3GPP TSG RAN meeting #90e										RP-20xxxx

Electronic Meeting, Dec. 7 - 11, 2020

Status Report to TSG

Agenda item:			9.8.8

| WI / SI Name | NR Multicast and Broadcast Services |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_MBS |  |  |  |  |
| Unique ID | 860048 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201038 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2021 | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: RAN1: 15% RAN2: 20% RAN3: 25% | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | Zhenzhen CAO |
|  | Company | Huawei |
|  | Email | caozhenzhen@huawei.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

Due to COVID-19 pandemic, this WI has been rescheduled and started only from Q3/2020 which is 6 months delay compared to the original schedule agreed in RP- RP-193248. For the new schedule, TUs need to be allocated for WG meetings in Q4/2020 and onwards.

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1#103-e agreements

Mechanisms to support group scheduling for RRC_CONNECTED UEs

Agreements: For convenience of discussion, consider the following clarification as RAN1 common understanding.

- PTP transmission: For RRC_CONNECTED UEs, use UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule UE-specific PDSCH which is scrambled with the same UE-specific RNTI.

- PTM transmission scheme 1: For RRC_CONNECTED UEs in the same MBS group, use group-common PDCCH with CRC scrambled by group-common RNTI to schedule group-common PDSCH which is scrambled with the same group-common RNTI. This scheme can also be called group-common PDCCH based group scheduling scheme.

- PTM transmission scheme 2: For RRC_CONNECTED UEs in the same MBS group, use UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule group-common PDSCH which is scrambled with group-common RNTI. This scheme can also be called UE-specific PDCCH based group scheduling scheme.

- Note: The ‘UE-specific PDCCH / PDSCH’ here means the PDCCH / PDSCH can only be identified by the target UE but cannot be identified by the other UEs in the same MBS group with the target UE.

- Note: The ‘group-common PDCCH / PDSCH’ here means the PDCCH / PDSCH are transmitted in the same time/frequency resources and can be identified by all the UEs in the same MBS group.

- FFS whether or not to have additional definition of transmission scheme(s)

Agreements: For RRC_CONNECTED UEs, if initial transmission for multicast is based on PTM transmission scheme 1, at least support retransmission(s) can use PTM transmission scheme 1.

- FFS: whether to support PTP transmission for retransmission(s).

- FFS: whether to support PTM transmission scheme 2 for retransmission(s).

- FFS: How to indicate the association between PTM scheme 1 and PTP transmitting the same TB.

- FFS: If multiple retransmission schemes are supported, then can different retransmission schemes be supported simultaneously for different UEs in the same group?

Working assumption:

For multicast of RRC-CONNECTED UEs, a common frequency resource for group-common PDCCH / PDSCH is confined within the frequency resource of a dedicated unicast BWP to support simultaneous reception of unicast and multicast in the same slot

- Down select from the two options for the common frequency resource for group-common PDCCH/ PDSCH

  - Option 2A: The common frequency resource is defined as an MBS specific BWP, which is associated with the dedicated unicast BWP and using the same numerology (SCS and CP)

    - FFS BWP switching is needed between the multicast reception in the MBS specific BWP and unicast reception in its associated dedicated BWP

  - Option 2B: The common frequency resource is defined as an ‘MBS frequency region’ with a number of contiguous PRBs, which is configured within the dedicated unicast BWP.

    - FFS: How to indicate the starting PRB and the length of PRBs of the MBS frequency region

- FFS whether UE can be configured with no unicast reception in the common frequency resource

- FFS on details of the group-common PDCCH / PDSCH configuration

- FFS whether to support more than one common frequency resources per UE / per dedicated unicast BWP subjected to UE capabilities

Agreements: Support TDM between one unicast PDSCH and one group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs.

Agreements: Support SPS group-common PDSCH for MBS for RRC_CONNECTED UEs

- FFS: use group-common PDCCH or UE-specific PDCCH for SPS group-common PDSCH activation/deactivation

- FFS: whether to support more than one SPS group-common PDSCH configuration per UE

- FFS: whether and how uplink feedback could be configured

- FFS: retransmission of SPS group-common PDSCH

Agreements: For PTM transmission scheme 1, the CORESET for group-common PDCCH is configured within the common frequency resource for group-common PDSCH.

- FFS: number of CORESET(s) for group-common PDCCH within the common frequency resource for group-common PDSCH

Agreements: For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, the CCE indexes are common for different UEs in the same MBS group.

Agreements: Down select from the two options for BDs/CCEs limit for Rel-17 MBS

- Option 1: the maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell defined in Rel-15 is kept unchanged for Rel-17 MBS.

- Option 2: For UEs supporting CA capability, the budget of BDs/CCEs of an unused CC can be used for group-common PDCCH to count the number of BDs/CCEs, which is similar to the method used for multi-DCI based multi-TRP in Rel-16.

Agreements:For RRC_CONNECTED UEs, support inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots (mandatory for the UE supporting MBS).

Agreements:Further study the following cases for simultaneous reception of unicast PDSCH and group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs.

- Case 1: support TDM between multiple TDMed unicast PDSCHs and one group-common PDSCH in a slot

- Case 2: support TDM among multiple group-common PDSCHs in a slot

- Case 3: support TDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

- Case 4: support FDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

- Case 5: support FDM among multiple group-common PDSCHs in a slot

- FFS: maximum number of PDSCHs in a slot simultaneous received per UE

Agreements:For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, further study the following options.

- Option 1: Define a new search space type specific for multicast

- Option 2: Reuse the existing CSS type(s) in Rel-15/16

  - FFS: whether modifications are needed for multicast

- Option 3: Reuse the existing USS in Rel-15/16 with necessary modifications for MBS

  - FFS: detailed modifications

Agreements:No specification enhancement in Rel-17 to support SDM between unicast PDSCH and group-common PDSCH in a slot for RRC_CONNECTED UEs.

Agreements: For PTM transmission scheme 1, if Option 2A or Option 2B for common frequency resource for group-common PDCCH/PDSCH is agreed, the FDRA field of group-common PDCCH is interpreted based on the common frequency resource.

Agreements: For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, further study the following options for the monitoring priority of search space set

- Option 1: The monitoring priority of search space set for multicast is the same as existing Rel-15/16 CSS

- Option 2: The monitoring priority of search space set for multicast is the same as existing Rel-15/16 USS

- Other options are not precluded

- The monitoring priority is used at least for PDCCH overbooking case

  - FFS for other cases (e.g., to prune PDCCH in terms of whether it’s unicast or multicast, etc.)

Mechanisms to improve reliability for RRC_CONNECTED UEs

Agreements:

For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1, support at least one of the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE feedback ACK or NACK.

  - From UEs within the group perspective,

    - FFS: PUCCH resource configuration for ACK/NACK feedback e.g., shared or separate PUCCH resources.

  - FFS details including conditions for it to be used

- NACK-only based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE only feedback NACK.

  - From UEs within the group perspective, further down-select between:

    - FFS: PUCCH resource configuration for NACK only feedback.

  - FFS details including conditions for it to be used

- To decide in RAN1#104-e whether or not to support only one or both of the above schemes

  - If both are supported, FFS configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback

Agreements:

For RRC_CONNECTED UEs receiving multicast, for ACK/NACK based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is, down-select one of the following options:

- Option 1: shared with PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 2: separate from PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 3: Option 1 or option 2 based on configuration

Agreements:

For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource configuration for HARQ-ACK feedback for unicast.

- FFS PUCCH format

Agreements:

Enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between:

- Option 1: DCI

- Option 2: RRC configures enabling/disabling

- Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

- FFS: Option 4: MAC-CE indicates enabling/disabling

- FFS: Option 5: RRC configures the enabling/ disabling function and MAC-CE indicates enabling /disabling

Agreements:

For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, further down-select among:

- Opt 1: by DCI

- Opt 2: by RRC

- Opt 3: by RRC+DCI

- FFS: Opt 4: by MAC-CE

- FFS: Opt 5: by RRC+MAC-CE

- FFS details for each option.

- FFS further enhancements for configuration of slot-level repetition

Agreements:

From the perspective of RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 initial transmission, retransmission supports, for the purpose of down-selection, options are:

- Option 1: group-common PDCCH scheduled group-common PDSCH

- Option 2: UE-specific PDCCH scheduled PDSCH

  - Alt 1: PDSCH is UE-specific PDSCH

  - Alt 2: PDSCH is group-common PDSCH

- Option 3: both option 1 and option 2

- FFS other options

- FFS CBG based retransmission

Agreements:

FFS whether CSI feedback enhancement is needed for MBS, including but not limited:

- New CQI measurement

- New CSI report formats

- Targeted BLER

- CSI-RS configuration

- A-CSI-RS transmission triggering

- SRS configuration

Agreements:

For ACK/NACK based HARQ-ACK feedback if supported, both Type-1 and Type-2 HARQ-ACK codebook are supported for RRC_CONNECTED UEs receiving multicast,

- FFS details of HARQ-ACK codebook design.

- FFS whether enhanced Type-2 and/or Type-3 HARQ-ACK codebook is supported or not.

Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI.

- FFS details

Agreements:

- For RRC_IDLE/RRC_INACTIVE Ues, beam sweeping is supported for group-common PDCCH/PDSCH.

  - FFS: Details for support of beam sweeping for group-common PDCCH/PDSCH.

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource(s) for group-common PDCCH/PDSCH.

- the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured.

- FFS: the relation of the common frequency resource(s) (if configured) and initial BWP.

- FFS: whether to configure one/more common frequency resources

- FFS: configuration and definition details of the common frequency resource

Agreements: From physical layer perspective, for broadcast reception, the same group-common PDCCH and the corresponding scheduled group-common PDSCH can be received by both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs.

- FFS details.

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, CSS is supported for group-common PDCCH.

- FFS: reuse current CSS type, define a new CSS type, etc.

- FFS other details.

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, a CORESET can be configured within the common frequency resource for group-common PDCCH/PDSCH. CORESET0 is used by default if the common frequency resource for group-common PDCCH/PDSCH is the initial BWP and the CORESET is not configured.

- FFS: configuration details of the CORESET for group-common PDCCH/PDSCH

2.1.2	Remaining Open issues

- Group scheduling mechanism for MBS in RRC_CONNECTED state, including enhancements to enable simultaneous operation with unicast reception

- Reliability improvement mechanisms for MBS in RRC_CONNECTED state

- Required changes for MBS in RRC_IDLE/ RRC_INACTIVE states

2.2	RAN2

2.2.1	Agreements

RAN2#112-e agreements

Broadcast and multicast sessions support, RRC states and other aspects related to SA2 LS

For Rel-17, R2 specifies two modes:

1: One delivery mode for high QoS (reliability, latency) requirement, to be available in CONNECTED (possibly the UE can switch to other states when there is no data reception TBD)

2: One delivery mode for “low” QoS requirement, where the UE can also receive data in INACTIVE/IDLE (details TBD).

R2 assumes (for R17) that delivery mode 1 is used only for multicast sessions.

R2 assumes that delivery mode 2 is used for broadcast sessions.

The applicability of delivery mode 2 to multicast sessions is FFS.

No data: When there is no data ongoing for the multicast session, the UE can stay in RRC_CONNECTED. Other cases FFS

It is up to SA2 to decide whether the multicast session activation/deactivation mechanism is supported or not, and RAN2 will discuss if there is any RAN2 impacts based on SA2 inputs.

It is up to SA2 to decide on the support of local MBS service, and RAN2 will discuss the RAN2 impacts based on SA2 inputs.

In general, Information of MBS services/groups subscribed by the UE (e.g. TMGI) and QOS requirements of a MBS service should be provided to RAN. Detail information e.g. for PTM PTP switch if any is FFS.

The function of mapping from QoS flows to MBS RBs in SDAP is needed for NR MBS. TBD whether any SDAP header is needed.

Layer 2 architecture

(Working assumption) no SDAP functions other than “mapping from QoS flows to radio bearers” and “transfer of user plane data” are supported for MBS. FFS whether to support QoS flows to radio bearers remapping.

In general: RAN2 wait for SA3’s progress for discussing security issues. TBD whether we need to send LS to SA3.

RoHC (at least U-mode) can be configured for NR MBS bearers. This is applicable for Mcast, assume this is applicable also to broadcast.

RoHC is located at PDCP.

The reordering and in-order delivery function in PDCP is supported for NR MBS.

The following PDCP functions are also supported for NR MBS: transfer of data; maintenance of PDCP SNs; duplicate discarding. Other PDCP functions are FFS.

RLC AM is supported for PTP transmission of NR MBS.

RLC UM is supported for PTP transmission of NR MBS.

RLC UM is supported for PTM transmission of NR MBS.

RLC TM is not supported for PTP transmission of NR MBS.

RLC TM is not supported for PTM transmission of NR MBS.

FFS for PTM if multiplexing/de-multiplexing of different logical channels are to be supported in MAC for NR MBS.

Working assumption: RLC-AM for PTM is not supported (can be revisited but it means that proponents of RLC-AM for PTM need to demonstrate the need, to change this).

Service continuity

R2 aim to support lossless handover for MBS-MBS mobility for service that requires this (TBD which detailed scenario but at least PTP-PTP)

In order to support the lossless handover for 5G MBS services, at least DL PDCP SN synchronization and continuity between the source cell and the target cell should be guaranteed by the network side to realize. The design of specific approach to realize this can be involved with WG RAN3.

From network side, the source gNB may forward the data to the target gNB and the target gNB will deliver the forwarding data. Meanwhile, the SN STATUS TRANSFER should be extended to cover the PDCP SN for MBS data; Then (TBD after or in parallel) the UE receives the MBS in the target cell by the target cell according to target configuration.

From UE side, PDCP status report may be supported as well.

Idle/Inactive support

UE receives the MBS configuration (for broadcast/delivery mode 2) by BCCH and/or MCCH (TBD), and this can be received in Idle / Inactive mode. Connected mode FFS (dep on UE cap and where service is provided etc). A notification mechanism is used to announce the change of MBS Control information.

2.2.2	Remaining Open issues

- Remaining issues for delivery mode 1, including but not limited to:

  - L2 architecture and MBS bearer configuration;

  - Dynamic switch issues (e.g. transparent or any signaling needed);

  - Use cases of PDCP status reporting and retransmission;

  - Remaining issues for lossless support.

- Remaining issues for delivery mode 2, including but not limited to:

  - BCCH and/or MCCH based PTM configuration;

  - PTM configuration change notification.

- RAN2 aspects of group scheduling, including but not limited to:

  - DRX;

  - SPS as agreed by RAN1.

2.3	RAN3

2.3.1	Agreements

RAN3#110-e agreements

General

- Baseline CRs to TS38.300, TS 38.401, TS 38.463, TS 38.423, TS 38.470, and TS 38.410 are endorsed.

Reply LS to SA2

- Reply LS to SA2 agreed in R3-207059, provide answers to the SA2 questions, ask SA2 to clarify some aspects for the MBS activation/deactivation, provide feedback about the provision of QoS requirement and feedback that RAN3 will develop protocol support to control both transmission modes for shared N3 transport between the MB-UPF and the NG-RAN.

Necessary Enhancements to NG-RAN Architecture

- An NR MBS Session is identified by an NR MBS Session Identifier which is unique within one PLMN

- The following agreements from RAN3#109-e on NR multicast are also applicable for NR broadcast:

  - WA pending SA2 progress (to progress discussion in RAN3):

    - One or more QoS flows may be used within a single MBS session

    - Each MB QoS flow belongs to one MBS Session

    - Each MB QoS flow is associated with a QoS profile

    - NR MBS supports both GBR and non-GBR QoS

    - One Shared NG-U tunnel is used per MBS session.

  - We Define MBS session resource in analogy with PDU session resource, e.g. including radio part, CP part, NG-UP part, MBS context in RAN

  - MBS session resource establishment is requested by 5GC (similarly to the PDU session establishment for unicast)

- 5GC shared MBS traffic delivery: as specified in TS 23.501.

- Agree TPs for 38.300 and 38.401 in R3-206384.

Session Management over NG

- Agree to have MBS Session Start/Release procedure for Broadcast but naming is FFS.

- Agree TP for 38410 in R3-207054 to introduce NGAP procedures to support Broadcast.

- Agreed that only PTM applies for broadcast (i.e. no PTP).

Dynamic Change between PTP and PTM for UEs in RRC_CONNECTED State

- Restrict the terms PTP and PTM for RAN internal delivery decision for the various mode. Agreed that for broadcast only PTM is applicable and for Multicast both PTP and PTM are applicable; PTP and PTM definitions need to be further discussed

- PTP and PTM modes can be used simultaneously in the same cell.

- The PTP-PTM Switching function is only applicable for a multicast MBS Session and resides in NG-RAN node. It enables the NG-RAN node to decide for which UEs to use PTP or PTM (PTP, PTM to be defined with RAN2) for the MBS session.

- The NG-RAN node takes its decision based on information such as MBS Session QoS requirements, number of joined UEs, UE individual feedback on reception quality, and other criteria. The same QoS requirements apply regardless of the decision.

- One TP to 38.300 agreed in R3-207055.

Bearer Management over F1/E1

- Use a shared F1-U tunnel for PTM transmission of an MBS radio bearer for an MBS Session

- About F1-U transport establishment

        - Support the method that gNB-DU assigns the DL F1-U GTP-U tunnel info, provides it to gNB-CU-CP and then gNB-CU-CP forwards it to gNB-CU-UP.

        - FFS if IP multicast method is supported or not.

- About MBS Context

        - Provide the MBS Session id, QoS profile from gNB-CU to gNB-DU.

        - Provide the MBS Session id, QoS profile from gNB-CU-CP to gNB-CU-UP.

- About MBS Bearer management

        - F1/E1 MBS Bearer management procedure can be discussed, but details on e.g. information to signal are pending RAN2/SA2 progress.

      - One TP to 38.401 agreed in R3-207056.

MBS Transmission Area

- Broadcast session is associated with Broadcast service area which is provided by 5GC.

- On NG-C interface, Broadcast service area info (e.g. a list of cell IDs) is indicated in the NGAP MBS session resource signaling, for broadcast sessions. FFS for multicast session.

Mobility with Service Continuity for UEs in RRC_CONNECTED State

Mobility between MBS Supporting Nodes

- For multicast, NR MBS shall provide means for minimization of data loss during mobility

- These means may be partly network deployment/implementation partly protocol support

- For multicast, in order to allow the UE to detect loss of data or duplication of data, RAN3 shall continue discussing solutions to support alignment of PDCP SNs in between gNBs.

- (in continuation of last meeting agreements):

        - Xn Handover Request and NG Handover Request message contain MBS context information for the UE.

        - MBS context information within the UE context shall contain all MBS multicast session information the UE has joined.

- (confirming the agreement form last meeting, contained in st2 TP):

        - The MBS configuration decided at target gNB is sent to the UE via the source gNB (details e.g. RRC container etc. pending RAN2 progress).

        - RAN3 will work on concepts to enable coordinated assignment of PDCP SNs to MBS user data packets within a gNB and between gNBs (to be coordinated with RAN2 if needed). Details FFS.

- One TP to 38.300 agreed in R3-207057.

Mobility between MBS Supporting and non-MBS Supporting Nodes

- RAN3 to deprioritize any detailed study on mobility between MBS-supporting gNBs and non-MBS-supporting gNBs, with the exception of studying impacts on Session management, until SA2 clarifies requirements and achieves some basic agreements

Others

- The discussion on CHO for MBS is deprioritized in R17.

- Proposals for Handover enhancements on reliable and low-latency NR MBS are deprioritized in R17.

Others

- Reception of broadcast service is supported in Rel-17 and according to RAN2 agreement, UE RRC state is of no relevance for reception of broadcast.

- Confirm that session management for broadcast services should be discussed in RAN3 (Already covered in AI22.2.2)

- For broadcast services reception, service reception continuity issues should be discussed in RAN3 based on the progress in RAN2.

- Whether the reception of multicast services is supported in idle/ inactive mode and the impact to RAN3, is pending RAN2 progress.

2.3.2	Remaining Open issues

- Session management over NG

- MBS Bearer management over F1 and E1

- Mobility with Service Continuity for UEs in RRC_CONNECTED State

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#103-E contributions:

- R1-2008033	Updated NR MBS work plan	CMCC

- R1-2008940	Summary#1 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS			Moderator (CMCC)

- R1-2009504	Summary#2 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS			Moderator (CMCC)

- R1-2009573	Summary#3 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS			Moderator (CMCC)

- R1-2009629	Summary#4 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS

- R1-2009677	Summary#5 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS			Moderator (CMCC)

- R1-2009744	Summary#6 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS			Moderator (CMCC)

- R1-2008034	Discussion on group scheduling mechanisms	CMCC

- R1-2007556	Group scheduling for MC/BC services	FUTUREWEI

- R1-2007562	Resource configuration and group scheduling for RRC_CONNECTED UEs	Huawei, HiSilicon

- R1-2007637	Group scheduling for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

- R1-2007691	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	vivo

- R1-2007835	Discussion on group scheduling mechanism for RRC_CONNECTED UEs in MBS	CATT

- R1-2008064	Support of group scheduling for RRC_CONNECTED UEs	LG Electronics

- R1-2008192	On mechanisms to support group scheduling for RRC_CONNECTED UEs	Samsung

- R1-2008242	Group scheduling for NR Multicast and Broadcast Services	OPPO

- R1-2008375	Considerations on MBMS group scheduling for RRC_CONNECTED UEs	Sony

- R1-2008449	Discussion on group scheduling mechanism for RRC_connected UEs	Apple

- R1-2008826	Mechanisms to Support Group Scheduling for RRC_CONNECTED UEs	ZTE

- R1-2008833	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	ETRI

- R1-2008882	Group Scheduling Mechanisms to Support 5G Multicast / Broadcast Services for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

- R1-2008926	Discussion on group scheduling mechanism for NR MBS	Lenovo, Motorola Mobility

- R1-2008961	Discussion on NR MBS group scheduling for RRC_CONNECTED UEs	MediaTek Inc.

- R1-2009000	Group Scheduling for NR-MBS	Intel Corporation

- R1-2009055	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	Asia Pacific Telecom co. Ltd

- R1-2009165	On group scheduling mechanism for NR multicast and broadcast	Convida Wireless

- R1-2009238	On Optimal Multiplexing for Simultaneous Operation of Broadcast/Multicast and Unicast Services		BBC

- R1-2009274	Views on group scheduling for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

- R1-2009305	Mechanisms to support group scheduling for RRC_CONNECTED Ues	Ericsson

- R1-2009464	FL summary on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2009539	FL summary#2 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2009654	FL summary#3 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2009716	FL summary#4 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2009275	Views on reliability enhancement for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

- R1-2007557	Improving reliability for MC/BC services	FUTUREWEI

- R1-2007563	Mechanisms to improve reliablity for RRC_CONNECTED UEs	Huawei, HiSilicon

- R1-2007638	Study on the reliability for RRC_CONNNECTED UEs	CHENGDU TD TECH LTD.

- R1-2007692	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	vivo

- R1-2007836	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT

- R1-2008035	Discussion on reliability improvement	CMCC

- R1-2008065	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

- R1-2008193	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

- R1-2008243	UL feedback for RRC-CONNECTED UEs in MBMS	OPPO

- R1-2008450	Discussion on MBS reliability improvement for RRC_connected UEs	Apple

- R1-2008715	Reliability improvement for RRC_CONNECTED UEs in MBS	Potevio

- R1-2008827	Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

- R1-2008883	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

- R1-2008893	Views on improving reliability for RRC_CONNECTED UEs in MBS	Google Inc.

- R1-2008927	Discussion on reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

- R1-2008962	Discussion on HARQ operation for NR MBS reliable transmission	MediaTek Inc.

- R1-2009001	Mechanisms to Improve Reliability for NR-MBS	Intel Corporation

- R1-2009166	On reliability enhancement for NR multicast and broadcast	Convida Wireless

- R1-2009306	Discussion on reliability mechanisms for NR MBS	Ericsson

- R1-2009465	Feature lead summary on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states	Moderator (BBC)

- R1-2009553	Summary#2 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states	Moderator (BBC)

- R1-2009554	Summary#3 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states	Moderator (BBC)

- R1-2007837	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs			CATT, CBN

- R1-2007564	Discussion on multicast support for IDLE/INACTIVE UEs	Huawei, HiSilicon

- R1-2007639	Basic functions for MBS for RRC_IDLE/RRC_INACTIVE UEs	CHENGDU TD TECH LTD.

- R1-2007693	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE Ues			vivo

- R1-2008036	Discussion on NR MBS in RRC_IDLE/RRC_INACTIVE states	CMCC

- R1-2008066	Basic function for broadcast/multicast	LG Electronics

- R1-2008194	On basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	Samsung

- R1-2008244	Discussion on enhancements for IDLE and INACTIVE state UEs	OPPO

- R1-2008828	Basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs	ZTE

- R1-2008884	Basic Functions for Broadcast / Multicast for  RRC_IDLE / RRC_INACTIVE UEs	Nokia, Nokia Shanghai Bell

- R1-2008928	Basic functions for broadcast/multicast in idle/inactive states	Lenovo, Motorola Mobility

- R1-2009002	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

- R1-2009167	On NR multicast and broadcast for RRC_IDLE/RRC_INACTIVE UEs	Convida Wireless

- R1-2009276	Discussion on broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	Qualcomm Incorporated

- R1-2009307	Support for NR multicast reception in RRC Inactive/Idle	Ericsson

- R1-2007641	Effects of NR MBS on PDSCH and PDCCH	CHENGDU TD TECH LTD.

- R1-2007694	Other issues for Rel-17 MBS	vivo

- R1-2007838	Discussion on search space type definition for group scheduling	CATT

- R1-2008067	Other aspects for MBS	LG Electronics

- R1-2008245	PUCCH resource allocation for UL feedback in MBMS	OPPO

- R1-2008317	Resource for receiving MBS	Huawei, HiSilicon

- R1-2008829	Preliminary Simulation Results of Rel-17 MBS	ZTE

- R1-2009308	Assumptions for Performance Evaluations of NR-MBS	Ericsson

RAN2#112-e contributions:

- R2-2008751	Reply LS on RAN impact of FS_5MBS Study (RP-202086; contact: Huawei)	RAN

- R2-2008755	LS on RAN impact of FS_5MBS Study (S2-2006044; contact: Huawei)	SA2

- R2-2008768	Reply LS on RAN impact of FS_5MBS Study (SP-200884; contact: Huawei)	SA

- R2-2008791	Discussion on Requirement and Architecture of MBS	CATT

- R2-2008792	Reliability Enhancement for PTM Transmission	CATT

- R2-2008793	Open Issues on Dynamic PTM and PTP Switch	CATT

- R2-2008794	Open Issues on Mobility with Service Continuity	CATT

- R2-2008795	Discussion on Miscellaneous Issues	CATT

- R2-2008796	Summary of Email Discussion Post111-e906 MBS Idle mode support	CATT

- R2-2008797	Further Discussion on MBS Idle Mode Support	CATT, CBN

- R2-2008865	Considerations on Protocol stack and network architecture	OPPO

- R2-2008866	Discussion on reliability for MBS reception	OPPO

- R2-2008867	Dynamic PTM and PTP switching with service continuity	OPPO

- R2-2008868	Discussion on mobility with MBS Service continuity	OPPO

- R2-2008869	Discussion on MBS reception of idle or inactive mode UE	OPPO

- R2-2008874	Discussion on group-based scheduling for MBS	OPPO

- R2-2008929	Discussioin on the protocol stack for NR MBS	CHENGDU TD TECH LTD.

- R2-2008930	Dynamic switch between PTM and PTP with service continuity	CHENGDU TD TECH LTD.

- R2-2008931	Discussion on mobility with service continuity	CHENGDU TD TECH LTD.

- R2-2008932	Consideration on reliability for NR MBS	CHENGDU TD TECH LTD.

- R2-2008933	NR MBS for RRC_IDLE/RRC_INACTIVE UE	CHENGDU TD TECH LTD.

- R2-2008934	RAN2 related aspects for NR MBS	CHENGDU TD TECH LTD.

- R2-2008940	IDLE/INACTIVE UE support for NR MBS	TCL Communication Ltd.

- R2-2008945	Reliability and latency handling during NR multicast mobility	TCL Communication Ltd.

- R2-2008989	Dynamic switch between PTM and PTP for service continuity	Intel Corporation

- R2-2008990	MBS service continuity in mobility	Intel Corporation

- R2-2008991	MBS support for IDLE and INACTIVE states	Intel Corporation

- R2-2009034	NR Multicast PTM bearer RLC AM mode operation	Qualcomm Inc, British Telecom, Kyocera, FirstNet, AT&T

- R2-2009035	NR Multicast Broadcast mobility enhancements with service continuity	Qualcomm Inc

- R2-2009036	NR Multicast Vs Broadcast comparison and Radio Bearer Architecture aspects	Qualcomm Inc

- R2-2009037	Enhancements for supporting loss less PTM PTP switching	Qualcomm Inc

- R2-2009038	NR Multicast-Broadcast services and configuration for UEs in different RRC states	Qualcomm Inc

- R2-2009054	HO for NR MBS	MediaTek Inc.

- R2-2009103	Consideration on PTP/PTM switching	Shanghai Jiao Tong University

- R2-2009126	HARQ operation for NR MBS reliable transmission	MediaTek Inc.

- R2-2009127	Dynamic PTM-PTP switch	MediaTek Inc.

- R2-2009128	NR MBS Radio Bearer Structure	MediaTek Inc.

- R2-2009154	Discussion on reliability of MBS service	Spreadtrum Communications

- R2-2009155	Discussion on dynamic PTM PTP switch	Spreadtrum Communications

- R2-2009156	Discussion on sevice continuity during mobility	Spreadtrum Communications

- R2-2009157	MBS for Idle and Inactive mode UE	Spreadtrum Communications

- R2-2009196	MBS L2 Architecture, user plane and control plane	Intel Corporation

- R2-2009197	MBS service reliability improvement	Intel Corporation

- R2-2009283	Discussion on NR MBS structure allowing service for idle UEs	Futurewei

- R2-2009303	MBS Protocol Architecture and Logical Channel Aggregation	Futurewei

- R2-2009304	ARQ of PTM with Logical Channel Aggregation	Futurewei

- R2-2009305	Service Continuity during Dynamic PTM/PTP Switch with Logical Channel Aggregation	Futurewei

- R2-2009313	PDCP Operation for MBS	Nokia, Nokia Shanghai Bell

- R2-2009314	MBS split bearer configuration and PTP/PTM switching	Nokia, Nokia Shanghai Bell

- R2-2009315	Miscellaneous Aspects of MBS	Nokia, Nokia Shanghai Bell

- R2-2009319	Consideration on MBS support in idle/inactive modes	ETRI

- R2-2009320	Discussion on RAN level QoS handling for MBS service area	TCL Communication Ltd.

- R2-2009334	Updated NR MBS workplan	Huawei, CMCC, HiSilicon

- R2-2009335	Discussion on SA2 LS on RAN impact of FS_5MBS Study	Huawei, HiSilicon

- R2-2009336	Draft reply LS to SA2 on RAN impact of FS_5MBS Study	Huawei, HiSilicon

- R2-2009337	Summary of Email discussion Post111-e-904 MBS L2 Architecture	Huawei, HiSilicon

- R2-2009338	Reliability enhancement for NR MBS	Huawei, HiSilicon

- R2-2009339	Support of dynamic switch between PTP and PTM	Huawei, HiSilicon

- R2-2009340	Service continuity during inter-cell mobility	Huawei, HiSilicon

- R2-2009341	General aspects for NR MBS	Huawei, HiSilicon

- R2-2009342	RRC states for MBS reception and Idle/Inactive UE support	Huawei, HiSilicon

- R2-2009343	38.300 running CR for NR MBS	Huawei, HiSilicon

- R2-2009440	Dynamic PTP PTM switch	LG Electronics Inc.

- R2-2009441	MBS in IDLEI NACTIVE	LG Electronics Inc.

- R2-2009444	MBS service continuity	LG Electronics Inc.

- R2-2009445	Consideration on properties of NR for multicastbroadcast	LG Electronics Inc.

- R2-2009461	General Considerations on Mobility with Service Continuity	Samsung R&D Institute India

- R2-2009494	Consideration on MBS transmission reliability	Apple

- R2-2009495	PTM PTP switch with MBS service continuity	Apple

- R2-2009496	Mobility with MBS service continuity	Apple

- R2-2009497	MBS reception in CONNECTED state	Apple

- R2-2009498	MBS reception in IDLE/INACTIVE state	Apple

- R2-2009537	Group Scheduling and Multiplexing Aspects	Samsung R&D Institute India

- R2-2009555	IDLE and INACTIVE state UE operation	Nokia, Nokia Shanghai Bell

- R2-2009575	Reliable MBS Transmission	Sharp

- R2-2009576	Dynamic switch between PTP and PTM	Sharp

- R2-2009579	Discussion on introducing counting and UE interest indication mechanism for UE in idle/inactive mode	China Unicom

- R2-2009600	Reliability Enhancements for NR MBS	Samsung

- R2-2009601	PTM PTP Switching and MBS Bearer Type	Samsung

- R2-2009611	IDLE /IN_ACTIVE UE support of MBS	NEC

- R2-2009612	Reliability of NR MBS	NEC

- R2-2009613	Service Continuity for Connected mode UE	NEC

- R2-2009614	Simultaneous transmission of multicast/unicast	NEC

- R2-2009641	Discussion on the counting scheme for dynamically switching PTM and PTP	ITRI

- R2-2009668	Framework for NR MBS Broadcast and Multicast services	Lenovo, Motorola Mobility

- R2-2009673	Clarification on the dynamic switching in MAC	Beijing Xiaomi Mobile Software

- R2-2009674	UE assistance information for connected mobility	Beijing Xiaomi Mobile Software

- R2-2009740	L2 architecture for NR MBS	ZTE, Sanechips

- R2-2009741	Consideration on MBS reliability guarantee	ZTE, Sanechips

- R2-2009742	Dynamic mode switching for NR MBS	ZTE, Sanechips

- R2-2009743	Consideration on lossless handover for NR MBS	ZTE, Sanechips

- R2-2009744	Support of Idle and Inactive mode UEs for NR MBS	ZTE, Sanechips

- R2-2009822	draft_Reply LS on RAN impact of FS_5MBS Study	ZTE, Sanechips

- R2-2009879	On HARQ and RLC for 5G MBS reliability	Lenovo, Motorola Mobility

- R2-2009880	5G MBS dynamic switch between PTP and PTM with service continuity	Lenovo, Motorola Mobility

- R2-2009881	Connected Mode Mobility with Service Continuity	Lenovo, Motorola Mobility

- R2-2009883	Security for PTP and PTM switching	Sony

- R2-2009884	PTP/PTM MRB and RLM	Sony

- R2-2009902	Open issues on MBS idle mode support	MediaTek Inc.

- R2-2009953	MBS reception in Idle and Inactive mode	Ericsson

- R2-2009954	SA2 questions about RRC state transitions for multicast	Ericsson

- R2-2009959	PTM to PTP Dynamic Switch	Ericsson

- R2-2009960	Mobility for NR MBS	Ericsson

- R2-2009961	Reliability for multicast operation	Ericsson

- R2-2009962	Aspects of Group Sscheduling	Ericsson

- R2-2010064	On Stage-2 aspects and overview of NR MBS	Samsung

- R2-2010078	RRC IDLE/ INACTIVE aspects of NR MBS	Samsung

- R2-2010139	Dynamic PTM/PTP Switching	Convida Wireless

- R2-2010143	MBS Mobility Management	Nokia, Nokia Shanghai Bell

- R2-2010145	On NR multicast and broadcast for RRC_IDLE/RRC_INACTIVE UEs	Convida Wireless

- R2-2010160	On reliability enhancement for NR multicast and broadcast	Convida Wireless

- R2-2010214	General considerations on NR MBS	vivo

- R2-2010215	Discussion reliability for RRC_CONNECTED UEs	vivo

- R2-2010216	Dynamic PTM PTP switch for RRC Connected UE	vivo

- R2-2010217	MBS Service Continuity for RRC Connected UE	vivo

- R2-2010218	Control of transmission area and group scheduling	vivo

- R2-2010219	Discussion on Idle and Inactive mode UEs	vivo

- R2-2010234	Consideration of control plane aspects for NR MBS	Kyocera

- R2-2010382	Consideration on Reliability Enhancement for MBS	CMCC

- R2-2010383	Discussion on Dynamic PTM PTP switch with service continuity	CMCC

- R2-2010384	Discussion on Mobility with Service Continuity	CMCC

- R2-2010385	Summary of [Post111-e][905][MBS] Connected Mode Mobility with Service Continuity (CMCC)	CMCC

- R2-2010386	Discussion on Beam Level MBS Deployment	CMCC

- R2-2010387	Discussion on Idle and Inactive UE MBS Reception	CMCC

- R2-2010411	Discussion on user-plane channel structure for MBS	LG Electronics Inc.

- R2-2010412	Discussion on reliability improvement and UL feedback in NR multicast	LG Electronics Inc.

- R2-2010643	Discussion on UE mode in CONNECTED states	TD Tech

- R2-2010644	Discussion on MBS support for UE in IDLE and INACTIVE states	TD Tech

- R2-2011022	Summary of [AT112-e][036][MBS] SA2 LS on MBS	Huawei, HiSilicon

- R2-2011133	Draft reply LS to SA2 on RAN impact of FS_5MBS Study	Huawei, HiSilicon

- R2-2011170	[DRAFT] Reply LS on RAN impact of FS_5MBS Study	Huawei, HiSilicon

- R2-2011271	Reply LS on RAN impact of FS_5MBS Study	RAN2

RAN3#110-e contributions:

- R3-205907	Introduction of NR MBS	Nokia, Nokia Shanghai Bell	draftCR

- R3-205909	Introduction of NR MBS	Huawei, CMCC	CR

- R3-205982	Discussion on NR MBS architecutre	CHENGDU TD TECH LTD.	discussion

- R3-206027	MBS bearer setup in F1 and E1	Samsung	discussion

- R3-206028	TP for MBS BLCR for 38.473-Addition of MBS feature	Samsung	other

- R3-206029	Dynamic Change Between PTP and PTM	Samsung	discussion

- R3-206030	Mobility Between MBS Supporting Nodes	Samsung	discussion

- R3-206031	TP for MBS BLCR for 38.413-Session Management over NG	Samsung	other

- R3-206058	Dynamic switch between PTM and PTP for RRC_CONNECTED UE	CHENGDU TD TECH LTD.	discussion

- R3-206059	Mobility between MBS supporting nodes	CHENGDU TD TECH LTD.	discussion

- R3-206060	Mobility between an MBS supporting node and a NON-MBS supporting node	CHENGDU TD TECH LTD.	discussion

- R3-206176	Session management procedure	Qualcomm Incorporated	discussion

- R3-206177	Loss-less handover procedure for NR multicast	Qualcomm Incorporated	discussion

- R3-206207	MBS assistance information	Intel Deutschland GmbH	discussion

- R3-206244	Enhancement to NG-RAN architecture for MBS	Nokia, Nokia Shanghai Bell	other

- R3-206245	MBS Stage 2 for Session management	Nokia, Nokia Shanghai Bell	other

- R3-206247	MBS Session management for joining/leaving	Nokia, Nokia Shanghai Bell	other

- R3-206248	MBS Session Management for NG-U resources setup	Nokia, Nokia Shanghai Bell	other

- R3-206250	MBS Stage 2 for PTP-PTM Switching	Nokia, Nokia Shanghai Bell	other

- R3-206251	MBS to MBS Mobility Key Principles	Nokia, Nokia Shanghai Bell	other

- R3-206252	MBS Stage 2 for Mobility Management 	Nokia, Nokia Shanghai Bell	other

- R3-206253	MBS Mobility Control Plane	Nokia, Nokia Shanghai Bell	other

- R3-206254	Support of MBS Seamless Mobility 	Nokia, Nokia Shanghai Bell	other

- R3-206299	Discussion on dynamic change between PTM and PTP	CATT	discussion

- R3-206300	Discussion on general principle for MBS service continuity	CATT	discussion

- R3-206301	Possible solutions to minimise data loss	CATT	discussion

- R3-206302	TP on TS 38.300 on MBS service continuity	CATT	other

- R3-206304	Minimising data loss on handover from non-MBS gNB toward MBS gNB	CATT	discussion

- R3-206305	Service continuity from MBS gNB toward non-MBS gNB	CATT	discussion

- R3-206306	Some Issues on Architecture of MBS	CATT	discussion

- R3-206307	Consideration on MBS Transmission Area	CATT	discussion

- R3-206309	MBS reception of Idle and In-active Ues	CATT	discussion

- R3-206310	Discussion on NG session management	CATT,CBN	discussion

- R3-206311	Draft LS to SA2 on multicast service area	CATT	LS out

- R3-206312	Introduction of MBS(BL CR for 38.463)	CATT	CR

- R3-206383	Introduction of NR Multicast and Broadcast Services	Ericsson	CR

- R3-206384	[TP for BL CR 38.300/38.401] Capturing agreements on NG-RAN architecture for 5G MBS	Ericsson	other

- R3-206385	[TP for BL CR 38.300] Session Management for NR MBS	Ericsson	other

- R3-206386	[TP for NGAP BL CR] - MBS Session Resource Management for NR MBS	Ericsson	other

- R3-206387	[TP for BL CR 38.300] Further Aspects of Dynamic PTP/PTM change for RRC_CONNECTED Ues	Ericsson	other

- R3-206388	On F1/E1 bearer management aspects	Ericsson	discussion

- R3-206389	On 5G MBS Service Area	Ericsson	discussion

- R3-206390	Minimization of data loss at mobility between MBS supporting nodes	Ericsson	discussion

- R3-206391	[TP for BL CR 38.413] Session control for Xn and NG based mobility	Ericsson	other

- R3-206392	[TP for BL CR XnAP] Session Control for Xn mobility	Ericsson	other

- R3-206393	Session control for Xn and NG based mobility - interworking with NG-RAN nodes not supporting MBS	Ericsson	discussion

- R3-206394	[TP for BL CR 38.413] Session control for Xn based mobility - interworking with NG-RAN nodes not supporting MBS	Ericsson	other

- R3-206395	Further aspects for Session control signalling design - Dual Connectivity	Ericsson	discussion

- R3-206396	IDLE and INACTIVE Support for NR MBS	Ericsson	discussion

- R3-206408	(TP to TS 38.300 BL CR) Inclusion of the agreements of RAN3#109	Huawei	other

- R3-206409	(TP to TS 38.401 BL CR) Inclusion of the agreements of RAN3#109	Huawei	other

- R3-206410	[DRAFT] Reply LS on LS on RAN impact of FS_5MBS Study	Huawei	LS out

- R3-206411	(TP to TS 38.410 BL CR) Session Management over NGAP	Huawei	other

- R3-206412	(TP to TS 38.401 BL CR) Decision on PTP and PTM	Huawei	other

- R3-206413	(TP to TS 38.401 BL CR) Bearer management over F1 and E1	Huawei	other

- R3-206414	(TP to TS 38.470 BL CR) Bearer management over F1 interface	Huawei	other

- R3-206415	(TP to TS 38.460 BL CR) Bearer management over E1 interface	Huawei	other

- R3-206416	(TP to TS 38.300 BL CR) Mobility procedure between MBS supporting nodes	Huawei	other

- R3-206417	(TP to TS 38.401 BL CR) Mobility procedure between MBS supporting nodes	Huawei	other

- R3-206418	Minimize Data Loss during Mobility between MBS supporting nodes	Huawei	discussion

- R3-206419	[DRAFT] LS on minimizing data loss during MBS mobility	Huawei	LS out

- R3-206420	Mobility between MBS supporting and non-supporting nodes	Huawei	discussion

- R3-206421	(TP to TS 38.410 BL CR) Support of Broadcast in NR MBS	Huawei	other

- R3-206483	Provisioning of Broadcast Services in RAN	Lenovo, Motorola Mobility	discussion

- R3-206484	Introduction of NR MBS	Lenovo, Motorola Mobility	CR

- R3-206485	Shared Delivery over F1-U for 5G MBS	Lenovo, Motorola Mobility	discussion

- R3-206486	MBS Session Management over NG interface	Lenovo, Motorola Mobility	discussion

- R3-206487	Configuration and Dynamic switch between PTP and PTM	Lenovo, Motorola Mobility	discussion

- R3-206488	5G MBS Transmission Mode and Area Control within NG-RAN	Lenovo, Motorola Mobility	discussion

- R3-206489	PDCP Count Value Synchronization and Data Forwarding	Lenovo, Motorola Mobility	discussion

- R3-206490	Support CHO for NR MBS with Service Continuity	Lenovo, Motorola Mobility	discussion

- R3-206526	MBS BL CR for TS38.410	ZTE	CR

- R3-206527	Discussion on MBS session management	ZTE	discussion

- R3-206528	Dynamic mode switching for NR MBS	ZTE	discussion

- R3-206529	Bearer Management for NR MBS	ZTE	discussion

- R3-206530	Discussion on multicast & broadcast transmission area	ZTE	discussion

- R3-206531	Consideration on MBS mobility procedure	ZTE	discussion

- R3-206532	Consideration on lossless handover for NR MBS	ZTE	discussion

- R3-206533	Discussion on UE mobility between an MBS-supporting gNB and a non-MBS-supporting gNB	ZTE	discussion

- R3-206534	Discussion on Broadcast service continuity in NR MBS	ZTE	discussion

- R3-206556	Discussion on support of reliable and low-latency NR multicast mobility 	TCL Communication Ltd.	discussion

- R3-206650	Issues on dynamic change between PTP and PTM	LG Electronics	discussion

- R3-206747	Consideration on Bearer Type Change for MBS Mobility	LG Electronics	discussion

- R3-206776	Consideration on Dynamic Control of the Broadcast/Multicast Transmission Area	LG Electronics	discussion

- R3-206784	On NG-RAN Architecture for 5G MBS	CMCC	discussion

- R3-206785	Session management over NG interface	CMCC	discussion

- R3-206802	Discussion on Mobility with Service continuity	CMCC	discussion

- R3-206835	Consideration on MBS context management over F1 and E1	CATT	discussion

- R3-206862	CB: # 23_NTN_RegUpdate_and_Paging - Summary of email discussion	Huawei - moderator	discussion

- R3-206863	CB: # 21_NTN_CellRelation - Summary of email discussion	ZTE - moderator	discussion

- R3-206865	CB: # 20_NTNcountry_specific_routing - Summary of email discussion	China Telecom, Qualcomm - moderator	discussion

- R3-206904	CB: # 28_MBS_general - Summary of email discussion	Huawei - moderator	discussion

- R3-206906	CB: # 57_MBS_LSs - Summary of email discussion	Nokia - moderator	discussion

- R3-206907	CB: # 58_MBSarch - Summary of email discussion	Ericsson - moderator	discussion

- R3-206908	CB: # 59_MBSsessionMgmt_NG - Summary of email discussion	Nokia - moderator	discussion

- R3-206909	CB: # 60_MBS_PTP-PTMdynChg - Summary of email discussion	Nokia - moderator	discussion

- R3-206910	CB: # 61_MBS_F1-E1bearerMgmt - Summary of email discussion	Huawei - moderator	discussion

- R3-206912	CB: # 63_MBSmobility_supporting_nodes - Summary of email discussion	Ericsson - moderator	discussion

- R3-206913	CB: # 64_MBSmobility_non-supporting_nodes - Summary of email discussion	ZTE - moderator	discussion

- R3-206914	CB: # 65_MBSmobility_AOB - Summary of email discussion	Lenovo - moderato	discussion

- R3-206915	CB: # 66_MBS_AOB - Summary of email discussion	CATT - moderator	discussion

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template