3GPP TSG-RAN WG1 #103-e		R1-20xxxxx

eMeeting, Oct 26th – Nov 13th, 2020

Source: 	Moderator (Ericsson)

Title:      	Summary of Email discussion [103-e-NR-1024QAM-01]

Agenda item:	8.16

Document for:	Discussion and Decision

1 Introduction

This document summarizes the discussions for email thread [103-e-NR-1024QAM-01]  under agenda item 8.16 for Introduction of DL 1024QAM for NR FR1.

2. Discussion

Below is a short moderator summary based on the tdocs [2-11] submitted for RAN1#103-e.

- 1024-QAM Constellation

  - Most companies propose reuse LTE 1024-QAM constellation [2] [3] [4] [5] [6] [9][10][11]

- CQI table with 1024-QAM

  - Reuse LTE CQI table with 1024-QAM entries [3][4][5][6][7][10] ([9] unless conflicts with MCS table)

  - At least 4 CQI entries for 1024-QAM [2]

  - Remove N entries from 256-QAM table, and add N new entries for 1024-QAM [11]

- MCS table with 1024-QAM entries

  - 5-bit vs 6-bit

    - Most companies seem to be OK with having a 5-bit MCS table [2][3][4][5] [6][7][9][10][11]

    - Some companies also suggest using a six-bit MCS table also [7][11]

      - Six-bit MCS table since five-bit MCS table may lead to reduced number of entries affecting transition point, non-uniform SE between MCS entries, better account for overhead

      - Adopt both 5-bits and 6-bits MCS tables [7]

        - Six-bit MCS table to indicate MCS and an overhead parameter to account for variable overhead.

    - Note the WI objective specifies the DCI overhead for MCS indication should be the same as in Rel-15.

  - MCS design principles

    - Most companies suggest starting with 256-QAM MCS table and remove M entries to accommodate M entries for 1024-QAM MCSes [2][3][4][5][6][7][10][11]

      - Several companies seem to be OK with M=5 [3][4][5][6][7][10], while one company mentioned M = 7 ([2]).

    - Implicit MCS entries

      - Most companies seem to prefer having five total implicit MCS entries  with one modulation order per MCS entry ([2][3][4][5][6][7][9][10]) while one company proposed to consider multiple modulation order per MCS entry with less than five total implicit MCS entries([11]).

    - Explicit MCS entries  (with modulation order/Target Code rate/Spectral efficiency)

      - Four entries for 1024-QAM [3][4][5][6][7][10]

      - Six entries for 1024-QAM [2]

      - Evaluate more [9][11] (including evaluation assumptions)

      - Regarding M=5 entries to be removed, different alternatives were mentioned:

        - remove {5,7,9,12,14} from the 256-QAM table [4][5][7]

        - remove {6, 8, 10, 12, 14} from the 256QAM table [3]

        - remove {2,4,6,8,10} from the 256-QAM table [10]

        - remove {1,3,5,7,9} from the 256-QAM table [11]

- RRC configuration and DCI formats

  - Configuration of 1024QAM CQI/MCS[3][9][10]

  - DCI formats and RNTIs for which 1024QAM is used/not used [3][5][9][10]

- Remaining aspects of 1024-QAM including spec impacts, etc

  - Specification impacts [6][9][10] – 38.201, 38.211, 38.212 38.214, including

    - PTRS reception procedure

    - LBRM procedure

  - Processing time relaxation [6]

  - UE capability reporting[3]

  - System-level simulation to identify the cell size(s) [2]

1st round

Proposal 1

- For supporting 1024-QAM in NR downlink, adopt the LTE 1024-QAM constellation.

Companies are requested to indicate their view about the above proposal in the Table below.

| Company Name | support/not support | Comments (Proposal 1) |
| --- | --- | --- |
| Intel | Support |  |
| Samsung | Support |  |
| vivo | Support |  |
| CATT | Support |  |
| ZTE,Sanechips | Support |  |

Proposal 2

- For supporting 1024-QAM in NR downlink, adopt the LTE 1024-QAM CQI table.

Companies are requested to indicate their view about the above proposal in the Table below.

| Company Name | support/not support | Comments (Proposal 2) |
| --- | --- | --- |
| Intel | Support |  |
| Samsung | Support |  |
| vivo | Support | Suggest to replace the entry {853, 8.3321} by {853, 8.3301} to ensure that the value of SE can be calculated according to the value of code rate. (853/1024*10≈8.3301) |
| CATT | Support | Fine with vivo’s proposal. |
| ZTE,Sanechips | Support |  |

Proposal 3

- For supporting 1024-QAM in NR downlink, adopt a five-bit MCS table with 1024-QAM entries.

  - Remove M (=5) entries from the NR 256QAM MCS table and add M new entries for 1024QAM

  - Introduce one implicit MCS entry corresponding to 1024QAM

  - Introduce 4 explicit MCS entries corresponding to 1024-QAM modulation

    - Highest MCS corresponding to code rate 948/1024, 1024-QAM

Companies are requested to indicate their view about the above proposal in the Table below.

| Company Name | support/not support | Comments (Proposal 3) |
| --- | --- | --- |
| Intel | Support |  |
| Samsung | Support |  |
| vivo | Support |  |
| CATT | Support |  |
| ZTE,Sanechips | Support |  |

Discussion point 1

- Indicate your preference on which M=5 MCS entries from 256-QAM can be removed, in order of preference from 1st to last.

    - Alt 1: remove {5,7,9,12,14} from the 256-QAM table

    - Alt 2: remove {6, 8, 10, 12, 14} from the 256QAM table

    - Alt 3: remove {2,4,6,8,10} from the 256-QAM table

    - Alt 4: remove {1,3,5,7,9} from the 256-QAM table

Companies are requested to indicate their view about the above discussion point in the Table below.

| Company Name | Preferred Alternative (most preferred first) | Comments (Discussion point 1) |
| --- | --- | --- |
| Intel | Alt 1 | Low MCS should be kept as they were already sub-sampled for 256QAM, i.e. Alt 3 and Alt 4 are not good options |
| Samsung | Alt. 1 | Do not support alts. 3 and 4 as these are not quite aligned with LTE 1024 QAM table. |
| vivo | Alt 2 | Follow NR MCS table design principle, CQI entries should be kept, so {5,7,9} should be kept as they were already included in the 1024-QAM CQI table. In addition, SE vs SNR curves at BLER=10% in fig.1 indicate that more equally spaced SNR can be obtained by adopting Alt 2, instead of Alt 1. Fig.1 SE vs SNR curves at BLER=10% by using NR LDPC coding |
| CATT | Alt. 1 |  |
| ZTE,Sanechips | Alt 1 | Firstly, we agree with the comments from Intel. Secondly, Alt 1 is based on the following agreements in LTE. Agreements in RAN1#90bis: - For introduction of 1024QAM MCS table: - Remove M entries from the 256QAM table while maintaining (close to) uniformly spaced SE, while keeping the lowest MCS - Add M new entries for 1024QAM, with (close to) uniformly spaced SE - Including 1 entry to support re-transmission with 1024 QAM Agreements in RAN1#92: The removed entries from 256QAM table are {5, 7, 9, 12, 14}.. Hence, we think Alt 1 is aligned with the following objective. - Specify corresponding MCS table with 1024QAM entries as defined in E-UTRA |

Discussion point 2

- Indicate your preference on which 4 explicit MCS entries for 1024-QAM can be added.

  - Alt 1:

| Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- |
| 10 | 806 | 7.8711 |
| 10 | 853 | 8.3321 |
| 10 | 900.5 | 8.7939 |
| 10 | 948 | 9.2578 |

  - Alt 2:

| Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- |
| 10 | 841 | 8.2129 |
| 10 | 885 | 8.6426 |
| 10 | 916.5 | 8.9502 |
| 10 | 948 | 9.2578 |

  - Alt 3:

| Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- |
| 10 | 805.5 | 7.8662 |
| 10 | 853 | 8.3301 |
| 10 | 900.5 | 8.7939 |
| 10 | 948 | 9.2578 |

Companies are requested to indicate their view about the above discussion point in the Table below.

| Company Name | Preferred Alternative | Comments (Discussion point 2) |
| --- | --- | --- |
| Intel | Alt 1 | Alt 2 doesn’t have the entry {853, 8.3321} from CQI table |
| Samsung | Alt 1 | According to proposal 2, if LTE CQI table is just reused, alt 1 is straightforward way. |
| vivo | Alt 3 | Prefer to replace the entry {853, 8.3321} by {853, 8.3301} to ensure that the value of SE can be calculated according to the value of code rate. (853/1024*10≈8.3301) Then for {805.5, 7.8662}, which SE is closer to the mean of the SEs of the previous and next entries compared to{806, 7.8711}, can be obtained by interpolating. |
| CATT | Alt 1 or Alt 3 |  |
| ZTE,Sanechips | Alt 1 | We support Alt 1. Reasons are as follows: - The spacing of SE in Alt1 is uniform, which satisfies with the following agreements in LTE, but it is obvious that the SE spacing in Alt2 is unequal . Agreements in RAN1#90bis: - For introduction of 1024QAM MCS table: - Remove M entries from the 256QAM table while maintaining (close to) uniformly spaced SE, while keeping the lowest MCS - Add M new entries for 1024QAM, with (close to) uniformly spaced SE - Including 1 entry to support re-transmission with 1024 QAM - Alt 1 includes the entry {853, 8.3321} from CQI table, but Alt 2 does not. - Simulation results in R1-2007977 show an equal SNR spacing between the adjacent MCS entries of Alt 1. |

Proposal 5

- Introduce new RRC signaling to indicate use of 1024-QAM CQI table.

| Company Name | support/not support | Comments (Proposal 5) |
| --- | --- | --- |
| Intel | Support |  |
| Samsung | Support |  |
| vivo | Support |  |
| CATT | Support |  |
| ZTE,Sanechips | Support |  |

Proposal 6

- Introduce new RRC signaling to indicate use of 1024-QAM MCS table for DCI format 1_1.

Companies are requested to indicate their view about the above proposal in the Table below.

| Company Name | support/not support | Comments (Proposal 6) |
| --- | --- | --- |
| Intel |  | The proposal should be modified to include “at least for DCI format 1_1” given the discussion point #3 |
| Samsung | Support |  |
| vivo | Support |  |
| CATT | Support | Agree with Intel’s addition. |
| ZTE,Sanechips | Support |  |

Discussion point 3

- Can 1024-QAM MCS table can be used with DCI format 1_2 ?

  - If yes, indicate your preferred alternative for enabling it.

    - Alt 1: Separate RRC signaling is used for DCI format 1_2

    - Alt 2 : Same RRC signaling applies to both DCI format 1_1 and DCI format 1_2

Companies are requested to indicate their view about the above discussion point in the Table below.

| Company Name | Yes/No | Preferred Alternative (if yes) | Comments (Discussion point 3) |
| --- | --- | --- | --- |
| Intel | Yes | Alt 2 | Open to consider DCI specific modulation, but the gains should be justified. |
| Samsung | Yes | Alt. 1 | Can provide better flexibility. It is noted that almost RRC parameters for DCI format 1_2 have been designed separately with DCI format 1_1. No strong point to have same RRC signaling. |
| vivo | No |  | DCI format 1_2 is used for URLLC scheduling, the motivation is unclear to use 1024QAM to achieve high reliability and low latency. |
| CATT | Yes | Alt. 1 | Agree with Samsung. |
| ZTE,Sanechips | No |  | DL 1024QAM is mainly deployed in FWA, IAB backhaul link and CPE link,etc. DCI format 1_2 is designed for URLLC. Hence, 1024-QAM MCS table cannot be used with DCI format 1_2. |

Proposal 7

- 1024-QAM MCS table can be used only with DCI format with CRC scrambled by C-RNTI or CS-RNTI.

Companies are requested to indicate their view about the above proposal in the Table below.

| Company Name | support/not support | Comments (Proposal 7) |
| --- | --- | --- |
| Intel | Support |  |
| Samsung | Support |  |
| vivo | Support |  |
| CATT | Support |  |
| ZTE,Sanechips | Support |  |

2nd round proposals (TBD)

Including spec impacts, etc

3 Conclusions

TBD

4 References

- RP-202044, New WID on Introduction of DL 1024QAM for NR FR1, RAN Meeting #89e, Sep 2020

- R1-2007617	On support of DL 1024QAM for NR FR1	Huawei, HiSilicon

- R1-2007700	On supporting DL 1024QAM for NR FR1	vivo

- R1-2007846	DL 1024QAM for NR FR1	CATT

- R1-2007977	Discussion on DL 1024QAM for NR FR1	ZTE

- R1-2008201	Discussion on DL 1024QAM for NR FR1	Samsung

- R1-2009009	Support of 1024QAM	Intel Corporation

- R1-2009171	Work plan on supporting 1024 QAM	Rapporteurs (Nokia, Ericsson)

- R1-2009172	Considerations for NR DL 1024 QAM in FR1	Nokia, Nokia Shanghai Bell

- R1-2009209	1024QAM for NR DL	Ericsson

- R1-2009282	Introduction of 1024-QAM modulation for NR PDSCH	Qualcomm Incorporated