3GPP TSG RAN WG1 Meeting #103-e                                   R1-20xxxxx

E-meeting, October 26th - November 13th, 2020

Agenda Item:			8.8.2.1

Source:			Moderator (China Telecom)

Title:			[103-e-NR-CovEnh-04] Email discussion for PUSCH coverage enhancement

Document for:		Discussion

- Introduction

In RAN #86 meeting, a new Rel-17 study item on NR coverage enhancements was approved [1]. The objective of this study item is to study potential coverage enhancement solutions for specific scenarios for both FR1 and FR2. The detailed objectives are as follows.

- The target scenarios and services include

  - Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1

  - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2.

  - TDD and FDD for FR1.

  - VoIP and eMBB service for FR1.

  - eMBB service as first priority and VoIP as second priority for FR2.

  - LPWA services and scenarios are not included.

- Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation

  - UL channels (including PUSCH and PUCCH) are prioritized for FR1.

  - Both DL and UL channels for FR2.

- Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services

  - The target channels include at least PUSCH/PUCCH

  - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions)

  - Study the additional enhanced solutions for FR2 if any

  - Evaluate the performance of the potential solutions based on link level simulation.

This contribution is a summary of the following email discussion:

[103-e-NR-CovEnh-04] Email discussion for PUSCH coverage enhancement – Jianchi (CT)

- 1st check point: 10/29

- 2nd check point: 11/4

- 3rd check point: 11/10

- Last check point 11/12

- Summary of potential solutions

2.1 Time domain based solutions

2.1.1 Enhancements on PUSCH repetition Type A

For enhancements on PUSCH repetition type A, there are three options:

- Option 1: Increase the maximum number of repetitions.

  - Support: HW, HiSi, CTC, Intel, CATT, Spreadtrum, OPPO, Sharp, Apple

- Option 2: The number of repetitions is counted on the basis of available UL slots.

  - Support: HW, HiSi, CTC, vivo, Intel, Samsung, ZTE, CATT, CMCC, Panasonic, OPPO, LG, Interdigital, NTT DOCOMO, ETRI, Apple

- Option 3: Flexible symbol resource allocation in different slots.

  - Support: CMCC, LG, ETRI

Option 1: Increase the maximum number of repetitions.

| Supported companies | HW, HiSi, CTC, Intel, CATT, Spreadtrum, OPPO, Sharp, Apple |
| --- | --- |
| Companies have concerns | vivo: If the actual number of repetitions is extended by simply extending the maximum number of repetitions, the actual number of repetition transmission may be not as expected. Sierra Wireless: For the eMBB use cases, do not recommend specifying increased repetition for the PUSCH. Nokia/NSB: The maximum number of repetitions for PUSCH repetition type A in release 15 is sufficient for FDD deployment. |
| Potential spec. impact | Increase the maximum number of repetitions for repetition type A, increase the entries of TDRA |

Option 2: The number of repetitions is counted on the basis of available UL slots.

| Supported companies | HW, HiSi, CTC, vivo, Intel, Samsung, ZTE, CATT, CMCC, Panasonic, OPPO, LG, Interdigital, NTT DOCOMO, ETRI, Apple |
| --- | --- |
| Companies have concerns | Nokia/NSB: The PUSCH repetition type B can be used to cope with the cancellation due to DL/UL collision in TDD deployment. Therefore, the consideration of counting repetition numbers based on non-consecutive slots for PUSCH repetition type A may not be needed. |
| Potential spec. impact | New mechanism to determine actual repetition times, the postponement rules for repetition type A should be supported |
| Other considerations | Samsung: SFI is used to ensure full flexibility for a gNB to adapt the slot structure to optimize resource utilization and a UE can transmit in all symbols indicated as UL symbols. Panasonic: If the cost of PDCCH is not high (i.e., no need of the large repetition for PDCCH) and/or DL/UL collision for TDD is not so frequent, just another PDCCH is used to request the remaining UL transmission would be more efficient because it can control whether the remaining repetition is sent or not. Therefore, this mechanism should be studied with the consideration of PDCCH overhead. |

- Option 3: Different symbol allocations applied in different slots.

| Supported companies | CMCC, LG, ETRI |
| --- | --- |
| Potential spec. impact | New mechanism to indicate UL symbols for each slot. |
| Other considerations | ETRI propose to study to indicate more than one SLIVs in a single UL grant. |

2.1.2 Enhancements on PUSCH repetition Type B

For enhancements on PUSCH repetition type B, there are three options:

- Option 1: Actual PUSCH transmission across the slot boundary and invalid symbols

  - Support: ZTE, CTC, Samsung, WILUS, ETRI

- Option 2: Actual PUSCH transmission across the slot boundary and invalid symbols, and the length of actual repetition larger than 14 symbols

  - Support: ZTE, CTC, Samsung, WILUS, Interdigital, NTT DOCOMO

- Option 3: RV enhancement

  - Support: vivo

Option 1: Actual PUSCH transmission across the slot boundary and invalid symbols

| Supported companies | ZTE, CTC, Samsung, WILUS, ETRI |
| --- | --- |
| Companies have concerns | Nokia/NSB: The enhancement on PUSCH repetition type B, e.g. actual repetition across the slot boundary, or the length of actual repetition larger than 14 symbols, is in the same category and should be discussed together with TB processing over multiple slots for PUSCH. The PUSCH coverage enhancement based on PUSCH repetition type B framework should be avoided. |
| Potential spec. impact | UE behaviour on handling of across slot boundary and invalid symbols, TBS determination, DM-RS pattern and DCI indication. |
| Other considerations | Intel: Enhancement on PUSCH repetition type B in time domain needs to be carefully studied in WI phase with considerations of impacts on UE implementation. |

Option 2: Actual PUSCH transmission across the slot boundary and invalid symbols, and the length of actual repetition larger than 14 symbols

| Supported companies | ZTE, CTC, Samsung, WILUS, Interdigital, NTT DOCOMO |
| --- | --- |
| Companies have concerns | Nokia/NSB: The enhancement on PUSCH repetition type B, e.g. actual repetition across the slot boundary, or the length of actual repetition larger than 14 symbols, is in the same category and should be discussed together with TB processing over multiple slots for PUSCH. The PUSCH coverage enhancement based on PUSCH repetition type B framework should be avoided. Spreadtrum: don’t support increasing maximum number of symbols for PUSCH. |
| Potential spec. impact | TBS determination, DM-RS pattern, DCI indication, SLIV table, hopping rules |
| Other considerations | WILUS: If across slot boundaries + actual repetition larger than 14 symbols are adopted, Additional consideration may be required for the hopping boundary determination Option 1: Slot boundary Option 2: Nominal repetition boundary Option 3: Aggregated actual repetition boundary Intel: Enhancement on PUSCH repetition type B in time domain needs to be carefully studied in WI phase with considerations of impacts on UE implementation. |

Option 3: Single TB, transmitted in parts over multiple actual repetition

| Supported companies | vivo |
| --- | --- |
| Potential spec. impact | enhanced or new rules of determining RV for each repetition, enhanced or new segment rules of actual repetitions. |

2.1.3 TB processing over multi-slot PUSCH

For TB processing over multi-slot PUSCH, there are two options:

- Option 1: TBS is determined based on single slot, transmitted in parts over multiple slots.

  - Support: Interdigital

- Option 2: TBS is determined based on multiple slots.

  - Support: IITH, IITM, CEWIT, Reliance Jio, Tejas Networks, Intel, CTC, LG, Qualcomm, Panasonic, Samsung, Interdigital, WILUS, Sharp

| Supported companies | vivo, IITH, IITM, CEWIT, Reliance Jio, Tejas Networks, Intel, CTC, LG, Interdigital, Qualcomm, Panasonic |
| --- | --- |
| Potential spec. impact | Uniform TDRA or start symbol and length, TBS and DMRS pattern, UE behaviour and related signalling. |
| Other considerations | vivo: For multi-slot PUSCH, applying the same TDRA over multiple slots would result in the discontinuous resource and inflexible allocation, the available resources cannot be fully utilized. CTC: - Option 1: TBS is determined based on single slot, transmitted in parts over multiple slots. - Option 2: TBS is determined based on multiple slots. - Option 2a: different RV version is transmitted in each slot. - Option 2b: different segment is transmitted in each slot. Nokia/NSB: There is a tradeoff between the time domain diversity gain from PUSCH repetition and the low coding rate gain brought by the potential TB processing over multiple slots. The applicability of this solution in TDD deployment is limited. |

2.1.4 OCC spreading based repetition

ZTE, Xiaomi: OCC spreading based PUSCH can be considered for NR coverage enhancement. Orthogonal code division multiplexing among different UEs can be considered in case of repetitions

2.1.5 Symbol-level repetition/combing

Panasonic: Symbol-level repetition could be considered as one of cross-slot channel estimation techniques combined with TB processing over multi-slot PUSCH. The same OFDM symbol is repeated in N consecutive symbols, where N is the number of repetitions. This structure is also suitable to perform symbol-level combining.

LG: Symbol-level combining should be studied to enhance PUSCH coverage. Symbol-level combining can be supported by applying the same RV value during a bundle of PUSCH slots.

2.1.6 TB interleaving

Sierra Wireless: Recommend that gaps between repeats be specified as a Rel 17 Coverage enhancement solution.

Figure 2a: Legacy scheduling (i.e. without gaps)

Figure 2b: No TBs scheduled within the gaps

Figure 2c: TBs scheduled within the gaps

Figure 2a illustrates how multiple TBs are in scheduled with repeats in legacy Rel 16 where each repeat is scheduled over contiguous slots. Figure 2b illustrates how a single TB is scheduled with a gap of 7 slots. Figure 2c illustrates the scheduling where the gap is filled with TBs #2-8. TBs #2-8 could be TBs for other users or for the same user as TB #1. If the TBs are all for the same user, this scheduling pattern provides the same data rate as if no gaps are used (i.e. figure 2a and 2c have the same data rate for that user).  Allowing the gaps to be filled with TBs for other users, improves the gNB’s scheduling flexibility which will result in lower latency for other users and improved capacity.

2.1.7 Early termination of PUSCH repetitions

| Supported companies | ZTE, CMCC, OPPO |
| --- | --- |
| Companies have concerns | Nokia/NSB: The potential advantage of introducing early termination of PUSCH repetition is unclear. |
| Potential spec. impact | Mechanism and signaling of early termination. |
| Other considerations | ZTE, Panasonic: DFI specified in Rel.16 NR-U for explicit ACK can be used as the starting point |

2.2 Frequency domain based solutions

2.2.1 Enhancements on inter-slot frequency hopping

- Inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions

  - Support: HW, HiSi, Xiaomi, vivo, ZTE, NEC, Sony, NTT DOCOMO, CTC, Spreadtrum, Apple

| Supported companies | HW, HiSi, Xiaomi, vivo, ZTE, NEC, Sony, NTT DOCOMO, CTC, Spreadtrum, Apple |
| --- | --- |
| Companies have concerns | Intel: Increasing the number of frequency hops from 2 to 4 for inter-slot frequency hopping may not be supported for NR coverage enhancement WI. |
| Potential spec. impact | Frequency hopping pattern and related signalling (RRC, DCI indication). |
| Other considerations | CATT: The suitable number of frequency hopping and the number of hopping offset candidates should be firstly determined, which may be dependent with BWP bandwidth. Then, it should be further studied how to flexibly indicate the hopping. Sony: For PUSCH frequency hopping, the gNB can dynamically adapt the frequency hopping pattern, based on which hops are more effective. |

2.2.2 Inter-slot frequency hopping with inter-slot bundling

- Inter-slot frequency hopping with inter-slot bundling to enable cross-slot channel estimation

  - Support: ZTE, Intel, CTC, NEC, Samsung, LG, Panasonic, Interdigital, Apple

| Supported companies | ZTE, Intel, CTC, NEC, Samsung, LG, Panasonic, Interdigital, Apple |
| --- | --- |
| Companies have concerns | CATT: Cross-slot channel estimation should be well studied before supporting this mechanism. |
| Potential spec. impact | Frequency domain hopping offset, time domain hopping interval and the related signalling, power consistency and the phase continuity within one bundle should be preserved. |
| Other considerations | Nokia/NSB: whether this solution should be supported or not depending on the outcome of cross-slot channel estimation solution. |

2.2.3 Enhancements on frequency hopping for PUSCH repetition type B

| Supported companies | CATT |
| --- | --- |
| Companies have concerns | vivo: Intra-repetition frequency hopping for PUSCH repetition type B should be deprioritized. Spreadtrum: Postpone the discussion on enhancements on frequency hopping for PUSCH repetition type B. |

2.2.4 Sub-PRB transmission

- Sub-PRB transmission with multi-slot aggregation for VoIP

  - Support: CTC, Sony, LG, NTT DOCOMO, Sierra Wireless, Samsung

| Supported companies | CTC, Sony, LG, NTT DOCOMO, Sierra Wireless, Samsung |
| --- | --- |
| Companies have concerns | vivo: sub-PRB transmission with single TB sized for multiple slots and transmitted over multiple slots suffers higher complexity and latency. Furthermore, sub-PRB transmission requires significant work on specification. Thus, sub-PRB transmission is not supported for PUSCH coverage enhancement. CATT: It can be foreseen that specifying Sub-PRB resource allocation requires heavy standard work, including sub-PRB pattern definition, FDRA/TDRA indication, hopping pattern within/between the PRBs, DMRS design, coexistence with legacy UEs, etc. Sub-PRB resource allocation may be supported, but the motivation needs more justification. Intel: Depending on coverage enhancement target for VoIP under various deployment scenarios in FR1 and FR2, further study is needed to conclude whether sub-PRB based resource allocation can be considered for PUSCH coverage enhancement. Spreadtrum: Too much specification efforts are needed. |
| Potential spec. impact | Sub-PRB pattern definition, FDRA/TDRA indication, hopping pattern within/between the PRBs, DMRS design, TBS determination, UE behaviour and related signalling. |
| Other considerations | Sierra Wireless: Recommend the 2 Tone Pi/2 BPSK (LTE-M) scheme be specified to improve coverage for VoIP. LTE-M and NB-IOT both support a near zero PAPR modulation schemes using the sub-PRB technique (2 tone DTF-spread pi/2 BPSK for LTE-M and single tone pi/2 BPSK for NB-IOT). The near-zero PAPR sub-PRB based modulation doesn’t increase coverage directly but instead reduce the need to define MPR and A-MPR values which would then require the UE to transmit at a higher power which increases coverage. CTC: - Option 1: TBS is determined based on sub-PRB and single slot. - Option 2: TBS is determined based on sub-PRB and multiple slots. For option 1, sub-PRB transmission can be in conjunction with repetition. For option 2, the basic principle is similar with TB processing over multi-slot PUSCH, while sub-PRB transmission can further benefit from PSD boosting. Samsung: frequency domain resource allocation, power control, DMRS related may or may not be needed, no spec impact on TBS determination. Nokia/NSB: There is a trade-off between Tx power per subcarrier vs SNR gain, which should be carefully considered. |

2.2.5 Enhancements on Intra-slot frequency hopping

| Supported companies | CTC, Spreadtrum, Nokia/NSB |
| --- | --- |
| Companies have concerns | Xiaomi: from the perspective of balancing complexity and performance gain, intra-slot frequency hopping should not to be supported. |
| Potential spec. impact | Frequency hopping pattern and related signalling (RRC, DCI indication). |
| Other considerations | Nokia/NSB: intra-frequency hopping with 3 hops and DMRS sharing. |

2.3 DM-RS enhancements

2.3.1 Cross-slot channel estimation

- Cross slot channel estimation

  - Support: HW, HiSi, Xiaomi, ZTE, ETRI, CTC, CMCC, NEC, Samsung, OPPO, Sharp, LG, Ericsson, Interdigital, NTT DOCOMO, Qualcomm, Intel, Panasonic, CATT, Apple

| Supported companies | HW, HiSi, Xiaomi, ZTE, ETRI, CTC, CMCC, NEC, Samsung, OPPO, Sharp, LG, Ericsson, Interdigital, NTT DOCOMO, Qualcomm, Intel, Panasonic, CATT, Apple |
| --- | --- |
| Potential spec. impact | Power consistency and phase continuity should be preserved, DMRS placement in special slot, DMRS configuration. |
| Other considerations | Vivo: To guarantee phase and power continuity, potential solution is to indicate UE the continuity/spatial relation among slots or transmissions implicitly or explicitly. ZTE: Further study whether phase continuity can be kept across slot boundary. CATT: It need to be further studied whether UE power un-consistency exists due to potential reasons like the updated pathloss measurement or modulation-order-dependent power reduction. NEC: The granularity of symbol number within one slot may not be suitable for cross channel estimation among multiple slots. To overcome this issue, different number of DMRS symbol in different slot could be studied. Panasonic: In order to support cross-slot channel estimation, RAN1 asks to RAN4 in what condition phase continuity can be kept. If cross-slot/cross-repetition channel estimation is supported, adaptive DMRS transmission, in which DMRS configuration can be different among slots or repetition could also be considered. Interdigital, Xiaomi: Support DMRS placement in a special slot which can be bundled with DMRS in the adjacent uplink slot. NTT DOCOMO: Cross-slot channel estimation with non-consecutive PUSCH slots should be also considered for the performance evaluation. |

2.3.2 Lower DMRS density

For lower DM-RS density, there are two options:

- Option 1: lower density in time domain, e.g., DMRS sharing among multiple PUSCH transmissions

  - Support: HW, HiSi, Spreadtrum, OPPO, Sharp, Nokia, NSB, xiaomi, Apple

- Option 2: lower density in frequency domain

  - ZTE

| Supported companies | HW, HiSi, Spreadtrum, OPPO, Sharp, Nokia, NSB, ZTE, xiaomi, Apple |
| --- | --- |
| Companies have concerns | CATT: DMRS enhancement is more related to MIMO enhancement, which may lead to heavy cross-topic specification work. It is suggested to justify the benefit of the DMRS density enhancement firstly. Intel: Lower DMRS density in time domain is not supported for PUSCH coverage enhancement. |
| Potential spec. impact | DM-RS pattern and related signaling. |

2.3.3 Higher DMRS density

| Supported companies | CTC: 1-comb DMRS (DM-RS with single port spans to occupy the whole DM-RS symbol) NTT DOCOMO: Denser DM-RS mapping (e.g. 3 additional DMRS for 2 symbol DMRS) |
| --- | --- |
| Companies have concerns | Intel, LG: Higher DMRS density in time domain may not be needed for PUSCH coverage enhancement. CATT: DMRS enhancement is more related to MIMO enhancement, which may lead to heavy cross-topic specification work. It is suggested to justify the benefit of the DMRS density enhancement firstly. |
| Potential spec. impact | DM-RS design and TBS determination. |

2.3.4 Adaptive DMRS configuration

| Supported companies | CMCC, Sony, Qualcomm |
| --- | --- |
| Other considerations | Qualcomm propose 2 options to enable efficient DMRS adaptation: - Option 1: Dynamic MAC-CE based activation + complementary DCI based selection of one of the activated DMRS configuration options. - Option 2: Single active DMRS configuration option that is dynamically activated by MAC-CE. Corresponding potential spec. impact: DMRS configurations, mechanism of DMRS configuration activation and selection. |

2.3.5 DM-RS balancing among frequency hops

Nokia/NSB: When PUSCH repetition type A and intra-slot frequency hopping are both applied, if an odd number of DMRS per slots is configured then the actual number of DMRS symbols per hop is not the same. This may lead to deterministic channel estimation accuracy differences across hops. This aspect should be considered when discussing DMRS enhancement solutions.

2.4 Power-domain based solutions

2.4.1 Waveform design to optimize MPR/A-MPR

| Supported companies | Interdigital, Qualcomm, Nokia, NSB |
| --- | --- |
| Companies have concerns | CATT: In RAN4, MPR/A-MPR is defined to put a requirement on range of UE power reduction. It only restricts the maximum reduced power, but the actual power reduction is up to UE implementation. The benefit for new waveform is not so clear, since the gain for optimizing power reduction is unclear and hard to be quantified. Anyway, RAN1 should firstly ask RAN4 the feasibility and availability, if it is indeed interesting and attractive. |
| Other considerations | Interdigital: For example, the gNB can determine based on e.g. uplink reference signals or based on power head room report if the UE needs more transmission power to achieve the performance target. The gNB can then indicate to the UE to use PAPR reduction techniques in the subsequent uplink transmissions. Qualcomm: Consider tone reservation principle for DFT-s-OFDM and CP-OFDM waveforms to further reduce PAPR. Nokia/NSB: The FDSS with spectral extension for QPSK is considered as potential solution to reduce MPR and to improve UL PUSCH coverage. |

2.4.2 Power boosting for pi/2 BPSK

IITH, IITM, CEWIT, Reliance Jio, Tejas Networks: Make pi/2 BPSK power boosting a function of the UL duty cycle. Send LS to RAN4 to study the feasibility of power boosting for pi/2 BPSK modulation beyond 26 dBm as a function of the UL duty cycle.

2.4.3 FDD high power UE

HW/HiSi: The most effective way to increase the SNR is directly using a higher instant output power at some specific slots while zero power is allocated to other slots of the FDD duration, thus maintaining the same total transmission power of all slots in the FDD duration and satisfying the SAR requirement.

Concerns:

CATT, OPPO, Sony: Power domain based enhancement should be carefully studied by RAN4 before starting any specification work in RAN1.

2.5 Spatial-domain based solutions

2.5.1 Multiple layer PUSCH transmission with DFT-S-OFDM

Ericsson:

- Non-coherent and partially coherent UE’s PAPR or cubic metric (CM) of multiple layer PUSCH transmission is not higher than 1-layer PUSCH transmission by coherent UE.

- Multiple layer PUSCH transmission with DFT-S-OFDM can improve PUSCH cell coverage.

- Multiple layer transmission is especially beneficial in the non-coherent UEs that are those most used in real deployment, since multi-layer transmission provides more power in these UEs.

  - Pure rank 1 transmission tends to be infrequent even for UEs in the poorest channel conditions when few gNB antennas are used.

  - When massive MIMO gNBs are used, rank 1 is almost never selected.

Concerns:

CATT: For multi-layer DFT-s-OFDM, the motivation is not so clear, since higher ranks are usually chosen only when the channel quality is good enough. In this case, the UL coverage is not the bottleneck, and the UE transmission power is not necessary to be high. Then, PAPR does not cause serious issue even if CP-OFDM waveform is applied, which is mandatorily supported by all NR UEs. In this case, CP-OFDM can be applied for multi-layer PUSCH transmission.

2.5.2 Open-loop/closed loop Tx diversity

NICT: The use of DFT-s-OFDM with Tx diversity should be one of the approaches for coverage enhancement.

Mitsubishi Electric: Alamouti-based transmit diversity is supported for PUSCH with DFT-s-OFDM.

OPPO: In order to harvest the spatial diversity gain, different PUSCH spatial filter parameters and different antenna ports can be applied for different PUSCH slots.

2.6 Others

2.6.1 Dynamic PUSCH waveform adaptation

Qualcomm: To enable efficient waveform adaptation in 5G NR, a new signaling mechanism for dynamic waveform reconfiguration is required.

- Option 1: Dynamic MAC-CE based activation of the active MCS table. Different RRC configured MCS tables are associated with different waveforms and the activated MCS table provides an implicit indication of the associated waveform.

- Option 2: A new hybrid MCS table will be defined as a combination of MCSs from the existing MCS tables for DFT-S-OFDM and CP-OFDM transmission schemes. PUSCH MSC idx signaled in a scheduling DCI (scheduled grant) or configured under ConfiguredGrantConfig in case of a configured grant for PUSCH will be used as an implicit signaling for the transmission scheme.

Potential specification impact:

For Option 1, MCS table configurations, mechanism for MCS table activation/selection based on MAC-CE signaling

For Option 2, new hybrid MCS table needs to be defined.

2.6.2 Coverage enhancement for voice

Ericsson:

- Indicate to CT1 and SA4 that 2KB SIP message sizes may impact VoNR coverage or setup latency in arduous coverage scenarios and ask if SigComP functionality can be supported to reduce SIP message overhead.

- Ask CT1/SA4 what SIP message packet sizes and arrival rates can be expected.

- Proposals (1st round)

- In RAN1 #102-e, following agreements have been achieved:

Agreements: - Study following power domain based solution for PUSCH enhancements - Waveform design to optimize MPR/A-MPR - [FDD high power UE] - Power boosting for pi/2 BPSK Note: if a LS to RAN4 (for the last two bullets) is deemed necessary, target sending the LS in the 1st week of RAN1#103-e.

- Q1: Is it necessary to send LS to RAN4?

- Q2: It is expected to receive reply LS from RAN4 during this meeting? If no, how to make conclusions in RAN1?

Companies are invited to answer the above questions.

| Companies | Comments |
| --- | --- |
| Sierra Wireless | Its unclear what the question(s) to RAN4 will be thus there is no need to send LS to RAN4. |
| Samsung | It seems no need to send an LS to RAN4. Also the questions to ask to RAN4 are unclear. |
| InterDigital | For waveform design to optimize MPR/A-MPR, we do not see a need to send LS to RAN4 |
| Intel | Share similar view as other companies. RAN1 needs to first understand better the questions for RAN4. |
| Panasonic | The question(s) to RAN4 should be clarified. |
| Nokia/NSB | We share similar concern as the FL that we may not receive the reply LS from RAN4 during this meeting and hence it is difficult to conclude the SI in this meeting. However, we are open to discuss the content of the LS, if the majority view is to send an LS to RAN4. |
| Xiaomi | Clarifying the question is the premise. |
| ZTE | For FDD higher power UE, we haven’t agreed to study yet. Thus, we don’t see a need to send LS to RAN4 at least for this one for now. Also, we agree that the questions for RAN4 should be clarified first. |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | For power boosting, we should definitely and immediately ask RAN4 whether it is possible to further boost the power beyond 26 dBm. Based on this the coverage goals will be determined. However, if sending an LS to RAN4 is a problem, we should at least decide in RAN1 which of these to be supported. We think due to 3 solutions being grouped together, there is confusion about which solutions are being talked about. Specifically for pi/2 BPSK power boosting, we have shown the gains achievable by power boosting. The group should consider studying this in more detail. |

- //Proposal 1-13: Conclusions or observations on details and specification impacts for each enhanced solution

- Proposal 1: Capture the followings into the TR

- Enhancements on PUSCH repetition type A were studied from several aspects, including increasing the maximum number of repetitions, the number of repetitions counted on the basis of available UL slots and flexible symbol resource allocation in different slots.

- Potential specification impacts of enhancements on PUSCH repetition type A include:

  - Increase the entries of TDRA, mechanism to determine actual repetition times, mechanism to indicate UL symbols for each slot.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Sierra Wireless | Consider making the sentence past tense (this goes for all the proposal to the TR). Enhancements on PUSCH repetition type A were studied from several aspects, including increasing the maximum number of repetitions, …. |
| Samsung | Fine in principle with the proposal but please note there are other aspects under discussion in Sec.2.1.2. We can come back to it after further discussion instead of agreeing now. |
| InterDigital | Support the proposal from the feature lead in principle. In addition, DMRS bundling between repetitions should be included in one of the proposals. A suggestion for the proposal is shown in Proposal 8. |
| Intel | In general, we are fine with the proposals. One comment for “mechanism to determine actual repetition times”, it may be better to update as “mechanism to determine transmission occasion of actual repetition times” to make it clearer. |
| NTT DOCOMO | We support the FL proposal. |
| Sharp | We are OK with the proposal. The wording suggested by Intel is preferred to us. |
| Nokia/NSB | We are fine with the FL’s proposal and the edits from Intel. |
| Xiaomi | We are fine with the FL proposal and the intel’s comment. |
| ZTE | We are fine the proposal. |
| LG | In general, we are fine with the proposal. For potential specification impacts of enhancements on PUSCH repetition type A, we would like to add “mechanism to determine actual starting OFDM symbol for each slot” |
| CATT | Fine with the proposal. |
| OPPO | Share similar view to update “mechanism to determine actual repetition times” in the proposal as “mechanism to determine transmission occasion of actual repetition times” |

- Proposal 2: Capture the followings into the TR

- Enhancements on PUSCH repetition type B are studied from several aspects, including actual PUSCH transmission across the slot boundary/invalid symbols and the length of actual repetition larger than 14 symbols.

- Potential specification impacts of enhancements on PUSCH repetition type B include:

  - TBS determination, DM-RS pattern, DCI indication, TDRA, hopping rules.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| InterDigital | Support the proposal from the feature lead in principle. In addition, DMRS bundling between repetitions should be included in one of the proposals. A suggestion for the proposal is shown in Proposal 8. |
| WILUS | Support FL proposal. |
| Intel | We are in general fine with the proposals, but not sure whether “DCI indication” is needed for potential spec impact. |
| Sharp | UCI multiplexing behaviour on PUCCH/PUSCH should be one of potential specification impacts for actual PUSCH transmission across the slot boundary. |
| Xiaomi | Support the FL proposal and in addition, we suggest to modify the “DCI indication” to“indication mechanism”. |
| ZTE | Support the proposal in principle. We suggest to make the potential spec impacts as examples since it may not be an complete list. |
| CATT | We are fine with the proposal. |
| InterDigital2 | There are examples in submitted contributions which capture repetition type B which has a repetition that is longer than 14 symbols. We suggest the following change in the proposal to make more general observation of the presented methods. Enhancements on PUSCH repetition type B are studied from several aspects, including nominal/actual PUSCH transmission across the slot boundary/invalid symbols and the length of nominal/actual repetition larger than 14 symbols. |

- Proposal 3: Capture the followings into the TR

- TB processing over multi-slot PUSCH is studied from several aspects, including TBS determined based on single slot, transmitted in parts over multiple slots and TBS determined based on multiple slots.

- Potential specification impacts of TB processing over multi-slot PUSCH include:

  - Time domain resource allocation, TBS determination, DM-RS pattern, RV determination.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Sierra Wireless | Its not clear what you mean with TBS – perhaps I think you instead mean Codeword. Also, for the case where the codewords are coded across multiple slots, the transmission still occurs across multiple slots where for TDD there may be gaps. Give this the following update is preferred: - TB processing over multi-slot PUSCH was studied from several aspects, including codewords determined based on single slot, transmitted in parts over multiple slots and codewords determined based on multiple slots, transmitted over multiple slots with and without gaps. |
| InterDigital | Support the proposal from the feature lead |
| WILUS | Support FL proposal with some additional clarification as below: - Proposal 3: Capture the followings into the TR - TB processing over multi-slot PUSCH is studied from several aspects, including TBS determined based on single slot, transmitted in parts over multiple slots and TBS determined based on multiple slots, transmitted over multiple slots. - Potential specification impacts of TB processing over multi-slot PUSCH include: - Time domain resource allocation, TBS determination, DM-RS pattern, RV determination. |
| Intel | We are in general fine with the proposal. For RV determination, it is good to clarify whether this is for TB spanning multiple slots in conjunction with repetition. |
| NTT DOCOMO | We support the FL proposal. |
| Qualcomm | We wish to clarify that if we go with simple TB scaling approach, then reusing the PUSCH repetition mechanism avoids any change to TDRA or even the necessity for consecutive slots. This is likely to have a very minimal impact on the spec. |
| Sharp | “DMRS pattern” can be deleted from the potential specification impacts. What kind of impacts on DMRS pattern is expected? |
| Nokia/NSB | Support the FL’s proposal in principle. However, the following sentence (highlighted in red) should be added for the completeness of the proposal: - “TB processing over multi-slot PUSCH is studied from several aspects, including TBS determined based on single slot, transmitted in parts over multiple slots and TBS determined based on multiple slots, transmitted over multiple slots”. |
| Xiaomi | We are in general fine with the proposal. But does the DMRS pattern relate to this solution? |
| ZTE | We are fine with the proposal. |
| LG | We are generally fine with the proposal. |
| CATT | We are not sure what new ‘DM-RS pattern’ or ‘RV determination’ rules are impacted here. |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Support the proposal. |

- Proposal 4: Capture the followings into the TR

- Enhancements on inter-slot frequency hopping are studied from several aspects, including more frequency offsets and more frequency hopping positions.

- Potential specification impacts of enhancements on inter-slot frequency hopping include:

  - Frequency domain hopping offsets, DM-RS pattern, TBS determination.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| InterDigital | Support the proposal from the feature lead |
| Intel | It is unclear to us why “DM-RS pattern, TBS determination” is related to enhancement on inter-slot frequency hopping. |
| NTT DOCOMO | We support the FL proposal. |
| Nokia/NSB | Support |
| Xiaomi | It is unclear why “TBS determination” is related to enhancement on inter-slot frequency hopping. |
| ZTE | Support the proposal in principle. Similarly, we are not sure why TBS determination is relevant here. |
| LG | We are generally fine with the proposal. |
| CATT | Fine with the proposal. |
| OPPO | why “TBS determination” is needed for frequency hopping enhancement? |

- Proposal 5: Capture the followings into the TR

- Potential specification impacts of inter-slot frequency hopping with inter-slot bundling to enable cross-slot channel estimation include:

  - Frequency domain hopping offset, time domain hopping interval, power consistency and the phase continuity within one bundle.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| InterDigital | Support the proposal from the feature lead |
| Intel | For “Frequency domain hopping offset”, it is not clear whether we need this. We can reuse same mechanism as for inter-slot FH. |
| Nokia/NSB | Support |
| ZTE | Support the proposal. |
| CATT | We are fine with the proposal. |

- Proposal 6: Capture the followings into the TR

- Sub-PRB transmission is studied from several aspects, including number of tones, sub-PRB transmission with single slot and sub-PRB transmission with multi-slot aggregation.

- Potential specification impacts of sub-PRB transmission include:

  - Frequency domain resource allocation, time domain resource allocation, TBS determination, DM-RS pattern, RV determination, hopping pattern within/between the PRBs

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Intel | We are in general fine with the proposal. For spec impact, we would like to add “PUSCH signal generation for DFT-s-OFDM waveform”. Our understanding is that we may want to use PUCCH format 4 like mechanism for sub-PRB transmission for PUSCH when DFT-s-OFDM waveform is employed. |
| NTT DOCOMO | We support the FL proposal. |
| Nokia/NSB | Support |
| Xiaomi | We are fine with this proposal |
| LG | We are generally fine with the proposal. |
| CATT | Fine with the proposal. |
| OPPO | We are generally fine with the proposal. |

- Proposal 7: Capture the followings into the TR

- Enhancements on intra-slot frequency hopping are studied from several aspects, including more frequency offsets, more frequency hopping positions and DM-RS sharing among multiple PUSCH transmissions.

- Potential specification impacts of enhancements on intra-slot frequency hopping include:

  - Frequency domain hopping offsets, DM-RS pattern, TBS determination, power consistency and phase continuity.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Nokia/NSB | Support |
| OPPO | “including more frequency offsets, more frequency hopping positions and DM-RS sharing among multiple PUSCH transmissions” it seems “more frequency hopping positions” is duplicated with “more frequency offsets”? |

- Proposal 8: Capture the followings into the TR

- Potential specification impacts of cross-slot channel estimation include:

  - Power consistency and phase continuity, DM-RS placement in special slot and DM-RS configuration.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| InterDigital | Support the proposal from the feature lead in principle. In Proposal 1,2 8, 9 or 10, DM-RS bundling between repetitions should be mentioned. One possible way to modify Proposal 8 as follows: - Potential specification impacts of cross-slot or cross-repetition channel estimation include: Power consistency and phase continuity, DM-RS placement in special slot and DM-RS configuration. |
| NTT DOCOMO | It is also beneficial to consider the cross-slot channel estimation with non-consecutive PUSCH slots e.g., TDD pattern of DDDSU as in the simulation assumption. |
| Sharp | We support the Interdigital’s proposal. Further, we suggest to remove “DM-RS placement in special slot and DM-RS configuration” from the potential specification impacts. Cross-slot channel estimation can be applied to the existing repetition type B without impacting the specification for DMRS placement in special slot and DMRS configuration. |
| Nokia/NSB | Support |
| Xiaomi | Does the DMRS configuration take the DMRS bundling into account? If yes, we are fine with the proposal. |
| ZTE | Fine with the proposal and also the revision from InterDigital. |
| LG | We are generally fine with the proposal. |
| CATT | Fine with either FL’s or InterDigital’s version. |
| InterDigital2 | To respond to the comment from Sharp, regarding “DM-RS placement in special slot and DM-RS configuration”, we would like to keep it as a potential specification impact. Cross-slot channel estimation can be applied to non-repetition PUSCH. For example, in the FL’s Proposal 2, an actual repetition of PUSCH that contains more than 14 symbols is considered. In such a configuration, DMRS can be placed in a special slot which will require cross-slot channel estimation. DMRS bundling operation may need a consideration for DMRS in a special slot as well. |

- Proposal 9: Capture the followings into the TR

- Lower DM-RS density in time domain is studied, including DM-RS sharing among multiple PUSCH transmissions.

- Potential specification impacts of DM-RS sharing among multiple PUSCH transmissions include:

  - DM-RS pattern and configuration, power consistency and phase continuity.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| InterDigital | Support the proposal from the feature lead. |
| Sharp | We are OK with the proposal. |
| Nokia/NSB | Support |
| Xiaomi | It is OK |
| ZTE | It was agreed to study lower DMRS density in both the time and frequency domain in the last meeting. In addition, we provide evaluation results show that lower DMRS density in the frequency domain could also provide about 1 dB gain. So, we suggest to change as follows: - Lower DM-RS density in time domain is studied, including DM-RS sharing among multiple PUSCH transmissions in the time domain and lower DMRS density in frequency domain. - Potential specification impacts of DM-RS sharing among multiple PUSCH transmissions include: - DM-RS pattern and configuration, power consistency and phase continuity. - Potential specification impacts of lower DMRS density in the frequency domain include: - DM-RS pattern and configuration. |
| OPPO | Support |

- Proposal 10: Capture the followings into the TR

- Higher DM-RS density is studied, including 1-comb DM-RS and additional DM-RS symbol position.

- Potential specification impacts of higher DM-RS density include:

  - DM-RS design, DM-RS position and TBS determination.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| InterDigital | Support the proposal from the feature lead. |
| Intel | Suggest to provide more details on “1-comb DMRS”, e.g., “DM-RS with single port spans to occupy the whole DM-RS symbol” to make it clear. Suggest to change “additional DM-RS symbol position” as “additional DM-RS symbol position in a slot”. |
| NTT DOCOMO | We support the FL proposal. |

- Proposal 11: Capture the followings into the TR

- Potential specification impacts of adaptive DM-RS configuration include:

  - Related signaling design.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Xiaomi | Need to add “DMRS configuration and pattern” in the potential spec impact. |
| OPPO | Support |

- Proposal 12: Capture the followings into the TR

- DM-RS balancing among frequency hops is studied. Potential specification impacts include related signaling design.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Intel | We may need to add “DMRS configuration and pattern” in the potential spec impact. |
| Nokia/NSB | Support |
| ZTE | Support |

- Proposal 13: Capture the followings into the TR

- UE transmit waveform design to reduce MPR is studies from several aspects, including tone reservation and FDSS with spectral extension for QPSK.

- Potential specification impacts include

  - UE transmit waveform design, RF requirements.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| InterDigital | Support the proposal from the feature lead |
| Nokia/NSB | Support |

Companies are invited to provide views on the others solutions, if any.

| Companies | Comments |
| --- | --- |

- Proposals (2nd round)

- //Proposal 1-13: Conclusions or observations on details and specification impacts for each enhanced solution

- Proposal 1: Capture the followings into the TR

- Enhancements on PUSCH repetition type A were studied from several aspects, including increasing the maximum number of repetitions e.g. 24, 32, the number of repetitions counted on the basis of available UL slots and flexible symbol resource allocation in different slots.

- Potential specification impacts of enhancements on PUSCH repetition type A include:

  - Increase the entries of TDRA (Time-Domain Resource Allocation).

  - Mechanism to determine transmission occasion of actual repetition times, e.g. postponement rules.

  - Mechanism to indicate UL symbols for each slot, e.g. mechanism to determine actual starting OFDM symbol for each slot.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Apple | For “flexible symbol resource allocation in different slots”, as we commented on GTW, this part seems more like PUSCH repetition type B. For PUSCH repetition type A, the starting symbol and length of transmission is the same for each repetition. Proposal 1 is on the PUSCH repetition type A enhancement, so it is preferred to take this part out of this proposal, also the sub-bullet under standard impact can be removed as well. |
| OPPO | For the 2nd sub-bullet “Mechanism to determine transmission occasion of actual repetition times, e.g. postponement rules”. We propose to remove the detail scheme “e.g. postponement rules” since there may be other mechanism as discussed in our contribution R1-2008271 to get the available UL slots for repetiotion. |
| vivo | Support the proposal in principle. But, there is concern about the maximum number. There seems to be unnecessary to provide the exact maximum number as example values or candidate values before evaluation. Thus, we suggest to remove “e.g. 24, 32”. |
| Qualcomm | General comment applicable to all proposal here: Can we also add number of companies who studied each scheme, how many support pursuing this scheme, and how many expressed concerns? We can add this information as the current meeting progresses and additional feedback in available, but it will be good to have it on record. |
| Ericsson | Examples can be removed in bullets, otherwise different companies may want to different examples which can be discussed in details in work item stage if needed: - “e.g. 24, 32” is not necessary which can be discussed based on the outcome of the evaluation; - It’s clear without examples in the 2nd and 3rd sub-bullet of the main bullet. |
| Intel | Support the proposal in principle. We had one question for the “Mechanism to indicate UL symbols for each slot”. Is this the intention to consider flexible resource allocation in time domain, e.g., different SLIV in different slots? It would be good to clarify this. |
| LG | We are generally fine with proposal. Regarding the 3rd sub-bullet “Mechanism to indicate UL symbols for each slot, e.g. mechanism to determine actual starting OFDM symbol for each slot.”, my understanding is that the main intention is to allow flexible resource allocation in time domain. So, it seems good to be listed up at same sub-bullet. But, it needs to clarify the difference of two examples in 3rd sub-bullet. ‘Indicate UL symbols’ may mean to use multiple SLIV indicators for each slot. On the other hand, ‘determine actual starting OFDM symbol for each slot’ may mean to use single SLIV indicator, and to determine the actual starting symbol by means of some rule or depending on condition (e.g., OFDM symbols for UL in special slot) . In this aspect, we suggest to remove ‘e.g.’ between two different examples at the 3rd sub-bullet. “Mechanism to indicate UL symbols for each slot, e.g. mechanism to determine actual starting OFDM symbol for each slot.” |
| Nokia/NSB | We share the same view as vivo and Ericsson that the examples on maximum number of repetitions may not be needed. Concerning the comment from LG, we think that capturing only “mechanism to determine actual starting OFDM symbol for each slot” should be sufficient, since it covers both implicit and explicit indication. Furthermore, we think it should be clear that even if flexible approach to configuring starting os for each repetition could be considered, duration of nominal and actual repetition would always coincide, given that it’s a Type A repetition. The current version of the proposal seems ambiguous in this regard. We propose to rewrite the third sub-bullet as: Mechanism to indicate UL symbols for each slot, e.g. mechanism to determine actual starting OFDM symbol for each slot provided that repetition length is constant across slots. |

- Proposal 2: Capture the followings into the TR

- Enhancements on PUSCH repetition type B were studied from several aspects, including actual PUSCH transmission across the slot boundary/invalid symbols and the length of actual repetition larger than 14 symbols, and RV enhancement.

- Potential specification impacts of enhancements on PUSCH repetition type B include:

  - TBS determination, DM-RS pattern, [DCI indication], SLIV table, hopping rules, RV determination, phase continuity.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Apple | We are not so clear the standard impact of hopping rule, the frequency hopping enhancement is in another proposal, any special consideration on hopping (inter-repetition hopping/inter-slot hopping) for PUSCH repetition type B enhancement? |
| Samsung | There were also other aspects considered (as summarized in Sec.2.1.1/2). We suggest to add at least “transmission with flexible symbol resource allocation”. |
| Panasonic | In Proposal 1, “TDRA” is used while SLIV is used in Proposal 2. We think the term should be aligned. |
| vivo | Support the revised proposal from FL. |
| Qualcomm | For spec impact, can we add “signalling mechanisms to indicate any potential changes to DMRS pattern/configuration”? |
| InterDigital | There are examples in submitted contributions which capture repetition type B which has a repetition that is longer than 14 symbols. We suggest the following change in the proposal to make more general observation of the presented methods. Enhancements on PUSCH repetition type B are studied from several aspects, including nominal/actual PUSCH transmission across the slot boundary/invalid symbols and the length of nominal/actual repetition larger than 14 symbols. |
| Ericsson | Not sure how frequency hopping will be affected by the enhancements mentioned in the first bullet, isn’t FH optimization in other proposals? |
| WILUS | For potential specification impacts, “phase continuity” seems a part of DMRS enhancement proposals. For the “hopping rules”, although its necessity is clear as proposed in our contribution [29], it should be considered in frequency domain enhancement as another proposal. |
| Intel | We share similar view as other companies that it is not clear whether there is spec impact on hopping rules. |
| LG | We are generally fine with the proposal 2. But, we share similar view with WILUS that ‘phase continuity’ and ‘hopping rules’ can be moved to other parts. |
| Nokia/NSB | We are fine with the proposal. |

- Proposal 3: Capture the followings into the TR

- TB processing over multi-slot PUSCH was studied from several aspects, including TBS determined based on single slot, transmitted in parts over multiple slots and TBS determined based on multiple slots, transmitted over multiple slots.

- Potential specification impacts of TB processing over multi-slot PUSCH include:

  - Time domain resource allocation, TBS determination, DM-RS pattern, RV determination.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| vivo | In our understanding, DMRS pattern may have impact on specification. For example, one potential TDRA for multi-slot PUSCH includes mapping type A, and first OFDM symbol as start symbol, and the length of 17. Therefore, second slot only occupies 3 OFDM symbols in time domain. It is well-known that there should be equal or greater than 4 OFDM symbols for PUSCH mapping type A. How DMRS symbol is mapped in time domain for the second slot? Thus, potential specification impacts should include DMRS pattern. |
| Ericsson | Phase coherency requirement for UE is needed in the multiple slot scheduled for one TB. |
| Sharp | - Why do we discuss TDRA with length 17 for multi-slot PUSCH? Even when you want a PUSCH with one repetition with length 14 in one slot and another repetition with length 3 in subsequent slot, you can apply PUSCH mapping type B. |
| LG | - We are generally fine with the proposal 3. Minor editorial comment: - TB processing over multi-slot PUSCH was studied from several aspects, including TBS determined based on single slot, transmitted in parts over multiple slots and , TBS determined based on multiple slots, and transmitted over multiple slots. |
| Nokia/NSB | - We are fine with the FL’s proposal in principle. However, as also mentioned by vivo, we think that there is potential spec impact for DMRS pattern as well. |

- Proposal 4: Capture the followings into the TR

- Enhancements on inter-slot frequency hopping were studied from several aspects, including:

  - More frequency offsets, e.g. 4 for BWP less than 50 PRBs, 8 for BWP greater than 50 PRBs.

  - More frequency hopping positions, e.g. 4.

- Potential specification impacts of enhancements on inter-slot frequency hopping include:

  - Frequency domain hopping offsets/positions, [DM-RS pattern], TBS determination.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Apple | We are fine with this proposal. |
| vivo | Support the revised proposal from FL. |
| Intel | We are fine with the proposal. |
| LG | We are fine with the proposal. |
| Nokia/NSB | Support |

- Proposal 5: Capture the followings into the TR

- Inter-slot frequency hopping with inter-slot bundling to enable cross-slot channel estimation was studied. Potential specification impacts of inter-slot frequency hopping with inter-slot bundling to enable cross-slot channel estimation include:

  - [Frequency domain hopping offset], time domain hopping interval, power consistency and the phase continuity within one bundle of multiple consecutive slots, e.g. 2, 4 slots.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Apple | We are fine with this proposal. |
| Panasonic | We suggest following description on potential specification impact: - The specification impact related to cross-slot channel estimation like power consistency and the phase continuity within one bundle - [Frequency domain hopping offset], time domain hopping interval |
| vivo | It is preferable to merge the proposal with proposal 8 as one sub-bullet or special case to discuss. |
| Qualcomm | For spec impact, can we add “signalling mechanisms to enable/disable inter-slot bundling and any potential changes to DMRS pattern/configuration”? |
| Ericsson | Does this proposal mean the study of the frequency hopping patterns among a set of repetitions on different slots? Maybe this one could be merged with proposal 8 as a sub-bullet since this is a study of how frequency hopping pattern will affect the cross-slot channel estimation. |
| Intel | It is still unclear to us why there is spec impact on “Frequency domain hopping offset”. Our understanding is that we would simply follow the design for inter-slot frequency hopping. Suggest to remove this. |
| Nokia/NSB | We support the FL’s proposal in principle. We are also fine to merge it with proposal 8 as suggested by some companies. In addition, we share the same view as Intel that the spec impact on “frequency domain hopping offset” needs more clarification. |

- Proposal 6: Capture the followings into the TR

- Sub-PRB transmission was studied from several aspects, including number of tones, sub-PRB transmission with single slot and sub-PRB transmission with multi-slot aggregation.

- Potential specification impacts of sub-PRB transmission include:

  - Frequency domain resource allocation, time domain resource allocation, TBS determination, DM-RS pattern, RV determination, hopping pattern within/between the PRBs, [PUSCH signal generation for DFT-s-OFDM waveform].

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Samsung | In the specification impact we suggest removing “time domain resource allocation, TBS determination, DM-RS pattern, RV determination”. For sub-PRB, the necessary change is to design new frequency domain resource allocation. Time domain resource allocation or TBS determination or RV determination can be impacted when sub-PRB is combined with other time domain solutions (i.e., repetition enhancement, TB processing over multi-slot PUSCH). The hopping pattern within/between PRBs and DMRS pattern in NR can be reused, so we added square brackets as follows. - Frequency domain resource allocation, [DM-RS pattern], [hopping pattern within/between the PRBs], [PUSCH signal generation for DFT-s-OFDM waveform].” |
| vivo | We are ok to capture in TR however sceptical about performance gain and there is significant impact on specification. |
| Qualcomm | New RAN4/RF requirements will be necessary as this is a new waveform for NR. |
| Intel | To clarify our proposal, PUSCH signal generation for DFT-s-OFDM waveform is based on PUCCH format 4 where a block-wised OCC is applied for PUSCH, which can help avoid the introduction of FFT size of 4 or 6 in the spec. |
| Nokia/NSB | Support |

- Proposal 7: Capture the followings into the TR

- Enhancements on intra-slot frequency hopping were studied from several aspects, including:

  - More frequency offsets, e.g. 4 for BWP less than 50 PRBs, 8 for BWP greater than 50 PRBs.

  - More frequency hopping positions, e.g. 3.

  - More hops, e.g. 3.

  - DM-RS sharing among multiple PUSCH transmissions.

- Potential specification impacts of enhancements on intra-slot frequency hopping include:

  - Frequency domain hopping offsets, DM-RS pattern, TBS determination, power consistency and phase continuity.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| vivo | The benefit of enhancements on intra-slot frequency hopping is unclear and overhead would increase. DMRS sharing among hops is impractical due to different frequency range. Whether the meaning of more hops is the same as more frequency hopping positions or not? What is the difference? |
| Ericsson | It’s not clear to us the “DM-RS sharing among multiple PUSCH transmissions” in case of intra-slot frequency hopping, more clarification text needed. |
| Sony | Similarly to vivo, we would like clarification on the difference in meaning between “more hops” and “more frequency hopping positions”. |
| Nokia/NSB | Support |

- Proposal 8: Capture the followings into the TR

- Cross-slot or cross-repetition channel estimation was studied from several aspects, including cross-slot channel estimation over consecutive slots, cross-slot channel estimation non-consecutive slots, and cross-repetition channel estimation.

- Potential specification impacts of cross-slot or cross-repetition channel estimation include:

  - Power consistency and phase continuity, [DM-RS placement in special slot and DM-RS configuration].

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Apple | We are fine with this proposal. |
| vivo | Support the revised proposal in principle. |
| CMCC | We propose to remove the bracket around “the DMRS placement in special slot and DMRS configuration”. The special slot with limited uplink symbols should be also considered for the cross channel estimation. And our intention is to fully use the symbols in the special slots. The DMRS optimization could be considered based on the performance of cross slot/repletion channel estimation. |
| Qualcomm | For spec impact, can we add “signalling mechanisms to enable/disable cross-slot or cross-repetition channel estimation? |
| InterDigital | We agree with CMCC. Regarding “DM-RS placement in special slot and DM-RS configuration”, we would like to keep it as a potential specification impact. Cross-slot channel estimation can be applied to non-repetition PUSCH. For example, in the FL’s Proposal 2, an actual repetition of PUSCH that contains more than 14 symbols is considered. In such a configuration, DMRS can be placed in a special slot which will require cross-slot channel estimation. DMRS bundling operation may need a consideration for DMRS in a special slot as well. |
| Ericsson | Maybe try to say “cross-slot” or “cross sub-slot” channel estimation is enough to cover all cases since “cross-repetition” may means cross repetitions which are in different slots or in same slot. And as we commented earlier, this proposal may capture proposal 5 as well, i.e. something like: “….including cross-slot channel estimation over consecutive slots, cross-slot channel estimation non-consecutive slots, and cross-repetition channel estimation, and cross-slot channel estimation with different frequency hopping patterns”. |
| Sharp | We prefer removing the contents in the square brackets. CMCC’s proposal (DMRS-less repetition in special slot) seems to be captured in lower DMRS (e.g., DMRS-less). InterDigital’s proposal (on actual PUSCH length more than 14 OFDM symbols) seems to be captured in Proposal 2. Proposal 2 captures “DMRS pattern” as one of potential specification impacts. |
| Intel | We suggest to remove “DM-RS placement in special slot”. Our view is that DMRS configuration would be sufficient. |
| Nokia/NSB | We are fine with the proposal. |

- Proposal 9: Capture the followings into the TR

- Lower DM-RS density in time domain was studied, including DM-RS sharing among multiple PUSCH transmissions in the time domain and lower DMRS density in frequency domain.

- Potential specification impacts of DM-RS sharing among multiple PUSCH transmissions include:

  - DM-RS pattern and configuration, power consistency and phase continuity.

- Potential specification impacts of lower DMRS density in the frequency domain include:

  - DM-RS pattern and configuration.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| vivo | The benefit of coverage enhancement from lower DMRS density in frequency domain is unclear. If lower DMRS density is introduced, there would be one added DMRS type, resulting in significant specification work. So, we support the 1st round proposal. |
| CMCC | Fine with the current version. |
| Qualcomm | For spec impact, can we add “signalling mechanisms (dynamic/implicit/static) to indicate changes in DMRS pattern/configuration”? |
| Ericsson | General comment (though we share similar view as Vivo that the benefit of these is not clear to us either) is that it’s better to merge all these DMRS density related optimizations together, e.g. lower density in time and/or frequency domain, adaptive/flexible DMRS density on different repetitions and/or different hops. |
| Sony | We are fine with the proposal, agree with Qualcomm that signalling will have spec impact. We find Ericsson’s proposal, to merge the DMRS density related topics, a good way forward. |
| Nokia/NSB | Proposals 9 and 10 could be grouped in one standalone “DMRS density proposal” accounting for lower/higher DMRS density. |

- Proposal 10: Capture the followings into the TR

- Higher DM-RS density was studied, including 1-comb DM-RS, e.g., DM-RS with single port spans to occupy the whole DM-RS symbol, and additional DM-RS symbol position in a slot.

- Potential specification impacts of higher DM-RS density include:

  - DM-RS design, DM-RS position and TBS determination.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Qualcomm | For spec impact, can we add “signalling mechanisms to indicate changes to DMRS pattern/configuration”? |
| Ericsson | General comment is that it’s better to merge all these DMRS density related optimizations together, e.g. lower density in time and/or frequency domain, adaptive/flexible DMRS density on different repetitions and/or different hops. |
| Nokia/NSB | Proposals 9 and 10 could be grouped in one standalone “DMRS density proposal” accounting for lower/higher DMRS density. |

- Proposal 11: Capture the followings into the TR

- Adaptive DM-RS configuration was studied. Potential specification impacts of adaptive DM-RS configuration include:

  - Related signaling design.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Apple | We are fine with this proposal. |
| CMCC | Fine with the proposal |
| Ericsson | General comment is that it’s better to merge all these DMRS density related optimizations together, e.g. lower density in time and/or frequency domain, adaptive/flexible DMRS density on different repetitions and/or different hops. |
| Sony | We are fine with the proposal. We find Ericsson’s proposal a good way forward. |
| Nokia/NSB | Support the proposal. We would merge only proposals related to DMRS density, such as P9 and P10, and not the ones related to DMRS configurations. |

- Proposal 12: Capture the followings into the TR

- DM-RS balancing among frequency hops was studied. Potential specification impacts include:

  - Related signaling design, DMRS configuration and pattern.

Companies are invited to provide views on the above proposal.

| Companies | Comments |
| --- | --- |
| Ericsson | General comment is that it’s better to merge all these DMRS density related optimizations together, e.g. lower density in time and/or frequency domain, adaptive/flexible DMRS density on different repetitions and/or different hops. |
| Nokia/NSB | Support the proposal. We would merge only proposals related to DMRS density, such as P9 and P10, and not the ones related to DMRS configurations. |

- Proposal 13: Capture the followings into the TR

- UE transmit waveform design to reduce MPR was studies from several aspects, including tone reservation and FDSS (Frequency Domain Spectral Shaping) with spectral extension for QPSK.

- Potential specification impacts include

  - UE transmit waveform design, RF requirements.

Companies are invited to provide views on the above proposal. Companies are encouraged to provide more details about the specification impacts of tone reservation and FDSS with spectral extension.

| Companies | Comments |
| --- | --- |
| vivo | In our understanding, FDSS could be up to implementation. We would like to clarify if there is any specification impact. |
| Qualcomm | For details on tone reservation, we can reference R1-2008626. If a single line summary is desired, then the following text can be added in brackets --- “a fraction of tones allocated to a UE are reserved for the UE to shape its waveform; no data is transmitted on these tones”. Regarding spec impact, we can add “signalling reserved tones” if UE transmit waveform design is deemed too generic. |
| Ericsson | Although this study is not clear to us and it may require RAN4 discussions as well if RAN1 has common understanding that this enhancement is necessary, we’re fine to capture all studied techniques including this one in the TR. |
| Nokia/NSB | Support the proposal, with the following editorial correction: “[…] was studied from several aspects”. Further details on FDSS (Frequency Domain Spectral Shaping) with spectral extension for QPSK can be found in R1-2008703. To further clarify the “UE transmit waveform design” in the spec impact, we can add “signalling design for spectral extension”. |

Companies are invited to provide views on the others solutions, if any.

| Companies | Comments |
| --- | --- |
| Qualcomm | Can we also add a proposal for dynamic PUSCH waveform adaptation (as summarized in Section 2.6.1? |
| Ericsson | Could u please capture power domain related proposals as well since we have studies already provided in our contribution for multi-layer transmission of PUSCH? Besides, the compression efficiency for SIP signal using compression algorithm is very high which can be up to 85% compression gain. So we would like to capture this possible solution to enhance compress large payload for PUSCH in VoNR as well. |

- Proposals (3rd round)

For this set of proposals, we are not talking about which solutions are supported or recommended for the follow-up WI, while we are analysing the details and the potential specification impacts of the solutions, which have been agreed to be studied.

Regarding the comments on supporting companies or companies having concerns, please refer to section 2.

Regarding signalling mechanism, we may not need to list it explicitly for each solution, as part of it is included in time domain resource allocation or frequency domain resource allocation, and RRC signalling need to be considered for most solutions.

- Proposal 1: Capture the followings into the TR

- Enhancements on PUSCH repetition type A were studied from several aspects, including increasing the maximum number of repetitions, e.g. 24, 32, the number of repetitions counted on the basis of available UL slots and flexible symbol resource allocation in different slots.

- Potential specification impacts of enhancements on increasing the maximum number of repetitions include:

  - TDRA (Time-Domain Resource Allocation).

- Potential specification impacts of enhancements on the number of repetitions counted on the basis of available UL slots include:

  - TDRA (Time-Domain Resource Allocation).

  - Mechanism to determine transmission occasion of actual repetition, [e.g. postponement rules].

- Potential specification impacts of enhancements on flexible symbol resource allocation include:

  - TDRA (Time-Domain Resource Allocation).

  - Mechanism to indicate UL symbols for each slot, [e.g. mechanism to determine actual starting OFDM symbol for each slot].

| Companies | Comments |
| --- | --- |
| FL | From FL perspective, although flexible symbol resource allocation is more like PUSCH repetition type B, since we are talking about enhancements, it can be studied for type A anyway. For better understanding, specification impacts are separated for each scheme. |
| Samsung | There can be overlap between enhancements considered in this section and R16 PUSCH repetitions Type B. Removing the flexible symbol resource allocation part would resolve that. The flexible symbol resource allocation needs to be added for repetition Type B. |
| LG | For enhancement of type A, the flexible symbol resource allocation can be considered. We are generally fine with FL’s proposal. For the 2nd sub-bullet in 4th bullet, we suggest to change as below: - Mechanism to indicate/determine UL symbols for each slot. |
| Apple | We share the similar view with Samsung. |
| vivo | We are fine without examples. In our understanding, “Mechanism to indicate UL symbols for each slot” is deemed generic, but “mechanism to determine actual starting OFDM symbol for each slot” is one more specific solution. So, we can just keep the former. |
| Huawei, HiSilicon | In determining the available UL slot, whether flexible slot which consists of several DL or flexible symbols can be regarded as available slot should be discussed. We suggest 3rd bullet proposal as follows: - Potential specification impacts of enhancements on the number of repetitions counted on the basis of available UL slots include: - TDRA (Time-Domain Resource Allocation). - Mechanism to determine transmission occasion of actual repetition, , [e.g. postponement rules]. Mechanism to determine whether flexible slot can be determined as an available UL slot. |
| Intel | We are generally fine with the proposal. We suggest to include some examples (e.g., postponement rules) to capture the exact study in SI phase. One editorial change: for the last bullet, it would be good the add “flexible symbol resource allocation in different slots” to align the main bullet. |

- Proposal 2: Capture the followings into the TR

- Enhancements on PUSCH repetition type B were studied from several aspects, including actual PUSCH transmission across the slot boundary/invalid symbols and the length of actual repetition larger than 14 symbols, and RV enhancement.

- Potential specification impacts of enhancements on PUSCH repetition type B include:

  - TBS determination, DM-RS pattern, TDRA (Time-Domain Resource Allocation), hopping rules, RV determination, phase continuity.

| Companies | Comments |
| --- | --- |
| Samsung | We suggest to add flexible symbol resource allocation in the description, and add to the specification impact “mechanisms for adjusting repetitions of a PUSCH transmission in the available UL symbols”. |
| Ericsson | “phase continuity” should be kept unless we remove the “and the length of actual repetition larger than 14 symbols” since phase continuity is only required within a slot in baseline. |
| vivo | Support the proposal. |

- Proposal 3: Capture the followings into the TR

- TB processing over multi-slot PUSCH was studied from several aspects, including TBS determined based on single slot and transmitted in parts over multiple slots, and TBS determined based on multiple slots and transmitted over multiple slots.

- Potential specification impacts of TB processing over multi-slot PUSCH include:

  - TDRA (Time-Domain Resource Allocation), TBS determination, [DM-RS pattern], RV determination, [phase continuity].

| Companies | Comments |
| --- | --- |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Support |
| Ericsson | Remove the brackets for “phase continuity” which is required for transmission in more than one slot. |
| vivo | According to our comment in 2nd round, there are always some cases to consider how to arrange DMRS symbols in time domain if multi-slot PUSCH. On the other hand, if mapping type B is always applied for multi-slot PUSCH, there is naturally no concern about DMRS pattern. However, mapping type for multi-slot PUSCH is not the key issue to discuss and needs further study to move forward. So, we would like to remove the brackets on DMRS pattern. |
| Huawei, HiSilicon | Whether frequency hopping pattern among multiple slots is included in the above filed of TDRA is not clear, which should be further clarified. We agree to remove the bracket for ‘phase continuity’. |
| Intel | We are not sure whether we need to have some enhancement on the DMRS pattern. Existing DMRS pattern in NR is already flexible enough, our view is that we can simply reuse the existing pattern for TB spanning multiple slots. |

- Proposal 4: Capture the followings into the TR

- Enhancements on inter-slot frequency hopping were studied from several aspects, including:

  - More frequency offsets, e.g. 4 for BWP less than 50 PRBs, 8 for BWP greater than 50 PRBs.

  - More frequency hopping positions, e.g. 4.

- Potential specification impacts of enhancements on inter-slot frequency hopping include:

  - Frequency domain hopping offsets/positions, [DM-RS pattern], TBS determination.

| Companies | Comments |
| --- | --- |
| vivo | Support the proposal. |

- Proposal 5: Capture the followings into the TR

- Inter-slot frequency hopping with inter-slot bundling to enable cross-slot channel estimation was studied. Potential specification impacts include:

  - [Frequency domain hopping offset], time domain hopping interval

  - The specification impact related to cross-slot channel estimation: power consistency and the phase continuity within one bundle of multiple consecutive slots, e.g. 2, 4 slots.

| Companies | Comments |
| --- | --- |
| FL | Proposal 5 “Inter-slot frequency hopping with inter-slot bundling” and proposal 8 “Cross-slot channel estimation” belong to different categories. If the majority suggest to treat it as a special case of cross-slot channel estimation, we are fine with it. |
| Ericsson | For cross-slot channel estimation, we have to consider how the frequency hopping is applied, e.g. hopping every other slot or every 2 slots or every 4 slots, which can be one aspect of proposal 8, considering cross slot channels estimation can only be applied on slots in the same hop according to our understanding. |
| vivo | It is preferable to merge the proposal with proposal 8 as one sub-bullet or special case to discuss. |
| Intel | We share similar view as FL that this proposal and proposal 8 “Cross-slot channel estimation” belong to different categories and it is better to separate these two. |

- Proposal 6: Capture the followings into the TR

- Sub-PRB transmission was studied from several aspects, including number of tones, sub-PRB transmission with single slot and sub-PRB transmission with multi-slot aggregation.

- Potential specification impacts of sub-PRB transmission with single slot include:

  - Frequency domain resource allocation, [TBS determination], [DM-RS pattern], hopping pattern within/between the PRBs, [PUSCH signal generation for DFT-s-OFDM waveform], [RF requirement].

- Potential specification impacts of sub-PRB transmission with multi-slot aggregation include:

  - Frequency domain resource allocation, time domain resource allocation, TBS determination, [DM-RS pattern], RV determination, hopping pattern within/between the PRBs, [PUSCH signal generation for DFT-s-OFDM waveform], [RF requirement].

| Companies | Comments |
| --- | --- |
| FL | Sub-PRB transmission with single slot and sub-PRB transmission with multi-slot aggregation may have different specification impacts. Specification impacts are separated. |
| Samsung | In the specification impact we suggest removing the parts in square brackets. For sub-PRB, the necessary change is to design new frequency domain resource allocation. |
| Apple | According to the last meeting agreement, we propose the following update, “Sub-PRB transmission for VoIP was studied from several aspects,…”. |
| vivo | We are fine to capture in TR, although we are still sceptical on the performance gain, and there is significant impact on specification. |
| CATT | Regarding the “time domain resource allocation, TBS determination, DM-RS pattern, RV determination, hopping pattern within/between the PRBs” in the 2nd bullet, they may be brought by the experience in Rel-15 when sub-PRB allocation is introduced in LTE MTC. Companies contributed great effort on the aforementioned parts and developed some new design different from NB-IoT. Considering that they are more like some ‘observations’ from the past, and the proposal is to capture ‘potential’ specification impact, we think they are OK to be captured. |
| Intel | For sub-PRB based transmission, it is not clear to us even for a single slot based transmission, why we do not need TBS determination. The number of REs in frequency is reduced to less than 12, which would definitely have impact on TBS determination. For PUSCH signal generation for DFT-s-OFDM waveform, we suggest to remove the bracket. Even if we reuse the existing signal generation mechanism, for DFT-s-OFDM waveform, we do not even have DFT size of 4 or 6, which is less than 12. It is unclear to us why there is no spec impact on this. |

- Proposal 7: Capture the followings into the TR

- Enhancements on intra-slot frequency hopping were studied from several aspects, including:

  - More frequency offsets, e.g. 4 for BWP less than 50 PRBs, 8 for BWP greater than 50 PRBs.

  - More frequency hopping positions, e.g. 3.

  - More number of hops in a slot, e.g. 3.

  - DM-RS sharing among multiple PUSCH transmissions with the same frequency position between two consecutive slots.

- Potential specification impacts of enhancements on intra-slot frequency hopping include:

  - Frequency domain hopping offsets, DM-RS pattern, TBS determination, power consistency and phase continuity.

| Companies | Comments |
| --- | --- |
| FL | More frequency hopping positions means the number of frequency hopping positions indicated by DCI is increased. More hops means the number hops in a slot is increased. |
| Apple | For sub-bullet DM-RS sharing among multiple PUSCH transmissions with the same frequency position between two consecutive slots, it’s not clear whether it is relevant to intra-slot frequency hopping, it seems more related to hopping proposal 4 inter-slot frequency hopping or proposal 9 DM-RS enhancement. |
| Ericsson | Could we understand that here it means we use single DCI to schedule multiple PUSCH (multiple frequency domain resource allocation) for this “more frequency hopping positions”? The “DM-RS sharing among multiple PUSCH transmissions with the same frequency position between two consecutive slots.” seems still not clear, does this mean different DMRS configurations are used in the 2 consecutive slots for the PUSCH repetitions with the same FDRA provided in DCI? |
| vivo | For 3rd sub-bullet, in our understanding, for intra-slot frequency hopping, the meaning of more hops is the same as more frequency hopping positions. Do more hops mean the number hops in a slot for multiple PUSCH transmissions? If so, the benefit is unclear, due to heavy DMRS overhead. Further clarification is needed. For 4th sub-bullet, we suggest to remove the 4th sub-bullet, and the new Proposal 9 includes this. We are fine to capture in TR, although we are still sceptical on the performance gain. |
| Intel | We share similar view as other companies that “DM-RS sharing among multiple PUSCH transmissions with the same frequency position between two consecutive slots.” seems not clear. This seems already covered by lower DMRS density. We suggest to remove this. |
| Sony | Similarly to vivo, we would like clarification on the difference in meaning between “more hops” and “more frequency hopping positions”. As we understand it: - more frequency hop positions relate to frequency-domain positions of hops, and - more hops relate to time-domain hop positions within a slot. This needs to be clear. We propose to change the wording of “more hops” to “more time hopping positions”. |

- Proposal 8: Capture the followings into the TR

- Cross-slot or cross-repetition channel estimation was studied from several aspects, including cross-slot channel estimation over consecutive slots, cross-slot channel estimation non-consecutive slots, and cross-repetition channel estimation.

- Potential specification impacts of cross-slot or cross-repetition channel estimation include:

  - Power consistency and phase continuity, [DM-RS placement in special slot and DM-RS configuration].

| Companies | Comments |
| --- | --- |
| FL | Regarding “DM-RS placement in special slot and DM-RS configuration”, it seems controversial, we may need more discussion or during GTW session. |
| Sharp | I share the understanding that utilization of special slots is an important use case for this item. If companies observe that further optimization of DMRS (time domain) location/granularity as an essential factor, we propose to explicitly mention it in the proposal, like following. - Cross-slot or cross-repetition channel estimation with/without optimization of DMRS location/granularity was studied from several aspects, including cross-slot channel estimation over consecutive slots, cross-slot channel estimation over non-consecutive slots, and cross-repetition channel estimation. - Potential specification impacts of cross-slot or cross-repetition channel estimation include: - Power consistency and phase continuity, DM-RS placement in special slot and DM-RS configuration. |
| Ericsson | What’s the difference between “cross-slot” and “cross-repetition”? it seems cross-slot can be cross 2 repetitions on 2 slots, cross-repetition can also be cross 2 PUSCH repetitions on 2 slots or in same slot. Based on our understanding, compared to legacy, the change is whether we do channel estimation across slots since phase coherency duration will be extended from single slot to more than one slot. |
| vivo | We are fine with the proposal. |
| Huawei, HiSilicon | Suggest to remove the bracket for ‘DM-RS placement in special slot and DM-RS configuration’ Besides, except above potential spec impacts of cross-slot channel estimation, the spec impact that same DMRS antenna ports for transmissions over multiple slots should be included, aiming to ensure that the estimated channel of each slot corresponds to the same fading link. Thus, we suggest the proposal as: - Potential specification impacts of cross-slot or cross-repetition channel estimation include: Power consistency and phase continuity, [DM-RS placement in special slot and DM-RS configuration, DMRS antenna ports] |
| CATT | We think at least the ‘DM-RS placement in special slot’ can be captured. The square bracket of DM-RS placement in special slot can be removed. |
| Intel | It is still not clear to us “DM-RS placement in special slot”. Does this mean in the spec slot, UE only transmits the DMRS but not PUSCH? If UE would also transmit the PUSCH in the spec slot, can we simply reuse the PUSCH repetition type B for DMRS pattern? |

- Proposal 9: Capture the followings into the TR

- Enhancements on DM-RS density were studied from several aspects, including DM-RS sharing among multiple PUSCH transmissions in the time domain, lower DMRS density in frequency domain, 1-comb DM-RS, e.g., DM-RS with single port spans to occupy the whole DM-RS symbol, and additional DM-RS symbol position in a slot.

- Potential specification impacts of DM-RS sharing among multiple PUSCH transmissions include:

  - DM-RS pattern and configuration, power consistency and phase continuity.

- Potential specification impacts of lower DMRS density in the frequency domain include:

  - DM-RS design, DM-RS pattern and configuration.

- Potential specification impacts of 1-comb DM-RS include:

  - DM-RS design, and TBS determination.

- Potential specification impacts of additional DM-RS symbol position in a slot include:

  - DM-RS position.

| Companies | Comments |
| --- | --- |
| FL | Based on the comments, proposal 9 and 10 are merged, while specification impacts are separated for each scheme. |
| Ericsson | As commented earlier in last round, it’s better to summarize these DMRS density related optimizations in one proposal. |
| CMCC | From current version, we cannot find any explicit description about lower density of DMRS in time domain. And if the density of DMRS could be changed, the TBS determination could also be impact. Furthermore, the TBS determination could be considered for single and multiple slots. Then the proposal is suggested as below, - Enhancements on DM-RS density were studied from several aspects, including DM-RS sharing among multiple PUSCH transmissions (including lower DMRS density in time domain) in the time domain, lower DMRS density in frequency domain, 1-comb DM-RS, e.g., DM-RS with single port spans to occupy the whole DM-RS symbol, and additional DM-RS symbol position in a slot. - Potential specification impacts of DM-RS sharing among multiple PUSCH transmissions (including lower DMRS density in time domain) include: - DM-RS pattern and configuration, TBS determination, power consistency and phase continuity. |
| vivo | We are fine to capture in TR, although we are still sceptical on the performance gain. |

- Proposal 11: Capture the followings into the TR

- Adaptive DM-RS configuration was studied. Potential specification impacts include:

  - Related signaling design.

| Companies | Comments |
| --- | --- |
| Ericsson | As commented earlier in last round, it’s better to summarize these DMRS density related optimizations in one proposal. |
| vivo | According to the revised Proposal 9, this proposal should be also included. |

- Proposal 12: Capture the followings into the TR

- DM-RS balancing among frequency hops was studied. Potential specification impacts include:

  - Related signaling design, DMRS configuration and pattern.

| Companies | Comments |
| --- | --- |
| Ericsson | As commented earlier in last round, it’s better to summarize these DMRS density related optimizations in one proposal. |

- Proposal 13: Capture the followings into the TR

- UE transmit waveform design to reduce MPR was studied from several aspects, including tone reservation and FDSS (Frequency Domain Spectral Shaping) with spectral extension for QPSK.

- Potential specification impacts include

  - UE transmit waveform design signalling reserved tones, signalling design for spectral extension, RF requirements.

Note: For tone reservation, a fraction of tones allocated to a UE are reserved for the UE to shape its waveform; no data is transmitted on these tones.

| Companies | Comments |
| --- | --- |
| Huawei, HiSilicon | In our understanding, FDSS can be implemented without spectral extension, and it can also apply to modulations other than QPSK. The intention to restrict the spectral extension and QPSK is not clear, and RAN1 should investigate different options to perform FDSS. Moreover, the detailed signalling and design for FDSS and reserved tones may impact the spec. In summary, we can support the proposal with the following revision: “ - UE transmit waveform design to reduce MPR was studies from several aspects, including tone reservation and FDSS (Frequency Domain Spectral Shaping) with spectral extension for QPSK. - Potential specification impacts include - UE transmit waveform design signalling reserved tones, signalling and/or design for FDSS spectral extension, RF requirements. Note: For tone reservation, a fraction of tones allocated to a UE are reserved for the UE to shape its waveform; no data is transmitted on these tones. “ |

- Proposal 14: Capture the followings into the TR

- Power domain based solutions were studies from several aspects, including multiple layer PUSCH transmission with DFT-S-OFDM and Open-loop/closed loop Tx diversity.

- Companies are encouraged to provide the potential specification impacts:

| Companies | Comments |
| --- | --- |
| Ericsson | Potential little specification impact: mechanism to determine the precoder, e.g. reuse a subset of the R15 codebooks. |

- Companies are encouraged to answer the following question:

- Q: Whether RAN1 to further study following solutions:

- Power boosting for pi/2 BPSK

- OCC spreading based repetition

- Symbol-level repetition/combing

- TB interleaving

- RV repetition

- Early termination of PUSCH repetitions

- Enhancements on intra-slot frequency hopping

- FDD high power UE

- Dynamic PUSCH waveform adaptation

- Compression enhancement for SIP invite message, e.g., SigComp

| Companies | Comments |
| --- | --- |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Can we understand why power boosting was moved into this category? Specifically, for proposal-14, only 2 or 3 companies provided results, but it is included in the TR. But 5 companies have provided results for the power boosting pi/2 BPSK case. We recommend that even this solution be captured in the TR. We strictly request this to be captured in the TR. Next, the gains of boosting are quite clear and easy for every to follow. 26 dBm is already supported in spec. We should only find out whether this can be further increased. Not sure what is the confusion on this solution. |
| Ericsson | SigComp was studied and described in our contribution and the summary of the enhancement results, we propose to at least capture the study results in TR according to the guidance and common understanding, and discuss the priority of it later based on the discussions on voice in the evaluation agendas. Example proposal: - Proposal xx: Capture the followings into the TR SIP signal compression was studied for enhancement large payload PUSCH including SigComp used for application information compression and the compression efficiency. Potential spec. impacts: also using compression algorithm to compress the large SIP signaling message in higher layer. |

- Proposals (4th round)

- Proposal 1: Capture the followings into the TR

- Enhancements on PUSCH repetition type A were studied from several aspects, including increasing the maximum number of repetitions, e.g. 24, 32, the number of repetitions counted on the basis of available UL slots and flexible symbol resource allocation in different slots.

- Potential specification impacts of enhancements on increasing the maximum number of repetitions include:

  - TDRA (Time-Domain Resource Allocation).

- Potential specification impacts of enhancements on the number of repetitions counted on the basis of available UL slots include:

  - TDRA (Time-Domain Resource Allocation).

  - Mechanism to determine transmission occasion of actual repetition, [e.g. postponement rules].

  - Mechanism to determine whether flexible slot can be determined as an available UL slot.

- Potential specification impacts of enhancements on flexible symbol resource allocation in different slots include:

  - TDRA (Time-Domain Resource Allocation).

  - Mechanism to indicate/determine UL symbols for each slot, [e.g. mechanism to determine actual starting OFDM symbol for each slot].

| Companies | Comments |
| --- | --- |
| FL | From FL perspective, although flexible symbol resource allocation is more like PUSCH repetition type B, since we are talking about enhancements, it can be studied for type A anyway. It seems the majority are fine with no examples. |
| Sharp | Wording change from "flexible slot" to "special slot" is proposed. Flexible slot can be interpreted as a slot whose slot format can be changed dynamically by DCI format 2_0. Original Huawei's proposal seems "special slot" which includes several DL or flexible symbols. The term "special slot" is already in proposal 8. Therefore, we think it's clearer. “Mechanism to determine whether flexible special slot can be determined as an available UL slot.” |
| LG | We are fine with the Proposal 1. |
| CATT | We are generally fine with Proposal 1. We also think Sharp’s modification is reasonable. |
| Nokia/NSB | Support |
| Huawei, HiSilicon | OK with the Proposal 1. |
| Intel | We are fine with Proposal 1 with Sharp’s update. |
| Samsung | When flexible symbols can be used as UL, the number of slots needed for PUSCH transmission with the indicated number of repetitions can change. To take this into consideration “length of the actual transmission” can be added in the last bullet. Also, “indicate” seems superfluous. - Mechanism to indicate/determine UL symbols for each slot and length of the actual transmission, [e.g. mechanism to determine actual starting OFDM symbol for each slot]. |
| Apple | The proposal updated by Sharp is fine for us. |
| OPPO | For the wording, “Mechanism to determine whether flexible slot can be determined as an available UL slot.”, it may be better to say “Mechanism to determine whether slot containing flexible symbols can be used for PUSCH repetition.” |
| FL | It seems the majority are basically fine with this proposal. Sharp, CATT, Intel, Apple suggest the following modification: “Mechanism to determine whether flexible special slot can be determined as an available UL slot.” From FL perspective, the suggestion is reasonable. @ Samsung, It seems “length of the actual transmission” can be covered by “determine UL symbols for each slot”. It is fine to remove “indicate”. |
| CMCC | Support FL and companies’ proposal that change the flexible to special. Since large repetition numbers are considered within this proposal, the early termination of repetitions should be considered to improve the system efficiency and resource utilization. Though the early termination is not a scheme directly enhance the performance of coverage, it could reduce the cost of network due to large number of repetition. |
| vivo | We support the newly revised proposal from FL. |
| FL | @CMCC, As discussed in RAN1 #102-e, early termination of repetitions is a separate issue and controversial. @all, No further concerns and proposal 1 seems stable. Proposal 1 is revised as follows incorporating the latest modification. - Proposal 1: Capture the followings into the TR - Enhancements on PUSCH repetition type A were studied from several aspects, including increasing the maximum number of repetitions, e.g. 24, 32, the number of repetitions counted on the basis of available UL slots and flexible symbol resource allocation in different slots. - Potential specification impacts of enhancements on increasing the maximum number of repetitions include: - TDRA (Time-Domain Resource Allocation). - Potential specification impacts of enhancements on the number of repetitions counted on the basis of available UL slots include: - TDRA (Time-Domain Resource Allocation). - Mechanism to determine transmission occasion of actual repetition, [e.g. postponement rules]. - Mechanism to determine whether flexible special slot can be determined as an available UL slot. - Potential specification impacts of enhancements on flexible symbol resource allocation in different slots include: - TDRA (Time-Domain Resource Allocation). - Mechanism to indicate/determine UL symbols for each slot, [e.g. mechanism to determine actual starting OFDM symbol for each slot]. |

- Proposal 2: Capture the followings into the TR

- Enhancements on PUSCH repetition type B were studied from several aspects, including actual PUSCH transmission across the slot boundary/invalid symbols, the length of actual repetition larger than 14 symbols, RV enhancement [and flexible symbol resource allocation].

- Potential specification impacts of enhancements on PUSCH repetition type B include:

  - TBS determination, DM-RS pattern, TDRA (Time-Domain Resource Allocation), RV determination,

  - [Mechanisms for adjusting repetitions of a PUSCH transmission in the available UL symbols for flexible symbol resource allocation]

  - Note that power consistency and phase continuity may or may not be required depending on factors such as cross-slot channel estimation, etc.

| Companies | Comments |
| --- | --- |
| FL | Regarding the “phase continuity”, it seems it’s similar with proposal 3. |
| LG | We are fine with Proposal 2 |
| CATT | We are fine with Proposal 2 in principle. We would appreciate if we can see some clarification on ‘flexible symbol resource allocation’. Does it mean different symbol numbers of (actual? nominal?) repetitions of type B repetition? |
| WILUS | We are generally fine with FL proposal. However, “Mechanisms for adjusting repetitions of a PUSCH transmission in the available UL symbols for flexible symbol resource allocation” seems abstract. |
| Nokia/NSB | We are fine with the proposal in principle. However, as also pointed out by CATT, clarification for “flexible symbol resource allocation” is needed, because we are not sure that this aspect has been studied. Whether it is the same as or different from “actual PUSCH transmission across the slot boundary/invalid symbols”? Since it is mentioned “were studied” in the text, let us make it clear. |
| Huawei, HiSilicon | OK with the Proposal. |
| Intel | We share similar view as other companies that the latest update with square bracket is not clear to us. Either it would be good to be clarified by proponent companies or we can remove it. |
| Ericsson | Agree with Intel and other companies that it’s better to remove the bullet in bracket if it’s not clear what to specify at all. Regarding “‐ Note that power consistency and phase continuity may or may not be required depending on factors such as cross-slot channel estimation, etc.”, does it mean we may assume different channels for different part of one PUSCH occasion and we may assume only one PUSCH transmission instance for power control? These (at least power control assumption) require clarification or additional changes in the specification. |
| Samsung | The proposal including the content of both square brackets is generally fine. The flexible symbol allocation can be realized by existing mechanisms, for example the direction of a symbol respect to an UL/DL configuration is changed by SFI. This can change the duration of the whole PUSCH transmission with repetitions and potentially impact the specifications. “flexible symbol resource allocation by SFI” can be used. The listed impacts to specifications may be due to independent solutions and not all impacts are necessary to support type B repetitions enhancement (e.g., RV enhancement, DM-RS pattern, etc.). Same comment applies also to other proposals. A statement can be added in the TR that applies to solutions discussed thereafter. |
| Apple | For the Flexible symbol resource allocation via the SFI, the benefits are not clear. If the UE missed the SFI indication, the gNB and UE would have different assumption on the UL repetition pattern. |
| FL | CATT, WILUS, Nokia/NSB, Ericsson, Apple have concerns on “flexible symbol resource allocation”, the proponent should clarify the details, otherwise we may have to remove it. Regarding “power consistency and phase continuity” in this proposal, it depends on whether joint channel estimation is always needed if the actual PUSCH transmission across the slot boundary. |
| CMCC | Support FL’s proposal. And share a similar concern on “flexible symbol resource allocation”. SFI may change the available symbols for uplink transmission dynamically. And there could some conflict due to dynamic change of available resource and the repetition transmission. And the benefit is not so clear. |
| vivo | Similar to some other comments, the meaning of “flexible symbol resource allocation” is not clear to us, which seems to require significant specification efforts. And does “flexible” mean independent resource allocation for each repetition or else? |
| Samsung | In our view the red part in the first bullet refers to the consideration of SFI for determining a repetition. The use of SFI seems reasonable as the DL has been shown by evaluation not to be the bottleneck link. In the first bullet, “[and flexible symbol resource allocation]” can be changed to “and operation with SFI”. Agree with previous comments to remove the sub-bullet “[Mechanisms for …”. |
| FL | CATT, WILUS, Nokia/NSB, Ericsson, Apple, CMCC, express concerns on “flexible symbol resource allocation”, so “flexible symbol resource allocation” is removed. - Proposal 2: Capture the followings into the TR - Enhancements on PUSCH repetition type B were studied from several aspects, including actual PUSCH transmission across the slot boundary/invalid symbols, the length of actual repetition larger than 14 symbols, RV enhancement [and flexible symbol resource allocation]. - Potential specification impacts of enhancements on PUSCH repetition type B include: - TBS determination, DM-RS pattern, TDRA (Time-Domain Resource Allocation), RV determination, - [Mechanisms for adjusting repetitions of a PUSCH transmission in the available UL symbols for flexible symbol resource allocation] - Note that power consistency and phase continuity may or may not be required depending on factors such as cross-slot channel estimation, etc. |
| Samsung | @FL What is the reason for not considering our proposal above? We have replied to the concern about the reliability of the SFI. We proposed to change the first bullet to describe our consideration of SFI for determining repetitions (details are in our contribution). |
| FL | @Samsung, From FL perspective, any solution can be studied. But it seems the concerns on SFI from other companies have not been addressed. If I misunderstood other companies’ concerns, please correct me, and I’m happy to include it. |
| FL | - Regarding “operation with SFI”, let’s discuss it during GTW session. - Proposal 2: Capture the followings into the TR - Enhancements on PUSCH repetition type B were studied from several aspects, including actual PUSCH transmission across the slot boundary/invalid symbols, the length of actual repetition larger than 14 symbols, RV enhancement [and operation with SFI]. - Potential specification impacts of enhancements on PUSCH repetition type B include: - TBS determination, DM-RS pattern, TDRA (Time-Domain Resource Allocation), RV determination, - Note that power consistency and phase continuity may or may not be required depending on factors such as cross-slot channel estimation, etc. |

- Proposal 3: Capture the followings into the TR

- TB processing over multi-slot PUSCH was studied from several aspects, including TBS determined based on single slot and transmitted in parts over multiple slots, TBS determined based on multiple slots and transmitted over multiple slots.

- Potential specification impacts of TB processing over multi-slot PUSCH include:

  - TDRA (Time-Domain Resource Allocation), TBS determination, DM-RS pattern, RV determination.

  - Note that power consistency and phase continuity may or may not be required depending on factors such as cross-slot channel estimation, etc.

| Companies | Comments |
| --- | --- |
| FL | As we are discussing the “potential” specification impacts, FL suggests to keep “DM-RS pattern” as commented by some companies. |
| InterDigital | From our perspective, DM-RS pattern is not part of the spec impact. DM-RS enhancement are discussed in other proposals (e.g., Proposal 9,11,12). Here, the aim is to obtain diversity gain in time domain by mapping a TB or segmented TB to multiple slots. Within each slot, existing DM-RS can be used. The UE can use the DM-RS within each slot, and perform channel estimation locally within each slot, as we commented online. This topic should not incorporate enhancements related to DMRS enhancement or cross-slot channel estimation; the aforementioned enhancements are add-ons to the enhancement discussed in this topic. Similar to the issue related to phase continuity, we propose to remove “DM-RS pattern” from the specification impact and modify the note as follows. - Note that power consistency, phase continuity and enhancements for DM-RS configurations may or may not be required depending on factors such as cross-slot channel estimation, etc. |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Support |
| Qualcomm | Same comments as Interdigital. |
| WILUS | We have similar view with InterDigital. |
| Nokia/NSB | We are fine with the proposal in principle. However, the specification impact on “TBS determination” needs further clarification. It is unclear to us why do we need to modify the TBS determination procedure? Although we may change the way REs are allocated in this case, the TBS determination procedure should not be modified. Maybe the terminology makes some confusions here. Concerning the DMRS pattern, if the intention is to consider this solution as an independent solution and joint channel estimation/DMRS sharing can be added on top then we are fine with the proposed text from InterDigital. |
| Huawei, HiSilicon | For TB transmission across multiple slots, we presented our concern on its difficulty on UE implementation in previous discussions. In UE implementation, slot boundary is a special time position for control and scheduling of hardware and software. If TB across slot boundary transmission is introduced with parts over multiple slots, the period of control and scheduling might be interrupted, which will increase the complexity significantly. Since the potential spec impact seems not to resolve any complexity issue yet, we suggest the following modification - “TB processing over multi-slot PUSCH was studied from several aspects, including TBS determined based on single slot and transmitted in parts over multiple integer slots, TBS determined based on multiple slots and transmitted over multiple slots.” - Thus TBS can across multiple slots but still end at slot boundary without blurring the usage of slot boundary |
| Intel | We are fine with the update from InterDigital. Enhancement on DMRS pattern may be only needed when cross-slot channel estimation is employed. |
| Ericsson | DMRS pattern doesn’t have to be affected even if cross-slot CE is applied. Besides, similar to the comment to earlier proposal, phase coherency here depends on whether only one channel is assumed for one PUSCH transmission when it crosses multiple slots. Power consistency depends on whether we assume only one PUSCH transmission instance even if it crosses multiple slots. And these (at least the power control assumption) requires specification changes to make it clear as we discussed in the GTW. |
| Samsung | DM-RS enhancement is treated in a dedicated proposal. No need to mention it as specification impact in association with every enhancement. |
| Apple | InterDigital’s upates are fine for us. |
| OPPO | InterDigital’s upates looks fine for us. |
| FL | From FL’s understanding, whether there is impact of DM-RS depends on the specific solution. The modification from InterDigital seems reasonable. - Note that power consistency, phase continuity and enhancements for DM-RS configurations may or may not be required depending on factors such as cross-slot channel estimation, etc. - @Nokia, Regarding the TBS determination, TBS determination procedure may not be modified, but the calculation of TBS need to be modified due to multi-slot PUSCH operation. @Ericsson, Regarding power consistency and phase continuity, it depends on the specific solutions and whether joint channel estimation is implemented. In that sense, “may or may not” seems reasonable. Regarding Huawei’s suggested modification, any comments? |
| Nokia/NSB | @FL: Could you please elaborate more on what is the difference between the calculation of TBS and the TBS determination procedure? Thank you! |
| vivo | We share the similar understanding as FL that whether there is impact of DM-RS depends on the specific solution. In addition, there may or may not be potential problems for DMRS pattern if solutions of TB processing over multi-slot PUSCH were supported. Anyway, we are OK with the comments above from FL. |
| FL | @Nokia, From my understanding, the calculation of TBS is based on multiple slots and the number of symbols for each slot may not be the same. This may not be identical with what we have in the current spec. If I misunderstood something and everyone is fine to remove “TBS determination”, I’m happy to remove it. Proposal 3 is revised as follows: - Proposal 3: Capture the followings into the TR - TB processing over multi-slot PUSCH was studied from several aspects, including TBS determined based on single slot and transmitted in parts over multiple slots, TBS determined based on multiple slots and transmitted over multiple slots. - Potential specification impacts of TB processing over multi-slot PUSCH include: - TDRA (Time-Domain Resource Allocation), TBS determination, DM-RS pattern, RV determination. - Note that power consistency, phase continuity and enhancements for DM-RS configurations may or may not be required depending on factors such as cross-slot channel estimation, etc. |

- Proposal 6: Capture the followings into the TR

- Sub-PRB transmission for VoIP was studied from several aspects, including number of tones, sub-PRB transmission with single slot and sub-PRB transmission with multi-slot aggregation.

- Potential specification impacts of sub-PRB transmission with single slot include:

  - Frequency domain resource allocation, TBS determination, [DM-RS pattern], hopping pattern within/between the PRBs, PUSCH signal generation for DFT-s-OFDM waveform, [RF requirement].

- Potential specification impacts of sub-PRB transmission with multi-slot aggregation include:

  - Frequency domain resource allocation, time domain resource allocation, TBS determination, [DM-RS pattern], RV determination, hopping pattern within/between the PRBs, PUSCH signal generation for DFT-s-OFDM waveform, [RF requirement].

| Companies | Comments |
| --- | --- |
| Nokia/NSB | Support |
| Intel | We support FL’s proposal. |
| FL | Seems no concerns. But we still have to address the brackets. Companies are encouraged to provide views on the brackets. |
| CMCC | One issue for the clarification. In the discussion of evaluation assumptions, 4 PRB was assumed for the VoIP traffic. Then how to realize an sub-PRB transmission for the VoIP? It seems that the sub-PRB transmission will cost a large number of uplink slots. |
| FL | @CMCC, From my understanding, yes, for sub-PRB transmission, multiple slots are needed. But it does not mean sub-PRB transmission may cause more resources than PRB based transmission. In order to enhance coverage, repetition is needed for PRB based transmission, which results in somehow similar time domain resources. Anyway, regarding the performance, we can discuss it later. @all, Regarding the brackets, can we remove the brackets or leave them for GTW session? |
| Qualcomm | Support removing brackets around RF requirements. Sub-PRB waveforms will need additional RAN4 input. |
| FL | Can we revise proposal 6 as follows? - Proposal 6: Capture the followings into the TR - Sub-PRB transmission for VoIP was studied from several aspects, including number of tones, sub-PRB transmission with single slot and sub-PRB transmission with multi-slot aggregation. - Potential specification impacts of sub-PRB transmission with single slot include: - Frequency domain resource allocation, TBS determination, [DM-RS pattern], hopping pattern within/between the PRBs, PUSCH signal generation for DFT-s-OFDM waveform, [RF requirement]. - Potential specification impacts of sub-PRB transmission with multi-slot aggregation include: - Frequency domain resource allocation, time domain resource allocation, TBS determination, [DM-RS pattern], RV determination, hopping pattern within/between the PRBs, PUSCH signal generation for DFT-s-OFDM waveform, [RF requirement]. |

- Proposal 7: Capture the followings into the TR

- Enhancements on intra-slot frequency hopping were studied from several aspects, including:

  - More frequency offsets, e.g. 4 for BWP less than 50 PRBs, 8 for BWP greater than 50 PRBs.

  - More frequency hopping positions, e.g. 3.

  - More time-domain hop positions within a slot, e.g. 3.

  - [DM-RS sharing among multiple PUSCH transmissions with the same frequency position between two consecutive slots]

- Potential specification impacts of enhancements on intra-slot frequency hopping include:

  - Frequency domain hopping offsets, DM-RS pattern, TBS determination.

  - [Power consistency and phase continuity for DM-RS sharing among multiple PUSCH transmissions]

| Companies | Comments |
| --- | --- |
| CATT | We are fine with FL’s proposal in principle. Regarding to the DMRS sharing bullet, a DMRS may be shared by more than 2 consecutive slots. Maybe deleting ‘two’ will be better. [DM-RS sharing among multiple PUSCH transmissions with the same frequency position between two consecutive slots] |
| Nokia/NSB | Support |
| Huawei, HiSilicon | Agree with CATT’s proposal for the removal of “two”. |
| Intel | We are fine with FL’s proposal. For DMRS sharing, our understanding is that this is already captured in lower DMRS density. Hence, we do not think we need to include this here for intra-slot frequency hopping. |
| Ericsson | Share similar view as Intel that DMRS sharing could be removed which can be covered by the DMRS optimization proposals (DMRS density/pattern related) in agreements below: - Agreements: Capture the followings into the TR - Enhancements on DM-RS density were studied from several aspects, including lower DM-RS density in time domain, DM-RS sharing among multiple PUSCH transmissions in the time domain, lower DMRS density in frequency domain, 1-comb DM-RS, e.g., DM-RS with single port spans to occupy the whole DM-RS symbol, and additional DM-RS symbol position in a slot. - Potential specification impacts of lower DM-RS density in time domain, and DM-RS sharing among multiple PUSCH transmissions include: - DM-RS pattern and configuration, power consistency, phase continuity, and TBS determination. - Potential specification impacts of lower DMRS density in the frequency domain include: - DM-RS design, DM-RS pattern and configuration. - Potential specification impacts of 1-comb DM-RS include: - DM-RS design, and TBS determination. - Potential specification impacts of additional DM-RS symbol position in a slot include: - DM-RS position. |
| Samsung | It is unclear that intra-slot FH can provide meaningful enhancements to coverage. As for the DM-RS design, it does not seem to have been studied specifically for this proposal, and DM-RS is already mentioned in several proposals. |
| Apple | We have a little concern on “More time-domain hop positions within a slot, e.g. 3”, first, the DM-RS overhead is increased; second, the time domain resources in a slot are segmented, it’s hard to schedule the left resources efficiently. |
| FL | From FL’s understanding, “DM-RS sharing” proposed by Nokia in this proposal is a bit different with what we have agreed for DM-RS density. “DM-RS sharing” in this proposal is combined with intra-slot hopping as illustrated in the following figure. Maybe Nokia can explain the details. @Samsung, Apple, In this proposal, we are not discussing the performance, which will be discussed later. |
| Nokia/NSB | We hope that the figure above in the FL’s comment could help to clarify. From our understanding, this case is similar to the case “inter-slot FH to enable cross-slot channel estimation” where “inter-slot” is replaced by “intra-slot” and “cross-slot channel estimation” is replaced by “DMRS sharing”. If cross-slot channel estimation and “inter-slot FH to enable cross-slot channel estimation” are captured separately, we don’t see any reason why we cannot do the same here. In addition, we would like to point out again that this proposal is to capture what has been studied in the TR as the matter of fact. |
| vivo | We are fine to capture in TR, although we are still sceptical on the performance gain. Furthermore, in our understanding, there is no need to keep frequency hopping positions and time domain hop positions. How does time domain hop positions relate to frequency hopping? So, we suggest to remove the following bullet. ‐ More time-domain hop positions within a slot, e.g. 3. |
| FL | @ vivo, As explained by Sony, frequency hopping positions and time domain hop positions are different. - more frequency hop positions relate to frequency-domain positions of hops, and - more time-domain hops relate to time-domain hop positions within a slot. @all, Regarding “DM-RS sharing”, based on FL and Nokia’s explanation, can we remove the brackets? |
| FL | Since no further comments, proposal 7 is revised as follows: - Proposal 7: Capture the followings into the TR - Enhancements on intra-slot frequency hopping were studied from several aspects, including: - More frequency offsets, e.g. 4 for BWP less than 50 PRBs, 8 for BWP greater than 50 PRBs. - More frequency hopping positions, e.g. 3. - More time-domain hop positions within a slot, e.g. 3. - [DM-RS sharing among multiple PUSCH transmissions with the same frequency position between two consecutive slots] - Potential specification impacts of enhancements on intra-slot frequency hopping include: - Frequency domain hopping offsets, DM-RS pattern, TBS determination. - [Power consistency and phase continuity for DM-RS sharing among multiple PUSCH transmissions] |
| Ericsson | If we understand correctly based on the clarification above, DMRS sharing is equal to cross-slot channel estimation or cross-repetition channel estimation. If we read the following agreements carefully, it seems the cross-slot/repetition channel estimation is already captured independently from the inter-slot frequency hopping. Besides, we would assume that cross-transmission channel estimation is only possible for the transmissions on the same frequency position/hop, meaning that no matter whether intra-slot hopping is enabled or not, the cross-transmission channel estimation can only happen on crossed repetitions or crossed slots which was new and not supported in legacy, but covered already by the agreements copied here. (Note, I try to use “cross-transmission” to cover both cross-slot and cross-repetition already agreed) Therefore, there seems still no need to mention “DMRS sharing” in this proposal based on our understanding. Agreements: Capture the followings into the TR - Joint channel estimation or DM-RS bundling with/without optimization of DMRS location/granularity was studied from several aspects, including cross-slot channel estimation over consecutive slots, cross-slot channel estimation over non-consecutive slots, cross-repetition channel estimation within one slot, and inter-slot frequency hopping with inter-slot bundling to enable cross-slot channel estimation. - Potential specification impacts of joint channel estimation or DM-RS bundling include: - Power consistency and phase continuity, DM-RS placement in special slot and DM-RS configuration. - Time domain hopping interval for inter-slot frequency hopping with inter-slot bundling |

- Proposal 8: Capture the followings into the TR

- Joint channel estimation or DM-RS bundling with/without optimization of DMRS location/granularity was studied from several aspects, including cross-slot channel estimation over consecutive slots, cross-slot channel estimation over non-consecutive slots, cross-repetition channel estimation within one slot, and inter-slot frequency hopping with inter-slot bundling to enable cross-slot channel estimation.

- Potential specification impacts of joint channel estimation or DM-RS bundling include:

  - Power consistency and phase continuity, DM-RS placement in special slot and DM-RS configuration.

  - Time domain hopping interval for inter-slot frequency hopping with inter-slot bundling

| Companies | Comments |
| --- | --- |
| FL | It seems the majority support merging proposal 5 “Inter-slot frequency hopping with inter-slot bundling” and proposal 8 “Cross-slot channel estimation”. Regarding the comments from Ericsson, maybe we can make the wording more generic as “joint channel estimation” or “DM-RS bundling”. Regarding “DM-RS placement in special slot”, as a number of companies support it, FL suggests to remove the brackets. Regarding “DMRS antenna ports”, is it included in “Power consistency and phase continuity”? |
| LG | - We are generally fine with Proposal 12. - As mentioned by companies, we need to clarify whether DMRS related specification impacts are included in other proposals or not. |
| Qualcomm | To be consistent with the edit made to Proposal 3, we would like to add TBS determination to this list of potential spec impacts, conditioned on whether this enhancement is coupled with Multi-slot PUSCH or not. |
| CATT | We are fine with the current version. |
| Nokia/NSB | We are fine with the proposal. |
| Huawei, HiSilicon | Fine with FL proposal |
| Intel | Regarding the “DM-RS placement in special slot”, it is still unclear to us the spec impact. It would be good that proponent companies would clarify it. |
| Ericsson | DMRS enhancement related enhancement is already covered quite much in - Agreements: Capture the followings into the TR - Enhancements on DM-RS density were studied from several aspects, including lower DM-RS density in time domain, DM-RS sharing among multiple PUSCH transmissions in the time domain, lower DMRS density in frequency domain, 1-comb DM-RS, e.g., DM-RS with single port spans to occupy the whole DM-RS symbol, and additional DM-RS symbol position in a slot. - Potential specification impacts of lower DM-RS density in time domain, and DM-RS sharing among multiple PUSCH transmissions include: - DM-RS pattern and configuration, power consistency, phase continuity, and TBS determination. - Potential specification impacts of lower DMRS density in the frequency domain include: - DM-RS design, DM-RS pattern and configuration. - Potential specification impacts of 1-comb DM-RS include: - DM-RS design, and TBS determination. - Potential specification impacts of additional DM-RS symbol position in a slot include: - DM-RS position. |
| FL | Regarding the “DM-RS placement in special slot”, CMCC, Huawei/HiSilicon, CATT, InterDigital suggest to keep “DM-RS placement in special slot”. InterDigital has explained it. The comments from InterDigital are copied below: Regarding “DM-RS placement in special slot and DM-RS configuration”, we would like to keep it as a potential specification impact. Cross-slot channel estimation can be applied to non-repetition PUSCH. For example, in the FL’s Proposal 2, an actual repetition of PUSCH that contains more than 14 symbols is considered. In such a configuration, DMRS can be placed in a special slot which will require cross-slot channel estimation. DMRS bundling operation may need a consideration for DMRS in a special slot as well. @Qualcomm, From FL’s understanding, joint channel estimation does not have to be combined with multi-slot PUSCH. For joint channel estimation itself, TBS determination seems not needed. @ Ericsson, The agreements on DM-RS do not cover joint channel estimation. |
| CMCC | Support FL’s proposal. |
| vivo | Support the proposal from FL. |
| FL | No further concerns and proposal 8 seems stable. |

- Proposal 13: Capture the followings into the TR

- UE transmit waveform design to reduce MPR was studied from several aspects, including tone reservation and FDSS (Frequency Domain Spectral Shaping) with spectral extension for QPSK.

- Potential specification impacts include

  - Signalling reserved tones, signalling and/ or design for spectral extension, RF requirements.

Note: For tone reservation, a fraction of tones allocated to a UE are reserved for the UE to shape its waveform; no data is transmitted on these tones.

| Companies | Comments |
| --- | --- |
| InterDigital | We think it is fine to keep QPSK in the text since FDSS was actually studied with QPSK in a contribution. However, we agree with the comment from Huawei that the work should not be limited to QPSK. From our view, for both FDSS and tone reservation, signalling is required. Thus, we have the following proposal for potential specification impacts. - Potential specification impacts include - Signalling, design for spectral extension, RF requirements. |
| Qualcomm | Support |
| Nokia/NSB | The first part of the proposal is just capturing what has been studied, as is being done for all other proposals and other channels. In this sense, we are not sure we should discuss on that part. Indeed, the solution was actually studied with spectral extension and for QPSK in particular. It would be rather inexact to capture something more generic. Therefore, we fully agree with InterDigital and should not remove the part related to QPSK. From our perspective, if Huawei's concerns are about the generality of the possible solutions RAN1 may envision to reduce MPR by waveform optimization/design, we believe that the edits proposed by InterDigital (agreeable to us) do not preclude any other non-QPSK options to be considered for the potential specification impacts. |
| Huawei, HiSilicon | OK with FL proposal. InterDigital seems fine with removal of “QPSK”. We prefer such removal. |
| FL | - Based on the comments, can we revise the proposal as follows: - UE transmit waveform design to reduce MPR was studied from several aspects, including tone reservation and FDSS (Frequency Domain Spectral Shaping), e.g., with spectral extension for QPSK. - Potential specification impacts include - Signalling reserved tones, signalling and/or design for spectral extension, RF requirements. Note: For tone reservation, a fraction of tones allocated to a UE are reserved for the UE to shape its waveform; no data is transmitted on these tones. |
| Nokia/NSB | - The modified version from the FL is not acceptable to us. If some companies have concerns on the potential specification impacts, that is totally OK and we are fine to further discuss. However, we again do not understand why companies still concern on the part which captures the fact that something has been studied. For this proposal in particular, are there any other sources showing results of FDSS without spectral extension and/or with other modulation order than QPSK? If there is no such diversity, then please capture exactly what have been studied here. Having said this, we can only agree to the proposal if the “e.g.” is removed. |
| InterDigital2 | - There was a misunderstanding about our comment so we attempt to clarify here. We support Nokia’s comment to capture what was studied in the observation (which was what we meant by “We think it is fine to keep QPSK in the text since FDSS was actually studied with QPSK in a contribution” in the earlier comment). |
| FL | - If only one company has concern on “with spectral extension for QPSK”, I would suggest to keep it. - Proposal 13: Capture the followings into the TR - UE transmit waveform design to reduce MPR was studied from several aspects, including tone reservation and FDSS (Frequency Domain Spectral Shaping) with spectral extension for QPSK. - Potential specification impacts include - Related signalling, design for spectral extension, RF requirements. Note: For tone reservation, a fraction of tones allocated to a UE are reserved for the UE to shape its waveform; no data is transmitted on these tones. |
| Huawei, HiSilicon | FDSS without spectral extension for pi/2 BPSK is feasible, and has been analysed and compared with FDSS with spectral extension for QPSK in [28], showing PAPR gain. Therefore, we suggest not to restrict FDSS only to spectral extension and QPSK, and investigate different options. To address Nokia’s concern, we suggest the following revision: - Proposal 13: Capture the followings into the TR - UE transmit waveform design to reduce MPR was studied from several aspects, including tone reservation, FDSS (Frequency Domain Spectral Shaping) without spectral extension for pi/2 BPSK, and FDSS (Frequency Domain Spectral Shaping) with spectral extension for QPSK. - Potential specification impacts include - Related signalling, design for spectral shaping with/without extension, RF requirements. Note: For tone reservation, a fraction of tones allocated to a UE are reserved for the UE to shape its waveform; no data is transmitted on these tones. |
| Nokia/NSB | FDSS without spectral extension for pi/2 BPSK, FDSS with and without spectral extension for QPSK have been analysed in our contribution to show that: FDSS with spectral extension for QPSK can achieve lower MPR compared to FDSS without spectral extension for QPSK and close to FDSS without spectral extension for pi/2 BPSK. However, FDSS with spectral extension for QPSK offers significant gain in link performance compared to the other two schemes. Regarding the edits from Huawei, we are fine to list all schemes in the first bullet. This is indeed a good catch. Following the same spirit, we propose a minor additional change (in blue), for completeness. Concerning the second bullet, from our perspective, potential specification impacts of this approach may exist only in case of spectral extension. Therefore, we would like to keep the second bullet as it was before. - Proposal 13: Capture the followings into the TR - UE transmit waveform design to reduce MPR was studied from several aspects, including tone reservation, FDSS (Frequency Domain Spectral Shaping) without spectral extension for pi/2 BPSK, and FDSS (Frequency Domain Spectral Shaping) with and without spectral extension for QPSK. - Potential specification impacts include - Related signalling, design for spectral shaping with/without extension, RF requirements. Note: For tone reservation, a fraction of tones allocated to a UE are reserved for the UE to shape its waveform; no data is transmitted on these tones. |

- Proposal 14: Capture the followings into the TR

- Spatial domain based solutions were studies from several aspects, including multiple layer PUSCH transmission with DFT-S-OFDM and Open-loop/closed loop Tx diversity.

- Potential specification impacts include

  - Mechanism to determine the precoder, e.g. reuse a subset of the R15 codebooks.

| Companies | Comments |
| --- | --- |
| Mitsubishi | Given the small amount of discussions on the topic, we don’t have any grounds for exemplifying only one particular scheme (e.g. subset of Rel.15 codebooks) over any of the other proposed schemes. For the sub-bullet in red, either add to e.g. list “Alamouti-based precoding” and any other proposal from different companies, or completely remove the bullet, since the spec impact is highly dependent on the specific scheme |
| Ericsson | To align with the Msg3 PUSCH discussions on spatial domain: - Potential specification impacts include indication to use close-loop or open-loop TX-D, mechanism to determine the precoder in random access procedure, e.g. reuse a subset of the R15 codebooks, and mechanism to determine the precoder cycling pattern during random access procedure, e.g. on different PUSCH repetitions. We’re also fine to include more examples for other schemes not considered here from other companies. |
| FL | From FL perspective, companies are encouraged to provide inputs on the potential specification impacts. |
| LG | In the spatial domain based solutions, three different solutions (i.e., multiple layer PUSCH transmission with DFT-s-OFDM, Open-loop Tx Diversity, and Close-loop MIMO) are listed up. If my understanding is correct, the multiple layer PUSCH transmission with DFT-s-OFDM is beneficial compared with the multiple layer PUSCH transmission with CP-OFDM in terms of PAPR/CM. But, we don’t think that multi-layer PUSCH transmission itself is not a performance bottleneck. In section 2.5.2, Open-loop based MIMO schemes (i.e., DFT-s-OFDM with Tx diversity (NICT), Alamouti-based transmit diversity (Mitsubishi), different PUSCH spatial filter parameters and different antenna ports (OPPO)) are listed up. There is no candidate solution of Close-loop based MIMO scheme. So, it is better to remove ‘multiple layer PUSCH transmission with DFT-S-OFDM’ and ‘close-loop’. Also, potential specification impact of open-loop Tx diversity can be included rather than ‘Mechanism to determine the precoder, e.g., reuse a subset of the R15 codebooks.’ - Spatial domain based solutions were studies from several an aspects, of including multiple layer PUSCH transmission with DFT-S-OFDM and Open-loop/closed loop Tx diversity. - Potential specification impacts include - Mechanism to determine the precoder, e.g. reuse a subset of the R15 codebooks. - Tx diversity for PUSCH with DFT-s-OFDM, and different PUSCH spatial filter parameters and different antenna ports for different PUSCH slots |
| Mitsubishi | We do not support Ericsson’s proposed wording, since the methods studied for MSG3 belong to coverage enhancements for channels other than PUSCH and PUCCH, were correctly captured there and are not to be transposed directly to PUSCH. Regarding multiple layer DFTsOFDM, we agree that it is a nice to have feature that increases the throughput with DFTsOFDM transmission and which should be supported as a general MIMO design enhancement, but we fail to see how this would provide higher PUSCH coverage as compared to Rel.16 single layer DFTsOFDM PUSCH. We generally support LG’s wording, capturing the class of technologies studied with respect to PUSCH coverage enhancements, and we propose the following clarified wording: - Spatial domain based solutions were studies from several an aspects, of including multiple layer PUSCH transmission with DFT-S-OFDM and Open-loop/closed loop Tx diversity. - Potential specification impacts include - Mechanism to determine the precoder, e.g. reuse a subset of the R15 codebooks. - Signalling related to support of Tx diversity for PUSCH with DFT-s-OFDM, and different PUSCH spatial filter parameters and different antenna ports for different PUSCH slots |
| FL | Suggest to further discussion based on Mitsubishi’ revisions. |
| Ericsson | @LG, and Mitsubishi, Right we’re focusing on the CM gain (by ~ 3dB) from DFT-S-OFDM compared to CP-OFDM. We understand the benefit for cell edge / coverage scenarios may be less obvious. In practice it turns out that rank 2 or higher transmission can be quite common in a cell, and that multilayer transmission can be a mechanism to deliver higher power especially for non-coherent UL MIMO UEs. Figure 1 we provided in the enhancement summary shows a histogram of the UL MIMO rank in a cell when the gNB has 4 or 32 Rx antennas. Rel-15 non-coherent UL MIMO transmission is used, and an FTP model 1 traffic is used. Resource utilization is roughly 40%. The detail setup of this simulation is provided in table 1 in Appendix 1 of our contribution R1-2008419. It can be seen that very few UEs transmit only rank 1. In the 4 Rx case, less than 1% of the UEs transmit rank 1, while for 32 gNB Rx antennas, rank 2 is always used. One major reason for the use of high rank is that non-coherent UL MIMO UEs gain 3 dB more power by transmitting two layers. Therefore, the cubic metric gain from DFT-S-OFDM can be reaped over the vast majority of the cell, instead of being constrained toward the center of the cell, and this can be seen as a coverage enhancement. For closed-loop TX-D during initial access, we agree that early SRS transmission is one way to make sure of the accurate CSI, however it is a complex solution. Depending on the open loop TX-D scheme for PUSCH, it may be possible to use the prior open-loop PUSCH transmissions for closed-loop TX-D e.g. using antenna selection. In this way, with both open loop and closed-loop TX-D, we may not even need any SRS for closed-loop TX diversity. Overall, closed-loop TX-D may be seen as an enhancement on top of open-loop TX-D for the purpose of coverage during initial access, so we’re OK to remove the closed-loop TX-D part in this proposal. |
| FL | - Based on the comments, proposal 14 is revised as follows: - Spatial domain based solutions were studies from several aspects, including multiple layer PUSCH transmission with DFT-S-OFDM and Open-loop/closed loop Tx diversity. - Potential specification impacts include - Mechanism to determine the precoder, e.g. reuse a subset of the R15 codebooks. - Signalling related to support of Tx diversity for PUSCH with DFT-s-OFDM, and different PUSCH spatial filter parameters and different antenna ports for different PUSCH slots |
| Ericsson | While we agree on deleting the closed-loop TX-D and also agree on the potential spec impacts added by other companies, the first bullet of potential spec. impact should be kept as well with adding the mechanism to indicate the support of the multiple-layer transmission with DFT-S-OFDM. Besides, different PUSCH transmissions/repetitions may not be on different slots, so it would be good to use e.g. “transmission” instead. - According to above, please find the updates from our side: - Proposal 14: Capture the followings into the TR - Spatial domain based solutions were studies from several aspects, including multiple layer PUSCH transmission with DFT-S-OFDM and Open-loop/closed loop Tx diversity. - Potential specification impacts include - Mechanism to indicate the support of multiple layer PUSCH transmission with DF-T-S-OFDM and to determine the precoder, e.g. reuse a subset of the R15 codebooks. - Signalling related to support of Tx diversity for PUSCH with DFT-s-OFDM, and different PUSCH spatial filter parameters and different antenna ports for different PUSCH slots transmissions |

- Proposal 15: Capture the followings into the TR

- Power boosting for pi/2 BPSK was studied, including beyond 26 dBm as a function of the UL duty cycle.

- Companies are encouraged to provide the potential specification impacts:

| Companies | Comments |
| --- | --- |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Support. |
| FL | Companies are encouraged to provide inputs on the potential specification impacts. |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Potential specification impacts include defining and allowing a UE to potentially power boost based on the UL time domain resource allocation. This can be either explicit or implicitly done. |
| FL | Based on the comments, proposal 15 is revised as follows: - Proposal 15: Capture the followings into the TR - Power boosting for pi/2 BPSK was studied, including beyond 26 dBm as a function of the UL duty cycle. - Potential specification impacts include - UE behavior for power boosting based on the UL time domain resource allocation, explicit or implicit signaling, RF requirement. |

- Proposal 16: Capture the followings into the TR

- SIP signal compression was studied for enhancement large payload PUSCH including SigComp used for application information compression and the compression efficiency.

- Potential specification impacts include

  - Using compression algorithm to compress the large SIP signaling message in higher layer.

| Companies | Comments |
| --- | --- |
| Ericsson | Fine. |
| FL | No further concerns and proposal 16 seems stable. |

- Proposal 17: Capture the followings into the TR

- Dynamic PUSCH waveform adaptation was studied. Potential specification impacts include:

  - Related signaling design.

| Companies | Comments |
| --- | --- |
| Qualcomm | Support. |
| FL | No further concerns and proposal 17 seems stable. |

Companies are invited to provide views on the others solutions, if any.

| Companies | Comments |
| --- | --- |

- Proposal: Capture the following observation into the TR.

- Observation 1:

- Five sources evaluate the performance of enhancements on PUSCH repetition type A.

  - Three sources show 1.0~6.8 dB performance gain when the actual number of repetition is increased for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type A.

  - Two sources show 2.0~6.4 dB performance gain when the actual number of repetition is increased for eMBB 100kbps at 10% iBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type A.

  - One source shows 1.6 dB performance loss when the maximum number of repetitions is increased to 16 at for eMBB 100kbps 10% iBLER for FR1 FDD, compared to Rel-16 PUSCH repetition type A.

| Companies | Comments |
| --- | --- |
| Qualcomm | Need clarity on what performance gain means. If it is just BLER gains for a fixed throughput, then it should be clarified. This stands in contrast to the gains being claimed for other enhancements below. |
| Ericsson | Increasing the number of repetitions may improve the SNR at target BLER in link level simulation but will also get worse antenna gain due to this lowered SNR. It’s better to check how much the additional actual antenna gain correction is needed as well. At lease it should be noted here that antenna gain loss is not considered in this observation. |

- Observation 2:

- Five sources evaluate the performance of enhancements on PUSCH repetition type B.

  - Four sources show 0.2~2.0 dB performance gain when the actual PUSCH transmission can across the slot boundary and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B.

  - One source shows around 1.4 dB performance gain when the actual PUSCH transmission can across the slot boundary and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR2 TDD, compared to Rel-16 PUSCH repetition type B.

  - One sources shows 0.33~0.38 dB performance gain when the actual PUSCH transmission can across the slot boundary and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B.

  - One source shows the number of RBs can be reduced from 38 to 33, when the actual PUSCH transmission can across the slot boundary and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B.

  - One source shows the number of RBs can be reduced from 30 to 26, when the actual PUSCH transmission can across the slot boundary and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR2 TDD, compared to Rel-16 PUSCH repetition type B.

  - One source shows around 2.0 dB performance gain for RV enhancement for eMBB at 10% iBLER for FR 1 TDD, compared to Rel-16 PUSCH repetition type B.

| Companies | Comments |
| --- | --- |

- Observation 3:

- Seven sources evaluate the performance of TB processing over multi-slot PUSCH.

  - Three sources show 0.6~6.2 dB performance gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1, compared to TB is determined based on single slot in Rel-16.

  - Five sources show 0.8~2.7 dB performance gain when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1, compared to TB is determined based on single slot in Rel-16.

  - One source shows 0.4~2.0 dB performance gain depending on the number of aggregated slots and modulation when TBS determined based on single slot and transmitted in parts over multiple slots for VoIP at 2% rBLER for FR1, compared to TB is determined based on single slot in Rel-16.

  - One source shows 0~1.75 dB performance gain depending on the number of aggregated slots and modulation when TBS determined based on single slot and transmitted in parts over multiple slots for eMBB at 10% iBLER for FR1, compared to TB is determined based on single slot in Rel-16.

| Companies | Comments |
| --- | --- |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Although we have not included in our TDOC, we have seen that the gains with multi slot TB processing improve with a greater number of slots. The gains depend on the number of PRBs and the slots used in the overall TB calculations. |
| InterDigital | Regarding the last 2 observations (starting with “One source shows 0.4~2.0 dB…” and “One source shows 0~1.75 dB performance gain”), these may come from Section 3 in our contribution [24], R1-2009168. If this is correct understanding, our evaluation assumptions actually belong to “TBS determined based on multiple slots and transmitted over multiple slots,” instead of “TBS determined based on single slot and transmitted in parts over multiple slots.” For example, as described in [24], throughout simulation, we made similar assumption compared to Section 2.3 in [10], where 1RB is placed across multiple slots and these RBs are aggregated. Similar to [12], as in option 2B, our assumption is similar to the following : “TBS is determined based on multiple slots and different segment is transmitted in each slot. Transmission in each slot is not self-decodable.” Finally, similar to option 2b in [12], we determined TBS based on N slots and 1RB/slot. Therefore, we would like to ask the feature lead to change “TBS determined based on single slot and transmitted in parts over multiple slots” to “TBS determined based on multiple slots and transmitted over multiple slots” in the last 2 observations. |

- Observation 4:

- Six sources evaluate the performance of inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions.

  - Five sources show 0.3~1.5 dB performance gain for inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions for eMBB at 10% iBLER for FR1, compared to Rel-16 inter frequency hopping.

  - One source shows no gain for inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions for eMBB at 10% iBLER for FR1, compared to Rel-16 inter frequency hopping.

| Companies | Comments |
| --- | --- |

- Observation 5:

- Five sources evaluate the performance of inter-slot frequency hopping with inter-slot bundling and cross-slot channel estimation.

  - Two sources show 0.5~2.5 dB performance gain for inter-slot frequency hopping with inter-slot bundling for VoIP at 2% rBLER depending on bundle size, DM-RS configurations for FR1, compared to Rel-16 inter-slot frequency hopping.

  - One source shows 1.0~1.55 dB performance gain for inter-slot frequency hopping with inter-slot bundling for VoIP at 2% rBLER depending on bundle size, DM-RS configurations for FR2, compared to Rel-16 inter-slot frequency hopping.

  - Three sources show 0.5~1.3 dB performance gain for inter-slot frequency hopping with inter-slot bundling for eMBB at 10% iBLER depending on bundle size for FR1, compared to Rel-16 inter-slot frequency hopping.

| Companies | Comments |
| --- | --- |

- Observation 6:

- Four sources evaluate the performance of sub-PRB transmission with multi-slot aggregation.

  - One source shows around 0.8 dB performance gain for sub-PRB transmission with 6 tones for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission.

  - One source shows 5.6 and 8.5 dB performance gain for sub-PRB transmission with 6 tones for VoIP at 2% rBLER and eMBB at 10% iBELR for FR1, respectively, compared to Rel-16 PRB-based transmission.

  - One source shows up to 5 dB performance gain for sub-PRB transmission with 2 tones for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission.

  - One source shows no gain for sub-PRB transmission for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission.

| Companies | Comments |
| --- | --- |

- Observation 7:

- Twelve sources evaluate the performance of cross-slot channel estimation.

  - Ten sources show 0.4~3 dB performance gain for cross-slot channel estimation for eMBB at 10% iBLER depending on the number of slots for FR1, compared to PUSCH transmission without cross-slot channel estimation.

  - Two sources show 0.9~1.3 dB performance gain for cross-slot channel estimation for VoIP at 2% rBLER depending on bundle size for FR1, compared to PUSCH transmission without cross-slot channel estimation.

  - One source shows 0.85~1.1 dB performance gain for cross-slot channel estimation for VoIP at 2% rBLER depending on bundle size for FR2, compared to PUSCH transmission without cross-slot channel estimation.

| Companies | Comments |
| --- | --- |

- Observation 8:

- Three sources evaluate the performance of lower DM-RS density.

  - One source shows around 1.4 dB performance gain for lower DM-RS density in time domain with cross-slot channel estimation for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density.

  - One source shows around 1.0 dB performance gain for lower DM-RS density in frequency domain for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density.

  - One source shows around 0.2 dB performance loss for lower DM-RS density in time domain for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density.

| Companies | Comments |
| --- | --- |

- Observation 9:

- Three sources evaluate the performance of higher DM-RS density.

  - One source shows 0.5~1.5 dB performance gain for 1-comb DM-RS for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density.

  - One source shows around 1.0 dB performance gain for additional DM-RS symbol position for VoIP at 2% rBLER for FR1, compared to Rel-16 DM-RS density.

  - One source shows around 0.05 dB performance loss for higher DM-RS density in time domain for eMBB 10% iBLER for FR1, compared to Rel-16 DM-RS density.

| Companies | Comments |
| --- | --- |

- Observation 10:

- One source evaluates the performance of adaptive DM-RS configuration and shows around 1.7 dB performance gain for eMBB at 10% iBLER for FR1, compared to Rel-16 semi-static DM-RS configuration.

- One source evaluates the performance of enhanced intra-slot frequency hopping with more frequency offsets/ more frequency hopping positions and shows around 1.8 dB performance gain for VoIP at 2% rBLER and 0.4 dB performance gain for eMBB at 2% rBLER for FR1, compared to Rel-16 intra-slot frequency hopping.

- One source evaluates the performance of power boosting for pi/2 BPSK and shows around 3 dB gain for UL duty cycle less than 50% and around 6 dB gain for UL duty cycle less than 25%.

- One source evaluates the performance of dynamic switching between DFT-S-OFDM and CP-OFDM and shows 1~1.5 dB gain, compared to semi-static switching between DFT-S-OFDM and CP-OFDM.

- One source evaluates the performance of UE transmit waveform design to reduce MPR and shows 1.0~1.5 dB gain, compared to Rel-16 DFT-S-OFDM and CP-OFDM.

- One source evaluates the performance of symbol level repetition and shows around 0.4 dB performance gain for UE speed 3km/h and around 0.3 performance loss for eMBB at 10% iBLER for FR1, compared to Rel-16 PUSCH repetition type A.

- One source evaluates the performance of Alamouti-based transmit diversity and shows 2-2.7dB performance gain for FR1 and 2-3dB performance gain with QPSK, and up to 8.5dB 2-3dB performance gain with 16QAM for FR2.

- One source evaluates the performance of multiple layer PUSCH transmission with DFT-S-OFDM and shows around 3 dB cubic metric gain, compared to multiple layer PUSCH transmission with CP-OFDM.

| Companies | Comments |
| --- | --- |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | As per chairman decision, we need to decide whether RAN1 will continue studying power boosting for pi/2 BPSK or not. As per our results, it is clear that we must continue this study. We suggest that this proposal be made to RAN1 and be captured. |
| Panasonic | On symbol-level repetition (6th bullet in Observation 10), we would like to clarify the UE speed as follows: - One source evaluates the performance of symbol level repetition and shows around 0.4 dB performance gain for UE speed 3km/h and around 0.3 dB performance loss for UE speed 120 km/h, respectively, for eMBB at 10% iBLER for FR1, compared to Rel-16 PUSCH repetition type A. |
| Qualcomm | For the first bullet, the dB gain is clear to see for a fixed MCS. Another way to characterize gains would be to look at increase in throughput gains. If possible, can we add the following line: “At low SNRs such as -10 to -12 dB, dynamic selection of the appropriate DMRS configuration can bring 10-50% increase in throughput compared to an ill-suited DMRS configuration. The exact throughput gains are dependent on factors such as UE speed, DMRS bundling, and PUSCH repetition.” These observations are drawn from the figures presented in the tdoc. For the bullet on dynamic waveform switching, the gains are proportional to the difference in tx power between DFT-S-OFDM and CP-OFDM. Using the MPR table as a reference, can we revise as follows: - One source evaluates the performance of dynamic switching between DFT-S-OFDM and CP-OFDM and shows 1.5-2 dB gain, compared to semi-static switching between DFT-S-OFDM and CP-OFDM when using QPSK modulation. |

- Proposals (5th round)

Proposal 18: Capture the following observation into the TR.

Observation:

- Six sources (China Telecom, ZTE, Intel, DOCOMO, Sierra Wireless, Apple) evaluate the performance of enhancements on PUSCH repetition type A.

  - Three sources (China Telecom, ZTE, DOCOMO) show 1.0~6.8 dB required SNR gain when the actual number of repetition is increased for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type A.

  - Two sources (Intel, DOCOMO) show 2.0 dB and 6.4 dB required SNR gain, respectively, when the actual number of repetition is increased for eMBB 100kbps at 10% iBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type A.

  - One source (Apple) shows 2.2 dB required SNR gain when the maximum number of repetitions is increased to 16 for eMBB at 10% iBLER for FR1 FDD, compared to Rel-16 PUSCH repetition type A.

  - One source (Sierra Wireless) shows 1.6 dB required SNR loss when the maximum number of repetitions is increased to 16 for eMBB 100kbps at 10% iBLER for FR1 FDD, compared to Rel-16 PUSCH repetition type A.

| Companies | Comments |
| --- | --- |
| Huawei, HiSilicon | Firstly, we have the following general question for clarification - Is it correct understanding that company name will be removed from the final observation? - Performance gain shown by simulation results may rely on some assumptions that are not agreed yet, sometime may be additionally at some costs like UE complexity. Currently all the observations conclude only the perspective of performance gain but no other perspective e.g. UE complexity, impact on receiver. Would the other perspectives be concluded at the same time? |
| Qualcomm | Many of the claimed gains need further qualification and a clear identification of the baseline that was used. Reporting 6+ dB gains without acknowledging the additional resources consumed does not seem right. Can we add details on how many additional repetitions were needed for the reported gains? |
| Ericsson | The baseline for gain here is Rel-16 PUSCH repetition, rather than retransmission. So the benefit actually seems to be reduced PDCCH overhead rather than PUSCH coverage. Since the PDCCH resource is proportional to the TDD DL:UL ratio, in these high TDD ratios where UL coverage is limited there can also be a high amount of PDCCH resource available. Also, our understanding of the simulation results shown by ZTE is that retransmission performs 1 to 1.5dB better than doubling the number of repetitions in their setup. Regarding the gains with repetition for eMBB, in some cases the simulation results seem to be for lower data rates than 100kbps, since larger TBS was not used with higher repetition. This should be clarified. Lastly, the array gain in AAS systems can decrease for lower operating SINR due to channel estimation effects. Therefore, the gain observed from these link level results may not be reflected in a full link budget analysis or in system simulations. As a summary, - Some BLER gain may be seen from simply increasing the number of repetitions with same TBS (i.e. lower data rate), but lower BLER also means lower antenna gain depending on the RX beamforming algorithm applied for uplink on the gNB side at least for mid-band; - Data rate targets need to be checked when considering repetition gains. - Since HARQ provides the same or better SINR gain than repetition, repetition can be seen as a way to reduce PDCCH overhead. So, subject to verification from companies providing the results, we propose the following updates to the observations, where the square brackets are removed after verification: - Six sources (China Telecom, ZTE, Intel, DOCOMO, Sierra Wireless, Apple) evaluate the performance of enhancements on PUSCH repetition type A. - Three sources (China Telecom, ZTE, DOCOMO) show 1.0~6.8 dB required SNR gain when the actual number of repetition is increased for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type A. [In simulations with more than 1.5 dB gain, HARQ is not used.] - Two sources (Intel, DOCOMO) show 2.0 dB and 6.4 dB required SNR gain, respectively, when the actual number of repetition is increased for eMBB [for data rates less than] 100kbps at 10% iBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type A. [HARQ is not used]. - One source (Apple) shows 2.2 dB required SNR gain when the maximum number of repetitions is increased to 16 for eMBB at 10% iBLER for FR1 FDD, compared to Rel-16 PUSCH repetition type A. [HARQ is not used]. - One source (Sierra Wireless) shows 1.6 dB required SNR loss when the maximum number of repetitions is increased to 16 for eMBB 100kbps at 10% iBLER for FR1 FDD, compared to Rel-16 PUSCH repetition type A. [HARQ is not used]. |
| Sierra Wireless | The observations for VoIP are acceptable, but we do not feel the above observations for eMBB proper represent the results. For example: Intel’s results in R1-2007952 have a fixed at TBS of 136 bits, thus the require date rates of ~ 100kbps is NOT maintained so there is evidence there will be a gain when the data rate will be maintained at 100kbps. In Apple’s R1-2008479 we only see results comparing no repeats vs 1 repeat where 1 repeat has no SNR gain when FH is disabled and ~ 1dB SNR gain with FH enabled. Also, we could not find repetition result for Docomo’s observation in R1-2008559 nor R1-2008557 – where are these located? |
| Samsung | This is a general comment applicable to all proposals in this section. We suggest to add some additional details in order to qualify better the gains and understand the evaluated solutions. For example, when a gain range is given, it is good to add under which conditions (in this proposal - how many repetitions are used to obtain the gain). A summary of the observed gains, including qualitative observations and other relevant considerations on complexity/implementation and/or assumptions can be added - this might be a further step after the agreement of the observations in current proposals. References to contribution numbers need to be added (company names can be removed). |
| FL | Incorporate the comments from the email. @Huawei, Yes, the company name will be removed from the final observations. Not sure what assumptions are not agreed, and not sure whether it is necessary to include UE complexity and impact on receiver into the observations on performance evaluation. If majority companies think it is indeed necessary, from FL perspective, the concerned companies are encouraged to provide a separate observation on UE complexity and impact on receiver. @Qualcomm, Samsung, Fine to add the number of repetition. @Sierra Wireless, DOCOMO submitted results in the thread “[103-e-NR-CovEnh-EvaluationResults] – enhancements”. The observations are made based on the inputs in that email thread. @Intel, Apple, Please clarify the concerns from Sierra Wireless. @Samsung, Fine to add Tdoc# as reference, but it seems some of the results submitted in “[103-e-NR-CovEnh-EvaluationResults] – enhancements” are not included in the Tdocs. @all, Please check the revision by Ericsson. |
| Apple | I just update our results in the draft folder, the performance of repetition with keeping the same target data rate is included. More specifically, we can get the 0.8dB gain with repetition and frequency hopping. The additional results just show the pure repetition gain, i.e. 2.2dB, without considering the TBS change, which is upper bound of repetition gain. |
| Intel | @ Sierra Wireless, yes, your understanding is correct. We used a fixed TBS of 136 bits in the simulations and no HARQ. We can change the observations as follows: - Two sources (Intel, DOCOMO) One resource (Intel) shows 2.0 dB and 6.4 dB required SNR gain, respectively, when the actual number of repetition is increased for eMBB with TBS of 136 bits [for data rates less than] 100kbps at 10% iBLER for FR1 TFDD, compared to Rel-16 PUSCH repetition type A. [HARQ is not used]. We are not sure the simulation assumption for DOCOMO so we separate the observations above. |
| Qualcomm | Support Ericsson’s changes. ZTE’s claim on 1.5 dB gain between repetition vs HARQ is not clear. Some insight on how/why this gain emerges will be good to understand. |
| ZTE | It seems Ericssion misunderstood our simulation results. In our paper R1-2007743, we show that, for VoIP service, 4 repetitions with maximum 1 re-transmission can provide about 1~1.5 dB gain over 2 repetitions with maximum 3 re-transmissions. That is, under similar latency requirement, repetition could provide more gains than re-transmission. |

Proposal 19: Capture the following observation into the TR.

Observation:

- Five sources (China Telecom, ZTE, vivo, InterDigital, Samsung) evaluate the performance of enhancements on PUSCH repetition type B.

  - Four sources (China Telecom, ZTE, InterDigital, Samsung) show 0.2~2.0 dB required SNR gain when the actual PUSCH transmission can across the slot boundary and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B.

  - One source (Samsung) shows around 1.4 dB required SNR gain when the actual PUSCH transmission can across the slot boundary and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR2 TDD, compared to Rel-16 PUSCH repetition type B.

  - One sources (InterDigital) shows 0.33~1.3 dB required SNR gain when the actual PUSCH transmission can across the slot boundary and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B.

  - One source (InterDigital) shows the number of RBs can be reduced from 38 to 33, when the actual PUSCH transmission can across the slot boundary and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B.

  - One source (InterDigital) shows the number of RBs can be reduced from 30 to 26, when the actual PUSCH transmission can across the slot boundary and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR2 TDD, compared to Rel-16 PUSCH repetition type B.

  - One source (vivo) shows around 2.0 dB required SNR gain for RV enhancement for eMBB at 10% iBLER for FR 1 TDD, compared to Rel-16 PUSCH repetition type B.

| Companies | Comments |
| --- | --- |
| InterDigital | We have a minor comment. In the observations, should “…can across the slot boundary…” be “…can cross the slot boundary…”? |
| Huawei, HiSilicon | For PUSCH transmission across slot-boundary, we have concerns on its difficulty of UE implementation. If TB across slot boundary transmission is introduced with parts over multiple slots, the period of control and scheduling might be interrupted, which will increase the complexity significantly. The proposal should make observation on gains together with its costs, including the increased UE complexity. |
| Qualcomm | Same comment as before. These gains come about because additional resources are used. They also seem to depend on the existence of an ‘S’ slot preceding a U slot. If an S slot exists, its because the gNB intends to use the S for certain purposes (e.g., SRS). To presuppose these slots to be available for data transmission does not seem right. FL: is there a way to capture qualitative comments/observations? |
| Ericsson | In our understanding, cross-slot channel estimation is used for when the actual PUSCH transmission crosses the slot boundary. If this understanding is correct, it should be stated, as the close relationship with cross slot could be missed. An alternative to extending PUSCH transmission across the slot boundary to add a small number of symbols is to increase the code rate. This will increase the required SINR, but not increase resource usage as is done with the extended PUSCH. The increased SINR from the higher code rate can be reduced through HARQ at the cost of overhead from retransmission. Since the use of HARQ could therefore affect the relative performance of cross-slot PUSCH transmission, we think it should be clarified if HARQ is used or not. Therefore, subject to confirmation from companies providing the simulation results, we suggest the following updates, where the square brackets are removed after confirmation: - Five sources (China Telecom, ZTE, vivo, InterDigital, Samsung) evaluate the performance of enhancements on PUSCH repetition type B. - Four sources (China Telecom, ZTE, InterDigital, Samsung) show 0.2~2.0 dB required SNR gain when the actual PUSCH transmission can across the slot boundary[, cross-slot channel estimation is used,] and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B. [HARQ is not used.] - One source (Samsung) shows around 1.4 dB required SNR gain when the actual PUSCH transmission can across the slot boundary[, cross-slot channel estimation is used,] and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR2 TDD, compared to Rel-16 PUSCH repetition type B. [HARQ is not used.] - One sources (InterDigital) shows 0.33~1.3 dB required SNR gain when the actual PUSCH transmission can across the slot boundary[, cross-slot channel estimation is used,] and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B. [HARQ is not used.] - One source (InterDigital) shows the number of RBs can be reduced from 38 to 33, when the actual PUSCH transmission can across the slot boundary[, cross-slot channel estimation is used,] and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B. [HARQ is not used.] - One source (InterDigital) shows the number of RBs can be reduced from 30 to 26, when the actual PUSCH transmission can across the slot boundary[, cross-slot channel estimation is used,] and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR2 TDD, compared to Rel-16 PUSCH repetition type B. [HARQ is not used.] - One source (vivo) shows around 2.0 dB required SNR gain for RV enhancement for eMBB at 10% iBLER for FR 1 TDD, compared to Rel-16 PUSCH repetition type B. |
| FL | @ InterDigital, Fine with the revision. @Huawei, Similar with proposal 18, not sure whether it is necessary to include UE complexity and impact on receiver into the observations on performance evaluation. If majority companies think it is indeed necessary, from FL perspective, the concerned companies are encouraged to provide a separate observation on UE complexity and impact on receiver. @Qualcomm, Not quite understand why additional resources are used. Regarding the comment “is there a way to capture qualitative comments/observations”, do you have any detailed suggestion? @all, Please check the revision by Ericsson. |
| ZTE | For our simulation above, we didn’t assume cross slot channel estimation. Basically, one additional DMRS is added when actual PUSCH transmission crosses the slot boundary. More details could be found in our Tdoc R1-2007743. So, we suggest to separate the observation as follows: - Four Three sources (China Telecom, ZTE, InterDigital, Samsung) show 0.2~2.0 dB required SNR gain when the actual PUSCH transmission can across the slot boundary[, cross-slot channel estimation is used,] and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B. [HARQ is not used.] - One source (ZTE) show 0.8 dB required SNR gain when the actual PUSCH transmission can across the slot boundary or the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B. HARQ is not used. |
| InterDigital | For our simulations, we assumed cross-slot channel estimation. All DMRS are used jointly to estimate channels in the actual PUSCH transmission. We did not use HARQ in our simulation. @ FL, we have extracted observations corresponding to our results. Please note that our observations cross Proposal 19 and 23. Our preference is to keep these observations in both Proposal 19 and 23. - One source (InterDigital) show 0.2~2.0 dB required SNR gain when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B. HARQ is not used. - One sources (InterDigital) shows 0.33~1.3 dB required SNR gain when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B. HARQ is not used. - One source (InterDigital) shows the number of RBs can be reduced from 38 to 33, when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B. HARQ is not used. - One source (InterDigital) shows the number of RBs can be reduced from 30 to 26, when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR2 TDD, compared to Rel-16 PUSCH repetition type B. HARQ is not used. |
| Qualcomm | @FL: We offer the following comment: “These schemes assume that a TDD pattern that has an S slot followed by U slot is used. It is further assumed that the gNB has provisioned this S slot but this S slot remains unused and not assigned for SRS or PUCCH transmissions and is therefore available for PUSCH. For TDD patterns without S slot this scheme offers no gains unless combined with cross-slot channel estimation.” We don’t think such a scenario is likely/typical. If an S slot is provisioned, it is most likely to accommodate SRS or PUCCH. Given that the gains are predicated on the existence of an S slot, the results here don’t easily generalize. |
| Samsung | Regarding the comments on the association with S slot, from our understanding, this is not the only usage of the cross slot boundary case. Even with two UL slots, the enhanced typeB could allow the actual transmission cross the slot boundary. So we think the limitation to tie with S slot is not necessary. Regarding the E///’s comments, we think the type B enhancement is trying to add more symbols in one transmission thus the coding rate will be lower (instead of higher code rate). For the cross slot channel estimation, yes, this could be combined with type B enhancement. But in actual design, it may not be necessary, e.g., the DMRS is per actual transmission basis even though the actual transmission is cross the slot boundary. |

Proposal 20: Capture the following observation into the TR.

Observation:

- Seven sources (China Telecom, IITH, Intel, Qualcomm, InterDigital, Nokia, Sierra Wireless) evaluate the performance of TB processing over multi-slot PUSCH.

  - Two sources (China Telecom, Qualcomm) show 0.6~2 dB required SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1, compared to TB is determined based on single slot with repetition in Rel-16.

  - Four sources (China Telecom, IITH, Qualcomm, Nokia) show 0.8~2.7 dB required SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1, compared to TB is determined based on single slot with repetition in Rel-16.

  - One source (InterDigital) shows 0.4 and 2.0 dB required SNR gain with different number of aggregated slots and modulation schemes when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 FDD, compared to TB is determined based on single slot without repetition in Rel-16.

  - One source (InterDigital) shows 0~1.75 dB required SNR gain depending on the number of aggregated slots and modulation when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1, compared to TB is determined based on single slot without repetition in Rel-16.

  - One source (Intel) shows 0.2 dB required SNR gain and 6.2 dB link budget gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 TDD, compared to TB is determined based on single slot without repetition in Rel-16.

  - One source (Sierra Wireless) shows 2 (w/ frequency hopping) and 2.5 dB (w/o frequency hopping) required SNR gain for codewords determined based on multiple slots and transmitted over multiple slots with gaps for eMBB at 10% iBLER for FR1, compared to Rel-16 where multiple TBs with repeats are scheduled over contiguous slots.

| Companies | Comments |
| --- | --- |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | - Please count our contribution as supported by 5 companies - In the enhancements email discussion, we have shown higher gains upto 3 dB for a VOIP type scenario. We have put the result also in that document. |
| InterDigital | Could you also capture our observation, “Up to 10log(N) dB power boosting gain can be obtained when TBS is determined base on N slots and transmitted over N slots” (Observation 9 in R1-2009583) |
| Huawei, HiSilicon | Similar comment as to proposal 19, for PUSCH transmission across slot-boundary, we have concerns on its difficulty of UE implementation. If TB across slot boundary transmission is introduced with parts over multiple slots, the period of control and scheduling might be interrupted, which will increase the complexity significantly. The proposal should make observation on gains together with its costs, including the increased UE complexity. |
| Nokia/NSB | We are fine with the proposal. Just a minor editorial comment: could you please change “Nokia” to “Nokia/NSB”? |
| Qualcomm | In response to Huawei comment, while we share your concerns, our original intent here was to pursue TB scaling with repetitions and RV cycling. This sidesteps the issues you bring up and preserves the meaning and relevance of a slot. A UE can continue to use its slot-based architecture. This does not require slots to be contiguous as well. |
| Ericsson | Our understanding of at least some of these results is that multi-slot TBS assumes that the number of PRBs is scaled down by the number of slots in the transmission, for example a 1 slot transmission would have 4 PRBs while a 4 slot would have one PRB. In this example, the EPRE rises by 6 dB in the 4 slot case, which improves channel estimation for the DMRS. If the 4 PRB transmission were instead repeated, we have an equivalent increase in the total PUSCH energy, although the EPRE for the DMRS in each transmission is lower. Therefore, a potential benefit of multi-slot TBS compared seems to be better channel estimation in the presence of the delay spread. If we understand correctly, neither HARQ is not used as a baseline. Since HARQ is also a way to provide additional PUSCH energy, we think it is important that this be captured. Furthermore, even if the repetitions are used, a company may only have used one RV for all the repetitions, which should be clarified as well. Therefore, subject to confirmation from companies providing the simulation results, we suggest the following updates, where the square brackets are removed after confirmation: - Seven sources (China Telecom, IITH, Intel, Qualcomm, InterDigital, Nokia, Sierra Wireless) evaluate the performance of TB processing over multi-slot PUSCH. - Two sources (China Telecom, Qualcomm) show 0.6~2 dB required SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1, compared to TB is determined based on single slot with repetition in Rel-16. [HARQ is not used] [The same redundancy version is used for repetitions]. - Four sources (China Telecom, IITH, Qualcomm, Nokia) show 0.8~2.7 dB required SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1, compared to TB is determined based on single slot with repetition in Rel-16. [HARQ is not used] [The same redundancy version is used for repetitions]. - One source (InterDigital) shows 0.4 and 2.0 dB required SNR gain with different number of aggregated slots and modulation schemes when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 FDD, compared to TB is determined based on single slot without repetition in Rel-16. [HARQ is not used] - One source (InterDigital) shows 0~1.75 dB required SNR gain depending on the number of aggregated slots and modulation when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1, compared to TB is determined based on single slot without repetition in Rel-16. [HARQ is not used] - One source (Intel) shows 0.2 dB required SNR gain and 6.2 dB link budget gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 TDD, compared to TB is determined based on single slot without repetition in Rel-16. [HARQ is not used] - One source (Sierra Wireless) shows 2 (w/ frequency hopping) and 2.5 dB (w/o frequency hopping) required SNR gain for codewords determined based on multiple slots and transmitted over multiple slots with gaps for eMBB at 10% iBLER for FR1, compared to Rel-16 where multiple TBs with repeats are scheduled over contiguous slots. [HARQ is not used] |
| Sierra Wireless | We are confused by the wording ‘required SNR gain’ in each of the observations. We don’t see why it is “required” so suggest deleted the word “required” to read as “SNR gain”. Although we preferred the word “codewords” vs “TBS”, we would like to be consistent with the other observations, so please update our observation as: One source (Sierra Wireless) shows 2 (w/ frequency hopping) and 2.5 dB (w/o frequency hopping) SNR gain for TBS determined based on multiple slots and transmitted over multiple slots with gaps for eMBB at 10% iBLER for FR1, compared to Rel-16 where multiple TBs with repeats are scheduled over contiguous slots. |
| FL | @IITH, Nokia, Regarding the number of companies, from FL understanding, the number of sources refers to the number of contributions or the number of results, while it does not refer to the number of companies. And the company name will be removed from the final observations. @IITH, It seems the results provided in section 2.1 is not aligned with the results in Appendix in [103-e-NR-CovEnh-EvaluationResults]. @InterDigital, The observations are made based on the thread “[103-e-NR-CovEnh-EvaluationResults] – enhancements”, please make sure all the results are included in the document. @Huawei, Similar with proposal 18, not sure whether it is necessary to include UE complexity and impact on receiver into the observations on performance evaluation. If majority companies think it is indeed necessary, from FL perspective, the concerned companies are encouraged to provide a separate observation on UE complexity and impact on receiver. @ Sierra Wireless, Fine to change “required SNR gain” to “SNR gain”, if everyone is fine. @all, Please check the revision by Ericsson. |
| China Telecom | For the baseline performance of TB determined based on single slot with repetition in Rel-16, we use different RVs for repetitions, i.e. RV = [0,2,3,1]. |
| Apple | If HARQ is no used, the gain will be overestimated, because if data in one slot could be coded correctly, the whole transmission will be retransmitted over multiple slots. |
| InterDigital | In our simulations, HARQ is not incorporated in the baseline. Regarding the power boosting gain, we will refer to the values shown in “[103-e-NR-CovEnh-EvaluationResults] – enhancements”. Please find the modified observations below. - One source (InterDigital) shows 0.4 and 2.0 dB required SNR gain and up to 10dB power boosting gain with different number of aggregated slots and modulation schemes when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 FDD, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. - One source (InterDigital) shows 0~1.75 dB required SNR gain and and up to 6.98dB power boosting gain depending on the number of aggregated slots and modulation when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. |
| Intel | In the simulations, we did not assume HARQ. We are fine to add HARQ is not used. |
| Qualcomm | @Ericsson, if we are to use HARQ for the 4 PRB case, we would have consumed a lot more RBs compared to the proposed scheme. In your example, if we allow for a total of 4 transmissions (3 retransmissions), we would have used a total of 16 RBs spread across 4 slots and will be comparing this with a scheme that uses a total of 4 PRBs. We felt this would not be a fair comparison. Our comparison was to instead look at a fixed cost to the network (4 PRBs say) and see what we would gain by TB scaling. Gains are expected due to improved coding performance and a reduction in overhead. Same arguments were made in LTE study. We offer the following edit: “HARQ is not used; if HARQ is enabled, legacy transmissions incur increased freq-domain resource use.” Regarding the comment on RV index, we can reword as follows: “In [Qualcomm] simulations, a single RV index is used when transmitting a TB across multiple slots. This is however not a prerequisite for this scheme and is open for further discussion.” In general, the intent here is to prevent unnecessary segmentation of the payload and to leverage coding gains by moving to larger TB sizes. We can include a comment to capture this: “Gains due to reduction in upper-layer (MAC/RLC/PDCP) headers is not reflected in [Qualcomm] simulations.” |

Proposal 21: Capture the following observation into the TR.

Observation:

- Six sources (ZTE, Intel, Qualcomm, vivo, Ericsson, Sierra Wireless) evaluate the performance of inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions.

  - Five sources (ZTE, Intel, vivo, Ericsson, Sierra Wireless) show 0.3~1.5 dB required SNR gain for inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions for eMBB at 10% iBLER for FR1, compared to Rel-16 inter frequency hopping.

  - One source (Qualcomm) shows no gain for inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions for eMBB at 10% iBLER for FR1, compared to Rel-16 inter frequency hopping.

  - One source (Ericsson) shows no gain for inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions and joint channel estimation over multiple slots is implemented for eMBB at 10% iBLER for FR1, compared to Rel-16 inter frequency hopping with joint channel estimation over multiple slots.

| Companies | Comments |
| --- | --- |
| Sierra Wireless | Sierra Wireless studied inter-slot frequency hopping with only one offset. We should not be included in this proposal. |
| FL | @ Sierra Wireless Fine. Corresponding revisions as follows: - FiveFour sources ([ZTE], [Intel], [vivo], [Ericsson], [Sierra Wireless]) show 0.3~1.5 dB required SNR gain for inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions for eMBB at 10% iBLER for FR1, compared to Rel-16 inter frequency hopping. Then, can we remove the results in [103-e-NR-CovEnh-EvaluationResults] accordingly? |
| Ericsson | Performance gain from increasing number of frequency hops depends on the delay spread and number of RX antennas and whether joint (e.g. cross-slot) channel estimation is used or not. With more BS Rx antennas, e.g. 4 RX, the gain from more hops can hardly be seen. With low delay spread, e.g. 30ns, the channel is flat in frequency domain and the gain from more hops can be little even if 2 RX is used. With cross-slot channel estimation, less number of frequency hops is preferred since single channel assumption can only be on the slots on the same hop, and the gain from increasing number of hops is zero. According to above, we propose some updates to this observation. Observation: - Six sources (ZTE, Intel, Qualcomm, vivo, Ericsson, Sierra Wireless) evaluate the performance of inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions. - Five sources (ZTE, Intel, vivo, Ericsson, Sierra Wireless) show 0.3~1.5 dB required SNR gain for inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions for eMBB at 10% iBLER for FR1, compared to Rel-16 inter frequency hopping. Cross-slot channel estimation is not used, 2 RX is assumed for gains higher than 0.3dB, [300ns] is assumed for gains higher than 0.3dB. - One source (Qualcomm) shows no gain for inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions for eMBB at 10% iBLER for FR1, compared to Rel-16 inter frequency hopping. - One source (Ericsson) shows no gain for inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions and joint channel estimation over multiple slots is implemented for eMBB at 10% iBLER for FR1, compared to Rel-16 inter frequency hopping with joint channel estimation over multiple slots. |

Proposal 22: Capture the following observation into the TR.

Observation:

- Four sources (China Telecom, Samsung, vivo, Sierra Wireless) evaluate the performance of sub-PRB transmission with multi-slot aggregation.

  - One source (China Telecom) shows around 0.8 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with repetition.

  - One source (Samsung) shows around 5.6 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission without repetition.

  - One source (Samsung) shows around 1.6 and 8.5 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for eMBB at 10% iBELR for FR1, respectively, depending on the number of aggregation slots, compared to Rel-16 PRB-based transmission without repetition.

  - One source (Sierra Wireless) evaluates the performance of sub-PRB transmission with 2 tones to reduce MPR and shows 0~5 dB  gain for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission.

  - One source (vivo) shows no gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with repetition.

| Companies | Comments |
| --- | --- |
| Ericsson | We do not understand how sub-PRB transmission can give more than a dB or 2 performance gain. A 23 dBm one PRB transmission carries the same energy as a 23 dBm half PRB transmission. The half PRB would have better EPRE which can lead to better channel estimation, and in our understanding is the main potential benefit of sub-PRB from a link level perspective. The second and third bullets with 5.6 and 8.5 dB gain compares 4 PRBs to ½ PRB if we understand correctly. Also, in our understanding HARQ is not used, which is relevant given the difference in the available resource for sub-PRB and the baseline Rel-16 transmission. Regarding the third fourth bullet with 5 dB gain, our understanding is that this is PAPR reduction, which overestimates the PA backoff reduction. Our guess is that the baseline is CP-OFDM, but we would ask Sierra to verify. Therefore, subject to confirmation from companies providing the simulation results, we suggest the following updates, where the square brackets are removed after confirmation: - Four sources (China Telecom, Samsung, vivo, Sierra Wireless) evaluate the performance of sub-PRB transmission with multi-slot aggregation. - One source (China Telecom) shows around 0.8 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with repetition. [HARQ was not used.] - One source (Samsung) shows around 5.6 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission [with 4 PRBs] without repetition. [HARQ was not used.] - One source (Samsung) shows around 1.6 and 8.5 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for eMBB at 10% iBELR for FR1, respectively, depending on the number of aggregation slots, compared to Rel-16 PRB-based transmission [with 4 PRBs] without repetition. [HARQ was not used.] - One source (Sierra Wireless) evaluates the performance of sub-PRB transmission with 2 tones to reduce MPR and shows 0~5 dB 5 dB PAPR reduction gain for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission [with CP-OFDM]. - One source (vivo) shows no gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with repetition. [HARQ was not used.] |
| Sierra Wireless | Support the observations |
| FL | @all, Please check the revision by Ericsson. |
| Apple | For sub-PRB transmission for eMBB, how can sub-PRB resource allocation fulfil the target data rate? The third bullet mention that sub-PRB transmission has 1.6dB and 8.5dB gain. Just checked the simulation assumption, with urban TDD@4GHz DDDSUDDSUU, even the highest coding rate for QPSK could not reach the target data rate100kbps. Half PRB transmission can get 3dB power boosting gain, but the cost is the coding rate increase, which diminish the gain or even cause the performance loss. |
| Samsung | We support the observation from FL, and fine with revision from E/// in principle. Regarding the revision from E///, specifically for the 3rd bullet, one correction is for 1.6dB gain case, the Rel-16 normal PUSCH is using 1PRB instead of 4PRBs, so suggested change on E///’s version: - One source (Samsung) shows around 1.6 and 8.5 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for eMBB at 10% iBELR for FR1, respectively, depending on the number of aggregation slots, compared to Rel-16 PRB-based transmission [with1PRb and 4 PRBs, respectively,] without repetition. [HARQ was not used.] Regarding the comments from Apple, for a quick calculation, for 100kbps data rate, 100bits per ms, since in our simulation we use SCS=30khz, then 50bits per slot (0.5ms), it’s ending up requiring a code rate around 0.18 for one PRB one slot (12 OFDM symbols). The lowest coding rate in Table 6.1.4.1-1 for QPSK is also around 0.18, certainly with higher code rate, larger data rate can achieved. Sub-PRB will increase the code rate, that’s the trade-off from coding gain to power gain. Our results are exactly to show in some cases, the obtained power gain can over-play the loss of coding gain which ends up better link budgets. |

Proposal 23: Capture the following observation into the TR.

Observation:

- Fourteen sources (China Telecom, ZTE, Intel, Qualcomm, Sharp, Panasonic, DOCOMO, Samsung, CMCC, vivo, Ericsson, Nokia, Apple, InterDigital) evaluate the performance of joint channel estimation.

  - Ten sources (China Telecom, ZTE, Qualcomm, Sharp, Panasonic, CMCC, vivo, Ericsson, Nokia, Apple) show 0.2~2 dB required SNR gain for joint channel estimation over multiple slots for eMBB at 10% iBLER depending on the number of slots for FR1, compared to Rel-16 PUSCH transmission without joint channel estimation.

  - One source (Intel) shows 2 dB required SNR gain for joint channel estimation over multiple non-consecutive slots with inter-slot frequency hopping for eMBB at 10% iBLER, compared to Rel-16 inter-slot frequency hopping without joint channel estimation.

  - Three sources (Samsung, NTT DOCOMO, Interditial) show 0.3~1.3 dB required SNR gain for joint channel estimation over multiple slots for VoIP at 2% rBLER depending on the number of slots for FR1, compared to Rel-16 PUSCH transmission without joint channel estimation.

  - One source shows (Samsung) 0.85~1.1 dB required SNR gain for joint channel estimation over multiple slots for VoIP at 2% rBLER depending on the number of slots for FR2, compared to Rel-16 PUSCH transmission without joint channel estimation.

  - One source (vivo) shows 0.8 dB required SNR gain for joint channel estimation over multiple repetition within a slot, compared to Rel-16 PUSCH transmission without joint channel estimation.

- Five sources (China Telecom, ZTE, Intel, Samsung, Ericsson) evaluate the performance of inter-slot frequency hopping with inter-slot bundling and joint channel estimation.

  - Two sources (China Telecom, Samsung) show 0.5~2.5 dB required SNR gain for inter-slot frequency hopping with inter-slot bundling for VoIP at 2% rBLER depending on bundle size, DM-RS configurations for FR1, compared to Rel-16 inter-slot frequency hopping.

  - One source (Samsung) shows 1.0~1.55 dB required SNR gain for inter-slot frequency hopping with inter-slot bundling for VoIP at 2% rBLER depending on bundle size, DM-RS configurations for FR2, compared to Rel-16 inter-slot frequency hopping.

  - Three sources (ZTE, Intel, Ericsson) show 0.5~3 dB required SNR gain for inter-slot frequency hopping with inter-slot bundling for eMBB at 10% iBLER depending on bundle size for FR1, compared to Rel-16 inter-slot frequency hopping.

  - One source (Intel) shows 1 dB required SNR gain for inter-slot frequency hopping with inter-slot bundling and joint channel estimation over multiple slot for eMBB at 10% iBLER for FR1, compared to Rel-16 inter-slot frequency hopping with joint channel estimation over multiple non-consecutive slots.

| Companies | Comments |
| --- | --- |
| InterDigital | Please also add an observation (Observation 6 from R1-2009583) which captures that for VoIP, 0.3dB BLER gain can be improved by performing cross-slot channel estimation with DMRS in a special slot. In the third sub-bullet, under the first bullet, “Interditial” should be “InterDigital.” |
| Nokia/NSB | We are fine with the proposal. Just a minor editorial comment: could you please change “Nokia” to “Nokia/NSB”? |
| FL | @InterDigital, Fine. Corresponding revisions as follows: - ThreeTwo sources ([Samsung], [NTT DOCOMO], [InterDigital]) show 0.30.9~1.3 dB required SNR gain for joint channel estimation over multiple slots for VoIP at 2% rBLER depending on the number of slots for FR1, compared to Rel-16 PUSCH transmission without joint channel estimation. - One source ([InterDigital]) shows 0.3 dB SNR gain for joint channel estimation over multiple slots with DMRS in a special slot for VoIP at 2% rBLER, compared to Rel-16 PUSCH transmission without joint channel estimation. @Nokia, Regarding the number of companies, from FL understanding, the number of sources refers to the number of contributions or the number of results, while it does not refer to the number of companies. And the company name will be removed from the final observations. |
| vivo | According to our comments for proposal 24, the observations related to joint channel estimation in proposal 24, should be included in proposal 23 rather than in proposal 24. And we suggest to revise the proposal as follows. - Fourteen sources (China Telecom, ZTE, Intel, Qualcomm, Sharp, Panasonic, DOCOMO, Samsung, CMCC, vivo, Ericsson, Nokia, Apple, InterDigital) evaluate the performance of joint channel estimation. - Ten sources (China Telecom, ZTE, Qualcomm, Sharp, Panasonic, CMCC, vivo, Ericsson, Nokia, Apple) show 0.2~2 dB required SNR gain for joint channel estimation over multiple slots for eMBB at 10% iBLER depending on the number of slots for FR1, compared to Rel-16 PUSCH transmission without joint channel estimation. - One source (Intel) shows 2 dB required SNR gain for joint channel estimation over multiple non-consecutive slots with inter-slot frequency hopping for eMBB at 10% iBLER, compared to Rel-16 inter-slot frequency hopping without joint channel estimation. - Three sources (Samsung, NTT DOCOMO, Interditial) show 0.3~1.3 dB required SNR gain for joint channel estimation over multiple slots for VoIP at 2% rBLER depending on the number of slots for FR1, compared to Rel-16 PUSCH transmission without joint channel estimation. - One source shows (Samsung) 0.85~1.1 dB required SNR gain for joint channel estimation over multiple slots for VoIP at 2% rBLER depending on the number of slots for FR2, compared to Rel-16 PUSCH transmission without joint channel estimation. - One source (vivo) shows 0.8 dB required SNR gain for joint channel estimation over multiple repetition within a slot, compared to Rel-16 PUSCH transmission without joint channel estimation. - Two sources (CMCC, vivo) show 1.0~1.4 dB required SNR gain for lower DM-RS density in time domain with joint channel estimation [and using multi-slot PUSCH with 4 symbols in the special slot] over multiple slots for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density without joint channel estimation. - One source (vivo) shows 1.0 dB required SNR gain for lower DM-RS density in time domain with joint channel estimation over multiple repetition within a slot for eMBB at 10% iBLER for FR1, compared to Rel-16 PUSCH transmission without joint channel estimation. |
| CMCC | Since the performance gain is mainly based on the cross channel estimation, which provide the opportunity to reduce the DMRS density, we suggest to capture our results under proposal 23. In addition, if the 1.4dB gain is from our contribution, I believe 1.22 dB is the correct value. The updated proposal is as below and the modification is highlighted. - Two sources (CMCC, vivo) show 1.0~1.4 1.22 dB required SNR gain for lower DM-RS density in time domain with joint channel estimation [and using multi-slot PUSCH with 4 symbols in the special slot] over multiple slots for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density without joint channel estimation. |

Proposal 24: Capture the following observation into the TR.

Observation:

- Four sources (ZTE, Intel, CMCC, vivo) evaluate the performance of lower DM-RS density.

  - Two sources (CMCC, vivo) show 1.0~1.4 dB required SNR gain for lower DM-RS density in time domain with joint channel estimation over multiple slots for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density without joint channel estimation.

  - One source (vivo) shows 1.0 dB required SNR gain for lower DM-RS density in time domain with joint channel estimation over multiple repetition within a slot for eMBB at 10% iBLER for FR1, compared to Rel-16 PUSCH transmission without joint channel estimation.

  - One source (ZTE) shows around 1.0 dB required SNR gain for lower DM-RS density in frequency domain for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density.

  - One source (Intel) shows around 0.2 dB required SNR loss for lower DM-RS density in time domain for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density.

| Companies | Comments |
| --- | --- |
| Ericsson | The result in the first bullet with 1.4 dB is from R1-2008026 if I understand correctly, and so the gain is from both using multi-slot PUSCH with 4 symbols in the special slot and lower DMRS density, so I suggest the following. Perhaps CMCC and vivo can verify. Actually, the gains seen in the 1st 2 bullets are close to what is already captured in proposal 23 for joint channel estimation, meaning that DMRS density optimization can hardly have gains when joint channel estimation is used. If results in the other bullets with respect to DMRS density in time domain are also using multi-slot PUSCH, they should be clarified as well. Besides, regarding the DMRS density reduction in frequency domain, we can understand power boosting from removing the comb, however not having DMRS in a whole PRB is another matter, and the gain provided in R1-2007743 assumes only 1 RX antenna (please correct if this is a typo), single front-loaded DMRS and without frequency hopping enabled, which should captured. Therefore, subject to confirmation from companies providing the simulation results, we suggest the following updates, where the square brackets are removed after confirmation: - Four sources (ZTE, Intel, CMCC, vivo) evaluate the performance of lower DM-RS density. - Two sources (CMCC, vivo) show 1.0~1.4 dB required SNR gain for lower DM-RS density in time domain with joint channel estimation [and using multi-slot PUSCH with 4 symbols in the special slot] over multiple slots for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density without joint channel estimation. - One source (vivo) shows 1.0 dB required SNR gain for lower DM-RS density in time domain with joint channel estimation over multiple repetition within a slot for eMBB at 10% iBLER for FR1, compared to Rel-16 PUSCH transmission without joint channel estimation. - One source (ZTE) shows around 1.0 dB required SNR gain for lower DM-RS density in frequency domain for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density. The results is based on the assumption with only 1 RX antenna, single front-loaded DMRS symbol and without frequency hopping. - One source (Intel) shows around 0.2 dB required SNR loss for lower DM-RS density in time domain for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density. |
| FL | @all, Please check the revision by Ericsson. |
| vivo | @ Ericsson For 1st sub-bullet, in fact, our simulation results are based on repetition type A, we did not consider the special slots. @ FL For performance gain obtained by low DMRS density, low DMRS density + joint CE should be compared with joint CE with Rel-16 density, rather than Rel-16 density w/o joint channel estimation. Actually, we would like to clarify that we consider lower DMRS density as one additional enhanced solution on the premise that joint channel estimation/DMRS bundling is configured. We do not see it as a standalone enhancement. Hence, we would like to move the 1st and 2nd sub-bullet to proposal 23, as follows - Four sources (ZTE, Intel, CMCC, vivo) evaluate the performance of lower DM-RS density. - Two sources (CMCC, vivo) show 1.0~1.4 dB required SNR gain for lower DM-RS density in time domain with joint channel estimation [and using multi-slot PUSCH with 4 symbols in the special slot] over multiple slots for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density without joint channel estimation. - One source (vivo) shows 1.0 dB required SNR gain for lower DM-RS density in time domain with joint channel estimation over multiple repetition within a slot for eMBB at 10% iBLER for FR1, compared to Rel-16 PUSCH transmission without joint channel estimation. - One source (ZTE) shows around 1.0 dB required SNR gain for lower DM-RS density in frequency domain for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density. The results is based on the assumption with only 1 RX antenna, single front-loaded DMRS symbol and without frequency hopping. - One source (Intel) shows around 0.2 dB required SNR loss for lower DM-RS density in time domain for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density. |
| CMCC | @E/// and FL Thanks for the detailed check. E///’s understanding of special slot is right. We have no problem with the additional description about that. For the question from E///, the 1.4dB performance gain should be compared with the 0.4dB pure cross channel estimation gain. Lower the density of DMRS without cross channel estimation cannot bring any performance enhancement. In addition, our evaluation results are firstly based on the cross slot channel estimation and then the system could see a chance to reduce the DMRS density in the time domain. From our side, we also prefer to move our results to the proposal 23, as the performance gain is mainly from cross channel estimation. |
| ZTE | We are fine with above modifications. |

Proposal 25: Capture the following observation into the TR.

Observation:

- Three sources (China Telecom, Intel, DOCOMO) evaluate the performance of higher DM-RS density.

  - One source (China Telecom) shows 0.5~1.5 dB required SNR gain for 1-comb DM-RS for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density.

  - One source (DOCOMO) shows around 1.0 dB required SNR gain for additional DM-RS symbol position for VoIP at 2% rBLER for FR1, compared to Rel-16 DM-RS density.

  - One source (Intel) shows around 0.05 dB required SNR loss for higher DM-RS density in time domain for eMBB 10% iBLER for FR1, compared to Rel-16 DM-RS density.

| Companies | Comments |
| --- | --- |
| Ericsson | For the first sub-bullet, the 2-comb DMRS in R16 should be able to do 3dB power boosting, which seems not considered in the baseline simulation and should be captured. And another issue, it’s not clear to us whether same EPRE is assumed for the baseline 2-comb DMRS and the enhanced 1-comb DRMS, maybe the proponent company can clarify. For the 2nd sub-bullet, the baseline is not using the maximum number of DMRS symbols supported in R16, only single DMRS is used based on our understanding. Observation: - Three sources (China Telecom, Intel, DOCOMO) evaluate the performance of higher DM-RS density. - One source (China Telecom) shows 0.5~1.5 dB required SNR gain for 1-comb DM-RS for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density without power boosting. - One source (DOCOMO) shows around 1.0 dB required SNR gain for additional DM-RS symbol position for VoIP at 2% rBLER for FR1, compared to Rel-16 DM-RS density. Only single DMRS symbol is used for baseline. - One source (Intel) shows around 0.05 dB required SNR loss for higher DM-RS density in time domain for eMBB 10% iBLER for FR1, compared to Rel-16 DM-RS density. |
| FL | @all, Please check the revision by Ericsson. |

Proposal 26: Capture the following observation into the TR.

Observation:

- One source (Qualcomm) evaluates the performance of adaptive DM-RS configuration and shows 1.7 dB required SNR gain for eMBB at 10% iBLER for FR1, compared to Rel-16 semi-static DM-RS configuration. For low SNR such as -10 to -12 dB, it shows that adaptive DM-RS configuration can bring 10-50% increase in throughput compared to an ill-suited DMRS configuration depending on factors such as UE speed, DMRS bundling, and PUSCH repetition.

- One source (China Telecom) evaluates the performance of enhanced intra-slot frequency hopping with more frequency offsets/ more frequency hopping positions and shows around 1.8 dB required SNR gain for VoIP at 2% rBLER and 0.4 dB required SNR gain for eMBB at 2% rBLER for FR1, compared to Rel-16 intra-slot frequency hopping.

- One source (IITH) evaluates the performance of power boosting for pi/2 BPSK and shows around 3 dB gain for UL duty cycle less than 50% and around 6 dB gain for UL duty cycle less than 25%.

- One source (Qualcomm) evaluates the performance of dynamic switching between DFT-S-OFDM and CP-OFDM and shows 2~3 dB gain, compared to semi-static switching between DFT-S-OFDM and CP-OFDM when using QPSK modulation.

- One source (Qualcomm) evaluates the performance of UE transmit waveform design to reduce MPR and shows 1 ~1.5 dB gain, compared to Rel-16 DFT-S-OFDM and CP-OFDM.

- One source (Panasonic) evaluates the performance of symbol level repetition and shows around 0.4 dB required SNR gain for UE speed 3km/h and around 0.3dB required SNR loss for UE speed 120km/h, respectively, for eMBB at 10% iBLER for FR1, compared to Rel-16 PUSCH repetition type A.

- One source (Mitsubishi) evaluates the performance of Alamouti-based transmit diversity and shows 2-2.7dB required SNR gain for FR1 and 2-3dB required SNR gain with QPSK, and up to 8.5dB 2-3dB required SNR gain with 16QAM for FR2.

- One source (Ericsson) evaluates the performance of multiple layer PUSCH transmission with DFT-S-OFDM and shows around 3 dB cubic metric gain, compared to multiple layer PUSCH transmission with CP-OFDM.

| Companies | Comments |
| --- | --- |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Please count our results on power boosting to be supported by 5 companies. |
| Qualcomm | Request for another minor edit to the first bullet: “For low SNR such as -10 to -12 dB, it shows that adaptive DM-RS configuration can bring 10-100% increase in throughput compared to an ill-suited DMRS configuration depending on factors such as UE speed, DMRS bundling, and PUSCH repetition.” |
| FL | @IITH, Regarding the number of companies, from FL understanding, the number of sources refers to the number of contributions or the number of results, while it does not refer to the number of companies. And the company name will be removed from the final observations. @Qualcomm, Fine. |

- Discussion (5th round)

We are starting the discussion on recommendations on the enhanced solutions for the follow-up WI. Companies are encouraged to fill in the following tables and provide comments.

8.1 Enhancements on PUSCH repetition type A

|  |  | Supporting companies | Companies have concerns |
| --- | --- | --- | --- |
| Enhancements on PUSCH repetition type A | increasing the maximum number of repetitions | HW, HiSi, CTC, Intel, CATT, Spreadtrum, OPPO, Sharp, Apple, Qualcomm |  |
|  | the number of repetitions counted on the basis of available UL slots and | HW, HiSi, CTC, vivo, Intel, Samsung, ZTE, CATT, CMCC, Panasonic, OPPO, LG, Interdigital, NTT DOCOMO, ETRI, Apple, Sierra Wireless |  |
|  | flexible symbol resource allocation in different slots. | CMCC, LG, ETRI | Nokia, NSB, Apple, Qualcomm |

| Companies | Comments |
| --- | --- |
| Nokia/NSB | We would like to ask for clarification on the intention of specifying the enhancements on PUSCH repetition type A. Is such enhancement aiming at extending PUSCH repetition type A functionality or a new PUSCH repetition type? |
| Sierra Wireless | Sierra Wireless supports: “the number of repetitions counted on the basis of available UL slots“ as this solve the issue for TDD of not enough “real” repeats. With the above fix, the need to “increasing the maximum number of repetitions” for eMBB is unclear but increasing from 16 to e.g. 32 maybe OK for the VoIP case. Also, going beyond 32 repeats would enter into the LPWA services area which goes beyond the SID scope which states: - “LPWA services and scenarios are not included.” Thus we would be OK to agree to “increasing the maximum number of repetitions to 32” |
| Apple | We share the similar view as Nokia/NSB, the definition of repetition type A is clearly defined in the spec. If the definition is changing, it could be better to name it as a new repetition type |
| Intel | @ Nokia/Apple, our understanding is that at least the first two proposals do not change the functionality of PUSCH repetition type A. However, for the last proposal, it seems a mixed version of PUSCH repetition type A and B. |
| Qualcomm | We have concerns on allowing flexible symbols across type A repetitions (last item). If this is allowed, this is a separate type of repetitions and we should recognize that. We are open to fixing the PUSCH rep counting issue by increasing the maximum number of reps rather than other explicit means. |
| LG | We are thinking that PUSCH repetition type A can be a starting point of enhancement for PUSCH coverage enhancement. We are fine that ‘Enhancements on PUSCH repetition type A’ is recommended on the enhanced solutions for the follow-up WI. In WI phase, we will discuss how to enhance the PUSCH repetition type A in details. Hence, in this discussion, it should avoid to be spent a time for selecting detail schemes among the candidates for enhancements on PUSCH repetition type A. |

8.2 Enhancements on PUSCH repetition type B

|  |  | Supporting companies | Companies have concerns |
| --- | --- | --- | --- |
| Enhancements on PUSCH repetition type B | actual PUSCH transmission across the slot boundary/invalid symbols | ZTE, CTC, Samsung, WILUS, ETRI | Huawei, HiSilicon, Nokia, NSB, Intel, Qualcomm |
|  | actual PUSCH transmission across the slot boundary/invalid symbols, and the length of actual repetition larger than 14 symbols | ZTE, CTC, Samsung, WILUS, Interdigital, NTT DOCOMO | Huawei, HiSilicon, Nokia, NSB, Intel, Qualcomm |
|  | RV enhancement | Vivo |  |

| Companies | Comments |
| --- | --- |
| Huawei, HiSilicon | For PUSCH transmission across slot-boundary, we have concerns on its difficulty of UE implementation. If TB across slot boundary transmission is introduced with parts over multiple slots, the period of control and scheduling might be interrupted, which will increase the complexity significantly. To avoid blurring the slot boundary, we have the following suggested proposal - actual PUSCH transmission over integer slots which intra slot boundary/invalid symbols can be acrossed - actual PUSCH transmission across the slot boundary/invalid symbols, and the length of actual repetition larger than 14 symbols, e.g. 28 symbols, 42 symbols. |
| Nokia/NSB | Similar to PUSCH repetition type A, we would like to ask for clarification on the intention of specifying the enhancements on PUSCH repetition type B as well. Is this going to be an extension of PUSCH repetition type B functionality or rather a new PUSCH repetition type? If it is simply an extension of PUSCH repetition type B, we think that such enhancement should be avoided or further discussion is needed on how not to violate the legacy PUSCH repetition type B scheme, which was carefully and recently designed for URLLC and having different target than what we are doing here. |
| Apple | We are not sure in the work item phase this enhancement should be done under eURLLC or coverage enhancement. Basically, repetition type B is designed for latency reduction; from coverage enhancement point of simple repetition is enough. |
| Intel | We share similar view as HW that some of the proposals including “actual PUSCH transmission across the slot boundary/invalid symbols and the length of actual repetition larger than 14 symbols” would have large impact on both UE and gNB implementation., where processing is slot based. We need to be careful to make decision on support of the enhancement for PUSCH repetition type B. |
| Qualcomm | Similar concerns as Apple. Gains are also marginal/questionable and don’t justify pursuing this. |
| Samsung | Regarding the interrupted for control: We don’t think one TB span multiple slots will bring additional impact than existing Type B repetition, same validation rule of Type B PUSCH repetition can be applied. Our main intention is to further enhance PUSCH type B repetition, without changing the valid symbol determination but just avoid some unnecessary segmentation, which may result in high code rate of each actual repetition. And potentially further reduce the code rate of each repetition by introducing L>14. Regarding the eURLLC vs CE: the most challenge part is to meet the coverage requirement in a certain TDD configuration with a required data rate. Type B PUSCH repetition can provide good utilization of UL symbols in TDD. The enhancement is to reduce code rate of type B repetition to improve coverage. |
| LG | Similar view with Nokia/Apple. It is confusing whether the enhancement of PUSCH repetition type B is targeting URLLC users or not and further it is proper methodology to URLLC users or not. If the target and/or purpose of it is different from that of the PUSCH repetition type B, it is desirable to define a new repetition type and further to clarify the difference between the new type and type B. |
| ZTE | Regarding interruption of control and scheduling, we share with Samsung. In addition, our understanding is that, most of the scheduling are transmission based not slot based. Thus, it would not bring much complexity. In TDD scenario, PUSCH repetition type B could improve the coverage by making a full use of all UL symbols. However, current mechanism would cause many segmentation when the transmission is across slot boundary or DL symbols. This would lead to very high data for each actual repetition as Samsung also mentioned. |

8.3 TB processing over multi-slot PUSCH

|  |  | Supporting companies | Companies have concerns |
| --- | --- | --- | --- |
| TB processing over multi-slot PUSCH | TBS determined based on single slot and transmitted in parts over multiple slots | Interdigital | Huawei, HiSilicon, Nokia, NSB |
|  | TBS determined based on multiple slots and transmitted over multiple slots | IITH, IITM, CEWIT, Reliance Jio, Tejas Networks, Intel, CTC, LG, Qualcomm, Panasonic, Samsung, Interdigital, WILUS, Sharp, NEC, Sierra Wireless | Huawei, HiSilicon, Nokia, NSB |

| Companies | Comments |
| --- | --- |
| Huawei, HiSilicon | Similar concern as for 8.2 on this solution that TB transmission with parts across multiple slots would increase the complexity of UE implementation. We suggest the following proposal to avoid blurring the slot boundary while TB across multiple slots can be well supported: - TBS determined based on single slot and transmitted in parts over multiple integer slots - TBS determined based on multiple slots and transmitted over multiple integer slots |
| Nokia, NSB | Same view as Huawei/HiSi. |
| NEC | Agree the view of Huawei/HiSi, however integer slots needs to be clarified. For example, is 12 OFDM symbols in each slot over 4 slots called over multiple integer slots? |
| Sierra Wireless | Sierra Wireless supports TBS determined based on multiple slots and transmitted over multiple slots. |
| Intel | We support TBS determined based on multiple slots and transmitted over multiple slots. |
| LG | Same view with NEC. |
| ZTE | We are fine with TBS determined based on multiple slots and transmitted over multiple slots. Regarding the ‘integer slots’ we have similar question as NEC. |
| Qualcomm | @Nokia, @Huawei, this can be accomplished without any impact to slot boundary. TB scaling + repetitions + RV cycling suffices. |

8.4 Enhancements on inter-slot frequency hopping

|  |  | Supporting companies | Companies have concerns |
| --- | --- | --- | --- |
| Enhancements on inter-slot frequency hopping | More frequency offsets, More frequency hopping positions | HW, HiSi, Xiaomi, vivo, ZTE, NEC, Sony, NTT DOCOMO, CTC, Spreadtrum, Apple | Intel |

| Companies | Comments |
| --- | --- |
| Intel | It highly depends on the gNB architecture, e.g., number of Rx antennas. If the number of Rx antennas is large, we do not see the need to increase the number of hops for inter-slot frequency hopping as the diversity order is already sufficiently large. |
| ZTE | If PUSCH is bottleneck channel in scenarios at 700MHz or 2GHz, more diversity gain is needed since the number of Rx chains is very small in such scenarios. |

8.5 Sub-PRB transmission for VoIP

|  |  | Supporting companies | Companies have concerns |
| --- | --- | --- | --- |
| Sub-PRB transmission for VoIP | sub-PRB transmission with single slot | Samsung |  |
|  | sub-PRB transmission with multi-slot aggregation | CTC, Sony, LG, NTT DOCOMO, Sierra Wireless, Samsung, Qualcomm |  |

| Companies | Comments |
| --- | --- |
| Qualcomm | We are open to considering this enhancement. We would however like to pursue this enhancement using CDM/OCC across a single PRB transmission similar to what is done for PUCCH Format 4. This will likely minimize RAN4 involvement as no new waveforms need to be specified. To this end, it will be good to edit the text in the second row to “sub-PRB transmission (including the use of CDM/OCC across a single PRB) with multi-slot aggregation” |
| Samsung | Regarding the comments from QC, the detailed solution of how to do sub-PRB transmission can be discussed in the WI phase. The PUCCH format 4 liked method (as also commented earlier by Intel), could be proposed a potential candidate and discussed in RAN1. Regarding the impact to RAN4, it will be premature to say which one has minimized impact, and we think the first priority should be given to the consideration of performance gains of the solution. We are also supportive to single slot case. In our simulations, we simulated one slot with one PRB (for normal PUSCH) vs 0.5PRBwith 2slot (for subPRB). For the subPRB case, we use TB over single slot, but just do a repetition in these 2slots. We can still observe a gain of 1.6dB in link budget. |
| Qualcomm | @Samsung, yes this was triggered by Intel’s comment. To the extent that we can see Intel’s approach should require no RAN4 involvement. Just to make sure it is not precluded, we prefer to add the edited text. |

8.6 Enhancements on intra-slot frequency hopping

|  |  | Supporting companies | Companies have concerns |
| --- | --- | --- | --- |
| Enhancements on intra-slot frequency hopping | More frequency offsets, More frequency hopping positions | CTC, Spreadtrum, Nokia, NSB | Intel, Qualcomm |
|  | More time-domain hop positions within a slot | CTC, Spreadtrum | Intel, Qualcomm |
|  | DM-RS sharing among multiple PUSCH transmissions with the same frequency position between two consecutive slots | Nokia, NSB | Intel, Qualcomm |

| Companies | Comments |
| --- | --- |
| Nokia, NSB | We are not sure how we can have more time-domain hopping positions within a slot without adding more frequency hopping positions. We added Nokia, NSB to “More frequency offsets, More frequency hopping positions” and removed from “More time-domain hop positions within a slot” to avoid confusion. |
| Intel | We do not support enhancement on intra-slot frequency hopping. More number of hops/times in a slot would make the implementation complicated. |
| Qualcomm | Share similar concerns as Intel. Don’t support pursuing this since it fragments DMRS resources. |
| LG | Same view with Qualcomm. If enhancement of frequency hopping is supported for coverage enhancement, targeting inter-slot frequency hopping seems enough. |

8.7 Joint channel estimation or DM-RS bundling

|  |  | Supporting companies | Companies have concerns |
| --- | --- | --- | --- |
| Joint channel estimation or DM-RS bundling | cross-slot channel estimation over consecutive slots | HW, HiSi, Xiaomi, ZTE, ETRI, CTC, CMCC, NEC, Samsung, OPPO, Sharp, LG, Ericsson, Interdigital, NTT DOCOMO, Qualcomm, Intel, Panasonic, CATT, Apple, vivo, Sierra Wireless |  |
|  | cross-slot channel estimation over non-consecutive slots | DOCOMO, Intel, Sierra Wireless, LG | Huawei, HiSilicon, Nokia, NSB, CATT, ZTE |
|  | cross-repetition channel estimation within one slot |  |  |
|  | inter-slot frequency hopping with inter-slot bundling | ZTE, Intel, CTC, NEC, Samsung, LG, Panasonic, Interdigital, Apple, NEC, Sierra Wireless |  |
|  | optimization of DMRS location/granularity | Interdigital, Xiaomi, Sharp, CMCC, Qualcomm |  |

| Companies | Comments |
| --- | --- |
| InterDigital | To align with the texts from the past rounds, please change “optimization of DMRS location/granularity” to “cross-slot channel estimation with DM-RS in a special slot”. In R1-2009583, DMRS placement in a special slot is explicitly stated in Proposal 7. |
| Huawei, HiSilicon | For ‘cross-slot channel estimation over non-consecutive slots’, we wonder whether phase continuity can be ensured in non-consecutive uplink transmission. |
| Nokia/NSB | We share similar concern as mentioned by Huawei. |
| Sharp | We prefer to keep “optimization of DMRS location / granularity” as agreed. We support the optimization of DMRS location/granularity with joint channel estimation. Our Tdoc (R1-2008399) indicates support for it. |
| Sierra Wireless | Sierra Wireless also supports: cross-slot channel estimation over consecutive slots cross-slot channel estimation over non-consecutive slots inter-slot frequency hopping with inter-slot bundling WRT cross-slot channel estimation over non-consecutive slots – Sierra feels that in many cases and many UE architectures, phase continuity can be maintained across non-consecutive slots, but we can leave this up to RAN4 to decide. So perhaps we can agree to recommend this only “if determined feasible by RAN4, cross-slot channel estimation over non-consecutive slots”. |
| CMCC | As we clarified and proposed in the section 7. The optimization of DMRS under the premise of cross channel estimation is preferred. And we also support to take the special slot into consideration. |
| Intel | We support - cross-slot channel estimation over consecutive slots - cross-slot channel estimation over non-consecutive slots - inter-slot frequency hopping with inter-slot bundling as we see the substantial performance gain for the above. We are fine with Sierra Wireless’s suggestions for “cross-slot channel estimation over non-consecutive slots” |
| Qualcomm | We have added support for “optimization of DMRS location/granularity” with the assumption that this also includes potential dynamic or adaptive DMRS configuration when DMRS bundling is enabled. |
| LG | LG supports ‘cross-slot channel estimation over non-consecutive slots’ in addition to ‘over consecutive slots’. We think a slot bundle for cross-slot channel estimation is composed to consecutive slots, but only a part of slots in the slot bundle can be actually utilized for cross-slot channel estimation. For example, if DL slots are included in the slot bundle, cross-slot channel estimation can be performed except the DL slots. |
| CATT | We also have concerns on cross-slot channel estimation over non-consecutive slots. The feasibility depends on implementation of RF components. Phase continuity is not easy to maintain if DL symbols/slots are inserted between UL slots. |
| ZTE | Similar view as several companies above, we have concerns on phase continuity over non-consecutive slots. |

8.8 Enhancements on DM-RS density

|  |  | Supporting companies | Companies have concerns |
| --- | --- | --- | --- |
| Enhancements on DM-RS density | lower DM-RS density in time domain, DM-RS sharing among multiple PUSCH transmissions in the time domain | HW, HiSi, Spreadtrum, OPPO, Sharp, Nokia, NSB, xiaomi, Apple, NEC | Intel, LG, CATT |
|  | lower DMRS density in frequency domain | ZTE | Intel, LG, CATT |
|  | 1-comb DM-RS | CTC | Intel, LG |
|  | additional DM-RS symbol position in a slot | DOCOMO |  |

| Companies | Comments |
| --- | --- |
| Intel | For low density in time domain or DMRS sharing, we think it highly depends on the number of symbols allocated for each PUSCH repetition and SNR operating point. Considering coverage enhancement scenario, where the number of symbols allocated for PUSCH repetition is relatively large and system is operating at low SNR regime, we do not see the performance gain of DMRS sharing. In this case, we actually need more DMRS symbols to achieve better channel estimation performance. We do not support lower/higher DMRS density in general. |
| LG | We share the view with Intel. Considering that UEs requiring coverage enhancements are in low-SNR region in general, acquiring enough channel estimation performance is important for coverage enhancement. We are not sure that lower DMRS density really beneficial to improve coverage. |
| CATT | Similar concern with Intel and LG on lower DMRS density. Regarding to 1-comb DMRS, we think it is some kind of higher density DMRS (frequency domain) but not lower DMRS density. |
| ZTE | For lower DMRS density in the frequency domain, the intention is to boost the power of DMRS if the density in the frequency domain is reduced. In the time domain, gNB can still configure one or more DMRS symbols depending on different situations. This is exactly to improve the channel estimation performance for a UE operating in low SNR region. |

8.9 Adaptive DM-RS configuration

|  | Supporting companies | Companies have concerns |
| --- | --- | --- |
| Adaptive DM-RS configuration | Qualcomm, CMCC |  |

| Companies | Comments |
| --- | --- |
| CMCC | Adaptive DMRS configuration could provide a flexibility for the DMRS optimization. If the cross channels estimation and other enhancements are adopted, the DMRS pattern could be optimized according to the specific enhancements. And a flexible method for the DMRS configuration could improve the system efficiency. |
| Qualcomm | Same views as CMCC. We need to address this given the number of DMRS enhancements we are considering. A joint proposal on DMRS enhancements may be a possible option. |

8.10 DM-RS balancing among frequency hops

|  | Supporting companies | Companies have concerns |
| --- | --- | --- |
| DM-RS balancing among frequency hops | Nokia, NSB |  |

| Companies | Comments |
| --- | --- |

8.11 UE transmit waveform design to reduce MPR

|  |  | Supporting companies | Companies have concerns |
| --- | --- | --- | --- |
| UE transmit waveform design to reduce MPR | tone reservation | Qualcomm |  |
|  | FDSS (Frequency Domain Spectral Shaping) without spectral extension for pi/2 BPSK | Huawei, HiSilicon |  |
|  | FDSS with and without spectral extension for QPSK | Nokia, NSB |  |

| Companies | Comments |
| --- | --- |
| InterDigital | FDSS should be considered for any modulation schemes, not limited to QPSK. Thus, we recommend to remove “QPSK” from “FDSS with and without spectral extension for QPSK”. Please also add InterDigital as a supporting company to both tone reservation and “FDSS with and without spectral extension.” We have shown our support for MPR reduction methods in Proposal 11 in R1-2009583. |
| Nokia/NSB | From our understanding, FDSS without spectral extension for pi/2 BPSK is already supported. In which sense would this be considered an enhancement? Is there different understanding between companies about this approach? Concerning the comment from InterDigital, from our study, FDSS with spectral extension is more beneficial for QPSK hence we think a reference to QPSK should be kept. Maybe the concern from InterDigital could be addressed by the following wording? “FDSS with and without spectral extension at least for QPSK”. We hope that the new wording is acceptable to InterDigital. |
| InterDigital2 | We are fine with the proposal from Nokia. We support ““FDSS with and without spectral extension at least for QPSK”.” |

8.12 Power boosting for pi/2 BPSK

|  | Supporting companies | Companies have concerns |
| --- | --- | --- |
| Power boosting for pi/2 BPSK | IITH, IITM, CEWIT, Reliance Jio, Tejas Networks |  |

| Companies | Comments |
| --- | --- |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Support. |

8.13 Spatial domain based solutions

|  |  | Supporting companies | Companies have concerns |
| --- | --- | --- | --- |
| Spatial domain based solutions | multiple layer PUSCH transmission with DFT-S-OFDM | Ericsson |  |
|  | Open-loop Tx diversity | Mitsubishi |  |

| Companies | Comments |
| --- | --- |

8.14 SIP signal compression

|  | Supporting companies | Companies have concerns |
| --- | --- | --- |
| SIP signal compression, SigComp | Ericsson |  |

| Companies | Comments |
| --- | --- |

8.15 Dynamic PUSCH waveform adaptation

|  | Supporting companies | Companies have concerns |
| --- | --- | --- |
| Dynamic PUSCH waveform adaptation | Qualcomm |  |

| Companies | Comments |
| --- | --- |

- Proposals (6th round)

Proposal 18: Capture the following observation into the TR.

Observation:

- Seven sources ([China Telecom, R1-2008874], [ZTE, R1-2007743], [Intel, R1-2007954], [DOCOMO, ?], [Sierra Wireless, R1-2007930], [Apple, R1-2008479], [Huawei, R1-2007583]) evaluate the performance of enhancements on PUSCH repetition type A.

  - One source ([China Telecom, R1-2008874]) shows 3.2 dB (O2I) and 4 dB (O2O) SNR gain when the actual number of repetition is increased from 3 to 8 (counted on the basis of available UL slots) for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type A with 8 nominal repetitions. HARQ is not used.

  - One source ([ZTE, R1-2007743]) shows 1~1.5 dB SNR gain for PUSCH transmission with 4 repetitions and maximum 1 re-transmissions for VoIP at 2% rBLER for FR1 TDD, compared to PUSCH transmission with 2 repetitions and maximum 3 re-transmissions.

  - One source ([DOCOMO, ?]) shows 6.8 dB SNR gain, when the actual number of repetition is increased for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type A. HARQ is not used.

  - One source ([DOCOMO, R1-2008559]) shows 6.4 dB  SNR gain, when the actual number of repetition is increased for eMBB [for data rates less than] 100kbps at 10% iBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type A. HARQ is not used.

  - One source ([Intel, R1-2007954]) shows 2.0 dB SNR gain, when the actual number of repetition is increased to 16 for eMBB with TBS of 136 bits at 10% iBLER for FR1 FDD, compared to Rel-16 PUSCH repetition type A with 8 repetitions. HARQ is not adopted.

  - One source ([Apple, R1-2008479]) shows 2.2 dB SNR gain when the maximum number of repetitions is increased to 16 for eMBB at 10% iBLER for FR1 FDD, compared to Rel-16 PUSCH repetition type A with 8 repetitions. HARQ is not used.

  - One source ([Apple, R1-2008479]) shows 0.8 dB SNR gain with the repetition and the frequency hopping is enabled. It also shows 2.2dB SNR gain when the maximum number of repetitions is increased to 16 for eMBB with TBS 120bits at 10% iBLER for FR1 FDD, compared to Rel-16 PUSCH repetition type A with 8 repetitions. HARQ is not used.

  - One source ([Huawei, R1-2007583]) shows about 2.0 dB SNR gain when the actual number of repetition is doubled, e.g. from 2 to 4, from 4 to 8, for eMBB at 10% iBLER for FR1 TDD. HARQ is not used.

  - One source ([Huawei, R1-2007583]) shows about 8.1dB SNR gain for PUSCH transmission with 3 retransmissions combined with 4 actual repetitions for VoIP at 2% rBLER for FR1 TDD, compared to PUSCH transmission with no repetition and no retransmission.

  - One source ([Sierra Wireless, R1-2007930]) shows 1.6 dB  SNR loss when the maximum number of repetitions is increased to 16 for eMBB 100kbps at 10% iBLER for FR1 FDD, compared to Rel-16 PUSCH repetition type A with 8 repetitions. HARQ is not used.

| Companies | Comments |
| --- | --- |
| Ericsson | It should be “loss” due to more repetitions, instead of “gain” according to ZTE’s assumptions and results provided in the summary: Baseline scheme: 4 repetitions with maximum 1 re-transmission (Max 8 transmissions) Enhanced scheme: 2 repetitions with maximum 3 re-transmissions (Max 8 transmissions). So we propose the updates for the corresponding bullet: - One source ([ZTE, R1-2007743]) shows 1~1.5 dB SNR gain loss for PUSCH transmission with 4 repetitions and maximum 1 re-transmissions for VoIP at 2% rBLER for FR1 TDD, compared to PUSCH transmission with 2 repetitions and maximum 3 re-transmissions. |
| ZTE | Very sorry that we mixed up the baseline scheme and enhanced scheme in ‘[103-e-NR-CovEnh-EvaluationResults] – enhancements’. 4 repetitions with maximum 1 re-transmission (Max 8 transmissions) should be the enhanced scheme. Please refer our contribution in R1-2007743, which correctly describes our simulation assumptions and results. |
| Qualcomm | @ZTE thanks for the clarification. But with no effective increase in transmission, we wonder where the gains are coming from. Are you not assuming LLR combining across different HARQ instances? |
| NTT DOCOMO | We are sorry for the missing information. Our document in R1-2008557 for FR1 evaluation, we use PUSCH repetition as defined, so that the gain in included in the results. And for the eMBB, the data rate is 1 Mbps. |
| Ericsson | @ZTE, ok, in your simulation description, you may mixed baseline and enhancement. But our earlier comments may still be valid if you used TDD configuration ‘DDDDDDDSUU’ mentioned in the enhancement simulation summary, i.e. “gain” should be changed to be “loss”. For your actual baseline simulation, i.e. 2 repetitions + 3 retransmissions + 1 initial for TDD configuration ‘DDDDDDDSUU’, you can have 8 actual repetitions. But in your actual enhancement simulation, when you use 4 repetitions and 2 transmissions(including initial transmission), 2 of the repetitions in each one of the 2 transmissions(including initial transmission) will be cancelled, thus you will have only 4 actual repetitions, and I guess that is why you have loss from this 4-reppetition configuration. And this set up proves that, with a proper retransmission and repetition combination supported in current spec., we can already be able to avoid having repetitions overlapping with DL/S slots in a TDD configuration. So, 4-repetition configuration has some loss instead of gain in this TDD ‘DDDDDDDSUU’ setup, compared to the 2-repetition configuration. |
| ZTE | I'd like to make a quick reply on Gokul's comments on Proposal 18. It seems the performance gain is from different RV settings. For the baseline scheme, we only used RV {0,2} for each (re-)transmission with 2 repetitions, while {0,2,3,1} for each (re-)transmission with 4 repetitions for enhanced scheme. We understand it is just one of the implementation. So, it's fine to add such simulation setting in the observation to make it clear. |

Revised Proposal 18: Capture the following observation into the TR.

Observation:

- Seven sources ([China Telecom, R1-2008874], [ZTE, R1-2007743], [Intel, R1-2007954], [DOCOMO, R1-2008557], [Sierra Wireless, R1-2007930], [Apple, R1-2008479], [Huawei, R1-2007583]) evaluate the performance of enhancements on PUSCH repetition type A.

  - One source ([China Telecom, R1-2008874]) shows 3.2 dB (O2I) and 4 dB (O2O) SNR gain when the actual number of repetition is increased from 3 to 8 (counted on the basis of available UL slots) for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type A with 8 nominal repetitions. HARQ is not used.

  - One source ([ZTE, R1-2007743]) shows 1.0~1.5 dB SNR gain for PUSCH transmission with 4 repetitions and maximum 1 re-transmissions (maximum 8 actual transmissions in total, redundancy version {0, 2, 3, 1}) for VoIP at 2% rBLER for FR1 TDD, compared to PUSCH transmission with 2 repetitions and maximum 3 re-transmissions (maximum 8 actual transmissions in total, redundancy version {0, 2}).

  - One source ([DOCOMO, R1-2008557]) shows 6.8 dB SNR gain, when the actual number of repetition is increased for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type A. HARQ is not used.

  - One source ([DOCOMO, R1-2008559]) shows 6.4 dB SNR gain, when the actual number of repetition is increased for eMBB 1Mbps at 10% iBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type A. HARQ is not used.

  - One source ([Intel, R1-2007954]) shows 2.0 dB SNR gain, when the actual number of repetition is increased to 16 for eMBB with TBS of 136 bits at 10% iBLER for FR1 FDD, compared to Rel-16 PUSCH repetition type A with 8 repetitions. HARQ is not adopted.

  - One source ([Apple, R1-2008479]) shows 2.2 dB SNR gain when the maximum number of repetitions is increased to 16 for eMBB at 10% iBLER for FR1 FDD, compared to Rel-16 PUSCH repetition type A with 8 repetitions. HARQ is not used.

  - One source ([Apple, R1-2008479]) shows 0.8 dB SNR gain with the repetition and the frequency hopping is enabled. It also shows 2.2dB SNR gain when the maximum number of repetitions is increased to 16 for eMBB with TBS 120bits at 10% iBLER for FR1 FDD, compared to Rel-16 PUSCH repetition type A with 8 repetitions. HARQ is not used.

  - One source ([Huawei, R1-2007583]) shows about 2.0 dB SNR gain when the actual number of repetition is doubled, e.g. from 2 to 4, from 4 to 8, for eMBB at 10% iBLER for FR1 TDD. HARQ is not used.

  - One source ([Huawei, R1-2007583]) shows about 8.1dB SNR gain for PUSCH transmission with 3 retransmissions combined with 4 actual repetitions for VoIP at 2% rBLER for FR1 TDD, compared to PUSCH transmission with no repetition and no retransmission.

  - One source ([Sierra Wireless, R1-2007930]) shows 1.6 dB SNR loss when the maximum number of repetitions is increased to 16 for eMBB 100kbps at 10% iBLER for FR1 FDD, compared to Rel-16 PUSCH repetition type A with 8 repetitions. HARQ is not used.

| Companies | Comments |
| --- | --- |
| FL | @DOCOMO, Please confirm whether the 4th bullet is correct. |
| NTT DOCOMO | Thank you very much for the update. For the 4th bullet, please refer R1-2008557 as well. |
| Intel | For the following one, suggest modify this as follows to reflect our simulation results and observations. - One source ([Intel, R1-2007954]) shows 2.0 dB SNR gain, when the actual number of repetition is doubled increased to 16 for eMBB with TBS of 136 bits at 10% iBLER for FR1 FDD, compared to Rel-16 PUSCH repetition type A with 8 repetitions. HARQ is not used adopted. |
| Ericsson | Regarding the bullet below, it would be better to clarify which TDD configuration and how many PUSCH repetitions (including those cancelled) are configured, e.g. DDDDDDDSUU. One source ([ZTE, R1-2007743]) shows 1.0~1.5 dB SNR gain for PUSCH transmission with 4 repetitions and maximum 1 re-transmissions (maximum 8 actual transmissions in total, redundancy version {0, 2, 3, 1}) for VoIP at 2% rBLER for FR1 TDD, compared to PUSCH transmission with 2 repetitions and maximum 3 re-transmissions (maximum 8 actual transmissions in total, redundancy version {0, 2}). |
| Sierra Wireless | We updated the results in “R1-20xxxxx - [103-e-NR-CovEnh-EvaluationResults] – enhancements - v022_ZTE_Sierra.doc” to again include our results for 4 and 8 repeats. Given this we would like to update our observation to show that we did find SNR gains for 4 and 8 repeats of 0.4 and 0.2 dB but had a SNR loss for 16 repeats of 1.6dB. So we would like to change our observation to be: - One source (Sierra Wireless) shows the when the TBS is adjusted to maintain the target data rate of 100kbps, [+0.4,+0.2,-1.6 dB] SNR gain was observed when the number of repetitions was increased to [4,8,16] respectively for eMBB at 10% iBLER for FR1 FDD using Rel-16 PUSCH repetition type A. We see wildly different result here b/c some papers (e.g. Apple R1-2008479, Intel R1-2007954, Huawei R1-2007583 ) are changing the data rate and thus seeing large gains which we feel needs to be noted. Some papers are also observing these gains below the required 100kbps for the eMBB use case which is against the agreement made so one could even make the argument that those observations should not be included. Thus we request the following text be inserted for the appropriate papers: “Note: the observed gain was for different data rates where the data rate was less than the required 100kbps for the eMBB use case”. |

Proposal 19: Capture the following observation into the TR.

Observation:

- Five sources ([China Telecom, R1-2008874], [ZTE, R1-2007743], [vivo, R1-2007680], [InterDigital, R1-2009168], [Samsung, R1-2009647]) evaluate the performance of enhancements on PUSCH repetition type B.

  - Three sources ([China Telecom, R1-2008874], [InterDigital, R1-2009168], [Samsung, R1-2009647]) show 0.2~2.0 dB SNR gain when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B. HARQ is not used.

  - One source ([ZTE, R1-2007743]) show 0.8 dB required SNR gain when the actual PUSCH transmission can across the slot boundary or the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B. HARQ is not used.

  - One source ([Samsung, R1-2009647]) shows around 1.4 dB SNR gain when the actual PUSCH transmission can cross the slot boundary and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR2 TDD, compared to Rel-16 PUSCH repetition type B.

  - One source ([InterDigital, R1-2009168]) shows 0.33~1.3 dB SNR gain when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B. HARQ is not used.

  - One source ([InterDigital, R1-2009168]) shows the number of RBs can be reduced from 38 to 33, when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B. HARQ is not used.

  - One source ([InterDigital, R1-2009168]) shows the number of RBs can be reduced from 30 to 26, when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR2 TDD, compared to Rel-16 PUSCH repetition type B. HARQ is not used.

  - One source ([vivo, R1-2007680]) shows around 2.0 dB SNR gain for RV enhancement for eMBB at 10% iBLER for FR 1 TDD, compared to Rel-16 PUSCH repetition type B.

| Companies | Comments |
| --- | --- |
| Ericsson | One general comment is to add “without joint channel estimation” after “compared to Rel-16 PUSCH repetition type B” for all bullets since the gain may not be able to be seen when joint channel estimation is supported as we discussed in proposal 23. For the simulation from ZTE, for the case 4 shown in the figured copied from R1-2007743 contributed by ZTE, when number of symbols is 16, isn’t the 2 DMRS will be jointly used for channel estimation? And for case 3, even if only 1 fronted DMRS used for the first repetition with 8 symbols crossing the slot boundary, the channel is assumed to be the same on the symbols in both slots. Maybe ZTE can clarify. If this is the correct understanding, we propose to capture the cross-slot channel estimation as well. Besides, lower MCS is used for longer duration PUSCH for the enhancement simulation, which should also be captured. - One source ([ZTE, R1-2007743]) show 0.8 dB required SNR gain when the actual PUSCH transmission can across the slot boundary or the length of actual repetition can be larger than 14 symbols and cross-slot channel estimation is used for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B without joint channel estimation. HARQ is not used. Same TB size is used for both baseline and enhancement. For the simulation from Samsung, cross-slot channel estimation should have been used when the number of symbols is more than 14 if we understand correctly, and this is where the gain comes from compared to R16 baseline without cross-slot channel estimation over repetitions. And this will overlap with cross-slot channel estimation discussions, we propose to capture this with the update shown below: - One source ([Samsung, R1-2009647]) shows around 1.4 dB SNR gain when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR2 TDD, compared to Rel-16 PUSCH repetition type B without joint channel estimation. As a summary, we proposal the update: Proposal 19: Capture the following observation into the TR. Observation: - Five sources ([China Telecom, R1-2008874], [ZTE, R1-2007743], [vivo, R1-2007680], [InterDigital, R1-2009168], [Samsung, R1-2009647]) evaluate the performance of enhancements on PUSCH repetition type B. - Three sources ([China Telecom, R1-2008874], [InterDigital, R1-2009168], [Samsung, R1-2009647]) show 0.2~2.0 dB SNR gain when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B without joint channel estimation. HARQ is not used. - One source ([ZTE, R1-2007743]) show 0.8 dB required SNR gain when the actual PUSCH transmission can across the slot boundary or the length of actual repetition can be larger than 14 symbols and cross-slot channel estimation is used for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B without joint channel estimation. HARQ is not used. Same TB size is used for both baseline and enhancement. - One source ([Samsung, R1-2009647]) shows around 1.4 dB SNR gain when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR2 TDD, compared to Rel-16 PUSCH repetition type B without joint channel estimation. - One source ([InterDigital, R1-2009168]) shows 0.33~1.3 dB SNR gain when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B without joint channel estimation. HARQ is not used. - One source ([InterDigital, R1-2009168]) shows the number of RBs can be reduced from 38 to 33, when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B without joint channel estimation. HARQ is not used. - One source ([InterDigital, R1-2009168]) shows the number of RBs can be reduced from 30 to 26, when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR2 TDD, compared to Rel-16 PUSCH repetition type B without joint channel estimation. HARQ is not used. - One source ([vivo, R1-2007680]) shows around 2.0 dB SNR gain for RV enhancement for eMBB at 10% iBLER for FR 1 TDD, compared to Rel-16 PUSCH repetition type B without joint channel estimation. |
| ZTE | @Ericsson, thanks for the clarification questions. I thought we didn’t use cross-slot channel estimation since there is one DMRS in each slot in both case 3 and case 4. After a further check, we indeed used cross-slot channel estimation in our simulation. So, we are fine with Eircsson’s modification. Sorry for confusion. |
| InterDigital | Some of the evaluation results presented here include use of DMRS in the special slot or DMRS optimization. Thus, they should be used to support Proposal 31 (cross-slot channel estimation). In addition, the comparison conducted in our evaluation compares 16-symbol actual repetition and 14-symbol actual repetition. While cross-slot channel estimation is performed for the 16-symbol actual repetition (due to placement of DMRS in special slot), the 14-symbol actual repetition does not perform cross-slot channel estimation (since all PUSCH are contained in one slot), the following clarification is sufficient. - One source ([InterDigital, R1-2009168]) shows 0.33~1.3 dB SNR gain when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B with 14-symbol actual repetition. HARQ is not used. - One source ([InterDigital, R1-2009168]) shows the number of RBs can be reduced from 38 to 33, when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B with 14-symbol actual repetition. HARQ is not used. - One source ([InterDigital, R1-2009168]) shows the number of RBs can be reduced from 30 to 26, when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR2 TDD, compared to Rel-16 PUSCH repetition type B with 14-symbol actual repetition. HARQ is not used. |

Revised Proposal 19: Capture the following observation into the TR.

Observation:

- Five sources ([China Telecom, R1-2008874], [ZTE, R1-2007743], [vivo, R1-2007680], [InterDigital, R1-2009168], [Samsung, R1-2009647]) evaluate the performance of enhancements on PUSCH repetition type B.

  - Three sources ([China Telecom, R1-2008874], [InterDigital, R1-2009168], [Samsung, R1-2009647]) show 0.2~2.0 dB SNR gain when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B without joint channel estimation. HARQ is not used.

  - One source ([ZTE, R1-2007743]) show 0.8 dB required SNR gain when the actual PUSCH transmission can across the slot boundary or the length of actual repetition can be larger than 14 symbols and cross-slot channel estimation is used for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B without joint channel estimation. HARQ is not used. Same TB size is used for both baseline and enhancement.

  - One source ([Samsung, R1-2009647]) shows around 1.4 dB SNR gain when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR2 TDD, compared to Rel-16 PUSCH repetition type B without joint channel estimation.

  - One source ([InterDigital, R1-2009168]) shows 0.33~1.3 dB SNR gain when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B with 14-symbol actual repetition without joint channel estimation. HARQ is not used.

  - One source ([InterDigital, R1-2009168]) shows the number of RBs can be reduced from 38 to 33, when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B with 14-symbol actual repetition without joint channel estimation. HARQ is not used.

  - One source ([InterDigital, R1-2009168]) shows the number of RBs can be reduced from 30 to 26, when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR2 TDD, compared to Rel-16 PUSCH repetition type B with 14-symbol actual repetition without joint channel estimation. HARQ is not used.

  - One source ([vivo, R1-2007680]) shows around 2.0 dB SNR gain for RV enhancement for eMBB at 10% iBLER for FR 1 TDD, compared to Rel-16 PUSCH repetition type B without joint channel estimation.

| Companies | Comments |
| --- | --- |

Proposal 20: Capture the following observation into the TR.

Observation:

- Seven sources ([China Telecom, R1-2008874], [IITH, R1-2007905, [Intel, R1-2007954, [Qualcomm, R1-2008626], [InterDigital, R1-2009168], [Nokia, ?], [Sierra Wireless, R1-2007930]) evaluate the performance of TB processing over multi-slot PUSCH.

  - Two sources ([China Telecom, R1-2008874], [Qualcomm, R1-2008626]) show 0.6~2 dB  SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1, compared to TB is determined based on single slot with repetition in Rel-16. HARQ is not used. Different redundancy version is used for repetitions in [China Telecom, R1-2008874]. The same redundancy version is used for repetitions in [Qualcomm, R1-2008626]. Gains due to reduction in upper-layer (MAC/RLC/PDCP) headers is not reflected in [Qualcomm, R1-2008626].

  - Four sources ([China Telecom, R1-2008874], [IITH, R1-2007905], [Qualcomm, R1-2008626], [Nokia, ?]) show 0.8~2.7 dB  SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1, compared to TB is determined based on single slot with repetition in Rel-16. HARQ is not used. Different redundancy version is used for repetitions in [China Telecom, R1-2008874]. The same redundancy version is used for repetitions in [IITH, R1-2007905], [Qualcomm, R1-2008626], [Nokia, R1-2008703]. Gains due to reduction in upper-layer (MAC/RLC/PDCP) headers is not reflected in [Qualcomm, R1-2008626].

  - One source ([InterDigital, R1-2009168]) shows 0.4 and 2.0 dB SNR gain and up to 10dB power boosting gain with different number of aggregated slots and modulation schemes when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 FDD, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used.

  - One source ([InterDigital, R1-2009168]) shows 0~1.75 dB SNR gain and up to 6.98dB power boosting gain depending on the number of aggregated slots and modulation when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used.

  - One source ([Intel, R1-2007954]) shows 0.2 dB SNR gain and 6.2 dB link budget gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 TDD, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used.

  - One source ([Sierra Wireless, R1-2007930]) shows 2 (w/ frequency hopping) and 2.5 dB (w/o frequency hopping)  SNR gain for codewords determined based on multiple slots and transmitted over multiple slots with gaps for eMBB at 10% iBLER for FR1, compared to Rel-16 where multiple TBs with repeats are scheduled over contiguous slots. HARQ is not used.

| Companies | Comments |
| --- | --- |
| Ericsson | Similar to proposal 19 and since the resource allocated for each PUSCH transmission in this enhancement is crossing multiple slots, we should capture “cross-slot channel estimation is used” and the actual gain we see is mainly from cross-slot channel estimation which can actually also be applied to PUSCH repetitions as we discussed in proposal 23. So we propose: Proposal 20: Capture the following observation into the TR. Observation: - Seven sources ([China Telecom, R1-2008874], [IITH, R1-2007905, [Intel, R1-2007954, [Qualcomm, R1-2008626], [InterDigital, R1-2009168], [Nokia, ?], [Sierra Wireless, R1-2007930]) evaluate the performance of TB processing over multi-slot PUSCH. - Two sources (China Telecom, R1-2008874], [Qualcomm, R1-2008626]) show 0.6~2 dB SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot with repetition in Rel-16 without cross-slot channel estimation. HARQ is not used. Different redundancy version is used for repetitions in [China Telecom, R1-2008874]. The same redundancy version is used for repetitions in [Qualcomm, R1-2008626]. Gains due to reduction in upper-layer (MAC/RLC/PDCP) headers is not reflected in [Qualcomm, R1-2008626]. - Four sources ([China Telecom, R1-2008874], [IITH, R1-2007905], [Qualcomm, R1-2008626], [Nokia, ?]) show 0.8~2.7 dB SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot with repetition in Rel-16 without cross-slot channel estimation. HARQ is not used. Different redundancy version is used for repetitions in [China Telecom, R1-2008874]. The same redundancy version is used for repetitions in [IITH, R1-2007905], [Qualcomm, R1-2008626], [Nokia, R1-2008703]. Gains due to reduction in upper-layer (MAC/RLC/PDCP) headers is not reflected in [Qualcomm, R1-2008626]. - One source ([InterDigital, R1-2009168]) shows 0.4 and 2.0 dB SNR gain and up to 10dB power boosting gain with different number of aggregated slots and modulation schemes when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 FDD and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. - One source ([InterDigital, R1-2009168]) shows 0~1.75 dB SNR gain and up to 6.98dB power boosting gain depending on the number of aggregated slots and modulation when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. - One source ([Intel, R1-2007954]) shows 0.2 dB SNR gain and 6.2 dB link budget gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 TDD and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. - One source ([Sierra Wireless, R1-2007930]) shows 2 (w/ frequency hopping) and 2.5 dB (w/o frequency hopping) SNR gain for codewords determined based on multiple slots and transmitted over multiple slots with gaps for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to Rel-16 where multiple TBs with repeats are scheduled over contiguous slots without cross-slot channel estimation. HARQ is not used. |
| Nokia/NSB | We are fine with the proposal. The above edits by Ericsson also look OK to us. One question for clarification from the FL: Is it a correct understanding that in the TR only Tdoc numbers are captured but not company names, or both will be captured in the TR? |
| Qualcomm | @Ericsson, we do not assume cross-slot channel estimation. Contiguous slots are not a prerequisite. Gains presented are purely due to coding gains because of larger TB sizes by aggregating across multiple slots. If we are going to add “HARQ is not used”, then we need to qualify this statement. Please restate as follows: “HARQ is not used; if HARQ is enabled, legacy transmissions incur increased freq-domain resource use and a fair comparison cannot be made.” |
| InterDigital | For our evaluation results, we implemented cross-slot channel estimation. We would like to rewrite the observations for our results as follows. - One source ([InterDigital, R1-2009168]) shows 0.4 and 2.0 dB SNR gain and up to 10dB power boosting gain with different number of aggregated slots and modulation schemes when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 FDD and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. Up to 10dB power boosting gain can be obtained depending on different number of aggregated slots when TBS determined based on multiple slots and transmitted over multiple slots for FR1 FDD VoIP. - One source ([InterDigital, R1-2009168]) shows 0~1.75 dB SNR gain and up to 6.98dB power boosting gain depending on the number of aggregated slots and modulation when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. Up to 6.98dB power boosting gain can be obtained depending on the number of aggregated slots when TBS determined based on multiple slots and transmitted over multiple slots for FR1 eMBB. We agree with Qualcomm that some of benefits of “TB over multiple slots” can be obtained regardless of whether cross-slot channel estimation is used or not. We also agree that cross-slot channel estimation is not pre-requisite for this enhancement. One of the benefits of “TB over multiple slots” is the use of smaller number of RBs used in the frequency domain; power boosting gain can be obtained. This gain is irrespective of whether cross-slot channel estimation is used or not, which is what we would like to stress with the above modification. We would also like to note that if the enhancement is to be compared against HARQ, the comparison should be done in latency critical applications such as VoIP. We also agree that HARQ consumes more resources compared to this enhancement. |
| FL | @Ericsson, China Telecom does not use cross-slot channel estimation. @Nokia, Based on the previous experience, only Tdoc numbers are captured but not company names. However, it seems some results reported in “[103-e-NR-CovEnh-EvaluationResults]” are not included in companies’ contributions. One way is to refer to the Tdoc number of “[103-e-NR-CovEnh-EvaluationResults]”. Or both will be captured in the TR. I would like to hear more views from other companies. Proposal 20 is revised as follows: Proposal 20: Capture the following observation into the TR. Observation: - Seven sources ([China Telecom, R1-2008874], [IITH, R1-2007905, [Intel, R1-2007954, [Qualcomm, R1-2008626], [InterDigital, R1-2009168], [Nokia, ?], [Sierra Wireless, R1-2007930]) evaluate the performance of TB processing over multi-slot PUSCH. - Two sources (China Telecom, R1-2008874], [Qualcomm, R1-2008626]) show 0.6~2 dB SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot with repetition in Rel-16 without cross-slot channel estimation. HARQ is not used. [If HARQ is enabled, legacy transmissions incur increased frequency domain resource use and a fair comparison cannot be made.] Different redundancy version is used for repetitions in [China Telecom, R1-2008874]. The same redundancy version is used for repetitions in [Qualcomm, R1-2008626]. Gains due to reduction in upper-layer (MAC/RLC/PDCP) headers is not reflected in [Qualcomm, R1-2008626]. - Two sources ([China Telecom, R1-2008874], [Qualcomm, R1-2008626]) show 1.0~2.7 dB SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot with repetition in Rel-16 without cross-slot channel estimation. HARQ is not used. [If HARQ is enabled, legacy transmissions incur increased frequency domain resource use and a fair comparison cannot be made.] Different redundancy version is used for repetitions in [China Telecom, R1-2008874]. The same redundancy version is used for repetitions in [Qualcomm, R1-2008626]. Gains due to reduction in upper-layer (MAC/RLC/PDCP) headers is not reflected in [Qualcomm, R1-2008626]. - Two sources ([IITH, R1-2007905], [Nokia, ?]) show 0.8~1.0 dB SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot with repetition in Rel-16 without cross-slot channel estimation. HARQ is not used. The same redundancy version is used for repetitions. - One source ([InterDigital, R1-2009168]) shows 0.4 and 2.0 dB SNR gain and up to 10dB power boosting gain with different number of aggregated slots and modulation schemes when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 FDD and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. Up to 10dB power boosting gain can be obtained depending on different number of aggregated slots when TBS determined based on multiple slots and transmitted over multiple slots for FR1 FDD VoIP. - One source ([InterDigital, R1-2009168]) shows 0~1.75 dB SNR gain and up to 6.98dB power boosting gain depending on the number of aggregated slots and modulation when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. Up to 6.98 dB power boosting gain can be obtained depending on the number of aggregated slots when TBS determined based on multiple slots and transmitted over multiple slots for FR1 eMBB. - One source ([Intel, R1-2007954]) shows 0.2 dB SNR gain and 6.2 dB link budget gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 TDD and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. - One source ([Sierra Wireless, R1-2007930]) shows 2 (w/ frequency hopping) and 2.5 dB (w/o frequency hopping) SNR gain for codewords determined based on multiple slots and transmitted over multiple slots with gaps for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to Rel-16 where multiple TBs with repeats are scheduled over contiguous slots without cross-slot channel estimation. HARQ is not used. |
| Ericsson | @Qualcomm and China Telecom, didn’t you use all the DMRS symbols for the single PUSCH occupying multiple slots when doing the joint channel estimation? Or you still assume different channels for different parts of one PUSCH transmission? What we mean is that in the enhancement, all DMRSs in the single PUSCH transmission across multiple slots are used for joint channel estimation, while the baseline with repetitions is not using cross-slot channel estimation, and this is the key point that we’re trying to capture. If cross-slot channel estimation is also used across all repetitions in the baseline, we do not see clear gain from multiple slot PUSCH. So our proposed updates based on Jianchi’s latest update according to above understanding: Proposal 20: Capture the following observation into the TR. Observation: - Seven sources ([China Telecom, R1-2008874], [IITH, R1-2007905, [Intel, R1-2007954, [Qualcomm, R1-2008626], [InterDigital, R1-2009168], [Nokia, ?], [Sierra Wireless, R1-2007930]) evaluate the performance of TB processing over multi-slot PUSCH. - Two sources (China Telecom, R1-2008874], [Qualcomm, R1-2008626]) show 0.6~2 dB SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot with repetition in Rel-16 without cross-slot channel estimation. HARQ is not used. [If HARQ is enabled, legacy transmissions incur increased frequency domain resource use and a fair comparison cannot be made.] Different redundancy version is used for repetitions in [China Telecom, R1-2008874]. The same redundancy version is used for repetitions in [Qualcomm, R1-2008626]. Gains due to reduction in upper-layer (MAC/RLC/PDCP) headers is not reflected in [Qualcomm, R1-2008626]. - Two sources ([China Telecom, R1-2008874], [Qualcomm, R1-2008626]) show 1.0~2.7 dB SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot with repetition in Rel-16 without cross-slot channel estimation. HARQ is not used. [If HARQ is enabled, legacy transmissions incur increased frequency domain resource use and a fair comparison cannot be made.] Different redundancy version is used for repetitions in [China Telecom, R1-2008874]. The same redundancy version is used for repetitions in [Qualcomm, R1-2008626]. Gains due to reduction in upper-layer (MAC/RLC/PDCP) headers is not reflected in [Qualcomm, R1-2008626]. - Two sources ([IITH, R1-2007905], [Nokia, ?]) show 0.8~1.0 dB SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot with repetition in Rel-16 without cross-slot channel estimation. HARQ is not used. The same redundancy version is used for repetitions. - One source ([InterDigital, R1-2009168]) shows 0.4 and 2.0 dB SNR gain and up to 10dB power boosting gain with different number of aggregated slots and modulation schemes when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 FDD and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. Up to 10dB power boosting gain can be obtained depending on different number of aggregated slots when TBS determined based on multiple slots and transmitted over multiple slots for FR1 FDD VoIP. - One source ([InterDigital, R1-2009168]) shows 0~1.75 dB SNR gain and up to 6.98dB power boosting gain depending on the number of aggregated slots and modulation when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. Up to 6.98 dB power boosting gain can be obtained depending on the number of aggregated slots when TBS determined based on multiple slots and transmitted over multiple slots for FR1 eMBB. - One source ([Intel, R1-2007954]) shows 0.2 dB SNR gain and 6.2 dB link budget gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 TDD and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. - One source ([Sierra Wireless, R1-2007930]) shows 2 (w/ frequency hopping) and 2.5 dB (w/o frequency hopping) SNR gain for codewords determined based on multiple slots and transmitted over multiple slots with gaps for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to Rel-16 where multiple TBs with repeats are scheduled over contiguous slots without cross-slot channel estimation. HARQ is not used. |

Revised Proposal 20: Capture the following observation into the TR.

Observation:

- Seven sources ([China Telecom, R1-2008874], [IITH, R1-2007905, [Intel, R1-2007954, [Qualcomm, R1-2008626], [InterDigital, R1-2009168], [Nokia, ?], [Sierra Wireless, R1-2007930]) evaluate the performance of TB processing over multi-slot PUSCH.

  - Two sources (China Telecom, R1-2008874], [Qualcomm, R1-2008626]) show 0.6~2 dB  SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot with repetition in Rel-16 without cross-slot channel estimation. HARQ is not used. [If HARQ is enabled, legacy transmissions incur increased frequency domain resource use and a fair comparison cannot be made.] Different redundancy version is used for repetitions in [China Telecom, R1-2008874]. The same redundancy version is used for repetitions in [Qualcomm, R1-2008626]. Gains due to reduction in upper-layer (MAC/RLC/PDCP) headers is not reflected in [Qualcomm, R1-2008626].

  - Two sources ([China Telecom, R1-2008874], [Qualcomm, R1-2008626]) show 1.0~2.7 dB SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot with repetition in Rel-16 without cross-slot channel estimation. HARQ is not used. [If HARQ is enabled, legacy transmissions incur increased frequency domain resource use and a fair comparison cannot be made.] Different redundancy version is used for repetitions in [China Telecom, R1-2008874]. The same redundancy version is used for repetitions in [Qualcomm, R1-2008626]. Gains due to reduction in upper-layer (MAC/RLC/PDCP) headers is not reflected in [Qualcomm, R1-2008626].

  - Two sources ([IITH, R1-2007905], [Nokia, ?]) show 0.8~1.0 dB SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot with repetition in Rel-16 without cross-slot channel estimation. HARQ is not used. The same redundancy version is used for repetitions.

  - One source ([InterDigital, R1-2009168]) shows 0.4 and 2.0 dB SNR gain and up to 10dB power boosting gain with different number of aggregated slots and modulation schemes when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 FDD and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. Up to 10dB power boosting gain can be obtained depending on different number of aggregated slots when TBS determined based on multiple slots and transmitted over multiple slots for FR1 FDD VoIP.

  - One source ([InterDigital, R1-2009168]) shows 0~1.75 dB SNR gain and up to 6.98dB power boosting gain depending on the number of aggregated slots and modulation when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. Up to 6.98 dB power boosting gain can be obtained depending on the number of aggregated slots when TBS determined based on multiple slots and transmitted over multiple slots for FR1 eMBB.

  - One source ([Intel, R1-2007954]) shows 0.2 dB SNR gain and 6.2 dB link budget gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 TDD and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used.

  - One source ([Sierra Wireless, R1-2007930]) shows 2 (w/ frequency hopping) and 2.5 dB (w/o frequency hopping) SNR gain for codewords determined based on multiple slots and transmitted over multiple slots with gaps for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to Rel-16 where multiple TBs with repeats are scheduled over contiguous slots without cross-slot channel estimation. HARQ is not used.

| Companies | Comments |
| --- | --- |
| China Telecom | @Ericsson, We confirm that cross-slot channel estimation is not used in our simulation. |
| Intel | We did not use cross-slot channel estimation. So we suggest to modify this as - One source ([Intel, R1-2007954]) shows 0.2 dB SNR gain and 6.2 dB link budget gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 TDD and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. |
| Ericsson | @China telecom and Intel, thanks for the feedback. Just for further clarification for better understanding: Do you mean the “single PUSCH transmitted over multiple slots” has different channels assumed for different part of this single PUSCH transmission in your simulation? I.e. channels on symbols in different slots for this single PUSCH transmission are estimated independently? Could you please also clarify how the DMRS are positioned? Is it based on number of symbols in each slot or total number of symbols in all slots? Thanks. Another question is for the baseline repetitions, did you use different RV or same RV? Thanks. |
| Sierra Wireless | We used perfect channel estimation. We also prefer to say that TBS is determined based on multiple slots, as to maintain consistency with other observations. We suggest the following modification: - One source ([Sierra Wireless, R1-2007930]) shows 2 (w/ frequency hopping) and 2.5 dB (w/o frequency hopping) SNR gain when TBS determined based on multiple slots and transmitted over multiple slots with gaps for eMBB at 10% iBLER for, compared to Rel-16 where repeats are scheduled over contiguous slots. HARQ is not used. |
| Qualcomm | @Ericsson, each PUSCH transmission in a slot is just like your legacy transmission. We are only manipulating the TB size here. The transmissions need not even be contiguous. |
| China Telecom | @Ericsson, Firstly, we want to clarify again that we didn’t use cross channel estimation over multiple slot, which exactly means different channels are assumed for different slots and estimated independently. In the simulation, we considered 14 PUSCH symbols, PUSCH mapping type B, and 2 DMRS symbols were configured on symbol 0 and 10 for each slot, refers to Table 6.4.1.1.3-3 in TS 38.211. For baseline repetitions, we used different RVs, i.e. RV = [0,2,3,1]. |

Proposal 21: Capture the following observation into the TR.

Observation:

- Six sources ([ZTE, R1-2007743], [Intel, R1-2007954], [Qualcomm, ?], [vivo, R1-2007680], [Ericsson, R1-2008419], [Huawei, ?]) evaluate the performance of inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions.

  - Five sources ([ZTE, R1-2007743], [Intel, R1-2007954], [vivo, R1-2007680], [Ericsson, R1-2008419], [Huawei, ?]) show 0.3~1.7 dB  SNR gain for inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions for eMBB at 10% iBLER for FR1, compared to Rel-16 inter frequency hopping. Cross-slot channel estimation is not used, 2 RX is assumed for gains higher than 0.3dB, [300ns] is assumed for gains higher than 0.3dB.

  - One source ([Qualcomm, ?]) shows no gain for inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions for eMBB at 10% iBLER for FR1, compared to Rel-16 inter frequency hopping.

  - One source ([Ericsson, R1-2008419]) shows no gain for inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions and joint channel estimation over multiple slots is implemented for eMBB at 10% iBLER for FR1, compared to Rel-16 inter frequency hopping with joint channel estimation over multiple slots.

| Companies | Comments |
| --- | --- |
| Qualcomm | We will include this result in a revised tdoc. It is currently only captured as a figure in the evaluation document. |
| FL | Since no comments, suggest to remove brackets of “300ns”. |
| Qualcomm | R1-2009729 is the revised tdoc with the figure that was originally included in the enhancement results document. |

Proposal 22: Capture the following observation into the TR.

Observation:

- Four sources ([China Telecom, R1-2008874], [Samsung, R1-2009647], [vivo, R1-2007680], [Sierra Wireless, R1-2007930]) evaluate the performance of sub-PRB transmission with multi-slot aggregation.

  - One source ([China Telecom, R1-2008874]) shows around 0.8 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with repetition. HARQ is not used.

  - One source ([Samsung, R1-2009647]) shows around 5.6 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with 4 PRBs without repetition. HARQ is not used.

  - One source ([Samsung, R1-2009647]) shows around 1.6 and 8.5 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for eMBB at 10% iBELR for FR1, respectively, depending on the number of aggregation slots, compared to Rel-16 PRB-based transmission with 1 PRB and 4 PRBs, respectively without repetition. HARQ is not used.

  - One source ([Sierra Wireless, R1-2007930]) evaluates the performance of sub-PRB transmission with 2 tones to reduce MPR and shows 5 dB PAPR reduction for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with CP-OFDM.

  - One source ([vivo, R1-2007680]) shows no gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with repetition. HARQ is not used.

| Companies | Comments |
| --- | --- |
| Ericsson | Similar to our comments to other proposals, cross slot channel estimation is used when the resource occupies multiple slots, this needs to be also captured in this proposal. And similar to proposal 23, capture the baseline is without cross-slot channel estimation. We propose the updates: Proposal 22: Capture the following observation into the TR. Observation: - Four sources ([China Telecom, R1-2008874], [Samsung, R1-2009647], [vivo, R1-2007680], [Sierra Wireless, R1-2007930]) evaluate the performance of sub-PRB transmission with multi-slot aggregation. - One source ([China Telecom, R1-2008874]) shows around 0.8 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1 when cross-slot channel estimation is used, compared to Rel-16 PRB-based transmission with repetition without cross-slot channel estimation. HARQ is not used. - One source ([Samsung, R1-2009647]) shows around 5.6 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with 4 PRBs without repetition. HARQ is not used. - One source ([Samsung, R1-2009647]) shows around 1.6 and 8.5 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for eMBB at 10% iBELR for FR1, respectively, depending on the number of aggregation slots, compared to Rel-16 PRB-based transmission with 1 PRB and 4 PRBs, respectively without repetition. HARQ is not used. - One source ([Sierra Wireless, R1-2007930]) evaluates the performance of sub-PRB transmission with 2 tones to reduce MPR and shows 5 dB PAPR reduction for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with CP-OFDM. - One source ([vivo, R1-2007680]) shows no gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with repetition. HARQ is not used. |
| FL | @Ericsson, China Telecom does not use cross-slot channel estimation. |
| Ericsson | @China Telecom, Maybe we mis-understand the “multi-slot aggregation” in the enhancement, do you mean repetitions instead of one PUSCH transmission crossing multiple slots? If it’s one PUSCH transmission across multiple slots, we would assume you use all the DMRS symbols for the single PUSCH occupying multiple slots to do a joint channel estimation. Or you still assume different channels for different parts of one PUSCH transmission in multiple slots? What we mean is that in the enhancement, all DMRSs in the single PUSCH transmission across multiple slots are used for joint channel estimation, while the baseline with repetitions may be not using cross-slot channel estimation, and this is the key point that we’re trying to capture. |

Revised Proposal 22: Capture the following observation into the TR.

Observation:

- Four sources ([China Telecom, R1-2008874], [Samsung, R1-2009647], [vivo, R1-2007680], [Sierra Wireless, R1-2007930]) evaluate the performance of sub-PRB transmission with multi-slot aggregation.

  - One source ([China Telecom, R1-2008874]) shows around 0.8 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1 when cross-slot channel estimation is used, compared to Rel-16 PRB-based transmission with repetition without cross-slot channel estimation. HARQ is not used.

  - One source ([Samsung, R1-2009647]) shows around 5.6 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with 4 PRBs without repetition. HARQ is not used.

  - One source ([Samsung, R1-2009647]) shows around 1.6 and 8.5 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for eMBB at 10% iBELR for FR1, respectively, depending on the number of aggregation slots, compared to Rel-16 PRB-based transmission with 1 PRB and 4 PRBs, respectively without repetition. HARQ is not used.

  - One source ([Sierra Wireless, R1-2007930]) evaluates the performance of sub-PRB transmission with 2 tones to reduce MPR and shows 5 dB PAPR reduction for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with CP-OFDM.

  - One source ([vivo, R1-2007680]) shows no gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with repetition. HARQ is not used.

| Companies | Comments |
| --- | --- |
| China Telecom | @Ericsson, We confirm that cross-slot channel estimation is not used for the enhancements in our simulation. If joint channel estimation is used, the gain should be larger. |
| Apple | For the observation on sub-PRB transmission on eMBB, 1.6dB and 8.5 dB gains are observed. It’s not clear the details of the simulation assumption, it seems the results are too optimistic. Sub-PRB transmission is aiming for VoIP evaluation, not sure it’s appropriate to put the eMBB observation here. |
| Ericsson | @China telecom, thanks for the feedback. Then this means you assume different channels for different parts of one PUSCH transmission in multiple slot. Could you clarify how you put the positions of the DMRS for this PUSCH across multiple slots? Based on the number of occupied symbols in each slot or based on the total number of symbols across multiple slots? Another question is in the baseline repetitions, did you use different RV or same RV? Thanks. |
| Sierra Wireless | The Ericsson’s edits are not exactly correct but we appreciate direction. We propose the following changes to the observation which maintain the direction Ericsson was proposing: - One source ([Sierra Wireless, R1-2007930]) evaluated the performance of sub-PRB transmission with 2 tones which showed 7 dB PAPR reduction. This PAPR reduction allows for MPR relaxation for VoIP use case at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with DFT-s-OFDM. |
| Samsung | Regarding the comments from apple, the discussion for sub-PRB was concluded for VoIP, which implies for low data rate services. Yes, for 8.5dB case, the assumption is kind of ideal, which is just used to show the benefits in this sub-PRB direction. The observation is just to capture what companies has been simulated. In the final recommendation, we are ok to focus on VoIP service for the subPRB design. |
| China Telecom | @Ericsson, In the simulation, we considered 14 PUSCH symbols, PUSCH mapping type B, and 2 DMRS symbols were configured on symbol 0 and 10 for each slot, refers to Table 6.4.1.1.3-3 in TS 38.211. For baseline repetitions, we used different RVs, i.e. RV = [0,2,3,1]. |

Proposal 23: Capture the following observation into the TR.

Observation:

- Fifteen sources ([China Telecom, R1-2008874], [ZTE, R1-2007743], [Intel, R1-2007954], [Qualcomm, R1-2008626], [Sharp, R1-2008399], [Panasonic, R1-2008378], [DOCOMO, R1-2008559], [Samsung, R1-2009647], [CMCC, R1-2008026], [vivo, R1-2007680], [Ericsson, R1-2008419], [Nokia, R1-2008703], [Apple, R1-2008479], [InterDigital, R1-2009168], [Huawei, R1-2007583]) evaluate the performance of joint channel estimation.

  - Eleven sources ([China Telecom, R1-2008874], [ZTE, R1-2007743], [Qualcomm, R1-2008626], [Sharp, R1-2008399], [Panasonic, R1-2008378], [CMCC, R1-2008026], [vivo, R1-2007680], [Ericsson, R1-2008419], [Nokia, R1-2008703], [Apple, R1-2008479], [Huawei, R1-2007583]) show 0.2~2.1 dB  SNR gain for joint channel estimation over multiple slots for eMBB at 10% iBLER depending on the number of slots for FR1, compared to Rel-16 PUSCH transmission without joint channel estimation.

  - One source ([Intel, R1-2007954]) shows 2 dB  SNR gain for joint channel estimation over multiple non-consecutive slots with inter-slot frequency hopping for eMBB at 10% iBLER, compared to Rel-16 inter-slot frequency hopping without joint channel estimation.

  - Three sources ([Samsung, R1-2009647], [NTT DOCOMO, R1-2008559]) show 0.9~1.3 dB  SNR gain for joint channel estimation over multiple slots for VoIP at 2% rBLER depending on the number of slots for FR1, compared to Rel-16 PUSCH transmission without joint channel estimation.

  - One source ([InterDigital, R1-2009168]) shows 0.3 dB SNR gain for joint channel estimation over multiple slots with DMRS in a special slot for VoIP at 2% rBLER, compared to Rel-16 PUSCH transmission without joint channel estimation.

  - One source shows ([Samsung, R1-2009647]) 0.85~1.1 dB SNR gain for joint channel estimation over multiple slots for VoIP at 2% rBLER depending on the number of slots for FR2, compared to Rel-16 PUSCH transmission without joint channel estimation.

  - One source ([vivo, R1-2007680]) shows 0.8 dB SNR gain for joint channel estimation over multiple repetition within a slot, compared to Rel-16 PUSCH transmission without joint channel estimation.

  - Two sources ([CMCC, R1-2008026], [vivo, R1-2007680]) show 1.0~1.22 dB required SNR gain for lower DM-RS density in time domain with joint channel estimation and using multi-slot PUSCH with 4 symbols in the special slot over multiple slots for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density without joint channel estimation.

  - One source ([vivo, R1-2007680]) shows 1.0 dB required SNR gain for lower DM-RS density in time domain with joint channel estimation over multiple repetition within a slot for eMBB at 10% iBLER for FR1, compared to Rel-16 PUSCH transmission without joint channel estimation.

- Five sources ([China Telecom, R1-2008874], [ZTE, R1-2007743], [Intel, R1-2007954], [Samsung, R1-2009647], [Ericsson, R1-2008419]) evaluate the performance of inter-slot frequency hopping with inter-slot bundling and joint channel estimation.

  - Two sources ([China Telecom, R1-2008874], [Samsung, R1-2009647]) show 0.5~2.5 dB  SNR gain for inter-slot frequency hopping with inter-slot bundling for VoIP at 2% rBLER depending on bundle size, DM-RS configurations for FR1, compared to Rel-16 inter-slot frequency hopping.

  - One source ([Samsung, R1-2009647]) shows 1.0~1.55 dB  SNR gain for inter-slot frequency hopping with inter-slot bundling for VoIP at 2% rBLER depending on bundle size, DM-RS configurations for FR2, compared to Rel-16 inter-slot frequency hopping.

  - Three sources ([ZTE, R1-2007743], [Intel, R1-2007954], [Ericsson, R1-2008419]) show 0.5~3 dB  SNR gain for inter-slot frequency hopping with inter-slot bundling for eMBB at 10% iBLER depending on bundle size for FR1, compared to Rel-16 inter-slot frequency hopping.

  - One source ([Intel, R1-2007954]) shows 1 dB  SNR gain for inter-slot frequency hopping with inter-slot bundling and joint channel estimation over multiple slot for eMBB at 10% iBLER for FR1, compared to Rel-16 inter-slot frequency hopping with joint channel estimation over multiple non-consecutive slots.

| Companies | Comments |
| --- | --- |
| FL | Proposal 23 seems stable. |

Proposal 24: Capture the following observation into the TR.

Observation:

- Two sources ([ZTE, R1-2007743], [Intel, R1-2007954]) evaluate the performance of lower DM-RS density.

  - One source ([ZTE, R1-2007743]) shows around 1.0 dB SNR gain for lower DM-RS density in frequency domain for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density. The results is based on the assumption with only 1 RX antenna, single front-loaded DMRS symbol and without frequency hopping.

  - One source ([Intel, R1-2007954]) shows around 0.2 dB SNR loss for lower DM-RS density in time domain for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density.

| Companies | Comments |
| --- | --- |
| FL | Proposal 24 seems stable. |

Proposal 25: Capture the following observation into the TR.

Observation:

- Three sources ([China Telecom, R1-2008874], [Intel, R1-2007954], [DOCOMO, R1-2008559]) evaluate the performance of higher DM-RS density.

  - One source ([China Telecom, R1-2008874]) shows 0.5~1.5 dB SNR gain for 1-comb DM-RS for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density without power boosting.

  - One source ([DOCOMO, R1-2008559]) shows around 1.0 dB SNR gain for additional DM-RS symbol position for VoIP at 2% rBLER for FR1, compared to Rel-16 DM-RS density with only single DMRS symbol.

  - One source ([Intel, R1-2007954]) shows around 0.05 dB SNR loss for higher DM-RS density in time domain for eMBB 10% iBLER for FR1, compared to Rel-16 DM-RS density.

| Companies | Comments |
| --- | --- |
| FL | Proposal 25 seems stable. |

Proposal 26: Capture the following observation into the TR.

Observation:

- One source ([Qualcomm, R1-2008626]) evaluates the performance of adaptive DM-RS configuration and shows 1.7 dB  SNR gain for eMBB at 10% iBLER for FR1, compared to Rel-16 semi-static DM-RS configuration. For low SNR such as -10 to -12 dB, it shows that adaptive DM-RS configuration can bring 10-100% increase in throughput compared to an ill-suited DMRS configuration depending on factors such as UE speed, DMRS bundling, and PUSCH repetition.

- One source ([China Telecom, R1-2008874]) evaluates the performance of enhanced intra-slot frequency hopping with more frequency offsets/ more frequency hopping positions and shows around 1.8 dB SNR gain for VoIP at 2% rBLER and 0.4 dB SNR gain for eMBB at 2% rBLER for FR1, compared to Rel-16 intra-slot frequency hopping.

- One source (IITH, R1-2007905)) evaluates the performance of power boosting for pi/2 BPSK and shows around 3 dB gain for UL duty cycle less than 50% and around 6 dB gain for UL duty cycle less than 25%.

- One source ([Qualcomm, R1-2008626]) evaluates the performance of dynamic switching between DFT-S-OFDM and CP-OFDM and shows 2~3 dB gain, compared to semi-static switching between DFT-S-OFDM and CP-OFDM when using QPSK modulation.

- One source ([Qualcomm, R1-2008626]) evaluates the performance of UE transmit waveform design to reduce MPR and shows 1 ~1.5 dB gain, compared to Rel-16 DFT-S-OFDM and CP-OFDM.

- One source ([Panasonic, R1-2008378]) evaluates the performance of symbol level repetition and shows around 0.4 dB SNR gain for UE speed 3km/h and around 0.3dB SNR loss for UE speed 120km/h, respectively, for eMBB at 10% iBLER for FR1, compared to Rel-16 PUSCH repetition type A.

- One source ([Mitsubishi, R1-2008743]) evaluates the performance of Alamouti-based transmit diversity and shows 2-2.7dB SNR gain for FR1, and 2-3dB SNR gain with QPSK and up to 8.5dB SNR gain with 16QAM for FR2.

- One source ([Ericsson, R1-2008419]) evaluates the performance of multiple layer PUSCH transmission with DFT-S-OFDM and shows around 3 dB cubic metric gain, compared to multiple layer PUSCH transmission with CP-OFDM.

| Companies | Comments |
| --- | --- |
| FL | Proposal 26 seems stable. |

Proposal 27: Capture the following observation into the TR.

- Enhancements on PUSCH repetition type A is beneficial for PUSCH coverage enhancements for TDD. It is recommended to support enhancements on PUSCH repetition type A in Rel-17, including:

  - Increasing the maximum number of repetitions, e.g., 32.

  - The number of repetitions counted on the basis of available UL slots.

| Companies | Comments |
| --- | --- |
| CATT | Support FL’s proposal. |
| Ericsson | Some general comment: we think that we’re not prepared to make recommendations to say which schemes are beneficial at this stage since the observations are not agreed yet and we do not know how to compare solutions. Specific comments on this proposal: As discussed earlier, we think that this is essentially a PDCCH overhead enhancement, since HARQ can repeat any number of times. The use case is then something like VoIP where there is a large number of UEs and PDCCH overhead is significant. Increasing the maximum number of repetitions seems to be the simplest approach, although it should not be excessive as Sierra points out. Increasing the actual number of repetitions seems a more complicated solution, although it may be feasible. We do not see the need to support both bullets and is a bit preferring the 1st bullet considering the specification impacts. So, we propose the following update: - Enhancements on PUSCH repetition type A is beneficial for PUSCH coverage enhancements for TDD. It is recommended to support enhancements on PUSCH repetition type A in Rel-17, including: - Increasing the maximum number of repetitions, e.g., 32. - The number of repetitions counted on the basis of available UL slots. |
| Samsung | - Performance gain of “increasing the maximum number of repetitions” is questionable based on the available results in Proposal 18, e.g., 2.2 dB gain vs. 1.6 dB loss. - Support “The number of repetitions counted on the basis of available UL slots” |
| Qualcomm | Same view as Ericsson. Support Ericsson’s revision. |
| InterDigital | We support the FL’s proposal. |
| Intel | We support the FL’s proposal. |
| FL | 11 (HW, HiSi, CTC, Intel, CATT, Spreadtrum, OPPO, Sharp, Apple, Qualcomm, Ericsson) companies support the 1st bullet. - 17 (HW, HiSi, CTC, vivo, Intel, Samsung, ZTE, CATT, CMCC, Panasonic, OPPO, LG, Interdigital, NTT DOCOMO, ETRI, Apple, Sierra Wireless) support 2nd 1st bullet. Based on majority’s view, FL suggests to keep both bullets. |
| Apple | We support the FL’s Proposal. |
| LG | We are fine with FL’s proposal |
| Samsung | We can support the FL support, but it will be good to justify the inconsistency on the simulation results when summarizing the results. |
| Sierra Wireless | We agree with several others that we don’t need to do both. We feel the biggest issue is with TDD not supporting enough “real” repetition for the VoIP case (eMBB is probably fine) so we feel the 2nd option is the best. However, if the first option has few spec impacts, we could agree to that as well assuming a maximum of 32 is considered. Since the group is rather divergent here, perhaps all we can agree to in the SI is that there are two options where the final decision is made in plenary where TU (time) management is considered. |
| Sharp | Agree with Sierra. Two schemes can be captured as two options. |

Proposal 28: Capture the following observation into the TR.

- TB processing over multi-slot PUSCH is beneficial for PUSCH coverage enhancements. It is recommended to support TB processing over multi-slot PUSCH in Rel-17, including:

  - TBS determined based on multiple slots and transmitted over multiple integer slots.

| Companies | Comments |
| --- | --- |
| Ericsson | Some general comment as we said previously: we think that we’re not prepared to make recommendations to say which schemes are beneficial at this stage since the observations are not agreed yet and we do not know how to compare solutions. Specific comments on this proposal: For multi-slot TBS, the benefit seems to be from cross-slot channel estimation, since the overhead can be the same with repetition. So before supporting multi-slot TBS, we think that the gains of multi-slot TBS without cross-slot channel estimation should be further studied. So this proposal is not needed to us and we can focus on the cross-slot/joint channel estimation proposal. |
| Qualcomm | Ericsson is mistaken in their understanding. Cross-slot channel estimation is not used. The improvement is due to coding gains and due to reduced overhead. |
| InterDigital | We support the FL’s proposal. Some of the gains from this enhancement can be obtained without using cross-slot channel estimation. |
| Ericsson | @Qualcomm, didn’t you use all the DMRS symbols for the PUSCH in the multiple slots when doing the channel estimation? What we mean is that in the enhancement, all DMRSs in the single PUSCH transmission across multiple slots are used for joint channel estimation, while the baseline with repetitions is not using cross-slot channel estimation, and this is the key point that we’re trying to capture. |
| Intel | We are fine with FL’s proposal. |
| FL | From FL perspective, 16 (IITH, IITM, CEWIT, Reliance Jio, Tejas Networks, Intel, CTC, LG, Qualcomm, Panasonic, Samsung, Interdigital, WILUS, Sharp, NEC, Sierra Wireless) companies support it and there is considerable gain. |
| LG | We are fine with FL’s proposal. |
| Sierra Wireless | We are fine with FL’s proposal. |
| Sharp | We support the FL proposal. We agree with Interdigital’ observation. |
| Qualcomm | @Ericsson: No we did not. This was just TB size scaling/optimization. We don’t even need contiguous slot transmissions. |

Proposal 29: Capture the following observation into the TR.

- Enhancements on inter-slot frequency hopping is beneficial for PUSCH coverage enhancements. It is recommended to support enhancements on inter-slot frequency hopping for PUSCH in Rel-17, including:

  - More frequency offsets, e.g. 4 for BWP less than 50 PRBs, 8 for BWP greater than 50 PRBs.

  - More frequency hopping positions, e.g. 4.

| Companies | Comments |
| --- | --- |
| CATT | Support FL’s proposal. |
| Ericsson | Some general comment as we said previously: we think that we’re not prepared to make recommendations to say which schemes are beneficial at this stage since the observations are not agreed yet and we do not know how to compare solutions. Specific comments on this proposal: If cross slot channel estimation is used, we don’t think there are gains from increasing the number of frequency hops. The gains are also dependent on the amount of delay spread. So, while we do not oppose a simple extension to inter-slot hopping that increases the number of hops, it may not be particularly beneficial enhancement to specify. |
| Intel | As we mentioned in the earlier comments, it highly depends on the gNB architecture, e.g., number of Rx antennas. If the number of Rx antennas is large, we do not see the need to increase the number of hops for inter-slot frequency hopping as the diversity order is already sufficiently large. We do not see the need for this proposal. |
| Apple | We support FL’s Proposal. |
| Samsung | We don’t think there are meaningful gains when increasing the number of hops for inter-slot frequency hopping. As mentioned by other companies, after considering diversity there is not much to gain with enhancing FH. |

Proposal 30: Capture the following observation into the TR.

- Sub-PRB transmission with multi-slot aggregation is beneficial for VoIP PUSCH coverage enhancements. It is recommended to support sub-PRB transmission with multi-slot aggregation in Rel-17.

| Companies | Comments |
| --- | --- |
| vivo | According to the observation provided in proposal 22, no or marginal performance gain for sub-PRB transmission over multiple slots is observed if compared with full PRB transmission with repetition. Some sources show performance gain due to sub-PRB over multiple slots are compared with full PRB without repetition which is not fair comparison, and the benefit is not justified. Besides, for OCC/CDM based sub PRB transmission, the main motivation is for multiplexing multiple UEs, which is not typical use case for poor coverage UEs. For PAPR reduction gain mentioned in previous proposal, since MPR for the legacy PF3 is already zero for inner RB allocation based on RAN4 spec, we do not see tx power can be further unlocked PAPR reduction, if any. |
| Ericsson | Some general comment as we said previously: we think that we’re not prepared to make recommendations to say which schemes are beneficial at this stage since the observations are not agreed yet and we do not know how to compare solutions. Specific comments on this proposal: As discussed in detail in the observation section, in our understanding, sub-PRB transmission should give no gains or limited gains stemming from channel estimation. The specification impacts are high, and other coverage enhancements are possible providing gains on those expected from sub-PRB. Thus, we do not think this proposal is needed. |
| Qualcomm | Sub-PRB transmission with multi-slot aggregation is a viable approach to support VoIP in uplink. This allows UE to boost EPRE without compromising on coding gains and also prevents loss due to over segmentation. Latency requirements for VoIP seems a little more relaxed, so this adds further credence to this approach. OCC/CDM approach sidesteps RAN4 involvement and is therefore good to pursue. |
| Samsung | We support FL’s proposal. Regarding the comments from Vivo, if full PRB case also uses repetition, then it will consume multiple times of the resources compared to Sub-PRB case. In Vivo’s results, it seems there is no obvious performance gain on link budget (while several other companies show gains), but the consumed T-F resource is reduced by half. Thus, we think the fair comparison is trying to align the resource consumption as much as possible. |
| Apple | We are not ready to support this proposal. at least the eMBB should not be included in the proposal. |
| Samsung | We are fine with considering that the design target is VoIP services, and suggest the following change: - Sub-PRB transmission with multi-slot aggregation is beneficial for VoIP PUSCH coverage enhancements. It is recommended to support sub-PRB transmission with multi-slot aggregation targeting VoIP services in Rel-17. |
| Sierra Wireless | We support FL’s proposal. Samsung’s suggest is also fine. |
| Qualcomm | Support Samsung revision. I recall IITH was also interested in half rate VoIP, this might be useful in that context as well. |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | Support FL as well as Samsung revision. |

Proposal 31: Capture the following observation into the TR.

- Joint channel estimation or DM-RS bundling is beneficial for PUSCH coverage enhancements. It is recommended to support Joint channel estimation or DM-RS bundling for PUSCH in Rel-17, including:

  - Cross-slot channel estimation over consecutive slots

  - Inter-slot frequency hopping with inter-slot bundling

  - Optimization of DMRS location/granularity, e.g., DM-RS placement in special slot, DM-RS sharing among multiple PUSCH transmissions in the time domain.

| Companies | Comments |
| --- | --- |
| CATT | Support FL’s proposal. |
| Ericsson | Some general comment as we said previously: we think that we’re not prepared to make recommendations to say which schemes are beneficial at this stage since the observations are not agreed yet and we do not know how to compare solutions. Specific comments on proposal: Cross slot or joint channel estimation should be the baseline. Optimizations such as DMRS granularity should be evaluated according to the gain they can provide in addition. And we do not see the addition gains so far. So we propose the following updates: Proposal 31: Capture the following observation into the TR. - Joint channel estimation or DM-RS bundling is beneficial for PUSCH coverage enhancements. It is recommended to support Joint channel estimation or DM-RS bundling for PUSCH in Rel-17, including: - Cross-slot channel estimation over consecutive slots - Inter-slot frequency hopping with inter-slot bundling - Optimization of DMRS location/granularity, e.g., DM-RS placement in special slot, DM-RS sharing among multiple PUSCH transmissions in the time domain. |
| Samsung | Support FL proposal. |
| InterDigital | We support the FL’s proposal. For some schemes, cross-slot channel estimation is enabled with placement of DMRS in special slot. Therefore, in some cases, optimization of DMRS is not an additional feature, but it is intrinsic for cross-slot channel estimation. @Ericsson, there are at least 3 sources captured in Proposal 23 that show performance gain obtained from “Optimization of DMRS location/granularity” using some of the examples in the FL’s proposal. From our perspective, some observations in Proposal 19 can also support the FL’s proposal. |
| Ericsson | @Interdigital, As we commented earlier, cross slot or joint channel estimation should be the baseline. Optimizations such as DMRS granularity should be evaluated according to the gain they can provide in addition. And we do not see the addition gains so far based on your results which overlaps with the joint channel estimation proposal. |
| Intel | We support the first two sub-bullet in the FL’s proposal. We share a similar view as Ericsson that last sub-bullet is not needed. As we explained in the earlier, for DMRS sharing, we think it highly depends on the number of symbols allocated for each PUSCH repetition and SNR operating point. Considering coverage enhancement scenario, where the number of symbols allocated for PUSCH repetition is relatively large and system is operating at low SNR regime, we do not see the performance gain of DMRS sharing. |
| Huawei, Hisilicon | We feel the 3rd bullet should be kept. Because DMRS sharing among multiple PUSCH transmissions not only include the lower DMRS density configuration averagely, but also include the scheme that joint channel estimation of multiple PUSCH transmission within a single slot, which cannot be covered by the 1st and 2nd bullet. Furthermore, we have analyzed in R1-2007583 that in poor SNR operation point, the scheduling would not be that accurate in reality and finer granularity retransmission of PUSCH by more retransmission times can contribute to a better performance. However, given current spec, DMRS overhead configured for each PUSCH transmission will cause a low spectral efficiency. Thus, DMRS sharing among multiple PUSCH transmissions is needed. Thus, we have the following suggestions: Option1: keep the “DM-RS sharing among multiple PUSCH transmissions in the time domain.” in the 3rd bullets Option2: revise Proposal 31 as follows: Proposal 31: Capture the following observation into the TR. - Joint channel estimation or DM-RS bundling is beneficial for PUSCH coverage enhancements. It is recommended to support Joint channel estimation or DM-RS bundling for PUSCH in Rel-17, including: - Cross-slot Joint channel estimation over consecutive PUSCH transmissions slots - Inter-slot frequency hopping with inter-slot bundling - Optimization of DMRS location/granularity, e.g., DM-RS placement in special slot, DM-RS sharing among multiple PUSCH transmissions in the time domain. |
| FL | @Huawei, It’s better to keep alignment with the following agreements: Agreements: Capture the followings into the TR - Joint channel estimation or DM-RS bundling with/without optimization of DMRS location/granularity was studied from several aspects, including cross-slot channel estimation over consecutive slots, cross-slot channel estimation over non-consecutive slots, cross-repetition channel estimation within one slot, and inter-slot frequency hopping with inter-slot bundling to enable cross-slot channel estimation. - Potential specification impacts of joint channel estimation or DM-RS bundling include: - Power consistency and phase continuity, DM-RS placement in special slot and DM-RS configuration. - Time domain hopping interval for inter-slot frequency hopping with inter-slot bundling |
| Apple | We support the FL’s proposal. |
| LG | We are generally fine with FL’s proposal. But, we don’t think reducing DMRS density by DMRS sharing is beneficial when we consider environments requiring coverage enhancement. Hence, we suggest to remove ‘or DM-RS bundling’ in main bullet, and ‘/granularity’ and ‘DM-RS sharing among multiple PUSCH transmissions in the time domain’ in 3rd sub-bullet. - Joint channel estimation or DM-RS bundling is beneficial for PUSCH coverage enhancements. It is recommended to support Joint channel estimation or DM-RS bundling for PUSCH in Rel-17, including: - Cross-slot channel estimation over consecutive slots - Inter-slot frequency hopping with inter-slot bundling - Optimization of DMRS location/granularity, e.g., DM-RS placement in special slot, DM-RS sharing among multiple PUSCH transmissions in the time domain. |
| Samsung | We support the FL proposal because optimization of DMRS location/granularity influences cross-slot channel estimation, and depending on scenarios/requirements it can be optimized to enhance performance or to maintain certain performance with fewer resources. |
| Sierra Wireless | We support the first two sub-bullet in the FL’s proposal. We share a similar view as Ericsson, and Intel that last sub-bullet is not needed and we should keep cross-slot channel estimation as a base line which would allow for some other optimization to support it as Interdigital suggested. In addition, at this stage, we really do not want to limit this to consecutive slots only as TDD configs DDDSU would not benefit at all and other TDD configs would benefit very little so we would like to leave detail for WI stage where we can develop educated views from RAN4 WRT phase continuity. So we suggest the following changes: - Joint channel estimation or DM-RS bundling is beneficial for PUSCH coverage enhancements. It is recommended to support Joint channel estimation or DM-RS bundling for PUSCH in Rel-17, including: - Cross-slot Joint channel estimation over consecutive PUSCH transmissions slots - Inter-slot frequency hopping with inter-slot bundling - Optimization of DMRS location/granularity, e.g., DM-RS placement in special slot, DM-RS sharing among multiple PUSCH transmissions in the time domain. |
| Sharp | We support the FL proposal since a kind of optimization of DMRS would lower the coding gain effectively at least in a case of DDDSU (S: 10D2S2U) configuration. |
| Qualcomm | Support first two bullets. For the third bullet, can we instead state “optimization of DMRS configuration”? This will be a necessary component depending on whether or not cross-slot channel estimation is enabled. |

Proposal 32: Capture the following observation into the TR.

- Power boosting for pi/2 BPSK is beneficial for PUSCH coverage enhancements. It is recommended to support Power boosting for pi/2 BPSK for PUSCH in Rel-17, including:

  - Beyond 26 dBm as a function of the UL duty cycle.

| Companies | Comments |
| --- | --- |
| vivo | For clarification, for which UE power class this enhancement is intended for? |
| CATT | Are we going to introduce the concept of ‘duty cycle’ from RAN1 perspective? |
| Ericsson | Some general comment as we said previously: we think that we’re not prepared to make recommendations to say which schemes are beneficial at this stage since the observations are not agreed yet and we do not know how to compare solutions. Specific comments on this proposal: This proposal is out of the scope of RAN1 and will depend on the RAN4 discussions, so we think this proposal is not needed. |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | This proposal is mainly for PC3 power class. The duty cycle is already part of RAN4 spec for 26 dBm power boosting. Same can be continued. To answer Ericsson, this initially started off as a RAN1 discussion whether or not to send a LS to RAN4 to seek their guidance. As per RAN1 chairman guidance, we must decide whether to support this feature or not. I think we can stick to that discussion. Furthermore, every other proposal in this round has similar wording, So we are not sure if there is any problem with this wording. We propose that this is a useful feature for RAN1 to consider further. |
| FL | @CATT, Ericsson, From FL perspective, if we agree to make recommendation on Power boosting for pi/2 BPSK, the corresponding work can be done in RAN4 during the work item phase. |
| Apple | As we commented before, we are not sure RAN1 should recommend the solutions based on single simulation results. In addition, RAN4 should confirm whether the UE transmission power beyond 26dBm is feasible from UE implementation and regulation point of view. |
| IITH, IITM, CEWIT, Reliance Jio, Tejas Networks | @Apple, we have come back a full round. The proposal initially was to send an LS to RAN4 to ask them if this is possible or not because we have shown that this is useful. But then several companies suggested RAN1 must first decide if this is necessary. As we have shown, using pretty much straightforward results, this is beneficial. So as FL mentioned, the main work can be handled in RAN4. But RAN1 can at least agree that this is beneficial. |

- Proposals (7th round)

Proposal 18: Capture the following observation into the TR.

Observation:

- Seven sources ([China Telecom, R1-2008874], [ZTE, R1-2007743], [Intel, R1-2007954], [DOCOMO, R1-2008557], [Sierra Wireless, R1-2007930], [Apple, R1-2008479], [Huawei, R1-2007583]) evaluate the performance of enhancements on PUSCH repetition type A.

  - One source ([China Telecom, R1-2008874]) shows 3.2 dB (O2I) and 4 dB (O2O) SNR gain when the actual number of repetition is increased from 3 to 8 (counted on the basis of available UL slots) for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type A with 8 nominal repetitions. HARQ is not used.

  - One source ([ZTE, R1-2007743]) shows 1.0~1.5 dB SNR gain for PUSCH transmission with 4 repetitions and maximum 1 re-transmissions (maximum 8 actual transmissions in total, redundancy version {0, 2, 3, 1}) for VoIP at 2% rBLER for FR1 TDD 4GHz with ‘DDDSUDDSUU’ configured by 16 repetitions, compared to PUSCH transmission with 2 repetitions and maximum 3 re-transmissions (maximum 8 actual transmissions in total, redundancy version {0, 2}).

  - One source ([DOCOMO, R1-2008557]) shows 6.8 dB SNR gain, when the actual number of repetition is increased for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type A. HARQ is not used.

  - One source ([DOCOMO, R1-2008557]) shows 6.4 dB SNR gain, when the actual number of repetition is increased for eMBB 1Mbps at 10% iBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type A. HARQ is not used.

  - One source ([Intel, R1-2007954]) shows 2.0 dB SNR gain, when the actual number of repetition is doubled increased to 16 for eMBB with TBS of 136 bits at 10% iBLER for FR1 FDD, compared to Rel-16 PUSCH repetition type A with 8 repetitions. HARQ is not used.

  - One source ([Apple, R1-2008479]) shows 2.2 dB SNR gain when the maximum number of repetitions is increased to 16 for eMBB at 10% iBLER for FR1 FDD, compared to Rel-16 PUSCH repetition type A with 8 repetitions. HARQ is not used.

  - One source ([Apple, R1-2008479]) shows 0.8 dB SNR gain with the repetition and the frequency hopping is enabled. It also shows 2.2dB SNR gain when the maximum number of repetitions is increased to 16 for eMBB with TBS 120bits at 10% iBLER for FR1 FDD, compared to Rel-16 PUSCH repetition type A with 8 repetitions. HARQ is not used.

  - One source ([Huawei, R1-2007583]) shows about 2.0 dB SNR gain when the actual number of repetition is doubled, e.g. from 2 to 4, from 4 to 8, for eMBB at 10% iBLER for FR1 TDD. HARQ is not used.

  - One source ([Huawei, R1-2007583]) shows about 8.1dB SNR gain for PUSCH transmission with 3 retransmissions combined with 4 actual repetitions for VoIP at 2% rBLER for FR1 TDD, compared to PUSCH transmission with no repetition and no retransmission.

  - One source ([Sierra Wireless, R1-2007930]) shows when the TBS is adjusted to maintain the target data rate of 100kbps, +0.4, +0.2, -1.6 dB SNR gain loss was observed when the maximum number of repetitions was increased to 4, 8, 16 respectively for eMBB at 10% iBLER for FR1 FDD using Rel-16 PUSCH repetition type A.

| Companies | Comments |
| --- | --- |

Proposal 20: Capture the following observation into the TR.

Observation:

- Seven sources ([China Telecom, R1-2008874], [IITH, R1-2007905, [Intel, R1-2007954, [Qualcomm, R1-2008626], [InterDigital, R1-2009168], [Nokia, ?], [Sierra Wireless, R1-2007930]) evaluate the performance of TB processing over multi-slot PUSCH.

  - Two sources (China Telecom, R1-2008874], [Qualcomm, R1-2008626]) show 0.6~2 dB  SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot with repetition in Rel-16 without cross-slot channel estimation. HARQ is not used. [If HARQ is enabled, legacy transmissions incur increased frequency domain resource use and a fair comparison cannot be made.] Different redundancy version {0, 2, 3, 1} is used for repetitions in [China Telecom, R1-2008874]. The same redundancy version is used for repetitions in [Qualcomm, R1-2008626]. Gains due to reduction in upper-layer (MAC/RLC/PDCP) headers is not reflected in [Qualcomm, R1-2008626].

  - Two sources ([China Telecom, R1-2008874], [Qualcomm, R1-2008626]) show 1.0~2.7 dB SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot with repetition in Rel-16 without cross-slot channel estimation. HARQ is not used. [If HARQ is enabled, legacy transmissions incur increased frequency domain resource use and a fair comparison cannot be made.] Different redundancy version {0, 2, 3, 1} is used for repetitions in [China Telecom, R1-2008874]. The same redundancy version is used for repetitions in [Qualcomm, R1-2008626]. Gains due to reduction in upper-layer (MAC/RLC/PDCP) headers is not reflected in [Qualcomm, R1-2008626].

  - Two sources ([IITH, R1-2007905], [Nokia, ?]) show 0.8~1.0 dB SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot with repetition in Rel-16 without cross-slot channel estimation. HARQ is not used. The same redundancy version is used for repetitions.

  - One source ([InterDigital, R1-2009168]) shows 0.4 and 2.0 dB SNR gain and with different number of aggregated slots and modulation schemes when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 FDD and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. Up to 10dB power boosting gain can be obtained depending on different number of aggregated slots when TBS determined based on multiple slots and transmitted over multiple slots for FR1 FDD VoIP.

  - One source ([InterDigital, R1-2009168]) shows 0~1.75 dB SNR gain when TBS determined based on multiple slots and transmitted over multiple slots for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used. Up to 6.98 dB power boosting gain can be obtained depending on the number of aggregated slots when TBS determined based on multiple slots and transmitted over multiple slots for FR1 eMBB.

  - One source ([Intel, R1-2007954]) shows 0.2 dB SNR gain and 6.2 dB link budget gain when TBS determined based on multiple slots and transmitted over multiple slots for VoIP at 2% rBLER for FR1 TDD and cross-slot channel estimation is used, compared to TB is determined based on single slot without repetition in Rel-16. HARQ is not used.

  - One source ([Sierra Wireless, R1-2007930]) shows 2 (w/ frequency hopping) and 2.5 dB (w/o frequency hopping) SNR gain for when codewords TBS determined based on multiple slots and transmitted over multiple slots with gaps for eMBB at 10% iBLER for FR1 and cross-slot channel estimation is used, compared to Rel-16 where multiple TBs with repeats are scheduled over contiguous slots without cross-slot channel estimation. HARQ is not used. Perfect channel estimation is assumed.

| Companies | Comments |
| --- | --- |

Proposal 22: Capture the following observation into the TR.

Observation:

- Four sources ([China Telecom, R1-2008874], [Samsung, R1-2009647], [vivo, R1-2007680], [Sierra Wireless, R1-2007930]) evaluate the performance of sub-PRB transmission with multi-slot aggregation.

  - One source ([China Telecom, R1-2008874]) shows around 0.8 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1 when cross-slot channel estimation is used, compared to Rel-16 PRB-based transmission with repetition without cross-slot channel estimation. HARQ is not used. Different redundancy version {0, 2, 3, 1} is used for Rel-16 PRB-based transmission with repetition.

  - One source ([Samsung, R1-2009647]) shows around 5.6 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with 4 PRBs without repetition. HARQ is not used.

  - One source ([Samsung, R1-2009647]) shows around 1.6 and 8.5 dB link budget gain for sub-PRB transmission with multi-slot aggregation (6 tones) for eMBB at 10% iBELR for FR1, respectively, depending on the number of aggregation slots, compared to Rel-16 PRB-based transmission with 1 PRB and 4 PRBs, respectively without repetition. HARQ is not used.

  - One source ([Sierra Wireless, R1-2007930]) evaluatesd the performance of sub-PRB transmission with 2 tones which to reduce MPR and showsed 5 7 dB PAPR reduction. This PAPR reduction allows for MPR relaxation for VoIP use case at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with CP-OFDM DFT-s-OFDM.

  - One source ([vivo, R1-2007680]) shows no gain for sub-PRB transmission with multi-slot aggregation (6 tones) for VoIP at 2% rBLER for FR1, compared to Rel-16 PRB-based transmission with repetition. HARQ is not used.

| Companies | Comments |
| --- | --- |

Proposal 29: Capture the following observation into the TR.

- Enhancements on inter-slot frequency hopping is beneficial for PUSCH coverage enhancements. It is recommended to support enhancements on inter-slot frequency hopping for PUSCH in Rel-17, including:

  - More frequency offsets, e.g. 4 for BWP less than 50 PRBs, 8 for BWP greater than 50 PRBs.

  - More frequency hopping positions, e.g. 4.

| Companies | Comments |
| --- | --- |

Proposal 30: Capture the following observation into the TR.

- Sub-PRB transmission with multi-slot aggregation is beneficial for VoIP PUSCH coverage enhancements. It is recommended to support sub-PRB transmission with multi-slot aggregation targeting VoIP services in Rel-17.

| Companies | Comments |
| --- | --- |

Proposal 32: Capture the following observation into the TR.

- Power boosting for pi/2 BPSK is beneficial for PUSCH coverage enhancements. It is recommended to support Power boosting for pi/2 BPSK for PUSCH in Rel-17, including:

  - Beyond 26 dBm as a function of the UL duty cycle.

| Companies | Comments |
| --- | --- |

- RAN1 #102-e agreements

Agreements:

- Capture the following updated structure in TR 38.830.

6.1		PUSCH coverage enhancements

6.1.1	Time-domain based solutions

6.1.2 	Frequency-domain based solutions

6.1.3	DM-RS enhancements

6.1.4 	Power-domain based solutions

6.1.5 	Spatial-domain based solutions

6.1.6	Others

Agreements:

- Prioritize the study on the performance and specification impacts on time domain based solutions for PUSCH enhancements, including

  - Increase the number of repetitions for PUSCH repetition type A

    - PUSCH repetition with non-consecutive slots/on the basis of available slots for TDD

    - Note: whether increasing the number of PUSCH repetition for FDD depends on the outcome of AI 8.8.1.1.

  - Enhancement on PUSCH repetition Type B

    - E.g., actual repetition across the slot boundary, or the length of actual repetition larger than 14 symbols, etc.

  - TB processing at least over multi-slot PUSCH

    - e.g., single TB, sized for a single slot, but transmitted in parts over multiple slots; or single TB, sized for multiple slots, transmitted over multiple slots, and in conjunction with repetition, etc.

- FFS

  - OCC spreading based repetition

  - Symbol-level repetition

  - TB interleaving

  - RV repetition

  - Early termination of PUSCH repetitions

Agreements:

- Following solutions are not considered for PUSCH enhancements in this study item in RAN1:

  - Enhancements to improve spherical coverage / beam correspondence

  - Reflective arrays

  - Polarization aspects of the UL and/or DL reference signals

Agreements:

- Prioritize the study on the performance and specification impacts on DM-RS enhancements for PUSCH, including

  - Cross-slot channel estimation

  - With a lower priority compared with cross-slot channel estimation (i.e., companies are encouraged to study it)

    - Lower density

      - E.g., DM-RS sharing among multiple PUSCH transmissions or lower DMRS density in the frequency domain.

    - Higher density

      - E.g., in time or frequency domain, e.g., 1-comb pattern

    - Adaptive configuration

    - DM-RS balancing among frequency hops

Agreements:

- Multiple layer PUSCH transmission with DFT-S-OFDM for PUSCH enhancements can be studied with low priority.

- Study open-loop/closed loop Tx diversity for PUSCH enhancements with low priority.

Agreements:

- Study the performance and specification impacts on frequency domain based solutions for PUSCH, including

  - Inter-slot frequency hopping

    - with more frequency offsets

    - with more frequency hopping positions.

  - Inter-slot frequency hopping with inter-slot bundling to enable cross-slot channel estimation

  - Enhancements on frequency hopping for PUSCH repetition type B

    - Note that the above inter-slot frequency hopping enhancement can apply for PUSCH repetition type B

  - Sub-PRB transmission for VoIP

    - FFS: details, e.g., number of tones, multi-slot aggregation

- FFS

  - Intra-slot frequency hopping

    - with more frequency offsets

    - with more frequency hopping positions.

[Note: Appropriate simulation assumptions are expected.]

Agreements:

- Study following power domain based solution for PUSCH enhancements

  - Waveform design to optimize MPR/A-MPR

  - [FDD high power UE]

  - Power boosting for pi/2 BPSK

Note: if a LS to RAN4 (for the last two bullets) is deemed necessary, target sending the LS in the 1st week of RAN1#103-e.

- RAN1 #103-e agreements

Agreements:

For the agreement made in RAN1 #102-e:

Agreements: - Study following power domain based solution for PUSCH enhancements - Waveform design to optimize MPR/A-MPR - [FDD high power UE] - Power boosting for pi/2 BPSK Note: if a LS to RAN4 (for the last two bullets) is deemed necessary, target sending the LS in the 1st week of RAN1#103-e.

RAN1 targets to make a decision whether to further study on power boosting for pi/2 BPSK during this e-meeting.

- Agreements: Capture the followings into the TR

- Enhancements on inter-slot frequency hopping were studied from several aspects, including:

  - More frequency offsets, e.g. 4 for BWP less than 50 PRBs, 8 for BWP greater than 50 PRBs.

  - More frequency hopping positions, e.g. 4.

- Potential specification impacts of enhancements on inter-slot frequency hopping include:

  - Frequency domain hopping offsets/positions.

- Agreements: Capture the followings into the TR

- Enhancements on DM-RS density were studied from several aspects, including lower DM-RS density in time domain, DM-RS sharing among multiple PUSCH transmissions in the time domain, lower DMRS density in frequency domain, 1-comb DM-RS, e.g., DM-RS with single port spans to occupy the whole DM-RS symbol, and additional DM-RS symbol position in a slot.

- Potential specification impacts of lower DM-RS density in time domain, and DM-RS sharing among multiple PUSCH transmissions include:

  - DM-RS pattern and configuration, power consistency, phase continuity, and TBS determination.

- Potential specification impacts of lower DMRS density in the frequency domain include:

  - DM-RS design, DM-RS pattern and configuration.

- Potential specification impacts of 1-comb DM-RS include:

  - DM-RS design, and TBS determination.

- Potential specification impacts of additional DM-RS symbol position in a slot include:

  - DM-RS position.

- Agreements: Capture the followings into the TR

- Adaptive DM-RS configuration was studied. Potential specification impacts include:

  - Related signaling design.

- Agreements: Capture the followings into the TR

- DM-RS balancing among frequency hops was studied. Potential specification impacts include:

  - Related signaling design, DMRS configuration and pattern.

- Agreements: Capture the followings into the TR

- Enhancements on PUSCH repetition type A were studied from several aspects, including increasing the maximum number of repetitions, the number of repetitions counted on the basis of available UL slots and flexible symbol resource allocation in different slots.

- Potential specification impacts of enhancements on increasing the maximum number of repetitions include:

  - TDRA (Time-Domain Resource Allocation).

- Potential specification impacts of enhancements on the number of repetitions counted on the basis of available UL slots include:

  - TDRA (Time-Domain Resource Allocation).

  - Mechanism to determine transmission occasion of actual repetition.

  - Mechanism to determine whether flexible special slot can be determined as an available UL slot.

- Potential specification impacts of enhancements on flexible symbol resource allocation in different slots include:

  - TDRA (Time-Domain Resource Allocation).

- Mechanism to determine UL symbols for each slot.

Agreements: Capture the followings into the TR

- TB processing over multi-slot PUSCH was studied from several aspects, including TBS determined based on single slot and transmitted in parts over multiple slots, TBS determined based on multiple slots and transmitted over multiple slots.

- Potential specification impacts of TB processing over multi-slot PUSCH include:

  - TDRA (Time-Domain Resource Allocation), TBS determination, RV determination.

- Note that power consistency, phase continuity and enhancements for DM-RS configurations may or may not be required depending on factors such as cross-slot channel estimation, etc.

Agreements: Capture the followings into the TR

- Joint channel estimation or DM-RS bundling with/without optimization of DMRS location/granularity was studied from several aspects, including cross-slot channel estimation over consecutive slots, cross-slot channel estimation over non-consecutive slots, cross-repetition channel estimation within one slot, and inter-slot frequency hopping with inter-slot bundling to enable cross-slot channel estimation.

- Potential specification impacts of joint channel estimation or DM-RS bundling include:

  - Power consistency and phase continuity, DM-RS placement in special slot and DM-RS configuration.

  - Time domain hopping interval for inter-slot frequency hopping with inter-slot bundling

Agreements: Capture the followings into the TR

- Power boosting for pi/2 BPSK was studied, including beyond 26 dBm as a function of the UL duty cycle.

- Potential specification impacts include

- UE behavior for power boosting based on the UL time domain resource allocation, explicit or implicit signaling, RF requirement.

Agreements: Capture the followings into the TR

- SIP signal compression was studied for enhancement large payload PUSCH including SigComp used for application information compression and the compression efficiency.

- Potential specification impacts include

  - Using compression algorithm to compress the large SIP signaling message in higher layer.

Agreements: Capture the followings into the TR

- Dynamic PUSCH waveform adaptation was studied. Potential specification impacts include:

  - Related signaling design.

- Agreements: Capture the followings into the TR

- Enhancements on PUSCH repetition type B were studied from several aspects, including actual PUSCH transmission across the slot boundary/invalid symbols, the length of actual repetition larger than 14 symbols, and RV enhancement.

- Potential specification impacts of enhancements on PUSCH repetition type B include:

  - TBS determination, DM-RS pattern, TDRA (Time-Domain Resource Allocation), RV determination,

  - Note that power consistency and phase continuity may or may not be required depending on factors such as cross-slot channel estimation, etc.

- Agreements:

- Sub-PRB transmission for VoIP was studied from several aspects, including number of tones, sub-PRB transmission with single slot and sub-PRB transmission with multi-slot aggregation.

- Potential specification impacts of sub-PRB transmission with single slot include:

  - Frequency domain resource allocation, TBS determination, DM-RS pattern, hopping pattern within/between the PRBs, PUSCH signal generation for DFT-s-OFDM waveform, RF requirement.

- Potential specification impacts of sub-PRB transmission with multi-slot aggregation include:

  - Frequency domain resource allocation, time domain resource allocation, TBS determination, DM-RS pattern, RV determination, hopping pattern within/between the PRBs, PUSCH signal generation for DFT-s-OFDM waveform, RF requirement.

- Agreements: Capture the followings into the TR

- Enhancements on intra-slot frequency hopping were studied from several aspects, including:

  - More frequency offsets, e.g. 4 for BWP less than 50 PRBs, 8 for BWP greater than 50 PRBs.

  - More frequency hopping positions, e.g. 3.

  - More time-domain hop positions within a slot, e.g. 3.

  - DM-RS sharing among multiple PUSCH transmissions with the same frequency position between two consecutive slots [add a reference to the section of DM-RS enhancements when capturing in the TR].

- Potential specification impacts of enhancements on intra-slot frequency hopping include:

  - Frequency domain hopping offsets, DM-RS pattern, TBS determination.

  - Power consistency and phase continuity for DM-RS sharing among multiple PUSCH transmissions

- Agreements: Capture the followings into the TR

- UE transmit waveform design to reduce MPR was studied from several aspects, including tone reservation, FDSS (Frequency Domain Spectral Shaping) without spectral extension for pi/2 BPSK, and FDSS with and without spectral extension for QPSK.

- Potential specification impacts include

  - Related signalling, design for spectral extension, RF requirements.

- Note: For tone reservation, a fraction of tones allocated to a UE are reserved for the UE to shape its waveform; no data is transmitted on these tones.

- Agreements: Capture the followings into the TR

- Spatial domain based solutions were studies from several aspects, including multiple layer PUSCH transmission with DFT-S-OFDM and Open-loop Tx diversity.

- Potential specification impacts include

  - Mechanism to indicate the support of multiple layer PUSCH transmission with DFT-S-OFDM and to determine the precoder, e.g. reuse a subset of the R15 codebooks.

  - Signalling related to support of Tx diversity for PUSCH with DFT-s-OFDM, and different PUSCH spatial filter parameters and different antenna ports for different PUSCH transmissions

- Reference

- 3GPP RP-200861, “Revised SID on Study on NR coverage enhancements”, China Telecom, RAN#88e, June 29th – July 3rd, 2020.

- “3GPP TSG RAN WG1 RAN1 #102e Chairman’s Notes”, e-Meeting, Aug. 17th – 28th, 2020.

- R1-2007583	Potential solutions for PUSCH coverage enhancement	Huawei, HiSilicon

- R1-2007640	PUSCH coverage enhancement	Beijing Xiaomi Mobile Software

- R1-2007680	Discussion on Solutions for PUSCH coverage enhancement	vivo

- R1-2007743	Discussion on potential techniques for PUSCH coverage enhancements  	ZTE

- R1-2007874	Discussion on potential techniques for PUSCH coverage enhancement 	CATT

- R1-2007905	PUSCH coverage enhancements  Indian Institute of Tech (H)

- R1-2007930	Potential techniques for NR coverage enhancements	  Sierra Wireless, S.A.

- R1-2007954	On potential techniques for PUSCH coverage enhancement  Intel Corporation

- R1-2007989	PUSCH coverage enhancement  ETRI

- R1-2007994	Discussion on PUSCH coverage enhancements  China Telecom

- R1-2008026	Discussion on the PUSCH coverage enhancement	CMCC

- R1-2008078	Discussion on PUSCH coverage enhancement	NEC

- R1-2008092	Potential solutions for PUSCH coverage enhancement	Spreadtrum Communications

- R1-2008181	PUSCH coverage enhancement  Samsung

- R1-2008271	Consideration on PUSCH coverage enhancement  OPPO

- R1-2008370	On PUSCH coverage enhancement techniques  Sony

- R1-2008378	Discussion on PUSCH coverage enhancements  Panasonic Corporation

- R1-2008399	PUSCH coverage enhancement  Sharp

- R1-2008403	Discussions on PUSCH coverage enhancement  LG Electronics

- R1-2008419	PUSCH coverage enhancement  Ericsson

- R1-2008479	On potential techniques for PUSCH coverage enhancement  Apple

- R1-2008483	PUSCH coverage enhancements  InterDigital, Inc.

- R1-2008559	Potential techniques for PUSCH coverage enhancements  NTT DOCOMO, INC.

- R1-2008626	Potential coverage enhancement techniques for PUSCH  Qualcomm Incorporated

- R1-2008700	On the use of Tx diversity in DFT-s-OFDM for PUSCH coverage enhancement  NICT

- R1-2008703	Discussion on approaches and solutions for NR PUSCH coverage enhancement  Nokia, Nokia Shanghai Bell

- R1-2008729	Discussion on potential techniques for PUSCH coverage enhancement  WILUS Inc.

- R1-2008743	On transmit diversity techniques for PUSCH coverage enhancement  Mitsubishi Electric RCE

- R1-2007483	[102-e-Post-NR-CovEnh-02] Phase 3: initial collection of simulation results for enhancements Moderator (China Telecom)

- Appendix

[3] R1-2007583  Huawei, HiSilicon

Observation 1: By joint channel estimation across consecutive PUSCH transmissions, a large coverage gain can be achieved as compared to conventional single slot channel estimation, i.e., 1.4 dB and 2.1 dB SNR gains are obtained at 10% BLER for 2 and 3 slots joint channel estimation, respectively. Observation 2: By finer scheduling granularity of (re)transmission, a better coverage performance could be obtained while the large DMRS overhead in each finer granularity (re)transmission would degrade the coverage performance. Observation 3: PUSCH transmissions with finer scheduling granularity and DMRS sharing among multiple PUSCH (re)transmissions can achieve an obvious SNR gain compared to the baseline (a scheduling granularity of 12OS) at 1Mbps target data rate, such as 1.4dB gain can be obtained by the finer scheduling granularity of 2OS. Observation 4: Enhancement of repetition cancellation in TDD (7:3 DL: UL) with doubled actual repetitions can obtain approximately >2dB SNR gain for both urban and rural scenarios. Observation 5: A large SNR gain is obtained by FDD higher power transmission as compared to original repetitions, e.g. 1 dB SNR gain at 10% BLER. Observation 6: By increasing the frequency hopping positions from 2 positions to 4 positions, a >1dB SNR gain can be obtained at 10% BLER for both 4 repetitions with frequency hopping and 8 repetitions with frequency hopping. Proposal 1: Joint channel estimation across consecutive PUSCH transmissions is recommended for PUSCH coverage enhancement. Proposal 2: Potential spec impacts to enable joint channel estimation are identical transmission power, precoding and DMRS port, etc., among consecutive PUSCH transmissions. Proposal 3: Shared DMRS among multiple consecutive PUSCH transmissions with finer scheduling granularity is recommended for PUSCH coverage enhancement, such as non-DMRS configured PUSCH should be supported to reduce DMRS overhead. Proposal 4: Enhancement of repetition cancellation in TDD to ensure more actual repetitions is recommended for PUSCH coverage enhancement. Proposal 5: FDD higher power UE transmission is recommended for PUSCH coverage enhancement. Proposal 6: More frequency hopping positions among multiple repetitions is recommended for PUSCH coverage enhancement.

[4] R1-2007640  Xiaomi

Proposal 1: For PUSCH type B, it is better to extension actual/nominal repetition to support non-consecutive slots and partial special slot for improving the PUSCH utilization efficiency and enhancing coverage. Proposal 2: Whether fixed RV repetition solution having an overall performance gain need to be further evaluated. Dynamic RV repetition or finer granularity RV repetition may be better. Proposal 3: OCC spreading based repetition can increase the spectrum efficiency, it should be considered into the candidate solutions for coverage enhancement. Proposal 4: The number of inter-slot frequency hops positions can be increased to at least 4 to further improve PUSCH coverage. Proposal 5: From the perspective of balancing complexity and performance gain, intra-slot frequency hopping should not to be supported. Proposal 6: Maybe dynamic indication window size and inter-slot bundling size for cross-slot channel estimation is more suitable Proposal 7: Support inter-repetition bundling to enable DMRS sharing. Proposal 8: At least actual repetitions segmented from the same nominal repetition in the same or continuous slots can share the same DMRS. Proposal 9: Support DMRS bundling in Repetition Type B which includes DMRS placement in a special slot.

[5] R1-2007680  vivo

Observation 1: Increase the number of actual repetitions for PUSCH repetition type A have potential specification impacts in the following aspects: - The maximum number of repetitions for repetition type A should be extended. - The postponement rules for repetition type A should be supported. Observation 2: Compared to RV cycling, enhanced RV solution can provide about 2 dB performance gain in some cases. Observation 3: Enhanced RV solution has potential specification impacts in the following aspects - Enhanced or new rules of determining RV for each repetition. - Enhanced or new segment rules of actual repetitions. Observation 4: For multi-slot PUSCH, applying the same TDRA over multiple slots would result in the discontinuous resource and inflexible allocation, the available resources cannot be fully utilized. Observation 5: Multi-slot PUSCH scheme has potential specification impacts in the following aspects - Uniform TDRA or start symbol and length is redesigned for multi-slot PUSCH. - The computation of TBS and DMRS pattern for multi-slot PUSCH is modified. Observation 6: Sub-PRB transmission does not have meaningful performance gain over PUSCH repetition type A with the same TBS. Observation 7: Sub-PRB transmission will lead to significant specification work, if supported. Observation 8: Phase continuity and power consistency among slots or transmissions is not required in current specification. Observation 9: Joint channel estimation have potential specification impacts in the following aspects - UE need to keep the same Tx power across PUSCH transmission if coherent transmission is configured; - Potential UE behavior if the coherency of PUSCH repetitions is impacted by other procedures, e.g. simultaneous transmission if configured with CA; - The granularity in time domain should be defined for joint channel estimation. Proposal 1: For PUSCH repetition type A, it is beneficial to postpone the PUSCH repetition if the resources are not available. Proposal 2: Enhanced RV solutions for PUSCH repetition type B transmission should be considered. Proposal 3: It is beneficial to support multi-slot PUSCH for coverage enhancement. Proposal 4: More frequency hops for inter slot frequency hopping should be considered with higher priority. Proposal 5: Intra-repetition frequency hopping for PUSCH repetition type B should be deprioritized. Proposal 6: sub-PRB transmission is not supported for PUSCH coverage enhancement. Proposal 7: It is beneficial for joint channel estimation to guarantee phase continuity and power consistency among multiple slots or transmission.

[6] R1-2007743  ZTE

Observation 1: For VoIP service, 4 repetitions with maximum 1 re-transmission can provide about 1~1.5 dB gain over 2 repetitions with maximum 3 re-transmissions. Observation 2: In scenario with TDD frame structure ‘DDDSU’ (S: 10D:2G:2U), - PUSCH repetition type B with actual repetition cross slot boundary can provide 0.8dB gain, and - the length of actual repetition with 16 symbols can provide 0.8dB gain. Observation 3: Increasing the number of frequency hopping positions from 2 to 4 could provide additional performance gain for PUSCH repetition. Observation 4: Frequency hopping together with cross-slot channel estimation can provide performance improvement for PUSCH repetition. Observation 5: Cross-slot channel estimation among 8 PUSCH repetitions can provide 1.8 dB gain in urban scenario. Observation 6: Lower DMRS density with only mapping DMRS on half of the PRBs could provide about 1 dB performance gain. Proposal 1: For PUSCH repetition type A, the number of repetitions indicated by gNB should be guaranteed in case of collisions. Proposal 2: For PUSCH repetition type B, support actual PUSCH transmission across the slot boundaries and the length of actual repetition larger than 14 symbols. Proposal 3: Early termination can be considered for NR coverage enhancement. Proposal 4: OCC spreading based PUSCH can be considered for NR coverage enhancement. Proposal 5: Enhancement to inter-slot frequency hopping is supported for NR coverage enhancement. - More frequency hopping positions can be considered. Proposal 6: For PUSCH repetition, support enhanced frequency hopping schemes to enable cross-slot channel estimation among repetitions per hop. Proposal 7: Cross-slot channel estimation among PUSCH repetitions should be supported. - Further study whether phase continuity can be kept across slot boundary. Proposal 8: Support lower DMRS density in the frequency domain for NR coverage enhancement.

[7] R1-2007874  CATT

Observation 1: The benefit of TB processing over multiple slots needs more justification. Observation 2: The motivation of introducing sub-PRB resource allocation needs more justification. Observation 3: The motivation of introducing multi-layer DFT-s-OFDM is not clear. Observation 4: Cross-slot channel estimation is up to gNB implementation. Observation 5: The benefit of DMRS density enhancement needs further justification. Proposal 1: Increasing the repetition number is supported, including: - Increasing the repetition number; - Supporting non-consecutive slots repetition on the basis of available slot/symbols. Proposal 2: Repetition type B enhancement should be studied for PUSCH. Proposal 3: Power domain based enhancement should be carefully studied by RAN4 before starting any specification work in RAN1.

[8] R1-2007905  Indian Institute of Tech (H)

Observation 1: 5G NR coverage enhancement should support additional [x] dB increase in MCL over rel-16 of 5G NR. Observation 2: Coverage enhancement SI should support higher MCL which directly results in higher ISD compared to existing IMT-2020 evaluations. Observation 3: UE with 26 dbm max Tx power for UL duty cycle < 50% provides a substantial increase in cell edge data rates. Proposal 1: Identify [x] dB via system and link-level simulations. Proposal 2: Study enhanced TBS calculations to increase the MCL for Rel-17 by supporting transmissions over multiple UL slots. Proposal 3: Make pi/2 BPSK power boosting a function of the UL duty cycle. Proposal 4: Send LS to RAN4 to study the feasibility of power boosting for pi/2 BPSK modulation beyond 26 dBm as a function of the UL duty cycle.

[9] R1-2007930  Sierra Wireless

Observation 1: Adding gaps between repeats to improve time diversity is a prioritized time-domain based solution Observation 2: Filling the gaps with TBs from the same user, maintains the data rate even when gaps are used. Observation 3: Allowing the gaps to be filled with TBs from other users, improves scheduling flexibility. Observation 4: With FH disabled, 2.5 dB of gain can be achieved when adding gaps between repeats. Observation 5: With FH enabled, 2.0 dB of gain can be achieved when adding gaps between repeats. Observation 6: The multi-slot encoding technique provides similar SNR gains to adding gaps between repeats. Observation 7: Advantages of gaps between repeats over multi-slot encoding: - Improved support for small TBS (e.g. VoIP TBS = 320bits) - Supports larger time diversity (e.g. beyond 64ms) - Scheduling flexibility (i.e. allows other users to be scheduled in gaps) Proposal 1: For the eMBB use cases, do not recommend specifying increased repetition for the PUSCH or PDSCH Proposal 2: Recommend that gaps between repeats be specified as a Rel 17 Coverage enhancement solution Proposal 3: e (LTE-M) scheme be specified to improve coverage for VoIP.

[10] R1-2007954  Intel

Observation 1 - ~2dB performance gain can be observed when doubling the repetition levels for PUSCH. Observation 2 - Compared to single slot transmission with same code rate, TB spanning multiple slots can deliver similar link level performance. - For TB spanning 4 slots with 1 PRB in each slot, ~6dB performance gain can be achieved in term of link budget over single slot transmission with 4 PRBs. Observation 3 - For Rel-15 inter-slot frequency hopping pattern, cross-slot channel estimation can provide ~2dB performance gain compared to the case without cross-slot channel estimation. - When employing cross-slot channel estimation, Rel-15 intra-slot and inter-slot frequency hopping patterns have similar performance. - When employing cross-slot channel estimation, ~1.0dB performance gain can be achieved by enhanced inter-slot frequency hopping pattern, compared to Rel-15 intra-slot and inter-slot frequency hopping pattern. - Compared to Rel-15 inter-slot frequency hopping without cross-slot channel estimation, substantial performance gain, i.e., ~3dB can be achieved by enhanced inter-slot frequency hopping with cross-slot channel estimation. Observation 4 - When 2 Rx antennas are used, ~1.5dB performance gain can be achieved for 4 frequency hops compared to 2 frequency hops. - When 4 Rx antennas are used, ~0.3dB performance gain can be achieved for 4 frequency hops compared to 2 frequency hops. Observation 5 - For 8 repetitions with inter-slot frequency hopping, 4 DMRS symbols can achieve better link level performance than 5 and 6 DMRS symbols for PUSCH. Observation 6 - For 8 repetitions with intra-slot frequency hopping, performance difference is small for the cases when DMRS symbols are not allocated in odd slots and when DMRS symbols are allocated in each slot. Proposal 1 - Maximum number of repetitions can be increased for PUSCH coverage enhancement, especially for TDD configuration with limited UL slots. - It is desirable to allow UE to postpone PUSCH transmission in TDD system, until all the configured/indicated number of repetitions is reached. Proposal 2 - Enhancement on PUSCH repetition type B in time domain needs to be carefully studied in WI phase with considerations of impacts on UE implementation. Proposal 3 - A TB with TBS determined for multiple slots and transmitted over multiple slots can be considered for PUSCH coverage enhancement. Proposal 4 - Inter-slot frequency hopping with inter-slot bundling is supported in conjunction with cross-slot channel estimation for PUSCH coverage enhancement. Proposal 5 - Increasing the number of frequency hops from 2 to 4 for inter-slot frequency hopping may not be supported for NR coverage enhancement WI, when considering practical gNB receiver architecture. Proposal 6 - Depending on coverage enhancement target for VoIP, sub-PRB based resource allocation may not be considered for PUSCH coverage enhancement. Proposal 7 - Higher DMRS density in time domain is not supported for PUSCH coverage enhancement. Proposal 8 - Lower DMRS density in time domain is not supported for PUSCH coverage enhancement.

[11] R1-2007989  ETRI

Proposal 1: For repetition type A enhancement, study the way to guarantee the number of repetition as being indicated. Proposal 2: For repetition type A enhancement, study to indicate more than one SLIVs in a single UL grant. Proposal 3: For repetition type B enhancement, relax the definition of a time window, i.e., include only valid symbols. Proposal 4: For joint channel estimation, study the enhanced power control to keep similar or even same power level during all repetitions.

[12] R1-2007994  China Telecom

Observation 1: The enhanced repetition mechanism, the number of repetitions counted on the basis of available UL slots, can improve the performance of voice service for both O2I and O2O scenario. About 3.2dB and 4dB gain at target 2% rBLER can be obtained for O2I and O2O respectively. Observation 2: Compared with Rel-16 repetition type B, the enhanced repetition type B can improve the performance of voice service for both O2I and O2O scenarios. About 0.8 dB gain at target 2% rBLER can be obtained for both O2I and O2O scenarios. Observation 3: Compared with Rel-16 repetition type A, TB processing over multi-slot PUSCH can improve the performance of voice service for both O2I and O2O scenarios. About 1.0 dB gain and 0.6dB gain at target 2% rBLER can be obtained for O2I and O2O scenarios respectively. Observation 4: TB processing over multi-slot PUSCH can improve the performance of eMBB for rural scenario. About 1.3 dB gain and 2.7dB gain at target 10% iBLER can be obtained for TB processing over 2 slots and 4 slots respectively. Observation 5: The enhanced frequency hopping scheme can improve the coverage performance. About 0.4dB gain at target 10% iBLER and 1.8dB gain at target 2% rBLER can be obtained for eMBB and VoIP respectively compared with Rel-16 frequency hopping scheme. Observation 6: Inter-bundle frequency hopping can improve the coverage performance. Compared with Rel-16 inter-slot frequency hopping, about 0.5dB gain at target 2% rBLER for VoIP service can be obtained for inter-bundle frequency hopping. Observation 7: Sub-PRB transmission can improve the coverage performance. Compared with Rel-16 repetition type A, about 0.8dB gain can be obtained at target 2% rBLER for sub-PRB transmission for voice service. Observation 8: Cross-slot channel estimation can improve the coverage performance. About 0.4dB and 0.8 dB gain at target 10% iBLER can be observed for TDD (DDDSUDDSUU) and FDD respectively. Observation 9: 1-comb DM-RS can improve the coverage performance. About 0.5dB and 1.5 dB gain at target 10% iBLER can be obtained for eMBB for urban and rural scenarios respectively.

[13] R1-2008026  CMCC

Observation 1: 0.4 dB gain could be achieved through the cross-slot channel estimation over 2 slots for PUSCH eMBB traffic. Observation 2: In addition to the cross-slot channel estimation, a lower DMRS density and fully utilized sources in special slot could improve the coverage of PUSCH. And about 1.22 dB coverage gain could be achieved. Proposal 1: Different symbol allocations applied in different slots for PUSCH repetition Type A can be supported. Proposal 2: PUSCH repetition on non-consecutive physical available resources for PUSCH transmission can be supported, both PUSCH repetition Type A and Type B can be considered. Proposal 3: Early termination for PUSCH repetitions can be supported to reduce unnecessary network UL resource occupation and reduce unnecessary UE power consumption. Proposal 4: Fully use of resources in the special slot should be considered for the PUSCH coverage enhancement. Proposal 5: A reduced DM-RS density could spare more resources for PUSCH to improve the coverage. Proposal 6: An adaptive configuration of DMRS could improve the operation flexibility according to different scenarios. Proposal 7: The lower density and adaptive configuration of DMRS should be prioritized.

[14] R1-2008078  NEC

Proposal 1: It’s beneficial to support cross channel. Based on legacy NR, channel across different slot are not inferable. Inferable state could be indicated to UE to support cross channel estimation. Proposal 2: To study different number of DMRS symbol in each slot to make up DMRS symbol pattern over multiple slots when cross slot channel estimation adopted. Proposal 3: It's beneficial to study providing more frequency hopping RB position to increase the coverage. Inter-slot frequency hopping with inter-slot bundling to enable cross-slot channel estimation should be supported.

[15] R1-2008092  Spreadtrum

Proposal 1. The repetition number of dynamic scheduled PUSCH/configured grant PUSCH should be increased, e.g., 32, 64, etc. Proposal 2. Don’t support increasing maximum number of symbols for PUSCH Proposal 3. For both inter/intra-slot hopping, the supported PUSCH hoping positions/number should be increased, e.g., 4, 8, etc. Proposal 4. Postpone the discussion on enhancements on frequency hopping for PUSCH repetition type B. Proposal 5. Sub-PRB transmission is not considered in PUSCH coverage enhancement. Proposal 6. Support to introduce DMRS-less transmission for PUSCH coverage enhancement in Rel-17.

[16] R1-2008181  Samsung

Observation 1: Rel-16 PUSCH repetition type B may have loss of coding gain. Proposal 1: Support an actual repetition across the slot boundary or invalid symbols. Proposal 2: Support a length of a repetition larger than 14 symbols. Observation 2: Rel-16 Type A and Type B PUSCH repetition cannot provide flexible utilization of UL symbols which may reduce coverage and resource utilization efficiency. Proposal 3: A UE considers the slot format indicated by dynamic SFI for adjusting repetitions of a PUSCH transmission to include only UL symbols. Observation 3: Dropping the transmission of repetitions has negative impact on PUSCH, especially for configured grant. There are trade-offs with both dropping and postponing cancelled repetitions and it is beneficial to enable a gNB to configure either dropping or postponing repetitions to a UE. Proposal 4: For Type A PUSCH repetition, support postponing/dropping cancelled repetitions for a PUSCH transmission. Proposal 5: Support enhancements on cross-slot channel estimation. Proposal 6: Support enhancements on frequency hopping with cross-slot channel estimation. Proposal 7: Support sub-PRB transmission for coverage enhancement. Observation 4: The necessary spec impact for sub-PRB transmission is frequency domain resource allocation as well as some consideration in pow

[17] R1-2008271  OPPO

Observation 1: Assuming the low fixed MCS, the BLER performance can be improve while the target throughput for eMBB in Urban Scenarios cannot be improved with repetition enhancement only. The while the target throughput for eMBB in Rural Scenarios can be enhanced by repetition. Observation 2: Repetition scheme can also enhance the coverage in lower SNR and data rate lower than the target eMBB data rate. Observation 3: When the maximum number of effective transmissions is fixed, adaptive repetition can achieve better performance. The adaptive repetition can be supported by HARQ mechanism with high control overhead or by enhance repetition mechanism. Observation 4: PUSCH/PDSCH slot aggregation would restrict flexible slots as uplink/downlink, which restricts the scheduling flexibility. Proposal 1: Slot aggregation and dynamic repetition can be considered as the baseline for Rel-17 PUSCH coverage enhancement. Proposal 2: Introducing higher aggregation factor for PUSCH repetition and adaptive PUSCH repetition with earlier termination. Proposal 3: Restricting PUSCH repetition in preconfigured valid slots. Proposal 4: Cross-slot estimation, DMRS-less and non-uniform distributed DMRS can be considered for PUSCH repetition. Supporting frequency hopping of PUSCH based on multiple slots. Proposal 5: During PUSCH repetition, different PUSCH spatial filter parameters and different antenna ports can be applied for different PUSCH slots. Proposal 6: High power UE for FDD shall be discussed in RAN4.

[18] R1-2008370  Sony

Proposal 1: Coverage enhancement supports the coverage enhancement of CG-UCI and CG-PUSCH. Observation 1: Pairwise repetition of CG-UCI and CG-PUSCH reduces PUSCH buffering and allows for earlier PUSCH decoding. Proposal 2: CG-UCI and CG-PUSCH are repeated in pairwise fashion. Observation 2: If the DM-RS density can be dynamically changed in both time and frequency domains, the throughput of the PUSCH can be optimized both with respect to time and frequency selective channel conditions. Proposal 3: Adaptive configuration of DM-RS to improve PUSCH coverage should be studied. Proposal 4: For PUSCH, frequency hopping with at least four hops is supported through multiple configured grants. Proposal 5: For PUSCH frequency hopping, the gNB can dynamically adapt the frequency hopping pattern, based on which hops are more effective. Observation 3: A higher transmit power UE can improve PUSCH coverage. Observation 4: An HD-FDD UE can transmit a higher power than an FD-FDD UE while using the same PA. Proposal 6: Coverage enhancement supports half-duplex FDD UEs. Proposal 7: Coverage enhancement supports sub-PRB PUSCH transmission. Proposal 8: Send an LS to RAN4 recommending that RAN4 study higher transmit power FDD UEs and negative MPR / power boosting.

[19] R1-2008378  Panasonic

Observation 1: The repetition enhancement such that the number of repetitions is counted on the basis of available UL slots is useful if the resource usage of PDCCH needs to be reduced like PDCCH repetition. Proposal 1: In order to support cross-slot channel estimation, RAN1 asks to RAN4 in what condition phase continuity can be kept. Observation 2: For the repetition with DMRS enhancement, DMRS distribution over the duration of repetition is important for the trackability to channel variation. Observation 3: DMRS allocation of PUSCH repetition Type A with cross-slot 2D MMSE channel estimation can provide better averaging and interpolation performance. Observation 4: Single TB, sized for multiple slots, transmitted over multiple slots could be considered up to 1/3 or 1/5 coding rate. Further lower coding rate is realized by the repetition. Observation 5: In the combination with cross-slot/cross-repetition channel estimation, frequency hopping enhancement such as configurable time domain hopping interval should be supported.

[20] R1-2008399  Sharp

Proposal 1: Capture DMRS bundling as a candidate solution for coverage enhancement in the TR. Proposal 2: DMRS-less repetition in the aggregated slot can be considered. Proposal 3: TBS scaling is required to meet the target data rate requirement in a PUSCH transmission with aggregated slot. Proposal 4: DMRS allocation can be based on the aggregated resource or the nominal repetition instead of the actual repetition. Proposal 5: For repetition type B, TBS scaling can be considered with Option 1: Scaling based on the amount of time resources or Option 2: Longer nominal repetition length than 14. Proposal 6: Increased number of repetitions can be considered for VoIP coverage enhancement.

[21] R1-2008403  LG

Proposal 1: To increase the number of PUSCH repetitions in unpaired spectrum, PUCCH repetition principle can be reused for PUSCH Proposal 2: Flexible symbol resource allocation depending on PUSCH repetition slot should be studied for PUSCH coverage enhancement. Observation 1: PUSCH TB mapping over multi-slot is beneficial in terms of performance gain and frequency-domain multiplexing. Proposal 3: Symbol-level combining should be studied to enhance PUSCH coverage. Symbol-level combining can be supported by applying the same RV value during a bundle of PUSCH slots. Proposal 4: To support cross-slot channel estimation, the same beam direction, precoding matrix, and PRB location should be applied for PUSCH repetitions during the slots. Proposal 5: Enhancement of DMRS density does not seems to be necessary. Proposal 6: To support bundled slot wise frequency hopping, the location and number of bundled slot need to be discussed further. Proposal 7: For sub-PRB transmission, related topics (i.e., TB mapping, DMRS mapping) should be studied.

[22] R1-2008419  Ericsson

Observations: - Non-coherent and partially coherent UE’s PAPR or cubic metric (CM) of multiple layer PUSCH transmission is not higher than 1-layer PUSCH transmission by coherent UE. - Multiple layer PUSCH transmission with DFT-S-OFDM can improve PUSCH cell coverage. - Multiple layer transmission is especially beneficial in the non-coherent UEs that are those most used in real deployment, since multi-layer transmission provides more power in these UEs. - Pure rank 1 transmission tends to be infrequent even for UEs in the poorest channel conditions when few gNB antennas are used. - When massive MIMO gNBs are used, rank 1 is almost never selected. - Cross-slot channel estimation brings gains, but further study is needed on how much needs to be specified vs. what can be done in gNB implementation (e.g. by estimating wideband phase corrections to combine slots). - Using 4 instead of 2 hops can bring modest gains in a limited set of scenarios. - SigComp can compress SIP packets at application layer before encryption is used. This feature should be considered for Voice coverage enhancement. It has better potential i.e. suitable for all scenarios regardless of whether packets are encrypted or unencrypted. - Early CSI may also benefit the Voice Service. Having accurate CSI for a UE in poor coverage that wants to send a large UL SIP packet such as INVITE can allow the network to apply schemes such as beamforming, frequency selective scheduling, robust modulation and coding schemes, etc. Based on the observations and discussions, we have following proposals. Proposals: - Consider at least the following areas for UL coverage enhancement: - Improvements to low PAPR transmission - Multi-antenna techniques - Cross-slot channel estimation - Specify multiple layer PUSCH transmission with DFT-S-OFDM. - Indicate to CT1 and SA4 that 2KB SIP message sizes may impact VoNR coverage or setup latency in arduous coverage scenarios and ask if SigComP functionality can be supported to reduce SIP message overhead. - Ask CT1/SA4 what SIP message packet sizes and arrival rates can be expected.

[23] R1-2008479  Apple

Proposal 1:The coverage enhancement solutions should be down selected further according to the performance and standard impacts Proposal 2: The benefits of TB processing over multi-slot, sub-PRB transmission and frequency hopping enhancement for PUSCH repetition type B need to be studied further. Observation: - At low SNR operating point, PUSCH performance is improved with one more DM-RS symbol. - Intra-slot Frequency hopping can provide benefit with smaller number of PRB allocation and a relative high SNR region. - Usage several techniques together could provide better performance than single technique applied.

[24] R1-2009583  InterDigital

Observation 1: The number of DMRS symbols placed in an uplink slot should be minimized without sacrificing channel estimation performance Observation 2: DMRS symbol can be placed in a special slot which is placed before the uplink slot, allowing channel estimation across the slot boundary Observation 3: The number of required PRBs can be lowered compared to the baseline scheme using the enhanced Type B transmission where the length of actual repetition is 16 Observation 4: The BLER performance can be improved by allocation of an additional DMRS symbol in the enhanced Type B transmission where the length of actual repetition is 16 Observation 5: BLER performance can be improved with optimized MCS and nearly 2 dB gain can be obtained over the baseline scheme in the enhanced Type B transmission where the length of actual repetition is 16 Observation 6: BLER gain of 0.3dB can be obtained by performing cross-slot channel estimation with DMRS in a special slot Observation 7: Relying solely on repetitions to meet PUSCH coverage can have the following shortcomings: 1) A non-narrow band frequency allocation, thus reducing the TB’s power spectral density 2) An increase of latency required to transmit the TB/reach the required HARQ operating point 3) Increased cell load, which may come at the cost of other service types or users in the cell. Observation 8: TB mapped over multiple slots can yield up to 2 dB gain Observation 9: Up to 10log(N) dB power boosting gain can be obtained when TBS is determined based on N slots and transmitted over N slots This contribution proposes to support the following enhancements Proposal 1:Support DMRS placement in a special slot which can be bundled with DMRS in the adjacent uplink slot Proposal 2:Support frequency hopping pattern which contains bundled DMRS in the same hop. Proposal 3: Support DMRS bundling for repetition type A and B Proposal 4: Support the use of uplink symbols in the adjacent special slot to extend duration of PUSCH Proposal 5 : Support enhanced Type B repetition where the length of a repetition is larger than 14 symbols Proposal 6: Support repetition of PUSCH over non-consecutive slots Proposal 7: Support DMRS placement in special slot for cross-slot channel estimation Proposal 8: Support TB scheduling over consecutive slots in the time domain without repetition Proposal 9 Support TB encoding for transmission of coded TB segments mapped over multiple slots Proposal 10: Support partial TB retransmission for TBs transmitted over a multi-slot PUSCH Proposal 11: Support methods to minimize MPR of the waveform

[25] R1-2008559  NTT DOCOMO

Proposal 1: Extension of PUSCH repetition to support non-consecutive slots can be one of the potential techniques for PUSCH coverage enhancement. Proposal 2: More efficient utilization of partial slot with next full slot for PUSCH can be one of the potential techniques for PUSCH coverage enhancement. Proposal 3: High PSD (small number of PRBs) with high coding rate may have advantage for coverage performance, and additional PRB unit with smaller number of subcarriers (e.g. half PRB with 6 subcarriers) can be one of the potential techniques for PUSCH coverage enhancement. Proposal 4: Frequency hopping with multiple frequency offsets can be one of the potential techniques for PUSCH coverage enhancement. Proposal 5: Denser DM-RS mapping (e.g., 2 for DM-RS symbol duration, and pos3 for additional DM-RS symbol position) can be expected for enhancement of coverage performance. Proposal 6: Cross-slot channel estimation with non-consecutive PUSCH slots should be also considered for the performance evaluation.

[26] R1-2008626  Qualcomm

Proposal 1: For enhancing the coverage of PUSCH, consider techniques for UE transmit waveform design that allow further reduction in the MPR values for DFT-S-OFDM and CP-OFDM waveforms. In particular, consider tone reservation principle for DFT-s-OFDM and CP-OFDM waveforms to further reduce PAPR. Proposal 2: Consider DMRS bundling technique for coverage enhancement in Rel-17. Observation: Using appropriate DMRS configuration could provide up to 1.7dB gain per fixed MCS and tens of percent of TPUT increase at cell edge scenarios for PUSCH. Proposal 3: Introduce a new mechanism for dynamic DMRS configuration signaling to enable DMRS adaptation in Rel-17. Proposal 4: Consider the following signaling options for dynamic DMRS configuration - Option 1: Dynamic MAC-CE based activation and complementary DCI based selection of one of the activated DMRS configuration options. - Option 2: Single active DMRS configuration option that is dynamically activated by MAC-CE to adopt DMRS configuration to channel and SNR conditions. Proposal 5: Introduce a new mechanism for dynamic signaling of the transmission scheme to enable waveform adaptation to reception conditions associated with PUSCH transmissions of a UE. Proposal 6: Consider the following options for dynamic implicit signaling of the transmission scheme for PUSCH: - Option 1: Dynamic MAC-CE based activation of the active MCS table. Different RRC configured MCS tables are associated with different waveforms and the activated MCS table provides an implicit indication of the associated waveform. - Option 2: To define a new hybrid MCS table that will contain a combination of MCSs from MCS tables for DFT-S-OFDM and CP-OFDM transmission schemes while different MCS index ranges will be associated with different waveforms. MSC index indicated for a PUSCH grant will be used as an implicit signaling for the transmission scheme. Proposal 7: Consider TBS scaling and optimization across multiple slots for PUSCH coverage enhancement in Rel-17.

[27] R1-2008700  NICT

Observation 1: DFT-s-OFDM is better waveform from viewpoint of reduction of PAPR than CP-OFDM. Observation 2: The use of Tx diversity is effective to reduce required SNR in PUSCH with DFT-s-OFDM at BLER of 10%. It could contribute to realizing coverage enhancement. Proposal 1: DFT-s-OFDM is better waveform from viewpoint of reduction of PAPR than CP-OFDM. Proposal 2: Tx diversity should be introduced in order to realize the reduction of necessary fading margin for coverage enhancement. Proposal 3: The use of DFT-s-OFDM with Tx diversity should be one of the approaches for coverage enhancement.

[28] R1-2008703  Nokia, Nokia Shanghai Bell

Observation 1. The coverage for data channel can be improved by using qam64-LowSE MCS index table (table 3), which enables lower code rate as compared to its 256QAM and 64QAM counterparts. Observation 2. For a fixed number of PRBs, using the lowest possible MCS index, which still guarantees the target throughput, can extend the cell coverage. Observation 3. The coverage of PUSCH can be enhanced by identifying the optimal combination of number of allocated PRBs and MCS index for PUSCH to meet the throughput target. Observation 4. The maximum number of repetitions for PUSCH repetition type A in release 15 is sufficient for FDD deployment. Observation 5. The PUSCH repetition type B can be used to cope with the cancellation due to DL/UL collision in TDD deployment. Therefore, the consideration of counting repetition numbers based on non-consecutive slots for PUSCH repetition type A may not be needed. Observation 6. There is a tradeoff between the time domain diversity gain from PUSCH repetition and the low coding rate gain brought by the potential TB processing over multiple slots. The applicability of this solution in TDD deployment is limited. Observation 7. The potential advantage of introducing early termination of PUSCH repetition and/or more RVs is unclear and, if any at system-level, likely absent at link-level. Observation 8. In TDD deployments, the coverage of PUSCH can be significantly enhanced by simply considering the frame structure that maximizes PUSCH coverage while ensuring that DL target throughput is met. Observation 9. Inter-slot frequency hopping with inter-slot bundling to enable cross-slot channel estimation can be further considered following the outcome of the discussion on cross-slot channel estimation solution. Observation 10. Specific enhancement for PUSCH repetition type B in frequency domain is not needed. However, generic enhancements agreed in frequency domain, if any, can be considered for PUSCH repetition type B. Observation 11. Intra-slot frequency hopping can help to improve PUSCH coverage. DMRS overhead can be reduced thanks to cross-slot joint channel estimation/channel information sharing. Observation 12. Introducing sub-PRB transmission may be beneficial for coverage, in case of low data rate applications. Observation 13. Cross-slot channel estimation and DMRS-less PUSCH transmission require several constraints to be applicable in practice. Observation 14. Extending the spectral shaping for QPSK is a potential solution to reduce MPR and to improve UL PUSCH coverage. Observation 15. FDSS without spectrum extension (defined for pi/2 BPSK in Rel-15) is not beneficial for QPSK due to the lack of significant gain in terms of CM and PAR. Observation 16. FDSS with spectrum extension is a potential candidate for shaping with QPSK because it can reduce both CM and PAR efficiently. Observation 17. The Output Back-Off at PA of the original QPSK waveform is reduced by 1.0-1.7 dB by applying FDSS with spectral extension. Observation 18. At the PRB allocations of interest for coverage, the OBO difference between pi/2 BPSK FDSS and QPSK FDSS with spectral extension is 0-0.3 dB. Observation 19. Link performance of QPSK with extend FDSS is comparable to the original QPSK waveform without FDSS. Observation 20. Link performance of QPSK with extend FDSS is always better compared to the pi/2 BPSK with FDSS. Observation 21. Coverage of the original QPSK waveform can be improved up to 2 dB by applying FDSS with spectral extension. Observation 22. At the PRB allocations of interest for coverage, the QPSK FDSS with spectral extension provides in most cases better, and the rest of the cases at least comparable coverage compared to pi/2 BPSK FDSS. Proposal 1. In the TR of Rel-17 NR coverage enhancement SI, the following observation should be captured: The coverage for data channel can be improved by using qam64-LowSE MCS index table (table 3), which yields more MCS indices with lower code rate as compared to its 256QAM and 64QAM counterparts, especially in scenarios with low(er) throughput requirements. Proposal 2. In the TR of Rel-17 NR coverage enhancement SI, the following observation should be captured: The coverage of PUSCH can be enhanced by identifying the optimal combination of number of allocated PRBs and MCS index for PUSCH to meet the throughput target. Proposal 3. The available features in NR Releases 15 and 16 should be considered when discussing work items for NR coverage enhancement. Proposal 4. The enhancement on PUSCH repetition type B, e.g. actual repetition across the slot boundary, or the length of actual repetition larger than 14 symbols, is in the same category and should be discussed together with TB processing over multiple slots for PUSCH. The PUSCH coverage enhancement based on PUSCH repetition type B framework should be avoided. Proposal 5. DMRS balancing should be considered for the design of new DMRS enhancement solutions. Proposal 6. The FDSS with spectral extension for QPSK is considered as potential solution to reduce MPR and to improve UL PUSCH coverage.

[29] R1-2008729  WILUS

Proposal 1: Discuss detailed methods about actual repetition across the slot boundary and the length of actual repetition larger than 14 symbols in WI phase. Proposal 2: Discuss options about RE calculation that extended to multiple slots in WI phase. -Redesign NRE' calculation -Redesign NRE calculation Proposal 3: Discuss determination of frequency hopping boundary based on time domain coverage enhancement in WI phase.

[30] R1-2008743  Mitsubishi Electric

Proposal 1: Alamouti-based transmit diversity is supported for PUSCH with DFTsOFDM.