3GPP TSG RAN WG1 Meeting #104-e	R1-2100093

e-Meeting, January 25th – February 5th, 2021

Source: 	ZTE

Title: 	Maintenance of Rel-16 MR-DC and CA

Agenda item:	7.2.10

Document for:	Discussion/Decision

# Introduction

In this contribution, discussion and analysis on remaining issues of Rel-16 MR-DC/CA are presented.

# Discussion

## SCell dormancy

### Issue1

During RAN1#103e meeting, the issue on HARQ-ACK priority of SCell dormancy indication was brought up and discussed preliminarily. Companies preferred to re-discuss this issue again in RAN1#104e meeting so that they can have more time to further check.

The enhancement of two HARQ-ACK codebooks with different priority is introduced in Rel-16. Based on the current TS38.213, HARQ-ACK priority determination for the corresponding PDSCH reception and the corresponding SPS release has been specified. The detailed spec is copied as below.

TS 38.213 Section 9 (HARQ-ACK priority determination for SPS and SPS release.) A PUSCH or a PUCCH transmission, including repetitions if any, can be of priority index 0 or of priority index 1. For a configured grant PUSCH transmission, a UE determines a priority index from phy-PriorityIndex, if provided. For a PUCCH transmission with HARQ-ACK information corresponding to a SPS PDSCH reception or a SPS PDSCH release, a UE determines a priority index from harq-CodebookID, if provided. For a PUCCH transmission with SR, a UE determines the corresponding priority as described in Clause 9.2.4. For a PUSCH transmission with semi-persistent CSI report, a UE determines a priority index from a priority indicator field, if provided, in a DCI format that activates the semi-persistent CSI report. If a priority index is not provided to a UE for a PUSCH or a PUCCH transmission, the priority index is 0.

TS 38.213 Section 9 (HARQ-ACK priority determination for corresponding PDSCH reception.) If in an active DL BWP a UE monitors PDCCH either for detection of DCI format 0_1 and DCI format 1_1 or for detection of DCI format 0_2 and DCI format 1_2, a priority index can be provided by a priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format 0_1 and DCI format 1_1 and for detection of DCI format 0_2 and DCI format 1_2, a DCI format 0_1 or a DCI format 0_2 can schedule a PUSCH transmission of any priority and a DCI format 1_1 or a DCI format 1_2 can schedule a PDSCH reception and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority.

However, the HARQ-ACK priority determination for SCell dormancy indication without scheduling PDSCH reception is still not specified.

This CR tries to clarify that the HARQ-ACK priority for SCell dormancy indication without scheduling PDSCH reception is determined by the priority indicator in DCI format 1_1. The corresponding change is copied below for convenience.

Proposal 1: RAN1 introduces the following TP for Section 9 of TS38.213 (Included in CR#1 also).

If in an active DL BWP a UE monitors PDCCH either for detection of DCI format 0_1 and DCI format 1_1 or for detection of DCI format 0_2 and DCI format 1_2, a priority index can be provided by a priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format 0_1 and DCI format 1_1 and for detection of DCI format 0_2 and DCI format 1_2, a DCI format 0_1 or a DCI format 0_2 can schedule a PUSCH transmission of any priority, a DCI format 1_1 or a DCI format 1_2 can schedule a PDSCH reception and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority, and a DCI format 1_1 can indicate SCell dormancy and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority.

### Issue2

During RAN1#103e meeting, the issue on how to generate HARQ-ACK for SPS PDSCH release when two transport blocks are configured has been resolved.

If maxNrofCodeWordsScheduledByDCI indicates reception of two transport blocks, when the UE receives a PDSCH with one transport block or a SPS PDSCH release, the HARQ-ACK information is associated with the first transport block and the UE generates a NACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is not provided and generates HARQ-ACK information with value of ACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is provided.

However, the same issue for SCell dormancy indication without scheduling a PDSCH still exists. This CR tries to clarify this issue.

Proposal 2: RAN1 introduces the following TP for Section 9.1.2.1 of TS38.213 (Included in CR#1 also).

If maxNrofCodeWordsScheduledByDCI indicates reception of two transport blocks, when the UE receives a PDSCH with one transport block, SCell dormancy indication or a SPS PDSCH release, the HARQ-ACK information is associated with the first transport block and the UE generates a NACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is not provided and generates HARQ-ACK information with value of ACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is provided.

# Conclusion

In this contribution, we provide our analysis and discussion for SCell dormancy.

Proposal 1: RAN1 introduces the following TP for Section 9 of TS38.213 (Included in CR#1 also).

If in an active DL BWP a UE monitors PDCCH either for detection of DCI format 0_1 and DCI format 1_1 or for detection of DCI format 0_2 and DCI format 1_2, a priority index can be provided by a priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format 0_1 and DCI format 1_1 and for detection of DCI format 0_2 and DCI format 1_2, a DCI format 0_1 or a DCI format 0_2 can schedule a PUSCH transmission of any priority, a DCI format 1_1 or a DCI format 1_2 can schedule a PDSCH reception and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority, and a DCI format 1_1 can indicate SCell dormancy and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority.

Proposal 2: RAN1 introduces the following TP for Section 9.1.2.1 of TS38.213 (Included in CR#1 also).

If maxNrofCodeWordsScheduledByDCI indicates reception of two transport blocks, when the UE receives a PDSCH with one transport block, SCell dormancy indication or a SPS PDSCH release, the HARQ-ACK information is associated with the first transport block and the UE generates a NACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is not provided and generates HARQ-ACK information with value of ACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is provided.