3GPP TSG RAN WG1 #104-e		R1-2100135

e-Meeting, January 25th – February 5th, 2021

Source:	OPPO

Title:	Draft TP on physical structure for NR sidelink

Agenda Item:	7.2.4

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss some open issues in current specifications on physical layer structure of NR-V2X and give text proposals for correction.

Discussion

  - PSSCH DMRS pattern for 3/4/5PSSCH OS

In RAN1#99 it was agreed that the minimum number of symbols that can be used for sidelink within a slot is 7 [1]:

Agreements: - For Rel-16, (normal CP) - Support 7, 8, 9,…, 14 symbols in a slot without SL-SSB for SL operation - Target reusing Uu DM-RS patterns for each of the symbol-length, with modifications as necessary - No other additional spec impact is expected for supporting 7, 8, …, 13 - # of DM-RS symbols - 2, 3, 4 - For a dedicated carrier, only 14-symbol is mandatory - There is a single (pre-)configured length of SL symbols in a slot without SL-SSB per SL BWP. - There is a single (pre-)configured starting symbol for SL in a slot without SL-SSB per SL BWP.

Considering the overhead of GAP and PSFCH, the minimum number of symbols for PSSCH within a slot are 3(including AGC symbol). However, for now, the DMRS pattern for PSSCH with less than 6 symbols (i.e. ld<6) are not defined yet [2], which implies that in case of less than 10 symbols are configured for sidelink on a BWP, and PSFCH resources are configured in a resource pool, there is no available DMRS pattern for PSSCH transmitted in a slot configured with PSFCH resources. One way to address the problem is to define DMRS patterns for 3≤ld<6, as suggested in Text Proposal 1, another way is to add one restriction in the specification that PSFCH resources shall not be configured in case of less than 10 symbols are configured for sidelink on a BWP, as suggested in Text Proposal 2 below.

Proposal 1: Adopt either Text Proposal 1 or Text Proposal 2 to correct the PSSCH DMRS issue in current specification.

===========================Text Proposal 1 (38.211) ===============================

8.4.1.1.2	Mapping to physical resources

<Not related omitted>

The position(s) of the DM-RS symbols is given by l according to Table 8.4.1.2.2-1 where ld is the duration of the scheduled resources for transmission of PSSCH and the associated PSCCH, including the OFDM symbol duplicated as described in clauses 8.3.1.5 and 8.3.2.3.

Table 8.4.1.2.2-1: PSSCH DM-RS time-domain location.

| ld in symbols | DM-RS position l |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSCCH duration 2 symbols |  |  | PSCCH duration 3 symbols |  |  |
|  | Number of PSSCH DM-RS |  |  | Number of PSSCH DM-RS |  |  |
|  | 2 | 3 | 4 | 2 | 3 | 4 |
| 3 | 1,2 |  |  | 1,2 |  |  |
| 4 | 1,3 |  |  | 1,3 |  |  |
| 5 | 1,4 |  |  | 1,4 |  |  |
| 6 | 1, 5 |  |  | 1, 5 |  |  |
| 7 | 1, 5 |  |  | 1, 5 |  |  |
| 8 | 1, 5 |  |  | 1, 5 |  |  |
| 9 | 3, 8 | 1, 4, 7 |  | 4, 8 | 1, 4, 7 |  |
| 10 | 3, 8 | 1, 4, 7 |  | 4, 8 | 1, 4, 7 |  |
| 11 | 3, 10 | 1, 5, 9 | 1, 4, 7, 10 | 4, 10 | 1, 5, 9 | 1, 4, 7, 10 |
| 12 | 3, 10 | 1, 5, 9 | 1, 4, 7, 10 | 4, 10 | 1, 5, 9 | 1, 4, 7, 10 |
| 13 | 3, 10 | 1, 6, 11 | 1, 4, 7, 10 | 4, 10 | 1, 6, 11 | 1, 4, 7, 10 |

==============================End of Text Proposal 3=======================

===========================Text Proposal 2 (38.213) ===============================

## 16.3	UE procedure for reporting HARQ-ACK on sidelink

A UE can be indicated by an SCI format scheduling a PSSCH reception, in one or more sub-channels from a number of Nsubch PSSCH sub-channels, to transmit a PSFCH with HARQ-ACK information in response to the PSSCH reception. The UE provides HARQ-ACK information that includes ACK or NACK, or only NACK.

A UE can be provided, by sl-PSFCH-Period-r16, a number of slots in a resource pool for a period of PSFCH transmission occasion resources. If the number is zero, PSFCH transmissions from the UE in the resource pool are disabled.

A UE expects that the number indicated by sl-PSFCH-Period-r16 is zero if the number of consecutive symbols for SL transmissions indicated by lengthSLsymbols is less than 10.

A UE expects that a slot t'kSL (0≤k<T'max) has a PSFCH transmission occasion resource if k mod NPSSCHPSFCH=0, where t'kSL is defined in [6, TS 38.214], and T'max is a number of slots that belong to the resource pool within 10240 msec according to [6, TS 38.214], and NPSSCHPSFCH is provided by sl-PSFCH-Period-r16.

A UE may be indicated by higher layers to not transmit a PSFCH in response to a PSSCH reception [11, TS 38.321].

<Not related omitted>

==============================End of Text Proposal 2=======================

Conclusion

In this contribution, we discussed some open issues on physical layer structures in current specifications, we have following proposals:

Proposal 1: Adopt either Text Proposal 1 or Text Proposal 2 to correct the PSSCH DMRS issue in current specification.

References

- 3GPP RAN1 #99 Chairman’s Notes, November 18th – 22nd, 2019

- 3GPP TS 38.211 V16.3.0 (2020-09), 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical channels and modulation (Release 16)