3GPP TSG RAN WG1 #104-e  	                                                         	     R1-2100147

e-Meeting, January 25th – February 5th, 2021

Source:	OPPO

Title:	Text proposals on type-3 HARQ-ACK codebook

Agenda Item:	7.2.2

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss an unclear case of type-3 HARQ-ACK codebook determination.

Discussion

Based on the current description in TS 38.213 (section 9.1.4), if a UE has obtained HARQ-ACK information for a given HARQ process, HARQ-ACK information for the given HARQ process is included into the type-3 codebook. However, if a UE has not obtained HARQ-ACK information corresponding to a given HARQ process, since:

- The given HARQ process has not been scheduled before; or

- There is no enough processing time for PDSCH decoding before type 3 codebook generation.

the procedure of type 3 codebook generation is not completely yet.

One example is shown in Figure 1, assuming 4 HARQ processes, i.e. HP 0~3, are configured to a UE, HARQ process #3 is never received by the UE, and the decoding timeline of HP 0 with NDI=1 cannot be satisfied. Based on the current description in Clauses 9.1.4 of TS38.213, type 3 codebook only contains the HARQ-ACK bits corresponding to HP 1 and HP 2. This does not align with the agreement (RAN1 #98bis) that a HARQ-ACK codebook containing all DL HARQ processes.

Figure 1: One example of Type 3 codebook

Proposal 1: Adopt TP1 for the generation of type-3 HARQ-ACK codebook.

- If the UE has not obtained HARQ-ACK information for a given HARQ process, NACK should be feedback for the given HARQ process.

-------------------------------------------- Start of TP1 38.213 V16.4.0 section 9.1.4----------------------------------------

9.1.4	Type-3 HARQ-ACK codebook determination

<Unchanged parts are omitted>

else

if NHARQ-ACK,cCBG/TB,max>0

while t<NTB,cDL

if UE has reported HARQ-ACK information for TB t for HARQ process number h on serving cell c, and has not subsequently detected a DCI format scheduling a PDSCH reception, or received a SPS PDSCH, with TB t for HARQ process number h on serving cell c

while g<NHARQ-ACK,cCBG/TB,max

j=j+1

g=g+1

end while

end if

if UE has obtained HARQ-ACK information for TB t for HARQ process number h on serving cell c corresponding to a PDSCH reception and has not reported the HARQ-ACK information corresponding to the PDSCH reception

while g<NHARQ-ACK,cCBG/TB,max

= HARQ-ACK information bit for CBG g of TB t for HARQ process number h of serving cell c

j=j+1

g=g+1

end while

elseif UE has not obtained HARQ-ACK information for TB t for HARQ process number h on serving cell c

while g<NHARQ-ACK,cCBG/TB,max

j=j+1

g=g+1

end while

end if

g=0

t=t+1

end while

else

while t<NTB,cDL

if UE has reported HARQ-ACK information for TB t for HARQ process number h on serving cell c and has not subsequently detected a DCI format scheduling a PDSCH reception, or received a SPS PDSCH, with TB t for HARQ process number h on serving cell c

= NACK

j=j+1

t=t+1

end if

if UE has obtained HARQ-ACK information for TB t for HARQ process number h on serving cell c corresponding to a PDSCH reception and has not reported the HARQ-ACK information corresponding to the PDSCH reception

if harq-ACK-SpatialBundlingPUCCH is not provided

= HARQ-ACK information bit for TB t for HARQ process h of serving cell c

else

= binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks for HARQ process h of serving cell c. If the UE receives one transport block, the UE assumes ACK for the second transport block

end if

j=j+1

t=t+1

elseif UE has not obtained HARQ-ACK information for TB t for HARQ process number h on serving cell c

= NACK

j=j+1

t=t+1

end if

end while

end if

t=0

end if

h=h+1

end while

h=0

c=c+1

end while

--------------------------------------------End of TP 1 38.213 V16.4.0 section 9.1.4------------------------------------------

Conclusions

In this contribution, we show our views on the remaining issue on type-3 HARQ-ACK codebook. The text proposal is provided in section 2, and the proposal includes:

Proposal 1: Adopt TP1 for the generation of type-3 HARQ-ACK codebook.

- If the UE has not obtained HARQ-ACK information for a given HARQ process, NACK should be feedback for the given HARQ process.