3GPP TSG RAN WG1 #104-e										          	R1-2100187

e-Meeting, Jan. 25th – Feb. 5th, 2021

Source:			OPPO

Title:			Discussion on multi-cell PDSCH scheduling via a single DCI

Agenda Item:	8.13.2

Document for:	Discussion and Decision

Introduction

In RAN1 103 e-meeting, some agreements on simulation assumptions for both link-level simulation and system-level simulation were achieved:

Agreements:

Further study multi-cell PDSCH scheduling via a single DCI with below simulation assumptions:

Table 1: Link level simulation assumptions

| Parameters | Values |
| --- | --- |
| Carrier frequency | Option 1: Inter-band CA (700MHz + 4GHz) Intra-band CA (2GHz) Option 2: Only 4GHz is considered |
| SCS | 15 kHz for 700MHz/2GHz 30 kHz for 4GHz |
| Bandwidth | Option 1: Baseline: PCell 10MHz + SCell 10/40MHz Optional: PCell 20MHz + SCell 20/40/100MHz Option 2: Baseline: Scheduling cell 100 MHz Optional: Scheduling cell 20 MHz |
| Channel model | TDL-C |
| Delay spread | 300 ns |
| Number of symbols for CORESET | [1], 2 or 3 |
| CORESET BW (contiguous PRB allocation) | 24/48/96 RBs depending on the bandwidth |
| CCE-to-REG mapping | Interleaved, [non-interleaved] |
| REG bundle size | 6 |
| Interleaver size | 2 |
| DCI payload size (excluding CRC) | Single PDSCH scheduling: 60 bits as baseline payload size Multi-cell PDSCH scheduling: 72/84/96/104 bits |
| BLER target for multi-cell scheduling DCI | Option 1: 1% Option 2: 0.5% |
| Number of BS antennas | 2 Tx for 700MHz/2GHz carrier frequency 4 Tx for 4GHz |
| Number of UE antennas | 2 Rx for 700MHz/2GHz carrier frequency 4 Rx for 4GHz carrier frequency |
| Modulation | QPSK |
| Channel coding | Polar code |
| UE speed | 3km/h |
| Aggregation level | 1/2/4/8/16 |
| Tx Diversity | One port precoder cycling |

Note 1: For two-cell scheduling via a single DCI, PDCCH transmitted on SCell schedules one PDSCH on the SCell and another PDSCH on PCell.

Note 2: For comparison, for single-cell scheduling, one PDCCH transmitted on SCell schedules one PDSCH on the SCell via self-scheduling and another PDCCH transmitted on the SCell schedules another PDSCH on PCell via cross-carrier scheduling.

Further discussion which rows are applicable to the scheduling cell/the scheduled cell for PDCCH

Agreements:

Further study with below simulation assumptions:

Simulation scenarios:

- For two-cell scheduling via a single DCI, PDCCH transmitted on a first cell schedules one PDSCH on the first cell and another PDSCH on a second cell.

- For single-cell scheduling (baseline), one PDCCH transmitted on a first cell schedules one PDSCH on the first cell via self-scheduling and another PDCCH transmitted on the first cell schedules another PDSCH on a second cell via cross-carrier scheduling.

  - Companies can optionally compare to the case of PDCCH transmitted on each of the two cells via self-scheduling. In this case, company should provide details on how to calculate the PDCCH blocking rate.

Simulation assumptions on carrier frequency, SCS, antenna configuration, carrier bandwidth as well as CORESET configuration

- Combination 1: 2 GHz, 15 kHz SCS, 2 Tx, 2 Rx, 20 MHz carrier BW, 2-symbol CORESET with 96RBs

- Combination 2: 4 GHz, 30 kHz SCS, 4 Tx, 4 Rx, 100 MHz carrier BW, 1-symbol CORESET with 270RBs

- [Combination 3: 700MHz, 15 kHz SCS, 2 Tx, 2 Rx, 10 MHz carrier BW, 3-symbol CORESET with 48RBs]

- [Combination 4: 4GHz, 30 kHz SCS, 4 Tx, 4 Rx, 40 MHz carrier BW, 2-symbol CORESET with 96RBs]

Payload size of two-cell scheduling DCI (excluding CRC):

- 60 for single-cell scheduling DCI (baseline).

- 72/84/96/108 for two-cell scheduling DCI.

- Companies are encouraged to report how the values are obtained, e.g., via separate or shared fields in DCI format.

Target BLER for two-cell scheduling DCI: 1% (baseline), 0.5%(optional)

Regarding the CCE-to-REG mapping, based on the agreed interleaved CCE-to-REG mapping, whether to adopt non-interleaved CCE-to-REG mapping is up to the proponent.

Agreements:

- Further study with below simulation assumptions:

Table 2: System level simulation assumptions

| Parameters | Values |
| --- | --- |
| Carrier frequency | For scheduling cell, follow agreed link level simulation assumptions For scheduled cell, consider 700MHz/2GHz with 10/20MHz BW (LTE overhead on DSS carrier can be optionally provided, up to proponent) |
| SCS |  |
| Simulation bandwidth |  |
| BS antenna height | 25 m |
| UE height | 1.5m |
| TRP transmit power | 46 dBm for 10MHz |
| Scenario | Urban Macro |
| ISD | 500m |
| TRP antenna configuration | (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1;1,1) for 700MHz (M,N,P,Mg,Ng;Mp,Np)= (2,8,2,1,1;1,1) for 2GHz (M,N,P,Mg,Ng;Mp,Np)= (8,4,2,1,1;1,1) for 4GHz |
| UE antenna configuration | (M,N,P,Mg,Ng;Mp,Np)= (1,1,2,1,1;1,1) for 700MHz/2GHz (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1;1,1) for 4GHz |
| Device deployment | 80% indoor, 20% outdoor |
| UE speeds of interest | Indoor users: 3km/h |
|  | Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB |
| BS antenna element gain | 8 dBi |
| UE noise figure | 9 dB |
| Thermal noise level | -174 dBm/Hz |
| Traffic | Full Buffer(baseline), FTP model 1 or 3 up to company |
| Macro sites | 19 |
| Number of UEs per cell | 10/15/20 UEs |
| Downtilt | 102° |
| Minimum BS to UE distance | 35m |

In this contribution, we evaluate and analyze performance of single DCI scheduling PDSCH on two cells.

Discussion

Carrier aggregation provides larger bandwidth and more transmission opportunities. Correspondingly, PDCCH overhead will increase. Especially for cross-carrier scheduling, PDCCH overhead in one scheduling carrier will be double even more. However, PDCCH resource is limited, especially for dynamic spectrum sharing (DSS), bandwidth for PDCCH resource is relatively narrow. PDCCH blockage may happen. One-to-two scheduling is one solution to reduce PDCCH overhead. One-to-two scheduling, to be specifically, one DCI schedules two PDSCHs in two carriers, is shown in Figure 1. To minimize DCI payload in one-to-two scheduling, some DCI fields for two PDSCHs will be shared and correspondingly, e.g. frequency domain resource, time domain resource, PUCCH resource and so on.

Figure 1 NR scheduling schemes

For one-to-two scheduling, On the one hand, saved PDCCH resource can be utilized for data transmission, on the other hand, it can reduce PDCCH blockage. So, we evaluate one-to-two scheduling from perspective of CCE saving and PDCCH blockage.

CCE saving

For one-to-two scheduling, larger DCI size is benefit for more flexible scheduling. However, larger DCI size requires larger aggregation level to meet reliability requirement, which will neutralize the gain from DCI number decrease. Therefore, a new DCI design is the trade-off between the flexibility of scheduling and PDCCH overhead reduction. Table 1-3 show demodulation SNR budget and distribution proportion for different DCI sizes.

Table 1 SNR budget for different DCI sizes (Target BLER=1%) for combination 1

| DCI size (including 24bit CRC) | AL1 |  | AL2 |  | AL4 |  | AL8 |  | AL16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR (dB) | Ratio | SNR (dB) | Ratio | SNR (dB) | Ratio | SNR (dB) | Ratio | SNR (dB) | Ratio |
| 84 | 11.6 | 0.22 | 3.7 | 0.27 | -0.6 | 0.29 | -3.95 | 0.14 | -7 | 0.08 |
| 96 | 13.3 | 0.18 | 5 | 0.25 | 0.44 | 0.27 | -3.5 | 0.20 | -6.625 | 0.10 |
| 108 | 16.5 | 0.11 | 6 | 0.27 | 0.857 | 0.29 | -2.91 | 0.21 | -6.25 | 0.12 |
| 120 |  |  | 7 | 0.35 | 1.138 | 0.30 | -2.4 | 0.21 | -5.7 | 0.14 |
| 132 |  |  | 7.5 | 0.34 | 2.1 | 0.24 | -2 | 0.26 | -5.375 | 0.16 |

Table 2 SNR budget for different DCI sizes (Target BLER=1%) for combination 2 and 4

| DCI size (including 24bit CRC) | AL1 |  | AL2 |  | AL4 |  | AL8 |  | AL16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR (dB) | Ratio | SNR (dB) | Ratio | SNR (dB) | Ratio | SNR (dB) | Ratio | SNR (dB) | Ratio |
| 84 | 4.55 | 0.45 | -1.10 | 0.35 | -4.87 | 0.14 | -8.00 | 0.04 | -10.41 | 0.02 |
| 96 | 6.00 | 0.39 | -0.28 | 0.37 | -4.45 | 0.18 | -7.40 | 0.04 | -10.00 | 0.02 |
| 108 | 8.30 | 0.32 | 0.10 | 0.41 | -3.93 | 0.19 | -7.10 | 0.05 | -9.62 | 0.03 |
| 120 |  |  | 0.98 | 0.66 | -3.50 | 0.24 | -6.65 | 0.07 | -9.20 | 0.03 |
| 132 |  |  | 1.45 | 0.63 | -3.00 | 0.25 | -6.20 | 0.09 | -8.95 | 0.03 |

Table 3 SNR budget for different DCI sizes (Target BLER=1%) for combination 3

| DCI size (including 24bit CRC) | AL1 |  | AL2 |  | AL4 |  | AL8 |  | AL16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR (dB) | Ratio | SNR (dB) | Ratio | SNR (dB) | Ratio | SNR (dB) | Ratio | SNR (dB) | Ratio |
| 84 | 11.67 | 0.24 | 3.75 | 0.30 | -0.45 | 0.29 | -4.02 | 0.14 | -7.01 | 0.03 |
| 96 | 13.3 | 0.21 | 5 | 0.27 | 0.44 | 0.28 | -3.5 | 0.20 | -6.6 | 0.04 |
| 108 | 16.5 | 0.15 | 5.9 | 0.28 | 0.86 | 0.30 | -2.9 | 0.21 | -6.32 | 0.05 |
| 120 |  |  | 6.9 | 0.40 | 1.42 | 0.28 | -2.4 | 0.25 | -5.75 | 0.07 |
| 132 |  |  | 7.5 | 0.37 | 2.1 | 0.27 | -2 | 0.28 | -5.37 | 0.08 |

As showed in Table 1-3, DCI size increase leads aggregation level increase. Take table 2 as an example, for new DCI size=96, smaller proportion of UEs’ aggregation level increases. It means that most UE could save PDCCH overhead by one-to-two scheduling. For new DCI size=132, around 69% UE’s aggregation level increases. It means that around 31% UE could save PDCCH overhead by one-to-two scheduling. So significant PDCCH overhead reduction can be expected for any DCI size.

Table 4-6 shows CCE saving ratio for different DCI sizes, CA ratio and combination scenarios. CCE saving ratio is calculated by the following formula:

CCE saving ratio=1-CA ratio*AL=1AL=16RatioAL(DCI size= i)*CCE overhead(AL)+1-CA ratio*AL=1AL=16RatioALDCI size=84*CCE overheadALCA ratio*2*AL=1AL=16RatioALDCI size=84*CCE overheadAL+1-CA ratio*AL=1AL=16RatioALDCI size=84*CCE overheadAL,

i=96,108,120,132

CA ratio is CA UE percentage. For CA UE, two DCIs are assumed as baseline and one new DCI is assumed for one-to-two scheduling. For nonCA UE, a DCI is always assumed. For combination 2 and 4, the CCE saving ratio are the same because the same carrier and antenna configuration are assumed in LLS evaluation.

Table 4 CCE saving ratio for combination 1

| CA ratio | New DCI size 96bits | New DCI size 108bits | New DCI size 120bits | New DCI size 132bits |
| --- | --- | --- | --- | --- |
| 0% | 0.00% | 0.00% | 0.00% | 0.00% |
| 10% | 7.87% | 6.98% | 6.07% | 5.20% |
| 20% | 14.43% | 12.80% | 11.13% | 9.53% |
| 30% | 19.99% | 17.72% | 15.41% | 13.19% |
| 40% | 24.75% | 21.94% | 19.07% | 16.34% |
| 50% | 28.87% | 25.60% | 22.25% | 19.06% |
| 60% | 32.48% | 28.80% | 25.04% | 21.44% |
| 70% | 35.66% | 31.63% | 27.49% | 23.54% |
| 80% | 38.49% | 34.14% | 29.67% | 25.41% |
| 90% | 41.02% | 36.38% | 31.62% | 27.08% |
| 100% | 43.30% | 38.40% | 33.38% | 28.59% |

Table 5 CCE saving ratio for combination 2/4

| CA ratio | New DCI size 96bits | New DCI size 108bits | New DCI size 120bits | New DCI size 132bits |
| --- | --- | --- | --- | --- |
| 0% | 0.00% | 0.00% | 0.00% | 0.00% |
| 10% | 8.24% | 7.42% | 5.39% | 4.85% |
| 20% | 15.10% | 13.61% | 9.89% | 8.89% |
| 30% | 20.91% | 18.84% | 13.69% | 12.31% |
| 40% | 25.89% | 23.32% | 16.95% | 15.24% |
| 50% | 30.20% | 27.21% | 19.77% | 17.78% |
| 60% | 33.98% | 30.61% | 22.24% | 20.01% |
| 70% | 37.31% | 33.61% | 24.42% | 21.97% |
| 80% | 40.27% | 36.28% | 26.36% | 23.71% |
| 90% | 42.92% | 38.67% | 28.10% | 25.27% |
| 100% | 45.31% | 40.82% | 29.66% | 26.68% |

Table 6 CCE saving ratio for combination 3

| CA ratio | New DCI size 96bits | New DCI size 108bits | New DCI size 120bits | New DCI size 132bits |
| --- | --- | --- | --- | --- |
| 0% | 0.00% | 0.00% | 0.00% | 0.00% |
| 10% | 7.84% | 6.89% | 5.44% | 4.66% |
| 20% | 14.38% | 12.64% | 9.97% | 8.55% |
| 30% | 19.90% | 17.50% | 13.80% | 11.84% |
| 40% | 24.64% | 21.67% | 17.09% | 14.65% |
| 50% | 28.75% | 25.28% | 19.94% | 17.10% |
| 60% | 32.34% | 28.44% | 22.43% | 19.23% |
| 70% | 35.52% | 31.23% | 24.63% | 21.12% |
| 80% | 38.33% | 33.71% | 26.58% | 22.80% |
| 90% | 40.86% | 35.92% | 28.33% | 24.30% |
| 100% | 43.13% | 37.92% | 29.90% | 25.64% |

From Table 4 to Table 6, we could see that

- As CA ratio increases, CCE saving ratio increases. For CA ratio=10%, CCE saving ratio is from 5% to 8% for different DCI size. For CA ratio =30%, CCE saving ratio is from 11% to 20% for different DCI sizes. For CA ratio =100%, CCE saving ratio is from 25% to 45% for different DCI sizes.

- For different combination scenarios, there is no significant difference in CCE saving ratio.

Observation 1: CCE saving ratio is more than 10% for any DCI size even CA ratio is not large, e.g. CA ratio=30%. And for different combination scenarios, there is no significant difference in CCE saving ratio.

PDCCH blockage

To evaluate PDCCH blockage, simplified SLS schedule model is applied:

- Based on "CA ratio" assumption, the number of UEs receiving single-cell scheduling (N1) and number of UEs receiving two-cell scheduling (N2) are determined.

- For each slot trial:

-- For each of N UEs (N=N1+N2), the simulation randomly chooses the AL based on the probabilities deduced by LLS result and the assumed DCI size.

-- Allocate CCE resources for each of N UE.

-- The number of DCIs that cannot be placed in resource plane is counted as blockage events and recorded. The number of corresponding dropped PDSCHs is also collected.

For one-to-two scheduling, one PDCCH blockage leads two PDSCH drops. So blocked PDCCH and dropped PDSCH are counted as metric 1 and metric 2. Blockage metric 1 and metric 2 is defined as:

Blockage metric 1 on PDCCH=total number of blocked PDCCHtotal number of PDCCH generated from traffic

Blockage metric 2 on PDSCH=total number of dropped PDSCHtotal number of PDSCH generated from traffic

Figure 2-5 shows PDCCH blockage metric 1 and metric 2 for different scenarios.

Figure 2 PDCCH blockage for combination 1

Figure 3 PDCCH blockage for combination 2

Figure 4 PDCCH blockage for combination 3

Figure 5 PDCCH blockage for combination 4

From figure 2 to figure 5, we could see that:

- As CA ratio increases, PDCCH blockage from perspective of PDCCH and PDSCH reduces significantly.

  - In combination 2, PDCCH blockage is reduced from 7% to 1-2%.

  - In combinations other than 2, PDCCH blockage reduction due to one-to-two scheduling is more than 10% for middle and higher CA ratio.

- The less PDCCH capacity, the more gain from one-to-two scheduling

Observation 2: One-to-two scheduling can reduce PDCCH blockage significantly.

When PDCCH resource is enough, CCE saving can be achieved by one-to-two scheduling. For agreed simulation assumption, in other words, typical scenarios, PDCCH blockage reduction due to one-to-two scheduling is significant.

Proposal 1: Considering performance from CCE saving ratio and PDCCH blockage reduction, One-to-two scheduling should be supported.

Conclusions

In this contribution, we show our views on multi-cell PDSCH scheduling via a single DCI enhancement with following observations:

Observation 1: CCE saving ratio is more than 10% for any DCI size even CA ratio is not large, e.g. CA ratio=30%. And for different combination scenarios, there is no significant difference in CCE saving ratio.

Observation 2: One-to-two scheduling can reduce PDCCH blockage significantly.

Proposal 1: Considering performance from CCE saving ratio and PDCCH blockage reduction, One-to-two scheduling should be supported.

Appendix

Table A-1 Link-level simulation scenario

| parameters | assumption |
| --- | --- |
| DCI Size (including 24bits CRC) | 84/96/108/120/132 |
| channel model | TDL-C DS=300ns |
| Transmission type | Interleaved with size=2 |
| bundle size | 6 |
| modulation | QPSK |
| channel coding | polar code |
| UE speed | 3km/h |
| Tx Diversity | One port precoder cycling |

References

- RP-193260 New WID on NR Dynamic spectrum sharing (DSS)

- R1-2006064 Multi-cell PDSCH scheduling via a single DCI

- R1-2007442 Summary of NR Dynamic spectrum sharing (DSS) in Email discussion [102-e-NR-DSS-DC_enh2-01]