3GPP TSG RAN WG1 #104-e	R1-2100325

# e-Meeting, January 25th – February 5th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on HARQ-ACK transmission in PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, NR_L1enh_URLLC-Core, LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For a type-2 HARQ-ACK codebook transmission in a PUSCH scheduled by DCI format 0_1, is used to indicate the total number of PDSCHs associated with PDCCH and PDCCH indicating SPS PDSCH release. In addition, could be used to indicate that there is no HARQ-ACK for PDSCHs associated with PDCCH and PDCCH indicating SPS PDSCH release in the PUSCH. To be more specific, it is specified in current specification that when , if UE does not receive any PDCCH scheduling PDSCH or indicating SPS PDSCH release and UE does not have HARQ-ACK for SPS PDSCH(s) to be transmitted in the PUSCH, UE does not multiplex HARQ-ACK in the PUSCH. However, when is used to indicate that there is no HARQ-ACK for PDSCHs associated with PDCCH and PDCCH indicating SPS PDSCH release in the PUSCH, but UE has HARQ-ACK for SPS PDSCH(s) to be transmitted in the PUSCH, UE would feed back 4+A bits HARQ-ACK (4 bits NACK for PDSCHs associated with PDCCH and PDCCH indicating SPS PDSCH release, and A bit(s) HARQ-ACK for quantity of A SPS PDSCH(s)) in the PUSCH according to current specification. This is not aligned with the behavior of HARQ-ACK transmission in PUCCH or in PUSCH not scheduled by a DCI format or scheduled by DCI format without UL DAI, since in the same case, HARQ-ACK codebook is determined as described in 9.1.3.1 based on DL DAI and the quantity of SPS PDSCH(s) which results in only A bit(s) HARQ-ACK for quantity of A SPS PDSCH(s) (for PDSCH associated with PDCCH and PDCCH indicating SPS PDSCH release is zero). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the UE behavior for only feeding back HARQ-ACK of SPS PDSCH(s) in a PUSCH when and there is only HARQ-ACK for SPS PDSCH(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When there is only HARQ-ACK for SPS PDSCH(s), there is no way for a UE to only feed back HARQ-ACK for SPS PDSCH(s) in the PUSCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has isolated impact on HARQ-ACK feedback in PUSCH scheduled by DCI format with and only SPS PDSCH reception(s). If the network is implemented according to the CR and the UE is not, or the UE is implemented according to this CR but the network is not, there may be different understandings between gNB and UE for the number of HARQ-ACK bits in PUSCH scheduled by DCI format with UL DAI when there is only HARQ-ACK for SPS PDSCH(s) in the PUSCH. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.1.3.2	Type-2 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DAI field, then

-	if the UE has not received any PDCCH within the monitoring occasions for DCI formats scheduling PDSCH receptions, or SPS PDSCH release, or DCI format 1_1 indicating SCell dormancy on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception, or in response to a detection of a DCI format 1_1 indicating SCell dormancy, to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission;

-	else, the UE generates the HARQ-ACK codebook as described in Clause 9.1.3.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH.

If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by a DCI format that includes a DAI field, the UE generates the HARQ-ACK codebook as described in Clause 9.1.3.1, with the following modifications:

-	For the pseudo-code for the HARQ-ACK codebook generation in Clause 9.1.3.1, after the completion of the c and m loops, the UE sets Vtemp2=VDAIUL where VDAIUL is the value of the DAI field according to Table 9.1.3-2

-	For the case of first and second HARQ-ACK sub-codebooks, the DCI format includes a first DAI field corresponding to the first HARQ-ACK sub-codebook and a second DAI field corresponding to the second HARQ-ACK sub-codebook

-	harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH.

If a UE is not provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format that includes a DAI field with value VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions or SPS PDSCH release or indicating SCell dormancy on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission.

If a UE is not provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format that includes a DAI field with value VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions or SPS PDSCH release or indicating SCell dormancy on any serving cell c and the UE has HARQ-ACK information in response to one or multiple activated SPS PDSCH receptions to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE multiplexes one HARQ-ACK information bit in response to one activated SPS PDSCH reception or HARQ-ACK information as described in Clause 9.1.2 in response to multiple activated SPS PDSCH receptions in the PUSCH transmission.

If a UE is provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format that includes a DAI field with first value VT-DAIUL=4 or with second value VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions or SPS PDSCH release, or DCI format 1_1 indicating SCell dormancy, on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information for the first sub-codebook or for the second sub-codebook, respectively, in the PUSCH transmission.

If a UE is provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format that includes a DAI field with first value VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions or SPS PDSCH release, or DCI format 1_1 indicating SCell dormancy, on any serving cell c for the first sub-codebook and the UE has HARQ-ACK information in response to one or multiple activated SPS PDSCH receptions to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE multiplexes one HARQ-ACK information bit in response to one activated SPS PDSCH reception or HARQ-ACK information as described in Clause 9.1.2 in response to multiple activated SPS PDSCH receptions for the first sub-codebook in the PUSCH transmission.

Table 9.1.3-2: Value of DAI

| DAI MSB, LSB | VT-DAIDL | Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH indicating SPS PDSCH release or DCI format 1_1 indicating SCell dormancy is present, denoted as X and X≥1 |
| --- | --- | --- |
| 0,0 | 1 | X-1mod4+1=1 |
| 0,1 | 2 | X-1mod4+1=2 |
| 1,0 | 3 | X-1mod4+1=3 |
| 1,1 | 4 | X-1mod4+1=4 |