3GPP TSG RAN WG1 #104-e		R1-2100350

e-Meeting, January 25th – February 5th, 2021

Source:	CATT

Title:	Evaluation of enhancements for multi-TRP

Agenda Item:	8.1.5

Document for:	Discussion and Decision

Introduction

HST-SFN deployment scenario is one of the most important scenarios with multiple TRPs. In order to have better performance in HST-SFN deployment scenario, the following objective was approved in the WID for Rel-17 NR FeMIMO [1]:

Enhancement to support HST-SFN deployment scenario: i. Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission ii. Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

In RAN1 #102 e-meeting, the following agreements on evaluation methodology of Rel-17 HST-SFN in FR1 were achieved:

Agreement The results should be reported Th - Per track location (at specific SNR) or - Throughput vs SNR at specific location - Ds/2 (mid track point) - Results for other locations can be reported by each company. Agreement - Perfect synchronization as baseline - Non-perfect time and frequency synchronization between the TRPs and UE, i.e., modeling of TRP CFO error (where CFO have temporal variation), UE receiver CFO, TRP timing errors may be optionally considered - Companies to provide details on how this was modelled in their evaluations - For example, uniform distribution between [-ppm ppm]*fc (Hz) for each simulation point where fc is the carrier center frequency and the values of maximum frequency error in ppm are captured TR 38.101-1/2 and TR 38.104 Agreement - It is recommended to provide results for SNR = 8, 12, 16, 20 dB - Other SNR values are not precluded - SNR defined relative ONLY to the reference point closest to TRP

In RAN1 #102 e-meeting, the following agreements on enhancements of Rel-17 HST-SFN were achieved:

Agreement For the discussion purpose consider the following categorization of the enhanced DL transmission schemes - Scheme 1: - TRS is transmitted in TRP-specific / non-SFN manner - DM-RS and PDCCH/PDSCH from TRPs are transmitted in SFN manner - Scheme 2: - TRS and DM-RS are transmitted in TRP-specific / non-SFN manner - PDSCH from TRPs is transmitted in SFN manner Agreement For discussion purpose consider the following three steps for TRP-based frequency offset pre-compensation scheme: - 1st step: Transmission of the TRS resource(s) from TRP(s) without pre-compensation - 2nd step: Transmission of the uplink signal(s)/channel(s) with carrier frequency determined based on the received TRS signals in the 1st step - 3rd step: Transmission of the PDCCH/PDSCH from TRP(s) with frequency offset pre-compensation determined based on the received signal/channel in the 2nd step - Note: A second set of TRS resource(s) may be transmitted at 3rd step.

In RAN1 #103 e-meeting, the following agreements on enhancements of Rel-17 HST-SFN were achieved:

Agreement Support at least the following configuration for HST scenario in Rel-17 - The same DMRS port(s) can associate with multiple TCI states - FFS other details Note: DMRS and PDCCH/PDSCH from different TRPs are transmitted in SFN manner Agreement When the same DMRS port(s) are associated with two TCI states containing TRS as source reference signal, at least one variant is supported for Rel-17 HST-SFN scenario based on further evaluations - Variant A: One of the TCI state can be associated with {average delay, delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - Variant B: One of the TCI state can be associated with {average delay, delay spread} and another TCI state with {Doppler shift, Doppler spread} (i.e., QCL-TypeB) - Variant C: One of the TCI state can be associated with {delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - Variant E: Both TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - FFS: Indication method to apply QCL, e.g., via new QCL-type, or reuse existing QCL-type while UE to ignore certain QCL properties - Note: Each TCI state in the above variants may be additionally associated with {Spatial Rx parameter} (i.e., QCL-TypeD) - Note: Companies are encouraged to provide evaluation results for the above variants based on agreed EVM from RAN1#102e meeting - Note: Above variants are applicable to scheme 1 and/or TRP based pre-compensation as a reference for evaluation. - This agreement is for the purpose of evaluation and does not imply the support or lack of support of scheme 1 and/or TRP based pre-compensation

In this contribution, we present our simulation results on transmission schemes for HST-SFN scenario.

Evaluation on schemes for HST-SFN deployment

In this section, we provide link-level simulation results for PDSCH with 4-taps channel model in FR1 according to the simulation assumptions of HST-SFN agreed in RAN1 #102-e meeting. Fig. 1 shows the HST-SFN deployment with 4 taps. In the simulation, we assume that the train is moving from TRP 2 to TRP 3 at a speed of 500km/h, PDSCH is SFN-ed transmitted from 4 TRPs with PRG-level precoding cycling in a carrier frequency of 3.5GHz, with numerology of 30kHz and band width of 20MHz. We also assume that there are 2 Rx antennas in UE and 2 Tx antennas in each TRP. Other simulation assumptions can be found in Table-I in appendix. The following evaluation results for different MCS levels are provided:

- Throughput at different positions between TRP 2 and TRP 3

- Throughput vs. SNR for the location that in the middle of the 4 TRPs

Figure 1 HST-SFN deployment

2.1	DL Transmission schemes

In this section, the performance comparisons of enhanced DL transmission scheme 1 and scheme 2 that proposed in RAN 1 #102 e-meeting is provided. The simulation results for Rel-15 SFN-ed transmission scheme that all of PDSCH, DMRS and TRS are transmitted in SFN-ed manner are also provided. Details of DL transmission scheme 1 and DL transmission scheme 2 are as follows:

  - DL transmission scheme 1: PDSCH and DMRS are SFN-ed transmitted from 4TRPs. TRS 1 is SFN-ed transmitted from TRP 1 and TRP 2, and TRS 2 is SFN-ed transmitted from TRP 3 and TRP 4.  Both TRS 1 and TRS 2 are used to estimate the Doppler profile and delay profile of the channel that convey the DMRS port.

  - DL transmission scheme 2: PDSCH is SFN-ed transmitted from 4 TRPs. A PDSCH layer is mapped to 2 DMRS ports in 2 CDM groups, where DMRS port 0 is SFN-ed transmitted from TRP 1 and TRP 2, DMRS port 1 is SFN-ed transmitted from TRP 3 and TRP 4. In order to have better frequency estimation and timing, TRS 1 is SFN-ed transmitted from TRP 1 and TRP 2, and TRS 2 is SFN-ed transmitted from TRP 3 and TRP 4. Each TRS is used to estimate the Doppler profile and delay profile for its corresponding DMRS port, and the channel estimation of the PDSCH layer is obtained through the 2 DMRS ports.

Fig. 2 to Fig. 4 show the performance comparisons of enhancement scheme 1 and enhancement scheme 2 with different MCS levels. It can be found that both scheme 1 and scheme 2 can achieve obvious gains over Rel-15 SFN-ed transmission scheme. That's because the Doppler profile and delay profile of PDSCH can be obtained more accurately through multiple TRS resources. It also can be seen that the performance of scheme 1 outperforms scheme 2. That’s because although scheme 2 may obtain more accurate channel estimation by utilizing more DMRS ports, the overhead of DMRS of scheme 2 is doubled, and it has more negative impact on the performance.

(a) Evaluation of different positions along the track                  (b) Evaluation of the position in the middle of 4 TRPs

Figure 2 Performance of enhanced DL Transmission schemes, MCS 4, Rank 1

(a) Evaluation of different positions along the track              (b) Evaluation of the position in the middle of 4 TRPs

Figure 3 Performance of enhanced DL Transmission schemes, MCS 13, Rank 1

- Evaluation of different positions along the track                  (b) Evaluation of the position in the middle of 4 TRPs

Figure 4 Performance of enhanced DL Transmission schemes, MCS 17, Rank 1

Observation 1:

- Compared with Rel-15 SFN transmission scheme, obvious performance gain can be observed with DL enhancement scheme 1 and DL enhancement scheme 2.

- DL enhancement scheme 1 outperforms DL enhancement scheme 2.

Proposal 1:

- DL enhancement scheme 2 is not supported.

2.2 Enhancements on Doppler shift pre-compensation

In this section, the performance of the following frequency offset pre-compensation schemes is provided. The detailed procedure of the schemes is provided in [2]:

- Pre-compensation scheme 1: TRS resource(s) are transmitted with frequency offset pre-compensation.

  - Pre-compensation scheme 1-1: PDSCH/DMRS/TRS are transmitted from 4 TRPs in SFN manner, with UE-specific frequency offset pre-compensated. For each UE, the pre-compensated frequency transmitted from each TRP is determined according to the SRS transmitted by the UE. This scheme can be seen as the Rel-15 SFN-ed transmission scheme +frequency offset pre-compensation.

  - Pre-compensation scheme 1-2: PDSCH/DMRS/TRS are transmitted from 4 TRPs in SFN manner, with UE-group common frequency offset pre-compensated. For each group of UEs, the pre-compensated frequency transmitted from each TRP is determined according to the SRS transmitted by a reference UE within the group. In this simulation, we assume that the distance between reference UE and the UE receiving PDSCH is 50m.

  - Pre-compensation scheme 1-3: PDSCH/DMRS are transmitted from 4 TRPs in SFN manner with UE-specific frequency pre-compensation. TRS 1 is transmitted from TRP 1 and TRP 2 in SFN manner, TRS 2 is transmitted from TRP 3 and TRP 4 in SFN manner. Both TRS 1 and TRS 2 are frequency pre-compensated with UE-specific frequency pre-compensation. For each UE, the pre-compensated frequency transmitted from each TRP is determined according to the SRS transmitted by the UE. This scheme can be viewed as the DL transmission scheme 1 in section 2.1 + UE-specific frequency offset pre-compensation.

  - Pre-compensation scheme 1-4: PDSCH/DMRS are transmitted from 4 TRPs in SFN manner, and TRS 1 with is transmitted from TRP 1 and TRP 2 in SFN manner, TRS 2 is transmitted from TRP 3 and TRP 4 in SFN manner. For each group of UEs, the pre-compensated frequency transmitted from each TRP is determined according to the SRS transmitted by a reference UE within the group. In this simulation, we assume that the distance between reference UE and the UE receiving PDSCH is 50m.

- Pre-compensation scheme 2: The TRS resource(s) are transmitted without frequency offset pre-compensation.

  - Pre-compensation scheme 2-1: PDSCH/DMRS are transmitted from 4 TRPs in SFN manner with UE-specific frequency pre-compensation in TRP 2, 3 and 4 and without frequency offset pre-compensation in TRP 1; TRS 1 without frequency pre-compensation is transmitted from TRP 1 and TRP 2 in SFN manner, TRS 2 without frequency pre-compensation is transmitted from 4 TRPs in SFN manner. TRS 1 is used for Doppler profile estimation and TRS 2 is used for delay profile estimation.

  - Pre-compensation scheme 2-2: PDSCH/DMRS are transmitted from multiple TRPs in SFN manner with UE specific frequency pre-compensation in TRP 2, 3 and 4 and without frequency offset pre-compensation in TRP 1; TRS 1 without frequency pre-compensation is transmitted from TRP 1 and TRP 2 in SFN manner, TRS 2 without frequency pre-compensation is transmitted from TRP 3 and TRP 4 in SFN manner. The Doppler profile of PDSCH is obtained by TRS 1 and the delay profile of PDSCH is obtained by TRS 1 and TRS 2.

Fig. 5 to Fig. 7 show performance comparisons of different schemes without CFO and without frequency estimation error in TRPs, where frequency estimation error in TRPs is used to characterize the error of the pre-compensated frequency. In each figure, figure (a) shows the performance comparisons of different schemes at different positions along the track, and figure (b) shows the performance comparisons of different schemes at the position in the middle of the 4 TRPs. It can be observed from the simulation results that

  - Compared with Rel-15 SFN-ed PDSCH transmission scheme, obvious performance gain can be achieved by the solutions with frequency offset pre-compensation;

  - The performance of the DL transmission scheme 1 in section 2.1 without frequency offset pre-compensation is similar as that of pre-compensation scheme 1-2 ( Rel-15 SFN-ed transmission + UE-group common frequency offset pre-compensation) and is worse than that of other frequency offset pre-compensation schemes (pre-compensation scheme 1-1, 1-3, 2-1, 2-2 and 1-4);

  - Similar performance can be achieved by pre-compensation schemes with PDSCH transmitted with UE-specific frequency offset pre-compensation schemes (pre-compensation scheme 1-1, 1-3, 2-1 and 2-2). That’s because that under the perfect frequency synchronization, when frequency estimation error in TRPs is not considered, through UE-specific frequency pre-compensation, the Doppler spread can be perfectly eliminated, and it is equivalent that there is only a single Doppler shift. Then the Doppler profile of the channel can be well estimated by a single TRS resource or by two TRS resources;

The performance of pre-compensation schemes with PDSCH transmitted with UE-specific frequency offset pre-compensated (pre-compensation scheme 1-1, 1-3, 2-1 and 2-2) outperforms the performance of pre-compensation schemes with PDSCH transmitted with UE-group common frequency offset pre-compensated (pre-compensation scheme 1-2 and 1-4). That’s because for scheme 1-2 and scheme 1-4, the frequency for pre-compensation in each TRP is determined by a reference UE, therefore the Doppler spread of the channel cannot be well eliminated;

(a) Evaluation of different positions along the track                   (b) Evaluation of the position in the middle of 4 TRPs

Figure 5 Performance comparisons of schemes, without CFO and frequency estimation error in TRPs, MCS 4

(a) Evaluation of different positions along the track                (b) Evaluation of the position in the middle of 4 TRPs

Figure 6 Performance comparisons of schemes, without CFO and frequency estimation error in TRPs, MCS 13

- Evaluation of different positions along the track                    (b) Evaluation of the position in the middle of 4 TRPs

Figure 7 Performance comparisons of schemes, without CFO and frequency estimation error in TRPs, MCS 17

With the above simulation results, we have the following observation:

Observation 2:

- Under perfect frequency synchronization, if frequency estimation error is not considered in TRPs,

  - Compared with Rel-15 SFN-ed transmission scheme and DL enhancement schemes without frequency pre-compensation, UE-specific frequency offset pre-compensation schemes can achieve notable gains.

  - The performance of schemes with UE-specific frequency offset pre-compensation outperforms the performance of schemes with UE-group common frequency offset pre-compensation.

  - For the schemes with UE-specific frequency offset pre-compensation, similar performance can be achieved by schemes with TRS resources without frequency offset pre-compensation and schemes with TRS resources with frequency offset pre-compensated.

Fig. 8 to Fig. 10 show evaluations of frequency offset pre-compensation schemes with a CFO of 0.05ppm and a Doppler estimation error of 0.05ppm in each TRP assumed. In each figure, figure (a) shows the performance comparisons of different schemes at different positions along the track, and figure (b) shows the performance comparisons of different schemes at the position in the middle of the 4 TRPs. When CFO and frequency estimation error is considered in each TRP, it can be observed from the simulation results that:

  - Compared with Rel-15 SFN-ed PDSCH transmission scheme, obvious performance gain can be achieved by the solutions with frequency offset pre-compensated;

  - The performance of the schemes of DL transmission scheme 1 in section 2.1 + frequency offset pre-compensation (pre-compensation scheme 1-3 and 1-4) outperforms the performance of the DL transmission scheme 1 in section 2.1 without frequency offset pre-compensation;

  - The performance of pre-compensation schemes with PDSCH transmitted with UE-specific frequency offset pre-compensation (pre-compensation scheme 1-1, 1-3, 2-1 and 2-2) outperforms the performance of pre-compensation schemes with PDSCH transmitted with UE-group common frequency offset pre-compensated and SFN-ed TRS transmission (pre-compensation scheme 1-2). That’s because although for scheme 1-1, 1-3, 2-1 and 2-2, the Doppler spread of the channel cannot be well eliminated due to CFO error and frequency estimation error in each TRP, the frequency pre-compensation of scheme 1-2 is even worse due to UE-group common frequency pre-compensation.

  - Similar performance can be achieved by pre-compensation scheme 1-1, 2-1 and 2-2, and their performance is worse than that of pre-compensation scheme 1-3 and 1-4. That’s because when CFO and frequency estimation error in each TRP is considered, the Doppler profile cannot be well eliminated for schemes with frequency offset pre-compensation. The performance of schemes with one TRS for Doppler profile estimation would be worse than the schemes with multiple TRS resources for Doppler profile estimation.

(a) Evaluation of different positions along the track                    (b) Evaluation of the position in the middle of 4 TRPs

Figure 8 Performance comparisons of schemes, with CFO = 0.05ppm and transmission frequency error = 0.05ppm in each TRP, MCS 4, rank 1

(a) Evaluation of different positions along the track                  (b) Evaluation of the position in the middle of 4 TRPs

Figure 9 Performance comparisons of schemes, with CFO = 0.05ppm and transmission frequency error = 0.05ppm in each TRP, MCS 13, rank 1

(a) Evaluation of different positions along the track               (b) Evaluation of the position in the middle of 4 TRPs

Figure 10 Performance comparisons of schemes, with CFO = 0.05ppm and transmission frequency error = 0.05ppm in each TRP, MCS 17, rank 1

With the simulation results with CFO and frequency estimation error in each TRP considered, we have the following observation:

Observation 3:

- When CFO and frequency estimation error is considered in each TRP,

  - Compared with Rel-15 SFN-ed transmission scheme without frequency pre-compensation, schemes with frequency offset pre-compensation schemes can achieve notable gains;

  - The performance of the DL transmission scheme 1 in section 2.1 with frequency offset pre-compensation outperforms the performance of the DL transmission scheme 1 in section 2.1 without frequency offset pre-compensation;

  - The performance of pre-compensation schemes with multiple TRS resources for Doppler shift estimation outperforms the performance of pre-compensation schemes with single TRS resource for Doppler shift estimation;

Conclusions

In this contribution, we provide our link simulation results for Rel-17 HST-SFN enhancement schemes. Based on the simulation results, we have the following observations and proposal:

Observation 1:

- Compared with Rel-15 SFN transmission scheme, obvious performance gain can be observed with DL enhancement scheme 1 and DL enhancement scheme 2.

- DL enhancement scheme 1 outperforms DL enhancement scheme 2.

Observation 2:

- Under perfect frequency synchronization, if frequency estimation error is not considered in TRPs,

  - Compared with Rel-15 SFN-ed transmission scheme and DL enhancement schemes without frequency pre-compensation, UE-specific frequency offset pre-compensation schemes can achieve notable gains.

  - The performance of schemes with UE-specific frequency offset pre-compensation outperforms the performance of schemes with UE-group common frequency offset pre-compensation.

  - For the schemes with UE-specific frequency offset pre-compensation, similar performance can be achieved by schemes with TRS resources without frequency offset pre-compensation and schemes with TRS resources with frequency offset pre-compensated.

Observation 3:

- When CFO and frequency estimation error is considered in each TRP,

  - Compared with Rel-15 SFN-ed transmission scheme without frequency pre-compensation, schemes with frequency offset pre-compensation schemes can achieve notable gains;

  - The performance of the DL transmission scheme 1 in section 2.1 with frequency offset pre-compensation outperforms the performance of the DL transmission scheme 1 in section 2.1 without frequency offset pre-compensation;

  - The performance of pre-compensation schemes with multiple TRS resources for Doppler shift estimation outperforms the performance of pre-compensation schemes with single TRS resource for Doppler shift estimation;

Proposal 1:

- DL enhancement scheme 2 is not supported.

References

[1] RP-193133, New WID: Further enhancements on MIMO for NR, RAN #86

[2] R1-2100347, On enhancements on HST-SFN deployment, CATT, 3GPP RAN 1 #104-e

Appendix

Table-I Link level simulation assumptions for HST-SFN

| Parameter | Value |
| --- | --- |
| Propagation condition | 4-tap channel model |
| TRP layout (Ds, Dmin, etc) | Ds=700m, Dmin=150m |
| Carrier frequency | 3.5 GHz |
| Speed | 500 km/h |
| BW | 20 MHz |
| SCS | 30 KHz |
| Number of RBs for PDSCH | 50 PRBs |
| Duplexing | TDD |
| TDD pattern | DDDDDDDSUU |
| gNB antenna configuration | 2Tx |
| UE antenna configuration | 2Tx |
| DMRS configuration | DM-RS type 1, 1+1+1 |
| TRS configuration | Periodic of 10ms, 2-slot pattern |
| MCS | MCS 13 based on 64QAM table |
| Rank | Rank 1 |