3GPP TSG RAN WG1 #104-e                                                                                    R1-2100392

e-Meeting, January 25th – February 5th, 2021

Source:              CATT

Title:                   Paging enhancement for UE power saving

Agenda Item:     8.7.1.1

Document for:   Discussion and Decision

Introduction

In RAN1#103 e-meeting, the following agreements were achieved[1]:

Agreements: Observation: For NR idle/inactive-mode UEs, UE sub-grouping indication within a PO can provide the following power saving gains w.r.t. Rel-16: - If the original group paging rate is 10%: - [0.3%] - [1.1%] where the baseline assumes 1 SS burst for synchronization before PO reception - [0.4%] - [0.8%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [0.3%] - [1.0%] where the baseline assumes 3 SS bursts for synchronization before PO reception - Some sources also evaluated performance if the original group paging rate is in the range between 20% and 80% and showed following results: - [0.7%] - [7.6%] where the baseline assumes 1 SS burst for synchronization before PO reception - [0.8%] - [3.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [0.5%] - [4.7%] where the baseline assumes 3 SS bursts for synchronization before PO reception The number of UE sub-groups evaluated ranges from 2 to 16. Some companies show concern on assuming group paging rate larger than 60%. Note: It is FFS in RAN1 another group paging rate > 10% for the evaluation of Rel-17 paging enhancement. Agreements: Observation: For NR idle/inactive-mode UEs, UE sub-grouping indication carried in paging early indication can provide the following power saving gains w.r.t Rel-16: - If the original group paging rate is 10%: - [10.6%] –[19.1%] where the baseline assumes 1 SS burst for synchronization before PO reception - [16.0%] –[36.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [14.3%] –[46.0%] where the baseline assumes 3 SS bursts for synchronization before PO reception - Some sources also evaluated performance if the original group paging rate is in the range between 20% and 60% and showed following results: - [8.0%] –[19.1%] where the baseline assumes 1 SS burst for synchronization before PO reception - [18.1%] –[34.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [20.6%] –[42.0%] where the baseline assumes 3 SS bursts for synchronization before PO reception The additional power saving gains w.r.t. paging early indication without UE sub-grouping are given as follows: - If the original group paging rate is 10%: - [0.6%] –[2.7%] where the baseline assumes 1 SS burst for synchronization before PO reception - [0.6%] –[4.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [0.6%] –[4.7%] where the baseline assumes 3 SS bursts for synchronization before PO reception - Some sources also evaluated performance if the original group paging rate is in the range between 20% and 60% and showed following results: - [1.3%] –[8.0%] where the baseline assumes 1 SS burst for synchronization before PO reception - [2.1%] –[13.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [3.3%] –[16.1%] where the baseline assumes 3 SS bursts for synchronization before PO reception The number of UE sub-groups evaluated ranges from 2 to 16. The power saving gains are dependent on the assumptions about placement of PEI and PO relative to SSB. Note: It is FFS in RAN1 another group paging rate > 10% for the evaluation of Rel-17 paging enhancement. Note: Not all sources providing results for paging early indication without UE sub-grouping also provide results for paging early indication with UE sub-grouping. Agreements: Observation: For NR idle/inactive-mode UEs with 10% group paging rate, cross-slot scheduling with K0 = 1, which can be supported by Rel-15/Rel-16 for Type 2 CSS, can provide the following power saving gains w.r.t. same-slot scheduling (K0 = 0): - [<1%] –[2.5%] where the baseline assumes 1 SS burst for synchronization before PO reception - [<1%] -[1.6%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [<1%] -[1.44%] where the baseline assumes 3 SS bursts for synchronization before PO reception One source shows that cross-slot scheduling with K0 = 32, which cannot be supported by Rel-15/Rel-16 for Type 2 CSS, can provide the following power saving gains w.r.t. same-slot scheduling (K0 = 0): - [0%] where the baseline assumes 1 SS burst for synchronization before PO reception - [6.3%] where the baseline assumes 3 SS bursts for synchronization before PO reception The power saving gain will become lower with higher group paging rate. Agreements: For NR idle/inactive-mode paging enhancement, paging early indication before paging occasion is supported from RAN1 perspective - FFS: Physical layer design based on DCI, SSS or TRS/CSI-RS - Send LS to inform RAN2 and kindly ask RAN2 to inform RAN1 if there is anything that RAN1 should take into consideration in the physical layer design for this feature, including any other progress RAN2 has made in this WI which may has RAN1 impact Agreements: Observation: For NR idle/inactive-mode UEs with 10% group paging rate, paging early indication without UE sub-grouping can achieve the following power saving gains w.r.t. Rel-16: - [0%] - [22.8%] where the baseline assumes 1 SS burst for synchronization before PO reception - Note: [0%] means UE can apply the baseline behavior if the time offset between the utilized SS burst and PO is small. - [5.0%] - [32.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [10.2%] - [67.7%] where the baseline assumes 3 SS bursts for synchronization before PO reception The power saving gains will become lower for higher group paging rate. The power saving gains are dependent on the assumptions about placement of PEI and PO relative to SSB. The power saving gains may vary with different paging early indication design.

In this contribution, we discuss the paging enhancement schemes. Next the PEI detection performance is provided. Power saving performance of sequence-based PEI and DCI-based PEI are evaluated respectively. The PEI procedure and configuration are analyzed.

Paging early indication

The paging early indication (PEI) can provide substantial power saving gain and is supported with agreement in RAN1#103 e-meeting. In this section, we further discuss the detailed designs of DCI-based PEI and sequence-based PEI.

The procedure of paging indication schemes in RRC_Idle/Inactive mode

The general procedure of paging reception in RRC_Idle/Inactive is shown as follow. Before UE is at a Paging Occasion (PO) as shown in Figure 1, UE needs to perform following steps:

1) Waking up at the time before PO for preparation and turning all components in preparation of data reception (warm up) - The preparation period is designed to accommodate all the processing requirements.

2) Timing acquisition from SSB and coarse synchronization – UE is out-of-sync from the network after long deep sleep.   The local oscillator at the UE is in free running mode without the calibration of frequency stability from DL received signals. UE uses the timing before deep sleep for cell search and timing acquisition.

3) Frequency and time tracking after coarse synchronization – After coarse synchronization, UE starts the front-end algorithm for frequency and time offset estimation.

4) Time and frequency offset compensation of receiving signals.

5) Calibration of local oscillator – the reference timing clock from local oscillator needs to be calibrated by the received signals with time and frequency offset compensated to correct the frequency drift of the local oscillator in achieving 0.1 ppm frequency stability requirements.

6) Demodulating/decoding the DCI from PDCCH for paging indication.

7) Demodulating/decoding PDSCH and retrieve the paging information.

8) If UE ID is included in the paging message, UE performs the subsequent processing, such as contention-based PRACH etc. Otherwise, UE goes back to sleep.

RRM measurement could be performed during SMTC window when UE wakes up to decode the paging message. Meanwhile, RRM measurement enhancement can be performed, such as increase the RRM measurement periodicity for no/low mobility UE based on Rel-16 optimization schemes.

Figure 1: Illustration of paging reception procedure in Rel-16

The related general processing timeline and power consumption is performed as following Table 1.

Table 1: Processing timeline

| UE operations in a paging cycle | Time duration (ms) | Energy contribution (relative power * slot) |
| --- | --- | --- |
| SS burst processing | 2 | PSSB * 4 |
| Light sleep | 18 | PLS * 36+ 100 Note1 |
| SS burst processing | 2 | PSSB * 4 |
| Light sleep | 18 | PLS * 36+ 100 Note1 |
| SS burst processing and intra-frequency RRM measurement | 2 | (PSSB + Pintra, search+meas *[1/4]Note2 + Pintra, search+meas *(1-[1/4]Note2))*0.85 Note3 * 4 |
| Light sleep | 8 | PLS * 16 + 100 |
| PO reception | 4 | (PPDCCH * 1 * (1 – RG Note4) + PPDCCH+PDSCH * 1 * RG Note4)*8 |
| Light sleep | 6 | PLS * 12 + 100 Note1 |
| Switch into another frequency layer | 0.5 | PMS * 1 |
| Inter-freq. RRM measurement | 5 | (PDS * (1-[1/4] Note2) + Pinter, search-only * [1/4]Note2)*10 |
| Switch back to serving frequency | 0.5 | PMS * 1 |
| Deep sleep | 1214 | PDS * 2428 + 450Note1 |
| (Total) | 1280 | Total Energy |
| Average power consumption = Total Energy /2560 (slot) |  |  |
| Note 1: Additional transition energy (relative power * ms) for light/deep sleep as specified in Table 19 of TR 38.840 Note 2: Cell search rate for intra/inter-frequency RRM measurement Note 3: Scaling convention to combine two different types of UE operations as in Section 8.1.3 of TR 38.840 Note 4: RG is the paging rate to the UE group |  |  |

PDCCH-based PEI:

TRS/CSI-RS configured for CONNECTED mode UE could be configured for UE in RRC_IDLE/Inactive state to perform time and frequency tracking. TRS/CSI-RS in RRC_IDLE/Inactive mode could be used to assist PDCCH-based paging indication for paging reception indication as well as reducing the false alarm. The procedure of TRS/CSI-RS assisted PDCCH-based paging indication is shown in Figure 2. In the procedure, one or more SSB and TRS/CSI-RS can be used to perform channel tracking and local oscillator calibration. PDCCH-based paging indication could indicate UE whether or not to monitor paging PDCCH in the subsequent PO. When UE receives PDCCH-based paging early indication, UE will be indicated to wake up and decode PDCCH/PDSCH carried paging message at next paging occasion. Otherwise, UE continues to sleep after PDCCH-based paging indication reception. The related procedure is shown as following figure.

Figure 2: Illustration of paging reception with TRS/CSI-RS assisted PDCCH-based paging indication

Sequence-based PEI:

Furthermore, the sequence-based paging indication is also considered as the candidate for reducing paging reception in achieving UE power saving as shown in Figure 3. In Figure 3, the sequence-based paging indication can not only be used for paging indication through non-coherent detection but also as the reference signals for channel tracking. The sequence-based paging indication can be combined with SSB to indicate UE whether to decode the subsequent PDSCH. If UE detects the paging early indication, UE would wake up to decode the paging.  Otherwise, UE continues sleeping and not to monitor the PDCCH and decode paging DCI. One SSB burst set is used RRM measurement and channel tracking. TRS is used to assist channel tracking and as Paging early indication. Its procedure is shown as following figure.

Figure 3: Illustration of paging reception with sequence-based PEI and channel tracking

According to the descriptions of two paging indication schemes, TRS/CSI-RS assisted PDCCH-based paging indication and sequence-based paging indication, UE can reduce the unnecessary Paging message reception via the paging indication obviously.

Link level evaluation for PEI

When IDLE UE is out of synchronization with the network after long deep sleep, the potential frequency error could be more than 1 ppm. After cell search and timing acquisition from 1 SSB, the frequency error would be reduced to 1 ppm. For evaluating the detection performance of PDCCH and PEI, the frequency errors of 0 ppm, 0.1ppm, 0.5 ppm and 1 ppm are assumed. And the detection performance with the frequency error of 0 ppm is baseline.

The simulation assumptions for detection performance

Based on the agreements in RAN1#102 e-meeting, the following simulation assumptions are used. And simulation results are provided according to chairman’s agreement assumptions.

Table 2 The LLS simulation assumptions

| Parameters | Values |
| --- | --- |
| Carrier Frequency | 4GHz (FR1) |
| Transmission BW | 20MHz (FR1) |
| Antenna Configuration | 2T/4R |
| Subcarrier spacing | 15kHz, 30kHz |
| Channels | TDL-C 300 ns delay spread 100 Hz Doppler shift |
| Residue frequency error | 0 ppm, 0.1 ppm, 0.5ppm, 1ppm |
| DCI-based PEI configuration | AL=4(288 REs)/8(576 REs)/16(1152 REs) 40 info + 24 CRC bits REG bundle size 6 |
| TRS-based PEI configuration | Number of RBs: 48 Number of symbols: - 2 symbols in a slot - 4 symbols in 2 slots Density: 3 REs/RB |
| SSS-based PEI configuration | Length: 144 Sequence: same as SSS Number of symbols: 2(288 REs) or 4(576 REs) |

Detection performance of PDCCH at the Paging Occasion

It is worth pursuing to reduce the power consumption of IDLE mode UE as much as possible, but the premise is to ensure the reliable detection performance. SSBs are used to mitigate residue frequency error. Residue frequency error will affect the detection performance of PDCCH. The detection performance of PDCCH with different frequency errors is shown in Figure 4 with l subcarrier spacing 15kHz and 30kHz.

(a): 15kHz SCS

(b): 30kHz SCS

Figure 4: Detection performance of PDCCH

From the above Figure 4, it could be observed that:

- For 15kHz subcarrier spacing, the detection performance of PDCCH is sensitive to the frequency error. The performance loss due to frequency error with 0.5ppm is 1.1~1.3 dB comparing to that without frequency error. For frequency error at 1ppm, the detection performance of PDCCH cannot reach the BLER at 0.1.

- For 30kHz subcarrier spacing, the detection performance of PDCCH is robust to the frequency error.

Observation 1: PDCCH is sensitive to the frequency error if the subcarrier spacing is 15kHz, the loss of detection performance is up to 1.1~1.3 dB for 0.5ppm. And PDCCH is robust to the frequency error if the subcarrier spacing is 30kHz.

Detection performance of DCI-based PEI and sequence-based PEI

The PEI can provide substantial power saving gain and is supported in RAN1#103 e-meeting. It is FFS whether the PEI is DCI-based or sequence-based. The Link level simulation with residue frequency error is performed to compare the DCI-based and sequence-based PEI detection performance.

For DCI-based PEI, sub-grouping is supported by including additional bits for the sub-group indication within the DCI.  The sequence-based PEI supports the indication of sub-grouping by transmitting different sequences for each sub-group. When the sub-grouping is supported in PEI, the DCI size and the number of sequence transmitted in a resource can have the impact on the PEI detection performance. When no more than one sequence is transmitted in a resource at a given time, the false alarm probability of sequence-based PEI shown in section 3.1 is negligible.

The DCI-based and sequence-based PEI link performances are evaluated respectively with frequency errors of 0 ppm, 0.5 ppm and 1 ppm. For DCI-based PEI, PDCCH detections with AL of 4/8/16 are evaluated. For sequence-based PEI, TRS with 1slot and 2 symbols are evaluated.

(a): 15kHz                                                           (b): 30kHz

Figure 5: Detection performance of DCI-based PEI and sequence-based PEI with 0 ppm

(a): 15kHz                                                           (b): 30kHz

Figure 6: Detection performance of DCI-based PEI and sequence-based PEI with 0.5 ppm

From the Figure 5 and Figure 6, it could be observed that

- When CFO compensation is performed, e.g., UE wakes up early to measure sufficient number of SSB burst set, to minimize the CFO closed to 0 ppm, sequence-based PEI has slightly better performance than that of DCI-based PEI with AL of 16 for both 15kHz and 30kHz subcarrier spacing.  The amount of the resource used for DCI-based PEI with AL of 16 is four times than that of sequence-based PEI. The Resource of DCI-based PEI with AL of 4 and the sequence-based PEI is same. The sequence-based PEI provides higher miss-detection performance gain over the DCI-based PEI.

- When the frequency error is 0.5 ppm, if the subcarrier spacing is 30kHz, the detection performance of DCI-based PEI and sequence-based PEI are similar to the performance with 0ppm frequency error. For subcarrier spacing 15kHz, the detection performance of sequence-based PEI is much better than that of DCI-based PEI with AL of 4/8/16 due to the DCI-based PEI detection performance is quite sensitive to frequency errors.

The CFO compensation is needed for DCI-based PEI to have compatible performance with that of sequence-based PEI. The CFO compensation requires UE power consumption in SSB or TRS processing. The frequency error tolerated sequence-based PEI show better performance gain than that of DCI-based PEI when the frequency error is not fully compensated.

Observation 2:  Whether the frequency error is compensated or not, sequence-based PEI shows better detection performance than that DCI-based PEI.

Detection performance of TRS-based PEI and SSS-based PEI

Sequence-based PEI can be TRS-based PEI or SSS-based PEI. We further evaluate the detection performance of TRS-based PEI and SSS-based PEI, the results as shown in Figure 7.

(a): 0 ppm                                                           (b): 0.5 ppm

Figure 7: Detection performance of TRS-based PEI and SSS-based PEI

From the above Figure 7, it could be observed that the detection performance of TRS-based PEI and SSS-based PEI is similar at 0ppm and 0.5 ppm.

Observation 3:  The detection performance of TRS-based PEI is similar to SSS-based PEI.

Power saving performance for PEI

The PEI can provide substantial power saving gain. In this section, the power saving performance  of  DCI-based PEI and sequence-based PEI are evaluated.

The simulation assumptions for power saving gain

The simulation assumptions are listed in Table 3 and others are as those listed in TR 38.840. Different paging cycles, i.e. 32rf, 64rf, 128rf, 256rf, are evaluated, respectively.

Table 3:  System parameter assumptions for Paging configuration

| Parameter | Assumption |
| --- | --- |
| Numerology | 30KHz, FR1 |
| SSB | 2SSB per slot, maximum 3SSB burst set. 20ms period |
| Paging cycle | 32rf, 64rf, 128rf, 256rf |
| Group paging rate | 0.1 |
| RRM measurement cycle | 4(32rf), 2(64rf),1(128rf,256rf) with RRM measurement |
| SMTC | 5ms |
| MGL | 6ms |

The power saving gain of two paging early indication schemes, i.e. DCI-based and sequence-based, are provided and the general paging procedure in Rel-16 is taken as the performance baseline. The evaluation cases are listed as following.

- Case 1: Rel-16 paging procedure as the baseline.

- Case 2: DCI-based PEI. The TRS/CSI-RS assisted DCI-based paging indication based on additional TRS/CSI-RS.

- Case 3: Sequence-based PEI. The sequence-based paging indication is used for channel tracking and paging indication.

Evaluation results of DCI-based PEI

In the DCI-based PEI scheme, the DCI-based PEI indicates whether UE receives paging or not in this paging cycle with TRS/CSI-RS assisting UE in performing channel tracking for PDCCH decoding. As evaluated in section 3.2, the PDCCH is sensitive to the frequency error if the subcarrier spacing is 15kHz, the loss of detection performance is up to 1.1~1.3 dB for 0.5ppm. So to ensure the PDCCH with reliable detection performance, three SSBs is required to compensate the CFO, as shown in Figure 2. The power saving gain of DCI-based PEI with three SSBs assumption is shown in Figure 8.

Figure 8: Power saving gain of DCI-based PEI with group paging rate 0.1 under different paging cycle

From the results in Figure 8, we observe that

- The DCI-based PEI achieves power saving gain, i.e., 4.41%~10.07%. The power saving gain is from reducing unnecessary paging reception. Because DCI decoding and demodulation needs high reliability, the number of SSBs in the preparation period is similar to that for decoding of paging DCI to achieve the same PDCCH performance.

Evaluation results on sequence-based PEI

In the sequence-based PEI scheme, the sequence signal is used for tracking and paging indication. The performance result is shown in the following figure.

Figure 9: Power saving gain of sequence-based PEI with group paging rate 0.1 under different paging cycle

From the results in Figure 9, we observe that

- The sequence-based PEI achieves power saving gain at 23.11%~51.59% power saving gain. The power saving gain is from reducing unnecessary paging reception and shortens the time in the preparation period since the non-coherent detection of sequence-based PEI has high tolerance in time and frequency error.

Observation 4: Sequence-based PEI could provide more power saving gains than DCI-based PEI.

In summary, Table 4 shows the comparison of different PEIs from the power saving gain and detection performance views. For sequence-based PEI with the similar overhead as the DCI-based PEI, it can provide higher detection performance and power saving gains over the DCI-based PEI. Thus, the sequence-based PEI should be applied to Rel-17 IDLE/Inactive mode power saving.

Table 4: Comparison of different PEIs

| PEI Types |  | Power saving gain | Detection performance |  |
| --- | --- | --- | --- | --- |
|  |  |  | Subcarrier spacing:15kHz | Subcarrier spacing:30kHz |
| DCI-based PEI |  | Low | Bad | Normal |
| Sequence-based PEI | TRS-based PEI | High | Good |  |
|  | SSS-based PEI | -- | Good |  |

Proposal 1:  The sequence-based PEI should be adopted in Rel-17 for UE in IDLE/Inactive mode for UE power saving.

Sub-grouping

The PEI could be used as early indication of paging reception in advance. If PEI indicated the following PO has its paging reception, UE could wakeup to receive paging message, otherwise, UE could skip paging PDCCH reception and related paging PDSCH reception. However, some UEs share the same PO. Once any UE within the UEs sharing the same PO is paged, PEI will indicate all UEs to decode the paging DCI and PDSCH, it causes false alarm issue and unnecessary power consumption for the UEs without paging. PEI with sub-grouping can solve this issue and further reduce the power consumption. The UEs sharing the same PO are divided to different sub-group UEs, and then different sub-grouping PEI indicate the different sub-group UEs to determine whether to decode the paging PDCH and PDSCH.

Sequence-based PEI with sub-grouping

Group WUS is supported in NB-IoT Rel-16. Different sequences are used to indicate different UE groups sharing the same PO. A UE supporting Group WUS can be configured to monitor a group WUS and a common WUS. Upon detecting either of them, UE shall monitor POs. Sequence-based PEI provides better detection performance and higher power saving gains. Whether to support sub-grouping in sequence-based PEI is analysed. The sequence-based PEI with sub-grouping can refer to the Group WUS design, there are two potential options to support sub-grouping:

Option 1: Multiple sub-grouping sequence-based PEI can be transmitted in a resource at a given time. Any sub-group UEs are paged, the corresponding sub-group sequence-based PEIs will be transmitted.

Option 2: Single sequence-based PEI is transmitted in a resource at a given time. If more than one sub-group of UEs are paged, then common sequence-based PEI is transmitted.

The main difference between option 1 and option 2 is that the maximum number of sequences transmitted in a resource. If more than two sub-group UEs are paged, common sequence-based PEI will be transmitted in option 2, which might cause the false alarm to UEs in other sub-group not paged. For option 1, difference sub-group sequence-based PEIs can be transmitted in a resource to reduce false alarm rate when more than two sub-group UEs are paged.  This reduces the UE power consumption in decoding each sub-group sequence-based PEI and more resources for sub-group sequence-based PEI repetition are needed to ensure the sub-group PEI detection performance. So there is a trade-off between false alarm probability and system resource overhead.

The paging rate and false alarm rate for different sub-group numbers are analysed as following. Assume that the paging rate for a PO is X and sub-group number for a PO is N.   Each sub-grouping paging rate (Y) can be calculated by formula: 1- (1-Y) ^N=X. The paging rates of 1, 2… N-1, N sub-groups from network perspective are calculated in Table 5, assume X is 10% and 40%, N is 8.

Table 5: Paging rate for different sub-group numbers from network perspective

| Case | Sub-group number | Calculate formula | Paging rate |  |
| --- | --- | --- | --- | --- |
|  |  |  | X:10%, Y:1.3% | X:40%, Y:6.2% |
| Case 1 | 1 sub-group | C81*Y*(1-Y)^7 | 9.5% | 31.7% |
| Case 2 | 2 sub-groups | C82*Y^2*(1-Y)^6 | 0.4% | 7.3% |
| Case 3 | 3 sub-groups | C83*Y^3*(1-Y)^5 | 1.2*10^(-4) | 1% |
| Case 4 | 4 sub-groups | C84*Y^4*(1-Y)^4 | 1.9*10^(-6) | 8*10^(-4) |
| Case 5 | 5 sub-groups | C85*Y^5*(1-Y)^3 | 2*10^(-8) | 4.2*10^(-5) |
| Case 6 | 6 sub-groups | C86*Y^6*(1-Y)^2 | 1.3*10^(-10) | 1.4*10^(-6) |
| Case 7 | 7 sub-groups | C87*Y^7*(1-Y) | 5*10^(-13) | 2.6*10^(-8) |
| Case 8 | 8 sub-groups | Y^8 | 8.2*10^(-16) | 2.2*10^(-10) |

As given in Table 5, 1, 2… 8 sub-groups UEs from network perspective can be paged simultaneously when N is 8. From UE perspective, option 1 can reduce the false alarm rate for the case 2…case 7 comparing to option 2.  For option 2, if more than one sub-group of UEs are paged, common PEI is transmitted. When there are 2, 3…7 sub-groups of UEs are paged, at least 1 sub-group wastes unnecessary power to decode PDCCH and PDSCH in option 2. Table 6 shows the false alarm rate for option 2 from UE perspective.

Table 6: False alarm rate for option 2 from UE perspective

| Case | Calculate formula | False alarm rate |  |
| --- | --- | --- | --- |
|  |  | X:10%, Y:1.3% | X:40%, Y:6.2% |
| Case 2 | C72*Y^2*(1-Y)^6 | 0.3% | 5.5% |
| Case 3 | C73*Y^3*(1-Y)^5 | 7.2*10^(-5) | 0.6% |
| Case 4 | C74*Y^4*(1-Y)^4 | 9.5*10^(-7) | 4*10^(-4) |
| Case 5 | C75*Y^5*(1-Y)^3 | 7.5*10^(-9) | 1.6*10^(-5) |
| Case 6 | C76*Y^6*(1-Y)^2 | 3.3*10^(-11) | 3.5*10^(-7) |
| Case 7 | C77*Y^7*(1-Y) | 6.2*10^(-14) | 3.3*10^(-9) |

In fact, the false alarm rate is very small as shown in Table 6. Thus, the false alarm rate of option 2 can be negligible.  Option 1 achieves the power saving gain than that of option 2 as shown in section 3.2. Therefore, it is desirable that common sequence-based PEI is transmitted when more than one sub-group of UEs are paged. This design of sequence-based PEI with sub-grouping only causes a little false alarm but the system resource overhead is not increased.

Observation 5:  For supporting sequence-based PEI with sub-grouping, if no more than one sequence is transmitted in a resource at a given time, the false alarm probability is negligible.

The simulation assumptions for sequence-based PEI with sub-grouping

The simulation assumptions are listed in Table 7 and others are as that in TR 38.840.

Table 7:  System parameter assumptions for sequence-based PEI with sub-grouping

| Parameter | Assumption |
| --- | --- |
| Numerology | 30KHz, FR1 |
| SSB | 2SSB per slot, maximum 3SSB burst set. 20ms period |
| Paging cycle | 128rf |
| Group paging rate | 0.1, 0.4 |
| RRM measurement cycle | 1 |
| SMTC | 5ms |
| MGL | 6ms |

The power saving gain of sequence-based PEI with sub-grouping is provided.  Rel-16 paging procedure (Case 1) and the sequence-based PEI (Case 3) in section 2.3.3 are taken as the performance baseline respectively. The evaluation cases are listed as following.

- Case 1: Rel-16 paging procedure.

- Case 3: Sequence-based PEI. The sequence-based paging indication is used for channel tracking and paging indication.

- Case 4: Sequence-based PEI with sub-grouping. The sub-grouping sequence-based paging indication is used for channel tracking and fine granularity paging indication.

Evaluation results on sequence-based PEI with sub-grouping

In the sequence-based PEI with sub-grouping scheme, the sub-grouping sequence signal is used for tracking and fine granularity paging indication. The additional power saving gain by introducing sub-grouping in sequence-based PEI is shown in Figure 10. The number of sub-groups with 2, 4, 8, 16, 32 and 64 are evaluated, and the paging rate is assumed to 0.1.

Figure 10: Additional power saving gain of sequence-based PEI with sub-grouping

From the results in Figure 10, we observe that the sequence-based PEI with sub-grouping achieves additional power saving gain at 30.19%~31.73% comparing to baseline case 1.  The power saving gain range 1.2%~2.21% power saving over case 3 sequence PEI without sub-grouping. Furthermore, with the increase of the number of sub-group, the gain of UE power saving is tending flat. The number of sub-group should be up to 8 considering the trade-off between the power saving gain and the overhead.

Observation 6: Introducing sub-grouping in sequence-based PEI can bring 30.19%~31.73% and 1.2%~2.21% power saving gain compare to Rel-16 paging and sequence-based PEI respectively.

Observation 7: With the increase of the number of sub-group, the gain of power saving gain is tending flat.

Proposal 2:  The sequence-based PEI with sub-grouping does not provide much power saving gain over that without sub-grouping and require large additional overhead.  Supporting sub-grouping should be further study.

Proposal 3:  For the sequence-based PEI with sub-grouping, the number of sub-groups should be up to 8 sub-groups considering the trade-off between power saving gain and system overhead.

For supporting sequence-based PEI with sub-grouping, different options are discussed in section 3.1. As given in Table 6, option 1 can reduce the false alarm probability for the case 2…case 7 compare to option 2, so the option 1 achieves some power saving gain than the option 2. The Figure 11 shows the total power saving gain of sequence-based PEI with sub-grouping for all cases in Table 6.

Figure 11: Power saving gain of option 1 compare to option 2 in section 2.2

From the results in Figure 11, we observe that the power saving gain of option 1 relative to option 2 is negligible for paging rate of 10%. Although the power saving gain is 2.01%~2.69% when the paging rate is 40%, it is not a typical assumption that the paging rate is 40%.

Observation 8: To support sequence-based PEI with sub-grouping, the power saving gain of option 1 relative to option 2 is negligible.

- Option 1: Multiple sub-grouping PEI can be transmitted in a resource.

- Option 2: Single sequence is transmitted in a resource.

PEI procedure and configuration

- PEI procedure and configuration

From the analysis and evaluation results, sequence-based PEI shows large power saving gain and good detection performance. The sequence-based PEI procedure and configuration are discussed in our combination contribution [2].  In general, the procedure of sequence-based PEI could be assumed as following steps:

Step1: TRS/CSI-RS frequency domain resource allocation through system broadcast (SIB) - examples of PEI parameters, such as:  period, slot pattern, PEI resource number and others.

Step2: get PEI time location

The TRS/CSI-RS resource of PEI in time domain for a given UE or a group of UEs could be derived by UE based on UE ID similar to the derivation of paging occasion. The TRS/CSI-RS resource of PEI in time domain is associated with the paging occasion.  UE could derive the TRS/CSI-RS resource of PEI in time domain from the starting slot of PO. An example of UE derivation of TRS/CSI-RS resource allocation in time domain associated with the first slot of PO is shown as follows,

  - Step2.1: Calculate PEI SFN

(PEI_SFN + PEI_offset) mod T = (T div N) * (UE_ID mode N)

Wherein, PEI_SFN is the SFN number of the resource allocation of PEI in time domain; PEI_offset is the slot offset from the SFN boundary for PEI; T is PEI cycle of a given UE or UE group; N is the number of total PEI radio frames in T; UE_ID is the 5G-S-TMSI mode 1024.

And all of parameters could be (pre)-configured by SIB or signaling through system broadcast (SIB).    The parameters for PEI resource in time domain could link to existing PO parameters, e.g., T is PEI cycle of the UE and related to PO cycle T; N is the number of total PEI frames in T and related to PO parameter N.

  - Step2.2:  calculate PEI subframe/slot time location

PEI_i_s = floor(UE_ID/N) mod Ns

Wherein, index of PEI_i_s is the index of the PEI within PEI SFN; Ns is the number of PEI occasions for a PEI SFN.  PEI subframe/slot time location PEI_slot is determined. Similar to step2.1, all parameters could be configured by SIB or higher layer signaling. They could also link to existing PO parameters, e.g., Ns is the number of PEI occasions for a PEI SFN and related to PO parameter Ns.

  - Step2.3: calculate PEI with different TCI state in  time domain if configured

PEI_beam_i_symbol = PEI_slot * symbol_number_per_slot + PEI_beam_i_offset

Wherein, PEI_beam_i_symbol is the i-th beam time location; PEI_beam_i_offset is the i-th beam time offset.

UE could update its PEI resource in time domain according to calculation predefined formula and related parameters. E.g., if sequence-based PEI parameters are configured according to PO parameters, the resource allocation of sequence-based PEI in time domain could be updated when PO parameters are updated.

Proposal 4: The sequence-based PEI configuration and procedure could be calculated by reference signal/channel, e.g., reusing the procedure of paging occasion computation for 38.304.

- QCL for PEI

In NR, the paging PDCCH and PDSCH are transmitted corresponding to the beams of the detected SSB. For paging enhancement in IDLE/Inactive mode, multiple beams of PEI should be supported to improve the PEI detection performance for the UEs in different beam coverage. The beams of PEI should be corresponding to the SSB beams and it is consistent with the beams used for paging PDCCH and PDSCH. This is to say, the spatial channel property of PEI should be QCLed with those of SSB corresponding to paging PDCCH/PDSCH.

Proposal 5: For IDLE/Inactive mode, multiple beams of PEI should be supported.

Proposal 6: For IDLE/Inactive mode, the spatial channel property of PEI is QCLed with the beams of SSB corresponding to paging PDCCH/PDSCH.

- Resource Collision between PEI and PO

As discussed in our combination contribution [3], there are two possible alternatives of the number of PEI to PO from network perspective.

- Alt 1  PEI: PO = 1: 1

- Alt 2  PEI: PO = 1: n

When the paging occasions within a DRX cycle is dense and one PEI is corresponding to one paging occasion, and the PEI is transmitted in multiple beams.  There might be issues of resource collision.  For the paging parameter N is T and Ns is 4, every ratio frame is paging frame and there are 4 paging occasions within a paging frame, as shown in Figure 12.  It is assumed that the gap between the starting of PEI and the starting of the corresponding PO is 4ms, and the beams number of PEI and PO are 4.

Figure 12 An illustrate of PEI and PO collisions

From the above Figure 12, it can be observed that several PEIs and paging occasions collide with each other. Once the PEIs and paging occasions are collided, eNB will transmit paging PDCCH to ensure the legacy UEs receive paging PDCCH. Rel-17 UEs need to decode paging PDCCH directly due to the de-prioritized PEI transmission, which cannot achieve the purpose of power saving. Thus, one PEI to indicate multiple PO is better alternative for the paging occasion’s dense scenario.

Proposal 7: The PEI design should consider a trade-off between the ratio of the number of PEI to PO and resource collision.

Conclusion

In this contribution, paging enhancement in RRC_IDLE/Inactive mode is discussed and analysed. Based on discussion, we have the following observations and proposals:

Observation 1: PDCCH is sensitive to the frequency error if the subcarrier spacing is 15kHz, the loss of detection performance is up to 1.1~1.3 dB for 0.5ppm. And PDCCH is robust to the frequency error if the subcarrier spacing is 30kHz.

Observation 2:  Whether the frequency error is compensated or not, sequence-based PEI shows better detection performance than that DCI-based PEI.

Observation 3:  The detection performance of TRS-based PEI is similar to SSS-based PEI.

Observation 4: Sequence-based PEI could provide more power saving gains than DCI-based PEI.

Observation 5:  For supporting sequence-based PEI with sub-grouping, if no more than one sequence is transmitted in a resource at a given time, the false alarm probability is negligible.

Observation 6: Introducing sub-grouping in sequence-based PEI can bring 30.19%~31.73% and 1.2%~2.21% power saving gain compare to Rel-16 paging and sequence-based PEI respectively.

Observation 7: With the increase of the number of sub-group, the gain of power saving gain is tending flat.

Observation 8: To support sequence-based PEI with sub-grouping, the power saving gain of option 1 relative to option 2 is negligible.

- Option 1: Multiple sub-grouping PEI can be transmitted in a resource.

- Option 2: Single sequence is transmitted in a resource.

Proposal 1:  The sequence-based PEI should be adopted in Rel-17 for UE in IDLE/Inactive mode for UE power saving.

Proposal 2:  The sequence-based PEI with sub-grouping does not provide much power saving gain over that without sub-grouping and require large additional overhead.  Supporting sub-grouping should be further study.

Proposal 3:  For the sequence-based PEI with sub-grouping, the number of sub-groups should be up to 8 sub-groups considering the trade-off between power saving gain and system overhead.

Proposal 4: The sequence-based PEI configuration and procedure could be calculated by reference signal/channel, e.g., reusing the procedure of paging occasion computation for 38.304.

Proposal 5: For Idle/Inactive mode, multiple beams of PEI should be supported.

Proposal 6: For Idle/Inactive mode, the spatial channel property of PEI is QCLed with the beams of SSB corresponding to paging PDCCH/PDSCH.

Proposal 7: The PEI design should consider a trade-off between the ratio of the number of PEI to PO and resource collision.

References

- RAN1#103 e-meeting, Chairman notes.

- R1- 2100394, Details of PEI configuration, CATT.

- R1- 2100396, System overhead analysis of PEI and TRS/CSI-RS for IDLE mode UE, CATT