3GPP TSG RAN WG1 #104-e                                                                             R1-2100394

e-Meeting, January 25th – February 5th, 2021

Source:              CATT

Title:                  Details of PEI configuration

Agenda Item:     8.7.1.3

Document for:   Discussion and Decision

Introduction

In RAN1#102b-e, it proposed potential paging enhancement options based on agreements [1], including

- Paging early indication before a target PO to indicate UE whether to monitor PDCCH scrambled with P-RNTI at the PO. Potential candidate indication methods include

  - DCI-based indication, e.g., based on

    - Extending existing DCI format 1_0 or 2_6

    - New DCI format

  - RS-based or sequence-based indication, e.g., based on TRS/CSI-RS or SSS

- Sub-grouping for paging, based on

  - Legacy paging DCI

  - Paging early indication

  - Additional reception occasions in time/frequency domain

  - Multiple P-RNTIs

- Cross-slot scheduling for paging PDSCH

- Other proposal is not precluded

In RAN1#103-e, it agreed PEI before paging occasion as NR idle/inactive-mode paging enhancement [2], including:

- For NR idle/inactive-mode paging enhancement, paging early indication before paging occasion is supported from RAN1 perspective

  - FFS: Physical layer design based on DCI, SSS or TRS/CSI-RS

  - Send LS to inform RAN2 and kindly ask RAN2 to inform RAN1 if there is anything that RAN1 should take into consideration in the physical layer design for this feature, including any other progress RAN2 has made in this WI which may has RAN1 impact

In this contribution, the PEI configuration and procedure for UE in RRC_IDLE/Inactive mode is discussed, including the procedure of PEI configuration for sequence-based PEI and DCI-based PEI. The initial evaluation results on PEI link performance are provided based on agreed simulation assumptions.

PEI configuration

PO configuration and procedure

For NR, the UE monitors one paging occasion (PO) per DRX cycle. A PO is a set of PDCCH monitoring occasions and can consist of multiple time slots where paging DCI can be sent. One Paging Frame (PF) is one Radio Frame and may contain one or multiple PO(s) or starting point of a PO. And for the PO calculation, the PF and PO for paging are determined by the following formula:

- SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

- Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns

When SearchSpaceId = 0 is configured for pagingSearchSpace, the PDCCH monitoring occasions for paging are same as for RMSI as defined in clause 13 in TS 38.213. Otherwise,

When SearchSpaceId other than 0 is configured for pagingSearchSpace, the UE monitors the (i_s + 1)th PO. A PO is a set of 'S*X ' consecutive PDCCH monitoring occasions with association of a number of transmitted SSBs and the firstPDCCH-MonitoringOccasionOfPO configuration. As an example, multiple POs are showed in Figure 1 derived from the paging formula aforementioned.

Figure 1: Paging occasions illustration

PEI configuration and procedure

For RRC-Idle mode, PEI is used to indicate whether there is a paging message for a given group before the PO. It was agreed that both sequence-based PEI and DCI-based PEI are considered as the candidates for PEI. The procedure and configuration of sequence-based and DCI-based PEI are discussed in this section.

Sequence-based PEI configuration and procedure

For sequence-based PEI, TRS/CSI-RS is an option. Other signals, e.g., SSS did not exclude. For sequence-based PEI configuration and procedure, the following steps could be considered. Taking TRS/CSI-RS-based PEI as example:

Step1: TRS/CSI-RS frequency domain resource allocation through system broadcast (SIB) - examples of PEI parameters, such as:

- Period: e.g.,10,20,40,80,160,320,640(slot)

- Slot pattern: the PEI located symbol position within slot. E.g.,(4,8),(5,9),(6,10)

- PEI resource number: e.g., 2 or 4 per period.

- Others.

Step2: get PEI time location

The TRS/CSI-RS resource of PEI in time location for a given UE or a group of UEs could be derived by UE based on UE ID similar to the derivation of paging occasion. The TRS/CSI-RS resource of PEI in time domain is associated with the paging occasion. UE could derive the TRS/CSI-RS resource of PEI in time domain from the starting slot of PO. An example of UE derivation of TRS/CSI-RS resource allocation in time domain associated with the first slot of PO is shown as follows,

  - Step2.1: Calculate PEI SFN

(PEI_SFN + PEI_offset) mod T = (T div N) * (UE_ID mode N)

Wherein, PEI_SFN is the SFN number of the resource allocation of PEI in time domain; PEI_offset is the slot offset from the SFN boundary for PEI; T is PEI cycle of a given UE or UE group; N is the number of total PEI radio frames in T; UE_ID is the 5G-S-TMSI mode 1024.

And all of parameters could be (pre)-configured by SIB or signaling through system broadcast (SIB). The parameters for PEI resource in time domain could link to existing PO parameters, e.g., T is PEI cycle of the UE and related to PO cycle T; N is the number of total PEI frames in T and related to PO parameter N.

  - Step2.2: calculate PEI subframe/slot time location

PEI_i_s = floor(UE_ID/N) mod Ns

Wherein, index of PEI_i_s is the index of the PEI within PEI SFN; Ns is the number of PEI occasions for a PEI SFN. PEI subframe/slot time location PEI_slot is determined. Similar to step2.1., all parameters could be configured by SIB or higher layer signaling. They could also link to existing PO parameters, e.g., Ns is the number of PEI occasions for a PEI SFN and related to PO parameter Ns.

  - Step2.3: calculate PEI with different TCI state in time domain if configured

PEI_beam_i_symbol = PEI_slot * symbol_number_per_slot + PEI_beam_i_offset

Wherein, PEI_beam_i_symbol is the i-th beam time location; PEI_beam_i_offset is the i-th beam time offset.

The procedure of PEI time location calculation is shown in Figure 2.

Figure 2: PEI time location calculation illustration

UE could be updated the PEI resource allocation in time domain according to the formula and related parameters, e.g., if PEI parameters are configured linked to PO parameters. The PEI resource allocation in time domain could be update when PO parameters are updated. With the derivation of PEI associated with PO, the PEI resource allocation in time domain could update based on the updated PO parameters.

Proposal 1: The sequence-based PEI configuration and procedure could be derived by reference of existing procedure and configuration parameters, e.g., reusing the procedure of paging occasion computation for 38.304.

DCI-based PEI configuration and procedure

For DCI-based PEI, existing DCI format could be used. For existing DCI, e.g., DCI format 1_0, DCI format 2_6 could be considered for PEI. Furthermore, considering existing DCI-based PEI configuration and procedure, the following steps could be considered.

Step1: Paging configuration and first PO PDCCH monitoring from DownlinkConfigCommonSIB IE as follows,

DownlinkConfigCommonSIB ::=     SEQUENCE {

frequencyInfoDL                 FrequencyInfoDL-SIB,

initialDownlinkBWP              BWP-DownlinkCommon,

bcch-Config                         BCCH-Config,

pcch-Config                         PCCH-Config,

...

}

BCCH-Config ::=                 SEQUENCE {

modificationPeriodCoeff         ENUMERATED {n2, n4, n8, n16},

...

}

PCCH-Config ::=             SEQUENCE {

defaultPagingCycle                  PagingCycle,

nAndPagingFrameOffset               CHOICE {

oneT                                NULL,

halfT                               INTEGER (0..1),

quarterT                            INTEGER (0..3),

oneEighthT                          INTEGER (0..7),

oneSixteenthT                       INTEGER (0..15)

},

ns                                  ENUMERATED {four, two, one},

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT                                   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT                                                      SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}      OPTIONAL,           -- Need R

...,

[[

nrofPDCCH-MonitoringOccasionPerSSB-InPO-r16                                  INTEGER (2..4)             OPTIONAL  -- Cond SharedSpectrum2

]]

}

-- TAG-DOWNLINKCONFIGCOMMONSIB-STOP

-- ASN1STOP

Step2: DCI-based PEI search space included in DownlinkConfigCommonSIB IE.

Proposal 2: DCI-based PEI configuration could be included in DownlinkConfigCommonSIB.

Initial evaluation results for PEI link performance

Simulation assumption and Initial evaluation results for PEI link performance

The performance of sequence-base PEI and DCI-based PEI is discussed with the analysis of results and observation in this section. According to agreements on simulation assumption, the following parameters are assumed.

Table 1: Simulation assumptions for sequence-based PEI and DCI-based PEI link performance evaluation

| Parameters | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Channel model | TDL-C 300ns 100Hz |
| Antenna number at BS | 2 |
| Antenna number at UE | 4 |
| Subcarrier spacing | 30 kHz |
| TRS like sequence | Number of RBs: 48 Number of symbols: - 2 symbols in a slot - 4 symbols in 2 slots Density: 3 REs/RB |
| PDCCH | AL=4(288RE),8(576RE),16(1152RE) DCI=40bit |
| CFO | 0,0.5 ppm |

The initial simulation results are shown as following figures.

Figure 3: Sequence-based PEI and DCI-based PEI performance

It could be observed that sequence-based PEI could perform better than DCI-based PEI for MDR when CFO is at the range of 0 ppm or 0.5 ppm. For sequence-based paging indication with the similar overhead as the DCI-based paging indication, it can provide higher miss-detection performance 5dB over the existing PDCCH based power saving signal/channel. Sequence-based power saving signal can be used for both channel tracking and paging indication and significant power saving gain also can be observed in our contribution. And a properly designed sequence-based power saving signal can support UE specific indication of paging reception which can provide more power saving gain than that of UE group based paging indication in our combination contribution [3]. Thus, the sequence-based paging indication should be applied to Rel-17 Idle/Inactive mode power saving.

Observation 1: Sequence-based PEI show better performance than that of DCI-based PEI regardless of whether the frequency error is fully compensated.

Proposal 3: The sequence-based paging indication should be supported in Rel-17 for UE in Idle/Inactive mode for UE power saving.

The UE procedure of PEI miss-detection

PEI has a probability of miss-detection. For UE in RRC-Idle mode, the miss-detection would be considered as no paging indication.

Proposal 4: For PEI miss-detection, UE could consider it as no paging and continue to sleep.

Conclusion

In this contribution, paging enhancement in RRC_IDLE/Inactive mode is discussed and analyzed. Based on discussion, we have the following observations and proposals:

Proposal 1: The sequence-based PEI configuration and procedure could be derived by reference of existing procedure and configuration parameters, e.g., reusing the procedure of paging occasion computation for 38.304.

Proposal 2: DCI-based PEI configuration could be included in DownlinkConfigCommonSIB.

Observation 1: Sequence-based PEI show better performance than that of DCI-based PEI regardless of whether the frequency error is fully compensated.

Proposal 3: The sequence-based paging indication should be supported in Rel-17 for UE in Idle/Inactive mode for UE power saving.

Proposal 4: For PEI miss-detection, UE could consider it as no paging and continue to sleep.

References

- RAN1#102b e-meeting, Chairman notes.

- RAN1#103 e-meeting, Chairman notes.

- R1-2100392, Paging enhancement for UE power saving, CATT.