3GPP TSG RAN WG1 #104-e	R1-2100407

# e-Meeting, January 25th – February 5th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | V15.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.213 Draft CR on PUCCH close loop power control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Release 15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For PUCCH rampup power calculation of close power control during the random access procedure or link failure recovery procedure, there is no need to calculate rampup power for no PUCCH transmission case. The UE only has to calculate rampup power in case the PUCCH transmission is performed. Although TS 36.213 exists with a description on PUCCH rampup power calculation for no PUCCH transmission case in Section 5.1.2.1 because LTE supports PUCCH PHR reporting without PUCCH transmission. However, NR does not support PHR type 2 so that the case of PUCCH PHR reporting without PUCCH transmission is not supported neither. Therefore, the current TS 38.213 section 7.2.1 which specifies the UE has to calculate rampup power for no PUCCH transmission case is redundant and pointless. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the rampup power calculation formula and corresponding descriptions for no PUCCH transmission case during the random access procedure or link failure recovery procedure in TS 38.213 section 7.2.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Redundant description on rampup power calculation for no PUCCH transmission case during random access procedure or link failure recovery procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has no impact on network behavior. This CR has no impact on UE behavior. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

7.2.1	UE behaviour

<Unchanged parts are omitted>

-	 is the current PUCCH power control adjustment state  for active UL BWP  of carrier  of serving cell  and PUCCH transmission occasion , where

-	The  values are given in Table 7.1.2-1

-	 is a sum of TPC command values in a set  of TPC command values with cardinality  that the UE receives between  symbols before PUCCH transmission occasion  and  symbols before PUCCH transmission occasion  on active UL BWP  of carrier  of serving cell  for PUCCH power control adjustment state, where  is the smallest integer for which  symbols before PUCCH transmission occasion  is earlier than  symbols before PUCCH transmission occasion

-	If the PUCCH transmission is in response to a detection by the UE of a DCI format 1_0 or DCI format 1_1,  is a number of symbols for active UL BWP  of carrier  of serving cell  after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUCCH transmission

-	If the PUCCH transmission is not in response to a detection by the UE of a DCI format 1_0 or DCI format 1_1,  is a number of  symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP  of carrier  of serving cell

-	If the UE has reached maximum power for active UL BWP  of carrier  of primary cell  at PUCCH transmission occasion  and , then

-	If UE has reached minimum power for active UL BWP  of carrier  of primary cell  at PUCCH transmission occasion  and , then

-	If a configuration of a  value for a corresponding PUCCH power control adjustment state  for active UL BWP  of carrier  of serving cell  is provided by higher layers,

-

If the UE is provided PUCCH-SpatialRelationInfo, the UE determines the value of  from the value of  based on a pucch-SpatialRelationInfoId value associated with the p0-PUCCH-Id value corresponding to  and with the closedLoopIndex value corresponding to ; otherwise,

-	Else,

-	, where , and  is the TPC command value indicated in a random access response grant corresponding to a PRACH transmission or is the TPC command value in a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI that the UE detects in a first PDCCH reception in a search space set provided by recoverySearchSpaceId if the PUCCH transmission is a first PUCCH transmission after 28 symbols from a last symbol of the first PDCCH reception, and, if the UE transmits PUCCH on active UL BWP  of carrier  of serving cell ,

;

where  is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last preamble for active UL BWP  of carrier  of primary cell , and  corresponds to PUCCH format 0 or PUCCH format 1

<Unchanged parts are omitted>