3GPP TSG RAN WG1 #104-e	                                     R1-2100484

e-Meeting, January 25th – February 5th, 2021

Source:	vivo

Title:	On supporting DL 1024QAM for NR FR1

Agenda Item:	8.16

Document for:	Discussion and Decision

- Introduction

In RAN #90e, the newly updated WI on DL 1024QAM for NR FR1 was approved in [1]. The main objectives of RAN1 are as below:

- Specify high order modulation for PDSCH [RAN1]

  - Specify 1024QAM constellation as specified in E-UTRA for DL PDSCH

  - Specify corresponding 5-bit MCS table with 1024QAM entries as defined in E-UTRA, with 5 bit DCI overhead for MCS indication

  - Specify corresponding CQI feedback with 1024QAM entries as defined in E-UTRA, with no changes to the CQI field and table sizes

In RAN1#103e, the following agreements on DL 1024QAM for NR FR1 were made [2]:

Agreements:

- Introduce new RRC signaling to indicate use of 1024-QAM CQI table.

- For supporting 1024-QAM in NR downlink, adopt the LTE 1024-QAM constellation.

- 1024-QAM MCS table can be used only with DCI format with CRC scrambled by C-RNTI or CS-RNTI.

Agreements:

- Introduce new RRC signaling to indicate use of 1024-QAM MCS table for at least DCI format 1_1

  - FFS : support of 1024-QAM MCS table for DCI format 1_2

    - Note: If 1024-QAM MCS table for DCI format 1_2 is supported, separate RRC signaling is used for each of the two DCI formats 1_1 and 1_2, respectively

  - FFS : whether the RRC signaling is only introduced in PDSCH-Config or it can also be separately configured in SPS-Config

Agreements:

- RRC signaling (mcs-Table-r17) to indicate use of 1024-QAM MCS table for DCI format 1_1 is present only in PDSCH-config

- When UE is configured with mcs-Table-r17 set to ‘qam1024’ in PDSCH-Config,

  - UE uses 1024-QAM MCS table for PDSCH scheduled with a DCI format 1_1 with CRC scrambled by C-RNTI,

  - UE uses 1024-QAM MCS table for PDSCH scheduled with the DCI format 1_1 with CRC scrambled by CS-RNTI if the UE is not configured with mcs-Table in SPS-Config

- Note: If 1024-QAM MCS table for DCI format 1_2 is supported, similar approach is used for 1024-QAM MCS table usage with DCI format 1_2

In this contribution, we present our views on introducing DL 1024QAM for NR PDSCH operation in FR1.

- CQI/MCS tables definition

According to RAN plenary guideline, for 1024QAM CQI table design, the CQI table defined in [3, Table 7.2.3-4] can be reused except a little modification (replace the entry {853, 8.3321} by {853, 8.3301} to ensure that the value of SE can be calculated according to the value of code rate), which is shown in Table1. The reasons for modification are as below:

- All efficiency values can be deduced based on code rate/1024*modulation order except the entry of ‘code rate x 1024 = 853’.

- When we use code rate and spectral efficiency to calculate TBS respectively, two different TBS values would be got, which is unexpected. The tiny spectral efficiency difference will amplify with the increase of allocated PRBs.

- The spectral efficiency values have an impact on MCS table design, since the medium 1024QAM entries are interpolated based on the spectral efficiency values of 1024QAM CQI table.

Table 1 4-bit CQI Table for 1024QAM

| CQI index | modulation | code rate x 1024 | Efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.3770 |
| 3 | QPSK | 449 | 0.8770 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.33218.3301 |
| 15 | 1024QAM | 948 | 9.2578 |

Proposal 1: For DL 1024QAM CQI table for NR FR1, support to reuse 1024QAM CQI table defined in LTE with the entry {853, 8.3321} replaced by {853, 8.3301}.

With reference to the 1024QAM MCS table specified in 36.213 section 7.1.7.1, the design principles for NR PDSCH 1024QAM MCS table can be summarized as below,

- Keep DCI field size unchanged (5-bit MCS table with 5 reserved entries for retransmission).

- Reuse CQI table values as starting point.

- Linearly interpolate the remaining MCS entries by adjacent spectral efficiencies.

According to above principles, we propose to add 5 new entries for 1024QAM based on the NR 256QAM MCS table specified in [4, Table 5.1.3.1-2]. One of the new entries is used to indicate adaptive retransmissions. In addition, the above two CQI entries for 1024QAM are directly taken as two MCS entries, and then the other two new entries are obtained through linear interpolation.

During email discussion in previous RAN1 meeting, the following four alternatives were proposed for MCS table design.

- Alt 1: remove {5, 7, 9, 12, 14} from the 256-QAM table

- Alt 2: remove {6, 8, 10, 12, 14} from the 256QAM table

- Alt 3: remove {2, 4, 6, 8, 10} from the 256-QAM table

- Alt 4: remove {1, 3, 5, 7, 9} from the 256QAM table

Considering that 64QAM MCS table and 1024QAM MCS table can be dynamically switched through changing of DCI formats, we think that all the 256QAM entries should be kept. In LTE, 5 MCS entries {5, 7, 9, 12, 14}, are removed from 256QAM table. However, follow NR MCS table design principle, CQI entries should be kept, so {5,7,9} should be kept as they were already included in the 1024-QAM CQI table. To make comparable with LTE, we prefer to remove entries {6, 8, 10, 12, 14} from the 256QAM table, while keeping the lowest MCS. The details of the proposed 1024QAM MCS table are shown in Table 2.

Table 2 MCS index table for 1024QAM

| MCS Index | Modulation Order | Target code Rate R x [1024] | Spectral |
| --- | --- | --- | --- |
| IMCS | Qm |  | efficiency |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.377 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.877 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 378 | 1.4766 |
| 6 | 4 | 434 | 1.6953 |
| 6 | 4 | 490 | 1.9141 |
| 8 | 4 | 553 | 2.1602 |
| 7 | 4 | 616 | 2.4063 |
| 10 | 4 | 658 | 2.5703 |
| 8 | 6 | 466 | 2.7305 |
| 12 | 6 | 517 | 3.0293 |
| 9 | 6 | 567 | 3.3223 |
| 14 | 6 | 616 | 3.6094 |
| 10 | 6 | 666 | 3.9023 |
| 11 | 6 | 719 | 4.2129 |
| 12 | 6 | 772 | 4.5234 |
| 13 | 6 | 822 | 4.8164 |
| 14 | 6 | 873 | 5.1152 |
| 15 | 8 | 682.5 | 5.332 |
| 16 | 8 | 711 | 5.5547 |
| 17 | 8 | 754 | 5.8906 |
| 18 | 8 | 797 | 6.2266 |
| 19 | 8 | 841 | 6.5703 |
| 20 | 8 | 885 | 6.9141 |
| 21 | 8 | 916.5 | 7.1602 |
| 22 | 8 | 948 | 7.4063 |
| 23 | 10 | 805.5 | 7.8662 |
| 24 | 10 | 853 | 8.3301 |
| 25 | 10 | 900.5 | 8.7939 |
| 26 | 10 | 948 | 9.2578 |
| 27 | 2 | reserved |  |
| 28 | 4 | reserved |  |
| 29 | 6 | reserved |  |
| 30 | 8 | reserved |  |
| 31 | 10 | reserved |  |

In this section, we also provide relevant link level simulation results to further down-select Alt1 and Alt2. In Figure 1, for a target BLER, nearly equal-interval SNR can be observed for different MCSs under a common modulation order for Alt2. In addition, comparing with Alt 1, SE vs SNR curves at BLER=10% in Figure 2 indicate that more equally spaced SNR can be obtained by adopting Alt 2.

Figure 1: simulation results for 1024QAM MCS table

Figure 2: SE vs SNR curves at BLER=10%

Proposal 2: For DL 1024QAM MCS table for NR FR1,

- Support to design 1024QAM MCS table based on NR 256QAM MCS table.

- Support to remove NR 256QAM MCS table entries {6, 8, 10, 12, 14}.

- Support to introduce the following 1024QAM entries.

| MCS Index |  | Modulation Order | Target code Rate R x [1024] |  | Spectral |
| --- | --- | --- | --- | --- | --- |
| IMCS |  | Qm |  |  | efficiency |
| 23 |  | 10 | 805.5 |  | 7.8662 |
| 24 |  | 10 | 853 |  | 8.3301 |
| 25 |  | 10 | 900.5 |  | 8.7939 |
| 26 |  | 10 | 948 |  | 9.2578 |
| 31 | 10 |  |  | reserved |  |

- Signalling design

Typically, DL 1024QAM is mainly deployed in high throughput eMBB scenario. For DCI format 1_2 for URLLC scheduling, the motivation is unclear to use 1024QAM. Therefore, we propose to only support 1024QAM MCS table for PDSCH scheduled by DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI. In Figure 3, we also provide relevant system level simulation results in indoor scenario. Comparing with NR 256QAM, it can be observed that the UPT performance is not obvious for 1024QAM. In addition, for edge users, the UPT performance is degraded due to removing low modulation order MCS entries.

Figure 3: Comparison of UPT performance between 1024QAM and 256QAM

Proposal 3: Don’t support 1024QAM MCS table for PDSCH scheduled by DCI format 1_2.

- UE capability reporting

DL 1024QAM has high requirements for the EVM of UE receive signal, so it should be supported as an optional UE feature, and UE capability for supporting DL 1024QAM can be reported per FSPC for NR FR1 only.

Proposal 4: Support of DL 1024QAM is reported per FSPC optionally for NR FR1 only.

- TP

For DL 1024QAM for NR FR1, transport block size determination related sentences in section 5.1.3.2 of TS 38.214 should be added.

Proposal 5: Support the following text proposal for TBS determination in section 5.1.3.2 of TS 38.214.

#### 5.1.3.2 Transport block TS38.214 V16.4.0 #### 5.1.3.2 Transport block size determination In case the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled, then one of the two transport blocks is disabled by DCI format 1_1 if IMCS = 26 and if rvid = 1 for the corresponding transport block. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword. For the PDSCH assigned by a PDCCH with DCI format 1_0, format 1_1 or format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or SI-RNTI, if Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤27, else if Table 5.1.3.1-4 is used and 0 ≤ IMCS ≤27, or a table other than Table 5.1.3.1-2 and Table 5.1.3.1-4 is used and 0 ≤ IMCS ≤28, the UE shall, except if the transport block is disabled in DCI format 1_1, first determine the TBS as specified below: … else if Table 5.1.3.1-2 is used and , - the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using 0 ≤ IMCS ≤27. If there is no PDCCH for the same transport block using 0 ≤ IMCS ≤27, and if the initial PDSCH for the same transport block is semi-persistently scheduled, the TBS shall be determined from the most recent semi-persistent scheduling assignment PDCCH. else if Table 5.1.3.1-4 is used and , - the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using 0 ≤ IMCS ≤27. If there is no PDCCH for the same transport block using 0 ≤ IMCS ≤27, and if the initial PDSCH for the same transport block is semi-persistently scheduled, the TBS shall be determined from the most recent semi-persistent scheduling assignment PDCCH. else - the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using 0 ≤ IMCS ≤28. If there is no PDCCH for the same transport block using 0 ≤ IMCS ≤28, and if the initial PDSCH for the same transport block is semi-persistently scheduled, the TBS shall be determined from the most recent semi-persistent scheduling assignment PDCCH. Unchanged parts are omitted

- Conclusions

In this contribution, we provide our views on supporting of DL 1024QAM for NR FR1. Based above discussion, the following proposals were made:

Proposal 1: For DL 1024QAM CQI table for NR FR1, support to reuse 1024QAM CQI table defined in LTE with the entry {853, 8.3321} replaced by {853, 8.3301}.

Proposal 2: For DL 1024QAM MCS table for NR FR1,

- Support to design 1024QAM MCS table based on NR 256QAM MCS table.

- Support to remove NR 256QAM MCS table entries {6, 8, 10, 12, 14}.

- Support to introduce the following 1024QAM entries.

| MCS Index |  | Modulation Order | Target code Rate R x [1024] |  | Spectral |
| --- | --- | --- | --- | --- | --- |
| IMCS |  | Qm |  |  | efficiency |
| 23 |  | 10 | 805.5 |  | 7.8662 |
| 24 |  | 10 | 853 |  | 8.3301 |
| 25 |  | 10 | 900.5 |  | 8.7939 |
| 26 |  | 10 | 948 |  | 9.2578 |
| 31 | 10 |  |  | reserved |  |

Proposal 3: Don’t support 1024QAM MCS table for PDSCH scheduled by DCI format 1_2.

Proposal 4: Support of DL 1024QAM is reported per FSPC optionally for NR FR1 only.

Proposal 5: Support following text proposal for TBS for TS 38.214.

#### 5.1.3.2 Transport block TS38.214 V16.4.0 #### 5.1.3.2 Transport block size determination In case the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled, then one of the two transport blocks is disabled by DCI format 1_1 if IMCS = 26 and if rvid = 1 for the corresponding transport block. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword. For the PDSCH assigned by a PDCCH with DCI format 1_0, format 1_1 or format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or SI-RNTI, if Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤27, else if Table 5.1.3.1-4 is used and 0 ≤ IMCS ≤27, or a table other than Table 5.1.3.1-2 and Table 5.1.3.1-4 is used and 0 ≤ IMCS ≤28, the UE shall, except if the transport block is disabled in DCI format 1_1, first determine the TBS as specified below: … else if Table 5.1.3.1-2 is used and , - the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using 0 ≤ IMCS ≤27. If there is no PDCCH for the same transport block using 0 ≤ IMCS ≤27, and if the initial PDSCH for the same transport block is semi-persistently scheduled, the TBS shall be determined from the most recent semi-persistent scheduling assignment PDCCH. else if Table 5.1.3.1-4 is used and , - the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using 0 ≤ IMCS ≤27. If there is no PDCCH for the same transport block using 0 ≤ IMCS ≤27, and if the initial PDSCH for the same transport block is semi-persistently scheduled, the TBS shall be determined from the most recent semi-persistent scheduling assignment PDCCH. else - the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using 0 ≤ IMCS ≤28. If there is no PDCCH for the same transport block using 0 ≤ IMCS ≤28, and if the initial PDSCH for the same transport block is semi-persistently scheduled, the TBS shall be determined from the most recent semi-persistent scheduling assignment PDCCH. Unchanged parts are omitted

# References

[1] RP-202886, 3GPP TSG RAN Meeting #90e, December 7 – 11, 2020

[2] RAN WG1 #103 e-meeting Chairman’s Notes.

[3] 3GPP TS 36.213

[4] 3GPP TS 38.214

# Appendix

Table A.1. Link-level evaluation assumptions

| Parameter | Value |
| --- | --- |
| Channel model | AWGN |
| Antenna configuration | 1T1R |
| Subcarrier spacing | 15 kHz |
| Rank | 1 |
| Channel estimation | real |
| Tx EVM | 0% |
| Rx EVM | 0% |
| Coding Scheme | NR LDPC |
| DMRS type | DM-RS type 1 |
| Number of DMRS symbols | 2 |
| Number of scheduled RBs | 10 |
| PDSCH mapping | Type A, Start symbol 3, Duration 10 |

Table A.2. System-level evaluation assumptions

| Parameter | Value |
| --- | --- |
| Scenarios | Scenario-1: Indoor Hotspot, 12 nodes in 50 m x 120 m |
| Carrier frequency | 4GHz |
| Bandwidth | 100 MHz, 1.72% Guard Band |
| Subcarrier spacing | 30kHz |
| Frame structure | DDDSU (S: 10D:2G:2U) |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32T: (4,4,2,1,1;4,4), (dH,dV) = (0.5, 0.5)λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | For 4R: (1,2,2,1,1;1,2), (dH, dV)=(0.5, N/A)λ |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |
| Min. BE-UT distance(2D) | 0 m |
| ISD | 20 m |
| BS height | 3 m |
| UE height | 1.5 m |
| Channel model | 3D channel model in TR 38.901 |
| BS Power | 31 dBm |
| Noise Figure | BS:5 dB, UE: 9 dB |
| Scheduler | MU-MIMO Proportional Fair |
| Max MCS | 256QAM \ 1024QAM |
| Device deployment | 100% indoor |
| Down-tilt | 90 degrees |
| UE speed | 3 km/h |
| Traffic model | Periodic traffic: Mean Pavg =104000 Bytes, STD=13000 Bytes, PMax=162500 Bytes, PMin=67 Bytes, Period=16.67 ms |
| PDB | 10ms |
| UE Number per cell | 4 |