3GPP TSG RAN WG1 Meeting #104-e	R1- 2100490

e-Meeting, January 25th – February 5th, 2021

Source:		SoftBank

Title:	Target of PUSCH Msg.3 coverage enhancements

Agenda Item:	8.8.3

Document for:	Discussion and Decision

Introduction

A new WI on NR coverage enhancement was approved in RP-202928 at RAN#90e.

The objective of this work item is to specify enhancements for PUSCH, PUCCH and Msg3 PUSCH for both FR1 and FR2 as well as TDD and FDD.

The detailed objectives of the work item are as follows:

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots.

  - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4 [RAN1, RAN4]

      - Potential optimization of DMRS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

- Specification of PUCCH enhancements [RAN1, RAN4]

  - Specify signaling mechanism to support dynamic PUCCH repetition factor indication [RAN1]

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

- Specify mechanism(s) to support Type A PUSCH repetitions for Msg3 [RAN1]

It is clearly mentioned that increasing the maximum number of repetitions up to a number to be determined during the course of the work in the first objective. However, noting is described for the number of PUSCH repetitions for Msg.3. This contribution discusses the necessary number of repetition for PUSCH Msg.3.

Discussion and proposal

The summary of the evaluations performed in the SI is available from the following link (https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_103-e/Inbox/drafts/8.8.1.1/6th_round/FR1-representative-values_r3.xlsx, see R1-2009809 for more details). From the results for service dependent requirements, it is revealed that the additional gain of 4.49 dB for PUSCH Msg.3 is necessary to achieve the target MCL value of -139.2dB.

As claimed during the SI phase, it is very important to achieve sufficient voice coverage compared with 3G services. Therefore, we propose the following:

Proposal

Support at least 2 and 4 repetitions for Type A PUSCH repetitoins for Msg.3