3GPP TSG RAN WG1 #104-e	R1-2100494

e-Meeting, January 25th – February 5th, 2021

Agenda item:	8.15.1

Source: 	Zhejiang Lab

Title: 	Scenarios applicable to NB-IoT/eMTC

Document for:	Discussion and Decision

Introduction

A new study WID on NB-IoT/eTMC support for NTN has been approved in RAN#86 and the following objectives were proposed for RAN1 in [1],

-	Bands of interest in sub 6 GHz

-	Device type with PC3 or PC5 (LEO and GEO)

-	Satellite constellation orbit LEO and GEO

-	Transparent payload.

-	Link budget

-	Aspects related to random access procedure/signals [RAN1, RAN2]

-	Mechanisms for time/frequency adjustment including Timing Advance, and UL frequency compensation indication [RAN1, RAN2]

-	Timing offset related to scheduling and HARQ-ACK feedback [RAN1, RAN2]

-    Aspects related to HARQ operation [RAN2, RAN1]

During RAN1#103e, some agreements for scenarios have been made in [2] as shown in the Appendix, where the main deployment scenarios and parameters have been agreed. In this contribution, we discuss the remaining issues for Scenarios applicable to NB-IoT/eMTC.

Discussion

Evaluation metrics

The proliferation of IoT devices that support a wide range of applications and services has driven the development of 5G and beyond networks to an unprecedented level, requiring massive connectivity, wide coverage and lower power consumption. In [1], a variety of industries have been identified for the application of IoT devices as follows,

-	Transportation (maritime, road, rail, air) & logistics

-	Solar, oil & gas harvesting

-	Utilities

-	Farming

-	Environment monitoring

-	Mining etc.

Since the IoT devices in above industries are often deployed in the remote areas without connection to terrestrial networks, satellites can provide complementary coverage in the remote areas. Necessary enhancements to enable eMBB services in NTN have been discussed and studied in Rel-17, whereas in this study the target is to support IoT device operation in NTN.

As the baseline scenarios and parameters have been agreed in [2], it is essential to define proper metrics to evaluate the performance and justify the necessary enhancements to support IoT devices in NTN. For some of the previously mentioned industries, e.g., transportation, low latency is required for the safety reason and for other exemplary industries, such as environment monitoring, relatively high UE throughout might be needed due to the transmission of vast amount of monitoring date. In this regard, some of the conventional metrics to evaluate eMBB service can also be applied to evaluate the performance of IoT devices with perhaps relatively lower requirements. However, one of the crucial requirements for IoT devices is low power consumption, especially for NTN where the distance between the transmitter and receiver is much longer than that in terrestrial networks. Therefore, power consumption should be taken into consideration for evaluation.

Proposal 1:  The following evaluation metrics should be considered,

- UE throughput;

- Latency;

- Power consumption.

Link budget assumptions

Two parameter sets have been defined in TR 38.821 for S-band as below,

Table 1: Set-1 Parameters

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [3] | Section 6.4.1 in [3] | Section 6.4.1 in [3] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter |  | 250 km | 90 km | 50 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Equivalent satellite antenna aperture | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m | 0.33 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |

Table 2: Set-2 Parameters

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [3] | Section 6.4.1 in [3] | Section 6.4.1 in [3] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture | S-band (i.e. 2 GHz) | 12 m | 1 m | 1 m |
| Satellite EIRP density |  | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |
| 3dB beamwidth |  | 0.7353 deg | 8.8320 deg | 8.8320 deg |
| Satellite beam diameter |  | 450 km | 190 km | 90 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture | S-band (i.e. 2 GHz) | 12 m | 1 m | 1 m |
| G/T |  | 14 dB K-1 | -4.9 dB K-1 | -4.9 dB K-1 |
| Satellite Rx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |
| Equivalent satellite antenna aperture | Ka-band (i.e. 30 GHz for UL) | 1.33 m | 0.13 m | 0.13 m |
| G/T |  | 20 dB K-1 | 5 dB K-1 | 5 dB K-1 |
| Satellite Rx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi |

Two additional sets of parameters were proposed and captured in [4] during the last meeting for more challenging scenarios. In our view, the two sets of parameters in TR38.821, i.e., set 1 and 2 should be the baseline for link budget analysis and two additional sets, i.e., set 3 and 4 can be optional.

Proposal 2:  Two parameter sets in TR 38.821, i.e., set 1 and 2, should be the base line for link budget analysis, and additional sets, i.e., set 3 and 4 can be optional.

Conclusions

In this contribution, we discussed the remaining issues for Scenarios applicable to NB-IoT/eMTC. Our proposals are summarized below.

Proposal 1:  The following evaluation metrics should be considered,

- UE throughput;

- Latency;

- Power consumption.

Proposal 2:  Two parameter sets in TR 38.821, i.e., set 1 and 2, should be the base line for link budget analysis, and additional sets, i.e., set 3 and 4 can be optional.

- References

- RP-193235, New Study WID on NB-IoT/eTMC support for NTN

- RAN1#103e Chairman’s Notes.

- TR 38.811, Study on New Radio (NR) to support non terrestrial networks.

- R1-2008868, “Email summary discussion on Scenarios applicable to NB-IoT/eMTC,” Eutelsat S.A., RAN1#103e.

- Appendix

Agreement:

IoT NTN scenarios A, B, and C are included in the study as shown below:

| NTN Configurations | Transparent satellite |
| --- | --- |
| GEO based non-terrestrial access network | Scenario A |
| LEO based non-terrestrial access network generating steerable beams (altitude 1200 km and 600km) | Scenario B |
| LEO based non-terrestrial access network generating fixed beams whose footprints move with the satellite (altitude 1200 km and 600km) | Scenario C |

Agreement:

The following IoT NTN reference scenario parameters are agreed:

| Scenarios | GEO based non-terrestrial access network - scenario A | LEO based non-terrestrial access network -Scenario B & C |
| --- | --- | --- |
| Orbit type | station keeping a nominally fixed position in terms of elevation/azimuth with respect to a given earth point | circular orbiting at low altitude around the earth |
| Altitude | 35,786 km | 600 km 1,200 km |
| Frequency Range (service link) | < 6 GHz (e.g. 2 GHz in S band) |  |
| Device channel Bandwidth (service link) (NOTE 7) | - NB-IoT 180 kHz (DL), Up to 180 kHz with all permissible smaller resource allocations 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz - eMTC: 1080 kHz (DL), Up to 1080 kHz with all permissible smaller resource allocations , including 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz (UL) |  |
| Payload | Transparent type | Transparent Type |
| Earth-fixed beams | Yes | Scenario B: Yes (steerable beams), see NOTE 1 Scenario C: No (the beams move with the satellite) |
| Max beam foot print size (edge to edge) regardless of the elevation angle | 3500 km (NOTE 3) | 1000 km (NOTE 2) |
| Min Elevation angle for both sat-gateway and C-IoT device | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and C-IoT device at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) |
| Max Round Trip Delay (propagation delay only) | 541.46ms (service and feeder links) | 25.77 ms (600km) (service and feeder links) 41.77 ms (1200km) (service and feeder links) |
| Max differential delay within a cell | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km |
| Max Doppler shift (earth fixed user equipment) (NOTE 6) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) |
| Max Doppler shift variation (earth fixed user equipment) (NOTE 6) | 0.000 045 ppm/s | 0.27 ppm/s (600km) 0.13 ppm/s (1200km) |
| C-IoT device motion on the earth | Min 0 km/s (stationary device), max 120 km/h | Min 0 km/s (stationary device), max 120 km/h |
| C-IoT device antenna types | Omnidirectional antenna with 0 dBi TX antenna gain and 0 dBi RX antenna gain (NOTE 4) |  |
| C-IoT device max Tx power | UE power class 3 with up to 200 mW (23dBm), UE power class 5 with up to 100 mW (20 dBm) |  |
| C-IoT device Noise Figure | Omnidirectional antenna: 7 dB or 9 dB (NOTE 5) |  |
| Service link | 3GPP defined Narrow Band IoT and eMTC |  |

NOTE 1:    Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite.

NOTE 2:   This beam size refers to the Nadir pointing of the satellite.

NOTE 3: The Maximum beam foot print size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation) or a single wide-beam.

NOTE 4: The use of a Circular polarized antenna is optional.

NOTE 5: Same Noise Figure of 7 dB as in Release 16 TR 38.821 or 9 dB as in Release 12 TR 36.888  for device can be assumed for link budget. The noise figure is device vendor implementation specific.

NOTE 6: Max Doppler shift and Max Doppler shift variation in the absence of any device pre-compensation of satellite Doppler shift on the service link.

NOTE 7: System bandwidth is FFS