3GPP TSG RAN WG1 Meeting #104-e                                       R1-2100544

Online, 25th January – 05th February 2021

| Agenda Item: | 8.7.1.1 |
| --- | --- |
| Source: | TCL Communication |
| Title: | Potential Paging Enhancements |
| Documents for: | Discussion and Decision |

Introduction

Potential NR power saving techniques have been studied SI RP-181463 and the report is available in TR 38.840 [2]. The resulting WI RP-191607 [3] focused on reducing PDDCH monitoring in RRC_CONNECTED by specifying a PDCCH-based wake-up signal. In Rel-17, a new WI RP-193239 [1] has been approved with the objective on power saving enhancements for RRC_IDLE/RRC_INACTIVE:

WID RP-193239:

1) Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1]

a) Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1]

- NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #100 meeting

b) Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive- mode UEs, minimizing system overhead impact [RAN1]

- NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

During the last meeting, several agreements were reached concerning the evaluation assumptions and metrics. The potential paging enhancements to study are:

Agreements:

For potential paging enhancements, RAN1 to study the following candidate schemes:

- Paging early indication before a target PO to indicate UE whether to monitor PDCCH scrambled with P-RNTI at the PO. Potential candidate indication methods include

  - DCI-based indication, e.g., based on

    - Extending existing DCI format 1_0 or 2_6

    - New DCI format

  - RS-based or sequence-based indication, e.g., based on TRS/CSI-RS or SSS

- Sub-grouping for paging, based on

  - Legacy paging DCI

  - Paging early indication

  - Additional reception occasions in time/frequency domain

  - Multiple P-RNTIs

- Cross-slot scheduling for paging PDSCH

- Other proposal is not precluded

Agreements: [RAN1#103-e]

For NR idle/inactive-mode paging enhancement, paging early indication before paging occasion is supported from RAN1 perspective

- FFS: Physical layer design based on DCI, SSS or TRS/CSI-RS

- Send LS to inform RAN2 and kindly ask RAN2 to inform RAN1 if there is anything that RAN1 should take into consideration in the physical layer design for this feature, including any other progress RAN2 has made in this WI which may has RAN1 impact

In this document we provide our view on potential paging enhancements.

# Discussion

Power-saving techniques are essential to enable low-power devices like industrial sensors, wearables etc. expected to last up to several years on a single charge. 3GPP has specified a machine-type communication (MTC) UE within the LTE release to allow implementation of such devices. For static and nomadic devices (IoT) like, pressure sensors, 3GPP has specified NB-IoT. However, NR will also introduce a reduced capability (REDCAP) UE with similar battery life requirements but different performance targets. Therefore, power saving enhancements for RRC_IDLE/INACTIVE is essential to enable efficient implementation of REDCAP UEs.

To conserve battery, low-power devices spend significant time in RRC_IDLE/INACTIVE mode, where they utilize discontinued reception (DRX) to turn off the RF and only wake up periodically to listen for eventual paging messages. However, decoding paging messages is complex and consumes a lot of power. Typically, a UE must do the following steps to monitor for paging:

1. UE wakes up before the paging occasion (PO)

2. Turns on RF and baseband

3. Time-frequency synchronization as well as serving cell confirmation

4. Attempts PDCCH decoding for DCI scrambled with P-RNTI

5. If no paging DCI is found, the UE goes back to DRX

This process can take several subframes depending on the potential PDCCH repetition scheme. Therefore, a wake-up signal (WUS) or paging indication (PI) has been introduced in LTE Rel-15 and NB-IoT. The PI is a sequence that is transmitted a configurable time before the paging occasion (PO) and indicates if a paging message is transmitted. Hence, the UE wakes up and searches for the PI. If the PI is detected, the UE wakes up the baseband and starts the procedure described above. If the PI is not detected, the UE goes back to sleep. The advantage is that the PDCCH decoding is only carried out if a paging message is to be received and hence unnecessary PDCCH decoding is avoided. The PI detector can be implemented as a simple correlator.

For RedCap UEs in RRC_IDLE/INACTIVE a PDCCH-based wakeup signal is not adequate since PDCCH detection requires time-frequency synchronization and involves complicated decoding which consumes a lot of power. Therefore, a RS-based wakeup signal is desirable since detection only involves simple correlations and no time-frequency tracking or complicated baseband processing is required.

Proposal 1: Consider a RS-based PI, e.g., based on SSS or TRS/CSI-RS.

A DCI-based PI has the advantage of a larger capacity to include grouping information but requires more power to receive. Thus, we propose to consider both approaches to coexist, RS-based PI for low power devices and DCI-based PI for regular devices.

Proposal 2: Consider coexistence of both RS-based and DCI-based PI.

Alternatively, both RS-based and DCI-based solutions can be used together for group refinement.

Proposal 3: Consider an RS-based PI followed by a DCI-based PI for group refinement.

# Conclusion

In this contribution the following proposals and observations have been made:

Proposal 1: Consider a RS-based PI, e.g., based on SSS or TRS/CSI-RS.

Proposal 2: Consider coexistence of both RS-based and DCI-based PI.

Proposal 3: Consider an RS-based PI followed by a DCI-based PI for group refinement.

# References

[1] RP-193239, “New WI: UE Power Saving Enhancements”

[2] TR-38.840, “Study on User Equipment (UE) power saving in NR”

[3] RP-191607, “New WID: UE Power Saving in NR”