3GPP TSG RAN WG1 Meeting #104-e                                     R1-2100576

e-Meeting, January 25th – February 5th, 2021

Agenda Item: 8.3.2

Source: MediaTek Inc.

Title: On the enhancements for unlicensed band URLLC/IIoT

Document for: Discussion and Decision

# Introduction

During RAN#88-e plenary [1], it was agreed to specify the required UL enhancements for URLLC to operate in unlicensed controlled environment. Specifying support for UE-initiated COT for FBE and harmonizing UL configured-grant enhancements in NR-U and URLLC were particularly emphasized.

# FBE in IDLE/INACTIVE mode

In RAN1#102e [2], it was agreed to support FBE UE-initiated COT in connected mode and still FFS for the idle/inactive mode.

Agreements:

- For semi-static channel access mode, support using the transmission of any scheduled/configured UL channel/signal to initiate a COT by a UE in RRC_CONNECTED mode

  - FFS the case when the UE is IDLE/INACTIVE mode

The question of whether and how to transmit PRACH when an idle mode UE can operate as a COT-initiating device based on semi-static channel access was raised.

On one hand, the focus in Rel-15 and Rel-16 URLLC was the enhancement of latency and reliability in connected mode and idle/inactive mode was not considered for enhancement. The motivation is that UEs operating with long DRX cycle and going frequently to idle/inactive mode are not mostly URLLC UEs since they stay in idle/inactive mode for long periods hence latency is not a priority for these sort of UEs.

On the other hand, some URLLC UEs are battery-powered devices like sensors which go frequently to idle/inactive mode for power saving and enhancing the RACH procedure latency could be very beneficial to reduce the time for the transition between idle/inactive and active mode. In this context, it would be very beneficial to transmit PRACH with UE-initiated COT for latency enhancement.

However, since this functionality is not required for all UEs and it is mainly useful for battery-powered UEs with high priority traffic, it should be configurable and under gNB control.

- The UE is configured to initiate a COT for PRACH transmission.

- E.g. UEs with high Priority traffic or mixed high/low priority traffic could have this functionality enabled by gNB.

If PRACH transmission is allowed to be within a UE-initiated COT, overlapping with the gNB idle period should be discussed, as it is not allowed in Rel-16. Also, sharing with the gNB this particular UE-initiated COT carrying PRACH should be discussed.

The UE-initiated COT carrying PRACH should be shared with the gNB to be able to continue with the RACH procedure. This could be explicitly indicated with an additional information carried by PRACH or directly specified.

- UE-initiated COT carrying PRACH is automatically shared with the gNB without any additional indication.

# UE-to-gNB COT sharing in semi-static channel access

In RAN1#102e [2], the following agreement has been made and the gNB can share a UE-initiated COT following the detection of an UL transmission from the UE starting at the beginning of the FFP:

Agreements:

- UE-to- gNB COT sharing in semi-static channel access mode is supported.

  - The gNB determines a COT in an FFP associated to a UE, that is initiated by the UE, if the gNB detects a UL transmission from the UE starting from the beginning of the FFP and ending before the idle period of the FFP.

    - FFS details

  - When the gNB determines a UE has initiated a COT in an FFP associated to the UE, the gNB can transmit within the FFP and before the idle period corresponding to the FFP.

    - FFS whether/how UE to gNB COT sharing when the gap is >16us

Few questions have been raised regarding the details of this scheme:

- How does the UE know the gNB has determined the UE initiated COT?

- If the detection of an UL transmission is enough for the gNB to decide the UE has initiated a COT?

UE UL transmission is not enough for the gNB to decide the UE has initiated its own COT. During a gNB-initiated COT, the UE may decide to initiate its own COT if it didn’t detect the gNB COT or if it has more UL data in its buffer and the gNB-initiated COT is not enough to transmit all the data. gNB couldn’t differentiate in that case between a UE sharing the gNB COT or a UE transmitting its own data.

| Scenario: - gNB LBT pass. gNB transmits DL data ( including UL grant for UE-1) - UE-1 starts transmitting in UL - UE-2 receives PDU data and wants to transmit to gNB. Starts LBT to initiate a COT and fails - UE-2 does another LBT after the end of UE-1 UL and succeeds. - UE-2 starts UL transmission on Configured Grant. |  |
| --- | --- |

Obviously if it is a CG transmission, the CG-UCI Channel Occupancy Time (COT) sharing information [TS38.212, 6.3.2.1.3	CG-UCI, Table 6.3.2.1.3-1] could be re-used to determine this information. However, this bit-field indicates the UE is sharing the COT and doesn’t indicate if the UE didn’t start its own COT. Hence, there is ambiguity that should be cleared.

- An explicit indication from UE to gNB that UE has initiated a COT

- If the UE has an UL CG transmission and if CG-UCI Channel Occupancy Time (COT) sharing information bit-field is enabled it is interpreted as the UE didn’t start its own COT.

- If the UE has an UL CG transmission and if CG-UCI Channel Occupancy Time (COT) sharing information bit-field is disabled, it is interpreted as the UE started its own COT.

# FFP parameters for UE-initiated COT

Regarding the signaling of the FFP parameters, the following agreement has been made in RAN1#102e [2]:

Agreements:

- For semi-static channel access mode,

- FFP parameters for UE-initiated COT can be provided to the UE by at least dedicated RRC signaling.

  - FFS on to be provided by SIB-1

- FFS whether the UE FFP periodicity is explicitly configured, or implicitly determined based on other higher layer parameters

First, we think reducing contention and especially restricting UEs with low priority traffic occupying the channel and exploiting UE-initiated COT should be further discussed. As one potential option, the UE COT-initiating should be a functionality under gNB control and the gNB should favor UEs with high priority traffic.

- UE COT-initiating functionality is RRC (or dynamically) configured to the UE.

Another possibility is to link the UE COT-initiating to the traffic priority.

- UE COT initiation enabling/disabling is determined from the traffic priority.

Regarding the FFP parameters signaling, according to the agreement above the FFP parameters could be signaled through RRC but it is still FFS if the FFP parameters could be provided by SIB-1. We support FFP parameters signalling in SIB-1 since in Rel-16 FFP parameters can already be provided by SIB-1 or dedicated RRC.

- FFP parameters for UE-initiated COT could be provided by SIB-1.

The second bullet point in the agreement above is about the UE FFP periodicity and if it could be determined implicitly from other higher layer parameters.

We support the use of implicit signalling in case there is no explicit signalling of the FFP periodicity and in that case other higher layer parameters could be used. One possibility is the periodicity of the CG/PRACH resources. The use of higher layer parameters could be overridden by the explicit signalling though.

- UE FFP periodicity determined from higher layer parameters but overridden by explicit dedicated signalling.

# DCI Indication for UE COT-initiation in next FFP

DCI indication to explicitly indicate to the UE whether or not to initiate a COT in a next FFP could have some benefit in giving more control to the gNB to enable/disable UEs especially controlling UEs transmitting low priority traffic.

However it comes with control overhead and the UE may need to acknowledge the reception of this information to avoid confusion between the gNB and the UE. If this DCI is missed, the UE is not be able to initiate a COT when the gNB is expecting it to initiate its own COT and the gNB may have already silenced other UEs to allow this particular UE to access the channel.

Hence, the aim of controlling UE COT-initiating could be done in the specification without including explicit signalling.

Restricting UE COT-initiation to high priority traffic (HP-CG, HP-SR, HP-HARQ-ACK …) is one potential option to explore and for the low priority traffic UEs could rely on gNB initiated COTs.

- No DCI indication for UE COT-initiation in next FFP and restrict UE COT-initiation to high priority traffic.

# COT associated with configured UL transmission

In RAN1#103e [3], the following agreement below has been made regarding the COT indication for configured UL transmission:

Agreements:

In semi-static channel access mode:

- When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down-select one of the following:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

  - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration.

- When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE:

  - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT.

- FFS on other conditions for determining the corresponding UE or gNB initiated COT

- Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

Different alternatives have been listed in the case when a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE. The different alternatives have been listed depending if an ongoing gNB COT should be taken into consideration or not.

Alt-a is prioritizing the sharing of the gNB-initiated COT if the transmission is confined within a gNB FFP before the idle period of that gNB FFP.

In Alt-a, the UE needs some time to detect any gNB DL transmission at the start of the gNB FFP and confirm that the gNB has initiated the COT. Hence having the transmission confined within a gNB FFP is not enough and needs further clarification. The transmission needs to be confined within [gNB_FFP_start + Δ1, gNB_idle_period_start] where Δ1 is the time required to receive and detect the gNB DL transmission at the start of the FFP. Also, the UE may need to parse the DCI to determine if the gNB allows for COT sharing or also if the gNB is not just sharing another UE’s COT.

Hence, the decision about the UE transmission will be based on the result of the DL detection at the start of the gNB FFP. But if the UE fails to detect the gNB DL transmission, it also needs some extra time for the CCA before initiating its own COT. Therefore, an additional time Δ2 is also required for CCA and the transmission needs to be confined within [gNB_FFP_start + Δ1 + Δ2, gNB_idle_period_start]

Alt-b means that the UE will not be able to transmit if the gNB has initiated a COT and it needs to wait to initiate its own COT which goes against the latency reduction objective of Configured Grant transmission. The UE should be allowed to transmit within the gNB-initiated COT exactly in a similar way as if operating in the licensed mode. Tying any configured UL transmission to UE-initiated COT is very restrictive.

- Support Alt-a with further reduction to the transmission confining interval to take UE processing time into consideration.

# Distinction between gNB COT initiation and sharing

Detection of DL signal at the beginning of the gNB FFP is not enough for the UE to distinguish between a gNB-initiated COT or the gNB sharing of another UE’s COT. A DL signal transmitted by the gNB could be sent in another UE-initiated COT. Therefore, a mechanism in place is needed for the UE to differentiate between the gNB initiating its own COT or sharing another UE’s COT. The UE cannot assume a gNB has initiated its own COT if the UE detects the unicast PDCCH at the beginning of gNB FFP.

Rel-16 design is not able to handle this ambiguity and a new differentiation is required. Multiple options have been proposed and most of them require PDCCH decoding and DCI parsing to determine the information which adds to the UE complexity each time it needs to detect a DL signal. The detection could be done in the PDP domain without much complexity and without the need for the PDCCH decoding and the parsing of the DCI by using different phase shifts for the DMRS.

- The gNB signals the initiating of its own COT or the sharing of a UE-initiated COT using different DMRS cyclic shifts.

# UE-initiated COT Cancellation

There is no need to cancel a UE-initiated COT and there is no clear motivation how this could be useful. If the motivation is to transmit a high priority DL traffic, the gNB can always share the UE-initiated COT for this purpose.  If the motivation is to prioritize another UE high priority UL traffic (e.g. following a previous reception of an SR from another UE) then we don’t think this is the best and optimum way to operate. This will add extra signalling overhead and will add to the complexity of the UE implementation to cancel an ongoing COT transmission. In that way, restricting the UE COT initiation to the high priority traffic is one potential solution.

- There is no need to support and specify a UE-initiated COT cancellation

But if the cancellation is to be supported, then a time duration t should be defined. This time duration is required for the UE to process the cancellation signal and prepare for the cancellation of the ongoing transmission.

- If COT cancellation is specified, a time duration t is to be introduced to process the cancellation signal and cancel the ongoing transmission.

If to be defined, the UE could be configured semi-statically (e.g. via RRC) about the time instants when it should cancel an ongoing UE-initiated COT (E.g. gNB FFP start boundary, E.g. another high priority UE FFP start boundary).

- If COT cancellation is specified, the UE should be configured semi-statically about the time instants when it should cancel an ongoing UE-initiated COT.

- If COT cancellation is specified, Cancellation should be defined as a UE capability. The UE signals its support of the cancellation.

# Nested and overlapping COTs

Can the UE initiate a COT within a gNB-initiated COT, and can gNB initiate a COT within a UE-initiated COT? We think both options should be supported and enabled/disabled by configuration. The UE can initiate a COT within a gNB-initiated COT if its UL transmission will overlap with the gNB FFP idle period or if the UE has data in its buffer and needs longer COT to transmit its data and sharing the gNB ongoing COT is not sufficient to transmit the UL data. This should be enabled particularly for UEs having high priority UL traffic to transmit, hence the need for a configuration to enable/disable this functionality.

gNB initiating a COT within a UE COT is also useful for the gNB to transmit and receive data from other UEs, probably with higher priority.

- UE initiating a COT within a gNB-initiated COT, and gNB initiating a COT within a UE-initiated COT is supported and configurable.

- UE initiating a COT within a gNB-initiated COT could be allowed only if UL transmission overlaps with the gNB FFP idle period.

# Conclusion

In this contribution, we have made the following proposals:

- The UE is configured to initiate a COT for PRACH transmission.

- E.g. UEs with high Priority traffic or mixed high/low priority traffic could have this functionality enabled by gNB.

- UE-initiated COT carrying PRACH is automatically shared with the gNB without any additional indication.

- An explicit indication from UE to gNB that UE has initiated a COT

- If the UE has an UL CG transmission and if CG-UCI Channel Occupancy Time (COT) sharing information bit-field is enabled it is interpreted as the UE didn’t start its own COT.

- If the UE has an UL CG transmission and if CG-UCI Channel Occupancy Time (COT) sharing information bit-field is disabled, it is interpreted as the UE started its own COT.

- UE COT-initiating functionality is RRC (or dynamically) configured to the UE.

- UE COT initiation enabling/disabling is determined from the traffic priority.

- FFP parameters for UE-initiated COT could be provided by SIB-1.

- UE FFP periodicity determined from higher layer parameters but overridden by explicit dedicated signalling.

- No DCI indication for UE COT-initiation in next FFP and restrict UE COT-initiation to high priority traffic.

- Support Alt-a with further reduction to the transmission confining interval to take UE processing time into consideration.

- The gNB signals the initiating of its own COT or the sharing of a UE-initiated COT using different DMRS cyclic shifts.

- There is no need to support and specify a UE-initiated COT cancellation

- If COT cancellation is needed, a time duration t is to be introduced to process the cancellation signal and cancel the ongoing transmission

- If COT cancellation is needed, the UE should be configured semi-statically about the time instants when it should cancel an ongoing UE-initiated COT.

- If COT cancellation is needed, Cancellation should be defined as a UE capability. The UE signals its support of the cancellation of an ongoing UE-initiated COT.

- UE initiating a COT within a gNB-initiated COT, and gNB initiating a COT within a UE-initiated COT is supported and configurable.

- UE initiating a COT within a gNB-initiated COT could be allowed only if transmission overlaps with the gNB FFP idle period.

# References

- RP-201310 “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR” Nokia, Nokia Shanghai Bell, 3GPP TSG RAN Meeting #88e.

- Chairman’s Notes RAN1#102e.

- Chairman’s Notes RAN1#103e.