3GPP TSG RAN WG1 Meeting #104-e	R1-2100581

E-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.9.1

Source:	Mediatek

Title:	Consideration on CQI report and Repetition applicability for 16QAM in R17

Document for:	Discussion and Decision

Introduction

In the RAN1 103-e meeting, following agreements have been captured in [1].

Working Assumption

- The following TBS indices are introduced for downlink

| I_TBS | I_SF |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | [552, 536] | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | [328, 296] | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | [2472, 2536] | 2984 | 4008 | 4968 |

- FFS: Support of legacy TBS indices with 16-QAM at least for some deployment modes.

- FFS: Mapping of (a subset of) TBS entries to modulation schemes for different deployment modes.

- FFS for I_SF > 7

Agreement

Repetitions larger than 2 are not supported in case of 16QAM for downlink

- FFS: Whether repetition of 2 is supported or not

In this contribution, first we investigate the MCS table and corresponding CQI report mechanism, secondly we restate our viewpoint on applicability of repetition for 16QAM downlink.

MCS table design and CQI report

MCS table design

According to above WA, there are 8 extended TBS indices originating from legacy LTE TBS table. This requires extending MCS field in DCI from 4 bits to 5 bits. Comparing to 4 bits MCS field solution (i.e. down-sampling solution), extending solution has more MCS indices and make more precise scheduling possible. On the other hand, as stated by Figure 3 in [3], the increased 1 bit DCI size has negligible impact on the NPDCCH decoding performance. Furthermore, we can limit the extended DCI to USS and associate it with 16QAM configuration in RRC signaling. Thus there is no problem on DCI blind decoding and backward compatibility.

Proposal1：The size of the MCS field in DCI N1/N0 in UE-specific search space is increased to 5 bits.

CQI report for Downlink 16QAM

The importance of CQI report for Downlink 16QAM scheduling is obvious. As we know, channel quality report can be put in Msg3 since R16 and the channel quality corresponds to the repetition achieving 1% BLER for the NPDCCH. However this metric isn’t suitable for 16QAM because it has no repetition. To solve this, [4] put forwards a solution to add 3 additional indices in Table 9.1.22.15-1 in 36.133. Thus R17 UEs can report one of these 3 indices in Msg3, or in AS RAI MAC upon receiving DCQR. If the modification to CQI report mechanism stops here, we have to face 2 obvious drawbacks, the first one is the CQI indices are too less to cover all 8 MCS indices, the second is the report latency is too large due to MAC CE method is based on Layer 2. If our design bases on suggested 5 bits MCS field in DCI, there will be a better solution. That is we can design a Layer1 solution in addition to Layer2 solution. For 5 bits MCS field, the 23 ~31 indices are spare, we can define the 31st index (0x11111) as a mark which indicates the DCI N0 is a request command for CQI report. When UE receives this specific DCI N0, it should provide the CQI report in the corresponding NPUSCH. Figure1 is the so called Layer1 CQI report diagram. For convenient comparison, Figure2 illustrates the legacy Layer2 CQI report diagram.

Figure1. Layer1 CQI report diagram

Figure2. Legacy Layer2 CQI report diagram

Combining the proposal1, to implement layer1 solution, the MCS table should be like Table1.Considering the CQI is just 3bits, TBS Index 0 and QPSK are enough to carry it.

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 2 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | 2 | 11 |
| 12 | 2 | 12 |
| 13 | 2 | 13 |
| 14 | 4 | 14 |
| 15 | 4 | 15 |
| 16 | 4 | 16 |
| 17 | 4 | 17 |
| 18 | 4 | 18 |
| 19 | 4 | 19 |
| 20 | 4 | 20 |
| 21 | 4 | 21 |
| reserved | … | … |
| 31 | 2 | 0 |

Table 1: DL MCS table for NB-IOT 16QAM

Accordingly the mapping between MCS index and CQI index is illustrated by Table 2.

| MCS Index | Modulation Order | CQI Index |
| --- | --- | --- |
| 14 | 4 | 0 |
| 15 | 4 | 1 |
| 16 | 4 | 2 |
| 17 | 4 | 3 |
| 18 | 4 | 4 |
| 19 | 4 | 5 |
| 20 | 4 | 6 |
| 21 | 4 | 7 |

Table 2: CQI index table

Except for the MCS field, the remaining field of DCI N0 keeps the same as before. Thus CQI report can share the same scheduling procedure as legacy NPUSCH including retransmission. Above discussion is just an example for the SA/GB operation mode. There will be a simple adjustment in Table1/Table2 for IB operation mode if RAN1 accept this Layer1 solution. Layer1 solution has very low latency than Layer2, We can find the latency of Layer1 solution is just 16ms, is less than half of Layer2 solution i.e. 40ms as illustrated by following Figure.

Proposal2: Introduce Layer1 CQI report method by utilizing spare index of MCS field (5 bits) in DCI N0.

Applicability of repetition for 16QAM

As we know, besides the time diversity gain, the repetition method essentially is an exchange between power/time resource and SNR. Here we consider two possible scenarios for repetition, one is using repetition-16QAM to replace normal 16QAM, and the other one is using repetition-16QAM to replace normal QPSK. Considering most of NB-IOT application scenarios are low speed and there is no fading channel, we did simulation based on approved simulation consumption i.e. AWGN. We select 3 typical cases to compare and Table 3 is the configuration detail, Figure 3/4/5 are the results. Here are 3 temporary definitions for convenience:

- Repetition-16QAM: with repetition

- Normal 16QAM: without repetition

- Normal QPSK: without repetition

|  | With 1 Repetition | without repetition | without repetition |
| --- | --- | --- | --- |
| Figure3 | 16QAM TBS=488 bits RU length=1 sub-frame Total time resource=2 ms | 16QAM TBS=552 bits RU length=2 sub-frames Total time resource=2 ms | QPSK TBS=488 RU length=2 sub-frames Total time resource=2 ms |
| Figure4 | 16QAM TBS=1000 RU length=2 sub-frames Total time resource=4 ms | 16QAM TBS=1128 RU length=4 sub-frames Total time resource=4 ms | QPSK TBS=1000 RU length=4 sub-frames Total time resource=4 ms |
| Figure5 | 16QAM TBS=2472 RU length=5 sub-frames Total time resource=10 ms | 16QAM TBS=2856 RU length=10 sub-frames Total time resource=10 ms | QPSK TBS=2536 RU length=10 sub-frames Total time resource=10 ms |

Table 3: Typical cases for repetition investigation

Figure3: BLER on 1ms and 2ms RU

Figure4: BLER on 2ms and 4ms RU

Figure5: BLER on 5ms and 10ms RU

From above simulation, we can observe that the repetition-16QAM is worse more than normal QPSK. Even comparing to normal 16QAM which has larger TBS (means higher code rate), the repetition-16QAM still be worse more than 1 db.

Proposal3: no repetition for 16QAM.

Conclusions

According to above discussion, we find that extending MCS field in DCI is simple and better method to support 16QAM and is suitable to support Layer1 CQI report method to decrease the report latency. Then we restate our viewpoint on applicability of repetition for 16QAM downlink.

Proposal1: The size of the MCS field in DCI N1/N0 in UE-specific search space is increased to 5 bits.

Proposal2: Introduce Layer1 CQI report method by utilizing spare index of MCS field (5 bits) in DCI N0.

Proposal3: no repetition for 16QAM.

- References

- “https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_103-e/Report/Draft_Minutes_report_RAN1%23103-e_v020.zip.

- R1-2005648, “Considerations on support of 16QAM in NB-IOT”, Mediatek

- R1-2008073,”Support of 16-QAM for NB-IoT”, Nokia, Nokia Shanghai Bell.

- R1-2008930,” Support of 16-QAM for unicast in UL and DL in NB-IoT Support of 16-QAM for unicast in UL and DL in NB-IoT”, Ericsson.