3GPP TSG RAN WG1 Meeting #103e  	R1-2009304

October 26th–November 13th, 2020

Agenda Item: 8.15.1

Source: MediaTek Inc.

Title: 8.15.2  IoT NTN Scenarios

Document for: Discussion

# Introduction

At the RAN#86 meeting, a new Study Item was approved for IoT Non Terrestrial Network (NTN) [1]. In this contribution, we provide link budget analysis for C/N for DL and UL using two satellite system parameter sets. We considered NB-IoT and eMTC in the analysis.

# IoT NTN Scenarios

IoT NTN scenarios are discussed in a separate agenda item. The main focus for RAN1 is the link budget analysis based on FDD, transparent payload, and frequency band below 6 GHz. The bands of interest on access link are the L band and S band. Two sets of parameters for link budget calculations for NB-IoT NTN based on [3, 4] are considered. We denote these sets of parameters as Set 3 and Set 4 based on Feature Lead recommendations in [5].

The link budget analysis aim to identify whether assumption for IoT NTN satellite with low EIRP and low G/T can be compatible with a reasonable working C/N on the DL and UL to allow typical IoT NTN operations. We show the list of cases for link budget in Table 1. The corresponding C/N values for DL and UL for NB-IoT are shown in Table 2. The assumptions for the free space path loss, atmospheric loss, shadow fading margin, scintillation loss, and polarisation loss are shown in Table 3 in ANNEX.

| Case | Satellite orbit | Parameter Set | Central beam elevation | Terminal | Frequency band | RAT |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | Set 3 | 12.5 deg | CIoT | S-band | NB-IoT |
| 2 | LEO-600 km | Set 3 | 30 deg | CIoT | S-band | NB-IoT |
| 3 | LEO-600 km | Set 4 | 30 deg | CIoT | S-band | NB-IoT |
| 4 | LEO-1200 km | Set 3 | 30 deg | CIoT | S-band | NB-IoT |

Table 1: List of Cases for Link Budget

| Cases | EIRP Density | EIRP per spot | DL C/N | G/T | UL C/N 3.75 kHz / 15 kHz / 3*15 kHz / 6*15 kHz / 180 kHz |
| --- | --- | --- | --- | --- | --- |
| 1 | 59.8 dBW/MHz | 84.4 dBm | -2.2 dB | 16.7 dB/K | 0.6 dB / -5.4 dB / -10.2 dB / -13.2 dB / -16.2 dB |
| 2 | 28.3 dBW/MHz | 50.9 dBm | -2.1 dB | -12.8 dB/K | 2.7 dB / -3.4 dB / -8.1 dB / -11.1 dB / -14.1 dB |
| 3 | 44.0 dBW/MHz | 44 dBm | -12.0 dB | -12.8 dB/K | -2.4 dB / -8.5 dB / -13.2 dB / -16.2 dB / -19.2 dB |
| 4 | 33.7 dBW/MHz | 56.3 dBm | -2.1 dB | -12.8 dB/K | -2.7 dB / -8.7 dB / -13.5 dB / -16.5 dB / -19.5 dB |

Table 2: Link Budget results

Cellular NB-IoT can support minimum performance requirement as follows:

- NPDSCH and NPDCCH with SNR = -10.2 dB and SNR=-11.4 dB with 256 and 1024 repetitions respectively on non-anchor carrier (TS 36.101 Table 8.12.1.1.2-2 and Table 8.12.2.1.1-1).

- NPBCH can be supported with minimum performance requirement with SNR=-11.5 dB (TS 36.101 Table 8.12.3.1.2.1-1).

- NPUSCH Format 1 and NPUSCH Format 2 with SNR = -12.2 dB and SNR=-10.9 dB with 64 repetitions respectively on non-anchor carrier (TS 36.104 Table 8.5.1.1.1-1 and Table 8.5.2.2.1-1).

- NPRACH can be supported with minimum performance requirement with SNR=-6.8 dB (TS 36.104 Table 8.5.3.2.1-1).

NPBCH uses a fixed schedule with a periodicity of 640 ms and repetitions made within 640 ms. The first transmission of the MIB-NB is scheduled in subframe #0 of radio frames for which the SFN mod 64 = 0. Repetitions are scheduled in subframe #0 of all other radio frames. The transmissions are arranged in 8 independently decodable blocks of 80 ms duration. In standalone deployment, NPBCH performance could be improved by 1.84 dB by including the first three symbols in the Release-8 control region and the REs corresponding to the 4 CRS ports. However, this solution does not seem necessary as would potentially be beneficial for case 3, but may already be achievable by most UE implementations which typically have some margin to meet the test requirements.

A UE can work at lower SNR than that shown above as NPDSCH can be scheduled with up to 2048 repetitions, NPDCCH with up to 1024, NPUSH with up to 128, NPRACH with up to 1024. Hence, the DL SNRs and UL SNRs required for the cases as shown in Table 2 should be well achievable. On the UL, the eNB may schedule UL transmission with an UL channel bandwidth of 3.75 kHz, or with 15 kHz or 3*15 kHz, with the needed number of repetitions.

Observation 1: NB-IoT can support minimum performance requirement for NPDSCH, NPDCCH, NPUSCH format 1 and 2, NPRACH for NB-IoT NTN Set 3 and Set 4 for cases 1, 2, 3, and 4 by using specified range of repetitions.

Observation 2: NB-IoT can support minimum performance requirement for NPBCH.

Observation 3: It is up to the eNB UL scheduler to select the sub-carrier spacing and UL channel bandwidth with the required number of repetitions to transmit a TBS on NPUSCH or to transmit HARQ feedback on NPUSCH format 2.

Proposal 1: List of Cases for Link Budget in Table 1 in R1-2100600 is used as working assumption for NB-IoT NTN

Proposal 2: Link Budget results for case 1, 2, 3, and 4 in Table 2 and losses in Table 3 in R1-2100600 are included in TR 36.763

# Conclusion

In this contribution, we summarize observations for the link budget analysis and user density analysis for IoT NTN for LEO and GEO.

Observation 1: NB-IoT can support minimum performance requirement for NPDSCH, NPDCCH, NPUSCH format 1 and 2, NPRACH for NB-IoT NTN Set 3 and Set 4 for cases 1, 2, 3, and 4 by using specified range of repetitions.

Observation 2: NB-IoT can support minimum performance requirement for NPBCH.

Observation 3: It is up to the eNB UL scheduler to select the sub-carrier spacing and UL channel bandwidth with the required number of repetitions to transmit a TBS on NPUSCH or to transmit HARQ feedback on NPUSCH format 2.

Proposal 1: List of Cases for Link Budget in Table 1 in R1-2100600 is used as working assumption for NB-IoT NTN

Proposal 2: Link Budget results for case 1, 2, 3, and 4 in Table 2 and losses in Table 3 in R1-2100600 are included in TR 36.763

# ANNEX

Path loss modelling is according to TR 38.821 in Table 6.1.3.3-1. As there were different views from companies on Free Space Loss, Atmospheric loss, Shadow margin, Polarisation loss, Additional losses, we used mainly worst case assumptions summarized in table below:

|  | GEO 35786 km | LEO 1200 km | LEO 600 km |  |
| --- | --- | --- | --- | --- |
| FSPL | 190.6 | 164.5 | 159.1 | dB |
| Scintillation losses | 2.2 | 2.2 | 2.2 | dB |
| atmospheric losses | 0.2 | 0.1 | 0.1 | dB |
| polarization loss | 3 | 3 | 3 | dB |
| shadow margin | 3 | 3 | 3 | dB |
| sum of all losses | 198.9 | 172.8 | 167.4 | dB |

Table 3: Losses in IoT NTN scenarios

# References

- RP-193235, “New Study WID on NB-IoT/eTMC support for NTN”, MediaTek, RAN#88-e, June 2020.

- TR 38.821 “Study on solutions for NR to support non-terrestrial networks”

- R1-2008815, Eutelsat, Inmarsat, Mediatek, Ligado, Thales, Hughes/EchoStar, ESA, Thales, Intelsat, Reference Link-Budget parameters for NB-IoT NTN, RAN1#103e, Nov 2020

- R1-2009098, Gatehouse, Sateliot, Discussion on scenarios applicable to IoT NTN, RAN1#103e, Nov 2020.

- R1-2008868, Eutelsat, Summary #4 of 8.15.1 IoT NTN Scenarios, RAN1#103e, Nov 2020.