3GPP TSG RAN WG1 #104-e	R1-21000625

e-Meeting, January 25 – February 5, 2021

Agenda item:	8.6.1

Source:	NEC

Title: 	Discussion on RedCap features

Document for:	Discussion and Decision

Introduction

In RAN1#103-e, RAN1 part of Study on support of reduced capability NR devices was completed, and in RAN#90, a new WI on support of reduced capability NR devices was approved [1]. Some features are still open whether/how they are captured in the WID and will be discussed and decided in RAN#91, e.g. number of Rx antenna ports for bands where “the UE is required to be equipped with a minimum of four Rx antenna ports”, support of maximum one layer for RedCap UE with two Rx antenna ports, etc.

In this contribution, we discuss further on some aspects within the scope of RedCap WI or still under study.

Discussion

Early identification

It was observed during study that early identification of RedCap UE type(s) e.g. during msg1 would be needed for coverage recovery.

There could be cell deployments where a serving cell has a good coverage, even for RedCap UE with reduced Rx antenna port(s) (or branches). For such a serving cell, coverage recovery for RedCap UE would be unnecessary.

Coverage recovery would also be unnecessary for RedCap UE which is equipped with two antenna ports for bands where “the UE is required to be equipped with a minimum of two Rx antenna ports” and would be expected to meet the requirements on receivers for non-RedCap UE specified in RAN4 specifications [3][4].

Observation:

- Necessity of early identification of RedCap UE type(s) intended for coverage recovery would depend on cell deployments

Observation:

- Early identification of RedCap UE type(s) intended for coverage recovery would be unnecessary for RedCap UE equipped with two antenna ports for bands where “the UE is required to be equipped with a minimum of two Rx antenna ports”

In RAN1#103-e, there was an agreement as follows:

Agreements:

- If early identification during initial access is supported, at least maximum supported UE BW during initial access is included in the set of L1 capabilities of the device type for RedCap early identification

  - Note: 20 MHz for FR1 and 100 MHz for FR2

  - FFS other L1 capabilities

Note: This does not preclude the case where the early indication only indicates whether it is a Redcap UE or which type of the Redcap UEs if multiple UE types are defined

On the other hand, there would be a case where maximum supported UE BW is not necessary for early identification, e.g. in case CBW (channel bandwidth) of a serving cell is ≤ 20MHz in FR1 or ≤ 100MHz in FR2 which are mandatorily supported by RedCap devices.

Observation:

- Necessity of early identification with regard to reduced BW would depend on CBW of the serving cell

As seen above, necessity of early identification of RedCap type(s) could depend on cell deployment (e.g. coverage, CBW).

Proposal:

- If early identification of RedCap type(s) is supported, it should be left for network deployment whether early identification is used

Rx antenna ports

Reduction of Rx antenna ports is supported for bands where “the UE is required to be equipped with a minimum of two Rx antenna ports”. It is still open until RAN#91 for bands where “the UE is required to be equipped with a minimum of four Rx antenna ports”.

In NR, number of Rx antenna ports itself is invisible from network while supported MIMO layer is indicated to the network per CC. Instead of indicating number of Rx antenna ports, requirements on UE receiver, minimum number of Rx antenna ports (branches) and receiver performance, are specified per band-basis [3][4].

For bands where “the UE is required to be equipped with a minimum of two Rx antenna ports”, one and two antenna ports (branches) are supported for RedCap UE, i.e. both RedCap UE with or without Rx antenna port reduction are supported.

For RedCap UE without Rx antenna reduction, it would be possible to adopt the existing specification for non-RedCap UE with regard to minimum number of Rx antenna ports and performance specified in RAN4 specifications as mentioned above.

Observation:

- Indication of the feature of “reduced Rx antenna ports” is unnecessary for RedCap UE with two antenna ports for bands where “the UE is required to be equipped with a minimum of two Rx antenna ports”

For RedCap UE with reduced number of Rx antenna ports, receiver performance may be degraded and relaxation of RAN4 specification with regard to UE receiver performance may be needed.

Observation:

- Relaxation of RAN4 specification with regard to UE receiver performance may be needed

MIMO

In NR, the following capability signaling is supported for DL MIMO:

4.2.7.6	FeatureSetDownlinkPerCC parameters

| maxNumberMIMO-LayersPDSCH Defines the maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signaling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE does not support MIMO on this carrier. | FSPC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

For RedCap UE with two Rx antenna ports, this capability signaling can be reused to indicate support of maximum two layers. If capability of maximum one layer is supported for RedCap UE with two Rx antenna branches, the capability can be indicated to the network by absence of this capability.

For RedCap UE with one Rx antenna branch, absence of this capability may be reused for indication of support of maximum one layer for DL.

For UL, existing capability signaling for MIMO can be reused.

Observation:

- Existing capability signaling of maximum number of MIMO layers can be reused for RedCap UE

No further reduced capability on MIMO other than reduced capability from reduced number of branches seems necessary, e.g. if DM-RS antenna ports are restricted, network flexibility would be heavily degraded.

Observation:

- No further restriction on MIMO would be needed for RedCap UE

Definition of the RedCap UE type

In RAN1#103-e, there was an agreement as follows:

Agreements:

- At least for RedCap UE identification, explicit definition of RedCap UE type(s) is needed. Pending conclusions on the reduced complexity features in AI8.6.1 and RedCap UE identification in AI8.6.5, the definition of the RedCap UE types can be based on one of:

  - Option 1: All the reduced capabilities recommended at the end of the RedCap study

  - Option 2: Only include the reduced capabilities that the network needs to know during initial access, if any

  - Option 3: All the recommended reduced capabilities as well as recommended power saving features

  - Option 4: The corresponding minimum set of the reduced capabilities that one RedCap UE type shall mandatorily support

FFS for other usages

On option 1 and option 3:

These options request “all” recommended reduced capability but would not be appropriate because RedCap UE is not mandated to implement all recommended reduced complexity features. For example, RedCap UE supports two Rx antenna ports (branches) for bands where “the UE is required to be equipped with a minimum of two Rx antenna ports”, i.e. no reduced capability with regard to number of Rx antenna ports (branches). These two options are unable to cover RedCap UE which is only implemented with part of recommended reduced capability features.

On option 2:

There would be case(s) where the network may want to identify RedCap UE if its coverage is not enough for RedCap UE, CBW/initial BWP is wider than 20MHz in FR1/100MHz in FR2, etc. On the other hand, there would be also case(s) where the network does not need to identify RedCap UE, e.g. a case where a serving cell has good coverage, or CBW of the serving cell is ≤ 20 MHz in FR1 or ≤ 100 MHz in FR2. With this option, definition may depend on cell deployments.

On option 4:

RedCap UE shall support BW of 20 MHz in FR1 or 100 MHz in FR2 including lower nominal BW. But for the cell whose CBW is ≤ 20MHz in FR1 or 100MHz in FR2, this reduced capability indication does not seem essential. Other reduced capabilities within the scope of RedCap work item, e.g. Rx antenna ports reduction, would not be mandatory.

Observation:

- For explicit definition of RedCap UE type option 1 and option 3, which request “All” recommended reduced capability, do not seem valid after completion of RAN1 study because it is allowed RedCap UE does not implement part of recommended reduced capability, e.g. number of reduced Rx antenna ports

Proposal:

- Further study explicit definition of RedCap UE type(s) for RedCap UE identification between option 2 and 4

Multiple BWP support

Support of multiple BWP by RedCap UE was discussed under power saving enhancement agenda. Support of multiple BWP may increase complexity of RedCap UE, though it would not be clear how much cost/power consumption of RedCap UE would increase as it was not evaluated during study. On the other hand, necessity of inhibiting RedCap UE to support multiple BWP would be unclear and could be left for implementation.

Observation:

- Support of multiple BWP could be optional for RedCap UE

Power saving

DRX settings suitable for traffic pattern of RedCap use cases would work well for reduction of power consumption of RedCap devices, while RedCap UE may be equipped with power saving features in Rel-16/17, e.g. for use cases where traffic pattern would be non-uniform.

Some Rel-16 power saving features, such as cross slot scheduling and MIMO layer adaptation, require multiple BWP support.

For some use cases of RedCap devices, traffic pattern would be expected uniform and would not change dynamically. Therefore, semi-static configuration/adoptation of power saving feature within active BWP would be reasonable. It would be preferable not to require support of multiple BWP for the sake of support of power saving feature.

Observation:

- Semi-static adoption of power saving feature within active BWP would be reasonable for some use cases of RedCap devices

If MIMO layer adaptation for power saving would be useful for RedCap UE for power saving, i.e. RedCap UE which is equipped with two Rx antenna ports and support maximum two layers may indicate one layer for power saving, it would be useful to allow RedCap UE equipped with two Rx antenna ports to support maximum one layer as capability.

Observation:

- It could be useful for power saving that RedCap UE with two Rx Antenna branches may support maximum one layer in DL, if MIMO layer adaptation for power saving would be expected useful for the RedCap UE

Summary

On early identification, we made the following observations and proposal:

Observation:

- Necessity of early identification of RedCap UE type(s) intended for coverage recovery would depend on cell deployments

Observation:

- Early identification of RedCap UE type(s) intended for coverage recovery would be unnecessary for RedCap UE equipped with two antenna ports for bands where “the UE is required to be equipped with a minimum of two Rx antenna ports”

Observation:

- Necessity of early identification with regard to reduced BW would depend on CBW of the serving cell

Proposal:

- If early identification of RedCap type(s) is supported, it should be left for network deployment whether early identification is used

On Rx antenna ports, we made the following observations:

Observation:

- Indication of the feature of “reduced Rx antenna ports” is unnecessary for RedCap UE with two antenna ports (branches) for bands where “the UE is required to be equipped with a minimum of two Rx antenna ports”

Observation:

- Relaxation of RAN4 specification with regard to UE receiver performance may be needed

On MIMO, we made the following observations:

Observation:

- Existing capability signaling of maximum number of MIMO layers can be reused for RedCap UE

Observation:

- No further restriction on MIMO would be needed for RedCap UE

On definition of the RedCap UE type at least for identification, we made the following observation and proposal:

Observation:

- For explicit definition of RedCap UE type option 1 and option 3, which request “All” recommended reduced capability, do not seem valid after completion of RAN1 study because it is allowed RedCap UE does not implement part of recommended reduced capability, e.g. number of reduced Rx antenna ports

Proposal:

- Further study explicit definition of RedCap UE type(s) for RedCap UE identification between option 2 and 4

On support of multiple BWP, we made the following observation:

Observation:

- Support of multiple BWP could be optional for RedCap UE

On Rel-16/17 power saving features, we made the following observation:

Observation:

- Semi-static adoption of power saving feature within active BWP would be reasonable for some use cases of RedCap devices

Observation:

- It could be useful for power saving that RedCap UE with two Rx Antenna branches may support maximum one layer in DL, if MIMO layer adaptation for power saving would be expected useful for the RedCap UE

Reference

[1] RP-202933	New WID on Support of reduced capability NR devices

[2] RAN1#103-e Chairman’s notes

[3] TS 38.101-1

[4] TS 38.101-2