3GPP TSG RAN WG1 #104-E			     R1-2100631

e-Meeting, January 25th – February 5th, 2021

Source:	Intel Corporation

Title:	Corrections to sidelink procedures

Agenda item:	7.2.4

Document for:	Discussion and Decision

- Introduction

This contribution is a part of Rel.16 maintenance work and it presents discussion on an open issue from RAN1#103-e related to physical layer procedure & Mode-2. Other corrections to R16 NR V2X are presented in [1]-[2].

- Discussion

In RAN1#103-e the issue of mapping/interpreting resources for reservation in a resource pool was discussed.

Q2. What option can you accept for the interpretation of sidelink slot for resource reservation period? # Option 0: No additional agreement Option 1: If a UE transmits a SCI format 1-A in slot n in a resource pool, and if “Resource reservation period” in the SCI format 1-A indicates P’ (following 8.1.7 of 38.214), - P’ is counted in t'0SL, t'1SL, t'2SL,… (i.e. The number of slots in the resource pool between slot n and slot n+P’ is always the same as P’ (including slot n+P’ itself)). Option 2: If a UE transmits a SCI format 1-A in slot n in a resource pool, and if “Resource reservation period” in the SCI format 1-A indicates P’ (following 8.1.7 of 38.214), P’ is counted in t0SL,t1SL,t2SL,... (i.e. The number of slots in the resource pool between slot n and slot n+P’ can be less than P’). - Option 2-1: No further change - Option 2-2: Add “UE expects tm+j×Prsvp_TX'SL belong to the set of slots assigned to the resource pool.” - Option 2-3: Add “If slot tm+j×Prsvp_TX'SL is not in the resource pool, the next slot in the resource pool should be used instead.” Option 3: If a UE transmits a SCI format 1-A in slot n in a resource pool, “Resource reservation period” P in the SCI format 1-A indicates the period in terms of ms. - If the physical slot after P ms is not in the resource pool, the next slot in the resource pool should be used instead. Option 4: If a UE transmits a SCI format 1-A in slot n in a resource pool, and if “Resource reservation period” in the SCI format 1-A indicates P’ (following 8.1.7 of 38.214), P’ is counted in the slots that can be used for SL transmission (i.e. the gap between the slots indicated in SCI within two periods is P). - Add “UE expects tm+j×Prsvp_TX'SL belong to the set of slots assigned to the resource pool.”

In our understanding, current specification is better to be corrected to minimize issues with jitter and resources not belonging to resource pools. From perspective of least spec impact & best possible minimization of the issue, we think Option 2 family should be adopted, while Option 1 can be considered as a fallback.

- Adopt one of the following options from the latest list in RAN1#103-e in descending preference order: Option 2-3, Option 2-2, Option 1

- Conclusions

In this contribution, we provided our view on the open issue from RAN1#103-e related to physical layer procedure & Mode-2 and have the following proposal:

- Proposal 1:

- Adopt one of the following options from the latest list in RAN1#103-e in descending preference order: Option 2-3, Option 2-2, Option 1

- References

- R1-2100629 “Correction to FD-OCC for PSCCH”, Intel Corporation, e-Meeting, January 2021.

- R1-2100630, “Corrections to Mode-2 resource allocation”, Intel Corporation, e-Meeting, January 2021.