3GPP TSG RAN WG1 #104-e	R1-2100716

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.8.3

Source: 	LG Electronics

Title: 	Discussion on coverage enhancement for Msg3 PUSCH

Document for:	Discussion and decision

Introduction

A new work item NR coverage enhancement was approved [1]. In WID, following objective in RAN1 perspective is included:

- Specify mechanism(s) to support Type A PUSCH repetitions for Msg3

In this contribution, we discuss on coverage enhancement for Msg3.

Discussion

In this section, we discuss indication of coverage enhancement for msg3 PUSCH and triggering condition of coverage enhancement for msg3 PUSCH.

Indication of Coverage Enhancement for msg3 PUSCH

For coverage enhancement for msg3 PUSCH, it needs that the configuration for PUSCH is inherited. For example, the parameters for PUSCH repetition (e.g., Number of repetition, DMRS bundling, frequency hopping, etc.) can be introduced in RA configuration. If dynamic indication for PUSCH repetition is necessary, the parameter can be included in RAR grant field for initial transmission and DCI format 0_0 with TC-RNTI for retransmission.

In addition, UE behavior for coverage enhancement for msg3 PUSCH can be described. For example, redundancy version for PUSCH transmission can be described for msg3 PUSCH transmission with repetition.

Proposal 1: The parameters for PUSCH repetition (e.g., Number of repetition, DMRS bundling, frequency hopping, etc.) can be introduced in RA configuration. Also, redundancy version for PUSCH transmission can be described for msg3 PUSCH transmission with repetition.

Triggering condition of Coverage Enhancement for msg3 PUSCH

In initial access procedure, system information may configure msg3 PUSCH repetition. But, there is an ambiguity how to trigger coverage enhancement for msg3 PUSCH. Following two alternatives can be considered:

- Alt.1:  UE reporting based triggering (i.e., UE selects coverage enhancement, and UE transmits PRACH preamble which is designated for indicating coverage enhancement.)

- Alt.2:  gNB indication based triggering (i.e., gNB indicates msg3 PUSCH repetition.)

Proposal 2: Discuss how to trigger coverage enhancement for msg 3 PUSCH.

- Alt.1:  UE reporting based triggering

- Alt.2:  gNB indication based triggering

Conclusion

In this contribution, we discuss on coverage enhancement for Msg3 PUSCH. From the discussion, we propose followings:

Proposal 1: The parameters for PUSCH repetition (e.g., Number of repetition, DMRS bundling, frequency hopping, etc.) can be introduced in RA configuration. Also, redundancy version for PUSCH transmission can be described for msg3 PUSCH transmission with repetition.

Proposal 2: Discuss how to trigger coverage enhancement for msg 3 PUSCH.

- Alt.1:  UE reporting based triggering

- Alt.2:  gNB indication based triggering

Reference

- China Telecom, “New WID on NR coverage enhancements,” RP-202928

- TS38.213

Annex

Table 1. Random Access Response Grant Content field size RAR grant field [2]

| RAR grant field | Number of bits |
| --- | --- |
| Frequency hopping flag | 1 |
| PUSCH frequency resource allocation | 14, for operation without shared spectrum channel access 12, for operation with shared spectrum channel access |
| PUSCH time resource allocation | 4 |
| MCS | 4 |
| TPC command for PUSCH | 3 |
| CSI request | 1 |
| ChannelAccess-CPext | 0, for operation without shared spectrum channel access 2, for operation with shared spectrum channel access |