3GPP TSG RAN WG1 #104	R1-2100721

e-Meeting, January 25th – February 5th, 2021

Agenda item:		8.13.3

Source:	Nokia, Nokia Shanghai Bell

Title:	On low latency Scell activation

Document for:		Discussion and Decision

# Introduction

RAN#86 approved a Further Multi-RAT Dual-Connectivity enhancements WI for Release 17 with the following objective [1]:

- Support efficient activation/de-activation mechanism for one SCG and SCells - Support for one SCG applies to (NG)EN-DC, and NR-DC [RAN2, RAN3, RAN4] - Support for SCells applies to NR CA, based on RAN1 leading mechanisms [RAN1, RAN2, RAN4] - This objective applies to FR1 and FR2 - Support of conditional PSCell change/addition [RAN2,RAN3, RAN4] - support scenarios which are not addressed in Rel-16 NR mobility WI

With respect to highlighted objective, the following agreements were made in last meeting

Working Assumption: Agreements: As working assumption, with respect to efficient SCell activation, reuse existing Rel-15/16 TRS structure for temporary RS - FFS: how many burst/symbols are required for both AGC settling and Time/Frequency tracking for different cases, e.g. FR1 and FR2, known and unknown SCell - A burst of temporary RS is notated as in S5.1.6.1.1 of TS 38.214 - “2-slot with four CSI-RSs resources (4 samples)” for FR1 - either “1-slot with two CSI-RSs resources (2 samples)” or “2-slot with four CSI-RSs resources (4 samples)” for FR2 - The working assumption can be confirmed after RAN4 check. (A LS R1-2009798 for such request is sent). Agreements: For efficient SCell activation, discuss and agree from the following alternatives at RAN1#104-e - Alt 1: the trigger of temporary RS is integrated into a single triggering signaling with the trigger of SCell activation transmitted on an activated cell. - FFS detailed design of this integrated triggering signaling. - Potential examples of single triggering signaling for further discussions - A PDSCH TB, e.g. containing two respective MAC-CEs for both triggers, one MAC-CE for both triggers - A DCI for both triggers - A PDSCH TB and its scheduling DL grant, e.g. MAC-CE for activation and DL grant for temporary RS - A DL grant and a UL grant received in the same slot/OFDM symbols of PDCCH where the DL grant is scheduling a MAC-CE for SCell activation and the UL grant is triggering the RS. - Rel-15/16 SCell activation MAC-CE and a specific configuration of temporary RS being implicitly triggered as well - Alt2: Triggering of temporary RS separately from SCell activation command is not precluded and both ‘separate’ triggers (examples below) and ‘integrated’ triggers (examples in Alt 1) are considered for SCell activation - FFS detailed design of separate triggering signaling. - Potential examples of separate triggering signaling for further discussions - Rel-15/16 SCell activation MAC-CE and Rel 15/16 DCI triggering - Rel-15/16 SCell activation MAC-CE and new DCI triggering for temporary RS - Note: temporary RS should be triggered by DCI or MAC-CE. - Note: the final mechanism of trigger signaling targets at applicability to one or more SCell activation. - FFS handling of SCell activation by existing Rel15/16 CA activation command when temporary RS is configured and triggered/not triggered

In this contribution we focus on the yellow-highlighted parts of the above agreements.

# On introduction of new SCell activation procedure using temporary RS

## On TRS structure

When it comes to

- TRS structure, e.g. whether to fully reuse existing Rel-15/16 TRS structure and configuration restriction (refer to 5.1.6.1.1 of TS 38.214), or any modification

we strongly prefer to reuse TRS structure from 5.1.6.1.1 of TS 38.214, i.e. TRS should be 4 symbols apart and in one or two consecutive slots. The benefit is low specification overhead and TRS with a minimum of 52RB (excluding special cases for paired spectrum and SCS=15kHz) and 4 symbol separation should be fully sufficient for time and frequency synchronization purposes.

Note: LS (R1-2009798) from RAN1 to RAN4 with questions regarding how many burst/symbols are required for both AGC settling and Time/Frequency tracking for different cases, e.g. FR1 and FR2, known and unknown SCell. The decision regarding how many A-TRS burst is required can be answered after the response from RAN4 is received.

## On triggering mechanism MAC-CE vs DCI

In this section we compare the pros and cons of MAC-CE vs DCI.

Table 1 MAC-CE and DCI comparison

| Field Names | MAC-CE | DCI |
| --- | --- | --- |
| Pros | - Relatively easy to add fields or define new MAC CE as compared to DCI. - No upper bound for number of information bits. - HARQ gain in MAC-CE. - Can be transmitted on any active cell. | - No need to decode and interpret the TB, just get the DCI - No issue from L2 point of view if the DCI is used for Scell Activation and Deactivation - Would not be able to indicate TCI state per SCell. This would require extra fields to be defined. No space in current Dormancy DCIs. - Slightly better latency (around 1ms less delay) performance. |
| Cons | - In case of ACK failure retransmission would be triggered leading to slightly higher latency. | - Increased computation complexity for interpreting the DCI information. - Number of information bits are limited by the structure of the DCI format. - DCI needs to be configured to be decoded on specific cell(s). Restrictions on Activation. - UE can monitor maximum (3 scrambled C-RNTI and 1 other) DCI formats over all slots. |
| Design Criteria | gNB shall be able to configure or indicate which activation mechanism it uses (Dynamically or RRC configured) for a UE. |  |

To conclude, DCI based activation, e.g. based on dormancy activation command, could be simple. However, the biggest challenge is to provide A-TRS trigger with correct QCL per each cell. From this point of view, MAC-CE is much more flexible and future proof. Therefore, we still suggest to adopt MAC-CE based SCell activation

Proposal-1: Adopt MAC-CE based Scell activation

### Design details for MAC-CE-based activation of known cell

For triggering with MAC-CE we propose the following options

- Option 1: New MAC-CE for A-TRS triggering and Scell Activation/Deactivation.

- Option 2: New MAC CE for A-TRS Triggering and Legacy MAC-CE for Activation/deactivation.

We prefer Option 1 over Option 2 for its relatively simplicity of one MACE-CE command that handles A-TRS  triggering and SCell activation. Option 2 seems complicated because of the two commands (New MAC CE for A-TRS Triggering and Legacy MAC-CE for Activation/deactivation) where order in which the two commands is received and the possible delays with which they can be received and the behaviour during the transient periods would need to be specified.

For the triggering purpose, a trigger list (similar to CSI-AperiodicTriggerStateList) including A-TRS/SSB resource and CSI report with reportQuantity set to 'none', could be configured.

In the Scell activation MAC-CE a gNB shall indicate A-TRS (form the trigger list) to be used for SCell activation, these could be SSB or CSI-RS;

- A-TRS to be triggered with an offset counted from slot of MAC-CE HARQ-ACK

- SSB of an index.

With triggering MAC-CE no CSI report would be triggered, i.e. reportQuantity set to 'none'. This is shown in Figure 1 and Figure 2, where a MAC-CE is sent in a PDSCH, the MAC-CE is activating SCell and triggering synchronization based on Aperiodic TRS (Figure 1) or synchronisation based on the earliest SSB (Figure 2). Both of these can be configured in a trigger list. In cases where triggered TRS would be colliding with SSB burst, a gNB may trigger SSB-based SCell activation instead of TRS based.

Figure 1 SCell activation with TRS

Figure 2 SCell activation with earliest SSB

Proposal-2: When R17 SCell activation command in MAC-CE for a SCell is received

  - Same MAC-CE in the same PDSCH indicates one state from preconfigured list of CSI trigger states for the first-active-BWP of each Scell

  - TCI-state associated with the triggered TRS is activated

## Other aspects

Moreover, as seen in the Figure 1 and Figure 2, the CSI framework of R15/R16 could be reused, upon activation, and synchronization based on single port A-TRS, UE may be able to measure CSI as per configured P-CSI-RS and/or based on separate trigger of A-CSI-RS either from the activated Scell or from Pcell.

Proposal-3: For acquisition of CSI after activation, reuse the existing R15/R16 framework.

# Conclusions

In this contribution, we discussed potential solutions to reduce SCell activation delay. Based on the discussion, we made the following proposals:

Proposal-1: Adopt MAC-CE based Scell activation

Proposal-2: When R17 SCell activation command in MAC-CE for a SCell is received

  - Same MAC-CE in the same PDSCH indicates one state from preconfigured list of CSI trigger states for the first-active-BWP of each Scell

  - TCI-state associated with the triggered TRS is activated

Proposal-3: For acquisition of CSI after activation, reuse the existing R15/R16 framework.

# References

- RP-201040, “Revised WID on Further Multi-RAT Dual-Connectivity enhancements”, Huawei