3GPP TSG RAN WG1 Meeting #104e                                                                  	     R1-2100761

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	6.2.1

Source:	Lenovo, Motorola Mobility

Title:	Corrections on multicast gap in Multiple TB

Document for:	Discussion and decision

Introduction

For multiple transport block transmission in multicast transmission, a scheduling gap is agreed among TBs in RAN1-99. In current TS36.213g40, text “the scheduling gap is inserted between TBs” is captured in section 7.1.11.

However，the subframes mapping for the transport blocks are not correct considering the scheduling gap. Furthermore, the scheduling gap counted by BL/CE DL subframe or absolute subfame needs RAN1 clarification. Two TP(s) proposal for TS36.213 corresponding to the scheduling gap counted with BL/CE DL subframe(s) or absolute subframe(s) are in section 2.

Agreement

For multicast, a scheduling gap can be inserted after each TB, where the gap length is configurable between {0, 2, 4, 8, 16, 32, 64, 128} subframes. The configuration is per cell.

Text proposals on scheduling gap in multiple TBs of multicast

TP1, scheduling gap is NGAP BL/CE DL subframe(s)

<Unchanged parts are omitted>

- 7.1.11	PDSCH subframe assignment for BL/CE UE

A BL/CE UE shall upon detection of a MPDCCH with DCI format 6-1A/6-1B/6-2 intended for the UE, decode the corresponding PDSCH in subframe(s) n+ki with i = 0, 1, …, NTBN-1 according to the MPDCCH, where

-	subframe n is the last subframe in which the MPDCCH is transmitted and is determined from the starting subframe of MPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI;

-	the value of is the number of scheduled TB determined in the corresponding DCI if present, otherwise;

-	If the UE is configured with higher layer parameter multiTB-Gap, PDSCH corresponding to MPDCCH with DCI CRC is scrambled by G-RNTI and,

-	subframe(s) n+sj with j=0,1,…, NTBN+(NTB-1)NGAP -1 are NTBN +(NTB-1)NGAP consecutive BL/CE DL subframe(s) where x=s0<s1<…<sNTBN+NTB-1NGAP-1.

-	 subframe(s) ni =n+ki, where ki=krN+l=srN+NGAP+l with, r=0,1,…NTB-1  and i = 0, 1, …, NTBN-1 are NTBN BL/CE DL subframe(s)

-	BL/CE DL subframes   with  are associated with TBr+1 , r=0,1,…NTB-1.

-	NGAP is the indicated value of higher layer parameter multiTB-Gap.

-	Otherwise

-		subframe(s) ni = n+ki with i=0,1,…, NTBN-1 are NTBN consecutive BL/CE DL subframe(s) where .

-	for ,

-		if the UE is configured with higher layer parameter interleaving in ce-PDSCH-MultiTB-Config, and PDSCH corresponding to a MPDCCH with DCI CRC scrambled by C-RNTI and  where  for BL/CE UE configured with CEModeA,  for BL/CE UE configured with CEModeB,

-	BL/CE DL subframes  with  are associated with TBr+1 ,

-		otherwise,

-	BL/CE DL subframes  with  are associated with TBr+1 , ,

-	the value of  is determined by the repetition number field in the corresponding DCI, where  are given in Table 7.1.11-1, Table 7.1.11-2 and Table 7.1.11-3, respectively and subframe n+x is the second BL/CE DL subframe after subframe n.

<Unchanged parts are omitted>

TP2, scheduling gap is NGAP absolute subframe(s)

<Unchanged parts are omitted>

- 7.1.11	PDSCH subframe assignment for BL/CE UE

A BL/CE UE shall upon detection of a MPDCCH with DCI format 6-1A/6-1B/6-2 intended for the UE, decode the corresponding PDSCH in subframe(s) n+ki with i = 0, 1, …, NTBN-1 according to the MPDCCH, where

-	subframe n is the last subframe in which the MPDCCH is transmitted and is determined from the starting subframe of MPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI;

-	the value of is the number of scheduled TB determined in the corresponding DCI if present, otherwise;

-	If the UE is configured with higher layer parameter multiTB-Gap, PDSCH corresponding to MPDCCH with DCI CRC is scrambled by G-RNTI and,

-	subframe(s) ni =n+ki, with i = 0, 1, …, NTBN-1 are NTBN BL/CE DL subframe(s)

-	subframe(s) nl = n+ kl with l=0,1,…, N-1 are N consecutive BL/CE DL subframe(s), x=k0<k1<…<kN-1 and subframe n+x is the second BL/CE DL subframe after subframe n

-	subframe(s) nr·N+l = n+ kr·N+l with l=0,1,…, N-1 are N consecutive BL/CE DL subframe(s), xr=krN<krN+1<…<krN+N-1 and subframe n+xr is the second BL/CE DL subframe after subframe n+krN-1+NGAP for each  r=1,…NTB-1

-	NGAP is the indicated value of higher layer parameter multiTB-Gap.

-	BL/CE DL subframes  with  are associated with TBr+1 ,

-	Otherwise

-	subframe(s) ni = n+ki with i=0,1,…, NTBN-1 are NTBN consecutive BL/CE DL subframe(s) where,  , and subframe n+x is the second BL/CE DL subframe after subframe n.

-	for ,

-		if the UE is configured with higher layer parameter interleaving in ce-PDSCH-MultiTB-Config, and PDSCH corresponding to a MPDCCH with DCI CRC scrambled by C-RNTI and  where  for BL/CE UE configured with CEModeA,  for BL/CE UE configured with CEModeB,

-	BL/CE DL subframes  with  are associated with TBr+1 ,

-	otherwise,

-	BL/CE DL subframes  with  are associated with TBr+1 , ,

-	The value of  is determined by the repetition number field in the corresponding DCI, where  are given in Table 7.1.11-1, Table 7.1.11-2 and Table 7.1.11-3, respectively.

<Unchanged parts are omitted>

- Proposal: RAN1 down selection one of the above TP adopted in TS36.213.

- References

3GPP TS 36.213 g30