3GPP TSG RAN WG1#104-e                              	                     R1- 2100818

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	7.2.2

Source:	Spreadtrum Communications

Title:	Maintenance of DL signals and channels for NR-U

Document for:	Discussion and decision

Introduction

In this contribution, we discuss the remaining issue of Rel-16 DMRS for PDSCH mapping type B.

In RAN1#99 meeting [1], the following two agreements are achieved.

Agreement: (RAN1#99) If a DMRS symbol falls in the last symbol of the type-B allocation, the DMRS symbol is dropped. - Note: This is same as specified in Rel-15 for durations 2/4/7. Agreement: (RAN1#99) For PDSCH type B durations of 12 and 13, if a DMRS symbol falls in the last symbol of the type-B allocation or on or after symbol 12 (indexing starts from 0) of the slot, the DMRS symbol is dropped.

Discussion

The DMRS position for PDCSH mapping Type B durations {3,5,6,8,9,10,11,12,13} have been captured in 38.211 v16.4.0 section 7.4.1.1.2 [2] according to the agreements. For all values of the PDSCH duration ld other than 2, 5, and 7 symbols, it is specified that UE is not expect to receive a DM-RS symbol beyond the ld-1:th symbol. Here for double-symbol DMRS, if the front-loaded DMRS of PDSCH allocation collides with resources reserved for a CORESET, the additional DMRS may shift to the last two symbols of the PDSCH allocation. According to the current specification, the last DMRS symbol will be dropped. However, in case of double-symbol DMRS, only one DMRS symbol cannot be used for channel estimation. Since the remaining DMRS symbol of the additional DMRS is useless, it can be discarded and the occupied REs can be used for PDSCH transmission to improve PDSCH performance. Figure 1 shows an example of double-symbol DMRS drop issue.

Figure 1

If the PDSCH duration ld is 12 or 13 symbols, it is also specified that the UE is not expected to receive a DM-RS symbol mapped to symbol 12 or later in the slot. Here for double-symbol DMRS, if the front-loaded DMRS of PDSCH allocation collides with resources reserved for a CORESET, the later DMRS symbol of the additional DMRS may shift to symbol 12 of the PDSCH. According to the current specification, this DMRS symbol will be dropped. Then the same issue for channel estimation occurs. Figure 2 shows another example of double-symbol DMRS drop issue.

Figure 2

The following changes are proposed to resolve these issues.

=====================TP for 38.211 Section 7.4.1.1.2 ============================= < Unchanged parts are omitted > For PDSCH mapping type B - if the PDSCH duration ld ∈2,3,4,5,6,7,8,9,10,11,12,13 OFDM symbols for normal cyclic prefix or ld∈2,4,6 OFDM symbols for extended cyclic prefix, and the front-loaded DM-RS of the PDSCH allocation collides with resources reserved for a search space set associated with a CORESET, shall be incremented such that the first DM-RS symbol occurs immediately after the CORESET and until no collision with any CORESET occurs, and - if the PDSCH duration ld is 2 symbols, the UE is not expected to receive a DM-RS symbol beyond the second symbol; - if the PDSCH duration ld is 5 symbols and if one additional single-symbol DMRS is configured, the UE expects the additional DM-RS to be transmitted on the fifth symbol; - if the PDSCH duration ld is 7 symbols for normal cyclic prefix or 6 symbols for extended cyclic prefix: - the UE is not expected to receive the front-loaded DM-RS beyond the fourth symbol, and - if one additional single-symbol DM-RS is configured, the UE only expects the additional DM-RS to be transmitted on the 5th or 6th symbol when the front-loaded DM-RS symbol is in the 1st or 2nd symbol, respectively, of the PDSCH duration, otherwise the UE should expect that the additional DM-RS is not transmitted; - if the PDSCH duration ld is 12 or 13 symbols, the UE is not expected to receive additional DM-RS mapped to symbol 12 or later in the slot; - for all values of the PDSCH duration ld other than 2, 5, and 7 symbols, the UE is not expected to receive a DM-RS symbol beyond the (ld-1):th symbol if single-symbol DMRS is used; - for all values of the PDSCH duration ld larger than 7 symbols, the UE is not expected to receive additional DM-RS beyond the (ld-1):th symbol if double-symbol DMRS is used; - if the PDSCH duration ld is less than or equal to 4 OFDM symbols, only single-symbol DM-RS is supported. - if the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, or lte-CRS-PatternList2 is configured, the PDSCH duration ld=10 symbols for normal cyclic prefix, the subcarrier spacing configuration μ=0, single-symbol DM-RS is configured, and at least one PDSCH DM-RS symbol in the PDSCH allocation collides with a symbol containing resource elements as indicated by the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, or lte-CRS-PatternList2, then l shall be incremented by one in all slots. The time-domain index l' and the supported antenna ports p are given by Table 7.4.1.1.2-5 where - single-symbol DM-RS is used if the higher-layer parameter maxLength in the DMRS-DownlinkConfig IE is not configured - single-symbol or double-symbol DM-RS is determined by the associated DCI if the higher-layer parameter maxLength in the DMRS-DownlinkConfig IE is equal to 'len2'. < Unchanged parts are omitted > =============================end of the TP ==================================

Conclusion

In this contribution, we discuss the remaining issues of Rel-16 DMRS for PDSCH mapping type B, and we provide a text proposal.

=====================TP for 38.211 Section 7.4.1.1.2 ============================= < Unchanged parts are omitted > For PDSCH mapping type B - if the PDSCH duration ld ∈2,3,4,5,6,7,8,9,10,11,12,13 OFDM symbols for normal cyclic prefix or ld∈2,4,6 OFDM symbols for extended cyclic prefix, and the front-loaded DM-RS of the PDSCH allocation collides with resources reserved for a search space set associated with a CORESET, shall be incremented such that the first DM-RS symbol occurs immediately after the CORESET and until no collision with any CORESET occurs, and - if the PDSCH duration ld is 2 symbols, the UE is not expected to receive a DM-RS symbol beyond the second symbol; - if the PDSCH duration ld is 5 symbols and if one additional single-symbol DMRS is configured, the UE expects the additional DM-RS to be transmitted on the fifth symbol; - if the PDSCH duration ld is 7 symbols for normal cyclic prefix or 6 symbols for extended cyclic prefix: - the UE is not expected to receive the front-loaded DM-RS beyond the fourth symbol, and - if one additional single-symbol DM-RS is configured, the UE only expects the additional DM-RS to be transmitted on the 5th or 6th symbol when the front-loaded DM-RS symbol is in the 1st or 2nd symbol, respectively, of the PDSCH duration, otherwise the UE should expect that the additional DM-RS is not transmitted; - if the PDSCH duration ld is 12 or 13 symbols, the UE is not expected to receive additional DM-RS mapped to symbol 12 or later in the slot; - for all values of the PDSCH duration ld other than 2, 5, and 7 symbols, the UE is not expected to receive a DM-RS symbol beyond the (ld-1):th symbol if single-symbol DMRS is used; - for all values of the PDSCH duration ld larger than 7 symbols, the UE is not expected to receive additional DM-RS beyond the (ld-1):th symbol if double-symbol DMRS is used; - if the PDSCH duration ld is less than or equal to 4 OFDM symbols, only single-symbol DM-RS is supported. - if the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, or lte-CRS-PatternList2 is configured, the PDSCH duration ld=10 symbols for normal cyclic prefix, the subcarrier spacing configuration μ=0, single-symbol DM-RS is configured, and at least one PDSCH DM-RS symbol in the PDSCH allocation collides with a symbol containing resource elements as indicated by the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, or lte-CRS-PatternList2, then l shall be incremented by one in all slots. The time-domain index l' and the supported antenna ports p are given by Table 7.4.1.1.2-5 where - single-symbol DM-RS is used if the higher-layer parameter maxLength in the DMRS-DownlinkConfig IE is not configured - single-symbol or double-symbol DM-RS is determined by the associated DCI if the higher-layer parameter maxLength in the DMRS-DownlinkConfig IE is equal to 'len2'. < Unchanged parts are omitted > =============================end of the TP ==================================

Reference

- RAN1#99 Chairman notes, 2019-11

- 3GPP TS 38.211 v16.4.0, Physical channels and modulation, 2020-12