3GPP TSG RAN WG1 #104-e	                                                                 R1-2101177

e-Meeting, January 25th – February 5th, 2021

Agenda Item:		7.2.5

Source:		Samsung

Title:		Remaining issues on PDCCH as PDSCH SLIV reference

Document for:		Discussion and decision

# Introduction

This contribution discusses the remaining issues on using PDCCH as PDSCH SLIV reference.

# Discussion

Following agreements were achieved during the email discussion [98b-NR-22].

Revised Proposal #1: For time domain resource allocation indication for PDSCH for Rel-16 URLLC in new DCI format, using the starting symbol of the PDCCH monitoring occasion in which the DL assignment is detected as the reference of the SLIV is supported. - A RRC parameter is used to enable the utilization of the new reference - When the RRC parameter enables the utilization of the new reference, the new reference is applied for TDRA entries with K0=0 - FFS: Other entries with K0>0 can also be included in the same TDRA table - For other entries (if any) in the same TDRA table, the reference is slot boundary as in Rel-15. Proposal #2: PDSCH mapping type A is not supported with the new reference.

Following agreements were achieved in RAN1#99 meeting

Agreements: Other entries with K0>0 can also be included in the same TDRA table where new reference of the SLIV is applied to the entries with K0=0 for DCI format 1_2. - For other entries (if any) in the same TDRA table, the reference is slot boundary as in Rel-15

The following text describes the determination of PDSCH candidate for Type-1 HARQ-ACK codebook.

TS 38.213 v16.3.0 #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel For a serving cell c, an active DL BWP, and an active UL BWP, as described in Clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based: a) on a set of slot timing values K1 associated with the active UL BWP a) If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for either DCI format 1_1 or DCI format 1_2 on serving cell c, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} b) If the UE is configured to monitor PDCCH for DCI format 1_1 and is not configured to monitor PDCCH for DCI format 1_2 for serving cell c, K1 is provided by dl-DataToUL-ACK c) If the UE is configured to monitor PDCCH for DCI format 1_2 and is not configured to monitor PDCCH for DCI format 1_1 for serving cell c, K1 is provided by dl-DataToUL-ACK-ForDCIFormat1_2 d) If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 for serving cell c, K1 is provided by the union of dl-DataToUL-ACK and dl-DataToUL-ACK-ForDCIFormat1_2 b) on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c a) if the UE is provided ReferenceofSLIV-ForDCIFormat1_2, for each row index with slot offset K0=0 and PDSCH mapping Type B in a set of row indexes of a table for DCI format 1_2 [6, TS 38.214], for each PDCCH monitoring occasion in a set of PDCCH monitoring occasions with different starting symbols within a slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0, if S+S0+L≤14 for normal cyclic prefix and S+S0+L≤12 for extended cyclic prefix, add a new row index in the set of row indexes of the table by replacing the starting symbol S of the row index by S+S0 …

The highlight part describes the determination of PDSCH candidate for Type-1 HARQ-ACK codebook using PDCCH as PDSCH SLIV reference. It only considers PDCCH monitoring occasions in a slot.

Using PDCCH as PDSCH SLIV reference can help reducing TDRA bits in DCI format 1_2, this is an important feature introduced in Rel-16 URLLC WI.  PDSCH repetition is another important feature for URLLC and it can help increase the reliability of PDSCH transmission. When both features are used in Type-1 HAR-ACK codebook, the candidate PDSCHs in a slot can be impacted by both PDCCH monitoring occasions and the number of PDSCH repetitions. A simple example is given in Figure 1.

Figure 1

In the TDRA table, there is only one entry, i.e., SLIV1. In slot 0, there are two PDCCH monitoring occasions. SLIV2 is an extended SLIV using PDCCH as reference. In slot 1, there is one PDCCH monitoring occasion. Without considering PDSCH repetition, SLIV1 is the only possible SLIV in slot 1. If UE is configured with PDSCH repetition, for example, pdsch-AggregationFactor = 2 configured in PDSCH-Config, in this case SLIV2 can be a valid SLIV in slot 1 as well. If pdsch-AggregationFactor is configured in PDSCH-Config, PDCCH monitoring occasions in slot n-pdsch-AggregationFactor+1 should be used for determining the candidate PDSCHs scheduled by DCI format 1_2  in slot n.

In Rel-16 IIOT, multiple SPS configurations are introduced and this feature is essential for URLLC because it can help reduce the latency as well as PDCCH signalling overhead. The number of PDSCH repetitions can also be configured in SPS-Config and there can be multiple SPS configurations with different values of pdsch-AggregationFactor. All the values of pdsch-AggregationFactor in all SPS-Config and PDSCH-Config should be considered for determining the candidate PDSCHs activated/scheduled by DCI format 1_2.

Another impact is the periodicity of SPS PDSCH configuration. If PDSCH is determined using PDCCH as SLIV reference, the SPS PDSCH is determined using the PDCCH carrying the activating DCI as SLIV reference. The location of the activating DCI should also be taken into consideration when determining the candidate PDSCHs in Type-1 HARQ-ACK codebook.

In Rel-16 MIMO, dynamic indication of the number of PDSCH repetitions is introduced. If this feature is enabled using DCI format 1_2, all the possible number of PDSCH repetitions should be considered when determining the candidate PDSCHs in Type-1 HARQ-ACK codebook.

Based on the above analysis, if UE is configured to use PDCCH as PDSCH SLIV reference and configured with Type-1 HARQ-ACK codebook, to ensure there are always HARQ-ACK bits for all possible PDSCH receptions following parameters need to be considered when determining the candidate PDSCHs in Type-1 HARQ-ACK codebook.

- the value of pdsch-AggregationFactor PDSCH-Config,

- the values of pdsch-AggregationFactor in all SPS-Config

- the values of periodicity of all the SPS PDSCH configurations

- all the possible numbers of PDSCH repetitions that can be dynamic indicated by DCI format 1_2

The size of Type-1 HARQ-ACK codebook would be increased if it is determined considering all the above parameters. An alternative solution is using slot boundary as PDSCH SLIV reference when UE is configured with Type-1 HARQ-ACK codebook. This solution is much easier compared with the former one.

Proposal: Slot boundary should be used as the reference of PDSCH SLIV if UE is configured with Type-1 HARQ-ACK codebook. The following 2 TPs should be adopted.

TP #1

TS 38.214 #### 5.1.2.1 Resource allocation in time domain When the UE is scheduled to receive PDSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocation table. The determination of the used resource allocation table is defined in Clause 5.1.2.1.1. The indexed row defines the slot offset K0, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PDSCH mapping type to be assumed in the PDSCH reception. Given the parameter values of the indexed row: - The slot allocated for the PDSCH is Ks, where , if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, and Ks = , otherwise, and where n is the slot with the scheduling DCI, and K0 is based on the numerology of PDSCH, and and are the subcarrier spacing configurations for PDSCH and PDCCH, respectively, and - Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset, for the cell receiving the PDCCH respectively, Nslot, offset, PDSCHCA and μoffset,PDSCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDSCH, as defined in clause 4.5 of [4, TS 38.211]. - The reference point S0 for starting symbol S is defined as: - if configured with pdsch-HARQ-ACK-Codebook = dynamic and configured with referenceOfSLIVForDCI-Format1-2-r16, and when receiving PDSCH scheduled by DCI format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI with K0=0, and PDSCH mapping Type B, the starting symbol S is relative to the starting symbol S0 of the PDCCH monitoring occasion where DCI format 1_2 is detected; - otherwise, the starting symbol S is relative to the start of the slot using S0=0. …

TP #2

TS 38.213 #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel For a serving cell c, an active DL BWP, and an active UL BWP, as described in Clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based: a) on a set of slot timing values K1 associated with the active UL BWP a) If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for either DCI format 1_1 or DCI format 1_2 on serving cell c, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} b) If the UE is configured to monitor PDCCH for DCI format 1_1 and is not configured to monitor PDCCH for DCI format 1_2 for serving cell c, K1 is provided by dl-DataToUL-ACK c) If the UE is configured to monitor PDCCH for DCI format 1_2 and is not configured to monitor PDCCH for DCI format 1_1 for serving cell c, K1 is provided by dl-DataToUL-ACK-ForDCIFormat1_2 d) If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 for serving cell c, K1 is provided by the union of dl-DataToUL-ACK and dl-DataToUL-ACK-ForDCIFormat1_2 b) on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c K0=0S0>0S+S0+L≤14S+S0+L≤12SS+S0 …

# Conclusions

This contribution discussed about remaining issues on using PDCCH as SLIV reference for PDSCH. Following is the summary of this contribution.

Proposal: Slot boundary should be used as the reference of PDSCH SLIV if UE is configured with Type-1 HARQ-ACK codebook. The following 2 TPs should be adopted.

TP #1

TS 38.214 #### 5.1.2.1 Resource allocation in time domain When the UE is scheduled to receive PDSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocation table. The determination of the used resource allocation table is defined in Clause 5.1.2.1.1. The indexed row defines the slot offset K0, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PDSCH mapping type to be assumed in the PDSCH reception. Given the parameter values of the indexed row: - The slot allocated for the PDSCH is Ks, where , if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, and Ks = , otherwise, and where n is the slot with the scheduling DCI, and K0 is based on the numerology of PDSCH, and and are the subcarrier spacing configurations for PDSCH and PDCCH, respectively, and - Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset, for the cell receiving the PDCCH respectively, Nslot, offset, PDSCHCA and μoffset,PDSCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDSCH, as defined in clause 4.5 of [4, TS 38.211]. - The reference point S0 for starting symbol S is defined as: - if configured with pdsch-HARQ-ACK-Codebook = dynamic and configured with referenceOfSLIVForDCI-Format1-2-r16, and when receiving PDSCH scheduled by DCI format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI with K0=0, and PDSCH mapping Type B, the starting symbol S is relative to the starting symbol S0 of the PDCCH monitoring occasion where DCI format 1_2 is detected; - otherwise, the starting symbol S is relative to the start of the slot using S0=0. …

TP #2

TS 38.213 #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel For a serving cell c, an active DL BWP, and an active UL BWP, as described in Clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based: a) on a set of slot timing values K1 associated with the active UL BWP a) If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for either DCI format 1_1 or DCI format 1_2 on serving cell c, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} b) If the UE is configured to monitor PDCCH for DCI format 1_1 and is not configured to monitor PDCCH for DCI format 1_2 for serving cell c, K1 is provided by dl-DataToUL-ACK c) If the UE is configured to monitor PDCCH for DCI format 1_2 and is not configured to monitor PDCCH for DCI format 1_1 for serving cell c, K1 is provided by dl-DataToUL-ACK-ForDCIFormat1_2 d) If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 for serving cell c, K1 is provided by the union of dl-DataToUL-ACK and dl-DataToUL-ACK-ForDCIFormat1_2 b) on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c K0=0S0>0S+S0+L≤14S+S0+L≤12SS+S0 …