3GPP TSG RAN WG1 #104-e 	                                                    	      R1-2101225

# e-Meeting, January 25th – February 5th, 2021

Agenda Item:		8.8.3

Source:				Samsung

Title:					Type A PUSCH repetitions for Msg3

Document for:	Discussion

# Introduction

The Rel-17 study item “Study on NR coverage enhancements” evaluated the baseline performance for both FR1 and FR2. PUSCH of Msg.3 has been identified as the potential bottleneck channels for both FR1 and FR2. As supported in the Rel-17 work item on NR coverage enhancements [1], the Msg.3 enhancement by using PUSCH repetition type A is approved.

This contribution discusses the aspects related to the enhancements for type A PUSCH repetitions for Msg3.

# Discussion

Since support of Msg3 PUSCH repetitions is likely to be a UE optional feature (either as an independent UE feature or as one part of the joint UE feature), whether or not the gNB needs to know the UE’s capability to support such feature needs to be discussed. Along with the repetition related configuration and the beam determination for the repetitions, this section considers the application of PUSCH enhancement techniques considered for normal PUSCH.

## UE capability differentiation

If support of msg3 PUSCH repetitions is a necessary feature for supporting coverage enhancements, UE differentiation can be in conjunction with support of coverage enhancements. This means once a UE declares support for Rel-17 coverage, the UE also supports msg3 PUSCH repetitions.

The following method can be considered:

- Msg.1 resource partition.

In this method, separate RO resources or separate preambles can be configured for UEs supporting or not supporting msg3 repetitions. By detecting the preamble in a RO, gNB can easily differentiate the UE who supports msg3 repetition. However, PRACH resources may also be partitioned for other purposes such as SSB association, 2step RACH vs 4step RACH, group A and group B, on demand SI, contention free vs contention based, and potentially Redcap UE differentiation etc. It is quite challenging to have another RACH partitioning because the preambles in one RO is limited to 64 and the grouping needs to consider each combination of so many purposes/features. Configuring separate ROs for each purpose is also not easy since DL signals (SSB and CSI-RS) association requires multiple maintains and the handling of overlaps will need further study.

- Msg.3 resource partition.

In this method, separate resources configured msg3 transmissions can be used for UEs supporting or not supporting msg3 repetitions. However, this method may require the gNB to perform multiple tests. For example, if using different time/frequency resources, the gNB needs to check which time/frequency resource are used. If using different msg3 transmission configurations (e.g., different/specific DMRS port/sequence, MCS etc.) with overlapped time/frequency resources, the gNB may need to detect a corresponding DMRS port/sequence or MCS. If different time/frequency resource are used, the worst case is some of the UL resource will be wasted; however, by proper gNB arrangement that if one of the resource is detected to be used, the other resource could be re-scheduled to other UEs. This re-scheduling could be more likely to happen when different transmission configurations are applied. E.g., when DMRS port 0 is used by UE not supporting msg3 PUSCH repetition by default and DMRS port 1 should be used by UE supporting msg3 PUSCH repetition, as shown in Fig. 1. If gNB detects the DMRS port 0, then it can understand the access UE is not supporting msg3 PUSCH repetition, so the resource for the rest of the configured PUSCH repetitions could be re-scheduled to other usage.

Fig. 1 – illustration of using different DMRS ports for UE differentiation.

As discussed above, both msg1-based and msg3-based method could work for UE differentiation, and both methods have pros and cons. Thus, the down selection or combination of both methods can be further studied.

Proposal 1: Both msg.1 and msg.3 resource partition can be considered for further down selection or combination.

## Repetition related configuration

Repetition number indication

The indication of repetition number is necessary to facilitate the msg3 PUSCH repetition. Possible mechanisms include using the RAR, DCI, and SIB1.

From flexibility perspective: via RAR > via DCI > via SIB1.

From overhead perspective: via RAR > via DCI > via SIB1.

The flexibility offered by RAR can be useful if gNB detection on the PRACH could be used for UE coverage level estimation, thus to avoid having always the maximum number of repetitions for the UE in worst coverage. However, since the interference level is usually high and gNB cannot tell how many times the UE ramped the power up for the detected preamble transmission, thus whether the reception of preamble is reliable for this purpose needs further study. Besides, dependence on the necessity of flexible coverage levels, e.g., UEs could be only categorized to be the one needs coverage enhancement and the other doesn’t need, so a cell specific repetition number might be enough. Thus, with the balance of the flexibility, complexity and necessity, signalling the repetition number of msg3 PUSCH in SIB1or in the RAR are considered for further down selection or combination.

Proposal 2: Signalling Msg3 PUSCH repetition number in SIB1 or in RAR can be considered for further down selection or combination.

Time domain resource allocation

In Rel-16, the UL grant in RAR indicates the time domain resource allocation (slot level offset and SLIV)  for a single slot. Given the type A repetition is supported, a UE can apply the SLIVto every slot where the repetitions are transmitted.

Proposal 3: Time domain resource allocation (slot level offset and SLIV) indicated by RAR UL grant is applicable to all repetitions.

## Beam determination

In Rel-16, the msg3 spatial setting is left to UE implementation and refinement of spatial setting from msg1 tx to msg3 tx was not finalized. For 2-step RACH, the PRACH and msgA PUSCH are specified to use a same spatial setting. Thus, from practical consideration, after a UE successfully detects a RAR corresponding to the transmitted preamble, that implies the spatial setting used for the PRACH transmission is qualified. There is no strong/identifiable motivation to change the spatial setting for msg3 PUSCH. Also, the relation between msg1 and msg3 affects power control as the msg.3 power is inherited from the msg.1 power setting with some msg.3-specific modification. Therefore, if the UE changes the spatial setting for msg.3 transmission compared to the one for msg1 transmission, the power setting may not be accurate. Thus, it is reasonable to use same spatial setting for msg.3 and msg.1.

Proposal 4: The repetitions for the msg3 PUSCH transmission that is scheduled by RAR use the same beam (spatial setting) as the one for the corresponding PRACH transmission.

Once UE send the first msg3, the contention resolution timer starts and it can last 64ms. Then for the msg3 retransmission scheduled by DCI 0-0 scrambled by TC-RNTI, the preamble beam may not be very stable due to a relatively long time that passed. Thus, it can be beneficial to allow the UE to select the beam to use for the msg3 re-transmissions.

Proposal 5: The UE can select the beam for msg3 re-transmissions.

## DCI Monitoring after msg3 transmission

In Rel-16, upon sending msg3, a UE starts the ra-ContentionResolutionTimer, during which it monitors for a DCI format 1_0 scheduling the PDSCH carrying msg4 or format 0_0 for msg3 retransmission. With msg3 repetitions allowed, there are different options for enabling such a DCI monitoring.

Option 1: The UE can start the monitoring after the end of the first msg3 transmission occasion among the set of scheduled msg3 repetitions. This option allows the UE to efficiently complete the RACH procedure once at least one msg3 transmission is correctly received by the gNB.

Option 2: The UE can start the monitoring after the end of the last msg3 repetition. This provides the easiest design of the procedure from a UE implementation. However, this comes at the expense of possible latency in the RACH procedure due to the transmission of possibly unneeded msg3 repetitions.

Proposal 6: Support starting	DCI Monitoring after the end of the first msg3 transmission.

## Other PUSCH enhancement techniques

Several proposals for coverage enhancement of non-msg3 PUSCH were made, e.g., time domain, frequency domain and power domain solutions. However, for Msg.3 PUSCH, contention resolution is not yet complete and the TBS is small.  It is unclear whether it is necessary, or even possible without additional designs, to apply all enhancements for non-msg3 PUSCH to msg.3 PUSCH. For example, a TB over multiple slots may not be necessary but the DMRS sharing/bundling techniques may be useful for msg3 PUSCH repetitions as well. The applicability of solutions for coverage enhancements of ‘normal’ PUSCH to msg.3 PUSCH should be separately considered for each solution.

Observation 1: The applicability of solutions for coverage enhancements of ‘normal’ PUSCH to msg.3 PUSCH should be separately considered for each solution.

# Conclusion

This contribution discusses the potential enhancements on channels in initial access procedure. Observations and proposals are summarized as follows:

Observation 1: The applicability of solutions for coverage enhancements of ‘normal’ PUSCH to msg.3 PUSCH should be separately considered for each solution.

Proposal 1: Both msg.1 and msg.3 resource partition can be considered for further down selection or combination.

Proposal 2: Signalling Msg3 PUSCH repetition number in SIB1 or in RAR can be considered for further down selection or combination.

Proposal 3: Time domain resource allocation (slot level offset and SLIV) for first repetition will follow the UL grant indicated by RAR.

Proposal 4: The repetitions for the msg3 PUSCH transmission that is scheduled by RAR use the same beam (spatial setting) as the one for the corresponding PRACH transmission.

Proposal 5: The UE can select the beam for msg3 re-transmissions.

Proposal 6: Support starting	DCI Monitoring after the end of the first msg3 transmission.

# References

- RP-202928, “New WID on NR coverage enhancements”, RAN meeting #90e, Dec 2020.