3GPP TSG RAN WG1 Meeting #104-e	R1-2101255

E-meeting, January 25 – February 5, 2021

Agenda Item:	8.14.3

Source:	Huawei, HiSilicon

Title:	Initial evaluation results for XR and Cloud Gaming

Document for:	Discussion and Decision

# Introduction

eXtended Reality (XR) and Cloud Gaming (CG) are important media applications enabled by 5G. In RAN#86, a new study item on XR evaluations for NR [1] was approved to identify XR applications, corresponding traffic models, KPIs, and evaluation methodologies, and then carry out performance evaluations. Some initial discussions and agreements have been made in RAN1#103-e on many aspects, e.g. applications, KPIs, and evaluation assumptions/methodologies [2].

In this contribution, we provide our initial system level evaluation results based on the agreed evaluation assumptions/methodologies.

# Simulation settings

In this contribution, the DL performance of XR and CG are evaluated. As shown in Table 1, Case 1 is for XR service with the typical downlink bitrate 60 Mbps, and Case 2 is for CG service with the typical downlink bitrate 35 Mbps [3].  A frame rate of 60 FPS (frames per second) is considered for both XR and CG services. The frame arrival interval is 1/60 s=16.67ms. The frame size is assumed to follow truncated Gaussian distribution, where the details are summarized in Table 2. We mainly consider Dense Urban and Urban Macro scenarios with FR1. The overall simulation settings are shown in the Annex.

Table 1. Two cases: XR and CG

| Simulation cases | DL bitrate | DL Frame Size distribution |
| --- | --- | --- |
| Case 1 (XR) | 60 Mbps | Truncated Gaussian |
| Case 2 (CG) | 35 Mbps | Truncated Gaussian |

Table 2. Detailed traffic model of Case 1 (XR) and Case 2 (CG)

| Traffic model | Case 1 (XR) | Case 2 (CG) |
| --- | --- | --- |
| Frame size distribution | Truncated Gaussian | Truncated Gaussian |
| Mean frame size (bits) | 1000200 | 583450 |
| STD of frame sizes (bits) | 100020 | 58345 |
| Packet arrival interval (ms) | 16.67 | 16.67 |

Note that from network transmission perspective, each video frame might be segmented into one or multiple IP packets, and transmitted from the server to the gNB. The size of the IP packets is limited by a maximum MTU (Maximum Transfer Unit) size, e.g. 1500 Bytes. At the gNB, each received packet is packaged into a SDAP/PDCP packet in the SDAP/PDCP layer. The SDAP/PDCP packets are further multiplexed into transmission blocks with or without segmentation in RLC/MAC layer, which are then delivered to physical layer for transmission. One transport block can include payload from multiple IP packets. One IP packet can also be transmitted over one or multiple transport blocks.

To simplify RAN1 evaluation, no frame segmentation is assumed in our simulation. Thus, one video frame results in one packet during simulation. Note that the packet size and packet arrival interval are the same as the frame size and frame arrival interval.

# Evaluation results for capacity

According to the agreement in RAN1#103-e [2], the capacity of a network is defined as the maximum number of users per cell for which the system’s user satisfaction rate is >= given threshold, where user satisfaction ratio=MsatisfiedMtotal, Msatisfied is number of users whose traffic requirement is deemed satisfied, and Mtotal is the total number of users. Here, we choose the threshold of user satisfaction ratio threshold as 90%. In the simulation, different numbers of users per cell, i.e., 1, 3, 5, 7, 10, 15 and 20, are considered.

## Capacity

As discussed in our companion paper [4], XQI is chosen as the KPI and is used to characterize the user experience or XR quality through RAN transmission. The performance requirement of a XR/CG user is deemed to be satisfied if the XQI score >= given threshold. In this subsection, XQI = 70 is chosen as the threshold (the maximum XQI score is 100).

The initial system level evaluation results for capacity of XR and CG in Dense Urban and Urban Macro are shown as follows.

### FR1 Dense Urban

- SU-MIMO

- MU-MIMO

Figure 1. Capacity simulation results (XQI>70) for XR and CG in FR1 Dense Urban

From Figure 1 a), it can be observed that

i)  For Case 1 (XR) service in FR1 Dense Urban SU-MIMO, the network capacity is 2 users per cell;

ii) For Case 2 (CG) in FR1 Dense Urban SU-MIMO, the network capacity is 5 users per cell.

From Figure 1 b), it can be observed that,

i)  For Case 1 (XR) service in FR1 Dense Urban MU-MIMO, the network capacity is 5 users per cell;

ii) For Case 2 (CG) in FR1 Dense Urban MU-MIMO, the network capacity is 16 users per cell.

It is obvious that the capacity of CG service is larger than XR service since CG service has lower data rate. In addition, for both VR and CG services, the user satisfaction rate degrades with increasing of the number of users per cell.

Observation 1: In FR1 Dense Urban, when XQI threshold is set 70 and the threshold of user satisfaction ratio is 90%, the number of supported users per cell is summarized in Table 3.

Table 3. The number of supported users per cell in FR1 Dense Urban

| No. of supported users | XR | CG |
| --- | --- | --- |
| SU-MIMO | 2 | 5 |
| MU-MIMO | 5 | 16 |

### FR1 Urban Macro

- SU-MIMO

- MU-MIMO

Figure 2. Capacity simulation results (XQI>70) for XR and CG in FR1 Urban Macro

From Figure 2 a), it can be observed that

i)  For Case 1 (XR) service in FR1 Urban Macro SU-MIMO, the network capacity is 1 users per cell;

ii) For Case 2 (CG) in FR1 Urban Macro SU-MIMO, the network capacity is 4 users per cell.

From Figure 2 b), it can be observed that,

i)  For Case 1 (XR) service in FR1 Urban Macro MU-MIMO, the network capacity is 3 users per cell;

ii) For Case 2 (CG) in FR1 Urban Macro MU-MIMO, the network capacity is 10 users per cell.

It is obvious that the capacity of CG service is larger than XR service since CG service has lower data rate. In addition, for both VR and CG services, the user satisfaction rate degrades with increasing of the number of users per cell.

Observation 2: In FR1 Urban Macro, when XQI threshold is set 70 and the threshold of user satisfaction ratio is 90%, the number of supported users per cell is summarized in Table 4.

Table 4. The number of supported users per cell in FR1 Urban Macro

| No. of supported users | XR | CG |
| --- | --- | --- |
| SU-MIMO | 1 | 4 |
| MU-MIMO | 3 | 10 |

## Impact of XQI on capacity

In this section, we present the simulation results of two XQI threshold, i.e., XQI>60 and XQI>70. Higher XQI threshold indicates better user experience, and also needs more stringent requirement.

- SU-MIMO

- MU-MIMO

Fig. 3  Capacity simulation results under different XQI requirement for XR and CG in FR1 Dense Urban

From Fig. 3 a), it can be observed that i) for Case 1 (XR) service in FR1 Dense Urban SU-MIMO, the network capacity of user requirement XQI>60 is 3 users per cell, which is 50% higher than that of XQI>70, ii) for Case 2 (CG) in this scenario, the network capacity of XQI>60 is 7 users per cell, which is 40% higher than that of XQI>70. As shown in Fig. 3b, for Case 1 (XR) service in FR1 Dense Urban MU-MIMO, the network capacity of XQI>60 is 8 users per cell, 60% higher than that of XQI>70, while for Case 2 (CG) in this scenario, the network capacity is 18 users per cell, 12.5% higher than that of XQI>70.

Observation 3: The system capacity depends on the user requirement of perceived experience. A more relaxed user experience requirement leads to higher system capacity.

# Conclusions

In this contribution, initial system level evaluation results of the scenario of FR1 dense urban and the scenario of FR1 urban macro for XR and CG service are discussed with the following observations:

Observation 1: In FR1 Dense Urban, when XQI threshold is set 70 and the threshold of user satisfaction ratio is 90%, the number of supported users per cell is summarized in Table 3.

Table 3. The number of supported users per cell in FR1 Dense Urban

| No. of supported users | XR | CG |
| --- | --- | --- |
| SU-MIMO | 2 | 5 |
| MU-MIMO | 5 | 16 |

Observation 2: In FR1 Urban Macro, when XQI threshold is set 70 and the threshold of user satisfaction ratio is 90%, the number of supported users per cell is summarized in Table 4.

Table 4. The number of supported users per cell in FR1 Urban Macro

| No. of supported users | XR | CG |
| --- | --- | --- |
| SU-MIMO | 1 | 4 |
| MU-MIMO | 3 | 10 |

Observation 3: The system capacity depends on the user requirement of perceived experience. A more relaxed user experience requirement leads to higher system capacity.

# References

RP-193241, 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019

Chairman notes, RAN1#103-e, November 2020.

R1-2100207, “Discussion on applications and traffic model for XR and Cloud Gaming”, Huawei, HiSilicon, RAN1#104-e, January 25 – February 5, 2021

R1-2100242, “Discussion on evaluation methodology for XR and Cloud Gaming” , Huawei, HiSilicon, RAN1#104-e, January 25 – February 5, 2021

# Annex-Simulation settings

Table I. System level simulation assumption for FR1 DL Scenario1 and Scenario2

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| Scenarios | Scenario-1: Dense Urban |  | Scenario-2: Urban Macro |
| Layout | 21cells with wraparound ISD: 200m |  | 21cells with wraparound ISD: 500m |
| Carrier frequency | 4.0 GHz |  |  |
| Bandwidth | 100 MHz |  |  |
| Subcarrier spacing | 30 KHz |  |  |
| Frame structure | Option 1: [DDDDU DDDDU] |  |  |
| Inter Site Distance | 200m | 500m |  |
| BS Antenna height | 25m |  |  |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 64T: (8,8,2,1,1;4,8) |  |  |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | For 4R: (1,2,2,1,1;1,2) |  |  |
| BS antenna pattern | 3-TRxP pattern, 8 dBi |  |  |
| UE antenna pattern | Omnidirectional, 0 dBi |  |  |
| TX Power | BS : 44 dBm per 20MHz |  | BS : 49 dBm per 20MHz |
| UE Power class | 23dBm |  |  |
| Noise Figure | BS:5 dB, UE:9 dB |  |  |
| Schedulingr | SU-MIMO Proportional Fair/MU-MIMO Proportional Fair |  |  |
| Max MCS | 256QAM |  |  |
| Device deployment | 80% indoor, 20% outdoor |  |  |
| UE speed | 3 km/h |  |  |
| Performance Metric | XQI score equal to or larger than a threshold |  |  |
| Channel Estimation | Ideal |  |  |