3GPP TSG-RAN WG1 Meeting #104-e	Tdoc R1-2101318

Online, January 25th – February 5th, 2021

Agenda Item:	8.1.5

Source:	Ericsson

Title:	Additional simulation results on multi-beam operation

Document for:	Discussion

# 1	Introduction

This is a companion paper to our main paper on enhancements of multi-beam operation [1], where we present in detail the simulation results for the topics of L1/L2-centric inter-cell mobility and maximum permissible exposure (MPE) mitigation.

We performed system level simulations (SLS) comparing a fast L1/L2 handover procedure and the, agreed in the evaluation methodology (EVM) [2],  L3 handover model for the Dense Urban Scenario for a UE speed of 120 km/h; the results are presented in section 2.1.

Additionally, we performed extensive simulations to investigate the potential benefits of fast panel switching due to panel blockage and MPE regulation; the results are presented in section 2.2	MPE mitigation SLS results. We simulated the Dense Urban and Indoor scenarios, both with the parameters agreed in EVM [2], but also with parameters corresponding to more challenging, but also realist, scenarios.

In RAN1#103e, the following was agreed for the fast uplink panel selection topic:

Agreement

In Rel-17 enhancement for facilitating fast uplink panel selection, the following use cases are assumed:

- MPE mitigation

- UE power saving

- UL interference management

- Support different configurations across panels

- UL mTRP

Agreement

In Rel-17 enhancement on MP-UE to facilitate fast UL panel selection and MPE mitigation, UL Tx panel(s) are assumed to be a same set or subset of DL Rx panel(s)

Agreement

In Rel.17 enhancement for facilitating fast uplink panel selection, UE-initiated UL panel selection/activation are supported:

- FFS: Whether NW-initiated panel selection/activation is also supported

- FFS: Whether specification support for this feature is necessary and if so the details of such spec support.

Agreement

On UE reporting for MPE mitigation for Rel-17, investigate and, if needed, specify the following:

- Reporting of P-MPR report based on Rel.16 framework.

  - FFS: Whether panel/beam level based P-MPR report is supported

  - FFS: Maximum reported number of panels, e.g. single or multiple

- Reporting SSBRI(s)/CRI(s) and/or indication of panel selection for the purpose of indicating:

  - Alt1: alternative UE panel(s) or TX beam(s) for UL transmission

  - Alt2: feasible UE panel(s) or TX beam(s) for UL transmission taking the MPE effect into account

  - FFS: indication of panel selection details (e.g. explicit/implicit)

- Any additional reporting content: down-select from the following in RAN1#104-e

  - Alt0: no additional reporting content

  - Alt1: Additional reporting content is included (for example P-MPR + L1-RSRP, virtual PHR + L1-RSRP, L1-RSRP/SINR with and without MPE effect, virtual PHR, P-MPR or virtual PHR + CRI/SSBRI, estimated max UL RSRP)

    - Note: Other options are not precluded

    - FFS: Whether the above reporting is triggered by UE or configured by NW

In RAN1#103e, the following was agreed for the L1/L2-centric inter-cell mobility topic:

Agreement

On Rel-17 enhancements to enable L1/L2-centric inter-cell mobility:

- The following use cases are assumed:

  - Network architecture:

    - NSA, i.e. LTE PCell and NR-PSCell

    - SA

  - Intra-band CA

    - FFS: If inter-band CA is also included

  - Intra- RAT (excluding inter-RAT)

  - Intra-frequency scenario:

    - The SSBs of non-serving cells have the same center frequency and SCS as the SSBs of the serving cell

    - An SSB of a non-serving cell is associated with a PCI different from the PCI of the serving cell

    - FFS: Support for inter-frequency scenario

  - FFS: Whether to support intra-DU only operation, or whether inter-DU is also allowed

- The following enhancement scope is assumed:

  - Facilitating measurement and reporting of non-serving RSs via incorporating non-serving cell info with some TCI(s), along with the necessary measurement and reporting scheme(s)

    - FFS: Detailed/exact method(s)

    - FFS: Whether this also implies the support of beam indication (TCI state update along with the necessary TCI state activation) for TCI(s) associated with non-serving cell RS(s)

    - FFS: Metric for the measurement and reporting, e.g. L1-RSRP or L3-RSRP or time- or spatial-domain-filtered L1-RSRP

    - FFS: Beam-level event-driven mechanism, using serving cell RS and/or non-serving cell RS

  - Facilitate serving cell to provide configurations for non-serving cell SSBs via RRC

    - FFS: details for the configurations, e.g. time/frequency location, transmission power, etc.

    - FFS: other information needed for inter-cell mobility

  - Note: In RAN1's understanding, non-serving cell SSB and non-serving cell RS can be part of the serving cell configuration

- FFS: The following enhancement scope is assumed by RAN1:

  - Whether RRC reconfiguration signaling is needed or not when a TCI associated with non-serving cell RS is indicated

    - A non-serving cell RS is an RS that is or has an SSB of a non-serving cell as direct or indirect QCL source

    - This implies no C-RNTI update when UE receives DL channel RS associated to non-serving cell RS as QCL source.

    - FFS whether TCI associated with non-serving cell can be indicated to or are applicable for all channels.

  - Whether some RRC parameters need to be updated without additional RRC signaling, e.g. some RRC parameters are pre-configured, which are associated with TCI states with neighbor cell RS as QCL source

  - Whether UE needs/can change serving cell during L1/L2-centric inter-cell mobility.

  - The above assumption to be verified by RAN2

# 2	Discussion

## 2.1	Inter-cell mobility SLS results

To investigate the potential performance impact of faster handover procedures for inter-cell mobility, we performed simulations for the Dense Urban scenario described in the EVM [2] with 120km/h UE speed and 10 UEs at random positions along the trajectory for each drop over a total of 100 drops. The handover procedure was evaluated with respect to two different handover models: L1/L2 and L3, where we set the handover model parameters as:

- Handover deactivation delay: L1/L2: 10 ms, L3: 1000 ms

- Handover interruption time with no data transfer: L1/L2: 5 slots, L3: 80 ms

### 2.1.1	Simulation parameters

Simulation parameters follow the EVM in [2] with clarification and specifics in Table 1.

Table 1 Simulation parameters for inter-cell mobility scenario

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz, SCS: 120 kHz, BW: 80 MHz |
| BS Antenna Configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2). (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ Using equivalent full array (8, 16, 2, 1, 1) for beamforming gNB Antenna has physical down-tilt of 15 degrees (improves coverage close to gNB) 132 gNB TX beam directions: Oversampled (x2) dual polarized DFT beams with 22 directions in azimuth [-60, 60] range and 6 directions in elevation [90,160] range (with directions corrected for physical down-tilt) |
| UE Antenna Configuration | Number/location of panels: 3 panels (left, right, and back) – “H” pattern Panel structure: 1x4x2 or (M, N, P) = (1, 4, 2), dH = 0.5 λ All panels are active 4 Dual polarized DFT beams per panel in azimuth [-90, 90] range |
| Link adaptation | Outer loop adaptation + Explicit CSI |
| Traffic Model | Full buffer |
| Control and RS overhead | No overhead modelled |
| Transmission scheme | Up to rank 2 transmission |
| Other simulation assumptions | TRP selection based on L1-RSRP Wideband scheduling, Proportional Fair Scheduling |
| Intra-cell beam management | Beam update mechanism: Exhaustive joint P2 & P3 sweep every 20 ms Beam activation delay: 5 slots Beam metric: L1-RSRP (from full bandwidth beamformed path gain) |
| Inter-cell handover procedure | Handover search every 100ms with a 3dB handover margin to all sectors comparing best BPL Handover metric: L1-RSRP (from full bandwidth beamformed path gain) Handover deactivation delay: {10 ms, 1000 ms} Handover interruption time (no data transfer): {5 slots, 80 ms} At a handover request, the best BPL found in the handover search is selected and activated after the deactivation delay and interruption time |
| Other potential impairments | Not modelled (assumed ideal) |
| Target BLER | 10% (Realized BLER is typically smaller for this scenario – only a few UEs per sector) |
| Scenario | Dense Urban (macro-layer only, TR 38.913), 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site), 100% outdoor, 10 UEs per drop (multiple UEs dropped along the trajectory to model intra- and inter-cell interference) |
| Channel model | UMa 5G (TR 38.901), outdoor, LOS only, Car Penetration loss Spatial consistency with UE mobility option B |
| UE mobility and trajectory | 120 km/h, Linear trajectory across multiple cell edges Random initial UE positions over multiple drops in rectangle from P-Q with width of 8 m (X from U[26,34]) with random direction of movement towards P or Q |
| UE panel orientation | The UEs are vertically oriented, with a random xy-orientation in each drop |
| Simulated time extent | 72000 slots (including 1000 slots warm-up period) corresponding to UE movement of approximately half the trajectory from P to Q |

### 2.1.2	Dense Urban, 120km/h, Handover model comparison

The handover procedure was evaluated with respect to two different handover models: L1/L2 and L3.

The resulting C.D.F. of User Throughput is shown in Figure 1, the mean and cell-edge user throughputs are shown in Figure 2. The mean user throughput evolution over time is shown in Figure 3, where the value is filtered over 100 slots.

We observe that the 1 s delay for the L3 handover model, which corresponds to 33.3 m UE movement which is more than half the average distance of 57.7 m between cell-edges along the trajectory, has a clear impact on the user throughput.

Figure 1 C.D.F of User Throughput for the L1/L2 and L3 handover procedure model

Figure 2 Mean and Cell-edge User throughput for the L1/L2 and L3 handover procedure model

Figure 3 Mean User Throughput over time for the L1/L2 and L3 handover procedure model

The C.D.F. of number of handovers per UE and the fraction of all handover requests that reverted the scheduler (cell) id to the previous scheduler is shown in Figure 4. This type of events are known as ping-pong handovers, and is traditionally considered undesirable. However, since there is very little performance impact of a L1/L2 handover, it is of less importance to avoid ping-pong handovers. The large delay for the L3 model compared to the L1/L2 handover model results in a clear reduction in the total number of handovers for each UE. We note that the average number of cell edges crossed is 5 per UE, however, the UEs also travels rather close to the intersection point between 3 sectors which may be one of the reasons more than 50% of the UEs have more than 7 handover requests for the L1/L2. Some of the additional handover request are also attributed to fast-fading effects around cell edges where some UEs change scheduler back and forth over the cell transition. Figure 5 shows the fraction of UEs that perform a handover at each handover search, which is about 6% or 8% for the L3 and L1/L2 models, respectively. This implies that each UE performs in average a handover every 1.67 s for the L3 model and every 1.25 s for the L1/L2 model. The UE moves 55.5 m in 1.67 seconds, which is rather close to the expected value of 57.7 m from the average distance between cell edges along the trajectory.

Figure 4 C.D.F of number of handovers per UE and fraction of handovers reverted to previous scheduler (cell) for the L1/L2 and L3 handover procedure model

Figure 5 Handover requests (% of all UEs) at each handover search (every 100 ms) for the L1/L2 and L3 handover procedure model

## 2.2	MPE mitigation SLS results

To investigate the benefit of introducing features to facilitate UL beam selection for UEs equipped with multiple panels, considering UL coverage loss mitigation due to panel blockage and MPE regulation, we performed UL system simulations for the Dense Urban and Indoor scenarios, both with the parameters agreed in EVM [2], but also with parameters corresponding to more challenging, but also realistic, use cases. In every simulation campaign, we compare the UL performance for the following three cases:

- No Blockage:

  - No blockage for any of the panels

  - Cell and UE panel selections are based on exhaustive gNB/UE beam sweep

  - This case is used as baseline for demonstrating the impact of panel blockage

- Panel blockage, gNB MPE-unaware:

  - Pathloss is increased with 10dB for one of the three panels, chosen randomly

  - Cell and UE panel selections are taking the extra 10dB pathloss into account

  - For UEs transmitting from a blocked panel, the max power is decreased by 10dB

  - This case is used as baseline for demonstrating the impact of MPE on scheduler decisions

- Panel blockage, gNB MPE-aware:

  - Pathloss is increased with 10dB for one of the panels, chosen randomly.

  - Cell selection is taking the pathloss increase into account.

  - For UEs transmitting from a blocked panel, the max power is decreased by 10dB.

  - UE panel selection is taking into account the pathloss increase and max power reduction

For each simulation campaign, we report the following results:

- CDF of the user throughput [bps/Hz]

- Mean and cell-edge user throughput (negative) gains of MPE cases over the no blockage baseline

- CDF of the best path-gain [dB], including the panel blockage and the gNB/UE beamforming gains

- CDF of the UEs TX power

### 2.2.1 Simulation parameters

Simulation parameters follow the EVM in [2] with clarification and specifics in Table 2. Additional parameters, corresponding to more challenging but also realistic scenarios, are marked with red text.

Table 2 Simulation parameters for MPE/MP-UE scenarios

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz, SCS: 120 kHz, BW: 80 MHz |
| BS Antenna Configuration (2.2.2.1, 2.2.3.1, 2.2.3.2) (2.2.2.2) | (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2). (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ Using equivalent full array (8, 16, 2, 1, 1) for beamforming Initial Beam selection based on L1-RSRP (from full bandwidth beamformed path gain) 128 dual polarized beams Antenna gain: 8 dBi (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1). (dV, dH) = (0.5, 0.5) λ 16 dual polarized beams |
| UE Antenna Configuration | Number/location of panels: 3 panels (left, right, and back) – “H” pattern Panel structure: 1x4x2 or (M, N, P) = (1, 4, 2), dH = 0.5 λ All panels are active 4 DFT Beams per panel in azimuth [-90, 90] range Antenna gain: 5 dBi |
| UL power control parameters | SNR target = 15 dB |
| Traffic Model | Full buffer |
| Control and RS overhead | No overhead modelled |
| Transmission scheme | Multi-antenna port transmission scheme, with up to rank 2 |
| Scenarios | - Dense urban (macro-layer only, TR 38.913) @FR2, 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per cell), - 100% outdoor (2.2.3.1) - 20% outdoor, 80% indoor (2.2.3.2) - Indoor (TR 38.901/802), 12 sites, 1 sector per site (2.2.2.X) |
| UE speed | 3 km/hr for indoor UEs, 30km/hr for outdoor UEs |
| Panel Blockage Modeling for MPE | Only one panel is blocked. The blocked panel is randomly selected at each drop Blocking entails an additional pathloss of 10dB applied to both DL and UL For simulation with full buffer traffic, a blocking event is determined, started at the beginning of each drop, and sustained throughout the entire drop |
| MPE Modeling | When MPE occurs, the max TX power for the blocked panel is reduced by 10dB P-MPR, i.e., the actual max TX transmit power = max UE TX power (23dBm) – P-MPR (10dB), where max UE TX power = 23 or 13 dBm (2.2.2.X, 2.2.3.1) (2.2.3.2) |
| UE-side panel switching latency | 0 ms for active panels (all the panel are active) |
| UE orientation | Vertical but random in azimuth |
| UE dropping | Random, 1 or 10 UE per sector (2.2.2.X, 2.2.3.1) (2.2.3.2) |

### 2.2.2 Indoor Hotspot

#### 2.2.2.1 EVM scenario

In this subsection, we provide simulation results for the indoor hotspot scenario according to the agreed EVM. The user throughput C.D.F. results are depicted in Figure 6, whereas the mean and cell-edge throughput (gains) are summarized in Table 3. The best BPL gain C.D.F. is shown in Figure 8. We evidence that in this scenario the panel blockage has negligible effect: about 3dB loss in the median path gain, resulting to less than 1% loss in mean throughput. Most importantly, the MPE-aware scheduler does not provide any significant gain over the MPE-unaware one. This is not surprising since the UEs in this scenario are not power-limited, as seen by the TX power C.D.F. in Figure 7.

Figure 6: CDF of user throughput

| User throughput [bps/Hz] (gain) | No panel blockage | Panel blockage gNB MPE-unaware | Panel blockage gNB MPE-aware |
| --- | --- | --- | --- |
| Mean | 3.73 | 3.72 (-0.5%) | 3.70 (-0.8%) |
| Cell-edge | 1.78 | 1.69 (-5.3%) | 1.67(-6.3%) |

Table 3: Mean and Cell-edge user throughput

Figure 7: CDF of the UEs TX power  		    Figure 8: CDF of the path-gain

#### 2.2.2.2 Smaller antenna array

In this subsection, we provide simulation results for the indoor hotspot scenario, but this time using a smaller than the one agreed in EVM gNB antenna array (4x4 cross-polarized), which may be more realistic for an indoor deployment. The throughput results are shown in Figure 9 and Table 4. The best BPL gain C.D.F. is shown in Figure 11. We evidence that in this scenario too the panel blockage has small effect, resulting to 3-4% loss in mean throughput. Even with a smaller gNB antenna array, the UEs are not power-limited, as seen by the CDF of the TX power in Figure 10. Hence, the result holds that the MPE-aware scheduler does not provide any significant gain over the MPE-unaware one.

Figure 9: CDF of user throughput

| User throughput [bps/Hz] (gain) | No panel blockage | Panel blockage gNB MPE-unaware | Panel blockage gNB MPE-aware |
| --- | --- | --- | --- |
| Mean | 3.29 | 3.18 (-3.3%) | 3.15 (-4.1%) |
| Cell-edge | 1.35 | 1.27 (-5.6%) | 1.55 (-7.5%) |

Table 4: Mean and Cell-edge user throughput

Figure 10: CDF of the UEs TX power 		Figure 11: CDF of the downlink path-gain

Based on the result in this section, it is clear that the MPE-aware scheduler provides no performance benefits:

For the indoor hotspot scenario, the UEs are not power-limited, and the MPE-aware scheduler provides no benefits.

### 2.2.3 Dense Urban

#### 2.2.3.1 EVM scenario

In this subsection, we provide simulation results for the Dense Urban scenario according to the agreed EVM. The user throughput C.D.F. results are depicted in Figure 12, whereas the mean and cell-edge throughput are summarized in Table 5. The best BPL gain C.D.F. is shown in Figure 14. In this scenario too, we see that the MPE-aware scheduler and the MPE-unaware scheduler have similar performance, since only a tiny portion of the UEs is power limited.

Figure 12: CDF of user throughput

| User throughput [bps/Hz] (gain) | No panel blockage | Panel blockage gNB MPE-unaware | Panel blockage gNB MPE-aware |
| --- | --- | --- | --- |
| Mean | 3.53 | 3.45 (-2.4%) | 3.45 (-2.3%) |
| Cell-edge | 1.62 | 1.55 (-3.9%) | 1.55 (-4.1%) |

Table 5: Mean and Cell-edge user throughput

Figure 13: CDF of the UEs TX power		 Figure 14: CDF of the downlink path-gain

We thus observe

For the dense urban scenario with the agreed EVM, the UEs are not power-limited and the MPE-aware scheduler provides no benefits.

#### 2.2.3.2 Power-limited scenario

In this subsection, we provide simulation results for the Dense Urban scenario with parameters tuned to depict a realistic power-limited scenario. To obtain these results, we deviate from the EVM as follows:

- The max TX power is decreased from 23 to 13 dBm

- The UEs are dropped both indoor (80%) and outdoor (20%)

- The number of UEs per sector are increased from 1 to 10

A new simulation case was added to provide another baseline on the results:

- Panel blockage with MPE-unaware scheduler and no MPR (Panel blockage, no MPR):

  - Pathloss is increased with 10dB for one of the panels, chosen randomly.

  - Cell and UE panel selections are taking the extra 10dB pathloss into account

Panel Blockage, MPR, gNB MPE-aware

Panel Blockage, MPR, gNB MPE-unaware

Panel Blockage, no MPR

No Blockage

Figure 15: User throughput CDF		Figure 16: Mean user throughput

Figure 17: Path-gain diff CDF 			      Figure 18: UE Tx power CDF

Even for this scenario, as seen by the user throughput results in Figure 18 and Figure 19, the MPE-aware scheduler does not offer better performance than the MPE-unaware one. It is also interesting to note that the case “Panel blockage, no MPR” has similar performance to the other two “Panel blockage” cases, despite not having the maximum power reduction due to the panel blockage.

Figure 21 shows that the UEs are almost always coverage limited, as only ~15% of the UEs do not use the maximum allowed TX power. From Figure 18 it’s also clear that ~10% of the UEs are completely out of coverage and thus the cell-edge user throughput is 0.

Figure 20 shows the difference in terms of path-gain between the best beam from the best panel, that will be used for the transmission, and the best beam from the second-best panel, that might be used in case the best panel is blocked. The result shows that in most of the cases (~95%) the path-gain difference is less than 10 dB and this means that the scheduler, in case of blockage of the best panel (that as agreed in the EVM introduces additional 10 dB pathloss) in most of the cases will switch to the best beam of the second-best panel losing less than 10 dB without the need of considering the MPR issue. This explains why in Figure 15 and Figure 16 there is a small difference between the “Panel blockage, no MPR” case and the “Panel blockage, MPR” cases and why the MPE-aware scheduler does not give any improvement over the MPE-unaware one: when the path-gain difference is less than 10 dB the panel switch is done regardless the possible MPR of the blocked panel.

The additional pathloss of the blocked panel leads to that UEs deselect the blocked panel without NW intervention.

# 3	Conclusion

We made the following observations:

Observation 1	For the indoor hotspot scenario, the UEs are not power-limited, and the MPE-aware scheduler provides no benefits.

Observation 2	For the dense urban scenario with the agreed EVM, the UEs are not power-limited and the MPE-aware scheduler provides no benefits.

Observation 3	The additional pathloss of the blocked panel leads to that UEs deselect the blocked panel without NW intervention.

# 4	References

R1-2101313, Enhancements on Multi-beam Operation, Ericsson, RAN1#104-e, January 2021

R1-2007151, R17 FeMIMO summary Item 1 EVM - final, Samsung, RAN1#102-e, August, 2020