3GPP TSG RAN WG1 #104-e                                                   	R1-2101356

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.1.4

Source:	Apple Inc.

Title:	Views on Rel-17 CSI enhancement

Document for:	Discussion/Decision

# Introduction

In RAN#86 meeting, Rel-17 FeMIMO WI was approved in RP-193133 [1]. Four main areas haven been identified for the Rel-17 FeMIMO, namely

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2

- Enhancement on SRS, targeting both FR1 and FR2

- Enhancement on CSI measurement and reporting

Regarding CSI measurement and reporting enhancement, the following objectives are approved as part of Rel-17 FeMIMO [1]

- Enhancement on CSI measurement and reporting: - Evaluate and, if needed, specify CSI reporting for DL multi-TRP and/or multi-panel transmission to enable more dynamic channel/interference hypotheses for NCJT, targeting both FR1 and FR2 - Evaluate and, if needed, specify Type II port selection codebook enhancement (based on Rel.15/16 Type II port selection) where information related to angle(s) and delay(s) are estimated at the gNB based on SRS by utilizing DL/UL reciprocity of angle and delay, and the remaining DL CSI is reported by the UE, mainly targeting FDD FR1 to achieve better trade-off among UE complexity, performance and reporting overhead

In the last RAN1 meeting RAN1#102e, the following agreement is reached [2]

Agreement Port selection codebook enhancements utilizing DL/UL reciprocity of angle and/or delay is supported in Rel-17. Agreement Rel-17 CSI measurement and reporting for DL multi-TRP and/or multi-panel transmission shall be enhanced to support and enable more dynamic channel/interference hypotheses for NCJT. Agreement Study following alternatives, and select one or a combination of multiple alternatives for Rel-17 in RAN1#104-e: Agreement For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, [at least for multi-DCI based and single-DCI based schemes (scheme 1a)], NZP CSI-RS resources for channel measurement are associated to different TRPs/TCI states at resource level - CMRs corresponding to different TRPs respectively shall be configured within the same resource set (i.e. scheme 1-2) and have the same number of ports among CMRs. - At least ‘typeI-SinglePanel’ codebook is supported - FFS: Other codebook types - Note that RAN1 shall strive to finalize NCJT CSI enhancement with single reporting setting firstly. - The support of larger than 32 ports across two CMRs is optional for a UE supporting Rel. 17 mTRP CSI Working Assumption For CSI measurement for multi-DCI based NCJT, down select one of following two options: - Option 1 (Explicit): CMRs corresponding to different TRPs can be associated with different reporting settings respectively, with the same configurations between two settings except for PUCCH/PUSCH resources and CMR/IMR resources setting(s) - Option 2 (Implicit): a single CSI reporting setting associated with each TRP where a NZP CSI-RS is configured for interference measurement from another TRP - FFS: how interference from CMR in the linked reporting settings in option 1 or from the NZP CSI-RS configured as IMR in option 2 is considered in CQI calculation Following restrictions apply to both options: - At least ‘typeI-SinglePanel’ codebook is supported - FFS: Other codebook types - Only ‘periodic’ and ‘semiPersistentOnPUCCH’ cases are supported; - The number of ports of two CMRs associated to two reporting settings for NCJT CSI measurement are the same; - The support of larger than 32 ports across two CMRs is optional for a UE supporting Rel. 17 mTRP CSI R1-2009530 Summary of Further Email discussion for Rel-17 CSI enhancements Moderator (Huawei) Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, the UE is expected to report - two RIs, two PMIs, two LIs and one CQI per codeword, for single-DCI based NCJT when the maximal transmission layers is less than or equal to 4 - FFS: Maximal transmission layers larger than 4 - FFS: Whether/how a subset of above reporting quantities are allowed to be configured to the UE - FFS: whether/how to support two RIs, two PMIs, two LIs and two CQIs, for multi-DCI based NCJT - FFS: whether/how to support CRI(s) to be reported in a CSI - FFS: restrictions among reported CSI quantities, e.g. among reported RIs and PMIs - FFS: whether/how to support non-PMI based port-selection - FFS: whether/how to support single value of reported LI Note that other NCJT CSI measurement/reporting enhancement for other scenarios is not precluded, e.g. for HST-SFN Agreement For a CSI reporting setting, support one or more of the following UE reporting mechanism: - Alt 1: the UE can be expected to report one CSI associated with the best single-TRP measurement hypothesis and one CSI associated with the best NCJT measurement hypothesis, if configured - FFS omission of CSI associated with NCJT measurement hypothesis - Alt 2: the UE can be expected to report one CSI associated with the best one among NCJT and/or single-TRP measurement hypotheses, if configured - FFS how to report recommended measurement hypothesis associated with that CSI report - Alt 3: the UE can be expected to report two CSIs associated with the two best single-TRP measurement hypotheses associated with CMRs from two TRPs and one CSI associated with the best NCJT measurement hypothesis, if configured - FFS omission of CSI associated with NCJT measurement hypothesis - Whether/How to report a subset of the CSI report quantities - FFS: CSI reporting configuration details Note supporting which one or more mechanisms is to be determined in RAN1#104-e Agreement For NCJT CSI measurement configured with single reporting setting, study following measurement resource configuration/association mechanism - Whether/how to support interference measurement based on NZP CSI-RS given by nzp-CSI-RS-ResourcesForInterference or based on CSI-IM given by csi-IM-ResourcesForInterference - Whether/how to interpret measurement based on CMRs associated with different TRPs/TCI states respectively for a NCJT measurement hypothesis - CMR/IMR resource configuration restrictions/associations, e.g. for reference resource/time domain behaviour/frequency domain behaviour Note that RAN1 shall strive for commonality of CSI measurement/reporting mechanisms for NCJT CSI measurement configured by single or two reporting settings

In this contribution, we provide our views on the Rel-17 FeMIMO CSI enhancement in the following areas

- CSI measurement and reporting enhancement for MTRP

- Type II port selection codebook enhancement utilizing partial DL/UL channel reciprocity

# CSI enhancement for MTRP

Rel-15/16 CSI measurement and reporting adopts the design framework of CMR and IMR, in which CMR is configured for channel measurement, and IMR is configurated for interference measurement. In terms of IMR, there are two types of IMR, i.e., NZP-IMR (a.k.a. NZP-CSI-RS) and ZP-IMR (a.k.a. CSI-IM).

For CSI enhancement for MTRP, the main focus is on the NJCT (Non-Coherent Joint Transmission) scheme, in which, the PDSCH transmitted from two TRP can overlap in frequency. For example, Multi-DCI based Multi-TRP operation and Single-DCI based Multi-TRP operation with SDM mode can be used to schedule explicit NCJT operation to the UE.

Rel-16 solution focused on the PDSCH scheduling enhancement, by introducing 6 different modes, 1 for Multi-DCI based Multi-TRP and 5 for Single-DCI based Multi-TRP. However, CSI enhancement is not allowed in Rel-16. Figure 1 illustrates a simple issue for NCJT. Assume TRP1 transmits one CSI-RS resource set and TRP 2 transmits the other CSI-RS resource set, we have the following observation

- There is no clear definition of CMR and IMR for NCJT

  - When TRP1 transmits its PDSCH, the CSI-RS resource set from TRP 1 is CMR, while CSI-RS resource set from TRP 2 is IMR

  - When TRP2 transmits its PDSCH, the CSI-RS resource set from TRP 2 is CMR, while CSI-RS resource set from TRP 1 is IMR

In other words, for UE to derive the optimal CSI reporting, UE needs to jointly consider the reciprocal CMR/IMR relationship between TRP1 and TRP2. Using PMI as an example, UE needs to jointly solve the following optimization problem, note that the problem formulation is loose and for illustration purpose only

MAXw1,w2CH1*W1H2*W2+CH2*W2H1*W1

Whereas

- H1 and H2 are the channel from TRP1 and TRP2 respectively

- W1 and W2 are the precoding matrix (PMI) of TRP1 and TRP2 respectively

- C∙ is the capacity function

Figure 1 Reciprocal relationship between CMR and IMR for NCJT

From the above analysis, we can see that, for UE to provide the optimal CSI reporting. The following two things are required

- The flexible CSI measurement RS configuration to reflect the reciprocal relationship between CMR and IMR

- The flexible CSI report quantity configuration to allow UE to report joint optimization results in terms of PMI/RI/CRI/LI/CQI.

When considering multi-DCI based multi-TRP, currently, gNB schedules two independent TB from each TRP. Therefore, for a CSI report associated with multi-DCI based Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, the UE is expected to report

- Two RIs, two PMIs, two LIs and two CQIs

Proposal 1 For a CSI report associated with multi-DCI based Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, the UE is expected to report

- Two RIs, two PMIs, two LIs and two CQIs

For interference measurement under NCJT, since the transmission from both TRP interfere with each other, it is reasonable that UE should consider the CMR, including RI/PMI decision, from the other TRP as the interference for the intended TRP. In other words, the RI and PMI decision from both TRP should be considered jointly and signals from one TRP should be considered as the interference to the other TRP.

Proposal 2 For interference measurement under NCJT, CMR, including RI/PMI decision, from one TRP should be considered as the interference, i.e. IMR, to the other TRP.

For reporting mechanism, we need to consider the following three cases

- Single TRP operation: Report the best TRP under the assumption that the other TRP is blanked

- Single TRP operation: Report each TRP under the assumption that the other TRP is blanked

- Multiple TRP operation: Report both TRP under NCJT operation

We have the following proposal

Proposal 3 For reporting mechanism, regarding the following three cases

- Single TRP operation: Report the best TRP under the assumption that the other TRP is blanked

- Single TRP operation: Report each TRP under the assumption that the other TRP is blanked

- Multiple TRP operation: Report both TRP under NCJT operation

gNB can configure one or multiple of them

- UE can independently indicate whether UE supports each reporting or not

- The CPU and active RS counting rule needs to be further discussed and clarified

# CSI enhancement with partial DL/UL channel reciprocity

Even for FDD spectrum, it is believed that DL and UL channel still have certain reciprocity, i.e., share similar channel statistic, even though the actual channel can be highly uncorrelated. For DL/UL channel partial reciprocity, it is generally the observation that the following two channel properties can be very similar between DL and UL

- Channel delay and power profile

- Channel angle in terms of AoA/AoD

In general, this is assuming that the DL and UL go through the similar reflection and scattering, and the wavelength of DL and UL signals are also similar even with frequency duplexing separation.

In last meeting, CSI-RS enhancement and potential SD-FD paring concept was discussed. Currently, we support CSI-RS with maximum 32 ports, which can be configured for both reciprocity and non-reciprocity based CSI report. For non-reciprocity based CSI report, gNB needs to sound all the transmit port for UE to feedback the CSI. However, for reciprocity based CSI, gNB already acquires certain DL channel information based on the UL channel estimate, and gNB already precode the CSI-RS with the consideration of the maximum MIMO layers the UE can support, i.e., the maximum Rx ports at the UE side. Normally, the maximum number of Rx ports at the UE side is much smaller than the maximum number of Tx ports at the gNB side, as results, 32 port CSI-RS should be enough for the reciprocity based CSI feedback, i.e., port selection codebook.

On the other side, CSI-RS capability can be an issue for reciprocity based CSI feedback since gNB needs to precode the CSI-RS differently for each user, as different user may experience different channel. There can be a need to increase the multiplexing order, or capability, of CSI-RS by allowing more users to be configured with different CSI-RS with the same amount of REs. After analyzing the CSI-RS configuration allowed by the current specification 38.211, we believe the current CSI-RS already allows very flexible CSI-RS configuration. If there is any enhancement needed, allowing lower density can be a simple and clean solution. In summary, we have the following proposal

Proposal 4 For port selection codebook enhancement,

- Do not introduce SD-FD pairing

- Do not introduce CSI-RS with more than 32 ports

- We can consider CSI-RS enhancement to allow more users to be multiplexed in the same amount of REs, solution can be allowing lower density CSI-RS, e.g., 0.5, 0.25

Delay and power profile determine the frequency selectivity of the channel. To handle the frequency selective channel, Rel-15/16 CSI allows both the Wideband and Subband CSI reporting. For Subband configuration, NW can select one of the two possible subband sizes via RRC for the BWP whose size is greater than or equal to 24 PRB. If NW can obtain the channel frequency selectivity more dynamically via the SRS measurement, more dynamic wideband and subband CSI reporting configuration can be considered

Proposal 5 For CSI enhancement utilizing partial reciprocity of DL/UL channels, more flexible wideband and subband CSI reporting configuration can be considered

# Conclusion

In RAN#86 meeting, Rel-17 FeMIMO WI was approved in RP-193133 [1]. Four main areas haven been identified for the Rel-17 FeMIMO, namely

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2

- Enhancement on SRS, targeting both FR1 and FR2

- Enhancement on CSI measurement and reporting

Regarding CSI measurement and reporting enhancement, in this contribution, we provide our views on the following areas

- CSI measurement and reporting enhancement for MTRP

- Type II port selection codebook enhancement utilizing partial DL/UL channel reciprocity

We have the following proposals

Proposal 1 For a CSI report associated with multi-DCI based Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, the UE is expected to report

- Two RIs, two PMIs, two LIs and two CQIs

Proposal 2 For interference measurement under NCJT, CMR, including RI/PMI decision, from one TRP should be considered as the interference, i.e. IMR, to the other TRP.

Proposal 3 For reporting mechanism, regarding the following three cases

- Single TRP operation: Report the best TRP under the assumption that the other TRP is blanked

- Single TRP operation: Report each TRP under the assumption that the other TRP is blanked

- Multiple TRP operation: Report both TRP under NCJT operation

gNB can configure one or multiple of them

- UE can independently indicate whether UE supports each reporting or not

- The CPU and active RS counting rule needs to be further discussed and clarified

Proposal 4 For port selection codebook enhancement,

- Do not introduce SD-FD pairing

- Do not introduce CSI-RS with more than 32 ports

- We can consider CSI-RS enhancement to allow more users to be multiplexed in the same amount of REs, solution can be allowing lower density CSI-RS, e.g., 0.5, 0.25

Proposal 5 For CSI enhancement utilizing partial reciprocity of DL/UL channels, more flexible wideband and subband CSI reporting configuration can be considered

# Reference

- RP-193133, "New WID: Further enhancements on MIMO for NR", 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019

- Chairman Notes, 3GPP TSG RAN WG1 Meeting #103-e, e-Meeting, October 26th – November 13th, 2020