3GPP TSG RAN WG1 Meeting #104-e	R1-2101407

Jan 25th – Feb 5th, 2021

Agenda Item:	8.8.1.1

Source:	IITH, CeWiT, IITM, Reliance Jio, Tejas Networks

Title:	PUSCH Repetitions for Coverage Enhancement

Document for:	Discussion and decision

# Motivation

In RAN#90e, NR coverage enhancement was approved as a work item [1] that will address the following objectives

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots.

  - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary, by RAN4 [RAN1, RAN4]

    - Potential optimization of DMRS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

- Specification of PUCCH enhancements [RAN1, RAN4]

  - Specify signaling mechanism to support dynamic PUCCH repetition factor indication [RAN1]

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

- Specify mechanism(s) to support Type A PUSCH repetitions for Msg3 [RAN1]

Further, the following are the coverage goals that should be achieved using these enhancements as agreed in [2]-

- For Scenario dependent targets, e.g., ISD/MPL

- For each scenario, multiple target ISD values can be used to draw observations, and a single target ISD value can be used to identify bottlenecks (if applicable)

- Target ISD values for each scenario are as follows:

- Urban 4GHz TDD –400, 500m for observation and 400m for bottleneck identification

- Urban 2.6GHz TDD –400, 500m for observation and 400m for bottleneck identification

- Rural 4GHz TDD NLOS O2I – 1732 and 3000 m for observation and 1732m with 33dBm/MHz BS transmit power for bottleneck identification

- Rural 2.6 GHz TDD NLOS O2I – 1732m for observation and bottleneck identification

- Rural 2 GHz FDD NLOS O2I – 1732m for observation and bottleneck identification

- Rural 700MHz FDD NLOS O2I –3000m, 4000m for observation and 4000m for bottleneck identification

In this document, we will discuss about how repetitions of the type-A can be used to enhance the coverage of the PUSCH channel. We focus on the Rural 4GHz deployment scenario.

# Coverage enhancement using Repetition Type-A

Repetition type A has been agreed as a technique to be studied further for coverage enhancement [1], [2]. For the repetition type-A simulations, we generate 1 new transport block (TB) and transmit it for numberOfRepetitions-r16 number of uplink slots, where, numberOfRepetitions-r16 is the RRC parameter defining the number of repetitions as defined in Rel-16. At the end of the numberOfRepetitions-r16 th slot transmission current TB is dropped and a new TB is generated for next numberOfRepetitions-r16 uplink slot transmission.

To show the performance of repetition type-A for coverage scenarios, we have considered MCS 3 and 4 PRBs to achieve defined throughput for the scenario defined in Table 2. A total of two DMRS symbols are allocated at 2nd and 11th position in the slot. We performed link level simulations to find the required numberOfRepetitions-r16 value to achieve the target ISD 1.732Km in Rural 4 GHz NLOS TDD setting.

The following simulation criteria is considered in our analysis

Table 1: Simulation settings for repetition type A

| Parameter | Value |
| --- | --- |
| Frequency | 4 GHz (TDD) |
| Frame structure for TDD | DDDSU only for 4GHz; No scheduling for UL in S slot. |
| BWP | 100MHz |
| SCS | 30 kHz |
| Channel Model | TDL-C, 300ns (valid for Rural) |
| UE velocity | 3 kmph |
| Frequency Hopping | No |
| Waveform | DFT-s-OFDM |
| BLER | 10% iBLER |
| HARQ | No |
| UE Tx | 1 |
| gNB Rx | 2 |
| Number of TxRUs for gNB | 2 receive chains simulated in LLS |
| DMRS configuration | 2 symbols, no multiplexing with data. 2nd and 11th OFDM symbols. Type-I |
| TBS size | 288 |
| Repetition | Type-A, 2, 4, 8, 16, 32, 64, 128 repetitions |
| PUSCH duration | 14 OS |
| PUSCH | Mapping type-A |
| Modulation | MCS 3; QPSK Table 6.1.4.1-1: MCS index table for PUSCH with transform precoding and 64QAM 38.214. |
| Channel Estimation | Non-Ideal (channel is estimated using appropriate methods) |

Parameters for link budget evaluation is in table 2 and all the link budget results are summarised in Table 3.

Table 2: Parameters for link budget evaluations

| Parameter | Value |
| --- | --- |
| Fc | 4GHz |
| BS Height | 35m |
| UE Height | 1.5m |
| Lognormal shadow fading std deviation (dB) | 8 dB |
| Pathloss Model | Rural, NLOS, O2I |
| Number of antenna elements | 64 |
| Number of RF chains in LLS | 2 |
| Antenna gain per element | 8 dBi |
| Antenna gain loss | 3 dB |
| Cable loss | 3 dB |
| BS Noise Figure | 5 dB |
| Thermal Noise Density | -174 dBm/Hz |
| Receiver Intf Density | -165 dBm/Hz (from ITU IMT 2020) |
| Receiver implementation margin (dB) | 2 dB |
| Shadow fading margin | 10.26 dB |
| HARQ gain(dB) | 0.5 |

| The required SNR | MCL | MIL | MPL | Range (m) | ISD (m) | LOS/ NLOS | O2I/ O2O | Key assumptions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.73 | 122.73 | 139.78 | 115.52 | 384.15 | 665.34 | NLOS | O2I | numberOfRepetitions-r16 = 1 |
| -1.9 | 125.36 | 142.41 | 118.15 | 449.34 | 778.25 | NLOS | O2I | numberOfRepetitions-r16 = 2 |
| -4.9 | 128.36 | 145.41 | 121.15 | 537.311 | 930.62 | NLOS | O2I | numberOfRepetitions-r16 = 4 |
| -7.2 | 130.48 | 147.53 | 123.27 | 609.67 | 1055.96 | NLOS | O2I | numberOfRepetitions-r16 = 8 |
| -9.08 | 132.54 | 149.59 | 125.33 | 689.323 | 1193.91 | NLOS | O2I | numberOfRepetitions-r16 = 16 |
| -11.64 | 135.1 | 152.15 | 127.89 | 802.95 | 1390.7 | NLOS | O2I | numberOfRepetitions-r16 = 32 |
| -14.2 | 137.66 | 154.11 | 130.45 | 935.29 | 1619.94 | NLOS | O2I | numberOfRepetitions-r16 = 64 |
| -16.82 | 140.28 | 157.33 | 133.07 | 1093 | 1893.15 | NLOS | O2I | numberOfRepetitions-r16 =128 |

Table 3: Link evaluation for repetition type-A for 100Kbps throughput

Figure 1: Link level performance results with different numberOfRepetitions-r16 values

From figure 1, which has link level performance results with different numberOfRepetitions-r16 values, there is ~2-3 dB gain in MPL value every-time when numberOfRepetitions-r16 value is doubled. To achieve the ISD target of the parameters defined in Table 2, a total of 128 repetitions are required to achieve the required coverage.

Proposal: Increase the numberOfRepetitions-r16 value to support 128 repetitions to support coverage enhancement.

Proposal: Handling of special slots in the TDD frame structure when the number of repetitions are increased to 128 may be studied in more detail.

# References

[1] RP-202928, New WID on NR coverage enhancements

[2] RAN1 Chairman’s Report, RAN1#103e, November 2020.