3GPP TSG RAN WG1 Meeting #104-e	R1- 2101436

Jan. 25th – Feb. 5th, 2021

Agenda item:	7.2.4

Source: 	Qualcomm Incorporated

Title: 	Remaining Issues in Mode 1 Resource Allocation

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss remaining issues and corrections for NR V2X specifications regarding Mode 1 resource allocation.

# HARQ Codebook

In [3], it was noted that the HARQ codebook determination when sidelink HARQ-ACK is multiplexed on PUSCH defines two cases based on the presence of the SAI field in the scheduling DCI. Those two cases are explicitly noted a DCI formats which have an SAI field, and DCI 0-0, which doesn’t. However, DCI 0-2 does not have an SAI field and is not captured by the procedure. The proposal in [3] was to update the procedure such that it applies in the same manner to both DCI 0-0 and DCI 0-2. This is a straightforward solution to the issue that minimizes specifications impact and we propose to adopt it.

Proposal 1: Adopt the following the text proposal in 38.213 to clarify how to multiplex SL HARQ-ACK on a PUSCH scheduled by DCI 0-2.

----------------------------------------------------begin text proposal for 38.213----------------------------------------------------

16.5.1.2 Type-1 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format without an SAI field, then

>>>>>>>>>>>>>>>>unchanged text omitted<<<<<<<<<<<<<<<<

16.5.2.2 Type-2 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format without an SAI field, then

-----------------------------------------------------end text proposal for 38.213-----------------------------------------------------

# Conclusion

Proposal 1: Adopt the following the text proposal in 38.213 to clarify how to multiplex SL HARQ-ACK on a PUSCH scheduled by DCI 0-2.

----------------------------------------------------begin text proposal for 38.213----------------------------------------------------

16.5.1.2 Type-1 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format without an SAI field, then

>>>>>>>>>>>>>>>>unchanged text omitted<<<<<<<<<<<<<<<<

16.5.2.2 Type-2 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format without an SAI field, then

-----------------------------------------------------end text proposal for 38.213-----------------------------------------------------

# References

- Chairman’s notes, RAN1 #102-e.

- Chairman’s notes, RAN1 #101-e.

- R1-2008530, “Maintenance for resource allocation mechanism mode 1”, NTT DOCOMO INC., RAN1 #103-e.

- 3GPP TS 38.211 V16.3.0, NR, Physical Channels and Modulation (Release 16).

- 3GPP TS 38.212 V16.3.0, NR, Multiplexing and Channel Coding (Release 16).

- 3GPP TS 38.213 V16.3.0, NR, Physical Layer Procedures for Control (Release 16).

- 3GPP TS 38.214 V16.3.0, NR, Physical Layer Procedures for Data (Release 16).