3GPP TSG RAN WG1 Meeting #104-e	R1-2101532

e-Meeting, January 25th – February 5th, 2021

Source:	Sharp

Title:	Remaining issues on physical layer structure and procedures for NR sidelink

Agenda Item:	7.2.4

Document for:	Discussion and Decision

Introduction

In this document, we share our views about remaining issues on physical layer structure and procedures for NR sidelink.

Discussion

Physical layer structure

Resource blocks for SL CSI-RS

RE mapping for SL CSI-RS is described in clause 8.4.1.5.3 of TS 38.211, as follows,

##### 8.4.1.5.3 Mapping to physical resources Mapping to resource elements shall be done according to clause 7.4.1.5.3 with the following exceptions: - only 1 and 2 antenna ports are supported, X∈1,2; - only density ρ=1 is supported; - zero-power CSI-RS is not supported; - the quantity βCSIRS is an amplitude scaling factor to conform with the transmit power specified in clause 8.2.1 of [6, TS 38.214].

And in clause 7.4.1.5.3 of TS 38.211, the frequency-domain mapping for CSI-RS is described as follows,

##### 7.4.1.5.3 Mapping to physical resources For each CSI-RS configured, the UE shall assume the sequence being mapped to resources elements k,lp,μ according to […] The frequency-domain location is given by a bitmap provided by the higher-layer parameter frequencyDomainAllocation in the CSI-RS-ResourceMapping IE or the CSI-RS-ResourceConfigMobility IE with the bitmap and value of ki in Table 7.4.1.5.3-1 given by - , ki-1=fi for row 1 of Table 7.4.1.5.3-1 - , ki-1=fi for row 2 of Table 7.4.1.5.3-1 - , ki-1=4fi for row 4 of Table 7.4.1.5.3-1 - , ki-1=2fi for all other cases where is the bit number of the bit in the bitmap set to one, repeated across every 1ρ of the resource blocks configured for CSI-RS reception by the UE. The starting position and number of the resource blocks in which the UE shall assume that CSI-RS is transmitted are given by the higher-layer parameters freqBand and density in the CSI-RS-ResourceMapping IE for the bandwidth part given by the higher-layer parameter BWP-Id in the CSI-ResourceConfig IE or given by the higher-layer parameters nrofPRBs in the CSI-RS-CellMobility IE where the the startPRB given by csi-rs-MeasurementBW is relative to common resource block 0.

However, the above yellow-highlighted text for NR Uu CSI-RS is not applicable to SL CSI-RS where the resource blocks are confined within the corresponding PSCCH/PSSCH transmission, as captured in the following RAN1 agreements:

Agreements: - Support at least Sidelink CSI-RS for CQI/RI measurement - Sidelink CSI-RS is confined within the PSSCH transmission

The following TP is proposed to clarify the resource blocks for SL CSI-RS as per the above RAN1 agreements:

##### 8.4.1.5.3 Mapping to physical resources Mapping to resource elements shall be done according to clause 7.4.1.5.3 with the following exceptions: - only 1 and 2 antenna ports are supported, X∈1,2; - only density ρ=1 is supported; - zero-power CSI-RS is not supported; - the quantity βCSIRS is an amplitude scaling factor to conform with the transmit power specified in clause 8.2.1 of [6, TS 38.214]. - The starting position and number of resource blocks in which the UE shall assume the CSI-RS is transmitted is respectively the lowest resource block of the lowest subchannel and the number of resource blocks of the corresponding PSCCH/PSSCH transmission.

Physical layer procedures

Simultaneous PSFCH transmission/reception

The following changes were introduced in RAN1#100bis-e,

#### 16.2.4.2 Simultaneous PSFCH transmission/reception If a UE - would transmit Nsch,Tx,PSFCH PSFCHs and receive Nsch,Rx,PSFCHPSFCHs, and - transmissions of the Nsch,Tx,PSFCH PSFCHs would overlap in time with receptions of the Nsch,Rx,PSFCH PSFCHs the UE transmits or receives only a set of PSFCHs corresponding to the smallest priority field value, as determined by a first set of SCI format 1-A and a second set of SCI format 1-A [5, TS 38.212] that are respectively associated with the Nsch,Tx,PSFCH PSFCHs and the Nsch,Rx,PSFCH PSFCHs.

The corresponding RAN1 agreements are as follows,

Agreements: For the prioritization between PSFCH TX and PSFCH RX, - When the UE is required to transmit more than one PSFCH, the highest priority of the associated PSCCH/PSSCH is used for prioritization of the PSFCH transmission. - When the UE is required to receive more than one PSFCH, the highest priority of the associated PSCCH/PSSCH is used for prioritization of the PSFCH reception.

As can be seen above, the intention of the RAN1 agreements for the yellow highlighted text was that the decision on whether to transmit one set of PSFCHs or receive another set of PSFCHs is based on which set contains a PSFCH with highest priority. However this was unfortunately not correctly reflected in the spec text, which says the selected “set of PSFCHs” should correspond to the smallest priority field value, e.g. if each PSFCH in a set “A”={PSFCH1, PSFCH2, PSFCH3, PSFCH4} for transmission has respective priority field values {2, 2, 3, 5} and each PSFCH in a set “B”={PSFCH5, PSFCH6, PSFCH7, PSFCH8} for reception has respective priority field values {3, 4, 5, 6}, then the spec text says the UE should transmit a subset of “A” = {PSFCH1, PSFCH2}, as they correspond to the smallest priority field value i.e. 2. However, the intention of the RAN1 agreements was actually that since set “A” for transmission contains a PSFCH with a smallest priority field value, the UE shall transmit (PSFCHs in set “A”) rather than receive (PSFCHs in set “B”).

Proposal 1: For simultaneous PSFCH transmission/reception, clarify that it is “a PSFCH” that corresponds to the smallest priority field value, rather than “a set of PSFCHs” that correspond to the smallest priority field value.

The following TP is proposed to clarify the intended UE behaviour in the RAN1 agreements:

#### 16.2.4.2 Simultaneous PSFCH transmission/reception If a UE - would transmit Nsch,Tx,PSFCH PSFCHs and receive Nsch,Rx,PSFCHPSFCHs, and - transmissions of the Nsch,Tx,PSFCH PSFCHs would overlap in time with receptions of the Nsch,Rx,PSFCH PSFCHs the UE transmits or receives only a set of PSFCHs containing a PSFCH that corresponds to the smallest priority field value, as determined by a first set of SCI format 1-A and a second set of SCI format 1-A [5, TS 38.212] that are respectively associated with the Nsch,Tx,PSFCH PSFCHs and the Nsch,Rx,PSFCH PSFCHs.

Interpretation of sidelink slot for resource reservation period

The interpretation of sidelink slot for resource reservation period was kept as a leftover issue in RAN1#103-e. The latest proposal is shown as follows,

Proposal 2 If a UE transmits a SCI format 1-A in slot n in a resource pool, and if “Resource reservation period” in the SCI format 1-A indicates P’ (following 8.1.7 of 38.214), - P’ is counted in t'0SL,t'1SL,t'2SL,… (i.e. The number of slots in the resource pool between slot n and slot n+P’ is always the same as P’ (including slot n+P’ itself)). - Updated equation in 8.1.7 (N is the number of slots belonging to the resource pool)

Besides the resource scattering issue which was raised and discussed for the proposal, another issue would be that the equation in clause 8.1.7 of TS38.214 needs correction. The outcome of [POST111-e][705][V2X] for SL CG resource determination in RAN2 is shown as follows,

Recommendation 1: RAN2 agree to keep the parameter timeReferenceSFN in the equation for sidelink configured grant occasion in section 5.8.3 in TS 38.321. Recommendation 2: RAN2 agree to define a separate sl-TimeReferenceSFN-Type1 in RRC configuration for sidelink configured grant type 1. Recommendation 3: RAN2 agree sl-TimeOffsetCGType1 should be the offset of a resource with respect to SFN = sl-TimeReferenceSFN-Type1. Recommendation 4: RAN2 agree to clarify the definition of N. Recommendation 5: RAN2 agree that N refers to the number of slots that can be used for SL transmission within 20ms, if configured, of TDD-UL-DL-ConfigCommon as specified in TS 38.331 [5] and clause 8.1.7 of TS 38.214 [7]. Recommendation 6: RAN2 agree to clarify the definition of numberOfSLSlotsPerFrame. Recommendation 7: RAN2 agree how to clarify the definition of numberOfSLSlotsPerFrame is FFS and will keep the current text for the moment. Recommendation 8: RAN2 confirms sl-TimeOffsetCG-Type1 refers to the number of logical slots that can be used for SL transmission, as specified in current specification.

It was noticed that RAN2 agreed the parameter N is the number of slots that can be used for SL transmission within 20ms and defined on top of the equation in clause 8.1.7 of TS38.214. If the latest proposal is to be agreed, updating the equation would cause extra effort for RAN2. Thus, we propose to keep the equation as it is in current specs and no specs change would be needed.

Proposal 2: If a UE transmits an SCI format 1-A in slot n in a resource pool, and if “Resource reservation period” in the SCI format 1-A indicates P’(following 8.1.7 of 38.214),

- P’ is counted in t0SL,t1SL,t2SL,... (i.e. The number of slots in the resource pool between slot n and slot n+P’ can be less than P’).

- UE expects tm+j×Prsvp_TX'SL belong to the set of slots assigned to the sidelink resource pool.

Conclusion

In this contribution, we discuss remaining issues on physical layer structure and procedures for NR sidelink, and provide corresponding proposals/TPs.

Proposal 1: For simultaneous PSFCH transmission/reception, clarify that it is “a PSFCH” that corresponds to the smallest priority field value, rather than “a set of PSFCHs” that correspond to the smallest priority field value.

Proposal 2: If a UE transmits an SCI format 1-A in slot n in a resource pool, and if “Resource reservation period” in the SCI format 1-A indicates P’(following 8.1.7 of 38.214),

- P’ is counted in t0SL,t1SL,t2SL,... (i.e. The number of slots in the resource pool between slot n and slot n+P’ can be less than P’).

- UE expects tm+j×Prsvp_TX'SL belong to the set of slots assigned to the sidelink resource pool.

References

RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei, 3GPP RAN#83.