3GPP TSG RAN WG1 #104-e		 R1-2101573

e-Meeting, January 25th – February 5th, 2021

Agenda Item:     7.2.1

Source:	Spreadtrum Communications

Title:	Discussion on remaining issues on 2-step RACH

Document for:	Discussion and decision

Introduction

In this contribution, we present our opinions on the remaining issue for MsgB within 2-step RACH, and provide related text proposal.

Discussion

In current specification [1], regarding to the multiplexing between MsgB and unicast PDSCH, the following restriction is applied. But it seems to allow MsgB and unicast PDSCH TDMed in a slot.

The UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if another PDSCH in the same cell scheduled with RA-RNTI or MSGB-RNTI partially or fully overlap in time.

Observation 1: According to 38.214 g40, MsgB and unicast PDSCH could be TDMed in a slot in one cell.

However, considering the payload size of MsgB is much larger than Msg2 and/or Msg4, it poses more challenge and difficulty for UE processing for the case when MsgB and unicast PDSCH TDMed multiplexing in a slot than Msg2 and unicast PDSCH TDMed multiplexing in a slot. In some degree, the processing capability requirement for MsgB could be equivalent to one unicast PDSCH. If one UE could not support two unicast PDSCHs TDMed in a slot, then it is possibly that the UE could not process MsgB when one MsgB and one unicast PDSCH TDMed multiplexing in a slot. Then, the accessing latency would be increased and the benefit of 2-step RACH would loss. Thus, we have the following proposal:

Proposal 1: The UE is not expected to be scheduled a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI, and another PDSCH in the same cell scheduled with MSGB-RNTI in a slot.

Based on the above proposal, we have the following text proposal#1:

Reason for change: The payload size of MsgB is much larger than Msg2 and/or Msg4. The processing capability requirement for MsgB could be equivalent to unicast PDSCH. For UEs not supporting two unicast PDSCHs TDMed in a slot per CC, MsgB could not be treated. The accessing latency would be increased and the benefit of 2-step RACH would loss. Summary of change: Not support MsgB and unicast PDSCH TDMed multiplexing in a slot per CC. Consequences if not approved: The benefit of 2-step RACH would loss, and even 2-step RACH could not be supported for UEs not supporting 2 unicast PDSCHs TDMed in a slot per CC. Clauses affected: TS38.214 g40, section 5.1 ---------------------------------------Start of Text proposal#1 for TS 38.214------------------------------------- - 5.1 UE procedure for receiving the physical downlink shared channel --------------------------------------------------Unchanged text omitted--------------------------------------------- The UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if another PDSCH in the same cell scheduled with RA-RNTI or MSGB-RNTI partially or fully overlap in time. The UE is not expected to be scheduled a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI, and another PDSCH in the same cell scheduled with MSGB-RNTI in a slot. ---------------------------------------End of Text proposal#1 for TS 38.214--------------------------------------

Conclusion

We have the following observation and proposals regarding to 2-step RACH:

Observation 1: According to 38.214 g40, MsgB and unicast PDSCH could be TDMed in a slot in one cell.

Proposal 1: The UE is not expected to be scheduled a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI, and another PDSCH in the same cell scheduled with MSGB-RNTI in a slot.

Text Proposal#1:

Reason for change: The payload size of MsgB is much larger than Msg2 and/or Msg4. The processing capability requirement for MsgB could be equivalent to unicast PDSCH. For UEs not supporting two unicast PDSCHs TDMed in a slot per CC, MsgB could not be treated. The accessing latency would be increased and the benefit of 2-step RACH would loss. Summary of change: Not support MsgB and unicast PDSCH TDMed multiplexing in a slot. Consequences if not approved: The benefit of 2-step RACH would loss, and even 2-step RACH could not be supported for UEs not supporting 2 unicast PDSCHs TDMed in a slot per CC. Clauses affected: TS38.214 g40, section 5.1 ---------------------------------------Start of Text proposal#1 for TS 38.214------------------------------------- - 5.1 UE procedure for receiving the physical downlink shared channel --------------------------------------------------Unchanged text omitted--------------------------------------------- The UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if another PDSCH in the same cell scheduled with RA-RNTI or MSGB-RNTI partially or fully overlap in time. The UE is not expected to be scheduled a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI, and another PDSCH in the same cell scheduled with MSGB-RNTI in a slot. ---------------------------------------End of Text proposal#1 for TS 38.214--------------------------------------

Reference

- 3GPP TS 38.214: "NR; Physical layer procedures for data", g40