3GPP TSG RAN WG1 Meeting #104-e	R1-21xxxxx

E-Meeting, January 25th – February 5th, 2021

Agenda Item:	6.1

Source:	Moderator (Huawei)

Title:	Feature lead summary #1 on [104-e-LTE-6.1CRs-03]

Document for:	Discussion and Decision

Introduction

This documents summarizes the discussions on 1024QAM proposed by companies [1, 2]

[103-e-LTE-Rel17_NB_IoT_eMTC-01] Email discussion on support of 16-QAM for unicast in UL and DL for NB-IoT – Yubo (Huawei)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Discussion

Issue 1: Correction on spectral efficiency of 1024QAM

| Reason for change: |  | For 1024QAM, the coding rate and efficiency in CQI table are not aligned. |
| --- | --- | --- |
| Summary of change: |  | The efficiency in CQI table for 1024QAM is corrected to be aligned with the coding rate. |
| Consequences if not approved: |  | There may be ambiguity for UEs to calculate the CQI value to be reported. |

<Changes to TS 36.213>

<Unchanged parts are omitted>

### 7.2.3	Channel Quality Indicator (CQI) definition

<Unchanged parts are omitted>

Table 7.2.3-4: 4-bit CQI Table 4

| CQI index | modulation | code rate x 1024 | Efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.3770 |
| 3 | QPSK | 449 | 0.8770 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3301 |
| 15 | 1024QAM | 948 | 9.2578 |

<Unchanged parts are omitted>

Please provide your comments for the proposed CR:

| Companies | Comments |
| --- | --- |

Issue 2: Correction for support of 1024QAM for PDSCH

| Reason for change: | Table 6.3.2-1 (Modulation schemes) does not include support of 1024QAM for PDSCH. |
| --- | --- |
| Summary of change: | Clarify in Table 6.3.2-1 that PDSCH supports 1024QAM. |
| Consequences if not approved: | Specification is inconsistent. |

<Changes to TS 36.211>

6.3.2	Modulation

For each codeword , the block of scrambled bits shall be modulated as described in clause 7.1 using one of the modulation schemes in Table 6.3.2-1, resulting in a block of complex-valued modulation symbols .

Table 6.3.2-1: Modulation schemes

| Physical channel | Modulation schemes |
| --- | --- |
| PDSCH | QPSK, 16QAM, 64QAM, 256QAM, 1024QAM |
| PMCH | QPSK, 16QAM, 64QAM, 256QAM |

Please provide your comments for the proposed CR:

| Companies | Comments |
| --- | --- |

Summary

- References

- R1-2101281	Correction on spectral efficiency of 1024QAM	Huawei, HiSilicon

- R1-2101508	Correction for support of 1024QAM for PDSCH	Qualcomm Incorporated