3GPP TSG RAN WG1 #104-e	R1-2101841

e-Meeting, January 25th – February 5th, 2021

Source:	Moderator (OPPO)

Title:	Summary#1 of email thread [104-e-NR-L1enh-URLLC-02]

Agenda Item:	7.2.5

Document for:	Discussion and Decision

# Introduction

In this paper, discussions under the following email thread in RAN1#104-e are summarized.

[104-e-NR-L1enh-URLLC-02] Email discussion/approval on remaining issues on UCI enhancements – Jia (OPPO) by Feb 3

- Issue 1: Timing for secondary cell activation / deactivation

- Issue 2: Limitation on the number of PUCCHs carrying HARQ-ACK in a slot/subslot

- Issue 3: Conflict between the first PUCCH repetition and semi-static configuration

- Issue 5: Correction for sub-slot based PUCCH

# Remaining issues

## Issue#1: Timing for secondary cell activation / deactivation

## Inputs from Tdocs

This issue was discussed in last meeting. And two options were considered. But no agreement was reached.

- Option 1: Adopt the TP with changing the definition of the slot n

- Option 2: Adopt the TP without changing the definition of the slot n

The proposals submitted to this meeting are listed below:

ZTE proposal:

- In our understanding, the slot with 2/7 OSs only exists in Clause 9 in TS 38.213, but in the other clauses, at least before Clause 9, a slot always contains 14 OSs. Therefore, even if a UE is provided subslotLengthForPUCCH-r16, the k1 in Clause 4 still should be a number of slots with 14 OSs for a PUCCH transmission with HARQ-ACK information for the PDSCH reception.

- Proposal 1: The current spec about the timing for secondary cell activation or deactivation is clear, no changes are needed.

E/// proposal:

Observation 1	When an indicated PDSCH-to-HARQ_feedback timing indicator field in the DCI format scheduling the PDSCH reception for SCell activation determines the number of sub-slots, inconsistency occurs in determination of k given by in Clause 4.3 of TS38.213 since k1 is given in number of sub-slots while  in number of slots.

Observation 2	Solutions to resolve the inconsistency in SCell activation timing due to sub-slot based PUCCH transmission, should maintain the slot granularity (i.e. slot constitute of 14 symbols) for reference slots for PUCCH transmissions and n and k indexes in Clause 4.3 of TS 38.213 to preserved alignment with timing granularity used in TS38.133 for SCell activation where NR slot length with respect to the numerology used in the SCell being activated is assumed.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt the following Text Proposal for Clause 4.3 in TS38.213 resolve the inconsistency in SCell activation timing due to sub-slot based PUCCH transmission.

---------------------------------Start of Text Proposal on TS 38.213 v16.4.0----------------------- - 4.3 Timing for secondary cell activation / deactivation With reference to slots for PUCCH transmissions each consisting of as defined in [4, TS 38.211],when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell ending in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot , except for the following: - the actions related to CSI reporting on a serving cell that is active in slot - the actions related to the sCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies in slot - the actions related to CSI reporting on a serving cell which is not active in slot that the UE applies in the earliest slot after in which the serving cell is active. The value of is where is a number of slots for where it is assumed is the slot with a PUCCH transmission with HARQ-ACK information for the PDSCH reception and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format scheduling the PDSCH reception as described in Clause 9.2.3 and is a number of slots per subframe for the SCS configuration of the PUCCH transmission as defined in [4, TS 38.211]. With reference to slots for PUCCH transmissions, if a UE receives a deactivation command [11, TS 38.321] for a secondary cell ending in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in slot . If the sCellDeactivationTimer associated with the secondary cell expires in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in the first slot that is after slot where is the SCS configuration for PDSCH reception on the secondary cell. ---------------------------------End of Text Proposal on TS 38.213 v16.4.0-----------------------

CATT proposal:

Proposal 2: Adopt the following TP for section 4.3 of TS38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

4.3	Timing for secondary cell activation / deactivation

With reference to slots for PUCCH transmissions, when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell ending in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot , except for the following:

-	the actions related to CSI reporting on a serving cell that is active in slot

-	the actions related to the sCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies in slot

-	the actions related to CSI reporting on a serving cell which is not active in slot that the UE applies in the earliest slot after  in which the serving cell is active.

The value of  is  where   is a number of slots from slot n to a slot for a PUCCH transmission with HARQ-ACK information for the PDSCH reception and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format scheduling the PDSCH reception as described in Clause 9.2.3 and  is a number of slots per subframe for the SCS configuration  of the PUCCH transmission.

With reference to slots for PUCCH transmissions, if a UE receives a deactivation command [11, TS 38.321] for a secondary cell ending in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in slot .

If the sCellDeactivationTimer associated with the secondary cell expires in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in the first slot that is after slot  where  is the SCS configuration for PDSCH reception on the secondary cell.

----------------------------------------------------- End of text proposal ------------------------------------------------------

vivo proposal:

Proposal 1: Slot should be used when defining the effective time of MAC-CE when a UE is provided subslotLength-ForPUCCH.

Proposal 2: Adopt the following text proposal for timing for SCell activation/deactivation in 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

4.3	Timing for secondary cell activation / deactivation

When a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell and would transmit a PUCCH with corresponding HARQ-ACK in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot , except for the following:

-	the actions related to CSI reporting on a serving cell that is active in slot

-	the actions related to the sCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies in slot

-	the actions related to CSI reporting on a serving cell which is not active in slot that the UE applies in the earliest slot after  in which the serving cell is active.

The value of  is   where  is a number of slots per subframe for the SCS configuration  of the PUCCH transmission.

If a UE receives a deactivation command [11, TS 38.321] for a secondary cell and would transmit a PUCCH with corresponding HARQ-ACK in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in slot .

If the sCellDeactivationTimer associated with the secondary cell expires in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in the first slot that is after slot  where  is the SCS configuration for PDSCH reception on the secondary cell.

*** Unchanged text is omitted ***

----------------------------------------------------- End of text proposal ------------------------------------------------------

Text proposal for correction for timing of applying spatial setting for a PUCCH transmission is provided in the following section 2.2.

Nokia proposal:

Proposal 2: Adopt the following draft CR to clarify the SCell activation / deactivation timing with sub-slot PUCCH configuration (changes in green, to be shown as track changes in the final CR)

<omitted text>

4.3	Timing for secondary cell activation / deactivation

With reference to slots for PUCCH transmissions, when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell ending in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot , except for the following:

-	the actions related to CSI reporting on a serving cell that is active in slot

-	the actions related to the sCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies in slot

-	the actions related to CSI reporting on a serving cell which is not active in slot that the UE applies in the earliest slot after  in which the serving cell is active.

The value of  is defined as , where m is the slot where UE transmits PUCCH with HARQ-ACK for the PDSCH carrying the activation command   is  where,  is a number of slots for a PUCCH transmission with HARQ-ACK information for the PDSCH reception and is indicated by the PDSCH-to-HARQfeedback timing indicator field in the DCI format scheduling the PDSCH reception as described in Clause 9.2.3 and  is a number of slots per subframe for the SCS configuration  of the PUCCH transmission.

<omitted text>

vivo proposal:

Proposal 3: Slot should be used when defining the effective time of MAC-CE when a UE is provided subslotLength-ForPUCCH.

Proposal 4: Adopt the following text proposal for timing for SCell activation/deactivation in 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

4.3	Timing for secondary cell activation / deactivation

With reference to slots for PUCCH transmissions, when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell and transmits corresponding HARQ-ACK in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot , except for the following:

-	the actions related to CSI reporting on a serving cell that is active in slot

-	the actions related to the sCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies in slot

-	the actions related to CSI reporting on a serving cell which is not active in slot that the UE applies in the earliest slot after  in which the serving cell is active.

The value of  is   where  is a number of slots per subframe for the SCS configuration  of the PUCCH transmission.

With reference to slots for PUCCH transmissions, if a UE receives a deactivation command [11, TS 38.321] for a secondary cell and transmits corresponding HARQ-ACK in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in slot .

If the sCellDeactivationTimer associated with the secondary cell expires in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in the first slot that is after slot  where  is the SCS configuration for PDSCH reception on the secondary cell.

*** Unchanged text is omitted ***

----------------------------------------------------- End of text proposal ------------------------------------------------------

Text proposal for correction for timing of applying spatial setting for a PUCCH transmission is provided in the following section 2.2.

Fujitsu proposal:

Observation: According to the current specification, in the case of sub-slot based PUCCH transmission, there is a time unit mismatch in the equation for calculating the slot for applying SCell activation or deactivation.

Proposal 1: In the case of sub-slot based PUCCH transmission, the timing for SCell activation or deactivation in Rel-16 shall be the same as the one in Rel-15.

- E.g. the reference point for the 3 ms MAC CE processing gap for the SCell activation or deactivation shall be counted from the end of the slot containing the PUCCH transmission.

Proposal 2: In the case of sub-slot based PUCCH transmission, the k1 for the determination of SCell activation or deactivation timing shall be

- the slot offset between the PDSCH carrying the SCell activation or deactivation command and the PUCCH for the corresponding HARQ-ACK information.

The corresponding text proposals are provided in Section 4.

Text proposal

# 4.3 Timing for secondary cell activation / deactivation With reference to slots for PUCCH transmissions, when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell ending in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot , except for the following: - the actions related to CSI reporting on a serving cell that is active in slot - the actions related to the sCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies in slot - the actions related to CSI reporting on a serving cell which is not active in slot that the UE applies in the earliest slot after in which the serving cell is active. The value of is where is a number of slots for a PUCCH transmission with HARQ-ACK information for the PDSCH reception and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format scheduling the PDSCH reception as described in Clause 9.2.3 and is a number of slots per subframe for the SCS configuration of the PUCCH transmission. In the case that the PUCCH transmission is according to subslotLength-ForPUCCH, is a slot offset between the PUCCH transmission and the corresponding PDSCH reception. With reference to slots for PUCCH transmissions, if a UE receives a deactivation command [11, TS 38.321] for a secondary cell ending in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in slot . If the sCellDeactivationTimer associated with the secondary cell expires in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in the first slot that is after slot where is the SCS configuration for PDSCH reception on the secondary cell. < Unchanged parts are omitted >

Nokia proposal:

Proposal 1: Adopt the following draft CR to clarify the SCell activation / deactivation timing with sub-slot PUCCH configuration (changes in green, to be shown as track changes in the final CR)

<omitted text>

4.3	Timing for secondary cell activation / deactivation

With reference to slots for PUCCH transmissions, when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell ending in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot , except for the following:

-	the actions related to CSI reporting on a serving cell that is active in slot

-	the actions related to the sCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies in slot

-	the actions related to CSI reporting on a serving cell which is not active in slot that the UE applies in the earliest slot after  in which the serving cell is active.

The value of  is  where  is a number of slots, composed of   symbols [4], from slot n to a slot for a PUCCH transmission with HARQ-ACK information for the PDSCH reception and is indicated by the PDSCH-to-HARQ feedback timing indicator field in the DCI format scheduling the PDSCH reception as described in Clause 9.2.3 and  is a number of slots per subframe for the SCS configuration  of the PUCCH transmission.

<omitted text>

Huawei proposal:

Observation 1: By changing the reference slot (slot n) for Scell activation or deactivation to the slot of the PUCCH transmission but not the slot of the PDSCH carrying the activation command, and modifying the value of k from  to , there remains no ambiguity in [1] and no NBC problem will occur.

Proposal 1: Change the referent slot for Scell activation or deactivation to the slot of the PUCCH transmission but not the slot of the PDSCH carrying the activation command in 38.213.

## 1st round proposal and discussion

Considering the TP from Ericsson, companies are encouraged to express your views:

---------------------------------Start of Text Proposal on TS 38.213 v16.4.0----------------------- - 4.3 Timing for secondary cell activation / deactivation With reference to slots for PUCCH transmissions each consisting of as defined in [4, TS 38.211],when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell ending in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot , except for the following: - the actions related to CSI reporting on a serving cell that is active in slot - the actions related to the sCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies in slot - the actions related to CSI reporting on a serving cell which is not active in slot that the UE applies in the earliest slot after in which the serving cell is active. The value of is where is a number of slots for where it is assumed is the slot with a PUCCH transmission with HARQ-ACK information for the PDSCH reception and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format scheduling the PDSCH reception as described in Clause 9.2.3 and is a number of slots per subframe for the SCS configuration of the PUCCH transmission as defined in [4, TS 38.211]. With reference to slots for PUCCH transmissions, if a UE receives a deactivation command [11, TS 38.321] for a secondary cell ending in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in slot . If the sCellDeactivationTimer associated with the secondary cell expires in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in the first slot that is after slot where is the SCS configuration for PDSCH reception on the secondary cell. ---------------------------------End of Text Proposal on TS 38.213 v16.4.0-----------------------

| Company | Comments |
| --- | --- |
| HW/HiSi | Fine with the intention of the TP. We suggest the following two minor modifications. - In the insertion in the first row, could we add “symbols” after the , i.e. transmissions each consisting of symbols as defined in [4, TS 38.211], - The second insertion, in the middle of the TP, maybe it is better to add “slot”, i.e. The value of is where is a number of slots for where it is assumed slot is the slot with.. |

## Issue#2: Limitation on the number of PUCCHs carrying HARQ-ACK in a slot/subslot

## Inputs from Tdocs

Nokia proposal:

Proposal 2: Adopt the following draft CR to clarify that more than one PUCCH with HARQ-ACK information may be transmitted in a sub-slot by UE with two PUCCH-configs  (changes in green, to be shown as track changes in the final CR).

9	UE procedure for reporting control information

<omitted text>

If a UE is provided two PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH-r16 in the first PUCCH-Config, the PUCCH resource for any SR configuration with priority index 0 or any CSI report configuration in any PUCCH-Config is within the subslotLengthForPUCCH-r16 symbols in the first PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH-r16 in the second PUCCH-Config, the PUCCH resource for any SR configuration with priority index 1 in any PUCCH-Config is within the subslotLengthForPUCCH-r16 symbols in the second PUCCH-Config

-	the UE shall separately apply the procedure described in Clause 9.2.3 for reporting HARQ-ACK information in a PUCCH resource of the first and second PUCCH-Config.

<omitted text>

Xiaomi proposal:

9.2.3  UE procedure for reporting HARQ-ACK

If UE is provided subslotLengthForPUCCH in PUCCH-Config, a UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a subslot, otherwise, a UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot

DOCOMO proposal:

---------------------------------Start of Text Proposal on TS 38.213 v16.4.0-----------------------

9.2.3	UE procedure for reporting HARQ-ACK

A UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot per priority.

<Unchanged parts are omitted>

----------------------------------End of Text Proposal on TS 38.213 v16.4.0-------------------------

## 1st round proposal and discussion

Considering the TP from Nokia and DOCOMO, companies are encouraged to express your views:

---------------------------------Start of Text Proposal on TS 38.213 v16.4.0----------------------- 9 UE procedure for reporting control information <omitted text> If a UE is provided two PUCCH-Config - if the UE is provided subslotLengthForPUCCH-r16 in the first PUCCH-Config, the PUCCH resource for any SR configuration with priority index 0 or any CSI report configuration in any PUCCH-Config is within the subslotLengthForPUCCH-r16 symbols in the first PUCCH-Config - if the UE is provided subslotLengthForPUCCH-r16 in the second PUCCH-Config, the PUCCH resource for any SR configuration with priority index 1 in any PUCCH-Config is within the subslotLengthForPUCCH-r16 symbols in the second PUCCH-Config - the UE shall separately apply the procedure described in Clause 9.2.3 for reporting HARQ-ACK information in a PUCCH resource of the first and second PUCCH-Config. <omitted text> ---------------------------------End of Text Proposal on TS 38.213 v16.4.0-----------------------

---------------------------------Start of Text Proposal on TS 38.213 v16.4.0----------------------- 9 9.2.3 UE procedure for reporting HARQ-ACK A UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot per priority. <Unchanged parts are omitted> ---------------------------------End of Text Proposal on TS 38.213 v16.4.0-----------------------

| Company | Comments |
| --- | --- |
| HW/HiSi | Agree with both TPs |

## Issue#3: Conflict between the first PUCCH repetition and semi-static configuration

## Inputs from Tdocs

CATT proposal:

Proposal 1: RRC configured UCI transmission with PUCCH repetition is dropped from the first slot if the PUCCH in the first slot collides with semi-static DL symbol(s) or SSB symbols.

A text proposal is provided below for TS38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9.2.6	PUCCH repetition procedure

<Unchanged text omitted>

For unpaired spectrum, the UE determines the  slots for a PUCCH transmission starting from a slot indicated to the UE as described in Clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in Clause 9.2.4 for SR reporting or in Clause 5.2.1.4 of [6, TS 38.214] for CSI reporting and having

-	an UL symbol, as described in Clause 11.1, or flexible symbol that is not SS/PBCH block symbol provided by startingSymbolIndex in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 as a first symbol, and

-	consecutive UL symbols, as described in Clause 11.1, or flexible symbols that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4

except that for repetitions of a PUCCH transmission which is not in response to a DCI format detection and a slot indicated to the UE as described in Clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in Clause 9.2.4 for SR reporting or in Clause 5.2.1.4 of [6, TS 38.214] for CSI reporting does not satisfy the above conditions, the UE does not transmit the PUCCH transmission.

For paired spectrum, the UE determines the  slots for a PUCCH transmission as the  consecutive slots starting from a slot indicated to the UE as described in Clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in Clause 9.2.4 for SR reporting or in Clause 5.2.1.4 of [6, TS 38.214] for CSI reporting.

----------------------------------------------------- End of text proposal ------------------------------------------------------

Huawei proposal:

Observation 2: In order to transmit UCI,

- Multiplexing before cancellation is better in case CSI overlaps with DL symbols and CG PUSCH, and CG PUSCH does not overlap with DL symbols (Example of  Figure 1)

- Cancellation before multiplexing is better when  CG PUSCH overlaps with DL symbols and CSI, and CSI does not overlap with DL symbols (Example of Figure 2)

Observation 3: For the case when CSI 1 overlaps with DL symbols only, CSI 2 overlaps with HARQ-ACK only, in order to transmit UCI as much as possible, which of the options is better and should be used depends on whether the PUCCH carrying multiplexed PUCCH indicated by gNB can overlap with DL symbols or not.

Proposal 2: In Rel-16, RAN1 needs to consider collision cases e.g. PUCCH/PUCCH collision and PUCCH/PUSCH collision to decide the processing order between multiplexing and cancellation.

Proposal 3: In Rel-16, RAN1 shall first clarify the processing order of multiplexing and cancellation when PUCCH/PUSCH with same priority collides with semi-static DL symbols/SSBs, then clarify the different priority cases if necessary.

## 1st round proposal and discussion

Considering the TP from CATT, companies are encouraged to express your views:

---------------------------------Start of Text Proposal on TS 38.213 v16.4.0----------------------- 9.2.6 PUCCH repetition procedure <Unchanged text omitted> For unpaired spectrum, the UE determines the slots for a PUCCH transmission starting from a slot indicated to the UE as described in Clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in Clause 9.2.4 for SR reporting or in Clause 5.2.1.4 of [6, TS 38.214] for CSI reporting and having - an UL symbol, as described in Clause 11.1, or flexible symbol that is not SS/PBCH block symbol provided by startingSymbolIndex in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 as a first symbol, and - consecutive UL symbols, as described in Clause 11.1, or flexible symbols that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 except that for repetitions of a PUCCH transmission which is not in response to a DCI format detection and a slot indicated to the UE as described in Clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in Clause 9.2.4 for SR reporting or in Clause 5.2.1.4 of [6, TS 38.214] for CSI reporting does not satisfy the above conditions, the UE does not transmit the PUCCH transmission. For paired spectrum, the UE determines the slots for a PUCCH transmission as the consecutive slots starting from a slot indicated to the UE as described in Clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in Clause 9.2.4 for SR reporting or in Clause 5.2.1.4 of [6, TS 38.214] for CSI reporting. ---------------------------------End of Text Proposal on TS 38.213 v16.4.0-----------------------

| Company | Comments |
| --- | --- |
| HW/HiSi | We do not agree with the TP. In our view the gNB should guarantee that the first repetition doesn’t collide with DL also in the case of semi-static PUCCH transmission. |

## Issue#5: Correction for sub-slot based PUCCH

## Inputs from Tdocs

Not all the slots in section 9 should be replaced by sub-slot when a UE is provided subslotLength-ForPUCCH

CATT proposal:

Proposal 3: For SR reporting and the timing of applying spatial setting for a PUCCH transmission, slot should not be replaced by sub-slot.

A text proposal is provided below for sub-slot based transmission in 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9	UE procedure for reporting control information

<Unchanged text omitted>

In the remaining of this Clause except for section 9.2.2 and 9.2.4, if a UE is provided subslotLengthForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLengthForPUCCH.

----------------------------------------------------- End of text proposal ------------------------------------------------------

vivo proposal:

Proposal 5: Adopt the following text proposal for sub-slot based PUCCH transmission in 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9	UE procedure for reporting control information

*** Unchanged text is omitted ***

In the remaining of this Clause except for section 9.2.2, 9.2.4, 9.2.5 for multiple PUCCH resources in a slot to transmit CSI reports and 9.2.6, if a UE is provided subslotLength-ForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLength-ForPUCCH.

*** Unchanged text is omitted ***

----------------------------------------------------- End of text proposal ------------------------------------------------------

## 1st round proposal and discussion

Considering the TP from vivo, companies are encouraged to express your views:

---------------------------------Start of Text Proposal on TS 38.213 v16.4.0----------------------- 9 UE procedure for reporting control information *** Unchanged text is omitted *** In the remaining of this Clause except for section 9.2.2, 9.2.4, 9.2.5 for multiple PUCCH resources in a slot to transmit CSI reports and 9.2.6, if a UE is provided subslotLength-ForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLength-ForPUCCH. *** Unchanged text is omitted *** ---------------------------------End of Text Proposal on TS 38.213 v16.4.0-----------------------

| Company | Comments |
| --- | --- |

# Conclusions

# References

- R1-2100092	Discussion on the timing for secondary cell activation and deactivation	ZTE

- R1-2100266	Maintenance of UCI for NR URLLC	Ericsson

- R1-2100337	Remaining issues on UCI enhancements	CATT

- R1-2100413	Maintenance on UCI enhancements	vivo

- R1-2100736	A remaining issue on timing for applying SCell activation or deactivation	Fujitsu

- R1-2100827	Draft CRs on timing of SCell activation/deactivation for sub-slot PUCCH and number of PUCCHs with HARQ-ACK in a slot/sub-slot	Nokia, Nokia Shanghai Bell

- R1-2101091	Correction on UE procedure for reporting HARQ-ACK	Xiaomi

- R1-2101236	Remaining issues on UCI enhancements	Huawei, HiSilicon

- R1-2101347	Remaining issues on UCI and PUSCH enhancements for eURLLC	Apple

- R1-2101584	Corrections on UCI enhancement for Rel-16 URLLC	NTT DOCOMO, INC.