3GPP TSG RAN WG1 Meeting #104-e			R1-2102247

e-Meeting, January 25th – February 5th, 2021

Title:	DRAFT LS on TCI State Update for L1/L2-Centric Inter-Cell Mobility

Response to:

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Source:	Samsung

To:	RAN2

Cc:	RAN3, RAN4

Contact Person:

#### Name:	Eko Onggosanusi

E-mail Address:	eko.o@samsung.com

1. Overall Description:

RAN1 discussed the support of L1/L2-centric inter-cell mobility and made the following agreement.

Agreement

On Rel.17 enhancements for L1/L2-centric inter-cell mobility,

- Discuss whether to support at least the source RS types already agreed for intra-cell mobility for the purpose of referencing to non-serving cell(s). Note: This implies the following source RS(s):

  - CSI-RS for BM configured for non-serving cell(s) for DL QCL and UL TX spatial references

  - CSI-RS for tracking (TRS) configured for non-serving cell(s) for DL QCL and UL TX spatial references

  - SSB configured for non-serving cell(s) for UL TX spatial references

  - SRS for BM configured for non-serving cell(s) for UL TX spatial references

  - FFS: whether to support CSI-RS for mobility

  - FFS: whether to support other source RS(s) potentially agreed later for intra-cell mobility

  - FFS: whether to support CSI-RS for BM and tracking configured for non-serving cell(s) and without non-serving cell SSB as QCL-TypeD source

- Send an LS to RAN2 on TCI state update (beam indication) using source RS configured for non-serving cell(s) for DL reception and UL transmission. The following topics are considered for the LS:

  - RRC configuration issues

  - Serving cell issues

  - C-RNTI issues

  - Issues related to CU-DU split

  - Inter-band CA issues

  - Inter-frequency issues

As a part of the Rel-17 NR_FeMIMO WID wherein the group is tasked to “identify and specify features to facilitate more efficient (lower latency and overhead) DL/UL beam management to support higher intra- and L1/L2-centric inter-cell mobility”, RAN1 is currently investigating TCI state update (beam indication) for DL reception from and UL transmission to non-serving cell(s) – at least on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH. In this case, the TCI is associated with a source RS configured for the non-serving cell(s).

For this purpose, RAN1 seeks a few answers from RAN2 on the following questions in order to proceed further.

Question 1: In regard of RRC configuration:

- Is RRC reconfiguration signaling needed for DL reception from or UL transmission to a non-serving cell, at least on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH? If so, which parameter(s)?

- Can some RRC parameters related to the non-serving cell(s) be updated via dynamic signaling (e.g. MAC CE and/or DCI) without any additional RRC reconfiguration signaling?

Question 2: In regard of serving cell, is a UE expected to change its serving cell for DL reception from or UL transmission to another (i.e. a non-serving) cell, at least on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH? If so, what would be the higher-layer impact?

Question 3: In regard of C-RNTI:

- In what condition(s) does a UE require C-RNTI update for DL reception from and UL transmission to a non-serving cell, at least on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH?

- In such condition(s), if any, is RRC reconfiguration signaling or some other (dynamic) signaling needed for the C-RNTI update?

Question 4: In regard of CU-DU split, by restricting the above feature only for intra-DU scenarios (instead of allowing inter-DU scenarios as well), what would be the difference in terms of the following?

- The associated RAN2 specification impact

- The implication in applicable use cases and inter-operability (e.g. across different gNB vendors)

Question 5: In regard of inter-band CA issues, what would be the higher-layer impact assuming intra-band CA as opposed to inter-band CA?

Question 6: In regard of inter-frequency issues, what would be the higher-layer impact assuming intra-frequency scenarios as opposed to inter-frequency scenarios?

2. Actions:

To: RAN2

ACTION: 	RAN1 respectfully asks RAN2 to provide answers for the above questions with additional details that RAN1 shall further consider

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #104bis-e                       12th – 20th April, 2021		E-meeting