3GPP TSG RAN WG1 Meeting #104-e			R1-2102247

e-Meeting, January 25th – February 5th, 2021

Title:	DRAFT LS on TCI State Update for L1/L2-Centric Inter-Cell Mobility

Response to:

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Source:	Samsung

To:	RAN2, RAN3, RAN4

Cc:	RAN

Contact Person:

#### Name:	Eko Onggosanusi

E-mail Address:	eko.o@samsung.com

1. Overall Description:

RAN1 discussed the support of L1/L2-centric inter-cell mobility and made the following agreement.

Agreement

On Rel.17 enhancements for L1/L2-centric inter-cell mobility,

- Discuss whether to support at least the source RS types already agreed for intra-cell mobility for the purpose of referencing to non-serving cell(s). Note: This implies the following source RS(s):

  - CSI-RS for BM configured for non-serving cell(s) for DL QCL and UL TX spatial references

  - CSI-RS for tracking (TRS) configured for non-serving cell(s) for DL QCL and UL TX spatial references

  - SSB configured for non-serving cell(s) for UL TX spatial references

  - SRS for BM configured for non-serving cell(s) for UL TX spatial references

  - FFS: whether to support CSI-RS for mobility

  - FFS: whether to support other source RS(s) potentially agreed later for intra-cell mobility

  - FFS: whether to support CSI-RS for BM and tracking configured for non-serving cell(s) and without non-serving cell SSB as QCL-TypeD source

- Send an LS to RAN2 on TCI state update (beam indication) using source RS configured for non-serving cell(s) for DL reception and UL transmission. The following topics are considered for the LS:

  - RRC configuration issues

  - Serving cell issues

  - C-RNTI issues

  - Issues related to CU-DU split

  - Inter-band CA issues

  - Inter-frequency issues

As a part of the Rel-17 NR_FeMIMO WID wherein the group is tasked to “identify and specify features to facilitate more efficient (lower latency and overhead) DL/UL beam management to support higher intra- and L1/L2-centric inter-cell mobility” as well as “QCL/TCI-related enhancements to enable inter-cell multi-TRP operations”, RAN1 is currently investigating TCI state update (beam indication) for DL reception from and UL transmission to non-serving cell(s) – at least on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH. In this case, the TCI can be associated with source RS(s) configured for the non-serving cell(s), if supported. It is noted that a non-serving cell is differentiated from the serving cell by PCI.

For the aforementioned purpose (in the context of L1/L2-centric inter-cell mobility and inter-cell multi-TRP operations), RAN1 seeks a few answers from RAN2 on the following questions in order to proceed further.

Question 1: In regard of serving cell,

- Is there a need for a UE to change its serving cell for DL reception from or UL transmission to another (non-serving) cell, at least on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH?

- If so, how can the addition, release or change of a non-serving cell for DL reception and/or UL transmission be done For example, would any of such actions require L3 handover and/or selection/activation among pre-configured candidate cells from RAN2 perspective?

- If so, how can the TCI states associated with the previous serving cell be handled?

- If so, what is the impact on the system information reception by the UE?

- If so, what is the impact on the RACH and PUCCH-related procedures and configurations?

- If not, what is the impact on the applicable use cases? That is, in what scenarios can the UE be configured for DL reception from or UL transmission to another (non-serving) cell, at least on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH, if the serving cell does not change?

Question 2: In regard of RRC configuration, RAN1 is discussing whether to allow a UE to be configured for DL reception from or UL transmission to a non-serving cell on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH. From RAN2 perspective

- Depending on the answer to question 1-1, what would be the impact of allowing the UE to transmit and/or receive on some or all of those channels and which RRC parameter(s) would need to be reconfigured for the UE?

- Is it feasible to update some of the above RRC parameter(s) via dynamic signaling (e.g. MAC CE and/or DCI, potentially selecting pre-configured values) without any additional RRC reconfiguration signaling?

Question 3: In regard of C-RNTI:

- Is there a need to assign a UE a separate C-RNTI for DL reception from and UL transmission to a non-serving cell, or can the same C-RNTI from the serving cell be reused, at least for transmission and reception on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH?

- In restricting the use of the same C-RNTI for serving and non-serving cells, what would be the impact in applicable use cases and/or required specification support, if any?

- If separate C-RNTIs are considered necessary, in some cases, for serving and non-serving cells, how would this be configured for UE, i.e. is RRC reconfiguration signaling or some other (dynamic) signaling needed for configuring the separate C-RNTI(s)?

Question 4: In regard of CU-DU split, from RAN2/3 perspective, is there any difference between supporting intra-DU only and supporting inter- in addition to intra-DU, in terms of the following?

- The associated RAN2 specification impact,

- Applicable use cases (e.g. deployment scenarios), and

- Network inter-operability (e.g. across different gNB vendors)

Question 5: In regard of CA issues, RAN1 is discussing whether the operation is supported only for intra-band CA scenario (i.e. UE is configured to operate with serving and non-serving cells that belong to the same frequency band) or for both intra-band CA and inter-band CA scenarios. Note that one common TCI state ID associated with a non-serving cell, if supported, may be optionally applied for CCs in a band.

- Are there specific RAN2/4 issues (including higher-layer impact) that need to be considered for deciding  between the two alternatives?

Question 6: In regard of inter-frequency issues, from RAN2/4 perspective, what would be the higher-layer and RRM impact assuming inter-frequency scenarios as opposed to intra-frequency scenarios? For intra-frequency scenario, it is assumed that SSBs of non-serving cells have the same center frequency and SCS as the SSBs of the serving cell.

- Note: RAN1 has agreed to support intra-frequency scenarios, whereas the support for inter-frequency scenarios is still for further study.

2. Actions:

To: RAN2, RAN3, RAN4

ACTION: 	RAN1 respectfully asks RAN2 to provide answers for the above questions related to signalling or connection control procedures (questions 1 to 6) with additional details that RAN1 shall further consider. RAN1 also respectfully asks RAN3 to provide answers for the above questions related to CU-DU split (question 4) with additional details that RAN1 shall further consider. RAN1 also respectfully asks RAN4 to provide answers for the above questions related to frequency band and CA (questions 5 and 6) with additional details that RAN1 shall further consider.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #104bis-e                       12th – 20th April, 2021		E-meeting