3GPP TSG RAN WG1 Meeting #104-e                                                                     R1-210xxxx

e-Meeting, Jan. 25th – Feb. 5th, 2021

Source:              Moderator (ZTE)

Title:                   FL summary #1 on SRS enhancements

Agenda Item:     8.1.3

Document for:   Discussion and Decision

- Antenna switching up to 8Rx

  - Resource set configurations

FL proposal 3-1:

- For aperiodic antenna switching SRS, support to configure N <=N_max resource sets, where totally K resources are distributed in the N resource sets flexibly based on RRC configuration.

- For 1T6R, K=6, N_max = [4], and each resource has 1 port.

- For 1T8R, K=8, N_max = [4], and each resource has 1 port.

- For 2T6R, K=3, N_max = [3], and each resource has 2 ports.

- For 2T8R, K=4, N_max = [4], and each resource has 2 ports.

- For 4T8R, K=2, N_max = [2], and each resource has 4 ports.

- At least more than one candidate value for N is supported for each xTyR. FFS the supported candidate values.

- FFS extension to increase N_max for 1T4R, 2T4R, T=R and 1T2R cases

- FFS the number of resources and resource sets for semi-persistent and periodic antenna switching SRS

Companies’ further views are collected as follows.

| Companies | Views |
| --- | --- |

- Coverage and capacity enhancements

FL Proposal 4-1: For Rel-17 SRS capacity and coverage enhancement, support the following

- Increase the maximum number of repetition symbols in one slot and one SRS resource to S

  - Support at least one S value from {8, 10, 12, 14}

    - FFS other candidate values

- Support to transmit SRS only in 1PFmSRS, BSRS contiguous RBs in one OFDM symbol, where mSRS, BSRS indicates the number of RBs configured by BSRS and CSRS

  - Support at least one PF value from {2, [3], 4, 8}

    - FFS other candidate values, e.g., non-integer values for PF

  - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued.

  - FFS it is applicable to frequency hopping only, or both frequency hopping and non-frequency hopping

  - FFS detailed signaling mechanism to determine PF and the location of the 1PFmSRS, BSRS RBs, potentially taking non-frequency hopping case into account

- Support Comb 8

  - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued.

- Support omitting SRS transmission on the whole mSRS, BSRS RBs in a frequency hop

- FFS whether and if needed, how to use harmonized approach to define the three supported schemes

- Note: other schemes for SRS capacity and coverage enhancements are not supported in Rel-17.

Companies’ further views are collected as follows.

| Companies | Views |
| --- | --- |

- Conclusion