3GPP TSG RAN WG1#104-e		R1-2xxxxxx

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.2.2

Source:	Moderator (none)

Title:	[Draft] PDCCH Monitoring Alternatives

Document for:	Discussion, Decision

# Discussion on PDCCH Monitoring Alternatives

## Current version (as of Tuesday 01:05 UTC) – with markup

- Alt 1: A fixed pattern of X slots.

  - The different X slot groups are consecutive and do not overlap

  - PDCCH could be configured in the first Y consecutive slots within each X slot group

  - BD/CCE budget is counted within the Y slots of each X slot group, and different X slot groups do not overlap

  - Alt 1-1: Y<X, BD/CCE budget is counted within the first Y slots of each X slot group, and the X slot groups do not overlap for different countings.

    - FFS: The Y slots are the first Y slots within the X slot group or not

  - Alt 1-2: Y=X, BD/CCE budget is counted for each X=Y slot group, and the X slot groups do not overlap for different countings.

  - Note: Y is used to facilitate discussion. If Alt 1-2 is agreed, Y is not needed.

- Alt 2: Use (X,Y) similar to the Rel-16 capability (pdcch-Monitoring-r16, (X, Y) span) as the baseline to define the new capability

  - Y<=X

  - PDCCH could be configured such that the developed span pattern by SS configuration satisfy (X,Y) requirement, i.e. the start of any two span of at most Y symbols/slots is separated by at least X symbols/slots

  - BD/CCE budget is counted for each span of at most Y symbols/slots

  - FFS: Values of X and Y and units in which they are defined

  - FFS: Whether number of slots within which the number of monitoring occasions is counted is needed and if needed, the value of the number of slots

- Alt 3: A sliding window of X=Y slots for defining multi-slot PDCCH monitoring capability.

  - The slot groups are sliding in unit of [1] slot

  - PDCCH could be configured in any slot

  - BD/CCE budget is counted within any slot group consecutive X=Y slots

  - FFS: Increments in which sliding occurs

## Clean version (as of Tuesday 1:05 UTC)

- Alt 1: A fixed pattern of X slots.

  - The different X slot groups are consecutive and do not overlap

  - PDCCH could be configured in Y consecutive slots within each X slot group

  - BD/CCE budget is counted within the Y slots of each X slot group

  - Alt 1-1: Y<X

    - FFS: The Y slots are the first Y slots within the X slot group or not

  - Alt 1-2: Y=X

  - Note: Y is used to facilitate discussion. If Alt 1-2 is agreed, Y is not needed.

- Alt 2: Use (X,Y) similar to the Rel-16 capability (pdcch-Monitoring-r16, (X, Y) span) as the baseline to define the new capability

  - Y<=X

  - PDCCH could be configured such that the developed span pattern by SS configuration satisfy (X,Y) requirement, i.e. the start of any two span of at most Y symbols/slots is separated by at least X symbols/slots

  - BD/CCE budget is counted for each span of at most Y symbols/slots

  - FFS: Values of X and Y and units in which they are defined

- Alt 3: A sliding window of X=Y slots for defining multi-slot PDCCH monitoring capability.

  - The slot groups are sliding in unit of [1] slot

  - PDCCH could be configured in any slot

  - BD/CCE budget is counted within any slot group

## Update from Company X