3GPP TSG RAN WG1#104-e		R1-2xxxxxx

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.2.2

Source:	Moderator (none)

Title:	[Draft] PDCCH Monitoring Alternatives

Document for:	Discussion, Decision

Discussion on PDCCH Monitoring Alternatives

Current version (as of Tuesday 01:05 UTC) – with markup

- Alt 1: A fixed pattern of X slots.

  - The different X slot groups are consecutive and do not overlap

  - PDCCH could be configured in the first Y consecutive slots within each X slot group

  - BD/CCE budget is counted within the Y slots of each X slot group, and different X slot groups do not overlap

  - Alt 1-1: Y<X, BD/CCE budget is counted within the first Y slots of each X slot group, and the X slot groups do not overlap for different countings.

    - FFS: The Y slots are the first Y slots within the X slot group or not

  - Alt 1-2: Y=X, BD/CCE budget is counted for each X=Y slot group, and the X slot groups do not overlap for different countings.

  - Note: Y is used to facilitate discussion. If Alt 1-2 is agreed, Y is not needed.

- Alt 2: Use (X,Y) similar to the Rel-16 capability (pdcch-Monitoring-r16, (X, Y) span) as the baseline to define the new capability

  - Y<=X

  - PDCCH could be configured such that the developed span pattern by SS configuration satisfy (X,Y) requirement, i.e. the start of any two span of at most Y symbols/slots is separated by at least X symbols/slots

  - BD/CCE budget is counted for each span of at most Y symbols/slots

  - FFS: Values of X and Y and units in which they are defined

  - FFS: Whether number of slots within which the number of monitoring occasions is counted is needed and if needed, the value of the number of slots

- Alt 3: A sliding window of X=Y slots for defining multi-slot PDCCH monitoring capability.

  - The slot groups are sliding in unit of [1] slot

  - PDCCH could be configured in any slot

  - BD/CCE budget is counted within any slot group consecutive X=Y slots

  - FFS: Increments in which sliding occurs

Clean version (as of Tuesday 1:05 UTC)

- Alt 1: A fixed pattern of X slots.

  - The different X slot groups are consecutive and do not overlap

  - PDCCH could be configured in Y consecutive slots within each X slot group

  - BD/CCE budget is counted within the Y slots of each X slot group

  - Alt 1-1: Y<X

    - FFS: The Y slots are the first Y slots within the X slot group or not

  - Alt 1-2: Y=X

  - Note: Y is used to facilitate discussion. If Alt 1-2 is agreed, Y is not needed.

- Alt 2: Use (X,Y) similar to the Rel-16 capability (pdcch-Monitoring-r16, (X, Y) span) as the baseline to define the new capability

  - Y<=X

  - PDCCH could be configured such that the developed span pattern by SS configuration satisfy (X,Y) requirement, i.e. the start of any two span of at most Y symbols/slots is separated by at least X symbols/slots

  - BD/CCE budget is counted for each span of at most Y symbols/slots

  - FFS: Values of X and Y and units in which they are defined

- Alt 3: A sliding window of X=Y slots for defining multi-slot PDCCH monitoring capability.

  - The slot groups are sliding in unit of [1] slot

  - PDCCH could be configured in any slot

  - BD/CCE budget is counted within any slot group

Update from Ericsson

- Alt 1: A fixed pattern of contiguous slot groups.

  - Each slot group consists of X slots

  - PDCCH monitoring can be configured in Y consecutive slots within each X slot group

  - BD/CCEs are counted toward the budget within the Y slots of each X slot group

    - Note: BD/CCEs are not counted across slot group boundaries

  - Alt 1-1: Y<X

    - FFS: Whether or not the Y slots are the first Y slots within each X slot group

  - Alt 1-2: Y=X

    - Note: Y is used to facilitate discussion. If Alt 1-2 is agreed, Y is not needed.

- Alt 2: Use (X,Y) similar to the Rel-16 capability (pdcch-Monitoring-r16, (X, Y) span) as the baseline to define the new capability

  - Y<=X

  - PDCCH monitoring can be configured such that the span pattern by search space configuration satisfies the (X,Y) requirement, i.e. X is the minimum time separation between the the start of two consecutive spans, including across slot groups

  - BD/CCEs  are counted toward the budget for each span of at most Y symbols/slots

  - FFS: Values of X and Y and units in which they are defined

- Alt 3: Same as Alt-1-2 (Y=X), except

  - BD/CCEs are counted toward the budget within an X slot sliding window that can cross a slot-group boundary

  - The window slides in unit of [1] slot

  - PDCCH monitoring can be configured in any slot within a slot group

Comments:

- For Alt-1, I modified the first two lines to make it more clear that the pattern is not X slots. Rather, the pattern consists of contiguous slot groups where each slot group consists of X slots

- For Alt-2, aligned the wording to be close to what is in he current 38.213 Section 10, However, what was missing from the Alt-2 description is the implicit sliding window. So I added "including across slot groups" analogous to current 38.213

A UE can indicate a capability to monitor PDCCH according to one or more of the combinations X,Y = (2, 2), (4, 3), and (7, 3) per SCS configuration of µ=0 and µ=1. A span is a number of consecutive symbols in a slot where the UE is configured to monitor PDCCH. Each PDCCH monitoring occasion is within one span. If a UE monitors PDCCH on a cell according to combination X,Y, the UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. A span starts at a first symbol where a PDCCH monitoring occasion starts and ends at a last symbol where a PDCCH monitoring occasion ends, where the number of symbols of the span is up to Y.

- Question to all: I'm not convinced that the following is accurate. What happens if there are two spans within a slot group that satisfy the (X,Y) requirement? Is it necessary to introduce a third variable N = number of slots in slot group?

"BD/CCEs  are counted toward the budget for each span of at most Y symbols/slots"

Update from vivo

- Alt 1: A fixed pattern of X slots.

  - The different X slot groups are consecutive and do not overlap

  - PDCCH could be configured in Y consecutive slots within each X slot group

  - BD/CCE budget is counted within the Y slots of each X slot group

  - Alt 1-1: Y<X

    - FFS: The Y slots are the first Y slots within the X slot group or not

  - Alt 1-2: Y=X

  - Note: Y is used to facilitate discussion. If Alt 1-2 is agreed, Y is not needed.

- Alt 2: Use (X,Y) similar to the Rel-16 capability (pdcch-Monitoring-r16, (X, Y) span) as the baseline to define the new capability

  - Y<=X

  - PDCCH could be configured such that the developed span pattern by SS configuration satisfy (X,Y) requirement, i.e. the start of any two consecutive span of at most Y symbols/slots is separated by at least X symbols/slots

  - BD/CCE budget is counted for each span of at most Y symbols/slots

  - FFS: Values of X and Y and units in which they are defined

  - FFS: Whether number of slots within which the number of monitoring occasions is counted the span pattern is repeated is needed and if needed, the value of the number of slots

- Alt 3: A sliding window of X=Y slots for defining multi-slot PDCCH monitoring capability.

  - The slot groups are sliding in unit of [1] slot

  - PDCCH could be configured in any slot

  - BD/CCE budget is counted within any slot group

Comments:

- For Alt-1, it seems that we already have common understanding on this. The wording refinement from Ericsson is also fine with us.

- For Alt-2, I think the original FFS is still needed but the wording should be adjusted to make it clearer. In single-slot monitoring capability defined in NR Rel-15/16, the multi-symbol span pattern is repeated every slot and there may be multiple spans within one slot. Similarly, to define multi-slot monitoring capability, the multi-symbol/slot span pattern should be repeated in multiple (e.g. M, M>X>=Y) slots.  One example could be that the span pattern is repeated in every subframe. Then N is actually the number of slots within which the span pattern is repeated, which is updated as above.

- For Alt-3, it is similar to Alt 1-2 except the BD/CCE counting.

In all the above alternatives, the above mentioned Y slots doesn’t mean all symbols in the slot are monitored. Which symbol needs to be monitored will be further discussed.