3GPP TSG RAN WG1 #104b-e		  R1-2102522

e-Meeting, April 12th – 20th, 2021

Source:	vivo

Title:	CSI feedback enhancements for Rel-17 URLLC

Agenda Item:	8.3.1.2

Document for:	Discussion and Decision

- Introduction

In RAN1#104 e-meeting, CSI enhancements are discussed. Only conclusion are achieved as following:

Conclusion: Continue evaluation of new reporting Case 1 and Case 2 for the schemes identified in Appendix B of R1-2102131.

- Companies are encouraged to provide their views on each scheme against each criterion in respective Tables in Appendix B.

- Companies are encouraged to provide additional evaluation results for as many schemes as possible, based on assumptions agreed in RAN1#102-e.

- Aim for down-selection at RAN1#104-b-e by taking into account evaluation results and assessment against criteria from Appendix B.

In RAN1#103 e-meeting, some agreements are listed as following:

Agreements

- No change of CSI processing time relative to Rel-16 CSI in this WI

- CSI processing time specific to a new CSI reporting quantity/type (if supported) can be studied

Agreement:

- For Case-2 new reporting, continue studying with focus on the new reporting type based on PDSCH decoding for OLLA performance enhancement for initial and re-transmissions of PDSCH.

Agreements:

For Case-1 New reporting, the following candidate schemes have been identified to address the fast interference change over time. Continue studying with focus on the identified schemes below for further study and evaluation.

- Scheme 1a: New reporting quantity based on CQI/SINR statistics, e.g.,

  - CQI/SINR statistics (e.g., mean, variance, etc.)

  - CSI prediction

- Scheme 1b: New reporting quantity of interference statistics (e.g., mean, variance, interference covariance matrix, etc.)

- Scheme 1c: New reporting quantity based on modifying existing reporting format, e.g.,

  - CQI reporting considering the worst subbands

  - Subband CQI granularity enhancement

- Scheme 1d: New reporting quantity related to CSI expiration time

- Scheme 1e: New reporting quantity with partial information update, e.g., CSI reporting with interference update only

In this contribution, we share our views on CSI feedback enhancements for URLLC.

- CSI feedback enhancements

In Rel-16, NR supports different types of CSI reports, including periodic CSI (P-CSI), semi-persistent CSI (SP-CSI) and aperiodic CSI (A-CSI) reports. A-CSI report is triggered by the CSI request field on a DCI that triggers a CSI report(s) on PUSCH. Either wideband or sub-band frequency granularities reporting are supported for A-CSI report on PUSCH. A-CSI on PUSCH supports Type I, Type II and Enhanced Type II CSI. By triggering one or multiple CSI report states, gNB can acquire the up-to-date channel state and interference state.

  - New CSI report triggering for A-CSI

In Rel-16, A-CSI is only triggered on PUSCH by UL grant. In case of DL traffic heavy case, gNB needs to transmit a DCI carrying UL grant to trigger an A-CSI on PUSCH, which may result in unnecessary DL control overhead. To facilitate efficient PDSCH (re)transmission and/or save control overhead for DL traffic heavy use case, A-CSI triggered on PUCCH is proposed.

Some triggering methods were proposed in RAN1 #104-e including:

- Option 1: CSI triggering on PUCCH by DL assignment

- Option 2: CSI-RS/SRS/A-CSI report triggering by NACK

- Option 3: CSI triggering on PUCCH by group DCI

Although there are differences for the triggering for these options, the key question is how to determine the resource for the triggered A-CSI.

One straightforward method is to use the PUCCH resource carrying the HARQ-ACK. Note that the PUCCH resource for HARQ-ACK is indicated by the DL grant. In this regard, A-CSI on PUCCH can be triggered by the DL grant, i.e. Option 1.

An alternative is to use separate resource from the PUCCH for HARQ-ACK, e.g., a separate PUCCH resource indicated/configured by gNB for the A-CSI, or a PUSCH resource similar to configured grant PUSCH for carrying A-CSI. The separate PUCCH or PUSCH resources can be indicated by the DL grant.

Therefore, for the resource for A-CSI, following alternatives can be considered.

- Alt.1: A-CSI and HARQ-ACK are multiplexed on the same PUCCH resource

- Alt.2: A-CSI is transmitted on a separated PUCCH resource from PUCCH for HARQ-ACK

- Alt.3: A-CSI is transmitted on a separated PUSCH resource

Proposal 1: For new CSI report triggering for A-CSI, A-CSI triggered by DL grant is preferred.

For Alt.1, the existing UCI multiplexing rules can be reused. The PUCCH resource set needs to be determined by the payload of HARQ-ACK and CSI. It should be clarified whether the PUCCH resource set configuration for HARQ-ACK is reused for multiplexing CSI and HARQ-ACK. Besides, the timeline for PDSCH reception and CSI computation should be met simultaneously. In current spec, the CSI computation time is defined for different cases as provided in Table 1 and Table 2. (Z1,Z1') of the Table 1 is defined for the case where there is only CSI report without a PUSCH with either transport block or HARQ-ACK or both and L = 0 CPUs are occupied. In Table 2, (Z1,Z1'), (Z2,Z2') are defined for the cases other than that for Table 1.

For URLLC service, PDSCH and PUSCH capability 2 processing time as in Table 3 and Table 4 are adopted to guarantee low latency. Compared to the PDSCH and PUSCH processing time capability 2, it can be observed the CSI computation time is much larger that PDSCH and PUSCH capability 2 processing time. In case of URLLC burst traffic, gNB can only rely on out-of-date CSI to schedule URLLC transmission due to the fact that a CSI report would need longer processing time than the preparation for the URLLC transmission. In this case, inaccurate CSI may lead to higher decoding failure probability at UE side. Therefore, the current CSI computation time is not beneficial for gNB obtaining CSI timely. On the other hand, if the conservative MCS is always selected due that gNB has no timely CSI, resource utilization efficiency would be decreased. Above all, the current CSI computation time is unfavorable for acquisition of up-to-date CSI information for URLLC service with low latency. Therefore, CSI computation time reduction is needed for URLLC in order for timely CSI acquisition, thereby enabling accurate MCS selection at gNB side.

Observation 1: CSI computation time defined in Rel-16 is much larger than PDSCH and PUSCH capability 2 processing time and is unfavorable for acquisition of accurate up-to-date CSI information.

Table 1 CSI computation delay requirement 1

|  | Z1 [symbols] |  |
| --- | --- | --- |
|  | Z1 | Z'1 |
| 0 | 10 | 8 |
| 1 | 13 | 11 |
| 2 | 25 | 21 |
| 3 | 43 | 36 |

Table 2: CSI computation delay requirement 2

|  | Z1 [symbols] |  | Z2 [symbols] |  |
| --- | --- | --- | --- | --- |
|  | Z1 | Z'1 | Z2 | Z'2 |
| 0 | 22 | 16 | 40 | 37 |
| 1 | 33 | 30 | 72 | 69 |
| 2 | 44 | 42 | 141 | 140 |
| 3 | 97 | 85 | 152 | 140 |

Table 3 PDSCH processing time

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | PDSCH processing capability 2 | PDSCH processing capability 1 |
| 0 | 3 | 8 |
| 1 | 4.5 | 10 |
| 2 | 9 for frequency range 1 | 17 |
| 3 | - | 20 |

Table 4 PUSCH processing time

|  | PUSCH preparation time N2 [symbols] |  |
| --- | --- | --- |
|  | PUSCH processing capability 2 | PUSCH processing capability 1 |
| 0 | 5 | 10 |
| 1 | 5.5 | 12 |
| 2 | 11 for frequency range 1 | 23 |
| 3 | - | 36 |

For an initial PDSCH transmission, gNB may not have the accurate CSI information. Thus, an inappropriate MCS is selected at gNB and it is mostly likely to result in decoding failure for the initial transmission at UE. In such case, NACK would be reported by UE for the PDSCH transmission. However, gNB has no opportunity to obtain a timely CSI by triggering an A-CSI on PUSCH before scheduling a retransmission, due that CSI computation time is longer than the PDSCH processing timeline where there is no enough time for UE to report CSI before HARQ-ACK feedback, as shown in the examples in Fig. 1. In order to obtain a timely CSI for more reliable retransmission scheduling, A-CSI on PUCCH can be triggered by gNB. For example, A-CSI can be reported with HARQ-ACK of initial PDSCH transmission, which can provide the channel state information for gNB for retransmission. To facilitate the A-CSI triggered on a PUCCH, CSI computation time needs to be reduced to align with PDSCH processing time N1.

Figure 1 CSI and HARQ-ACK processing time for 15kHz

Proposal 2: For A-CSI triggered by DL grant where CSI and HARQ-ACK are multiplexed on the same PUCCH resource, CSI computation time needs to be reduced, e.g align with PDSCH processing time N1.

For Alt.2, separate PUCCH resource configurations are adopted between CSI and HARQ-ACK. It should be clarified which of the parameters of PUCCH resource configurations need to be configured for the PUCCH for CSI. In particular, it should be further discussed on the number of PUCCH resource sets for CSI report, whether/how to indicate the PUCCH resource for A-CSI, whether/how to determine the power for CSI report. In such case, the existing timeline for CSI computation can be reused due to the separate resource for CSI. However, there would be non-trivial specification impacts.

An alternative way to transmit the A-CSI is to trigger a A-CSI on a PUSCH by the DL grant, i.e. Alt.3. The PUSCH can be configured by RRC, e.g. similar to the Type 1 configured grant resource. In such case, the configuration of Type 1 CG can be reused.

Proposal 3: For A-CSI triggered by DL grant where CSI and HARQ-ACK are transmitted on separate resource, A-CSI transmitted on a separate PUCCH resource or PUSCH resource can be considered.

  - New CSI report for Case 1 and Case 2 reporting

In the past meetings, Case 1 and Case 2 new CSI reporting were discussed intensively. Different variants are proposed for Case 1 and Case 2 CSI reporting.

For Case 1 CSI reporting, CQI/SINR based on statistics (e.g. average, variance) or partial updated measurements are adopted. In this case, the existing measurement/report procedures for generating CSI report, including CSI measurement, CSI computation and multiplexing CSI on the UL channel may be reused.

In fact, for the typical URLLC use case, UE mobility is low and channel state including the spatial characteristics does not change frequently. Therefore, it is not necessary to update the RI or PMI frequently. For CSI report instance, CQI update only can be applied and the latest RI/PMI based on the previous CSI measurement for RI/PMI can be assumed, as shown in Fig.2(a). CSI processing time can be reduced by simplifying the CSI measurement and computation processing.

Furthermore, there could be a case when channel part for CSI measurement varies slowly and sporadic interference becomes the main factor for the channel state change, due to the random inter-cell interference variation. From this perspective, frequent measurement and reporting for channel part are not required. UE can perform interference measurement only to obtain CQI while channel part and RI/PMI base on the latest report. This method is beneficial to capture the dynamic interference variance timely.

As shown in Figure 2 (b), DCI only triggers interference measurement RS, which is related to the latest relative CSI report configuration including both channel part measurement and interference measurement. CQI computation based on interference measurement RS and the latest channel measurement results assuming the same RI/PMI.

Figure 2 (a) CQI based on both channel part and interference measurement, RI/PMI based on the latest report

Figure 2 (b) CQI acquisition based on interference measurement only

Proposal 4: Support Case 1 New reporting quantity with CQI update only, where the latest RI/PMI based on the previous CSI measurement for RI/PMI can be assumed.

For Case 2 new reporting, it should be pointed out that the new reporting type is based on PDSCH decoding for OLLA performance enhancement. The Case 2 new reporting would help improve the accuracy of MCS selection for the next transmission, including retransmission and new transmission. Although there might be benefit for Case 2 new reporting, the disadvantage of Case 2 new reporting should not be ignored.

For case 2 new reporting, the reported information is only based on the decoding on the scheduled PRBs in the previous transmission and the same precoder needs to be assumed. It hard to use the reported information to adjust the scheduling and MCS selection for the next transmission (new transmission or retransmission), if different time/frequency resources or TBS would be used.

In fact, if gNB schedules a PDSCH for a UE on a given set of PRBs and UE fails to decode the PDSCH, it would be safer for gNB to schedule the retransmission for this UE on a different set of PRBs, since UE may occur strong interference or large fading in the PRBs for initial transmission. In this sense, reporting the channel/interference information only based on the initially scheduled resources would be not useful due to lack of any information for the different sub-band used for re-transmission.

When the case 2 new reporting is applied for a new initial transmission, the reported channel/interference information may be expired and not applicable for a new transmission if the new transmission is scheduled with a period of time after the last transmission. Especially for the burst traffic for URLLC, the reporting information based on last scheduled PDSCH decoding is not sufficient.

Observation 2: For the burst traffic for URLLC, Case 2 new reporting based on the reported information for the previously scheduled PRBs is not sufficient to adjust the scheduling resources and MCS selection for the next transmission (new transmission or retransmission) for gNB.

- Performance evaluation

  - Simulation results

In this section, the performances with existing CSI report mechanism and the above-mentioned new CSI report scheme are evaluated. In the simulation, the following cases are assumed.

- Scheme 1: Rel-15 CSI feedback mechanism with 10ms and 40ms feedback periodicities, respectively. For each CSI report instance, CQI with channel and interference measurement and RI/PMI are both updated simultaneously, as shown in Figure 3 (a).

- Scheme 2-0: for each CSI report instance with 40ms periodicity, RI, PMI and CQI are updated based on channel part measurement and interference measurement. Within 40ms, three CSI report instances with 10ms periodicity are configured. For CSI report with 10ms periodicity, only interference measurement is performed and channel part is not measured. CQI only report is applied and the latest channel part, RI and PMI are reused, as shown in Figure 3(b).

- Scheme 2-1: this scheme is similar to scheme 2-0, for CSI report with 10ms periodicity, both channel part measurement and interference measurement are performed. CQI only is reported and the latest RI, PMI are reused, as shown in Figure 3(c).

Figure 3(a) CSI report scheme 1with 40ms periodicty

Figure 3(b) CSI report scheme 2-0

Figure 3(c) CSI report scheme 2-1

Table 5 UE capability and resource utilization

| Schemes | Percentage of UEs satisfying BLER reliability requirement | Resource utilization |
| --- | --- | --- |
| Scheme 1(10ms) | 98.25% | 48.19% |
| Scheme 1(40ms) | 67.22% | 77.45% |
| Scheme 2-0 | 70.87% | 56.34% |
| Scheme 2-1 | 89.13% | 52.43% |

According to the simulation results listed in table 5, Scheme 1 has the best performance because the more accurate channel state acquisition and the shorter CSI report periodicity are beneficial for more accurate MCS selection at gNB side. However, the overhead and power consumption for CSI report for scheme 1 are larger than that for the other schemes. On the other hand, Scheme 2-1 can reduce the CSI computation and overhead compared to Scheme 1 with short periodicity, while the performance loss of Scheme 2-1 is limited. Compared to Scheme 2-0, Scheme 2-1 achieves better performance. It can be observed that both the instantaneous channel and interference variation influence the CQI accuracy. Scheme 1 with 40 ms report periodicity has the worst performance due to inaccurate CSI.

Observation 3: Scheme 1 of CSI report with full CQI/RI/PMI report and short periodicity can achieve the best performance among all schemes, while the overhead and power consumption for CSI report for scheme 1 are larger than that for the other schemes.

Observation 4: Compared to Scheme 1 with short periodicity, Scheme 2-1 can reduce the CSI computation and overhead while the performance loss of Scheme 2-1 is limited.

Observation 5: Scheme 2-1 achieves better performance compared to Scheme 2-0 because both the instantaneous channel and interference variation influence the CQI accuracy.

  - Observations for other CSI report schemes

In the last meeting, total 17 schemes are summarized. Some simulation results for case 1 and case 2 new reporting were provided.

Based on the discussions and simulation results from companies, we have the following observations.

- SINR/CQI/CSI statistics

Firstly, we think the mean and variance CQI can be derived by gNB if there are sufficient CQI reports, which may require UE frequently measures channel/interference and reports CSI. In this sense, the main benefits for reporting mean/variance CQI are reduction of CSI reporting overhead. We think the motivation of introducing mean/variance CQI should be justified.

Secondly, how to make use of the mean/variance CQI reported from UE by gNB for scheduling should be further clarified. Besides, how to use the mean-CQI and variance-CQI is up to implementation. One  may use predicted CQI = mean-CQI – X* variance-CQI, while other may use predicted CQI = mean-CQI -Y * variance-CQI, where X or Y is scale factor and X ≠ Y. How to test the performance of mean/variance CQI reporting is another issue.

Thirdly, how to obtain the mean/variance CQI in case of sub-band CQI reporting should be clarified. If sub-band CQI is considered for obtaining mean-CQI and variance-CQI, the variance-CQI may be enlarged due to the potential frequency selection. Limited advantage could be achieved compared to sub-band CQI report, because gNB can schedule resource excluding PRB with the lower CQI.

Furthermore, it is necessary to obtain mean-CQI and variance-CQI from sufficient samples of CQI/SINR. It seems only with the sufficient CQI samples, the corresponding statistics distribution character can be assumed. If there is no sufficient number of samples, the expected results can not be obtained.

Observation 6: For Case 1 CSI reporting based on SINR/CQI/CSI statistics,

  - main benefits for reporting mean/variance CQI are reduction of CSI reporting overhead.

  - it should be further clarified how to obtain and make use of the mean/variance CQI.

  - it is necessary to ensure sufficient samples of CQI/SINR to obtain the SINR/CQI/CSI statistics.

Contribution [2] shows the performances of sub-band 4-bit CQI and SINR statistics schemes. According to Fig. 5, it can be observed that both sub-band 4-bit CQI and SINR statistics can achieve 100 percent UEs satisfying requirement. SINR statistics method has the lower average PRB utilization compared to Sub-band 4-bit CQI method.

According to Fig. 6-A in [3], it can be observed that both sub-band 4-bit CQI and SINR statistics methods have the similar block error rate.

According to [4], CQI statistics methods has no performance gain compared with baseline case, i.e., P-CSI feedback with Wideband CSI report and SP-CSI feedback with Subband CSI report.

In [5], it can be observed that the scheme of statistical CSI scheme provides some gains. But, the performance includes PER range from 10^-4 to 10^-1. In our opinion, 10^-5 target PER is not evaluated. And the higher PER is not typical target PER for URLLC service e.g. 10^-3~10^-1. Around 10^-4 PER, full sub-band CQI reporting with 4bits has the similar performance with statistics CSI method.

Observation 7: From the existing simulation results, SINR/CQI/CSI statistics scheme does not show obvious performance gain compared to full sub-band 4-bit CQI report.

- Worst-M CQI

Contribution [2] shows the performances of sub-band 4-bit CQI and worst M CQI methods. According to Fig. 5, it can be observed that both sub-band 4-bit CQI and worst-M CQI methods can achieve 100 percent UEs satisfying requirement. Worst-M CQI has the lower average PRB utilization compared to sub-band 4-bit CQI methods.

In [3], worst-M CQI is slightly better than sub-band 4-bit CQI scheme. In simulation, gNB randomly scheduling for worst-M CQI scheme while gNB always use best sub-bands for sub-band 4-bit CQI scheme. In our understanding, the gain may be from randomly scheduling countering random character of interference. Actually, gNB can also schedule the resources randomly based on implementation for sub-band 4-bit CQI scheme.

Observation 8: From the existing simulation results, worst-M CQI report does not show obvious gain compared to full sub-band 4-bit CQI report.

- PDSCH-based CSI feedback, including soft ACK (Decoding margin), (Delta) CQI/MCS/SINR and Block error probability

There is no much difference among soft ACK (Decoding margin), (Delta) CQI/MCS/SINR, Block error probability schemes. These schemes are reporting some information in different forms according to the PDSCH decoding processing and results. One question is that the reported information is only based on the decoding on the scheduled PRBs in the previous transmission and the same precoder needs to be assumed. It may not be helpful to make use of the reported information to adjust the scheduling and MCS selection for the next transmission (new transmission or retransmission), if different time/frequency resources and length would be used.

In contribution [6], the simulation of Full CQI vs. soft ACK schemes are performed. For Full CQI method, Subband PMI and subband CQI are reported. It can be observed in the “Factory automation” scenario, all schemes allow for 100% packet success rate. On the other hand, for the “R15 enabled use case”, all schemes provide the similar improvement in terms of packet failure, e.g. 99.985% vs. 100% packet success rate for Full CQI scheme and slow soft ACK scheme, respectively. No obvious performance gain is observed for Full CQI vs. soft ACK schemes.

Observation 9: From the existing simulation results, soft ACK schemes does not show obvious gain compared to full sub-band CQI report.

For (Delta) CQI/MCS/SINR schemes, in contribution [7] and [8], the performances of conventional scheme and new scheme, i.e. PDSCH-based CSI feedback are evaluated. In conventional scheme, SRS with every 5-slot periodicity is used for SNR estimation.

According to simulation results, for 20UEs, 60UEs and 100UEs cases, 100 percent UE can satisfy BLER reliability requirement for both methods. The only benefit seems that the new scheme can save percentage of resources compared to conventional scheme.

Observation 10: From the existing simulation results for PDSCH-based CSI feedback, no performance gains are observed compared to the conventional scheme, i.e. periodic SRS.

For Block error probability based scheme, according to [9], performance gain is not obvious compared to other schemes e.g. statistic CQI, delta SINR, slow soft-ACK. In some cases even is worse than slow Soft-ACK method.

Observation 11: For Block error probability based scheme, no performance gains are observed compared to other schemes e.g. stat.CQI, deltaSINR, slow soft-ACK.

Above all, according to evaluation results from companies, the new reporting types 1 and 2, e.g. SINR/CQI/CSI statistics methods, Worst-M CQI, PDSCH-based CSI feedback schemes group can only provide sub-optimal performance compared to the full sub-band CSI report with 4 bits with short reporting periodicity. Hence, no remarkable gain is observed for these new CSI report schemes. Full sub-band CSI report with 4 bits extends Rel-16 method with improving granularity. This scheme is mature and easy to be tested and evaluated. Meanwhile, low specification impact is observed.

The disadvantage of full sub-band CQI method with 4 bits is mainly signaling overhead of CSI report due to dense report instances and fine report granularity. To reduce the overhead and power consumption for CSI report, new reporting types can be considered.

However, for URLLC service, signaling overhead is not the essential issue because the latency and reliability requirements should be met. It should be clarified firstly whether Rel-17 CSI enhancement is targeted for overhead reduction. If overhead reduction is needed for URLLC CSI enhancement, in particular, how to reduce the CSI computation complexity and the overhead for CSI report based on the existing CSI measurement/reporting mechanisms should be prioritized.

Observation 12: The new reporting types e.g. CQI/SINR statistics Worst-M CQI, PDSCH-based CSI feedback schemes can only provide sub-optimal performance compared to the full sub-band CSI report with 4bits with short reporting periodicity.

Proposal 5: Full sub-band 4-bit CQI reporting is preferred for Rel-17 CSI enhancement.

Proposal 6: If overhead reduction is needed for URLLC CSI enhancement, how to reduce the CSI computation complexity and the overhead for CSI report based on the existing CSI measurement/reporting mechanisms should be prioritized.

- Conclusion

In the contribution, we have some investigations on CSI feedback enhancement for URLLC/IIOT, and propose that,

Observation 1: CSI computation time defined in Rel-16 is much larger than PDSCH and PUSCH capability 2 processing time and is unfavorable for acquisition of accurate up-to-date CSI information.

Observation 2: For the burst traffic for URLLC, Case 2 new reporting based on the reported information for the previously scheduled PRBs is not sufficient to adjust the scheduling resources and MCS selection for the next transmission (new transmission or retransmission) for gNB.

Observation 3: Scheme 1 of CSI report with full CQI/RI/PMI report and short periodicity can achieve the best performance among all schemes, while the overhead and power consumption for CSI report for scheme 1 are larger than that for the other schemes.

Observation 4: Compared to Scheme 1 with short periodicity, Scheme 2-1 can reduce the CSI computation and overhead while the performance loss of Scheme 2-1 is limited.

Observation 5: Scheme 2-1 achieves better performance compared to Scheme 2-0 because both the instantaneous channel and interference variation influence the CQI accuracy.

Observation 6: For Case 1 CSI reporting based on SINR/CQI/CSI statistics,

  - main benefits for reporting mean/variance CQI are reduction of CSI reporting overhead.

  - it should be further clarified how to obtain and make use of the mean/variance CQI.

  - it is necessary to ensure sufficient samples of CQI/SINR to obtain the SINR/CQI/CSI statistics.

Observation 7: From the existing simulation results, SINR/CQI/CSI statistics scheme does not show obvious performance gain compared to full sub-band 4-bit CQI report.

Observation 8: From the existing simulation results, worst-M CQI report does not show obvious gain compared to full sub-band 4-bit CQI report.

Observation 9: From the existing simulation results, soft ACK schemes does not show obvious gain compared to full sub-band CQI report.

Observation 10: From the existing simulation results for PDSCH-based CSI feedback, no performance gains are observed compared to the conventional scheme, i.e. periodic SRS.

Observation 11: For Block error probability based scheme, no performance gains are observed compared to other schemes e.g. stat.CQI, deltaSINR, slow soft-ACK.

Observation 12: The new reporting types e.g. CQI/SINR statistics Worst-M CQI, PDSCH-based CSI feedback schemes can only provide sub-optimal performance compared to the full sub-band CSI report with 4bits with short reporting periodicity.

Proposal 1: For new CSI report triggering for A-CSI, A-CSI triggered by DL grant is preferred.

Proposal 2: For A-CSI triggered by DL grant where CSI and HARQ-ACK are multiplexed on the same PUCCH resource, CSI computation time needs to be reduced, e.g align with PDSCH processing time N1.

Proposal 3: For A-CSI triggered by DL grant where CSI and HARQ-ACK are transmitted on separate resource, A-CSI transmitted on a separate PUCCH resource or PUSCH resource can be considered.

Proposal 4: Support Case 1 New reporting quantity with CQI update only, where the latest RI/PMI based on the previous CSI measurement for RI/PMI can be assumed.

Proposal 5: Full sub-band 4-bit CQI reporting is preferred for Rel-17 CSI enhancement.

Proposal 6: If overhead reduction is needed for URLLC CSI enhancement, how to reduce the CSI computation complexity and the overhead for CSI report based on the existing CSI measurement/reporting mechanisms should be prioritized.

- References

[1] Chairman notes RAN1#104 e-meeting

[2] R1-2008862	CSI feedback enhancements for URLLC/IIoT use cases. Nokia, Nokia Shanghai Bell, RAN1#103e meeting

[3]R1-2100835 CSI feedback enhancements for URLLC/IIoT use cases, Nokia, Nokia Shanghai Bell, RAN1#104e

[4] R1-2100102 Discussion on CSI feedback enhancements for eURLLC ZTE, RAN1#104e

[5] R1-2100650 CSI feedback enhancements for URLLC/IIoT Intel Corporation, RAN1#104e

[6] R1-2008936	CSI feedback enhancements for enhanced URLLC/IIoT. InterDigital, Inc. RAN1#103e meeting

[7] R1-2009258	CSI enhancement for IOT and URLLC. Qualcomm Incorporated, RAN1#103e meeting

[8] R1-2101460	CSI enhancement for IOT and URLLC Qualcomm Incorporated, RAN1#104e meeting

[9] R1-2100830 CSI feedback enhancements InterDigital, Inc. RAN1#104e meeting

- Annex

  - Simulation assumption for Rel-15 enabled use case (e.g. AR/VR) use case in TR 38.824

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid, 21 cells |
| Inter-BS distance | 500m |
| Carrier frequency | 4 GHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 2) dH = 0.5λ, dV = 0.8λ |
| antenna tilt | 102o |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | Panel model 1: Mg=1, Ng=1, P=2, dH=0.5 (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2); |
| UE antenna height | Follow the modelling of TR 38.901. |
| UE antenna gain | 0dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| BS receiver | MMSE-IRC |
| Number of UEs per cell | URLLC UEs:20 eMBB UEs: 5 |
| Simulation bandwidth | 40 MHz |
| SCS | 30 kHz |
| UE distribution | 80% of users are outdoors and 20% of users are indoors Use 3 km/h for modelling fading channel |
| penetration loss | low loss model |
| Channel estimation | Realistic |
| Traffic mode (URLLC UE) | FTP model 3 (100p/s) Latency: 4ms (1.5Kbytes) |
| Traffic mode (eMBB UE) | FTP model 3 Packet size: 0.1Mbytes mean inter-arrival time: 200ms |
| Reliability | 99.999 |
| Max Rank | 1 |