3GPP TSG RAN WG1 #104b-e																																																			R1-2102905

e-Meeting, April 12th – 20th, 2021

Source:	CMCC

Title:	Discussion on scenarios applicable to NB-IoT and eMTC

Agenda Item:	8.15.1

Document for:	Discussion and Decision

Introduction

In RAN1#104-e, agreements on the parameter assumptions for the purpose of link budget calibration were achieved for NB-IoT/eMTC application over NTN [1]:

Agreement: The following assumptions are agreed for a common set of link budget parameters: - UE power class (PC5=20 dBm) - UE Noise Figure (NF=9 dB) - Channel Bandwidth for NB-IoT and eMTC as was included in IoT NTN reference scenario parameters agreed in RAN1#103e - NB-IoT 180 kHz (DL), Up to 180 kHz with all permissible smaller resource allocations 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz - eMTC: 1080 kHz (DL), Up to 1080 kHz with all permissible smaller resource allocations, including 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz (UL) - Other losses Other Losses; GEO (35786 km); LEO (1200 km); LEO (600 km). Scintillation losses; 2.2; 2.2; 2.2. Atmospheric losses; 0.2; 0.1; 0.1. Polarization loss; 3; 3; 3. Shadow margin; 3; 3; 3. NOTE 1: With PC3 (23 dBm) there is a 3dB gain compared to the PC5 (20 dBm) assumption on UL. NOTE 2: With NF=7 dB, there is a 2 dB improvement compare to NF=9 dB on DL. NOTE 3: Link budgets with other link budget parameters are not excluded from being captured in the TR. NOTE 4: These parameters are only for the purpose of link budget calculations. NOTE 5: Atmospheric losses are a function of elevation angle. Agreement: Link budget analysis assumes 3 dB polarization loss for DL and 3 dB polarization loss on UL for satellite parameters Set 1, Set 2, Set 3, and Set 4 Agreement: Include in TR 36.763, the 3 dB beam width (HPBW), central beam center elevation and central beam edge elevation in the satellite parameter set(s) to be used in link budget calculations – (Corresponding satellite parameter Set 3 and Set 4 are given in Section 9.4) SET 3; GEO 35786 km; LEO-600 km; LEO-1200 km. 3 dB Beam width (HPBW); 0.735 degree; 22.0631 degree; 22.0631 degree. Central beam center elevation; 20.88 degree; 43.78 degree; 46.05 degree. Central beam edge elevation; 12.5 degree; 30 degree; 30 degree. Central beam edge satellite-UE distance; 40316 km; 1074 km; 1998 km. SET 4; LEO-600 km. 3 dB Beam width (HPBW); 104.7 degree. Central beam center elevation; 90 degree. Central beam edge elevation; 30 degree. Central beam edge satellite-UE distance; 1076 km. NOTE 1: The 3 dB beam width (HPBW) is already included in satellite parameter set 1 and Set 2 in TR 38.821 Table 6.1.1.1-1 and Table 6.1.1.1-2 respectively. The central beam center elevation for Set-1 and Set-2 is defined as the target elevation angle that is included in in TR 38.821 Table 6.1.3.2-1. The central beam edge satellite-UE distance can be derived from the central beam edge elevation and does not need to be included. NOTE 2: Central beam center elevation is the beam center elevation of the central beam in the beam layout. NOTE 3: Central beam edge elevation is the minimum beam edge elevation of the central beam in the beam layout. NOTE 4 In SLS evaluation with a multiple beam layout, the central beam is the serving beam for UEs. The outer beams have beam center elevation that is different from the central beam center elevation. For the interference modelling, the interference due to the outer beams is determined by using their respective beam center elevations. NOTE 5: For the multiple-beam satellite cell, the longest beam edge distance will correspond to the minimum beam edge elevation of the most outer beam as illustrated in figure below. Agreement: Include the following tables in TR 36.763: - Set 1 satellite parameters (based on TR 38.821, Table 6.1.1.1-1) - Set 2 satellite parameters (based on TR 38.821, Table 6.1.1.1-2) - Set 3 satellite parameters (Eutelsat R1-2101146 with central beam edge elevation 12.5 degree for GEO, and 30 degree for LEO-600 km and 1200 km) Satellite orbit; GEO; LEO-1200; LEO-600. Satellite altitude; 35786 km; 1200 km; 600 km. Central beam edge elevation; 12.5 degree; 30 degree; 30 degree. Central beam center elevation; 20.9 degree; 46.05 degree; 43.8 degree. Payload characteristics for DL transmissions. Equivalent satellite antenna aperture (NOTE 1); S-band (i.e. 2 GHz); 12 m; 0.4m; 0.4 m. Satellite EIRP density; 59.8 dBW/MHz; 33.7 dBW/MHz; 28.3 dBW/MHz. Satellite Tx max Gain; 45.7 dBi; 16.2 dBi; 16.2 dBi. 3dB beam width (HPBW); 0.7353 degree; 22.1 degree; 22.1 degree. Satellite beam diameter (NOTE 2); 459km; 470 km; 234 km. Payload characteristics for UL transmissions. Equivalent satellite antenna aperture (NOTE 1); S-band (i.e. 2 GHz); 12 m; 0.4 m; 0.4 m. G/T; 16.7dB K-1; -12.8 dB K-1; -12.8 dB K-1. Satellite Rx max Gain; 45.7 dBi; 16.2 dBi; 16.2 dBi. NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of TR 38.811 NOTE 2: Satellite beam diameter is at Nadir point NOTE 3: Central beam center elevation is referred to as central beam elevation in TR 38.821 NOTE 4: Central beam edge elevation is the minimum beam edge elevation of the central beam in the beam layout. - Set 4 satellite parameters (Thales, Sateliot, Gatehouse R1-2101019) Satellite orbit; LEO-600. Satellite altitude; 600 km. Central beam edge elevation; 30 degree. Central beam center elevation; 90 degree. Payload characteristics for DL transmissions. Equivalent satellite antenna aperture (NOTE 1); S-band (i.e. 2 GHz); 0.097 m. Satellite EIRP density; 21.45 dBW/MHz. Satellite Tx max Gain; 11 dBi. 3dB beam width (HPBW); 104.7 degree. Satellite beam diameter (Note 2); 1700 km. Payload characteristics for UL transmissions. Equivalent satellite antenna aperture (Note1); S-band (i.e. 2 GHz); 0.097 m. G/T; - 18.6 dB·K-1. Satellite Rx max Gain; 11 dBi. NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of TR 38.811 NOTE 2: Satellite beam diameter is at Nadir point NOTE 3: Central beam center elevation is referred to as central beam elevation in TR 38.821 NOTE 4: Central beam edge elevation is the minimum beam edge elevation of the central beam in the beam layout.

In this contribution, we will update the link budget results based on the latest parameters for link budget calibration, and then discuss on additional impact factors for link budget analysis.

Discussion

Link budget analysis for calibration

The parameters for link budget calibration are summarized in Table 1~4 [2] [3].

Table 1: Satellite parameters.

|  | Set-1 |  |  | Set 2 |  |  | Set 3 |  |  | Set 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite orbit | GEO | LEO-1200 | LEO-600 | GEO | LEO-1200 | LEO-600 | GEO | LEO-1200 | LEO-600 | LEO-600 |
| Satellite altitude (km) | 35786 | 1200 | 600 | 35786 | 1200 | 600 | 35786 | 1200 | 600 | 600 |
| Central beam center elevation (deg) | 12.5 | 30 | 30 | 20 | 30 | 30 | 20.88 | 46.05 | 43.78 | 90 |
| Central beam edge elevation (deg) | 2.3 | 26.3 | 27 | 11 | 22.2 | 23.8 | 12.5 | 30 | 30 | 30 |
| Satellite beam diameter (km) | 250 | 90 | 50 | 450 | 190 | 90 | 459 | 470 | 234 | 104.7 |
| Payload characteristics for DL transmissions (S-band, 2 GHz) |  |  |  |  |  |  |  |  |  |  |
| Equivalent satellite antenna aperture (m) | 22 | 2 | 2 | 12 | 1 | 1 | 12 | 0.4 | 0.4 | 0.097 |
| Satellite EIRP density (dBW/MHz) | 59 | 40 | 34 | 53.5 | 34 | 28 | 59.8 | 33.7 | 28.3 | 21.45 |
| Satellite Tx max Gain (dBi) | 51 | 30 | 30 | 45.5 | 24 | 24 | 45.7 | 16.2 | 16.2 | 11 |
| 3dB beamwidth (HPBW) (deg) | 0.4011 | 4.4127 | 4.4127 | 0.7353 | 8.832 | 8.832 | 0.735 | 22.0631 | 22.0631 | 104.7 |
| Satellite beam diameter (km) | 250 | 90 | 50 | 450 | 190 | 90 | 459 | 470 | 234 | 1700 |
| Payload characteristics for UL transmissions (S-band, 2 GHz) |  |  |  |  |  |  |  |  |  |  |
| Equivalent satellite antenna aperture (m) | 22 | 2 | 2 | 12 | 1 | 1 | 12 | 0.4 | 0.4 | 0.097 |
| G/T （dB/K) | 19 | 1.1 | 1.1 | 14 | -4.9 | -4.9 | 16.7 | -12.8 | -12.8 | -18.6 |
| Satellite Rx max Gain (dBi) | 51 | 30 | 30 | 45.5 | 24 | 24 | 45.7 | 16.2 | 16.2 | 11 |

NOTE: Central beam center elevation is used for link budget calibration.

Table 2: UE characteristics for link budget calibration.

| Characteristics |  | C-IoT device |
| --- | --- | --- |
| Basic | Frequency band | S band (2 GHz) |
|  | Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
|  | Polarisation | Linear: +/-45°X-pol |
| DL | Channel Bandwidth | NB-IoT: 180 kHz eMTC: 1080 kHz |
|  | Rx Antenna gain | 0 dBi |
|  | Antenna temperature | 290 K |
|  | UE Noise figure | 9dB |
| UL | UE transmit power | UE power class 5: 20dBm |
|  | Tx antenna gain | 0 dBi |
|  | Channel Bandwidth | - NB-IoT: 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz - eMTC: 1080kHz, 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz |

Table 3: Free space path loss (FSPL).

|  | Set-1 |  |  | Set 2 |  |  | Set 3 |  |  | Set 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite orbit | GEO | LEO-1200 | LEO-600 | GEO | LEO-1200 | LEO-600 | GEO | LEO-1200 | LEO-600 | LEO-600 |
| Satellite altitude (km) | 35786 | 1200 | 600 | 35786 | 1200 | 600 | 35786 | 1200 | 600 | 600 |
| Central beam center elevation (deg) | 12.5 | 30 | 30 | 20 | 30 | 30 | 20.88 | 46.05 | 43.78 | 90 |
| FSPL (dB) | 190.6 | 164.5 | 159.1 | 190.4 | 164.5 | 159.1 | 190.4 | 162.3 | 156.9 | 154.0 |

Table 4: Other loss for link budget calibration.

|  | GEO 35786 km | LEO 1200 km | LEO 600 km |
| --- | --- | --- | --- |
| Scintillation losses (dB) | 2.2 | 2.2 | 2.2 |
| Atmospheric losses (dB) | 0.2 | 0.1 | 0.1 |
| Polarization loss (dB) | 3 | 3 | 3 |
| Shadow margin (dB) | 3 | 3 | 3 |

NOTE:	Atmospheric losses are a function of elevation angle.

A preliminary link budget is provided based on the above parameters for calibration with a selected candidates of UL channel bandwidth, with a summary in Table 5. The detailed calculation can be found in Appendix A1-A4.

- NB-IoT: 180kHz, 3.75kHz

- eMTC: 1080kHz, 30kHz

Table 5: Summary of preliminary link budget for calibration.

|  |  |  |  | Set-1 |  |  | Set 2 |  |  | Set 3 |  |  | Set 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Satellite orbit | GEO | LEO-1200 | LEO-600 | GEO | LEO-1200 | LEO-600 | GEO | LEO-1200 | LEO-600 | LEO-600 |
|  |  |  | Satellite altitude (km) | 35786 | 1200 | 600 | 35786 | 1200 | 600 | 35786 | 1200 | 600 | 600 |
|  |  |  | Central beam center elevation (deg) | 12.5 | 30 | 30 | 20 | 30 | 30 | 20.88 | 46.05 | 43.78 | 90 |
|  |  |  | FSPL (dB) | 190.6 | 164.5 | 159.1 | 190.4 | 164.5 | 159.1 | 190.4 | 162.3 | 156.9 | 154.0 |
|  | UL/DL | BW (kHz) |  |  |  |  |  |  |  |  |  |  |  |
| NB-IoT | DL | 180 | CNR (dB) | -5.0 | 2.2 | 1.6 | -10.3 | -3.8 | -4.4 | -4.0 | -2.0 | -1.9 | -5.9 |
|  | UL | 180 |  | -13.9 | -5.6 | -0.3 | -18.8 | -11.6 | -6.3 | -16.0 | -17.4 | -11.9 | -14.9 |
|  | UL | 3.75 |  | 2.9 | 11.2 | 16.6 | -2.0 | 5.2 | 10.6 | 0.8 | -0.6 | 4.9 | 1.9 |
| eMTC | DL | 1080 |  | -5.0 | 2.2 | 1.6 | -10.3 | -3.8 | -4.4 | -4.0 | -2.0 | -1.9 | -5.9 |
|  | UL | 1080 |  | -21.7 | -13.4 | -8.0 | -26.5 | -19.4 | -14.0 | -23.8 | -25.2 | -19.7 | -22.7 |
|  | UL | 30 |  | -6.2 | 2.1 | 7.5 | -11.0 | -3.9 | 1.5 | -8.3 | -9.6 | -4.1 | -7.1 |

Based on the above link budget results for the purpose of calibration, it can be seen that:

- For GEO with Set 2 satellite parameter, the UL CNR will reach -18.8dB level for NB-IoT with 180kHz BW, and reach -26.5dB level for eMTC with 1080kHz BW.

- For LEO at 1200km with Set 3 satellite parameter, the UL CNR will reach -17.4dB level for NB-IoT with 180kHz BW, and reach -25.2dB level for eMTC with 1080kHz BW.

- For LEO at 600km with Set 4 satellite parameter, the UL CNR will reach -14.9dB level for NB-IoT with 180kHz BW, and reach -22.7dB level for eMTC with 1080kHz BW.

Observation 1: Based on the latest parameters for link budget calibration, it can be observed that:

- For GEO with Set 2 satellite parameter, the UL CNR will reach -18.8dB level for NB-IoT with 180kHz BW, and reach -26.5dB level for eMTC with 1080kHz BW.

- For LEO at 1200km with Set 3 satellite parameter, the UL CNR will reach -17.4dB level for NB-IoT with 180kHz BW, and reach -25.2dB level for eMTC with 1080kHz BW.

- For LEO at 600km with Set 4 satellite parameter, the UL CNR will reach -14.9dB level for NB-IoT with 180kHz BW, and reach -22.7dB level for eMTC with 1080kHz BW.

Discussion on additional impact factors for link budget analysis

In our view, link budget analysis can be used to evaluate the basic coverage performance of IoT NTN. Then the following additional impact factors in real deployment scenario should be considered.

- Carriage and container penetration loss for logistics application.

- Vegetation loss for outdoor application.

- Additional FSPL for lower elevation angle (e.g., in the edge of the farthest beam).

Additional path losses

Regarding the additional losses, carriage and container penetration loss and vegetation loss are considered.

For logistics application, carriage and container penetration loss is non-negligible. O2I car penetration loss modeling in [4] may be reused to calculate the carriage and container penetration loss, i.e.,

PLcar=Nμ,σP2

μ=9, σP=5. Optionally, for metallized car windows, μ=20 can be used.

For outdoor application, a UE may be surrounded by the vegetation, e.g., woods. In this case, the vegetation loss is non-negligible. The vegetation loss (PLv) can be derived as [5]

PLv=l∙γ

where, l is the length of path within the tree canopy (m), and γ is specific attenuation for very short vegetative paths (dB/m).

Figure 1: Specific attenuation due to woodland.

Figure 1 shows typical values for specific attenuation derived from various measurements over the frequency range 30 MHz to about 30 GHz in woodland [5]. According to Figure 1, at 2 GHz, γ≈0.4 dB/m.

Figure 2 gives an example of alee-trees. In this case, the length of path within the tree canopy (m) l can be calculated as,

l=hsinα∙Wd

where, α is the elevation angle (rad), h is the height of tree canopy, W is the width of tree canopy, d is the interval distance between two trees. When the elevation angle is 10 deg, h=6.5m, W=3m, d=5m, l is about 22.5 m.

Then the vegetation loss (PLv) can be calculated as

PLv=l∙γ=0.4dB / m*22.5m=9dB

Figure 2: Example of alee-trees.

In summary, the following additional path loss needs to be considered in link budget analysis.

- Carriage and container penetration loss (9~20dB) for logistics application.

- Vegetation loss (e.g., 9dB) for outdoor application.

Observation 2: Additional path loss can be observed in some deployment scenarios.

- Carriage and container penetration loss (9~20 dB) for logistics application.

- Vegetation loss (e.g., 9 dB) for outdoor application.

Lower elevation angle

As discussed in RAN1#104-e, the central beam center elevation is used for link budget calibration [1].

Figure 3: Illustration of elevation angle.

As shown in Figure 3,

Farthest beam edge elevation < Central beam edge elevation < Central beam center elevation

Thus, a UE in the edge of the farthest beam may suffer from much larger path loss than in the center of the central beam.

Table 6: Summary of FSPL for some other elevation angles.

|  |  | Set-1 |  |  | Set 2 |  |  | Set 3 |  |  | Set 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Satellite orbit | GEO | LEO-1200 | LEO-600 | GEO | LEO-1200 | LEO-600 | GEO | LEO-1200 | LEO-600 | LEO-600 |
|  | Satellite altitude (km) | 35786 | 1200 | 600 | 35786 | 1200 | 600 | 35786 | 1200 | 600 | 600 |
| Elevation angle (deg) | Center of a central beam | 12.5 | 30 | 30 | 20 | 30 | 30 | 20.88 | 46.05 | 43.78 | 90 |
|  | Edge of a central beam | 2.3 | 26.3 | 27 | 11 | 22.2 | 23.8 | 12.5 | 30 | 30 | 30 |
|  | Minimum elevation | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| FSPL (dB) | Center of a central beam | 190.6 | 164.5 | 159.1 | 190.4 | 164.5 | 159.1 | 190.4 | 162.3 | 156.9 | 154.0 |
|  | Edge of a central beam | 190.8 | 165.1 | 159.7 | 190.6 | 165.9 | 160.4 | 190.6 | 164.5 | 159.1 | 159.1 |
|  | Maximum FSPL | 190.6 | 168.4 | 164.2 | 190.6 | 168.4 | 164.2 | 190.6 | 168.4 | 164.2 | 164.2 |

As shown in Table 6, additional 0~10 dB FSPL can be experienced by a UE in locations other than in the center of the central beam.

Observation 3: Additional 0~10 dB FSPL can be experienced by a UE in locations other than in the center of the central beam.

Summary of additional path loss

Based on above discussion on section 2.2.1 and 2.2.2, it is noted that compare with link budget results for calibration, additional path loss should be considered for evaluating the basic coverage performance of IoT NTN in real deployment conditions.

- Carriage and container penetration loss (9~20 dB) for logistics application.

- Vegetation loss (e.g., 9 dB) for outdoor application.

- Additional FSPL (0~10 dB) for lower elevation angle.

Proposal 1: Compare with link budget results for calibration, additional path loss should be considered for evaluating the basic coverage performance of IoT NTN in real deployment conditions.

- Carriage and container penetration loss for logistics application.

- Vegetation loss for outdoor application.

- Additional FSPL for lower elevation angle.

Conclusions

In this contribution, we update the link budget results based on the latest parameters for link budget calibration, and then discuss on additional impact factors for link budget analysis. The observations and proposals are summarized as follows:

Observation 1: Based on the latest parameters for link budget calibration, it can be observed that:

- For GEO with Set 2 satellite parameter, the UL CNR will reach -18.8dB level for NB-IoT with 180kHz BW, and reach -26.5dB level for eMTC with 1080kHz BW.

- For LEO at 1200km with Set 3 satellite parameter, the UL CNR will reach -17.4dB level for NB-IoT with 180kHz BW, and reach -25.2dB level for eMTC with 1080kHz BW.

- For LEO at 600km with Set 4 satellite parameter, the UL CNR will reach -14.9dB level for NB-IoT with 180kHz BW, and reach -22.7dB level for eMTC with 1080kHz BW.

Observation 2: Additional path loss can be observed in some deployment scenarios.

- Carriage and container penetration loss (9~20 dB) for logistics application.

- Vegetation loss (e.g., 9 dB) for outdoor application.

Observation 3: Additional 0~10 dB FSPL can be experienced by a UE in locations other than in the center of the central beam.

Proposal 1: Compare with link budget results for calibration, additional path loss should be considered for evaluating the basic coverage performance of IoT NTN in real deployment conditions.

- Carriage and container penetration loss for logistics application.

- Vegetation loss for outdoor application.

- Additional FSPL for lower elevation angle.

References

- Chairman's Notes RAN1#104-e, January 25th – February 5th, 2021.

- TR 36.763, “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN) (Release 17)”, V0.1.0 (2021-03)

- TR 38.821, “Solutions for NR to support non-terrestrial networks (NTN) (Release 16)”, V16.0.0 (2019-12)

- TR38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”.

- ITU-R, P.833-8, “Attenuation in vegetation”.

- Appendix

Table A-1: Link budget calibration of DL transmissions (2 GHz) for NB-IoT.

| NB-IoT DL |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite orbit | Satellite parameters | UL/DL | TX: EIRP [dBW] | Noise figure [dB] | RX: G/T [dB] | Atmospheric loss [dB] | Free space path loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | BW (kHZ) | CNR [dB] |
| GEO | Set-1 | DL | 51.6 | 9.0 | -33.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 180.0 | -5.0 |
| GEO | Set-2 | DL | 46.1 | 9.0 | -33.6 | 0.2 | 190.4 | 3.0 | 2.2 | 3.0 | 180.0 | -10.3 |
| GEO | Set-3 | DL | 52.4 | 9.0 | -33.6 | 0.2 | 190.4 | 3.0 | 2.2 | 3.0 | 180.0 | -4.0 |
| LEO-1200 | Set-1 | DL | 32.6 | 9.0 | -33.6 | 0.1 | 164.5 | 3.0 | 2.2 | 3.0 | 180.0 | 2.2 |
| LEO-1200 | Set-2 | DL | 26.6 | 9.0 | -33.6 | 0.1 | 164.5 | 3.0 | 2.2 | 3.0 | 180.0 | -3.8 |
| LEO-1200 | Set-3 | DL | 26.3 | 9.0 | -33.6 | 0.1 | 162.3 | 3.0 | 2.2 | 3.0 | 180.0 | -2.0 |
| LEO-600 | Set-1 | DL | 26.6 | 9.0 | -33.6 | 0.1 | 159.1 | 3.0 | 2.2 | 3.0 | 180.0 | 1.6 |
| LEO-600 | Set-2 | DL | 20.6 | 9.0 | -33.6 | 0.1 | 159.1 | 3.0 | 2.2 | 3.0 | 180.0 | -4.4 |
| LEO-600 | Set-3 | DL | 20.9 | 9.0 | -33.6 | 0.1 | 156.9 | 3.0 | 2.2 | 3.0 | 180.0 | -1.9 |
| LEO-600 | Set-4 | DL | 14.0 | 9.0 | -33.6 | 0.1 | 154.0 | 3.0 | 2.2 | 3.0 | 180.0 | -5.9 |

Table A-2: Link budget calibration of DL transmissions (2 GHz) for eMTC.

| eMTC DL |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite orbit | Satellite parameters | UL/DL | TX: EIRP [dBW] | Noise figure [dB] | RX: G/T [dB] | Atmospheric loss [dB] | Free space path loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | BW (kHZ) | CNR [dB] |
| GEO | Set-1 | DL | 59.3 | 9.0 | -33.6 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 1080.0 | -5.0 |
| GEO | Set-2 | DL | 53.8 | 9.0 | -33.6 | 0.2 | 190.4 | 3.0 | 2.2 | 3.0 | 1080.0 | -10.3 |
| GEO | Set-3 | DL | 60.1 | 9.0 | -33.6 | 0.2 | 190.4 | 3.0 | 2.2 | 3.0 | 1080.0 | -4.0 |
| LEO-1200 | Set-1 | DL | 40.3 | 9.0 | -33.6 | 0.1 | 164.5 | 3.0 | 2.2 | 3.0 | 1080.0 | 2.2 |
| LEO-1200 | Set-2 | DL | 34.3 | 9.0 | -33.6 | 0.1 | 164.5 | 3.0 | 2.2 | 3.0 | 1080.0 | -3.8 |
| LEO-1200 | Set-3 | DL | 34.0 | 9.0 | -33.6 | 0.1 | 162.3 | 3.0 | 2.2 | 3.0 | 1080.0 | -2.0 |
| LEO-600 | Set-1 | DL | 34.3 | 9.0 | -33.6 | 0.1 | 159.1 | 3.0 | 2.2 | 3.0 | 1080.0 | 1.6 |
| LEO-600 | Set-2 | DL | 28.3 | 9.0 | -33.6 | 0.1 | 159.1 | 3.0 | 2.2 | 3.0 | 1080.0 | -4.4 |
| LEO-600 | Set-3 | DL | 28.6 | 9.0 | -33.6 | 0.1 | 156.9 | 3.0 | 2.2 | 3.0 | 1080.0 | -1.9 |
| LEO-600 | Set-4 | DL | 21.8 | 9.0 | -33.6 | 0.1 | 154.0 | 3.0 | 2.2 | 3.0 | 1080.0 | -5.9 |

Table A-3: Link budget calibration of UL transmissions (2 GHz) for NB-IoT.

| NB-IoT UL |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite orbit | Satellite parameters | UL/DL | UE power class | TX: EIRP [dBm] | RX: G/T [dB] | Atmospheric loss [dB] | Free space path loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | BW (kHZ) | CNR [dB] |
| GEO | Set-1 | UL | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 180.00 | -13.9 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 3.75 | 2.9 |
| GEO | Set-2 | UL | Class 5 | 20.0 | 14.0 | 0.2 | 190.4 | 3.0 | 2.2 | 3.0 | 180.00 | -18.8 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.4 | 3.0 | 2.2 | 3.0 | 3.75 | -2.0 |
| GEO | Set-3 | UL | Class 5 | 20.0 | 16.7 | 0.2 | 190.4 | 3.0 | 2.2 | 3.0 | 180.00 | -16.0 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.4 | 3.0 | 2.2 | 3.0 | 3.75 | 0.8 |
| LEO-1200 | Set-1 | UL | Class 5 | 20.0 | 1.1 | 0.1 | 164.5 | 3.0 | 2.2 | 3.0 | 180.00 | -5.6 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.1 | 164.5 | 3.0 | 2.2 | 3.0 | 3.75 | 11.2 |
| LEO-1200 | Set-2 | UL | Class 5 | 20.0 | -4.9 | 0.1 | 164.5 | 3.0 | 2.2 | 3.0 | 180.00 | -11.6 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.1 | 164.5 | 3.0 | 2.2 | 3.0 | 3.75 | 5.2 |
| LEO-1200 | Set-3 | UL | Class 5 | 20.0 | -12.8 | 0.1 | 162.3 | 3.0 | 2.2 | 3.0 | 180.00 | -17.4 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.1 | 162.3 | 3.0 | 2.2 | 3.0 | 3.75 | -0.6 |
| LEO-600 | Set-1 | UL | Class 5 | 20.0 | 1.1 | 0.1 | 159.1 | 3.0 | 2.2 | 3.0 | 180.00 | -0.3 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.1 | 159.1 | 3.0 | 2.2 | 3.0 | 3.75 | 16.6 |
| LEO-600 | Set-2 | UL | Class 5 | 20.0 | -4.9 | 0.1 | 159.1 | 3.0 | 2.2 | 3.0 | 180.00 | -6.3 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.1 | 159.1 | 3.0 | 2.2 | 3.0 | 3.75 | 10.6 |
| LEO-600 | Set-3 | UL | Class 5 | 20.0 | -12.8 | 0.1 | 156.9 | 3.0 | 2.2 | 3.0 | 180.00 | -11.9 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.1 | 156.9 | 3.0 | 2.2 | 3.0 | 3.75 | 4.9 |
| LEO-600 | Set-4 | UL | Class 5 | 20.0 | -18.6 | 0.1 | 154.0 | 3.0 | 2.2 | 3.0 | 180.00 | -14.9 |
|  |  |  | Class 5 | 20.0 | -18.6 | 0.1 | 154.0 | 3.0 | 2.2 | 3.0 | 3.75 | 1.9 |

Table A-4: Link budget calibration of UL transmissions (2 GHz) for eMTC.

| eMTC UL |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite orbit | Satellite parameters | UL/DL | UE power class | TX: EIRP [dBm] | RX: G/T [dB] | Atmospheric loss [dB] | Free space path loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | BW (kHZ) | CNR [dB] |
| GEO | Set-1 | UL | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 1080.00 | -21.7 |
|  |  |  | Class 5 | 20.0 | 19.0 | 0.2 | 190.6 | 3.0 | 2.2 | 3.0 | 30.00 | -6.2 |
| GEO | Set-2 | UL | Class 5 | 20.0 | 14.0 | 0.2 | 190.4 | 3.0 | 2.2 | 3.0 | 1080.00 | -26.5 |
|  |  |  | Class 5 | 20.0 | 14.0 | 0.2 | 190.4 | 3.0 | 2.2 | 3.0 | 30.00 | -11.0 |
| GEO | Set-3 | UL | Class 5 | 20.0 | 16.7 | 0.2 | 190.4 | 3.0 | 2.2 | 3.0 | 1080.00 | -23.8 |
|  |  |  | Class 5 | 20.0 | 16.7 | 0.2 | 190.4 | 3.0 | 2.2 | 3.0 | 30.00 | -8.3 |
| LEO-1200 | Set-1 | UL | Class 5 | 20.0 | 1.1 | 0.1 | 164.5 | 3.0 | 2.2 | 3.0 | 1080.00 | -13.4 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.1 | 164.5 | 3.0 | 2.2 | 3.0 | 30.00 | 2.1 |
| LEO-1200 | Set-2 | UL | Class 5 | 20.0 | -4.9 | 0.1 | 164.5 | 3.0 | 2.2 | 3.0 | 1080.00 | -19.4 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.1 | 164.5 | 3.0 | 2.2 | 3.0 | 30.00 | -3.9 |
| LEO-1200 | Set-3 | UL | Class 5 | 20.0 | -12.8 | 0.1 | 162.3 | 3.0 | 2.2 | 3.0 | 1080.00 | -25.2 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.1 | 162.3 | 3.0 | 2.2 | 3.0 | 30.00 | -9.6 |
| LEO-600 | Set-1 | UL | Class 5 | 20.0 | 1.1 | 0.1 | 159.1 | 3.0 | 2.2 | 3.0 | 1080.00 | -8.0 |
|  |  |  | Class 5 | 20.0 | 1.1 | 0.1 | 159.1 | 3.0 | 2.2 | 3.0 | 30.00 | 7.5 |
| LEO-600 | Set-2 | UL | Class 5 | 20.0 | -4.9 | 0.1 | 159.1 | 3.0 | 2.2 | 3.0 | 1080.00 | -14.0 |
|  |  |  | Class 5 | 20.0 | -4.9 | 0.1 | 159.1 | 3.0 | 2.2 | 3.0 | 30.00 | 1.5 |
| LEO-600 | Set-3 | UL | Class 5 | 20.0 | -12.8 | 0.1 | 156.9 | 3.0 | 2.2 | 3.0 | 1080.00 | -19.7 |
|  |  |  | Class 5 | 20.0 | -12.8 | 0.1 | 156.9 | 3.0 | 2.2 | 3.0 | 30.00 | -4.1 |
| LEO-600 | Set-4 | UL | Class 5 | 20.0 | -18.6 | 0.1 | 154.0 | 3.0 | 2.2 | 3.0 | 1080.00 | -22.7 |
|  |  |  | Class 5 | 20.0 | -18.6 | 0.1 | 154.0 | 3.0 | 2.2 | 3.0 | 30.00 | -7.1 |