3GPP TSG RAN WG1 #104b-e	                                                                     R1-2102933

e-Meeting, April 12th – 20th, 2021

Source:	vivo

Title:	Discussion on UE behavior when DCI format 2_0 is not monitored outside DRX active time

Agenda Item:	7.1

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the UE behavior when DCI format 2_0 is not monitored outside DRX active time and provide the text proposal for specification maintenance.

- Discussion

As described in clauses 11.1.1 of TS 38.213 [1], when a UE does not detect a DCI format 2_0, the UE behavior in the slot containing a set of flexible symbols have been defined. Besides, if the monitoring occasion of DCI format 2_0 with CRC scrambled by SFI-RNTI occurs outside DRX active time, UE will not attempt to detect the DCI format 2_0 since it is not required to so according to clause 5.7 of TS 38.321 [2].

5.7 Discontinuous Reception (DRX) The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, and AI-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

However, it should be noted that the case UE is not required to perform the detection of DCI format 2_0 cannot be covered by 38.213 text ‘UE does not detect a DCI format 2_0’ as the sentence is supposed to cover the case where the detection of DCI format 2_0 fails. In the light of the analyses aforementioned, when a UE is not required to monitor DCI format 2_0 outside DRX active time, UE behavior in the slot containing a set of flexible symbols is missing from current specification.

For simplicity, we can adopt the same UE behavior as that of the case ‘UE does not detect a DCI format 2_0’. The associated text proposal (TP) is provided as below:

<Unchanged part omitted> For a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE is not required to monitor DCI format 2_0 providing a slot format for the slot as described in Clauses 5.7 of [11, TS 38.321] or the UE does not detect a DCI format 2_0 providing a slot format for the slot - the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format - the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR - the UE receives PDCCH as described in Clause 10.1 - if the UE is configured by higher layers to receive PDSCH in the set of symbols of the slot, the UE does not receive the PDSCH in the set of symbols of the slot - if the UE is configured by higher layers to receive CSI-RS in the set of symbols of the slot, the UE does not receive the CSI-RS in the set of symbols of the slot, except when UE is provided CO-DurationsPerCell and the set of symbols of the slot are within the remaining channel occupancy duration. - if the UE is configured by higher layers to receive DL PRS in the set of symbols of the slot, the UE receives the DL PRS - if the UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in the set of symbols of the slot and the UE is not provided enableConfiguredUL, the UE - does not transmit the PUCCH, or the PUSCH, or the PRACH in the slot and does not transmit the SRS in symbols from the set of symbols in the slot, if any, starting from a symbol that is after PUSCH preparation time Tproc,2 for the corresponding PUSCH timing capability [6, TS 38.214] assuming d2,1=1 after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0 and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0 - does not expect to cancel the transmission of the SRS, or the PUCCH, or the PUSCH, or the PRACH in symbols from the set of symbols in the slot, if any, starting before a symbol that is after the PUSCH preparation time Tproc,2 for the corresponding PUSCH timing capability [6, TS 38.214] assuming d2,1=1 after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0 and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0 - if the UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in the set of symbols of the slot and the UE is provided enableConfiguredUL, the UE can transmit the SRS, or PUCCH, or PUSCH, or PRACH, respectively. <Unchanged part omitted>

Proposal 1: Agree the TP above for TS 38.213 (from Rel-15) to specify the UE behavior when DCI format 2_0 is not monitored outside DRX active time.

- Conclusion

In the contribution, we provide our view on the UE behavior when DCI format 2_0 is not monitored outside DRX active time, with the following proposal:

Proposal 1: Agree the TP above for TS 38.213 (from Rel-15) to specify the UE behavior when DCI format 2_0 is not monitored outside DRX active time.

Reference

- 3GPP TS 38.213, “NR; Physical layer procedures for control”, v16.5.0 (2021-03).

- 3GPP TS 38.321, “NR; Medium Access Control (MAC) protocol specification”, v16.3.0 (2020-12).