3GPP TSG- RAN WG1 Meeting #104b-e                                                 R1-2103126

# e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.13.1

Source:	Apple Inc.

Title:	Views on Rel-17 DSS SCell scheduling PCell

Document for:	Discussion/Decision

# Introduction

Dynamic spectrum sharing allows NR to be co-channel deployed with LTE, i.e., NR and LTE share the same spectrum, to address the spectrum availability issue from the operator. However, from performance or user experience perspective, it is very hard for DSS deployment to deliver better performance with NR compared to LTE. In Rel-15/16, multiple solutions are adopted to allow DSS in many areas such as (1) symbol level rate matching, including DMRS, mini-slot scheduling, flexible CORESET starting symbol within a slot, etc. (2) RB level rate matching via RRC configuration (3) RE level PDSCH rate matching to LTE.

In the Rel-17 “New WID on NR Dynamic spectrum sharing (DSS)”, the following objective was approved to address the PCell scheduling restriction with DSS

This work item is limited to FR1, and includes the following objectives for NR Dynamic Spectrum Sharing (DSS): - PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2] - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI - The number of cells can be scheduled at once is limited to 2 - The increase in DCI size should be minimized - Note: The total PDCCH blind decoding budget should not be changed as a result of this work - Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

In RAN1#104e meeting, the following agreement was reached [2]

Agreement When CCS from sSCell to PCell/PSCell is configured, - Out of order scheduling is not allowed between a) PDSCH on PCell/PSCell scheduled by PDCCH on PCell/PSCell and b) PDSCH on PCell/PSCell scheduled by PDCCH on sSCell - Out of order scheduling is not allowed between a) PUSCH on PCell/PSCell scheduled by PDCCH on PCell/PSCell and b) PUSCH on PCell/PSCell scheduled by PDCCH on sSCell FFS: Whether this agreement requires RAN1 specification impact. Agreement When CCS from sSCell to PCell/PSCell is configured, - Simultaneous reception of a) unicast PDSCH on PCell/PSCell scheduled from PCell/PSCell and b) unicast PDSCH on PCell/PSCell scheduled from sSCell is not allowed - Simultaneous transmission of a) PUSCH on PCell/PSCell scheduled from PCell/PSCell and b) PUSCH on PCell/PSCell scheduled from sSCell is not allowed - Note: Simultaneous implies full/partial time overlapping FFS: Whether this agreement requires RAN1 specification impact. Agreement - When CCS from sSCell to PCell/PSCell is configured, CA activation/deactivation operation for the sSCell is supported R1-2102101 Summary#3 of Email discussion [104-e-NR-DSS-01] Moderator (Ericsson) Working Assumption - When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 that schedule PDSCH/PUSCH on PCell/PSCell on PCell/PSCell USS set(s), and/or on sSCell USS set(s) - The WA to be confirmed after agreements are made on PDCCH BD/CCE handling and PDCCH overbooking handling for CCS from sSCell to PCell/PSCell - Specs also allow UEs supporting functionality of only Alt-1. Capability signaling details, if any, can be handled during the UE capability discussion for Rel17 - FFS: Whether the UE can monitor PDCCH from both cells in the same slot. Agreement - When CCS from sSCell to PCell/PSCell is configured, UE monitors ‘DCI formats 0_0 and 1_0 in CSS that schedule PDSCH/PUSCH on PCell/PSCell’ only on the PCell/PSCell and not on the sSCell

In this contribution, we provide our views on the objective of PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell.

# Existing PCell scheduling restrictions

## Cross carrier scheduling (CCS) restrictions

In Rel-15 and Rel-16, cross carrier scheduling (CCS) is allowed for both PDSCH and PUSCH, with the following restrictions

- No cross CG cross carrier scheduling

- PCell can only be scheduled by itself

- For each scheduled cell, only one scheduling cell can be RRC configured to schedule it

- A cell that is configured to be scheduled by the other cell, cannot be configured to schedule any cell

- A cell when configured to be scheduling cell, has to support at least self-scheduling

Below is the corresponding specification in 38.331 and 38.213

## PCell PDCCH configurations

In the current specification, there are three ways to configure the PDCCH

- pdcch-ConfigSIB1 in MIB

- PDCCH-ConfigCommon

- PDCCH-Config

In terms of the cross-carrier scheduling, there are the following different DCI Formats excluding the DCI designed for sidelink and IAB

- Fall back DCI, i.e. Format 0_0 and Format 1_0, do not support cross carrier scheduling

- Non-Fall back DCI, i.e. DCI Format 0_1, Format 0_2, Format 1_1 and Format 1_2, support cross carrier scheduling

- Special DCI, i.e. DCI Format 2_0, Format 2_1, Format 2_2, Format 2_3, Format 2_4, Format 2_6, most of which support cross carrier scheduling

In terms of the search space configuration, there are the following possible configurations

- Type0-PDCCH CSS: searchSpaceSIB1, searchSpaceZero

  - Only fall back DCI is allowed

- Type0A-PDCCH CSS: searchSpaceOtherSystemInformation

  - Only fall back DCI is allowed

- Type1-PDCCH CSS: pagingSearchSpace

  - Only fall back DCI is allowed

- Type2-PDCCH CSS: ra-SearchSpace

  - Only fall back DCI is allowed

- Type3-PDCCH CSS

  - Both fall back DCI and special DCI are allowed

- USS

  - Both Non-fall back DCI and Fall back DCI are allowed

It is also important to note that if PCell is required to support initial acquisition, it becomes almost impossible to move all the PDSCH/PUSCH scheduling to SCell due to the lack of RRC connection

## NR CORESET configuration

In DSS, NR CORESET is expected to be configured in the symbols without LTE CRS.

- When LTE has 1 or 2 port CRS, CRS occupies symbol {0, 4, 7, 11}

- When LTE has 4 port CRS, CRS occupies symbol {0, 1, 4, 7, 8, 11}

In Rel-15, FG3-2 was defined to enhance DSS support, i.e., “PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot”. For a UE that supports FG3-2, a DSS NW can configure up to 3 symbol CORESET when LTE has 1or 2 port CRS, or, up to 2 symbol CORESET when LTE has 3 or 4 port CRS

Observation 1: For a UE that supports FG3-2, a DSS NW can configure up to 3 symbol CORESET when LTE has 1 or 2 port CRS, or, up to 2 symbol CORESET when LTE has 3 or 4 port CRS.

# PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell

When sSCell is configured to allow SCell to schedule SpCell together with the SpCell itself, in order to reduce the unnecessary impact on the UE implementation, we have the following views

When CCS from sSCell to PCell/PSCell is configured, the configuration of Type 3 CSS set for DCI formats 2_5 and 2_6 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16.

Proposal 1: When CCS from sSCell to PCell/PSCell is configured, the configuration of Type 3 CSS set for DCI formats 2_5 and 2_6 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16.

When CCS from sSCell to PCell/PSCell is configured, UE cannot be configured to monitor fall back DCI formats 0_0, 1_0 on the sSCell and can be configured to monitor them only on PCell/PSCell, including both CSS (agreed in RAN1#104) and USS.

Proposal 2: When CCS from sSCell to PCell/PSCell is configured, UE cannot be configured to monitor fall back DCI formats 0_0, 1_0 on the sSCell and can be configured to monitor them only on PCell/PSCell including both CSS and USS.

With the above restrictions and the existing agreement, we also need to consider the UE PDCCH monitoring complexity related issues, since we now allow two cells (SpCell and sSCell) to schedule one cell (SpCell), including the following issues

- BC/CCE counting

- Overbooking

- Number of unicast DCI counting

- The maximum number of CCs that UE supports linear scaling of BC and CCE

- Handling of the different SCS of sSCell and SpCell if it is allowed

Proposal 3: When CCS from sSCell to PCell/PSCell is configured, following issues related to UE PDCCH monitoring complexity should be discussed and clarified

•	BC/CCE counting

•	Overbooking

•	Number of unicast DCI counting

•	The maximum number of CCs that UE supports linear scaling of BD and CCE

•	Handling of the different SCS of sSCell and SpCell if it is allowed

# Conclusion

Dynamic spectrum sharing allows NR to be co-channel deployed with LTE, i.e., NR and LTE share the same spectrum, to address the spectrum availability issue from the operator. However, from performance or user experience perspective, it is very hard for DSS deployment to deliver better performance with NR compared to LTE. In the Rel-17 “New WID on NR Dynamic spectrum sharing (DSS)”, one objective was approved to consider PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell.

We have the following observation and proposals

Proposal 1: When CCS from sSCell to PCell/PSCell is configured, the configuration of Type 3 CSS set for DCI formats 2_5 and 2_6 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16.

Proposal 2: When CCS from sSCell to PCell/PSCell is configured, UE cannot be configured to monitor fall back DCI formats 0_0, 1_0 on the sSCell and can be configured to monitor them only on PCell/PSCell including both CSS and USS.

Proposal 3: When CCS from sSCell to PCell/PSCell is configured, following issues related to UE PDCCH monitoring complexity should be discussed and clarified

•	BC/CCE counting

•	Overbooking

•	Number of unicast DCI counting

•	The maximum number of CCs that UE supports linear scaling of BD and CCE

•	Handling of the different SCS of sSCell and SpCell if it is allowed

# Reference

- RP-193260, “New WID on NR Dynamic spectrum sharing (DSS)”, 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019

- Chairman Notes, 3GPP TSG RAN WG1 Meeting #104-e, e-Meeting, January 25th – February 5th, 2021