3GPP TSG RAN1#104-bis-e	R1-2103197

e-Meeting, April 12th – 20th, 2021

Agenda item: 		7.2.11

Source:				Nokia, Nokia Shanghai Bell

Title:	Remaining issues on NR Rel-16 UE features

Document for:		Discussion

# 1	Introduction

In RAN1#104-bis-e the RAN1 Rel-16 UE feature list has been updated in [1]. In this contribution we address the remaining issue regarding MIMO FGs, as summarized in [2].

# 2	Resource counting for MIMO FGs

Conclusion:

- If one resource is used for one or multiple of BFD/RLM, it is counted as one (basic usage1)

- If one resource is used for one or multiple of NBI/PL-RS/RSRP, add 1 (basic usage 2)

- If one resource is used for L1-SINR in addition to basic usage 1 & 2, add N if referred N times by one or more CSI Reporting Settings

In [2], the following conclusion has been agreed upon:

For CSI-RSRP determination CSI reference signals transmitted on antenna port 3000 according to TS 38.211 [4] shall be used. If CSI-RSRP is used for L1-RSRP, CSI reference signals transmitted on antenna ports 3000, 3001 can be used for CSI-RSRP determination.

Unfortunately, RAN1 was unable to conclude on the exact way to capture the conclusions above in the feature list, as some companies still showed concerns on how the resources are counted in some cases. In one attempt to simplify the definition it has been proposed to limit the definition to 1-port RS and leave the other RS definitions open. However, this is incomplete, given that in TS 38.215 [3] CSI-RSRP is defined for 1 and 2 ports:

It is our understanding that the main concern raised by companies is for the cases where more than 1-port RS may be configured, as the UE would then need to process a CDM operation in addition to regular CSI-RS processing. A simple way to address this issue is to consider the CDM operation as one factor in determining the resource counting.

Proposal: Consider the CDM operation as one factor in determining the resource counting.

# 2	Potential ways forward

In the table below we make proposals on how to capture the CDM operation in the feature descriptions. The text in red is from tentative agreements in [2], with modifications in yellow highlights. In the version below, the note is describing only the handling of CSI-RS/CSI-IM, as there is no ambiguity on how SSB is counted.

|  | 16-1a-1 | SSB/CSI-RS for L1-SINR measurement | Per slot limitations: - The max number of SSB/CSI-RS (1Tx) for CMR - The max number of CSI-IM/NZP-IMR resources - The max number of CSI-RS (2Tx) resources for CMR Memory limitations: - The max number of SSB/CSI-RS resources as CMR - The max number of CSI-IM/NZP IMR resources Other limitations: - Supported density of CSI-RS (CMR) - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 - Supported SINR measurements | 2-21, 2-22 or 2-23, 2-23a | Yes | N/A |  | Per band | No | No |  | Component 1: Candidate values {8, 16, 32, 64} Component 2: Candidate values {8, 16, 32, 64} Component 3: Candidate values {0, 4, 8, 16, 32, 64} Component 4: Candidate values {8, 16, 32, 64 , 128} Component 5: Candidate values {8, 16, 32, 64 , 128} Component 6: Candidate values {‘1 only’, ‘3 only’, ‘1 and 3’} Component 7: Candidate values {2, 4, 8, 16, 32, 64} Component 8: Candidate values: bitmap with entries {SSB as CMR with dedicated CSI-IM, SSB as CMR with dedicated NZP IMR, CSI-RS as CMR with dedicated NZP IMR configured, CSI-RS as CMR without dedicated IMR configured} If a UE supports FG 16-1a-1 it must support CMR(CSI-RS) + dedicated CSI-IM Note1: The reference slot duration is the shortest slot duration defined for the FR where the reported band belongs Note2: For component 4 and 5 the configured CSI-RS resources for both active and inactive BWPs are counted Note3: For components 1, 2 and 3, CSI-RS resources configured as CMR without dedicated IMR are counted both as CMR and IMR Note4: For components 1, 2, 3, 7, a SSB/CSI-RS resource is counted within the duration of a reference slot in which the corresponding reference signals are transmitted Note5: For components 1, 2, 3, 7, if one resource used for L1-SINR measurement is referred N times by one or more CSI reporting settings with reportQuantity-r16= ssb-Index-SINR-r16 or cri-SINR-r16, it is counted N times. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16-1g | Resources for beam management, pathloss measurement, BFD, RLM and new beam identification | - The maximum total number of SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification - The maximum total number of SSB/CSI-RS/CSI-IM resources configured across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification | 2-24, 2-31 | Yes | N/A |  | Per UE | No | Yes |  | Component-1: candidate value set is {2, 4, 8, 12, 16, 32, 64, 128} Component-2: candidate value set is {2, 4, 8, 12, 16, 32, 40, 48, 64, 72, 80, 96, 128, 256} Note: For RS configured for new beam identification, they are always counted regardless of beam failure event Note: The “configure to measure” RS (component1) only counts those in active BWP but the configured RS (component2) counts all configured including both active and inactive BWP Note: the reference slot duration is the shortest slot duration defined for the reported FR supported by the UE Note: The “configured to measure” RS is counted within the duration of a reference slot in which the corresponding reference signals are transmitted Note: - If one CSI-RS/CSI-IM resource is used for one or multiple of BFD/RLM, it is counted as (1 x the CDM order for the RS) (basic usage1) - If one CSI-RS/CSI-IM resource is used for one or multiple of NBI (New Beam Identification)/PL-RS/L1-RSRP, add (1 x the CDM order for the RS) (basic usage 2) - L1-RSRP measurement includes cases associated with reports with reportQuantity set to ‘cri-RSRP’ or with reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info is not configured - If one resource is used for L1-SINR in addition to basic usage 1 & 2, add (N times the CDM order for the RS) if referred N times by one or more CSI Reporting Settings with reportQuantity-r16= ‘cri-SINR-r16’ | Optional with capability signaling |
|  | 16-1g-1 | Resources for beam management, pathloss measurement, BFD, RLM and new beam identification across frequency ranges | - The maximum total number of SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification - The maximum total number of SSB/CSI-RS/CSI-IM resources configured across all CCs for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification | 2-24, 2-31, 16-1g | Yes | N/A |  | Per UE | No | No |  | Component-1: candidate value set is {2, 4, 8, 12, 16, 32, 64, 128} Component-2: candidate value set is {2, 4, 8, 12, 16, 32, 40, 48, 64, 72, 80, 96, 128, 256} Note: This FG indicates the maximum number of resources across all FR(s) that are supported by the UE Note: The signalled values apply to the shortest slot duration defined in any FR(s) that are supported by the UE Note: The “configured to measure” RS is counted within the duration of a reference slot in which the corresponding reference signals are transmitted Note: - If one CSI-RS/CSI-IM resource is used for one or multiple of BFD/RLM, it is counted as (1 x the CDM order for the RS) (basic usage1) - If one CSI-RS/CSI-IM resource is used for one or multiple of NBI (New Beam Identification)/PL-RS/L1-RSRP, add (1 x the CDM order for the RS) (basic usage 2) - L1-RSRP measurement includes cases associated with reports with reportQuantity set to ‘cri-RSRP’ or with reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info is not configured - If one resource is used for L1-SINR in addition to basic usage 1 & 2, add (N times the CDM order for the RS) if referred N times by one or more CSI Reporting Settings with reportQuantity-r16= ‘cri-SINR-r16’ | Optional with capability signaling |

Proposal: Adopt the changes above to 16-1a-1, 16-1g, and 16-1g-1.

# 4	Conclusions

In this contribution, we have made the following proposals on the remaining issues of MIMO UE features:

Proposal: Consider the CDM operation as one factor in determining the resource counting.

Proposal: Adopt the changes above to 16-1a-1, 16-1g, and 16-1g-1.

# 5	References

[1]		R1-2102006, “Updated RAN1 UE features list for Rel-16 NR after RAN1#104-e,” Moderators (AT&T, NTT DOCOMO, INC.), Jan. 2021

[2]		R1-2102051, “Summary of email discussion/approval [104-e-NR-UEFeatures-eMIMO-04],” Moderator (AT&T), Febr. 2021

[3]		TS-38.215, “Physical layer measurements”, Jan. 2021