3GPP TSG RAN WG1 Meeting #104bis-e  			    			            R1- 2103331

e-Meeting, April 12th – 20th, 2021

# Source:	ETRI

# Title:		Discussion on resource allocation for power saving

# Agenda item:	8.11.1.1

# Document for:	Discussion

Introduction

In the last meeting, the following conclusion and agreements have been made for resource allocation for power saving [1]:

Agreements: - Random resource selection is applicable to both periodic and aperiodic transmissions - FFS conditions for random resource selection Conclusion: - PSFCH reception is not included for Type A UE - S-SSB reception is not included for Type A UE - SL reception Type B is additionally added - Type B: Same as Type A with an exception of performing PSFCH and S-SSB reception - Note: the same conditions as in RAN1#103-e regarding the context of the discussion of Type A and Type D still apply (also applicable to type B) Agreements: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, where - FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE - The resource selection window is [n+T1, n+T2] - As a baseline, T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4] - Further discuss whether or not to introduce a threshold to re-define T1 and T2 such that - T1 ≥ 0 (subject to processing time constraint Tproc, 1), and T2 ≤ remaining PDB - T2-T1 ≤ (pre-)configured threshold - A minimum value for Y is (pre-)configured from a range of values, FFS details - FFS any restriction to determine Y candidate slots (including its relationship with SL-DRX) - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX - Note: The terminology “periodic-based partial sensing” is based on the “partial sensing” used in LTE-V and it is intended to be used for the design and discussion of partial sensing in Rel-17. Agreements: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, the UE monitors slots of at least one a set of periodic sensing occasions, where a periodic sensing occasion is a set of slots according to if tvSL is included in the set of Y candidate slots. - Preserve is a periodicity value from the configured set of possible resource reservation periods allowed in the resource pool (sl-ResourceReservePeriodList). Down select to one: - Option 1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList - Option 2: PreservePreserve corresponds to a subset of values from the configured set sl-ResourceReservePeriodList - FFS how to determine the subset (e.g., by (pre-)configuration, UE determination) - Option 3: PreservePreserve is a common divisor among values in the configured set sl-ResourceReservePeriodList - Option 4: FFS others - k equals tois selected according to (down select to one) - Option 1: Only the most recent sensing occasion within sensing window for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction - Option 2: The two most recent sensing occasions within sensing window for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction - Option 3: All possible sensing occasions after n –T0 - Option 4: Only one periodic sensing occasion for one reservation period. The k value is up to UE implementation. Max value for k is (pre-)configured. - Option 5: k is (pre-)configured, including multiple values - Option 6: (pre-)configuration of a bitmap, same as in LTE-V - Option 7: FFS others - FFS relationship between periodic sensing occasions and SL-DRX - FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE - Note: companies are encouraged to show performance data for the down selections Agreements: - In a resource pool (pre-)configured with at least partial sensing, if UE performs contiguous partial sensing and resource (re-)selection is triggered in slot n, support the following option: - Option 1: For the purpose of resource (re-)selection, the UE monitors slots between [n+TA, n+TB] and performs identification of candidate resources, in or after slot n+TB, based on all available sensing results, including periodic-based partial sensing results (if applicable). - FFS TA, TB (including the possibility of equal to zero, positive or negative) and remaining details (in particular, whether there should be exclusion of slots, changes in TA/TB values for different purposes, etc.) - FFS whether n can be replaced by e.g., index of some of Y candidate slots - FFS condition(s) in which contiguous partial sensing is performed by UE - FFS interaction with SL-DRX, if any - FFS interaction with periodic-based partial sensing, if any - Other options are not precluded - Note: This option is not to replace random resource selection only without sensing or re-evaluation and pre-emption checking

Based on the above conclusion and agreements, we introduce our views on resource allocation for power saving in this contribution.

Sidelink power saving

Random resource selection and partial sensing

In a resource pool (pre-)configured with at least partial sensing, there are at least two types of partial sensing considering periodic and aperiodic transmissions. One is periodic-based partial sensing, and the other is contiguous partial sensing. In case of periodic-based partial sensing for periodic transmission, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window as in LTE. Regarding the resource selection window [n+T1, n+T2], the same way as in Rel-16 NR V2X is a baseline. Since there is no strong motivation to re-define T1 and T2, it is proposed to confirm that the definitions of T1 and T2 are the same as in Rel-16 NR V2X. Regarding a periodicity value(s) from the configured set of possible resource reservation periods allowed in the resource pool (sl-ResourceReservePeriodList), it is proposed to consider a subset of values from the configured set sl-ResourceReservePeriodList which is larger than a certain threshold value. In Rel-16 NR V2X, there is a broad range of periodicities for periodic reservation, i.e. {(1,…,99), 100, 200, 300, 400, 500, 600, 700, 900, 1000} ms, and up to 16 periodic reservation values can be (pre-)configured per resource pool. Considering periodic transmission with small value(s) of periodicity such as 1 or 2 ms, there is no big difference between periodic transmission with short periodicity and continuous transmission. Therefore, partial sensing for such a periodic transmission with short periodicity can be covered by contiguous partial sensing which will be mentioned later. Since a subset of periodicity values which are larger than a certain threshold value is considered for periodic-based partial sensing, only the most recent sensing occasion within sensing window for a given reservation periodicity configured would be sufficient. Therefore, regarding the periodic-based partial sensing, the followings are proposed:

Proposal 1: For the periodic-based partial sensing,

- It is proposed to confirm the definitions of T1 and T2 as in Rel-16 NR V2X

- It is proposed to consider a subset of values from the configured set sl-ResourceReservePeriodList which are larger than a certain threshold value. FFS for the threshold value, e.g., 100 ms.

- It is also proposed to consider only the most recent sensing occasion, i.e., k = 1 for a given reservation periodicity

In case of contiguous partial sensing, it is proposed for aperiodic transmission. It is also applicable for periodic transmission with short periodicity as mentioned in the above. Therefore, the sensing window [n+TA, n+TB] subject to triggering resource selection in slot n can be configured variously for different purposes. For example, for aperiodic transmission, both TA and TB would be positive values as shown in Figure 1.

Figure 1. Contiguous partial sensing for aperiodic transmission

For periodic transmission with short periodicity, TA or both TA and TB would be negative values, and it could be better to replace n in the definition of the sensing window by the first slot of Y candidate slots in order to keep consistency for sensing procedure of periodic transmission as shown in Figure 2. If a periodicity for periodic transmission is too short, a sensing window could be configured after selection triggering slot n.

Figure 2. Contiguous partial sensing for periodic transmission with short periodicity

In case of random selection, both TA and TB would be equal to zero. Therefore, regarding the contiguous partial sensing, the followings are proposed:

Proposal 2: For the contiguous partial sensing,

- The sensing window [n+TA and n+TB] can be configured variously for different purposes e.g., both TA and TB are positive values for aperiodic transmission, either or both of TA and TB are negative values for periodic transmission with short periodicity, both TA and TB are equal to zero for random selection.

- In case of periodic transmission with short periodicity, it is proposed to replace n in the definition of the sensing window by the first slot of Y candidate slots.

Reception capability of power saving UEs

In the last meeting, in addition to Type-A and Type-D, Type-B was newly introduced. Type-B does not have any reception capability except for PSFCH and S-SSB reception. Since Type-B UE cannot perform sensing operation same as Type-A UE, it is defined for random resource selection. However, the reception capability of Type-B UE can be utilized to improve power saving and reliability. Randomly selected resources will cause resource collision with higher probability than sensing-based selected resources. In case of periodic transmission, the collision might be occurred continuously with other UE’s periodic resource reservation. Since the increasing number of retransmissions due to continuous collision can cause power consumption and deteriorate reliability. In order to avoid continuous collision, the limited reception capability can be utilized. For example, with PSFCH reception capability, Type-B UE can count the number of NACKs and if the number of NACKs is higher than a certain threshold, Type-B UE can re-select resource(s) randomly.

Proposal 3: It is proposed to study how to utilize the limited reception capability of Type-B for random resource selection.

Coexistence of power saving RA scheme(s)

In LTE sidelink, for random resource selection and partial sensing UEs a specific resource pool is configured, and the resource pool is separated from other resource pool(s) for full sensing UEs. However, in Rel-17 NR, considering efficient resource utilization and various use cases, it was agreed that a sidelink mode 2 TX resource pool can be (pre-)configured to support coexistence between power saving UEs and full sensing UEs. In order to avoid potential collision between power saving UEs and full sensing UEs, it is necessary to study potential solutions. For example, full sensing UEs can avoid the resources for power saving UEs by configuring different priorities between them i.e., higher priority for power saving UEs than one for full sensing UEs, or an indication for power saving UEs can be included in 1st or 2nd SCI to avoid resource confliction. By detecting higher priority or indication for power saving UEs, full sensing UEs can avoid the resources for power saving UEs.

Proposal 4: It is proposed to study how to support coexistence between power saving UEs and full sensing UEs in a same resource pool.

Sidelink DRX impacts

Regarding sidelink DRX, if SL DRX wake-up timing between UEs is aligned, it will be beneficial for power saving since the UEs can communicate each other during periodic wake-up time. Even though it seems to be suitable for periodic traffic, it can be extended to aperiodic traffic with additional mechanisms for aperiodic traffic (e.g., wake-up signal). In case of unicast communication, PC5-RRC connection procedure can be considered one of candidate methods to exchange the information on SL DRX configuration. However, in case of groupcast and broadcast communications, another approach/procedure for the delivery of the information on SL DRX should be studied. In addition to data transmission and reception including retransmission subject to SL DRX, it is also needed to study other aspects such as CSI reporting, PSFCH reporting, etc. For example, if a UE receives SL data in SL DRX on-duration time and the corresponding PSFCH is transmitted in SL DRX off-duration time, a proper UE’s behavior should be defined. With respect to CSI reporting, it can be expected a similar situation. Therefore, it is proposed to study potential impacts of SL DRX on power saving.

Proposal 5: It is proposed to study potential impacts of SL DRX on power saving.

Additional enhancements

In addition to random resource selection and partial sensing, additional enhancements for power saving can be considered. First of all, the concept of partial sensing can be extended in different domain (e.g., frequency) rather than time domain. In Uu link, BWP adaptation has been used for power saving via BWP switching. Supporting narrow BWP (and/or reduced resource pool) operation with BWP switching could be one of solutions. For example, in power saving mode, a UE can operate in narrow BWP in order to reduce power consumption for monitoring. When a large amount of data transmission/reception is needed, the UE can switch its BWP into larger one. In order to support BWP switching, it may need to support configuration of multiple SL BWPs, cross-BWP switching, cross-slot switching, SCI only transmission in single subchannel and so on. SL resource pool switching can be also considered instead of SL BWP switching. Since it may cause a heavy specification workload, it should be studied carefully. Even though the main objective of inter-UE coordination is for enhanced reliability and reduced latency in resource allocation mode 2, the inter-UE coordination can be also used for power saving. For example, if a UE selects resource(s) for its own transmission within a white list which is provided as a coordination message, the power consumption for resource selection can be reduced. Other enhancements such as reduced number of blind decoding/estimations, simplified sensing with 1st stage SCI only, etc. could be also studied.

Proposal 6: Additional enhancements besides random resource selection and partial sensing for power saving should be also discussed.

Summary

In this contribution, we made the following proposals for resource allocation for sidelink power saving.

Proposal 1: For the periodic-based partial sensing,

- It is proposed to confirm the definitions of T1 and T2 as in Rel-16 NR V2X

- It is proposed to consider a subset of values from the configured set sl-ResourceReservePeriodList which are larger than a certain threshold value. FFS for the threshold value, e.g., 100 ms.

- It is also proposed to consider only the most recent sensing occasion, i.e., k = 1 for a given reservation periodicity

Proposal 2: For the contiguous partial sensing,

- The sensing window [n+TA and n+TB] can be configured variously for different purposes e.g., both TA and TB are positive values for aperiodic transmission, either or both of TA and TB are negative values for periodic transmission with short periodicity, both TA and TB are equal to zero for random selection.

- In case of periodic transmission with short periodicity, it is proposed to replace n in the definition of the sensing window by the first slot of Y candidate slots.

Proposal 3: It is proposed to study how to utilize the limited reception capability of Type-B for random resource selection.

Proposal 4: It is proposed to study how to support coexistence between power saving UEs and full sensing UEs in a same resource pool.

Proposal 5: It is proposed to study potential impacts of SL DRX on power saving.

Proposal 6: Additional enhancements besides random resource selection and partial sensing for power saving should be also discussed.

References

- Chairman’s notes, RAN1#104-e, January 2021.