3GPP TSG RAN WG1 #104b-e						R1-2103351

e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.3.4

Source: 	LG Electronics

Title: 	Discussion on propagation delay compensation enhancements

Document for:	Discussion and decision

Introduction

In the last e-meeting, following agreement was made [1-2]:

Agreements: - Take 65 ns as the assumption of transmit timing error for evaluation of the overall time synchronization error for control-to-control. - Asymmetry between downlink and uplink channel for smart grid scenario is not considered. - errorBS,DL,TXerrorBS, DL, TX is included in the equation for calculating the overall time synchronization error. Agreements: TA adjustment accuracy is not considered for the evaluation of time synchronization error. Agreements: For evaluation of the overall time synchronization error for smart grid, companies can take one of the following two options as the assumption for BS transmit timing error: - Option 1: 200 ns - Option 2: 65 ns Agreements: Take ±100 ns as the assumption for downlink frame timing detection error (errorUE,DL,RXerrorUE, DL, RX) at the UE for evaluation of the overall time synchronization error for TA based propagation delay compensation, if downlink frame timing detection error needs to be considered separately. - Send a LS to RAN4 to ask for clarification on whether downlink frame timing detection error is included in Te or not - In the LS, to include more details about option 1 (included) & option 2 (not included); also including the necessary background - FFS whether to apply the same value to RTT-based propagation delay compensation, and the corresponding condition (if any) if the same value will be applied

In this contribution, we discuss and provide our views on propagation delay compensation enhancements and necessity of RAN1 involvement.

Discussion

  - Uu interface budget among 5GS synchronicity budget requirement

From previous RAN2 email discussion, synchronicity budget per Uu interface has been agreed as following [2]:

| Scenario | Single Uu interface Budget |
| --- | --- |
| Control-to-Control | ±145ns to ±275ns |
| Smart Grid | ±795ns to ±845ns |

According to description in [3], these values are determined with assumptions such that network-side synchronization for Control-to-control and Smart Grid is based on gPTP and GNSS, respectively. The error caused by the limited granularity of referenceTimeInfo-r16 IE (±5ns) is already included in the other part budget, so that error doesn’t need to be considered.

According to the maximum timing synchronization error results with UE propagation delay compensation summarized in the NR Industrial Internet of Things (IoT) study item, it could meet Uu interface synchronization error budget for Smart Grid scenarios.

Table 1: Summary of maximum timing synchronization error results 
with UE propagation delay compensation from IIoT SI [4].

|  | 15kHz SCS | 30kHz SCS | 60kHz SCS |
| --- | --- | --- | --- |
| Source R1-1900156 [21] | 488ns | 357.5ns | 276.5ns |
| Source R1-1901334 [22] | 505ns | 371ns | 287.5ns |
| Source R1-1900935 [23] | 472.5ns | 338.5ns |  |
| Source R1-1901252 [24] | 536ns | 438ns | 357ns |

Observation #1: It is not necessary to enhance propagation delay compensation for Smart Grid scenario in the RAN1 perspective.

Proposal #1: Take below table as a baseline for synchronicity budget per Uu interface in the control-to-control scenario.

| Scenario | Single Uu interface Budget |
| --- | --- |
| Control-to-Control | ±145ns to ±275ns |

  - Simple analysis of PD compensation

For PD compensation, TA mechanism and RTT based mechanism have been discussed. Figure 1 and 2 show brief synchronization procedure of each mechanism.

| Figure 1. Synchronization procedure of TA mechanism. | Figure 2. Synchronization procedure of RTT mechanism |
| --- | --- |

For TA mechanism, it has been discussed how we derive error component. In short, For DL, PD compensation error budget is consists of three parts; gNB Tx error, PD estimation error and UE Rx error

the total synchronicity error would be following according to previous meetings.

errortotal≤errorBS, DL, TX+errorBS, UL,RX+errorTAindication+errorUE, DL,RX+errorUE, DL,TX2+errorUE, DL, RX

- BS transmit timing error (errorBS, DL, TX):

  - For control-to-control, FFS whether to use ±32.5 ns or 65 ns for the evalation.

  - For smart grid, it was agreed to use 65ns or 200ns for the evaluation

  - We assume errorBS, DL, TX is 65 ns

- BS detecting error (errorBS, UL, RX) : 100 ns

- TA indicating error (errorTA_indication): 8*64*Tc/2 , 130ns or 260ns depending on SCS.

- Downlink frame timing error (errorUE, DL, RX): Work as reference point of Te. We assume 100 ns

- UE Initial transmit timing error (Te) : from 260 ns to 325 ns ((from 8 to 10)*64*Tc )

  - We assume errorUE, DL,TX = Te

Based on the above analysis, total error budget would be up to 360 ns. Considering 275ns error budget of CtC scenario, it would be necessary to enhance TA mechanism for CtC scenario. However, it is also possible to reduce previous assumption by using reduced Te or increasing TA granularity. It is necessary to discuss again if variable in assumption is changed for any reason.

For RTT-based mechanism, the total synchronicity error would be simple as following

errortotal, RTT≤errorBS, DL, TX+errorUE, DL, RX+errorUE, UL, TX+errorBS, UL, RX+errorRTT_indication2

Comparing to TA mechanism, errorRTT_indication is newly added. When UE reports its time stamp to gNB, the accuracy of value is limited to the reporting granularity. Thus, errorRTT_indication refers the reporting granularity of time stamp. In the above analysis, with brief assumption where errorUE, UL, TX is as a half of Te and errorRTT_indication is same as TA indication, total error budget of RTT-based mechanism could be under the 275ns.

Currently, RAN1 sent LS to RAN4 for possibility of enhancement on timing errors. As we can see, some error component take dominant potion in error budget. If it is possible to reduce such error component, the result can change totally. It is highly recommended to re-consider after RAN4 reply LS for PD compensation scheme.

Observation #2: Up to value of errorUE, UL, TX and errorRTT_indication, RTT-based mechanism could meet error budget under 275ns.

Proposal #2: It is necessary to discuss again after RAN4 reply LS is sent if variables in assumption are able to be changed by RAN4.

  - Possible options on RAN1 involvement

We see any kind of RAN1 involvement is necessary in order to support control-to-control scenario. In the last meeting, following two options (actually, four options) has been considered:

- Option 1: TA-based propagation delay

  - Option 1a: Propagation delay estimation based on legacy Timing advance (potentially with enhanced TA indication granularity).

  - Option 1b: Propagation delay estimation based on timing advanced enhanced for time synchronization (as 1a but with updated RAN4 requirements to TA adjustment error and Te)

  - Option 1c: Propagation delay estimation based on a new dedicated signaling with finer delay compensation granularity (Separated signaling from TA so that TA procedure is not affected)

- Option 2: RTT based delay compensation:

  - Propagation delay estimation based on an RAN managed Rx-Tx procedure intended for time synchronization (FFS to expand or separate procedure/signaling to positioning).

Considering synchronization error budget of TA mechanism, Option 1b should be considered as baseline. As mentioned in section 2.2, error portion of initial timing error Te is quite dominant among error components. If it is still not feasible to meet synchronization error budget after minimization of Te, enhanced PD compensation need to be designed.

With the legacy TA granularity which contributes 65ns to sync inaccuracy, UE needs finer TA values to compensate PD precisely and make error budget smaller. Thus, Option 1c also can be considered for a new dedicated signaling with finer delay compensation granularity.

In addition, if PD can be estimated more precisely by Option 2, the Option 2 can be considered to provide more accurate PD update information along with finer PD granularity. For Option 2, we can bring the design studied in positioning WI.

Proposal #3: Take Option 1b as a baseline for PD compensation enhancement. Consider Option 1c or Option 2 for enhanced PD compensation under Option 1b if necessary.

Conclusions

In this contribution, propagation delay compensation enhancements and necessity of RAN1 involvement were discussed, and the followings are proposed.

Observation #1: It is not necessary to enhance propagation delay compensation for Smart Grid scenario in the RAN1 perspective.

Proposal #1: Take below table as a baseline for synchronicity budget per Uu interface in the control-to-control scenario.

Observation #2: Up to value of errorUE, UL, TX and errorRTT_indication, RTT-based mechanism could meet error budget under 275ns.

Proposal #2: It is necessary to discuss again after RAN4 reply LS is sent if variables in assumption are able to be changed by RAN4.

Proposal #3: Take Option 1b as a baseline for PD compensation enhancement. Consider Option 1c or Option 2 for enhanced PD compensation under Option 1b if necessary.

References

RAN1 Chairman notes, RAN1#103-e

RAN1 Chairman notes, RAN1#104-e

TS 22.104 v17.4.0, Service requirements for cyber-physical control applications in vertical domains; Stage 1 (Release 17)

TR 38.825 v16.0.0, Study on NR Industrial Internet of Things (IoT)