3GPP TSG RAN WG1 Meeting #104bis-e	R1-2103467

e-Meeting, April 12th – 20th, 2021

Source:	Sharp

Title:	Remaining issues on resource allocation for NR sidelink

Agenda Item:	7.2.4

Document for:	Discussion and Decision

Introduction

In this document, we share our views on remaining issues on both resource allocation mode 1 and mode 2 for NR sidelink.

Discussion

Resource allocation mode 1

Editorial corrections

A number of editorial corrections are proposed in TP#1-1 and TP#1-2 in section 5.1, on the following points:

- In the second paragraph of section 16.5 of TS 38.213, the sentence “the UE generates one HARQ-ACK information bit in response to the PSFCH receptions to multiplex in a PUCCH transmission occasion…” is prone to be misinterpreted such that it is “the PSFCH receptions” (rather than the actually intended “one HARQ-ACK information bit”) that are to be multiplexed in a PUCCH transmission.

- In section 16.5.1, according to the pseudo code for Type-1 HARQ-ACK codebook generation, “MA” is a notation used for a set of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions. However, some spec text is phrased such that “MA” is the size of the set, e.g. “within the MA occasions for …”, and “according to the corresponding MA occasions”.

- Clarification that for a given “occasion for candidate PSSCH transmission”, there is only one “corresponding PSFCH reception occasion”, i.e. the “occasions” should be replaced with “occasion” in “an occasion for candidate PSSCH transmission with corresponding PSFCH reception occasions” in section 16.5.1.1. Note that “PSFCH reception occasion” is a time-domain concept, and it is not the same as “PSFCH resource” as specified in section 16.3.

- Clarification that there is only one active DL BWP in any configured serving cell, hence “… across active DL BWPs of the configured serving cell…” should be replaced by “… in the active DL BWP of the configured serving cell…”. Note that the wording came from Clause 9 of TS 38.213 where it says “across active DL BWPs of configured serving cells”. Here for SL, with the change of “serving cells” to “serving cell”, i.e. only a single cell is involved, “across active DL BWPs” is not valid any more.

- Clarification that in section 10.1 of TS 38.213, dci-FormatsSL indicates the UE to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1. Note that similar changes relating to dci-FormatsExt were already agreed in RAN1#104-e meeting, see the following RAN1 agreements under agenda item 7.2.5 in [1]:

Agreement Text proposal in section 2.1 (issue A-1) of R1-2102221 for correction on dci-FormatsExt in section 10.1 in TS 38.213 is endorsed in R1-2102222 (TS38.213, Rel-16, CR#0204, Cat. F).

- Correction that the reference of PDCCH validation in clause 8.1.2 of TS 38.214 is clause 10.2A of TS 38.213, instead of clause 10.3 of TS 38.213.

Interpretation of Configuration index field in DCI format 3_0

In NR SL, a “Configuration index” field in DCI format 3_0 was introduced to activate/de-activate SL configured grant Type 2 with the corresponding value of the field. The interpretation of the field is copied as following,

[TS38.212, sub-clause 7.3.1.4.1] - Configuration index – 0 bit if the UE is not configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI; otherwise 3 bits as defined in clause 8.1.2 of [6, TS 38.214]. If the UE is configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI, this field is reserved for DCI format 3_0 with CRC scrambled by SL-RNTI.

It is noticed that if UE is configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI, the 3 bits field of Configuration index is defined in clause 8.1.2 of TS38.214, in which only leads to PDCCH validation for SL configured grant Type 2 specified in clause 10.2A of TS38.213. Therefore, the related contents in TS38.213 is the only place for interpretation of Configuration index field in DCI format 3_0.

[TS38.214, clause 8.1.2] In sidelink resource allocation mode 1: - for PSSCH and PSCCH transmission, dynamic grant, configured grant type 1 and configured grant type 2 are supported. The configured grant Type 2 sidelink transmission is semi-persistently scheduled by a SL grant in a valid activation DCI according to Clause 10.3 of [6, TS 38.213].

[TS38.213, clause 10.2A] A UE validates, for scheduling activation or scheduling release, a SL configured grant Type 2 PDCCH if - the CRC of a corresponding DCI format 3_0 is scrambled with a SL-CS-RNTI provided by sl-CS-RNTI, and - the new data indicator field in the DCI format 3_0 for the enabled transport block is set to '0'

We noticed that in PDCCH validation for SL configured grant Type 2, one of the conditions met is that the NDI field in the DCI format 3_0 is set to ‘0’. If the NDI field is set to ‘1’, and the CRC is scrambled with SL-CS-RNTI, then it means the DCI format 3_0 is for retransmission scheduling of a SL configured grant Type 1/2. It is noted that the interpretation of Configuration index field in DCI format 3_0 for the case of retransmission scheduling for SL CG is not specified yet. In our view, it is straightforward to set the Configuration index field as ‘reserved’ for the above case.

A TP (i.e. TP#2) is provided in section 5.2 for clause 7.3.1.4.1 in TS 38.212.

Priority value of a HARQ-ACK information bit in exceptional cases

It is specified in section 16.5 in TS 38.213 that “the priority value of HARQ-ACK information is same as the priority value of the PSSCH transmissions that is associated with the PSFCH reception occasions providing the HARQ-ACK information”. As identified in our previous contributions e.g. [2], there are a few exceptional cases where it is unclear in the specs which priority value should be used for such a HARQ-ACK bit. However, during the preparation phase of RAN1#104-e meeting it was argued by one company in email thread “[104-e-Prep-NR-5G_V2X] Preparation phase discussion for Rel-16 5G V2X maintenance” that for HARQ-ACK bit with no actual PSSCH transmission, the UE can simply ignore the priority value of such a HARQ-ACK bit, i.e. the UE does not take such a HARQ-ACK bit into account when determining the priority value of the corresponding PUCCH.

Even if we fully follow the argument quoted above in email thread “[104-e-Prep-NR-5G_V2X] Preparation phase discussion for Rel-16 5G V2X maintenance”, it is worth clarifying that in the few cases we listed in [2], there is only one case corresponding to the scenario (i.e. no actual PSSCH transmission) in the above quoted argument. Here we use a simple example to show why there is a problem in other cases, if the priority value of HARQ-ACK information is not clearly specified. As illustrated in Figure 1, assuming Type-1 HARQ-ACK codebook is used, and the set of slot timing values K1 includes values “3” and “5”, according to the 3rd paragraph of sub clause 16.5.1, TS 38.213,

- For PUCCH#1, k=3 corresponds to PSFCH occasion#1 in slot n=11, for which four HARQ-ACK bits will be included in the HARQ-ACK codebook, respectively for candidate PSSCH transmissions in slots #6, #7, #8 and #9. For an actual PSSCH transmission in slot #9, e.g. PSSCH#1 with k=3 indicated in DCI format 3_0, and priority = 0 indicated in the corresponding SCI format 1-A, the HARQ-ACK value obtained from PSFCH reception is copied to the HARQ-ACK bit corresponding to occasion for candidate PSSCH transmission in slot #9. In addition, since the priority value of PUCCH#1 is the smallest priority value of all reported HARQ-ACK bits, it is (correctly) set to 0 here since the priority value of the HARQ-ACK bit corresponding to PSSCH#1 is already 0, i.e. the smallest value in all possible values.

- For PUCCH#2, k=5 corresponds to PSFCH occasion#1 in slot n=11, for which four HARQ-ACK bits will be included in the HARQ-ACK codebook, respectively for candidate PSSCH transmissions in slots #6, #7, #8 and #9. For the same actual PSSCH transmission in slot #9, i.e. PSSCH#1 with k=3 indicated in DCI format 3_0, and priority = 0 indicated in SCI format 1-A, unlike for PUCCH#1, since k=5 used to derive the PSFCH occasion #1 does not match the k=3 indicated in DCI format 3_0, NACK is always set for the HARQ-ACK bit corresponding to occasion for candidate PSSCH transmissions in slot #9. In addition, the priority value of PUCCH#2 is also 0 here since the one corresponding to PSSCH#1 is already 0, i.e. the smallest value in all possible values. This is definitely undesirable, because other HARQ-ACK bits conveyed by PUCCH#2, especially those for which the k indicated in DCI format 3_0 is used to derive the PSFCH occasion, may have a priority value much larger than 0, in which case the intended priority value for PUCCH#2 should have been much larger than 0, i.e. with much lower priority.

Figure 1  Both PUCCH#1 and PUCCH#2 convey a HARQ-ACK bit for PSSCH#1, with the difference that the one in PUCCH#1 is actually intended, and the one in PUCCH#2 is “always set to NACK”; the priority of PUCCH#1 is correctly set to “0” due to a priority value 0 for PSSCH#1, and the priority of PUCCH#2 is (incorrectly) set to “0” (i.e. highest priority) even if other, actually intended, HARQ-ACK bits correspond to PSSCHs with priority greater than 0.

It is thus proposed to clarify in the specs that a largest priority value is used for a HARQ-ACK information bit at least in the above case, for which a TP (i.e. TP#3) is provided in section 5.3.

Monitoring of SL related DCI formats in Type0/0A/1/2-PDCCH CSSs

The following two yellow-highlighted places were first introduced in an initial version of the Editor’s draft CR to TS 38.213 for email approval after RAN1#99, among which the second one was removed in the finally endorsed document R1-1913680 ([3]); the first one was, however, accidentally kept in TS 38.213. One straightforward option is to remove the first one as well, although we think it may be also beneficial to the system if monitoring of DCI formats 3_0/3_1 in Type0/0A/1/2-PDCCH CSSs is instead allowed (e.g. when no SL related USS is configured).

TPs for the two options (i.e. TP#4-1 and TP#4-2) are provided in section 5.4. It is proposed to adopt either TP#4-1 or TP#4-2.

If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and - a C-RNTI, an MCS-C-RNTI, a CS-RNTI, a SL-RNTI, a SL-CS-RNTI, or a SL-L-CS-RNTI the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI, the SL-RNTI, the SL-CS-RNTI, or the SL-L-CS-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI, or P-RNTI.

Slot offset between PSFCH and HARQ-ACK reporting

It is currently specified that the requirement on time offset between the end of the last PSFCH symbol and the start of PUCCH/PUSCH providing corresponding HARQ-ACK information is at least N+1≥14+1=15 symbols for μ=min⁡(μSL,μUL) (see Table 16.5-1 in TS 38.213, reproduced below in Table 1), i.e. larger than 1 slot.

Table 1: Values of N (reproduction of Table 16.5-1 in TS 38.213)

| μ | N |
| --- | --- |
| 0 | 14 |
| 1 | 18 |
| 2 | 28 |
| 3 | 32 |

On the other hand, the slot offset between the last PSFCH reception occasion and the PUCCH/PUSCH providing corresponding HARQ-ACK information, i.e. k, can take a value of zero, “k=0 corresponds to a last slot for a PUCCH transmission that would overlap with the last PSFCH reception occasion assuming that the start of the sidelink frame is same as the start of the downlink frame [4, TS 38.211]”. By definition, in case of k=0, the time offset between last PSFCH symbol and the start of the corresponding PUCCH/PUSCH is always within 1 slot for μ=min⁡(μSL,μUL), i.e. the timing requirement is not fulfilled for k=0.

It is thus proposed to remove the definition of k=0, and clarify that k is always greater than zero. A TP (i.e. TP#5) is provided in section 5.5.

Applicability of HARQ-ACK information reporting for Type-1 and Type-2 HARQ-ACK codebooks

In TS 38.213, section 16.5 provides some general text on how HARQ-ACK information is generated based on PSFCH reception, and section 16.5.1 and section 16.5.2 are respectively for Type-1 and Type-2 HARQ-ACK codebook generation. One problem of the text in section 16.5 is that some text there is only applicable to one of the HARQ-ACK codebook types.

For example, the first yellow-highlighted place below is only true for Type-2 HARQ-ACK codebook where one HARQ-ACK information bit is generated for one time period of a CG. For Type-1 HARQ-ACK codebook the number of HARQ-ACK information bits are generated based on the number of occasions for candidate PSSCH transmissions, and is not necessarily equal to one. It is proposed to fix this in a similar way as the second yellow-highlighted place (i.e. remove “one” and “bit”).

Note that we noticed the following comments in [4]:

“Regarding the above changes, we do not think that these changes are needed. First of all, in our understanding, this section describes how a UE generate valid HARQ-ACK information bits. In this case, even for the semi-static codebook, the UE will generate a single valid HARQ-ACK information bit in resepnse to the PSFCH reception. Therefore, removing “one” is not needed. Company’s concern was that the UE still need to generate HARQ-ACK bit for the non-scheduled part for Type 1-codebook, …”.

Regarding why it is not necessarily “one” bit, using Figure 1 in this document as an example, for PSFCH receptions in PSFCH occasion #1, for Type-1 HARQ-ACK codebook, four (rather than one) bits are always generated, each for one “occasion for PSSCH transmission”.

Note also that the same logic here holds for both dynamic grant and configured grant, and as can be seen below (yellow highlight in the second quoted paragraph below), the spec correctly says “HARQ-ACK information” for dynamic grant, and only (incorrectly) says “one HARQ-ACK information bit” for configured grant.

## 16.5 UE procedure for reporting HARQ-ACK on uplink […] For SL configured grant Type 1 or Type 2 PSSCH transmissions by a UE within a time period provided by sl-PeriodCG, the UE generates one HARQ-ACK information bit in response to the PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource, in a set of time resources. For PSSCH transmissions scheduled by a DCI format 3_0, a UE generates HARQ-ACK information in response to PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource in a set of time resources provided by the DCI format 3_0.

A TP (i.e. TP#6) is provided for sub clause 16.5 of TS 38.213 in section 5.6.

Resource allocation mode 2

Identification of candidate single-slot resources in the sensing procedure

Regarding the time domain location of PSSCH DMRS, it was noticed that the duration of the resources of PSSCH transmission including the duplicated OFDM symbol (i.e. ld) is always larger than or equal to 6 [6]. If the determined ld is less than 6, the slot cannot be used for PSSCH transmission since the DMRS pattern is not specified.

Table 8.4.1.1.2-1: PSSCH DM-RS time-domain location.

| ld in symbols | DM-RS position l |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSCCH duration 2 symbols |  |  | PSCCH duration 3 symbols |  |  |
|  | Number of PSSCH DM-RS |  |  | Number of PSSCH DM-RS |  |  |
|  | 2 | 3 | 4 | 2 | 3 | 4 |
| 6 | 1, 5 |  |  | 1, 5 |  |  |
| 7 | 1, 5 |  |  | 1, 5 |  |  |
| 8 | 1, 5 |  |  | 1, 5 |  |  |
| 9 | 3, 8 | 1, 4, 7 |  | 4, 8 | 1, 4, 7 |  |
| 10 | 3, 8 | 1, 4, 7 |  | 4, 8 | 1, 4, 7 |  |
| 11 | 3, 10 | 1, 5, 9 | 1, 4, 7, 10 | 4, 10 | 1, 5, 9 | 1, 4, 7, 10 |
| 12 | 3, 10 | 1, 5, 9 | 1, 4, 7, 10 | 4, 10 | 1, 5, 9 | 1, 4, 7, 10 |
| 13 | 3, 10 | 1, 6, 11 | 1, 4, 7, 10 | 4, 10 | 1, 6, 11 | 1, 4, 7, 10 |

In a slot, the number of symbols used for sidelink (i.e. sl-LengthSymbols) is (pre-)configured per SL BWP, with the range [7, 14] symbols and PSFCH including the Gap symbol is located in the last three symbols in a sidelink slot. As per the discussion above, we noticed that slots with PSFCH resource(s) if sl-LengthSymbols≤9 are not available for PSSCH transmission, e.g. sl-LengthSymbols=9, then ld=9-3PSFCH-1Gap symbol for PSSCH=5<6. For resource allocation mode 2, MAC layer would randomly select resources for PSSCH transmission, within the subset of resources determined by the physical layer. The worst case might be every resource selected by MAC layer cannot be used since the determination of the candidate single-slot resources (i.e. Step 1 in clause 8.1.4 of TS38.214) does not consider the above case which still regards the slots with PSFCH when sl-LengthSymbols≤9 as candidate resources. In RAN1#104e, companies proposed to specify DMRS patterns for the ld<6 cases, while considering we’re already at maintenance stage for Rel-16 NR V2X, such case is better to be avoided. Meanwhile, one company proposed a solution that such configuration is not expected by the UE. Note that the proposed solution would mean that there is no PSFCH and accordingly no SL A/N in 3 (i.e. sl-LengthSymbols=7,8,9) out of 8 total configurations (i.e. sl-LengthSymbols=[7,14]). Therefore, we propose to exclude the slots with PSFCH when sl-LengthSymbols≤9 in the identification of candidate single-slot resources in the sensing procedure. With the proposed way, resources selected by the MAC layer are always available for PSSCH/PSCCH transmission.

A TP (i.e. TP#7) is provided in section 5.7 for clause 8.1.4 in TS 38.214.

Clarification on the moment for the re-evaluation

In sub-clause 5.22.1.2a of TS38.321, the moment to trigger the re-evaluation is specified as following,

- 5.22.1.2a Re-evaluation and Pre-emption A resource(s) of the selected sidelink grant for a MAC PDU to transmit from multiplexing and assembly entity is re-evaluated by physical layer at T3 before the slot where the SCI indicating the resource(s) is signalled at first time as specified in clause 8.1.4 of TS 38.214 [7].

It was noticed that for a pre-selected resource which would be signalled in slot m, the re-evaluation for the resource is performed at least at the moment m-T3 (i.e. m-Tproc,1SL). From our perspective, the re-evaluation applies for the pre-selected resources within the initial period which are selected and not signalled yet, e.g. a resource in slot (n+T1) is selected for the initial period, wherein n is the slot on which MAC layer triggers the PHY to determine the subset of resources and 0≤T1≤Tproc,1SL as in [7], then, according to the specified moment, the re-evaluation for such resource is performed in slot (n+T1-Tproc,1SL)≤n which means the re-evaluation might even happen before slot n, i.e. the moment on which the resource in slot (n+T1) is selected. In such cases, when MAC layer selects resource(s) for the initial period, the moment of triggering re-evaluation already passes. Therefore, we propose not to perform re-evaluation for such resources. Considering the re-evaluation triggering moment is captured in RAN2 specification, an LS would be needed.

Proposal 1: A resource(s) of the selected sidelink grant is re-evaluated at slot r which is T3 before the slot m where it will be first signalled if slot n when it is triggered to determine the selected sidelink grant is before slot r.

Proposal 2: Send an LS to RAN2 to inform the above agreement.

Conclusion

In this contribution, we discuss remaining issues on both resource allocation mode 1 and mode 2 for NR sidelink, and propose one TP for editorial corrections, and a number of other TPs to clarify the following aspects:

- Interpretation of Configuration index field in DCI format 3_0.

- Priority value of a HARQ-ACK information bit in exceptional cases.

- Monitoring of SL related DCI formats in Type0/0A/1/2-PDCCH CSSs.

- Slot offset between PSFCH and HARQ-ACK reporting.

- Applicability of HARQ-ACK information reporting for Type-1 and Type-2 HARQ-ACK codebooks.

- Identification of candidate single-slot resources in the sensing procedure.

In addition, we make the following proposals:

Proposal 1: A resource(s) of the selected sidelink grant is re-evaluated at slot r which is T3 before the slot m where it will be first signalled if slot n when it is triggered to determine the selected sidelink grant is before slot r.

Proposal 2: Send an LS to RAN2 to inform the above agreement.

References

RAN1#104-e chairman’s notes.

R1-2101533, “Remaining issues on resource allocation for NR sidelink”, Sharp, RAN1#104-e.

R1-1913680, “Introduction of V2X in NR”, Samsung, RAN1#99.

R1-2100515, “Discussion on essential corrections in resource allocation for Mode 1 and 2”, LGE, RAN1#104-e.

TS38.321, “MAC protocol specification”, v16.4.0.

TS38.211, “Physical channels and modulation”, v16.5.0.

TS38.214, “Physical layer procedures for data”, v16.5.0.

Text proposals

TP#1-1 (for TS 38.213) and TP#1-2 (for TS38.214)

-------------------------------------------- Start of TP#1-1 ---------------------------------------

16.5	UE procedure for reporting HARQ-ACK on uplink

< Unchanged parts are omitted >

For SL configured grant Type 1 or Type 2 PSSCH transmissions by a UE within a time period provided by sl-PeriodCG, the UE generates, in response to the PSFCH receptions, one HARQ-ACK information bit to multiplex in a PUCCH transmission occasion that is after a last time resource, in a set of time resources.

< Unchanged parts are omitted >

16.5.1	Type-1 HARQ-ACK codebook determination

This Clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

If a UE is configured a SL configured grant Type 1, and the UE is configured a SL configured grant Type 2 or to monitor PDCCH for detection of DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI, and the UE is provided a set of slot timing values K1 associated with a SL BWP by sl-PSFCH-ToPUCCH and sl-PSFCH-ToPUCCH-CG-Type1, the sl-PSFCH-ToPUCCH-CG-Type1 is one of sl-PSFCH-ToPUCCH.

A UE reports HARQ-ACK information for PSSCH transmissions with corresponding PSFCH reception occasions in slot n only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot n+k, where k is a number of slots indicated by the PSFCH-to-HARQ_feedback timing indicator field in a DCI format 3_0 scheduling the PSSCH transmissions, or by a value of PSFCH-to-HARQ feedback timing indicator field in a DCI format 3_0 activating a SL configured grant Type-2 transmission, or by a value of sl-PSFCH-ToPUCCH for a SL configured grant Type-1. If the UE reports HARQ-ACK information for the PSSCH transmissions with corresponding PSFCH reception occasions in a slot other than slot n+k, the UE sets a value for each corresponding HARQ-ACK information bit to NACK.

If a UE reports HARQ-ACK information in a PUCCH only for

-	PSFCH reception occasions associated with PSSCH transmissions scheduled by a DCI format 3_0 with counter SAI field value of 1, or

-	PSFCH reception occasions associated with PSSCH transmissions corresponding to a SL configured grant

within a set MA of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions as determined in Clause 16.5.1.1, the UE determines a HARQ-ACK codebook only for the PSFCH reception occasion associated with PSSCH transmissions scheduled by a DCI format 3_0 or only for the PSFCH reception occasion associated with PSSCH transmissions corresponding to a SL configured grant according to corresponding set MA of occasions, where a value of a counter SAI in DCI format 3_0 is according to Table 16.5.2.1-1. Otherwise, the procedures in Clause 16.5.1.1 and in Clause 16.5.1.2 for a HARQ-ACK codebook determination apply.

16.5.1.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a SL BWP on a serving cell c and an active UL BWP on the primary cell, as described in Clause 12, a UE determines a set MA of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on:

a)	a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-PSFCH-ToPUCCH for DCI format 3_0 or by sl-PSFCH-ToPUCCH-CG-Type1

b)	the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively

c)	a set of sidelink resource pool bitmaps

d)	a value of a period of PSFCH transmission occasion resources for each sidelink resource pool provided by a respective sl-PSFCH-Period

For the set of slot timing values K1, the UE determines a set MA of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code.

< Unchanged parts are omitted >

The cardinality of the set MA defines a total number M of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions corresponding to the HARQ-ACK information bits. A UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of  OACK HARQ-ACK information bits as ojACK = HARQ-ACK information bit for candidate PSSCH transmission with index j with corresponding PSFCH reception, for  0 ≤j<M, as described in Clause 16.5. If the UE does not transmit a PSSCH in an occasion for candidate PSSCH transmission with corresponding PSFCH reception occasion, due to the UE not detecting a corresponding DCI format 3_0, the UE generates a NACK value for the occasion for candidate PSSCH transmission with corresponding PSFCH reception occasion.

If OACK≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in Clause 7.2.1, as nHARQ-ACK=m=0M-1Nmreceived where Nmreceived is a number of HARQ-ACK information bits determined for corresponding PSSCH transmissions with corresponding PSFCH reception occasions in PSFCH reception occasion m.

16.5.1.2	Type-1 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by DCI format 0_0, then

-	if the UE

-	has not received any PDCCH with a DCI format 3_0 scheduling PSSCH transmissions with corresponding PSFCH reception occasions that the UE transmits corresponding HARQ-ACK information in the PUSCH, based on a value of a respective PSFCH-to-HARQ_feedback timing indicator field in a DCI format scheduling the PSSCH transmissions or on the value of PSFCH-to-HARQ feedback timing indicator field in a DCI format 3_0 activating a SL configured grant Type 2 transmission, or

-	has not been provided PSSCH resources with corresponding PSFCH reception occasions that the UE transmits corresponding HARQ-ACK information based on the value of sl-PSFCH-ToPUCCH-CG-Type1 for a SL configured grant Type 1, in any of the set MA of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions, as described in Clause 16.5.1.1,

the UE does not multiplex HARQ-ACK information in the PUSCH transmission;

-	else the UE generates the HARQ-ACK codebook as described in Clause 16.5.1.1, unless the UE generates HARQ-ACK information only for

-	PSFCH reception occasions associated with PSSCH transmissions corresponding to a SL configured grant, or

-	PSFCH reception occasions associated with PSSCH transmissions that are scheduled by DCI format 3_0 with a counter SAI field value of 1 in the set MA of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions,

in which case the UE generates HARQ-ACK information only for the PSFCH reception occasions as described in Clause 16.5.1.

A UE sets to NACK value in the HARQ-ACK codebook any HARQ-ACK information corresponding to PSFCH reception occasions associated with PSSCH transmissions scheduled by a DCI format 3_0 that the UE detects in a PDCCH monitoring occasion that starts after a PDCCH monitoring occasion where the UE detects a DCI format scheduling the PUSCH transmission.

If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by a DCI format that includes a SAI field, the UE generates the HARQ-ACK codebook as described in Clause 16.5.1.1 when a value of the SAI field in the DCI format is VT-SAIUL=1. The UE does not generate a HARQ-ACK codebook for multiplexing in the PUSCH transmission when VT-SAIUL=0 unless the UE generates HARQ-ACK information only for

-	PSFCH reception occasions associated with PSSCH transmissions corresponding to a SL configured grant, or

-	PSFCH reception occasions associated with PSSCH transmissions that are scheduled by a DCI format 3_0 with a counter SAI field value of 1 in the set MA of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions as described in Clause 16.5.1.

VT-SAIUL=0 if the SAI field in the DCI format is set to '0'; otherwise, VT-SAIUL=1.

16.5.2	Type-2 HARQ-ACK codebook determination

This Clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic.

16.5.2.1	Type-2 HARQ-ACK codebook in physical uplink control channel

A UE determines monitoring occasions for PDCCH with DCI format 3_0 for scheduling PSSCH transmissions with associated PSFCH reception occasions on an active DL BWP of a serving cell c, as described in Clause 10.1, and for which the UE transmits HARQ-ACK information in a same PUCCH in slot n based on

-	PSFCH-to-HARQ_feedback timing indicator field values, or a value provided by sl-PSFCH-ToPUCCH-CG-Type1, for PUCCH transmission with HARQ-ACK information in slot n in response to PSFCH receptions

-	Time gap field in DCI format 3_0 for scheduling PSSCH transmissions with associated PSFCH receptions

-	Time resource assignment in DCI format 3_0 for scheduling PSSCH transmissions with associated PSFCH receptions

-	A set of configured sidelink resource pool bitmaps

-	A value of a period of PSFCH resources provided in sl-PSFCH-Period.

-	A value of a minimum time gap provided in sl-MinTimeGapPSFCH.

The set of PDCCH monitoring occasions for DCI format 3_0 for scheduling PSSCH transmissions with associated PSFCH reception occasions is defined as the PDCCH monitoring occasions in the active DL BWP of the configured serving cell, indexed in ascending order of start time of the associated search space sets. The cardinality of the set of PDCCH monitoring occasions defines a total number M of PDCCH monitoring occasions.

< Unchanged parts are omitted >

if a SL configured grant Type 1 is configured for a UE, or a SL configured grant Type 2 is configured and activated for a UE, and the SL configured grant provides a grant for PSSCH transmissions with PSFCH reception occasions in a slot n-K1, where K1 is the PSFCH-to-HARQ-feedback timing value for the SL configured grant

OACK=OACK+1;

oOACK-1ACK= HARQ-ACK information bit associated with the PSFCH reception occasions associated with the PSSCH transmissions scheduled by the SL configured grant

end if

< Unchanged parts are omitted >

10.1	UE procedure for determining physical downlink control channel assignment

< Unchanged parts are omitted >

-	if search space set s is a CSS set

-	an indication by dci-Format0-0-AndFormat1-0 to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0

-	an indication by dci-Format2-0 to monitor one or two PDCCH candidates, or to monitor one PDCCH candidate per RB set if the UE is provided freqMonitorLocations for the search space set, for DCI format 2_0 and a corresponding CCE aggregation level

-	an indication by dci-Format2-1 to monitor PDCCH candidates for DCI format 2_1

-	an indication by dci-Format2-2 to monitor PDCCH candidates for DCI format 2_2

-	an indication by dci-Format2-3 to monitor PDCCH candidates for DCI format 2_3

-	an indication by dci-Format2-4 to monitor PDCCH candidates for DCI format 2_4

-	an indication by dci-Format2-6 to monitor PDCCH candidates for DCI format 2_6

-	if search space set s is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or an indication by dci-FormatsExt to monitor PDCCH candidates for DCI format 0_2 and DCI format 1_2, or for DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2, or an indication by dci-FormatsSL to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1

< Unchanged parts are omitted >

-------------------------------------------- End of TP#1-1 ----------------------------------------

-------------------------------------------- Start of TP#1-2 ---------------------------------------

8.1.2	Resource allocation

In sidelink resource allocation mode 1:

-	for PSSCH and PSCCH transmission, dynamic grant, configured grant type 1 and configured grant type 2 are supported. The configured grant Type 2 sidelink transmission is semi-persistently scheduled by a SL grant in a valid activation DCI according to Clause 10.2A of [6, TS 38.213].

< Unchanged parts are omitted >

-------------------------------------------- End of TP#1-2 ----------------------------------------

TP#2 (for TS 38.212)

-------------------------------------------- Start of TP#2 -------------------------------------------

< Unchanged parts are omitted >

7.3.1.4.1   Format 3_0

DCI format 3_0 is used for scheduling of NR PSCCH and NR PSSCH in one cell.

The following information is transmitted by means of the DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI:

< Unchanged parts are omitted >

-	Configuration index – 0 bit if the UE is not configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI; otherwise 3 bits as defined in clause 8.1.2 of [6, TS 38.214]. If the UE is configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI, this field is reserved for DCI format 3_0 with CRC scrambled by SL-RNTI and for DCI format 3_0 with CRC scrambled by SL-CS-RNTI wherein the DCI format 3_0 indicates retransmission(s) as defined in [8, TS 38.321].

< Unchanged parts are omitted >

-------------------------------------------- End of TP#2 --------------------------------------------

TP#3 (for TS 38.213)

-------------------------------------------- Start of TP#3 -------------------------------------------

< Unchanged parts are omitted >

16.5.1	Type-1 HARQ-ACK codebook determination

< Unchanged parts are omitted >

A UE reports HARQ-ACK information for PSSCH transmissions with corresponding PSFCH reception occasions in slot n only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot n+k, where k is a number of slots indicated by the PSFCH-to-HARQ_feedback timing indicator field in a DCI format 3_0 scheduling the PSSCH transmissions, or by a value of PSFCH-to-HARQ feedback timing indicator field in a DCI format 3_0 activating a SL configured grant Type-2 transmission, or by a value of sl-PSFCH-ToPUCCH for a SL configured grant Type-1. If the UE reports HARQ-ACK information for the PSSCH transmissions with corresponding PSFCH reception occasions in a slot other than slot n+k, the UE sets a value for each corresponding HARQ-ACK information bit to NACK, and the priority value of the NACK is same as the largest priority value among the possible priority values.

< Unchanged parts are omitted >

-------------------------------------------- End of TP#3 --------------------------------------------

TP#4-1 (for TS 38.213) and TP#4-2 (for TS 38.213)

------------------------------------------- Start of TP#4-1------------------------------------------

< Unchanged parts are omitted >

10.1	UE procedure for determining physical downlink control channel assignment

< Unchanged parts are omitted >

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and

-	a C-RNTI, an MCS-C-RNTI, or a CS-RNTI

the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI, or P-RNTI.

< Unchanged parts are omitted >

------------------------------------------ End of TP#4-1 --------------------------------------------

------------------------------------------- Start of TP#4-2------------------------------------------

< Unchanged parts are omitted >

10.1	UE procedure for determining physical downlink control channel assignment

< Unchanged parts are omitted >

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and

-	a C-RNTI, an MCS-C-RNTI, a CS-RNTI, a SL-RNTI, a SL-CS-RNTI, or a SL Semi-Persistent Scheduling V-RNTI

the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, the CS-RNTI, or for DCI format 3_0 with CRC scrambled by the SL-RNTI, or the SL-CS-RNTI, or for DCI format 3_1 with CRC scrambled by the SL Semi-Persistent Scheduling V-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI, or P-RNTI.

< Unchanged parts are omitted >

------------------------------------------ End of TP#4-2 --------------------------------------------

TP#5 (for TS 38.213)

-------------------------------------------- Start of TP#5 -------------------------------------------

< Unchanged parts are omitted >

16.5	UE procedure for reporting HARQ-ACK on uplink

< Unchanged parts are omitted >

A UE does not expect to be provided PUCCH resources or PUSCH resources to report HARQ-ACK information that start earlier than N+1∙2048+144∙κ∙2-μ∙Tc after the end of a last symbol of a last PSFCH reception occasion, from a number of PSFCH reception occasions that the UE generates HARQ-ACK information to report in a PUCCH or PUSCH transmission, where

-	κ and Tc are defined in [4, TS 38.211]

-	μ=min⁡(μSL,μUL), where μSL is the SCS configuration of the SL BWP and μUL is the SCS configuration of the active UL BWP on the primary cell

-	N is determined from μ according to Table 16.5-1

Table 16.5-1: Values of N

| μ | N |
| --- | --- |
| 0 | 14 |
| 1 | 18 |
| 2 | 28 |
| 3 | 32 |

With reference to slots for PUCCH transmissions and for a number of PSFCH reception occasions ending in slot n, the UE provides the generated HARQ-ACK information in a PUCCH transmission within slot n+k, subject to the overlapping conditions in Clause 9.2.5, where k>0, and k is a number of slots indicated by a PSFCH-to-HARQ_feedback timing indicator field, if present, in a DCI format indicating a slot for PUCCH transmission to report the HARQ-ACK information, or k is provided by sl-PSFCH-ToPUCCH-CG-Type1.k=0

< Unchanged parts are omitted >

-------------------------------------------- End of TP#5 --------------------------------------------

TP#6 (for TS 38.213)

-------------------------------------------- Start of TP#6 -------------------------------------------

< Unchanged parts are omitted >

16.5	UE procedure for reporting HARQ-ACK on uplink

< Unchanged parts are omitted >

For SL configured grant Type 1 or Type 2 PSSCH transmissions by a UE within a time period provided by sl-PeriodCG, the UE generates HARQ-ACK information in response to the PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource, in a set of time resources.

For PSSCH transmissions scheduled by a DCI format 3_0, a UE generates HARQ-ACK information in response to PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource in a set of time resources provided by the DCI format 3_0.

< Unchanged parts are omitted >

-------------------------------------------- End of TP#6 --------------------------------------------

TP#7 (for TS 38.214)

-------------------------------------------- Start of TP#7 -------------------------------------------

8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

< Unchanged parts are omitted >

1)	A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource except the ones in the slots with PSFCH resources when sl-LengthSymbols≤9, where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL  , where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

The total number of candidate single-slot resources is denoted by Mtotal.

< Unchanged parts are omitted >

-------------------------------------------- End of TP#7 --------------------------------------------