3GPP TSG RAN WG1 Meeting #104b-e                                                                 	   R1-2103535

e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.6.1.2

Source:	Lenovo, Motorola Mobility

Title:	Reduced number or Rx branches for RedCap

Document for:	Discussion and decision

# Introduction

In RAN#91e, the RedCap WID [1] was updated and agreed. One objective is to specify support of reduced number of Rx branches for RedCap UEs,

- For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands.

- For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands.

- A means shall be specified by which the gNB can know the number of Rx branches of the UE.

In this contribution, we provide views on the reduced number of Rx branches based on the WID description.

# Discussion

In the WID, it was clarified that for UL, the coverage enhancement solutions specified in the NR Coverage Enhancement WI are available also to RedCap UEs by default.

- Uplink coverage enhancement solutions specified in the NR Coverage Enhancement WI (NR_cov_enh) shall be assumed to be available also to RedCap UEs by default (with small modifications for RedCap UEs if found necessary).

While for DL, there is no clarification whether coverage recovery is still in the WI scope. From the RedCap SI outcome, there is clear need of coverage recovery for PDCCH CSS and Msg4 for the scenario of 1Rx and 24 dBm/MHz DL PSD. In addition, RAN1#103e determined two alternatives for coverage recovery,  either single coverage recovery target for initial access and non-initial access, or two coverage targets for initial access and non-initial access, respectively. The choice of the alternative impacts which DL and UL channel needs to have coverage recovery.

Proposal 1: A clarification is needed on whether DL coverage recovery is in the scope of RedCap WI.

It was agreed in the RedCap WID to specify one RedCap UE type,

- Specify definition of one RedCap UE type including capabilities for RedCap UE identification and for constraining the use of those RedCap capabilities only for RedCap UEs, and preventing RedCap UEs from using capabilities not intended for RedCap UEs including at least carrier aggregation, dual connectivity and wider bandwidths. [RAN2, RAN1]

Since both 1Rx and 2Rx branches are supported in each frequency band, the single RedCap UE type definition will not contain a specific number of Rx branches.

Proposal 2: The single RedCap UE type definition does not contain a specific number of Rx branches.

The gNB could anyway obtain the number of Rx branches based on UE capability reporting. Besides, it is beneficial to obtain the number of Rx branches early for e.g., better link adaptation for DL channel in initial access. An identification could then be performed for RedCap UEs with 1Rx based on e.g., dedicated PRACH occasions or PRACH preambles. Alternatively, considering the purpose of early identification, it is reasonable to identify RedCap UEs with poor channel conditions instead of based on number of Rx branches. It is noted in CE WI, there is similar discussion on the possibility of allocating PRACH resources for poor channel conditions.

Proposal 3: Support early identification of RedCap UEs with 1Rx branches or with poor channel conditions in Msg1.

As mentioned, there has been discussion in CE WI on identification for UEs supporting UL CE for Msg3, with one option of using separate PRACH resources. Since the solutions defined in UL CE WI applicable also for RedCap UEs by default, one issue is whether needs to differentiate RedCap UEs and non-RedCap UEs with or without Rel.17 CE. In our understanding, differentiation is needed since the network shall identify the RedCap UEs to get the capability of reduced bandwidth and potentially reduced number of Rx branches. These are relevant for the proper data scheduling and transmission during initial access [2].

Proposal 4: UE identification is needed to differentiate RedCap UEs from non-RedCap UEs with or without Rel.17 CE.

There was concern regarding the potential degradation of end user performance and spectral efficiency of the network with the introduction of 1Rx RedCap UEs. Therefore, it was agreed in the WID to specify a system information indication for barring the RedCap UEs,

- Specify a system information indication to indicate whether a RedCap UE can camp on the cell/frequency or not; it shall be possible for the indication to be specific to the number of Rx branches of the UE. [RAN2, RAN1]

Similar with the discussion in early identification, it is be more reasonable to restrict the access of RedCap UEs based on the undergoing channel condition than based on number of Rx branches, since serving 1Rx UEs with good channel condition will not sacrifice the system performance.

Proposal 5: Barring of RedCap UEs could be based on DL channel condition, instead of simply based on number of Rx branches.

# Conclusions

As a summary, we have the following proposals on reduced number of Rx branches for RedCap UEs,

Proposal 1: A clarification is needed on whether DL coverage recovery is in the scope of RedCap WI.

Proposal 2: The single RedCap UE type definition does not contain a specific number of Rx branches.

Proposal 3: Support early identification of RedCap UEs with 1Rx branches or with poor channel conditions in Msg1.

Proposal 4: UE identification is needed to differentiate RedCap UEs from non-RedCap UEs with or without Rel.17 CE.

Proposal 5: Barring of RedCap UEs could be based on DL channel status, instead of simply based on number of Rx branches.

# References

RP-210918, “Revised WID on support of reduced capability NR devices”, RAN#91e

R1-2103534, “Reduced maximum UE bandwidth for RedCap”, Lenovo, Motorola Mobility, RAN1#104e